# USB DAC + Balanced Headphone Amplifier — Design Specification v0.1

Product requirements and architecture for a personal PCB project

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 23 Sep 2026 | v0.1 (draft for review) | Product requirements + architecture |

# 1. Purpose and scope

This specification defines the requirements and architecture of a USB-C Hi-Res DAC with a single DAC chip and a balanced headphone amplifier able to drive the AKG K1000 and the HIFIMAN Susvara.

Portfolio project: an end-to-end, industrial-style PCB workflow (spec → schematic → layout → JLCPCB fabrication and assembly (PCBA) → bring-up → automated measurement → revision) aimed at the NVIDIA “PCB Design EDA and AI Engineer – New College Graduate 2027” role (Taipei/Hsinchu). The posting emphasises PCB design, Python/EDA scripting, GitLab CI/CD, AI agents and DFF/DFA/DFT, so design automation is a first-class deliverable, not an extra.

Goal: “best measured performance for the money”, proven by published measurements; not “beats every product”, which a hobby analyzer cannot prove (QuantAsylum itself says its QA403 won’t match a US$30,000 analyzer).

| In scope for v1 | Out of scope for v1 |
| --- | --- |
| USB-C Hi-Res DAC with a single AK4493SEQ DAC chip | Paralleled DACs (possible v2 experiment) |
| Balanced headphone amplifier (4.4 mm) plus a 6.35 mm single-ended output | 4-pin XLR output |
| Relay-based headphone protection | Hardware volume knob and display |
| Design automation: CI checks, BOM cost gate, DFF/DFA/DFT rules, AI datasheet review, automated measurement | Enclosure design (3D-printed case for prototypes only) |

# 2. Key targets

All values below are proposed targets for v0.1; every board revision is measured against this table (Section 8).

| Parameter | Target | Minimum acceptable | Conditions | How verified |
| --- | --- | --- | --- | --- |
| PCM sample rate | 44.1–384 kHz, up to 32-bit | 384 kHz | USB Audio Class 2.0 | Host playback sweep at every rate |
| DSD (stretch goal) | DoP | Optional | — | Host playback |
| Drivers | Driverless on macOS/Linux; Windows 10/11 built-in USB Audio Class 2 driver | — | — | Host playback on each OS |
| THD+N at 1 kHz | ≤ −110 dB | ≤ −105 dB | Balanced, 2 Vrms, 300 Ω load, low gain, 20 Hz–20 kHz bandwidth | QA403 analyzer via Python |
| Dynamic range | ≥ 120 dB(A) | ≥ 117 dB(A) | Balanced, low gain | QA403 |
| Output power (Variant B) | ≥ 1.0 W into 120 Ω; ≥ 1.2 W into 60 Ω | — | Balanced, high gain, 1 kHz | QA403 + dummy loads |
| Output impedance | < 0.5 Ω | — | 1 kHz | Load/no-load method, 32 Ω and 300 Ω |
| Frequency response | ±0.1 dB, 20 Hz–20 kHz | — | 48 kHz and 384 kHz | QA403 |
| Crosstalk at 1 kHz | ≤ −100 dB (300 Ω); ≤ −90 dB (32 Ω) | — | — | QA403 |
| Channel balance | ±0.1 dB | — | — | QA403 |
| Output DC offset | ≤ ±2 mV | — | — | — |
| Protection | Output relay closes 2–3 s after power is good; opens within 100 ms on output DC > ±1 V or loss of power | — | — | Oscilloscope |
| Power | Data port ≤ 500 mA at 5 V (USB 2.0 budget); amplifier power from a separate USB-C PD port at 15 V | — | ≥ 20 W charger | — |
| Thermal | All parts within ratings | — | 30 min at full rated power, 0–40 °C ambient | Thermocouple / IR camera |
| PCB | 4 layers, ≤ 100 × 70 mm, all SMT parts on top side, 90 Ω controlled-impedance USB pair | — | — | ERC/DRC and layout-rule checks in CI (Section 8) |
| Cost | ≈ 1,000 NTD per assembled board | — | 100 units | BOM cost gate on every commit (Section 8) |

# 3. Architecture

## 3.1 Signal and control path

Figure 1. Signal and control path (Variant B output stage shown).

The USB-C data port brings USB 2.0 High-Speed audio in through ESD protection; its 5 V bus powers the digital section.

The XMOS XU316 runs XMOS’s free sw_usb_audio reference firmware (USB Audio Class 2.0), boots from an external QSPI flash, uses a 24 MHz crystal, sends I²S/DSD audio plus MCLK to the DAC and controls it over I²C.

Two audio master-clock oscillators (22.5792 MHz for the 44.1 kHz family, 24.576 MHz for the 48 kHz family) feed both the XU316 and the DAC; the XU316 selects the oscillator.

The AKM AK4493SEQ delivers differential L/R outputs and applies host volume in its hardware attenuator.

The output stage (4 × TI OPA1622) makes each output leg a difference amplifier plus low-pass filter, with two op-amps paralleled per leg through small ballast resistors; a 2-position switch sets the gain.

A delay/DC-detect protection chip drives the relays on all four output legs (Section 5.3).

The 4.4 mm balanced jack carries all four legs; the 6.35 mm single-ended jack uses the + legs only.

## 3.2 Power path

The digital section is bus-powered from the USB-C data port (5 V, within the 500 mA USB 2.0 budget): a 0.9 V buck feeds the XU316 core and a 3.3 V LDO feeds the XU316 I/O and the DAC digital supply.

The analog section runs from a separate USB-C power port: a USB PD sink chip requests 15 V from a ≥ 20 W charger and feeds a +12 V low-noise LDO, an inverting buck-boost converter to about −14 V followed by a −12 V low-noise LDO, and a +5 V low-noise LDO for the DAC analog supply (VDDL/R).

If the PD charger is absent, the analog section stays off and the outputs stay muted.

Figure 2. Power path.

## 3.3 Volume and gain structure

Volume comes from the host via USB Audio Class and is applied in the AK4493SEQ’s hardware attenuator (255 levels, 0.5 dB steps plus mute); a hardware knob is deferred to v2. The DAC full scale is ≈ 2 Vrms differential per channel, and the 2-position gain switch sets the output stage as follows.

| Gain setting | Differential gain | Balanced full-scale output | Notes |
| --- | --- | --- | --- |
| High gain | 5.48× (+14.8 dB) | 10.96 Vrms (5.48 Vrms, 7.75 V peak per leg) | Fits within the ±12 V rails |
| Low gain | 1× (0 dB) | 2 Vrms | For sensitive headphones |

# 4. Key parts

| Part | Role | Key specs | Package / assembly note | Unit price @100 | Source |
| --- | --- | --- | --- | --- | --- |
| XMOS XU316-1024-QF60B | USB audio | USB 2.0 High-Speed audio with XMOS’s free sw_usb_audio reference firmware, up to 384 kHz; core 0.855–0.945 V, 3.3 V I/O; ~300 mA typical; 0–70 °C commercial grade; boots from external QSPI flash | 60-pin QFN, 7 × 7 mm, 0.4 mm pitch; leadless, X-ray fee | US$9.21 | Digi-Key |
| AKM AK4493SEQ | DAC | 32-bit stereo; PCM up to 768 kHz, DSD512; S/N 123 dB (2 Vrms), 125 dB (2.7 Vrms), 128 dB (mono mode); THD+N −115 dB (2 Vrms), −110 dB (2.7 Vrms); differential voltage output; supplies VDDL/R 4.75–5.25 V, TVDD/AVDD 3.0–3.6 V; I²C, 3-wire or pin control | 48-pin LQFP, 7 × 7 mm; leaded, no X-ray fee | US$6.08 | Mouser: 2,824 in stock, 24-week factory lead time (Sept 2026). LCSC lists AK4493SEQ-L; to be confirmed |
| TI OPA1622 (×4 in Variant B, ×2 in Variant A) | Output stage | Dual op-amp; ±2 V to ±18 V supply; output current +145 mA / −130 mA; THD+N −119 dB at 142 mW into 32 Ω; shutdown pin | 10-pin VSON, 3 × 3 mm, with thermal pad; leadless, X-ray fee | US$1.59 | LCSC / TI |
| WCH CH224K | USB PD sink | Requests 5/9/12/15/20 V. Supply: now listed at LCSC as discontinued with minimal stock; use a newer WCH PD sink chip (e.g., CH224Q), confirm at schematic stage | JLCPCB Extended part | US$0.28 | LCSC |

# 5. Headphone drive

## 5.1 Load requirements

Maker specifications set the drive requirement. Current, not voltage, is the limit: 129 mA peak sits right at the OPA1622’s −130 mA output-current rating, so the design limit is 100 mA peak per op-amp (about 75% of the rating).

| Headphone | Impedance, sensitivity | Design point | Voltage | Peak current | Result |
| --- | --- | --- | --- | --- | --- |
| AKG K1000 | 120 Ω, 74 dB/mW | Rated 1 W | 10.95 Vrms, 15.5 V peak | 129 mA | ≈ 104 dB SPL |
| HIFIMAN Susvara | 60 Ω, 83 dB/mW | 0.5 W for 110 dB SPL peaks | 5.48 Vrms, 7.76 V peak | 129 mA | 110 dB SPL peaks |

## 5.2 Output-stage variants

Maximum power per channel (balanced, high gain), limited by 100 mA per op-amp or by the high-gain full scale:

| Load | Variant A: 2 × OPA1622, 1 op-amp per leg | Variant B: 4 × OPA1622, 2 paralleled per leg |
| --- | --- | --- |
| AKG K1000 (120 Ω) | 0.60 W → 101.8 dB SPL | 1.00 W → 104.0 dB SPL (full rated power) |
| HIFIMAN Susvara (60 Ω) | 0.30 W → 107.8 dB SPL | 1.20 W → 113.8 dB SPL |
| 300 Ω (voltage-limited by high-gain full scale) | 0.40 W | 0.40 W |
| 32 Ω | 0.16 W | 0.64 W |
| Decision | Do-not-fit option: the PCB has footprints for all 4 op-amps | Baseline: meets both headphone goals |

## 5.3 Power, thermal and protection

Driving the K1000 at 1 W on both channels draws ≈ 3.9 W from the ±12 V rails (≈ 164 mA average per rail). Variant A would dissipate ≈ 1 W per OPA1622; Variant B spreads this over four chips (≈ 0.5 W each). Each thermal pad gets a copper pour and a thermal-via array.

Protection is required, because these headphones are expensive and hard to replace:

| Function | Implementation | Requirement |
| --- | --- | --- |
| Power-on delay | Relays on all 4 output legs, driven by the protection chip | Close 2–3 s after power is good |
| DC-offset detect | Protection chip opens the relays on a DC fault | Open within 100 ms on output DC > ±1 V |
| Power-loss disconnect | Protection chip opens the relays when power is lost | Open within 100 ms on loss of power |
| Pop-free start and stop | OPA1622 shutdown pin | — |
| Bring-up safety | 60 Ω and 120 Ω dummy resistors only | Never test on real headphones |

# 6. Cost budget

Planning allocations at 100 units, 31.7 NTD per USD. Main-chip prices are distributor quotes; other lines are estimates, to be replaced by live LCSC prices in the automated BOM check.

| Item | NTD per board |
| --- | --- |
| Main chips, Variant B: XU316 + AK4493SEQ + 4 × OPA1622 | 686 |
| QSPI boot flash | 8 |
| 24 MHz crystal | 3 |
| 2 audio oscillators | 38 |
| USB PD sink chip | 10 |
| Digital power (0.9 V buck + 3.3 V LDO) | 13 |
| Analog power (inverting converter, ±12 V and 5 V regulators) | 38 |
| Headphone protection (protection chip + 2 relays) | 44 |
| Connectors (2 × USB-C, 4.4 mm, 6.35 mm) | 51 |
| USB ESD protection | 6 |
| Passives (~120 parts) | 48 |
| Gain switch and LEDs | 10 |
| 4-layer bare PCB | 25 |
| JLCPCB assembly fees (amortised) | 60 |
| Other items total (estimates) | 353 |
| Total, Variant B (≈ 4% over the 1,000 NTD target) | ≈ 1,040 |
| Total, Variant A (main chips 585 with 2 × OPA1622) | ≈ 939 |

Line items are rounded to whole NTD, so they do not add exactly to the subtotal. Excluded: enclosure, shipping and import tax.

Prototype note: 5 prototype boards with JLCPCB Economic PCBA cost ≈ 2,300 NTD in assembly fees alone (≈ 460 NTD per board), before parts, bare PCB and shipping. This assumes ~15 Extended part types and 3 leadless chips per board.

| Cost-down lever (to bring Variant B to ≤ 1,000 NTD) | Estimated saving | Note |
| --- | --- | --- |
| Use the XU316’s internal application PLL for the audio clock instead of the two oscillators | ≈ 38 NTD | Measure the jitter impact, which is itself a good experiment |
| Drop the 6.35 mm single-ended jack | ≈ 15 NTD | — |
| Replace JLCPCB Extended parts with Basic library parts wherever possible | Not yet quantified | Each unique Extended part type adds a US$3.07 loading fee (Section 7) |

# 7. Design for JLCPCB (DFF/DFA/DFT rules)

These rules apply to every revision; the ones that can be checked automatically also run in CI (Section 8).

| ID | Type | Rule |
| --- | --- | --- |
| R01 | DFA | Prefer Basic parts: each unique Extended part type adds a US$3.07 loading fee. |
| R02 | DFA | Economic PCBA places parts on one side only, so all SMT parts go on the top side. |
| R03 | DFA | Each leadless chip (QFN/VSON) adds an X-ray inspection fee. Here the XU316 and OPA1622 are leadless; the AK4493SEQ is leaded. |
| R04 | DFA | Turn on “Confirm Parts Placement” (US$0.45) and check the 3D preview for rotation and polarity errors. |
| R05 | DFF/DFA | More than 30 boards on Economic PCBA must be ordered as one panel of ≤ 250 × 250 mm; otherwise use Standard PCBA. |
| R06 | DFA | Through-hole connectors need hand soldering at extra cost; prefer SMT-mount jacks where available. |
| R07 | DFT | Test point on every power rail and on the key clock and I²S nets. |
| R08 | DFT | I²S input header, so the DAC and amplifier can be tested on their own (the QA403 can drive I²S directly). |
| R09 | DFT | Fiducials and polarity marks. |
| R10 | DFF | Use JLCPCB’s standard 4-layer impedance-controlled stack-up for the 90 Ω USB pair; solid ground plane; digital and analog areas separated. |

# 8. Automation and verification plan

The part most relevant to the NVIDIA role: every check runs automatically with an explicit pass/fail rule.

| Check | Runs when | Pass/fail rule |
| --- | --- | --- |
| ERC and DRC (KiCad command line) | Every commit (GitLab CI) | Zero errors |
| Manufacturing outputs: Gerbers, drill, BOM, pick-and-place, schematic PDF, for both variants | Every commit | Files generated |
| BOM check | Every commit | Every part has an LCSC number and is in stock; count of Extended parts and leadless chips reported; cost at 5 and 100 units; FAIL if the 100-unit cost > 1,000 NTD |
| ~10 Python DFF/DFA/DFT and layout rules | Every commit | All rules pass, e.g., no digital traces through the analog area, USB pair length-matched, decoupling capacitor near every IC power pin, test point on every rail, fiducials present |
| AI datasheet-review agent | Every merge request | Reads the datasheets for the DAC, amplifier and regulators, extracts requirements, checks the schematic and comments on the MR. Scored on 10–20 deliberately planted errors (catch rate and false alarms) |
| Automated measurement suite (Python + QuantAsylum QA403, ~US$600, or a school-lab analyzer) | Each board revision | THD+N, dynamic range, frequency response, crosstalk, output impedance, max power at 32/60/120/300 Ω; auto-generated report compared against Section 2 |

# 9. Risks and open decisions

## 9.1 Risks

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Variant B budget ≈ 4% over target | Cost target | Cost-down levers (Section 6); automated cost gate |
| OPA1622 output current at its limit | Clipping or distortion at high SPL | Variant B paralleling; 100 mA design limit |
| AK4493SEQ supply: 24-week factory lead time; LCSC stock unconfirmed | Schedule | Buy the prototype quantity early; consign parts to JLCPCB if needed |
| CH224K discontinued at LCSC | Redesign | Choose the successor PD chip at the schematic stage |
| XU316 0.4 mm-pitch QFN | Layout and assembly yield | Follow the XMOS reference layout; X-ray inspection |
| Two USB ports: possible ground-loop noise from the PC | Noise floor | Measure; USB isolation as a v2 option |
| Headphone damage during bring-up | Very costly | Relay protection; dummy loads only |

## 9.2 Open decisions (to close during schematic)

| Decision | Options and notes |
| --- | --- |
| Rail regulators | Low-cost LM317/LM337-class, or lower-noise TI TPS7A4901/TPS7A3001, whose 150 mA / 200 mA limits may be too low for Variant B at full power |
| Clock source | Two oscillators, or the XU316 internal PLL (saves ≈ 38 NTD; jitter to be measured) |
| PD sink chip | Successor to the CH224K, e.g., CH224Q |
| Connectors | Add a 4-pin XLR output? (out of scope for v1 as currently drafted) |
| Volume control | Hardware volume knob in v2 |

# 10. Next steps (schematic phase, weeks 1–3)

| # | Action | Done |
| --- | --- | --- |
| 1 | Create the GitLab repository and KiCad project with the CI skeleton (ERC/DRC + outputs) before drawing anything. | ☐ |
| 2 | Download datasheets and reference material: AK4493SEQ datasheet and AKD4493S evaluation-board manual (AKM website, request form); XU316 datasheet and XMOS sw_usb_audio reference design; OPA1622 datasheet. | ☐ |
| 3 | Draw the schematic block by block (power → USB/XU316 → clocks → DAC → output stage → protection) and write down the calculations for each block. | ☐ |
| 4 | Assign an LCSC part number to every part and run the BOM cost gate. | ☐ |
| 5 | Close the open decisions in Section 9.2. | ☐ |
| 6 | Get a schematic review from an experienced engineer, then start layout. | ☐ |
| 7 | Apply to the NVIDIA posting once the repository and CI are public. | ☐ |

# Appendix A. Reference documents

| Ref | Document | Source |
| --- | --- | --- |
| [1] | AK4493SEQ datasheet | AKM |
| [2] | AKD4493S evaluation-board manual | AKM website (request form) |
| [3] | XU316-1024-QF60B datasheet | XMOS |
| [4] | sw_usb_audio reference firmware and reference design | XMOS |
| [5] | OPA1622 datasheet | Texas Instruments |

# Appendix B. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Draft for review |