# USB DAC + Balanced Headphone Amplifier — Design Specification v0.2

Product requirements and architecture for a personal PCB project

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 23 Sep 2026 | v0.2 (draft for review) | Product requirements + architecture |

Companion document: “Schematic Design Notes v0.1” holds the block-level circuit guidance; this specification states the requirements and architecture it implements.

# 1. Purpose and scope

This specification defines the requirements and architecture of a USB-C Hi-Res DAC with a single DAC chip and a balanced headphone amplifier able to drive the AKG K1000 and the HIFIMAN Susvara. The outputs are a 3.5 mm unbalanced jack and a 4.4 mm balanced jack, for one headphone at a time; K1000 and Susvara owners need a 4.4 mm cable or a 4.4 mm-to-4-pin-XLR adapter.

Portfolio project: an end-to-end, industrial-style PCB workflow (spec → schematic → layout → JLCPCB fabrication and assembly (PCBA) → bring-up → automated measurement → revision) aimed at the NVIDIA “PCB Design EDA and AI Engineer – New College Graduate 2027” role (Taipei/Hsinchu). The posting emphasises PCB design, Python/EDA scripting, GitLab CI/CD, AI agents and DFF/DFA/DFT, so design automation is a first-class deliverable, not an extra.

Goal: “best measured performance for the money”, proven by published measurements; not “beats every product”, which a hobby analyzer cannot prove (QuantAsylum itself says its QA403 won’t match a US$30,000 analyzer).

Budget: ≤ 1,500 NTD per assembled board at 100 units is acceptable; 1,000 NTD is no longer a hard limit (Section 6).

| In scope for v1 | Out of scope for v1 |
| --- | --- |
| USB-C Hi-Res DAC with a single AK4493SEQ DAC chip | Paralleled DACs (possible v2 experiment) |
| Balanced headphone amplifier: 4.4 mm balanced jack plus 3.5 mm unbalanced jack (one headphone at a time) | 4-pin XLR output (use a 4.4 mm cable or a 4.4 mm-to-4-pin-XLR adapter) |
| Relay-based headphone protection on both jacks | Hardware volume knob and display |
| Design automation: CI checks, BOM cost gate, DFF/DFA/DFT rules, AI datasheet review, automated measurement | Enclosure design (3D-printed case for prototypes only) |

# 2. Key targets

All values below are proposed targets for v0.2, checked against the AK4493S, AKD4493S-A and OPA1622 datasheets; every board revision is measured against this table (Section 8). Design estimates are labelled as such.

| Parameter | Target | Minimum | Conditions | How verified |
| --- | --- | --- | --- | --- |
| PCM sample rate | 44.1–384 kHz, up to 32-bit | 384 kHz | USB Audio Class 2.0 | Host playback sweep at every rate |
| DSD (stretch goal) | DoP | Optional | — | Host playback |
| Drivers | Driverless on macOS/Linux; Windows 10/11 built-in USB Audio Class 2 driver | — | — | Host playback on each OS |
| THD+N at 1 kHz | ≤ −110 dB | ≤ −105 dB | 0 dBFS, low gain (4 Vrms balanced), 300 Ω load, 20 Hz–20 kHz bandwidth | QA403 via Python. Basis: AK4493S datasheet −115 dB typ (−105 dB max); AKD4493S-A evaluation board measured −114.6/−113.8 dB (L/R) at 44.1 kHz |
| Dynamic range | ≥ 120 dB(A) | ≥ 117 dB(A) | Both gains, both jacks | QA403. Design estimate ≈ 121 dB(A) (DAC 123 dB typ, 118 dB min) |
| Output power, 4.4 mm balanced | ≥ 1.0 W into 120 Ω; ≥ 1.2 W into 60 Ω | — | High gain, 1 kHz | QA403 + dummy loads |
| Output power, 3.5 mm unbalanced | ≥ 0.5 W into 60 Ω; ≥ 0.6 W into 32 Ω | — | High gain, 1 kHz | QA403 + dummy loads |
| Output impedance | < 0.5 Ω | — | 1 kHz, both jacks | Load/no-load method, 32 Ω and 300 Ω |
| Frequency response | +0.1/−0.3 dB, 20 Hz–20 kHz | — | 44.1, 48 and 384 kHz sample rates | QA403. Basis: DAC digital filter + switched-capacitor filter alone +0.1/−0.2 dB at 44.1 kHz; analog filter adds ≈ −0.06 dB at 20 kHz |
| Crosstalk at 1 kHz | Balanced ≤ −100 dB (300 Ω); unbalanced ≤ −75 dB (32 Ω, at the jack) | — | Unbalanced limited by the shared ground return of a 3-pole plug | QA403 |
| Channel balance | ≤ 0.3 dB; optional per-unit firmware trim to ±0.05 dB | — | — | QA403. Basis: DAC inter-channel gain mismatch 0.15 dB typ / 0.3 dB max |
| Output DC offset | ≤ ±5 mV (design estimate 0.7 mV unbalanced / 1.5 mV balanced typical; worst case ≈ 5 mV / 11 mV) | — | Both jacks | Multimeter at the jack; a DC-servo footprint is provided as a fallback |
| Protection | Relays close 2–3 s after power is good and open within 100 ms on a fault | — | Both jacks; fault = output DC > ±1 V or loss of power | Oscilloscope |
| Power | Data port ≤ 500 mA at 5 V; amplifier from a separate USB-C PD port at 15 V; maximum PD draw ≈ 10.5 W (0.70 A at 15 V) | — | ≥ 30 W charger (only chargers above 27 W must offer 15 V under USB PD power rules; many 18–20 W phone chargers do not) | Measure the PD draw at 1.2 W into 60 Ω, both channels |
| Thermal | All parts within ratings | — | 30 min at full rated power, 0–40 °C ambient | Thermocouple / IR camera |
| PCB | 4 layers, ≤ 100 × 70 mm, all SMT parts on the top side, 90 Ω controlled-impedance USB pair | — | ≥ 70 × 70 mm with edge rails and fiducials, as JLCPCB Standard assembly requires | ERC/DRC and layout-rule checks in CI (Section 8) |
| Cost | ≤ 1,500 NTD per assembled board | — | 100 units | Automated BOM cost gate at 1,500 NTD on every commit (Section 8) |

# 3. Architecture

## 3.1 Signal and control path

Figure 1. Signal and control path.

The USB-C data port brings USB 2.0 High-Speed audio in through ESD protection; its 5 V bus powers the digital section.

The XMOS XU316-1024-QF60A runs XMOS’s free sw_usb_audio reference firmware (USB Audio Class 2.0). All of its I/O is 1.8 V only: it boots from a 1.8 V QSPI flash, takes a 24 MHz crystal, and sends I²S/DSD audio and I²C control to the DAC at 1.8 V.

Two 1.8 V oscillators with output enable supply MCLK: 22.5792 MHz for the 44.1 kHz family and 24.576 MHz for the 48 kHz family. The MCLK net also reaches XU316 pin X1D11, so the XU316’s internal application PLL can generate MCLK instead, an experiment that needs no rework.

The AKM AK4493SEQ runs with its internal LDO disabled and TVDD/AVDD/DVDD at 1.8 V; it delivers differential L/R outputs and applies host volume in its hardware attenuator.

The DAC outputs are AC-coupled into Stage 1: one TI OPA1612 per channel, wired as a unity-gain difference amplifier plus low-pass filter.

In the output stage, each channel has a leg+ (non-inverting, 1× or 2.74× set by the gain switch) and a leg− (unity inverter of leg+). Each leg is one OPA1622 with both halves paralleled through 0.47 Ω ballast resistors.

A protection chip drives relays on all four legs (Section 5.3). The 4.4 mm balanced jack carries L+, L−, R+, R− and GND; the 3.5 mm unbalanced jack carries L+, R+ and GND.

## 3.2 Power path

The digital section is bus-powered from the USB-C data port (5 V, within the 500 mA USB 2.0 budget): a ≥ 1 A 0.9 V buck feeds the XU316 core (up to 830 mA maximum per datasheet), a 3.3 V LDO the USB PHY analog supply, and a 1.8 V LDO the XU316 I/O, USB 1.8 V supply, DAC TVDD and flash. A quiet 1.8 V LDO for the DAC AVDD/DVDD and the oscillators is enabled after the main 1.8 V rail.

The analog section runs from a separate USB-C power port: a WCH CH224Q requests 15 V from a ≥ 30 W charger and feeds a +12 V LDO (LM317-class) and an inverting buck-boost converter (≥ 36 V, ≥ 1.5 A buck, e.g., TI LMR36015/TPS54360-class) to about −14 V, followed by a −12 V LDO (LM337-class). An ADI LT3045 ultra-low-noise LDO makes +5 V from +12 V for the DAC VDDL/R and VREFHL/R, through 10 Ω + 470 µF per AKM.

Without PD power the analog section stays off, the DAC is held in reset and the relays stay open.

Figure 2. Power path.

## 3.3 Volume and gain structure

Volume comes from the host via USB Audio Class and is applied in the AK4493SEQ’s hardware attenuator (255 levels, 0.5 dB steps plus mute); a hardware knob is deferred to v2. The DAC full scale is ≈ 2 Vrms differential (±2.8 Vpp per datasheet). Stage 1 has unity gain, so 2 Vrms reaches the output stage, where the 2-position gain switch sets leg+ as follows; leg− mirrors it, doubling the balanced level.

| Gain setting | Leg+ gain | 3.5 mm unbalanced full scale | 4.4 mm balanced full scale | Notes |
| --- | --- | --- | --- | --- |
| High gain | 2.74× (+8.8 dB) | 5.48 Vrms | 10.96 Vrms (7.75 V peak per leg) | Fits within the ±12 V rails |
| Low gain | 1× (0 dB) | 2.0 Vrms | 4.0 Vrms | For sensitive headphones |

# 4. Key parts

| Part | Role | Key specs | Package / assembly note | Unit price @100 | Source |
| --- | --- | --- | --- | --- | --- |
| XMOS XU316-1024-QF60A-C24 | USB audio | 16 cores, 600 MHz; USB 2.0 High-Speed PHY; all I/O 1.8 V only; core 0.855–0.945 V; core current 225 mA typ, 830 mA max; USB needs a 24 MHz (or 12 MHz) clock; boots from QSPI flash; 0–70 °C; MSL3 | 60-pin QFN, 7 × 7 mm, 0.4 mm pitch; leadless, X-ray fee | US$9.21 | Digi-Key: 563 in stock, 12-week lead time; LCSC C5375653 out of stock |
| AKM AK4493SEQ | DAC | 32-bit; PCM 8–768 kHz; DSD64–512; THD+N −115 dB typ (−105 dB max); DR 123 dB typ (118 dB min); S/N 125 dB (large-amplitude mode); channel isolation 120 dB; output ±2.8 Vpp differential (≈ 2 Vrms); load ≥ 450 Ω, ≤ 25 pF; MCLK 2.048–49.664 MHz; characterized at TVDD = AVDD = DVDD = 1.8 V, VDDL/R = VREFH = 5.0 V | 48-pin LQFP; leaded, no X-ray fee | US$6.08 | Mouser: 2,824 in stock, 24-week factory lead time |
| TI OPA1622 (× 4) | Output legs | ±2 to ±18 V; short-circuit current +145/−130 mA; THD+N −123 dB (10 mW, 128 Ω), −116 dB (10 mW, 32 Ω); 2.8 nV/√Hz; offset ±0.5 mV max; enable threshold ≈ 0.82 V referenced to its GND pin (1.8 V logic OK); RθJA 47.6 °C/W; thermal pad to V− | 10-pin VSON, 3 × 3 mm; leadless, X-ray fee | US$1.59 | LCSC / TI |
| TI OPA1612 | Stage-1 difference amp + filter | 1.1 nV/√Hz; ±2.25 to ±18 V; the op-amp on AKM’s AKD4493S-A evaluation board | SOIC-8; leaded | US$0.88 | LCSC C94590 (in stock) |
| ADI LT3045 | DAC 5 V supply | 20 V input, 500 mA, 0.8 µVrms noise | 10-pin DFN, 3 × 3 mm; leadless | US$7.15 (JLCPCB); US$5.31 @121 (LCSC) | JLCPCB / LCSC |
| WCH CH224Q | USB PD sink | PD 3.2; 5/9/12/15/20/28 V; resistor-configurable; replaces the CH224K (discontinued at LCSC) | 10-pin DFN, 2 × 2 mm; leadless | US$0.26 | LCSC C46061833: out of stock (9 Sep 2026); JLCPCB pre-order |

# 5. Headphone drive

## 5.1 Load requirements

Maker specifications set the drive requirement. Current, not voltage, is the limit: 129 mA peak sits right at the OPA1622’s −130 mA output-current rating, so the design limit is 100 mA peak per op-amp (about 75% of the rating), or 200 mA per leg with both halves of an OPA1622 paralleled.

| Headphone | Impedance, sensitivity | Design point | Voltage | Peak current | Result |
| --- | --- | --- | --- | --- | --- |
| AKG K1000 | 120 Ω, 74 dB/mW | Rated 1 W | 10.95 Vrms, 15.5 V peak | 129 mA | ≈ 104 dB SPL |
| HIFIMAN Susvara | 60 Ω, 83 dB/mW | 0.5 W for 110 dB SPL peaks | 5.48 Vrms, 7.76 V peak | 129 mA | 110 dB SPL peaks |

## 5.2 Output power by jack and gain

Maximum power per channel at full scale, limited by the design limit of 200 mA peak per paralleled leg or by the full-scale voltage; SPL from the maker sensitivity:

| Load | Balanced, high gain (10.96 Vrms full scale) | Balanced, low gain (4.0 Vrms) | Unbalanced, high gain (5.48 Vrms) | Unbalanced, low gain (2.0 Vrms) |
| --- | --- | --- | --- | --- |
| AKG K1000 (120 Ω) | 1.00 W (104.0 dB SPL) | 0.13 W (95.2 dB SPL) | 0.25 W (98.0 dB SPL) | 0.03 W (89.2 dB SPL) |
| HIFIMAN Susvara (60 Ω) | 1.20 W (113.8 dB SPL) | 0.27 W (107.3 dB SPL) | 0.50 W (110.0 dB SPL) | 0.07 W (101.2 dB SPL) |
| 300 Ω | 0.40 W | 0.05 W | 0.10 W | 0.01 W |
| 32 Ω | 0.64 W | 0.50 W | 0.64 W | 0.12 W |
| 16 Ω | 0.32 W | 0.32 W | 0.32 W | 0.25 W |

## 5.3 Power, thermal and protection

Thermal budget per OPA1622 (one chip per leg, ±12 V rails, RθJA 47.6 °C/W): the K1000 at 1 W balanced dissipates 0.61 W per chip (+29 °C), the Susvara at 1.2 W balanced 1.05 W (+50 °C), and the unbalanced worst case, 32 Ω at maximum power, 1.01 W (+48 °C). At 1.2 W into 60 Ω on both channels the rails supply ≈ 388 mA (+12 V) and ≈ 283 mA (−12 V). Each thermal pad (tied to V−) gets a copper pour and a thermal-via array.

Protection is required, because these headphones are expensive and hard to replace. The relays cover both jacks:

| Function | Implementation | Requirement |
| --- | --- | --- |
| Power-on delay | Relays on all 4 output legs (both jacks), driven by the protection chip | Close 2–3 s after power is good |
| DC-offset detect | Protection chip opens the relays on a DC fault | Open within 100 ms on output DC > ±1 V |
| Power-loss disconnect | Protection chip opens the relays when power is lost | Open within 100 ms on loss of power |
| Pop-free start and stop | OPA1622 enable pins, driven from 1.8 V logic | — |
| Bring-up safety | 60 Ω and 120 Ω dummy resistors only | Never test on real headphones |

# 6. Cost budget

Budget at 100 units, 31.7 NTD per USD, excluding enclosure, shipping and import tax. Quoted lines use the distributor prices in Section 4; allocations are estimates.

| Item | Basis | NTD per board |
| --- | --- | --- |
| XMOS XU316-1024-QF60A-C24 | Quote: US$9.21 (Digi-Key) | 292 |
| AKM AK4493SEQ | Quote: US$6.08 (Mouser) | 193 |
| 4 × TI OPA1622 | Quote: 4 × US$1.59 (LCSC/TI) | 202 |
| TI OPA1612 | Quote: US$0.88 (LCSC) | 28 |
| ADI LT3045 | Quote: US$7.15 (JLCPCB) | 227 |
| WCH CH224Q | Quote: US$0.26 (LCSC) | 8 |
| Subtotal, quoted parts | Distributor quotes | 949 |
| QSPI boot flash (1.8 V) | Estimate | 8 |
| 24 MHz crystal | Estimate | 3 |
| 2 × 1.8 V low-jitter oscillators with output enable | Estimate | 95 |
| Digital power (0.9 V buck, 3.3 V and 1.8 V LDOs) | Estimate | 19 |
| Quiet 1.8 V LDO | Estimate | 10 |
| ±12 V rails (inverting buck-boost + LM317/LM337-class LDOs) | Estimate | 60 |
| Headphone protection (protection chip + 2 relays) | Estimate | 44 |
| Gain switch and LEDs | Estimate | 10 |
| Connectors (2 × USB-C, 4.4 mm 5-pole, 3.5 mm) | Estimate | 55 |
| USB ESD protection | Estimate | 6 |
| Passives (~160, incl. 0.1% thin-film, C0G, electrolytics) | Estimate | 79 |
| 4-layer bare PCB | Estimate | 25 |
| JLCPCB Standard PCBA fees (amortised) | Estimate | 156 |
| Subtotal, allocations | Estimates | 571 |
| Total per assembled board | At the 1,500 NTD limit | ≈ 1,520 |
| Total with the fallback: mid-grade 5 V LDO instead of the LT3045 | Estimate | ≈ 1,340 |

Lines are rounded to whole NTD, so each subtotal can differ by 1 NTD from the sum of its lines.

At ≈ 1,520 NTD the estimate sits at the 1,500 NTD limit; the automated BOM check will replace the allocations with live LCSC prices, and the last row shows the mid-grade 5 V LDO fallback.

Prototype estimate: 5 boards on Economic PCBA cost ≈ 4,600 NTD in fees (≈ 30 Extended part types, 8 leadless parts per board) plus ≈ 7,300 NTD in parts at 10-piece prices, ≈ 11,900 NTD in total, excluding shipping and import tax.

Assembly note: at 100 units, X-ray inspection is the largest assembly line (≈ US$200 for ≈ 800 leadless parts at US$0.25 each), followed by SMT joints (≈ US$96) and feeder loading (≈ US$84 for ~55 part types).

# 7. Design for JLCPCB (DFF/DFA/DFT rules)

These rules apply to every revision and use JLCPCB’s PCBA price list of 8 Sep 2026; the ones that can be checked automatically also run in CI (Section 8).

| ID | Type | Rule |
| --- | --- | --- |
| R01 | DFA | Standard PCBA is needed for the 100-unit run: setup US$25.56 (single side), stencil US$8.21, feeder loading US$1.53 per part type (Basic or Extended), fixtures US$41.04 for 100–199 boards. |
| R02 | DFF/DFA | Standard PCBA requires edge rails and fiducials and a single board of at least 70 × 70 mm; this board stays within 100 × 70 mm. |
| R03 | DFA | Prototypes use Economic PCBA: setup US$8.18, stencil US$1.53, US$3.07 per Extended part type (so prefer Basic parts), 2–50 boards per order, one side only (all SMT parts on the top side), minimum IC pin pitch 0.4 mm (the XU316 is exactly 0.4 mm). |
| R04 | DFA | Prefer leaded packages where a leaded equivalent exists: X-ray inspection costs US$0.82 per leadless part (11–50 parts) or US$0.25 per part (501–1,000 parts). Leadless here: XU316, OPA1622, LT3045, CH224Q; leaded: AK4493SEQ, OPA1612. |
| R05 | DFA | Joint fees: SMT US$0.0016 per joint; through-hole (manual) US$0.0164 per joint + US$3.58 per order, so prefer SMT-mount jacks where available. |
| R06 | DFA | Turn on “Confirm Parts Placement” (US$0.45) and check the 3D preview for rotation and polarity errors. |
| R07 | DFT | Test point on every power rail and on the key clock and I²S nets. |
| R08 | DFT | I²S input header, so the DAC and amplifier can be tested on their own (the QA403 can drive I²S directly). |
| R09 | DFT | Fiducials and polarity marks. |
| R10 | DFF | Use JLCPCB’s standard 4-layer impedance-controlled stack-up for the 90 Ω USB pair; solid ground plane; digital and analog areas separated. |

# 8. Automation and verification plan

The part most relevant to the NVIDIA role: every check runs automatically with an explicit pass/fail rule.

| Check | Runs when | Pass/fail rule |
| --- | --- | --- |
| ERC and DRC (KiCad command line) | Every commit (GitLab CI) | Zero errors |
| Manufacturing outputs: Gerbers, drill, BOM, pick-and-place, schematic PDF | Every commit | Files generated |
| BOM check | Every commit | Every part has an LCSC number and is in stock; count of Extended parts and leadless chips reported; cost at 5 and 100 units; FAIL if the 100-unit cost > 1,500 NTD |
| ~10 Python DFF/DFA/DFT and layout rules | Every commit | All rules pass, e.g., no digital traces through the analog area, USB pair length-matched, decoupling capacitor near every IC power pin, test point on every rail, fiducials present |
| AI datasheet-review agent | Every merge request | Reads the DAC, amplifier and regulator datasheets, extracts requirements, checks the schematic and comments on the MR. The datasheet-derived rule list in the Schematic Design Notes seeds the agent and its planted-error test set; scored on 10–20 planted errors (catch rate and false alarms) |
| Automated measurement suite (Python + QuantAsylum QA403, ~US$600, or a school-lab analyzer) | Each board revision | THD+N, dynamic range, frequency response, crosstalk, output impedance, DC offset, max power at 16/32/60/120/300 Ω on both jacks; auto-generated report compared against Section 2 |

# 9. Risks and open decisions

## 9.1 Risks

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Estimated cost ≈ 1,520 NTD vs the 1,500 NTD limit | Cost target | Mid-grade 5 V LDO fallback (≈ 1,340 NTD); automated cost gate with live LCSC prices |
| XU316-1024-QF60A has 1.8 V-only I/O | Interface damage or redesign | Flash, oscillators, DAC TVDD, I²C pull-ups and relay/enable drivers must all be 1.8 V-compatible; checked in schematic review |
| 15 V USB PD availability | No analog power from small chargers | Require a ≥ 30 W charger (only chargers above 27 W must offer 15 V); without 15 V the analog section stays off and the relays stay open |
| Stock: XU316 out of stock at LCSC (Digi-Key 563); AK4493SEQ 24-week factory lead time; CH224Q out of stock at LCSC; LT3045 relies on JLCPCB stock | Schedule | Buy prototype quantities early; consign parts to JLCPCB |
| Sustained OPA1622 output short (e.g., wrong plug) | Thermal-shutdown cycling | Rely on the OPA1622 current limit and thermal shutdown; test with a shorted plug on the dummy-load bench |
| XU316 0.4 mm-pitch QFN | Layout and assembly yield | Follow the XMOS reference layout; X-ray inspection |
| Two USB ports: possible ground-loop noise from the PC | Noise floor | Measure; USB isolation as a v2 option |
| Headphone damage during bring-up | Very costly | Relay protection; dummy loads only |

## 9.2 Decisions (closed in v0.2 and still open)

| Decision | Status | Resolution / next step |
| --- | --- | --- |
| Output stage | Closed | Baseline: 4 × OPA1622, both halves paralleled per leg |
| Rail regulators | Closed | LT3045 for the DAC 5 V; LM317/LM337-class for ±12 V |
| Clock source | Closed | Two 1.8 V oscillators with output enable; MCLK also routed to X1D11 for the XU316 PLL option |
| PD sink chip | Closed | CH224Q (alternate: CH224A) |
| Connectors | Closed | 3.5 mm unbalanced + 4.4 mm balanced |
| Volume control | Closed | Host volume via the AK4493SEQ hardware attenuator |
| Paralleling topology | Open | Follower-assisted vs two independent gain stages; decide by SPICE simulation |
| DC servo | Open | Fit only if the measured offset exceeds 5 mV (footprint provided) |
| Volume knob | Open (v2) | Hardware volume knob in v2 |

# 10. Next steps (schematic phase, weeks 1–3)

| # | Action | Done |
| --- | --- | --- |
| 1 | Set up the GitLab repository and KiCad project with the CI skeleton (ERC/DRC + outputs) before drawing anything. | ☐ |
| 2 | Draw the schematic block by block, following the companion Schematic Design Notes. | ☐ |
| 3 | Simulate the output stage with TI’s OPA1622 and OPA1612 SPICE models for stability, offset and noise. | ☐ |
| 4 | Assign an LCSC part number to every part and run the 1,500 NTD cost gate. | ☐ |
| 5 | Get a schematic review from an experienced engineer. | ☐ |
| 6 | Start layout once the schematic review is closed. | ☐ |
| 7 | Apply to the NVIDIA posting once the repository is public. | ☐ |

# Appendix A. Reference documents

| Ref | Document | Source |
| --- | --- | --- |
| [1] | AK4493SEQ datasheet | AKM |
| [2] | AKD4493S-A evaluation-board manual | AKM website (request form) |
| [3] | XU316-1024-QF60A datasheet | XMOS |
| [4] | sw_usb_audio reference firmware and reference design | XMOS |
| [5] | OPA1622 datasheet | Texas Instruments |
| [6] | JLCPCB PCBA price list (8 Sep 2026) | JLCPCB |
| [7] | Schematic Design Notes v0.1 (companion document: block-level circuit guidance) | This project |

# Appendix B. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Draft for review |
| v0.2 | 23 Sep 2026 | Diego Yang | Budget raised to ≤ 1,500 NTD; outputs changed to 3.5 mm unbalanced + 4.4 mm balanced; parts and targets verified against the XU316-1024-QF60A, AK4493S, AKD4493S-A and OPA1622 datasheets; USB chip changed to XU316-1024-QF60A (1.8 V I/O); OPA1612 stage-1 and LT3045 added; CH224K replaced by CH224Q; JLCPCB fees updated to the 8 Sep 2026 price list; most open decisions closed. |