# USB DAC + Balanced Headphone Amplifier — Schematic Design Notes v0.1

Companion to Design Specification v0.2: datasheet-derived requirements and starting component values for the KiCad schematic

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 23 Sep 2026 | v0.1 | Schematic design notes (companion to Design Specification v0.2) |

# 0. Purpose and references

This document collects the datasheet-derived requirements and starting component values for drawing the KiCad schematic of the USB DAC + balanced headphone amplifier. It is a companion to Design Specification v0.2 and is organized by block: key decisions, the power tree, per-block notes, the XU316 signal plan, firmware requirements, a schematic review checklist and the items still to be verified.

The designer owns and verifies every value. Circuit values are checked in SPICE with TI's OPA1622 and OPA1612 models, and pin-level requirements against the datasheets below (cited as R1 to R4). Figures quoted from a datasheet, or computed from datasheet values, are used exactly as stated; where a value is only a starting point it is marked “starting value — verify in SPICE/datasheet”.

| Ref | Document | Publisher | Document no. | Date |
| --- | --- | --- | --- | --- |
| R1 | XU316-1024-QF60A Datasheet | XMOS | XM014034B | 2023-08-29 |
| R2 | AK4493S Datasheet | AKM | 211100143-E-02 | 2022/03 |
| R3 | AKD4493S-A Evaluation Board Manual | AKM | KM136001 | 2025/08 |
| R4 | OPA1622 Datasheet | TI | SBOS727B | May 2016 |

# 1. Key design decisions

The decisions below fix the architecture that the schematic implements. The Basis column records why each choice was made, so a later change can be checked against the original reason.

| Decision | Choice | Basis |
| --- | --- | --- |
| USB audio processor | XU316-1024-QF60A-C24; all I/O 1.8 V only (QF60A); core 0.9 V | Matches the supplied datasheet; 1.8 V logic everywhere |
| DAC digital supplies | AK4493S internal LDO off (LDOE = L); TVDD = AVDD = DVDD = 1.8 V | Exactly AKM's characterization condition (analog specifications are given at AVDD = TVDD = DVDD = 1.8 V, VDDL/R = VREFH = 5.0 V) and matches the XU316's 1.8 V I/O |
| DAC analog supplies | VDDL/R and VREFHL/R = 5.0 V from an ultra-low-noise LDO (LT3045) | Output full scale scales with VREFH − VREFL |
| DAC output coupling | AC-couple each DAC output pin (as in AKM's Example Circuit 3) | Removes the ≈ 2.5 V common-mode DC so the DC-coupled amplifier stays near 0 V |
| Stage 1 | OPA1612 unity-gain difference amplifier + low-pass filter per channel | Cancels even-order and common-mode errors so the 3.5 mm output (leg+ only) is clean; AKM's evaluation board uses OPA1612 |
| Output stage | Leg+ non-inverting (1× / 2.74×); leg− is a unity inverter of leg+; each leg is one OPA1622 with both halves paralleled | One gain resistor per channel to switch; 200 mA peak per leg for K1000/Susvara |
| Analog supplies | ±12 V from 15 V USB PD | 7.75 V peak per leg with headroom (OPA1622 operates from ±2 V to ±18 V) |
| Clocks | Two 1.8 V oscillators + MCLK net to XU316 pin X1D11 | X1D11 is the XU316 application-PLL output pin, so a PLL-generated MCLK can be tried without rework |

# 2. Power tree and current budget

Two USB-C ports feed separate domains. The data port supplies the digital rails from VBUS_DATA (5 V); a USB PD sink on the power port supplies the analog rails from VBUS_PD (15 V). Currents are datasheet typical and maximum figures, or budget estimates where marked.

## 2.1 Regulation chains

| Domain | Input | Regulation chain | Rails |
| --- | --- | --- | --- |
| Digital | USB-C data port, VBUS_DATA 5 V | 5 V → synchronous buck (≥ 1 A) → 0V9; 5 V → LDO → 3V3; 3V3 → LDO → 1V8_D; 3V3 → low-noise LDO (EN from 1V8_D) → 1V8_A | 0V9, 3V3, 1V8_D, 1V8_A |
| Analog | USB-C power port with CH224Q, VBUS_PD 15 V | 15 V → LM317-class LDO → +12 V; +12 V → LT3045 (set 5.0 V) → +5 V_A; 15 V → inverting buck-boost → ≈ −14 V → LM337-class LDO → −12 V | +12 V, +5 V_A, −14 V (intermediate), −12 V |

## 2.2 Rail budget

| Rail | From → regulator | Loads | Current | Notes |
| --- | --- | --- | --- | --- |
| VBUS_DATA 5 V | USB-C data port | Digital rails | ≈ 0.3 A worst case (estimate) | 1–10 µF on VBUS required by USB (2.2 µF typical); ESD array |
| 0V9 | 5 V → synchronous buck, ≥ 1 A | XU316 VDD; PLL_AVDD via ferrite (600 Ω at 100 MHz, DCR < 1 Ω, e.g., BKH1005LM601) + 1 µF | 225 mA typ, 830 mA max (C24) | 0.855–0.945 V; ≥ 8 × 100 nF + ≥ 10 µF bulk |
| 3V3 | 5 V → LDO | XU316 USB_VDD33 (0.8 mA HS, up to 25 mA FS transmit); feeds the 1.8 V LDOs | < 150 mA | 3.0–3.6 V |
| 1V8_D | 3V3 → LDO | XU316 VDDIOL/R/T/B18, USB_VDD18 (30 mA typ, 36 mA max HS), QSPI flash, AK4493 TVDD (1–1.5 mA), I²C pull-ups, debug VREF | ≈ 100 mA | All XU316 VDDIO on this one rail, so the on-chip power-on reset works (XMOS checklist G.3); 100 nF per VDDIO pin + ≥ 10 µF |
| 1V8_A (quiet) | 3V3 → low-noise LDO, EN from 1V8_D | AK4493 AVDD (1–1.5 mA) + DVDD (6/11/17 mA typ at 44.1/96/192 kHz, 26 mA max at 192 kHz) + two oscillators | ≤ ≈ 60 mA | AK4493 rule: TVDD must come up before or with AVDD and DVDD |
| VBUS_PD 15 V | USB-C power port, CH224Q | Analog rails | ≤ 0.70 A (10.5 W at 1.2 W into 60 Ω, both channels) | ≥ 30 W charger; TVS + bulk |
| +12 V | 15 V → LM317-class LDO (ADJ bypass cap) | 4× OPA1622, OPA1612, relay coils, +5 V_A LDO | Up to ≈ 390 mA | ≈ 1.2 W LDO dissipation at max, so SOT-223/DPAK + copper |
| −12 V | 15 V → inverting buck-boost to ≈ −14 V → LM337-class LDO | 4× OPA1622, OPA1612 | Up to ≈ 280 mA | Converter IC sees Vin + ¦Vout¦ ≈ 29 V: pick ≥ 36 V, ≥ 1.5 A (e.g., TI LMR36015 or TPS54360 class); small hot loop, far from analog |
| +5 V_A | +12 V → LT3045 (set 5.0 V) | AK4493 VDDL + VDDR (27 mA typ, 41 mA max) + VREFHL + VREFHR (1–1.5 mA) | ≈ 45 mA | VREFHL/R each via 10 Ω; VDDL/R routed separately from the regulator (AKM) |

## 2.3 Power sequencing

The sequencing rules from the datasheets are met by the rail structure and the hardware defaults, so the board powers up safely before firmware runs.

| # | Requirement | Implementation | Source |
| --- | --- | --- | --- |
| 1 | AK4493 TVDD must come up before or with AVDD and DVDD | TVDD is on 1V8_D; the 1V8_A low-noise LDO is enabled from 1V8_D | R2 |
| 2 | AK4493 PDN low while the supplies come up and for ≥ 150 ns after all are up | 100 kΩ pull-down on PDN; firmware releases PDN only after PD_GOOD and rail settling (Section 5) | R2 |
| 3 | No MCLK, BICK or LRCK until all AK4493 supplies are on | Oscillators off at power-up and enabled only after the rails settle; I²S starts after register configuration | R2 |
| 4 | VREFH must come up with or after VDDL/R | VREFHL/R fed from +5 V_A through 10 Ω into 470 µF | R2 |
| 5 | XU316 on-chip power-on reset | All VDDIO pins on the single 1V8_D rail (XMOS checklist G.3); no external reset circuit | R1 |
| 6 | QSPI flash ready within 300 µs | Select a flash that meets this; otherwise hold RST_N until the flash is ready | R1 |
| 7 | Headphone outputs isolated during power-up and on PD power loss | Relays close 2–3 s after all rails are good and the DAC is running; they open immediately on PD power loss (3.5) | Design |

# 3. Block notes

Pin numbers refer to the XU316-1024-QF60A and AK4493SEQ packages. Entries are datasheet requirements unless they are marked as starting values.

## 3.1 XU316-1024-QF60A (USB audio processor)

### 3.1.1 Requirements and starting values

| Topic | Requirement / value | Notes |
| --- | --- | --- |
| Crystal | 24 MHz crystal (USB needs 12 or 24 MHz): 12 pF load, ESR ≤ 60 Ω, Rf = 1 MΩ, Rd = 680 Ω, CL1 = CL2 = 22 pF | Example part from the datasheet table: Seiko Epson FA-238 24.0000MD30X-W5 |
| Clock alternative | 1.8 V oscillator into XIN | The oscillator must be running before reset is released |
| Decoupling | VDD (0V9): ≥ 8 × 100 nF + ≥ 10 µF bulk; 100 nF per VDDIO pin + ≥ 10 µF on 1V8_D; PLL_AVDD via ferrite (600 Ω at 100 MHz, DCR < 1 Ω) + 1 µF | Place the PLL_AVDD ferrite and 1 µF close to the pin; rail details in 2.2 |
| Boot flash pins | QSPI on fixed pins: SS = X0D01; D0–D3 = X0D04–X0D07; SCLK = X0D10 | Six pins; the remaining port plan is in Section 4 |
| Boot flash part | 1.8 V; supported by the XMOS tools; in quad mode at power-up (Quad Enable set); ready within 300 µs | If the flash cannot be ready within 300 µs, hold RST_N. The part number is an open item (Section 7) |
| Boot-mode pins | X0D04–X0D06 are sampled as boot-mode pins and have internal pull-downs; 000 = QSPI boot | Do not pull them up |
| USB data | USB_DP pin 29, USB_DM pin 28; 90 Ω differential; ≤ 1 mm skew | XMOS example geometry W 0.12 mm / S 0.10 mm over 0.10 mm dielectric; recompute for the JLCPCB stack-up. Keep ≥ 1.27 mm from other high-speed nets and ≥ 0.51 mm from low-speed nets |
| USB-C sink | 5.1 kΩ from CC1 to GND and from CC2 to GND | VBUS decoupling (1–10 µF) and ESD array: see 2.2 |
| VBUS sensing | Optional, because the design is bus-powered | If self-powered: 220 kΩ series, 100 kΩ to GND for 1.8 V I/O, 47 kΩ bleeder, 2.2 µF |
| Reset | RST_N has an internal pull-up; with a single 1.8 V VDDIO rail no external reset is required | Bring RST_N to the debug header |
| Debug | JTAG-only xSYS2 0.05" 10-pin header or TAG-Connect TC2030 footprint (pinouts in 3.1.2) | Needs a 1.8 V XTAG4 adapter |
| Layout and thermal | 16 vias (4 × 4) in the ground paddle; 4 vias under each of the four VDD paddles; decoupling around the device with short ground returns | MSL3 (168 h floor life) |
| Logic levels | XU316 VOH ≥ 1.35 V at 2 mA; AK4493 VIH ≥ 80% × TVDD = 1.44 V at 1.8 V | Works only because the AK4493 inputs draw ≤ 10 µA, so the XU316 outputs sit near 1.8 V. Keep TVDD on the same 1V8_D rail and flag this at review |

### 3.1.2 Debug connectors

| Pin | xSYS2 10-pin, 0.05" (JTAG only) | TAG-Connect TC2030 |
| --- | --- | --- |
| 1 | VDDIOB18 (with decoupler) | TCK |
| 2 | TMS | GND |
| 3 | GND | TMS |
| 4 | TCK | TDI |
| 5 | GND | VREF |
| 6 | TDO | TDO |
| 7 | GND | Not present (6-pin) |
| 8 | TDI | Not present (6-pin) |
| 9 | GND | Not present (6-pin) |
| 10 | RST_N | Not present (6-pin) |

## 3.2 Clocks

Two 1.8 V CMOS oscillators with output enable, one per sample-rate family, drive a single MCLK net. The XU316 enables one oscillator at a time, and MCLK feeds both the AK4493 and XU316 pin X1D11 so that a PLL-generated MCLK can be tried later without rework.

### 3.2.1 Clock requirements

| Item | Requirement / value | Notes |
| --- | --- | --- |
| Oscillators | 22.5792 MHz (44.1 kHz family) and 24.576 MHz (48 kHz family); 1.8 V CMOS with output enable | Part numbers (1.8 V, OE, low phase noise) and LCSC stock are open items (Section 7) |
| Oscillator supply | 1V8_A | Quiet rail, shared with AK4493 AVDD and DVDD |
| Oscillator outputs | Each through a ≈ 22–33 Ω series resistor onto the one MCLK net; enabled one at a time by OSC_EN_44K1 and OSC_EN_48K | Series resistor: starting value — verify in SPICE/datasheet |
| MCLK to AK4493 | MCLK pin 46 via a damping resistor | AKM recommends a damping resistor; without it THD+N can degrade |
| MCLK to XU316 | X1D11 (port 1D, tile 1) | Application-PLL output pin: input in oscillator mode, output in PLL mode |
| AK4493 MCLK limits | 2.048–49.664 MHz; duty cycle 40–60%; pulse width ≥ 9.05 ns | Both oscillator frequencies are inside the range |
| Clock changes | MCLK may change only while the DAC is in reset (PDN = L or RSTN bit = 0) | Firmware sequence in Section 5 |
| Routing | Keep MCLK short and away from the VREF pins and the analog outputs | MCLK test point in 3.7 |

### 3.2.2 Sample-rate ratios

| Sample rate | MCLK | MCLK / fs | AK4493S |
| --- | --- | --- | --- |
| 384 kHz | 24.576 MHz | 64 fs | Allowed: the 216–388 kHz range allows 32/48/64/96 fs |
| 352.8 kHz | 22.5792 MHz | 64 fs | Allowed (same 216–388 kHz range) |
| 192 kHz | 24.576 MHz | 128 fs | Allowed |
| 96 kHz | 24.576 MHz | 256 fs | Allowed |
| 48 kHz | 24.576 MHz | 512 fs | Allowed |
| 44.1 kHz | 22.5792 MHz | 512 fs | Allowed |

## 3.3 DAC (AK4493SEQ)

The AK4493SEQ runs in register mode over I²C with its internal LDO disabled. All digital supplies are 1.8 V, which is AKM's characterization condition, and the analog supplies and references are 5.0 V from +5 V_A.

### 3.3.1 Pin connections

| Function | Pin (no.) | Connection | Requirement / note |
| --- | --- | --- | --- |
| Mode select | PSN (13), I2C (14), CSN (7) | PSN = L; I2C = H; CSN → DVSS | Register mode over I²C |
| I²C address | CAD0 (12), CAD1 (17) | Strap to set the I²C address | Record the chosen address for the firmware register map |
| I²C bus | SCL (8), SDA (9) | XU316 1-bit open-drain ports | 2.2 kΩ pull-ups to 1V8_D |
| LDO enable | LDOE (16) | DVSS | Internal LDO off (LDOE = L) |
| Digital I/O supply | TVDD (48) | 1V8_D | Must come up before or with AVDD and DVDD |
| Core supplies | AVDD (44), DVDD (1) | 1V8_A | DVDD 6/11/17 mA typ at 44.1/96/192 kHz, 26 mA max at 192 kHz |
| Analog supplies | VDDL (33, 34), VDDR (27, 28) | +5 V_A | Route VDDL and VDDR separately from the regulator (AKM) |
| Power-down | PDN (2) | XU316 GPIO with 100 kΩ pull-down | Low while the supplies come up and for ≥ 150 ns after all are up; no MCLK/BICK/LRCK until all supplies are on |
| Master clock | MCLK (46) | MCLK net via damping resistor | Limits and ratios in 3.2 |
| Audio interface | BICK (3), SDATA (4), LRCK (5) | XU316 I²S outputs BCLK, SDATA, LRCLK | Test points and I²S input header in 3.7 |
| Reference, high | VREFHL (41, 42), VREFHR (19, 20) | +5 V_A via 10 Ω each | ≈ 34 Hz low-pass with 470 µF; VREFH comes up with or after VDDL/R |
| Reference, low | VREFLL (39, 40), VREFLR (21, 22) | Analog ground (optional 10 Ω) | Output full scale scales with VREFH − VREFL |
| Common mode | VCML (38), VCMR (23) | 10 µF electrolytic each to VREFL | No other load |
| Grounds | AVSS, DVSS, VSSL, VSSR | One solid analog ground plane | THD+N degrades with impedance between VSS pins |
| Unused pins | NC (24, 37, 43); TESTE (18); WCK (6); DEM0 (15) | NC → analog ground; TESTE → DVSS or open; WCK and DEM0 → DVSS | WCK and DEM0 are not used in register-mode PCM |
| Zero detect | DZFL (10), DZFR (11) | Open or to GPIO | Optional DZF input on the XU316 (Section 4) |
| Analog outputs | AOUTLP (36), AOUTLN (35), AOUTRP (25), AOUTRN (26) | Analog chain (3.4) | Load ≥ 450 Ω and ≤ 25 pF per pin; internal current limit 85 mA per pin; filter resistors ≤ 1% (AKM), 0.1% here for common-mode rejection |

### 3.3.2 Decoupling (AKM typical connection, LDO disabled)

| Pin or pin pair | Decoupling | Note |
| --- | --- | --- |
| TVDD | 0.1 µF + 10 µF | Supplied from 1V8_D |
| AVDD | 0.1 µF + 10 µF | Supplied from 1V8_A |
| DVDD | 1 µF + 0.1 µF | Supplied from 1V8_A |
| VDDL, VDDR | Each 0.1 µF + 10 µF | Supplied from +5 V_A, routed separately |
| VREFHL–VREFLL, VREFHR–VREFLR | Each 0.1 µF ceramic at the pins + 470 µF electrolytic | With the 10 Ω feed this forms the ≈ 34 Hz low-pass |
| VCML (38), VCMR (23) | Each 10 µF electrolytic to VREFL | No other load on VCM |

## 3.4 Analog chain (per channel)

Each channel runs from the DAC pins through a passive interface and the OPA1612 difference amplifier (stage 1) to two OPA1622 legs, then through the relays to the jacks. Every component value in 3.4.1 is a starting value — verify in SPICE/datasheet; the results are computed from these values.

### 3.4.1 Starting values

| Stage | Values | Result |
| --- | --- | --- |
| DAC pin interface | 100 Ω series per pin → 2.2 nF C0G across P–N → 100 µF coupling capacitor per pin (audio/bipolar electrolytic) → 10 kΩ bias to ground → stage 1 | Passive pole ≈ 362 kHz; coupling time constant with the 10 kΩ bias ≈ 1 s |
| Stage 1 (one OPA1612 half per channel) | Unity-gain difference amplifier: four 1.00 kΩ 0.1% thin-film resistors; 820 pF C0G across the feedback resistor and across the ground-leg resistor | Active pole ≈ 194 kHz. Combined response −0.06 dB at 20 kHz, −0.23 dB at 40 kHz, −0.89 dB at 80 kHz. Output 2 Vrms at full scale |
| Leg+ (OPA1622 channel A, non-inverting) | Rf = 348 Ω, Rg = 200 Ω (0.1%); 22–47 pF C0G across Rf (verify stability) | 2.74× (+8.8 dB) high gain; the gain switch opens Rg for 1.00× low gain. Low Rf keeps the offset from the OPA1622 input bias current (1.2 µA typ, 2 µA max) at 0.42 mV typ |
| Leg− (OPA1622 channel A, inverting) | Rin = Rf = 1.00 kΩ 0.1%; non-inverting input to ground via 499 Ω | Unity inverter of leg+; the 499 Ω provides bias-current compensation |
| Paralleling, Option 1 (simulate first) | Channel A is the gain stage with feedback from the leg output node; channel B is a unity follower of channel A's output; each output reaches the node through 0.47 Ω 1% (≥ 0.25 W) | One gain network per leg; low output impedance |
| Paralleling, Option 2 (fallback) | Both channels as identical gain stages, each with feedback taken before its own 0.47 Ω ballast | Output impedance ≈ 0.24 Ω; needs a 4-pole gain switch |
| Option footprints | Arrange the footprints so that either option is a 0 Ω-link change | The choice follows the SPICE stability check (Section 7) |
| Gain switch | DPDT slide switch, one pole per channel, in the Rg path; optional 3rd/4th pole to an XU316 input | With the extra pole fitted, firmware soft-mutes during a gain change |
| Enable | Both halves of a package share EN; all four EN pins are driven from one XU316 GPIO with a 100 kΩ pull-down | EN threshold ≈ 0.82 V referenced to the OPA1622 GND pin; EN is never floating or high-impedance. In shutdown the output is not high-impedance, so the relays isolate the jacks |
| Thermal | OPA1622 thermal pad to V− copper with vias | Dissipation up to 1.05 W per chip (+50 °C at 47.6 °C/W) |
| Do-not-fit footprints | Output Zobel per leg (10 Ω + 100 nF); DC servo per channel | Not fitted by default; the footprints keep both options open |

### 3.4.2 Output connectors (after the relays)

| Signal | 4.4 mm 5-pole | 3.5 mm TRS |
| --- | --- | --- |
| L+ | Tip | Tip |
| L− | Ring 1 | Not connected (leg+ only) |
| R+ | Ring 2 | Ring |
| R− | Ring 3 | Not connected (leg+ only) |
| GND | Sleeve | Sleeve |

### 3.4.3 Computed estimates

| Parameter | Estimate | Basis |
| --- | --- | --- |
| Dynamic range | ≈ 121 dB(A) at both gains, both jacks | DAC 1.42 µV and stage 1 0.94 µV, A-weighted |
| DC offset, typical | 0.7 mV unbalanced / 1.5 mV balanced | Computed from the 3.4.1 values |
| DC offset, worst case | ≈ 5.0 mV unbalanced / 11.0 mV balanced | Computed; a DC servo footprint is provided (do not fit) |
| OPA1622 dissipation | Up to 1.05 W per chip | +50 °C at 47.6 °C/W; thermal pad to V− copper |
| Output drive per leg | 7.75 V peak, 200 mA peak | K1000/Susvara; the ±12 V rails give headroom |
| Stage 1 full-scale output | 2 Vrms | DAC full scale scales with VREFH − VREFL |

## 3.5 Protection

Relays keep both jacks disconnected until the system is running, and disconnect them again on a DC fault or on loss of PD power. The XU316 can also force a mute.

| Function | Requirement |
| --- | --- |
| Relay placement | 2 × DPDT relays on L+, L−, R+ and R−, ahead of both jacks |
| Power-up | Open at power-up; close 2–3 s after all rails are good and the DAC is running |
| DC fault | Open within 100 ms on > ±1 V DC on any leg |
| PD power loss | Open immediately on PD power loss: sense VBUS_PD, so the relays open before ±12 V fall out of regulation |
| Firmware mute | The XU316 can force a mute (MUTE output, Section 4) |
| Detector | μPC1237-class protection IC, or a comparator window + delay |
| Coil drive | Coils from +12 V via an NPN transistor or logic-level MOSFET driven by a 1.8 V GPIO; flyback diodes |
| Short circuit | The OPA1622 tolerates a continuous short on one amplifier per package; with paralleled halves, a sustained short relies on thermal shutdown (175 °C) |

## 3.6 Power input

| Item | Requirement |
| --- | --- |
| PD sink | CH224Q in resistor-configuration mode requesting 15 V; take the resistor value from the WCH datasheet (open item, Section 7) |
| Charger | ≥ 30 W charger; worst-case draw ≤ 0.70 A (10.5 W at 1.2 W into 60 Ω, both channels) |
| PD_GOOD | VBUS_PD divider into a 1.8 V XU316 GPIO input |
| Input protection | TVS + bulk capacitance on VBUS_PD |
| −14 V converter | Inverting buck-boost; follow TI's inverting buck-boost guidance. The IC sees Vin + ¦Vout¦ ≈ 29 V, so pick ≥ 36 V, ≥ 1.5 A (e.g., TI LMR36015 or TPS54360 class); keep the hot loop small and far from the analog section |

## 3.7 Test and DFT

| Item | Requirement |
| --- | --- |
| Rail test points | VBUS_DATA, 0V9, 3V3, 1V8_D, 1V8_A, VBUS_PD, +12 V, −14 V, −12 V, +5 V_A, VREFHL, VREFHR |
| Signal test points | MCLK, BCLK, LRCLK, SDATA and PDN; relay drive; each leg before the relay |
| I²S input header | MCLK, BCLK, LRCLK, SDATA, GND, with 0 Ω links (or the XU316 held in reset) so a QA403 can drive the DAC directly |
| Assembly | Three fiducials; edge rails; board ≥ 70 × 70 mm for Standard PCBA |

# 4. XU316 signal plan

- About 15 GPIO are needed beyond the six QSPI pins, and 28 are available (34 − 6). Two port rules apply: never mix inputs and outputs on one multi-bit port, and treat X0D04–X0D07 as output-only (XMOS checklist G.7); here those pins carry the QSPI data lines, so no GPIO is mapped to them. Start from XMOS's xcore.ai USB Audio reference design port map and confirm that the map builds in the XMOS tools before layout.

| Signal | Dir | Port rule | Pin / notes |
| --- | --- | --- | --- |
| QSPI: SS, SCLK, D0–D3 (6 signals) | Output, I/O | Fixed pins | SS = X0D01; D0–D3 = X0D04–X0D07; SCLK = X0D10 |
| MCLK | Input (oscillator mode) / output (PLL mode) | X1D11 = port 1D, tile 1 | Application-PLL output pin |
| BCLK, LRCLK, SDATA | Output | 1-bit ports on the tile running I²S | To the AK4493 and the I²S test header |
| I²C SCL, SDA | Open-drain | 1-bit ports, open-drain | 2.2 kΩ pull-ups to 1V8_D |
| PDN, OSC_EN_44K1, OSC_EN_48K, AMP_EN, MUTE, 2 LEDs | Output | May share one multi-bit output port | PDN and AMP_EN each have a 100 kΩ pull-down |
| PD_GOOD, GAIN_STATE (optional), DZF (optional) | Input | Inputs on a separate port | Never on a port that also carries outputs |
| USB DP, DM | USB | Dedicated USB pins | DP pin 29, DM pin 28 |
| JTAG and reset | JTAG | Dedicated pins | TCK 24, TMS 23, TDI 18, TDO 20, RST_N 21 |

# 5. Firmware requirements

- The hardware defaults (pull-downs on PDN and EN, relays open) hold the board in a safe state until firmware takes over. Firmware then follows the sequences below; if the optional gain-switch pole is fitted, it also soft-mutes during a gain change (3.4).

| # | Event | Required behavior |
| --- | --- | --- |
| 1 | Power-up | PDN low, oscillators off, OPA1622 EN low, relays open |
| 2 | On PD_GOOD | Wait for the rails to settle (≈ 100 ms) → enable the oscillator for the current family → release PDN → configure registers over I²C → start I²S → enable the OPA1622s → wait ≥ 2 s → close the relays |
| 3 | Sample-rate family change | Soft mute → RSTN = 0 (or PDN low) → switch oscillator → RSTN = 1 → unmute |
| 4 | PD power loss | Hardware opens the relays; firmware sets PDN low and disables EN |
| 5 | Volume | Through the AK4493 attenuator (0.5 dB steps); optional per-channel trim, since the DAC gain mismatch is up to 0.3 dB |
| 6 | First programming | First flash programming through the XTAG4 adapter |

# 6. Schematic review checklist

These rules are the first rule set for the AI datasheet-review agent, and each one can be turned into a planted-error test case. ERC is KiCad's electrical rules check; a netlist script checks connections, part values and counts in the exported netlist; the AI agent reviews rules that need datasheet or part-level interpretation.

| # | Rule | Source | Check method |
| --- | --- | --- | --- |
| 1 | XU316 VDD: ≥ 8 × 100 nF + ≥ 10 µF | R1 | Netlist script |
| 2 | XU316 VDDIO: 100 nF per VDDIO pin + ≥ 10 µF | R1 | Netlist script |
| 3 | PLL_AVDD: ferrite + 1 µF, placed close | R1 | Netlist script; AI agent (ferrite part) |
| 4 | All XU316 VDDIO on one 1.8 V rail | R1 (G.3) | Netlist script |
| 5 | 24 MHz crystal network: 12 pF load, ESR ≤ 60 Ω, Rf = 1 MΩ, Rd = 680 Ω, CL1 = CL2 = 22 pF | R1 | Netlist script; AI agent (crystal part) |
| 6 | QSPI flash on the fixed pins; 1.8 V part; Quad Enable set; ready within 300 µs | R1 | Netlist script (pins); AI agent (part) |
| 7 | X0D04–X0D06 not pulled up (boot mode 000 = QSPI) | R1 | Netlist script |
| 8 | No multi-bit port mixes inputs and outputs | R1 (G.7) | Netlist script with the port map |
| 9 | USB: 90 Ω differential, ≤ 1 mm skew; VBUS 1–10 µF; 5.1 kΩ on CC1 and CC2 | R1; USB | Netlist script; AI agent (differential pair) |
| 10 | Ground paddle 16 vias (4 × 4); 4 vias under each VDD paddle | R1 | AI agent (footprint) |
| 11 | Debug header present (xSYS2 or TC2030) with RST_N | R1 | Netlist script |
| 12 | AK4493 LDOE = L; TVDD = AVDD = DVDD = 1.8 V; TVDD first | R2 | Netlist script; AI agent (sequencing) |
| 13 | AK4493 PDN pull-down | R2 | Netlist script |
| 14 | VREFH pairs 0.1 µF + 470 µF; VCML and VCMR 10 µF each | R2 | Netlist script |
| 15 | VDDL and VDDR 0.1 µF + 10 µF each | R2 | Netlist script |
| 16 | Single analog ground plane for AVSS, DVSS, VSSL, VSSR | R2 | Netlist script |
| 17 | MCLK damping resistor present; MCLK ≤ 49.664 MHz | R2 | Netlist script; AI agent (oscillator) |
| 18 | DAC output pin load ≥ 450 Ω, ≤ 25 pF | R2 | AI agent (load calculation) |
| 19 | Unused and NC pin handling (NC, TESTE, WCK, DEM0, DZFL/DZFR) | R2 | ERC; netlist script |
| 20 | XU316 VOH vs AK4493 VIH margin flagged (1.35 V vs 1.44 V) | R1, R2 | AI agent |
| 21 | OPA1622 thermal pad to V−; EN never floating; paralleled halves in one package | R4 | ERC (EN); netlist script |
| 22 | Relays on all four legs before both jacks | Design (3.5) | Netlist script |

# 7. Open items to verify

The items below are still open. Each one points to the section where the result is used.

| # | Item | What to verify | Section |
| --- | --- | --- | --- |
| 1 | QSPI flash part | 1.8 V; Quad Enable set; supported by the XMOS tools; ready within 300 µs | 3.1 |
| 2 | Oscillator part numbers | 1.8 V, output enable, low phase noise; LCSC stock | 3.2 |
| 3 | Inverting converter and inductor | ≥ 36 V, ≥ 1.5 A class for the ≈ −14 V rail (e.g., TI LMR36015 or TPS54360 class) | 2.2, 3.6 |
| 4 | Relay part and coil current | 2 × DPDT; coil current fits the +12 V budget | 3.5 |
| 5 | Protection IC | μPC1237-class IC or comparator window + delay; > ±1 V DC trip within 100 ms | 3.5 |
| 6 | Paralleling topology stability | SPICE with TI's OPA1622 model; Option 1 first, Option 2 as fallback | 3.4 |
| 7 | OPA1622 output swing at 100 mA | Datasheet Figures 38/39 | 3.4 |
| 8 | USB impedance on the JLCPCB stack-up | 90 Ω differential; recompute the XMOS example geometry | 3.1 |
| 9 | XU316 port map | Map builds in the XMOS tools before layout | 4 |
| 10 | CH224Q resistor value for 15 V | From the WCH datasheet | 3.6 |

# Appendix A. Rails and signals

The rails and signals referenced in this document, with their source and destination, as a starting list for the schematic net labels.

| Rail or signal | Type | From → to | Section |
| --- | --- | --- | --- |
| VBUS_DATA | Power, 5 V | USB-C data port → 0V9 buck and 3V3 LDO | 2.2 |
| 0V9 | Power | Synchronous buck → XU316 VDD; PLL_AVDD via ferrite | 2.2 |
| 3V3 | Power | LDO → XU316 USB_VDD33; input to the 1.8 V LDOs | 2.2 |
| 1V8_D | Power | LDO → XU316 VDDIO and USB_VDD18, QSPI flash, AK4493 TVDD, I²C pull-ups, debug VREF | 2.2 |
| 1V8_A | Power, quiet | Low-noise LDO (EN from 1V8_D) → AK4493 AVDD and DVDD, both oscillators | 2.2 |
| VBUS_PD | Power, 15 V | USB-C power port (CH224Q) → +12 V LDO, inverting converter, PD_GOOD divider | 2.2, 3.6 |
| +12 V | Power | LM317-class LDO → four OPA1622, OPA1612, relay coils, +5 V_A LDO | 2.2 |
| −14 V | Power, intermediate | Inverting buck-boost → LM337-class LDO | 2.2 |
| −12 V | Power | LM337-class LDO → four OPA1622, OPA1612 | 2.2 |
| +5 V_A | Power | LT3045 → AK4493 VDDL and VDDR; VREFHL and VREFHR feeds | 2.2 |
| VREFHL, VREFHR | Reference | +5 V_A via 10 Ω each → AK4493 VREFHL (41, 42) and VREFHR (19, 20) | 3.3 |
| MCLK | Clock | Enabled oscillator → AK4493 MCLK (46) via damping resistor; XU316 X1D11 | 3.2 |
| BCLK, LRCLK, SDATA | I²S | XU316 → AK4493 BICK (3), LRCK (5), SDATA (4); I²S input header | 3.3, 3.7 |
| SCL, SDA | I²C | XU316 ↔ AK4493 SCL (8), SDA (9); 2.2 kΩ pull-ups to 1V8_D | 3.3 |
| PDN | Control | XU316 → AK4493 PDN (2); 100 kΩ pull-down | 3.3 |
| OSC_EN_44K1, OSC_EN_48K | Control | XU316 → output enable of the 22.5792 MHz and 24.576 MHz oscillators | 3.2 |
| AMP_EN | Control | XU316 → EN of all four OPA1622; 100 kΩ pull-down | 3.4 |
| MUTE | Control | XU316 → relay protection (force mute) | 3.5 |
| PD_GOOD | Status | VBUS_PD divider → XU316 input | 3.6 |
| GAIN_STATE | Status, optional | Gain-switch 3rd/4th pole → XU316 input | 3.4 |
| DZF | Status, optional | AK4493 DZFL (10) / DZFR (11) → XU316 input | 3.3 |
| RST_N | Reset | XU316 RST_N (21) ↔ debug header | 3.1 |
| USB_DP, USB_DM | USB | USB-C data port ↔ XU316 pins 29 and 28; 90 Ω differential | 3.1 |
| CC1, CC2 | USB-C | Data port configuration channels; 5.1 kΩ each to GND | 3.1 |
| L+, L−, R+, R− | Analog output | OPA1622 leg outputs → relays → 4.4 mm and 3.5 mm jacks | 3.4, 3.5 |

# Appendix B. Abbreviations

| Term | Meaning |
| --- | --- |
| C0G | Class 1 ceramic dielectric (NP0), stable over voltage and temperature |
| DFT | Design for test |
| DNF | Do not fit: footprint placed, part not assembled |
| DPDT | Double-pole, double-throw (relay or switch) |
| DZF | Zero-detect flag outputs of the AK4493 (DZFL, DZFR) |
| ERC | Electrical rules check (KiCad) |
| ESD | Electrostatic discharge |
| fs | Sampling frequency |
| FS / HS | USB full-speed / high-speed |
| I²C | Two-wire control bus (SCL, SDA) |
| I²S | Three-wire audio serial interface (BCLK, LRCLK, SDATA) |
| LDO | Low-dropout linear regulator |
| MCLK | Master clock |
| MSL | Moisture sensitivity level |
| OE | Output enable |
| PCBA | Printed circuit board assembly |
| PD | USB Power Delivery |
| PDN | Power-down input of the AK4493 |
| PLL | Phase-locked loop |
| QE | Quad Enable bit of a QSPI flash |
| QSPI | Quad serial peripheral interface |
| THD+N | Total harmonic distortion plus noise |
| TVS | Transient voltage suppressor |
| VIH / VOH | Input-high threshold / output-high level |
| xSYS2 / XTAG4 | XMOS debug header / XMOS debug adapter |

# Appendix C. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Initial issue: datasheet-derived requirements and starting values for schematic capture; companion to Design Specification v0.2 |