# USB DAC + Balanced Headphone Amplifier — Design Specification v0.5

Product requirements and architecture for a personal PCB project

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 24 Sep 2026 | v0.5 (draft for review) | Product requirements + architecture |

Companion documents: “Schematic Design Notes v0.4” holds the block-level circuit guidance, and “Parts List v0.3” (DAC_HPA_Parts_List_v0.3.xlsx) is the source of truth for cost. This specification states the requirements and architecture they implement.

# 1. Purpose and scope

This specification defines the requirements and architecture of a single USB-C, bus-powered Hi-Res DAC with one ESS ES9018K2M DAC chip and a balanced headphone amplifier. The outputs are a 3.5 mm unbalanced jack and a 4.4 mm balanced jack, for one headphone at a time.

v0.5 answers the fourth review round (R4-A, R4-B, R4-C; Appendix C). The v0.4 architecture stays: CH32V307 + AGRV2K USB bridge, ES9018K2M in differential current mode with one I/V converter per pin, 2 × OPA1622 difference-amplifier legs, LM27762 charge-pump rails and a hardware fault latch. v0.5 lowers the headphone rails to ±3.78 V, calibrates every unit to 1.75 Vrms per leg, enforces the analog supply current in hardware (TPS2553), switches the power domains per USB state, runs the DAC in asynchronous mode from a 100 MHz MCLK and completes the protection. The v0.5 amendment of 24 Sep 2026 adds a 24.576 MHz and a 22.5792 MHz oscillator for the two sample-rate families, so that every rate gets an exact 50:50 bit clock (the ES9018K2M needs 45:55 to 55:45), and moves the USB firmware and the CPLD logic to our own implementation.

Goal: “best measured performance for the money”, proven by published measurements; not “beats every product”, which a hobby analyzer cannot prove (QuantAsylum itself says its QA403 won’t match a US$30,000 analyzer).

Budget: ≤ 1,500 NTD per board refers to the 5-board sample run (what the owner actually pays: ≈ 7,500 NTD for five assembled boards, including the bare PCBs and JLCPCB assembly fees, excluding shipping, import tax, programmers and enclosure). It is not a 100-unit average. On 24 Sep 2026 the owner accepted a second oscillator and then requested a third (≈ +62 and +64 NTD per board) and set the approved ceiling to 1,700 NTD per board; the current estimate is 1,666 NTD per board (8,330 NTD for five boards), a conservative planning figure (Section 6).

Portfolio context is kept in the repository README.

| In scope for v1 | Out of scope for v1 |
| --- | --- |
| USB-C Hi-Res DAC with a single ES9018K2M DAC chip, bus-powered from one USB-C port (data + 5 V) | DSD in v1 (stretch goal: the ES9018K2M supports DSD; depends on firmware) |
| Balanced headphone amplifier: 4.4 mm balanced jack plus 3.5 mm unbalanced jack (one headphone at a time) | K1000-class 1 W drive (v0.5 gives ≈ 102 mW into the K1000, Section 5) |
| PCM 44.1–384 kHz (including 352.8 kHz; all eight rates with exact clocks), 16–32 bit, USB Audio Class 2.0 High-Speed | USB PD power |
| USB power modes: Default (≤ 500 mA, hardware-limited) and High (only with a ≥ 1.5 A Type-C advertisement) | A guaranteed USB suspend current (2.5 mA is a target in v1) |
| DC-fault protection: fault latch and PGOOD hardware kill that mute the amplifiers and turn the headphone rails off | Output relays |
| Host volume in the DAC’s attenuator; per-unit level calibration at the end-of-line test | Hardware volume knob |
| Design automation: CI checks, BOM cost gate, DFF/DFA/DFT rules, AI datasheet review, automated measurement and calibration | Enclosure design (3D-printed case for prototypes only) |

# 2. Key targets

All values below are proposed targets for v0.5, based on the ES9018K2M, OPA2197, OPA1622, LM27762 and TPS2553 datasheets and the v0.5 design calculations; every board revision is measured against this table (Section 8). Design estimates and assumptions are labelled as such.

| Parameter | Target | Minimum | Conditions | How verified |
| --- | --- | --- | --- | --- |
| PCM sample rate | 44.1–384 kHz, 16–32 bit: all eight rates (44.1, 48, 88.2, 96, 176.4, 192, 352.8 and 384 kHz) with exact clocks and a 50:50 bit clock | 384 kHz | USB Audio Class 2.0, High-Speed; DAC bit-clock duty 45:55 to 55:45 (Section 5.3) | Host playback sweep at every rate; all eight rates bit-perfect in the proof of concept |
| DSD | Not in v1 (stretch: the ES9018K2M supports DSD; depends on firmware) | — | — | — |
| Hosts | Windows 10/11 built-in UAC2 driver, macOS, Linux; Android over USB-C (High-Speed) | — | — | Host playback on each OS |
| THD+N at 1 kHz | ≤ −105 dB | ≤ −100 dB | Calibrated 0 dBFS, 300 Ω, both jacks, 20 Hz–20 kHz (proposed) | QA403 via Python (Section 8) |
| Dynamic range | ≥ 115 dB(A) | ≥ 112 dB(A) | Both jacks (estimate ≈ 117 dB(A) unbalanced, ≈ 120 dB(A) balanced) | QA403 |
| Level at 0 dBFS | 1.75 Vrms unbalanced / 3.5 Vrms balanced after calibration (± 0.1 dB) | — | Per-unit master trim stored in MCU flash; before calibration −3.5 dB (≤ 1.73 Vrms) | Automated end-of-line test (Section 7) |
| Output power, High mode | Unbalanced ≥ 70 mW at 32 Ω; balanced ≥ 150 mW at 60 Ω, ≥ 90 mW at 120 Ω, ≥ 36 mW at 300 Ω | — | 1 kHz, both channels; rated at 1 % THD+N or calibrated 0 dBFS, whichever comes first; estimates in Section 5.2 | QA403 + dummy loads |
| Output power, Default mode | Balanced ≥ 50 mW at 60 Ω; other loads as High mode; 32 Ω balanced not rated | — | As High mode; analog current within the TPS2553 Default limit | QA403 + dummy loads |
| Output impedance | ≤ 0.5 Ω | — | 1 kHz, both jacks, including contacts | Load/no-load method, 32 Ω and 300 Ω |
| Frequency response | +0.1/−0.3 dB, 20 Hz–20 kHz | — | 44.1, 48 and 384 kHz sample rates; estimate −0.05 dB at 20 kHz | QA403 |
| Crosstalk at 1 kHz | Balanced ≤ −100 dB (300 Ω); unbalanced ≤ −75 dB (32 Ω, at the jack) | — | Layout rule for the shared ground (Section 7) | QA403 |
| Channel balance | ≤ 0.3 dB; per-channel fine trim to ±0.05 dB by MCU digital scaling at calibration | — | — | Automated end-of-line test |
| DC offset | ≤ ±10 mV unbalanced, ≤ ±20 mV balanced | — | Worst-case estimate 7.9 / 15.9 mV (RSS ≈ 4.1 / 6.4 mV); 100 % screening | Automated test on every board |
| Protection | Every leg ≤ 0.1 V into 32 Ω within 100 ms of the fault onset; ≤ 1 mJ delivered after the trip; starts latched at power-up | — | Defined at the headphone terminals; no false trip for full-scale content down to ≈ 4 Hz | DC fault injected by a digital DC test signal (Section 7) |
| Pop | ≤ 5 mV peak for controlled transitions; ≤ 20 mV peak at unplug or brownout (target) | — | Controlled: power-on after configuration, suspend/resume, rate and rate-family change, fault recovery; 32 Ω and 300 Ω | Oscilloscope |
| USB power | Before configuration ≤ 100 mA (estimate ≤ 83 mA); Default mode ≤ 500 mA, hardware-enforced (≈ 0.473 A worst case); High mode only with a ≥ 1.5 A Type-C advertisement (≈ 0.758 A worst case); suspend 2.5 mA is a target, not guaranteed in v1 | — | Bus-powered descriptors (bmAttributes 0x80, bMaxPower 0xFA) | USB power meter in every state; Type-C 1.5 A source and USB-A port |
| Inrush | Attach charge ≤ 50 µC (estimate ≈ 38 µC) | — | Nominal capacitance; domains staggered and soft-started | Charge at attach; VBUS droop at each domain enable |
| Thermal | All parts within ratings; the board recovers | — | Overload tests, 30 min each, High mode, maximum volume (Section 7) | Thermocouple / IR camera |
| PCB | 4 layers, target ≤ 80 × 60 mm, all SMT parts on the top side except the 4.4 mm jack, 90 Ω USB pair | — | JLCPCB Economic PCBA | ERC/DRC and layout-rule checks in CI (Section 8) |
| Cost | ≤ 1,500 NTD per board for the 5-board run (target); owner-approved ceiling 1,700 NTD per board (estimate 1,666 NTD) | — | Including PCB and PCBA fees; excluding shipping, tax and tools | Cost gate: WARN > 1,500, FAIL > 1,700 (Section 8) |

# 3. Architecture

## 3.1 Signal and control path

Figure 1. Signal and control path (100 MHz oscillator for the DAC MCLK and the CPLD system clock, 24.576 MHz or 22.5792 MHz family oscillator selected in front of the CPLD PLL, OPA2197 I/V converters, ±3.78 V output legs).

The single USB-C port carries USB 2.0 High-Speed audio and the 5 V bus. D+, D− and VBUS pass through a USBLC6-2SC6 ESD array. CC1 and CC2 keep their 5.1 kΩ Rd pull-downs and also go through 10 kΩ to MCU ADC inputs, which read the Type-C current advertisement (Section 3.2).

USB bridge: a WCH CH32V307RCT6 RISC-V MCU with a built-in USB 2.0 High-Speed PHY runs our own USB Audio Class 2.0 firmware, built on WCH’s register-level high-speed USB device examples; the CPLD logic is ours as well. The open-source USBAudioToI2S project (Apache-2.0; MCU + CPLD asynchronous USB audio) is kept only as a known-good comparison on the same hardware. v1 depends on a pre-layout firmware gate, run as a two-stage proof of concept (Section 8). The 8 MHz HSE crystal is confirmed (the firmware derives the 4 MHz USB HS PLL reference as HSE ÷ 2); the Basic 5.0 × 3.2 mm YXC X50328MSB2GI is used with 33 pF load capacitors.

Clock: three 3.3 V oscillators on 3V3D. X201, 100 MHz (YXC OT3225100MJBA4SL, enable tied high), drives the DAC MCLK directly and the CPLD system clock (SPI registers, input side, FIFO write side), each through 33 Ω. X202, 24.576 MHz (YXC OT322524.576MJBA4SL), for the 48 kHz family and X203, 22.5792 MHz (SCTF SX2M22.5792M20F30TNN), for the 44.1 kHz family share one node at the CPLD’s dedicated PLL clock input, each through its own 33 Ω (R210, R211). The ES9018K2M runs in its normal (asynchronous, DPLL) mode, the mode its datasheet performance is specified in. It requires MCLK > 192 × FSR (73.7 MHz at 384 kHz) and allows up to 100 MHz with DVCC 3.3 V and an external 1.3 V DVDD, so the ±10 ppm oscillator sits at that limit (open item: a stocked 80–98 MHz oscillator would add margin).

An AGM AGRV2KQ32 CPLD makes BCLK = 64 × fs and LRCLK. The ES9018K2M needs a bit-clock duty cycle of 45:55 to 55:45 (datasheet v3.7, Audio Interface Timing), which a divider from 100 MHz cannot hold at the top rates, so each rate family has its own oscillator, selected before the PLL as reviewer R4-C suggested. The CPLD drives the two enable pins, OSC48_EN for X202 (100 kΩ pull-up R212, on by default) and OSC44_EN for X203 (100 kΩ pull-down R213, off by default), and exactly one family oscillator runs at a time. The CPLD’s single PLL doubles it to 49.152 MHz or 45.1584 MHz, divided by 2/4/8/16 for 384/192/96/48 kHz or 352.8/176.4/88.2/44.1 kHz: all eight rates are exact integer ratios with a 50:50 bit clock (Section 5.3). There is no PLL in the DAC MCLK path.

Rate-family change: DAC soft mute → I²S output stopped (held low) → PLL held in reset → oscillator enables switched → wait ≥ 10 ms for start-up → PLL released and locked → output restarts → DAC re-locks (lock status, register 64) → unmute. There is no clock multiplexer in the CPLD fabric, and the 100 MHz MCLK to the DAC is never touched. X202 is tri-state when disabled (YXC datasheet: enable ≥ 70 % VDD or floating, disable ≤ 30 % VDD, start-up ≤ 3 ms); the shared node also needs X203’s disabled output to be high-impedance (open item: confirm in the SCTF datasheet; fallback: a small 2:1 logic multiplexer in front of the PLL input). Both family oscillators and the shared node sit next to the CPLD PLL clock pin with short traces, and the enables come from the CPLD on the same 3V3D domain, so nothing back-powers the oscillators when 3V3D is off.

The ESS ES9018K2M (32-bit, PCM to 384 kHz via I²S) is configured by the MCU over I²C (address 0x90) and runs in differential current mode. Each output pin is equivalent to a voltage source (3.05 Vpp around 1.65 V) in series with 806 Ω ±11 % (datasheet p. 31), and ESS specifies DNR and THD+N only in differential current mode, so the design follows the ESS application circuit (p. 27) with one I/V converter per pin. The MCU drives RESETB (100 kΩ pull-down) and reads the DPLL lock status.

Signal chain: DAC (current mode) → 2 × OPA2197 I/V converters (one amplifier per DAC pin, VREF ≈ 1.70 V) → 2 × OPA1622 difference-amplifier legs → jacks, all on ±3.78 V. U403 serves DACL/DACLB and U404 DACR/DACRB. Each feedback is 836 Ω (1 kΩ ∥ 5.1 kΩ, 1 %) with 10 Ω + 660 pF (two 330 pF C0G in parallel), a 288 kHz pole. VREF = 1.703 V from a 15 kΩ over 15 kΩ + 1 kΩ divider on 3V3A (10 µF + 100 nF, τ ≈ 77 ms), at the DAC’s zero-DC-current point (≈ 1.702 V). The OPA2197 (Vos ±0.25 mV max, 5.5 nV/√Hz, rail-to-rail I/O) was chosen for its maximum offset (Appendix C, A-H2).

Output stage: 2 × TI OPA1622, one amplifier per leg, with no paralleling. Each amplifier is a 4-resistor 2.00 kΩ (0.1 %) difference amplifier with 330 pF C0G (a 241 kHz pole), fed by the I/V outputs: leg+ = I/V(DAC−) − I/V(DAC+), leg− = the swap. The leg input common mode is VREF, so VREF errors are mostly common mode. U401 drives L+ and L−, U402 drives R+ and R−. The clean output envelope is ≤ 80 mA peak per leg within the OPA1622 swing curve (Section 5).

Protection: an LM393 window comparator detects DC on any leg and sets an N-MOSFET fault latch on the always-on 3V3M domain. The latch pulls RAIL_EN (LM27762 EN+/EN−) and AMP_EN (both OPA1622 EN pins) low, and LM27762 PGOOD pulls AMP_EN low in hardware whenever the rails are not good. The set path is blanked for ≈ 60 ms after PGOOD goes good without clearing the latch. Power-up starts latched, the rails are first on and last off, and there are no relays (Section 5.3).

The 4.4 mm balanced jack carries L+, L−, R+, R− and GND; the 3.5 mm unbalanced jack carries L+, R+ and GND. Both jacks are live, so only one headphone is connected at a time.

## 3.2 Power path

The single USB-C port supplies data and 5 V; the board is bus-powered (bmAttributes 0x80, bMaxPower 0xFA = 500 mA). Only 5 × 1 µF sit on VBUS before any switch (connector and the inputs of ME6211 #1, ME6211 #2, LP5907 and TPS2553). At attach only 3V3M starts (4.0 µF behind it), so the attach charge is ≈ 38 µC against the 50 µC USB-IF limit (nominal values; DC-bias derating lowers the real figure).

3V3M, always on: ME6211C33 #1 powers the MCU, the fault latch and the LEDs. Before USB configuration only this domain runs (62 / 83 mA typical / maximum estimate; assumption on the MCU current).

3V3D, switched: ME6211C33 #2 (CE = AUD_EN) powers the CPLD, the three oscillators (100 MHz, 24.576 MHz and 22.5792 MHz), the I²C pull-ups and the DAC DVCC (through the GZ1608D601TF ferrite, ≈ 14 mA). A fixed, ceramic-stable 1.3 V LDO (ME6216A13M3G, ±1 %, 300 mA) fed from 3V3D makes the external DVDD, so DVDD cannot exceed DVCC during the ramps.

3V3A, switched: the LP5907-3.3 (EN = AVCC_EN) supplies VCCA, AVCC_L/AVCC_R and the VREF divider.

Headphone rails: a TI TPS2553 current-limited switch (EN = ANA_EN; built-in soft start; FAULT open drain with 7.5 ms deglitch) feeds 5V_ANA through the Murata BLM18PG121SN1D ferrite (2 A, 50 mΩ) to the LM27762, which makes +3.780 V (3.723–3.837 V) and −3.782 V (−3.726 to −3.838 V) with 215 kΩ/100 kΩ and 210 kΩ/100 kΩ feedback; each top resistor is built from Basic values (100 kΩ + 100 kΩ + 15 kΩ or 10 kΩ). EN = RAIL_EN; PGOOD goes to the MCU and to the AMP_EN kill.

Current limit: RILIM = 100 kΩ + 47 kΩ = 147 kΩ gives the Default limit (157 / 183 / 213 mA minimum / nominal / maximum, including 1 % resistors); a 2N7002 switches 100 kΩ in parallel for High mode (59.5 kΩ: 393 / 442 / 498 mA). Default mode holds the board to ≤ 0.473 A worst case (27 mA below 0.5 A); High mode (≤ 0.758 A) is used only when the Type-C advertisement read on CC1/CC2 is ≥ 1.5 A (bands: Default 0.25–0.61 V, 1.5 A 0.70–1.16 V, 3.0 A 1.31–2.04 V). A USB-A-to-C cable reads as Default, and many phones limit OTG to ≤ 500 mA and run in Default mode.

Supply headroom at the design corner (VBUS 4.60 V at the receptacle: host minimum 4.75 V minus 150 mV cable and connector drop, assumption): LM27762 VIN 4.546 V; at the worst clean load (4.4 mm, 32 Ω, 80 mA per leg) the negative-rail capability is −3.956 V (margin 0.17 V) and the positive 4.466 V (margin 0.69 V), with a 3.0 Ω charge-pump resistance (assumption; 2.5 Ω is specified only at 5.5 V). At a 4.40 V stress corner the negative rail is 26 mV short at current peaks (brief dropout; the 95 % PGOOD threshold is not reached).

VBUS sense: 100 kΩ / 47 kΩ into an ADC (5.0 V → 1.60 V) with an analog watchdog at 4.30 V, which drops AMP_EN first on unplug or brownout (target < 5 µs).

Figure 2. Power path (switched power domains with the three oscillators on 3V3D, TPS2553 current limit, LM27762 ±3.78 V rails).

## 3.3 Volume, gain and level

Gain is fixed. Volume comes from the host via USB Audio Class (Feature Unit volume and mute) and is applied in the ES9018K2M attenuator (0 to −127.5 dB in 0.5 dB steps), with MCU fine scaling for channel trim; a hardware knob is out of scope. Per leg, L = (Rf/Rs) × 2.156 Vrms with Rf = 836 Ω and Rs = 806 Ω ±11 %: nominal 2.236 Vrms, unit spread 1.955–2.589 Vrms including Rf 1 % and AVCC ±2 %. Every unit is therefore calibrated to 1.75 Vrms per leg at 0 dBFS with the ES9018K2M 32-bit master trim, stored in MCU flash at the automated end-of-line test.

| Parameter | Value | Basis |
| --- | --- | --- |
| Calibrated full scale | 1.75 Vrms per leg (2.475 V peak): 3.5 mm 1.75 Vrms, 4.4 mm 3.5 Vrms | End-of-line calibration to ± 0.1 dB |
| Master trim | −0.96 dB (lowest-gain unit) to −3.40 dB (highest-gain unit); nominal −2.13 dB | ES9018K2M 32-bit master trim, stored in MCU flash |
| Before calibration | −3.5 dB, so ≤ 1.73 Vrms on any unit | Firmware default |
| Channel trim | ±0.05 dB | MCU digital scaling |
| Output envelope | ≤ 80 mA peak per leg; Vpk ≤ (¦rail¦min − 0.9 V)/(1 + 8 Ω/Rleg) | OPA1622 Fig. 38/39 (typical, 25 °C); bench-verify at ±3.78 V |

# 4. Key parts

| ZQ-c04-r00-c0-QZ | ZQ-c04-r00-c1-QZ | ZQ-c04-r00-c2-QZ | ZQ-c04-r00-c3-QZ | ZQ-c04-r00-c4-QZ | ZQ-c04-r00-c5-QZ |
| --- | --- | --- | --- | --- | --- |
| ZQ-c04-r01-c0-QZ | ZQ-c04-r01-c1-QZ | ZQ-c04-r01-c2-QZ | ZQ-c04-r01-c3-QZ | ZQ-c04-r01-c4-QZ | ZQ-c04-r01-c5-QZ |
| ZQ-c04-r02-c0-QZ | ZQ-c04-r02-c1-QZ | ZQ-c04-r02-c2-QZ | ZQ-c04-r02-c3-QZ | ZQ-c04-r02-c4-QZ | ZQ-c04-r02-c5-QZ |
| ZQ-c04-r03-c0-QZ | ZQ-c04-r03-c1-QZ | ZQ-c04-r03-c2-QZ | ZQ-c04-r03-c3-QZ | ZQ-c04-r03-c4-QZ | ZQ-c04-r03-c5-QZ |
| ZQ-c04-r04-c0-QZ | ZQ-c04-r04-c1-QZ | ZQ-c04-r04-c2-QZ | ZQ-c04-r04-c3-QZ | ZQ-c04-r04-c4-QZ | ZQ-c04-r04-c5-QZ |
| ZQ-c04-r05-c0-QZ | ZQ-c04-r05-c1-QZ | ZQ-c04-r05-c2-QZ | ZQ-c04-r05-c3-QZ | ZQ-c04-r05-c4-QZ | ZQ-c04-r05-c5-QZ |
| ZQ-c04-r06-c0-QZ | ZQ-c04-r06-c1-QZ | ZQ-c04-r06-c2-QZ | ZQ-c04-r06-c3-QZ | ZQ-c04-r06-c4-QZ | ZQ-c04-r06-c5-QZ |
| ZQ-c04-r07-c0-QZ | ZQ-c04-r07-c1-QZ | ZQ-c04-r07-c2-QZ | ZQ-c04-r07-c3-QZ | ZQ-c04-r07-c4-QZ | ZQ-c04-r07-c5-QZ |
| ZQ-c04-r08-c0-QZ | ZQ-c04-r08-c1-QZ | ZQ-c04-r08-c2-QZ | ZQ-c04-r08-c3-QZ | ZQ-c04-r08-c4-QZ | ZQ-c04-r08-c5-QZ |
| ZQ-c04-r09-c0-QZ | ZQ-c04-r09-c1-QZ | ZQ-c04-r09-c2-QZ | ZQ-c04-r09-c3-QZ | ZQ-c04-r09-c4-QZ | ZQ-c04-r09-c5-QZ |
| ZQ-c04-r10-c0-QZ | ZQ-c04-r10-c1-QZ | ZQ-c04-r10-c2-QZ | ZQ-c04-r10-c3-QZ | ZQ-c04-r10-c4-QZ | ZQ-c04-r10-c5-QZ |
| ZQ-c04-r11-c0-QZ | ZQ-c04-r11-c1-QZ | ZQ-c04-r11-c2-QZ | ZQ-c04-r11-c3-QZ | ZQ-c04-r11-c4-QZ | ZQ-c04-r11-c5-QZ |
| ZQ-c04-r12-c0-QZ | ZQ-c04-r12-c1-QZ | ZQ-c04-r12-c2-QZ | ZQ-c04-r12-c3-QZ | ZQ-c04-r12-c4-QZ | ZQ-c04-r12-c5-QZ |

All other parts are chosen in Schematic Design Notes v0.4 (part number, LCSC number, stock and price). Parts List v0.3 is the source of truth for cost; every line in it has an LCSC number and a verified JLCPCB class.

# 5. Headphone drive and estimates

## 5.1 Load requirements

Current limits the low-impedance loads. The clean output envelope is ≤ 80 mA peak per leg within the OPA1622 swing; overload behaviour is handled separately by the TPS2553 current limit (Section 5.3). The OPA1622’s +145/−130 mA figure is a typical short-circuit current and its current limit is ≈ 110–120 mA at an 85 °C die, so neither is a linear output rating.

| Requirement | Value | Basis |
| --- | --- | --- |
| Loads | 16–600 Ω on both jacks | Load matrix in Section 5.2 |
| Clean envelope | ≤ 80 mA peak per leg | One amplifier per leg (U401 = L+/L−, U402 = R+/R−) |
| Swing model | Vpk ≤ (¦rail¦min − 0.9 V)/(1 + 8 Ω/Rleg) | OPA1622 Fig. 38/39, typical, 25 °C; extrapolated to ±3.78 V (bench-verify) |
| Rails | +3.780 V (3.723–3.837 V), −3.782 V (−3.726 to −3.838 V) | LM27762 feedback-reference tolerance |
| Full-scale level | 1.75 Vrms per leg (2.475 V peak) after calibration | Section 3.3 |
| Output impedance | < 0.1 Ω plus jack contacts (the OPA1622 legs drive the jacks directly, no ballast); target ≤ 0.5 Ω on both jacks | Estimate |
| Reference headphones | HIFIMAN Susvara (60 Ω, 83 dB/mW); AKG K1000 (120 Ω, 74 dB/mW) | Maker specifications; SPL in Section 5.2 |

## 5.2 Output power and SPL

Rated power is defined at 1 kHz, both channels driven, at 1 % THD+N or calibrated 0 dBFS, whichever comes first. The values below are design estimates at the calibrated level, with the limiting factor in brackets. Default mode differs from High mode only where the analog current exceeds the TPS2553 Default limit (minimum 0.157 A); the 3.5 mm values are the same in both modes.

| Load | 3.5 mm unbalanced | 4.4 mm balanced, High mode | 4.4 mm balanced, Default mode |
| --- | --- | --- | --- |
| 16 Ω | 51 mW (80 mA envelope) | — | — |
| 32 Ω | 80 mW (swing) | 102 mW (80 mA envelope) | ≈ 31 mW (Default current limit) |
| 60 Ω | 51 mW (full scale) | 166 mW (swing) | ≈ 59 mW (Default current limit) |
| 120 Ω | — | 102 mW (full scale) | 102 mW (full scale) |
| 300 Ω | 10 mW (full scale) | 41 mW (full scale) | 41 mW (full scale) |
| 600 Ω | — | 20 mW (full scale) | 20 mW (full scale) |

SPL from the maker sensitivity, 4.4 mm balanced at the calibrated 0 dBFS (estimates). The higher power figures of v0.4 are withdrawn: they assumed rails and an output swing that the USB supply cannot hold.

| Headphone | Impedance, sensitivity | Mode | Power | SPL |
| --- | --- | --- | --- | --- |
| HIFIMAN Susvara | 60 Ω, 83 dB/mW | High | 166 mW | ≈ 105 dB SPL |
| HIFIMAN Susvara | 60 Ω, 83 dB/mW | Default | 59 mW | ≈ 101 dB SPL |
| AKG K1000 | 120 Ω, 74 dB/mW | High or Default | 102 mW | ≈ 94 dB SPL |

## 5.3 Power, offset, noise, bit clock, thermal and protection

USB current per state (estimates at 5 V, typical / maximum; the MCU and CPLD currents are assumptions, the oscillators are counted at their datasheet maxima (X201 up to 35 mA, X202 5 mA, X203 10 mA; both family oscillators counted, conservatively) with assumed typical values, and the external DVDD current is scaled from 80 MHz to 100 MHz MCLK):

| State | Powered domains | Current | Limit / note |
| --- | --- | --- | --- |
| Attached, not configured | 3V3M only (MCU, latch, LEDs) | 62 / 83 mA | ≤ 100 mA unit load; measure (assumption on the MCU current) |
| Configured, idle | 3V3M, 3V3D, 3V3A and the headphone rails | 225 / 302 mA | Digital 190 / 260 mA |
| Playing, Default mode | All; analog current ≤ TPS2553 Default limit | ≤ 0.473 A worst case (0.260 + 0.213 A) | ≤ 500 mA, enforced in hardware (27 mA margin; measure) |
| Playing, High mode | All; analog current ≤ TPS2553 High limit | ≤ 0.758 A worst case | Only with a ≥ 1.5 A Type-C advertisement |
| Suspend | Audio domains off; MCU in low-power mode | Target 2.5 mA | Not guaranteed in v1 (open item) |

Analog current at the clean output (maximum quiescent): 3.5 mm 16/32/60/300 Ω draw 146/134/96/53 mA; 4.4 mm 32/60/120/300/600 Ω draw 249/235/149/85/64 mA, so the 4.4 mm 32 Ω and 60 Ω loads are reduced in Default mode. Quiescent current per rail (maximum): OPA1622 13.2 mA, OPA2197 4.8 mA, LM393 1 mA, plus 3.4 mA DC from the I/V outputs into the leg networks (positive rail).

DC offset per leg, worst case at 25 °C with datasheet maxima:

| Term | Per leg | Basis |
| --- | --- | --- |
| Common-mode leakage | 3.40 mV | VREF 1.703 V × 0.2 % (0.1 % resistors, gain 1) |
| OPA1622 Vos | 1.00 mV | 0.5 mV max × 2 |
| OPA1622 Ios | 0.10 mV | Ios × 1 kΩ × 2 |
| OPA2197 Vos | 1.08 mV | 0.25 mV max × noise gain up to 2.17, two amplifiers |
| DAC pin-to-pin DC mismatch | 1.18 mV | ≤ 1 mV (assumption, measure) × Rf/Rs |
| VREF-to-null error | 1.17 mV | Divider tolerance + assumed ±20 mV null-point uncertainty, × 3 % pair-gain mismatch |
| Total, unbalanced | ≤ 7.9 mV worst (≈ 4.1 mV RSS) | Limit ±10 mV (margin 2.1 mV) |
| Total, balanced | ≤ 15.9 mV worst (≈ 6.4 mV RSS) | Limit ±20 mV (margin 4.1 mV): 20 mV into 16 Ω balanced = 25 µW DC |

Every board’s offset is measured by the automated test (100 % screening); the DAC-mismatch assumption stays open until measured. Noise (estimates): the I/V output is ≈ 12.4 nV/√Hz (OPA2197 5.5 nV/√Hz × 2.04 plus the Rf and DAC-source thermal noise) and each leg ≈ 20.1 nV/√Hz, or 2.84 µV in 20 kHz. Dynamic range is ≈ 115.8 dB unweighted and ≈ 117.7 dB(A) for the analog stages; with the DAC (127 dB(A) DNR, referred to the untrimmed level) it is ≈ 117.0 dB(A) unbalanced and ≈ 120.0 dB(A) balanced. The I/V pole (288 kHz) and the leg pole (241 kHz) give −0.05 dB at 20 kHz and −0.20 dB at 40 kHz.

Bit-clock duty cycle (design calculation, worst case per rate). The ES9018K2M requires a duty cycle of 45:55 to 55:45, high and low times ≥ 4.5 ns and a cycle ≥ 10 ns, with data setup 4.1 ns and hold 2 ns to the rising edge (datasheet v3.7, Audio Interface Timing). A divider from 100 MHz would reach only 40:60 at 352.8/384 kHz; with the selected family oscillator doubled by the CPLD PLL, every rate is an exact integer division:

| Rate | Bit clock (64 × fs) from | Worst duty | Against 45:55 to 55:45 |
| --- | --- | --- | --- |
| 48 kHz | X202 24.576 MHz × 2 (CPLD PLL) = 49.152 MHz ÷ 16 | 50.0 % | Inside (exact ratio) |
| 96 kHz | 49.152 MHz ÷ 8 | 50.0 % | Inside (exact ratio) |
| 192 kHz | 49.152 MHz ÷ 4 | 50.0 % | Inside (exact ratio) |
| 384 kHz | 49.152 MHz ÷ 2 | 50.0 % | Inside (exact ratio) |
| 44.1 kHz | X203 22.5792 MHz × 2 (CPLD PLL) = 45.1584 MHz ÷ 16 | 50.0 % | Inside (exact ratio) |
| 88.2 kHz | 45.1584 MHz ÷ 8 | 50.0 % | Inside (exact ratio) |
| 176.4 kHz | 45.1584 MHz ÷ 4 | 50.0 % | Inside (exact ratio) |
| 352.8 kHz | 45.1584 MHz ÷ 2 | 50.0 % | Inside (exact ratio) |

Thermal: the analog input power is bounded by the TPS2553 limit (≤ 1.06 W Default, ≤ 2.49 W High); the board worst case is ≈ 3.8 W (High mode, overload). A 4.4-to-3.5 mm adapter shorts leg−, one amplifier per package: ≈ 0.35 W in the shorted amplifier + ≈ 0.1 W in the other, ≈ 0.45 W per package, +21 °C (θJA 47.6 °C/W). A 4.4 mm plug shorting leg+ to leg− loads both amplifiers of one package with ≈ 0.55 W per package, outside TI’s one-amplifier continuous-short rating, so it gets a bench survival test.

Protection is required, because the target headphones are expensive and hard to replace. There are no relays; the outputs are kept safe by the rails-first-on/last-off sequence, a hardware PGOOD kill and the fault latch:

| Function | Implementation | Requirement |
| --- | --- | --- |
| DC detect | Each leg → 147 kΩ (100 k + 47 k) + 1 µF (τ = 147 ms) → BAV99 into MAX and MIN nodes (1 MΩ loads); LM393 on the rails compares against ±0.405 V (100 kΩ / 12 kΩ from ±3.78 V) → trip at ≈ ±0.75 V DC on a leg | ≈ 70 ms for 2 V DC, ≈ 46 ms for a leg stuck at a rail; full-scale 20 Hz gives 0.13 V after the filter, 5 Hz 0.52 V, 4 Hz 0.65 V → no false trip down to ≈ 4 Hz |
| Hardware kill paths | AMP_EN (both OPA1622 EN; 100 kΩ pull-down; MCU through 10 kΩ) is pulled low by Q608 when the latch is set and by Q609 whenever LM27762 PGOOD is not good (disabled or unpowered). RAIL_EN (LM27762 EN+ and EN−; 100 kΩ pull-down; MCU through 10 kΩ) is pulled low by Q607 when the latch is set | No firmware needed; EN stays below the OPA1622 input limit whenever the rails are off |
| Fault latch | N-MOSFETs only (2N7002): LM393 outputs → 10 kΩ → node F (47 kΩ to 3V3M, 1N4148WS clamp: ≈ −0.7 V on a fault, ≤ 0.31 mA) → Q601 inverter → set request S → Q602 sets the Q604/Q605 cross-coupled latch (100 kΩ pull-ups; FLT high = fault); Q606 (MCU FLT_CLR) clears it; 100 nF makes it power up latched; the MCU reads FLT | The 3.3 V domain never sees the −3.8 V comparator output; a clear does not stick while a set request is present |
| Blanking | Q603 holds S low for ≈ 60 ms after PGOOD goes good (1 MΩ + 100 nF from PGOOD) | Blocks only the set request; it cannot clear a set latch, so there is no hiccup |
| Energy at the terminals | Before the trip ≤ 16.5 mJ (16 Ω, rail-stuck leg for 46 ms; 11.4 mJ at 32 Ω, 6.1 mJ at 60 Ω). After the trip the rails are disabled and each holds ≈ 30.6 µF → ≤ 0.22 mJ; the LM27762 50 kΩ discharge is not relied on | ≤ 1 mJ after the trip; OPA1622 shutdown is not treated as isolation (TI: the output is not high-impedance) |
| Back-powering | With the rails off and the DAC and VREF powered, each DAC pin can push ≤ 1.7 mA and VREF ≤ 0.16 mA into the unpowered I/V inputs (OPA2197 input-current limit 10 mA); firmware removes the source (LP5907 off, DAC reset) within 10 ms of a fault | The normal sequence never has the DAC powered while the rails are off |
| Overload | Analog current reaches the TPS2553 limit → rails droop → PGOOD → amplifiers off in hardware → firmware soft-mutes, lowers the volume ceiling by 3 dB (down to −12 dB) and re-enables after 1 s; the ceiling is kept until replug; TPS2553 FAULT is also read | Overload tests (Section 7) |
| Unplug and brownout | The VBUS analog watchdog (4.30 V) drops AMP_EN first; the PGOOD kill is the hardware backstop (the +3.78 V LDO loses regulation near VIN ≈ 3.8 V while 3V3M holds to ≈ 3.5 V) | Pop ≤ 20 mV peak (target); ordering at a fast VBUS collapse is not guaranteed, so it is measured |
| Adapter short | A 4.4-to-3.5 mm adapter shorts leg−; with U401 = L+/L− and U402 = R+/R−, only one amplifier per package is shorted (TI continuous-short rating) | ≈ 0.45 W per package; warning in the manual |
| Jack ESD | 4 × LESD5D5.0CT1G (5 V standoff, 12 pF), one per jack leg | — |
| Bring-up safety | Dummy resistors only | Never test on real headphones |

Power sequence after USB configuration (power-down and suspend run in reverse, rails last off):

| Step | Action | Note |
| --- | --- | --- |
| 1 | ANA_EN → TPS2553 soft start | Headphone rails first |
| 2 | +5 ms: FLT_CLR, then RAIL_EN → ±3.78 V → PGOOD good | Hardware arming ≈ 60 ms later |
| 3 | AVCC_EN → LP5907 (VCCA and AVCC together); VREF settles (τ ≈ 77 ms) | ESS: VCCA first, AVCC at the same time or later |
| 4 | +5 ms: AUD_EN → 3V3D (DVCC, CPLD, oscillators); the 1.3 V DVDD LDO is fed from 3V3D | ESS: DVCC at the same time as VCCA or later; DVDD ≤ DVCC during the ramp |
| 5 | MCLK running → RESETB released ≥ 1 ms after all supplies and MCLK are stable → I²C init (asynchronous mode, 32-bit I²S, mute, master trim from flash) | MCU outputs toward the CPLD stay low or high-impedance while 3V3D is off; I²C pull-ups sit on 3V3D |
| 6 | When PGOOD has been good ≥ 100 ms, ≥ 400 ms after step 3 and the latch is clear → AMP_EN high → soft unmute | Rate change: MCLK never changes; soft mute → I²S output stopped → new divider; for a family change the PLL is held in reset while the oscillator enables switch (≥ 10 ms start-up), then released and locked → output restarts → DAC re-locks (register 64) → soft unmute; AMP_EN stays high |

# 6. Cost budget

Budget for the 5-board sample run on JLCPCB Economic PCBA, at the fixed rate of 31.7 NTD per US$ that the cost gate uses. Pricing rule: the LCSC tier that matches the run quantity (1+ tier for one-per-board ICs, 10+ for two-per-board parts), with a US$0.10 minimum per purchasing line for parts under US$0.05 (attrition and minimum-quantity allowance). Parts List v0.3 is the source of truth for cost.

| Item | Basis | US$, 5 boards | NTD per board |
| --- | --- | --- | --- |
| USB-C input and sensing | LCSC prices | 1.53 | 10 |
| USB bridge: CH32V307 + CPLD + crystal + three oscillators | LCSC prices | 37.89 | 240 |
| DAC (ES9018K2M) and its supplies | LCSC prices | 19.08 | 121 |
| I/V converters and output legs: 2 × OPA2197 + 2 × OPA1622 + 0.1 % resistors | LCSC prices | 33.99 | 215 |
| Power: domains, current limit and ±3.78 V rails | LCSC prices | 10.04 | 64 |
| Protection and control | LCSC prices | 2.06 | 13 |
| Outputs, indicators, test, board (the 4.4 mm jack is 218 NTD of this) | LCSC prices | 36.33 | 230 |
| Minimum-spend allowance (US$0.10 per purchasing line under US$0.05) | Pricing rule | 0.95 | 6 |
| Subtotal, parts (4,497 NTD) | Parts List v0.3 | 141.88 | 899 |
| 5 bare 4-layer PCBs | Estimate | 10.00 | 63 |
| Economic PCBA setup | JLCPCB price list | 8.18 | 52 |
| Stencil | JLCPCB price list | 1.53 | 10 |
| Extended part types (19 × US$3.07) | Includes the through-hole 4.4 mm jack (conservative) and X202 (class assumed) | 58.33 | 370 |
| SMT joints (681 per board × US$0.0016 × 5) | JLCPCB price list | 5.45 | 35 |
| X-ray (8 leadless parts per board × 5 × US$0.82) | CPLD, DAC, 2 × OPA1622, LM27762, three oscillators (oscillators conservative) | 32.80 | 208 |
| Through-hole joints (4.4 mm jack, 7 joints × 5 × US$0.0164 + US$3.58) | JLCPCB price list | 4.15 | 26 |
| Confirm Parts Placement | JLCPCB price list | 0.45 | 3 |
| Subtotal, JLCPCB fees | JLCPCB price list | 110.89 | 703 |
| Total, 5-board run (8,330 NTD) | Parts + PCBs + fees | 262.77 | 1,666 |
| Cost-gate status: target 1,500 NTD per board; approved ceiling 1,700 NTD | WARN above 1,500, FAIL above 1,700; 166 NTD over the target, 34 NTD under the ceiling | — | WARNING |

Lines are rounded to whole NTD, so a subtotal can differ by 1 NTD from the sum of its lines.

Excluded: shipping, import tax, the WCH-LinkE and CPLD programming cable, the enclosure and the proof-of-concept hardware for the firmware gate (Section 8).

Counts: 126 BOM lines (121 fitted), 48 purchasing lines, 227 placed parts per board, 19 Extended + 29 Basic part types, 681 SMT joints + 7 through-hole joints per board, 8 leadless parts per board (X-ray).

Extended types (19): CH32V307RCT6, AGRV2KQ32, ES9018K2M, OPA1622IDRCR, OPA2197IDR, LM27762DSSR, TPS2553DBVR, LP5907MFX-3.3, ME6211C33M5G-N (two fitted, one type), ME6216A13M3G, USBLC6-2SC6, the three oscillators X201 (YXC OT3225100MJBA4SL, C4353707; Extended confirmed on its JLCPCB page), X202 (YXC OT322524.576MJBA4SL, C2831388; class not shown on its JLCPCB page, assumed Extended) and X203 (SCTF SX2M22.5792M20F30TNN, C5452663; Extended per its JLCPCB part page), RT0603BRD072KL (2.00 kΩ 0.1 %), the USB-C receptacle, the 3.5 mm jack, the 4.4 mm jack and LESD5D5.0CT1G. Everything else is Basic, verified on the JLCPCB part page or the basic-parts list.

Cost gate: target 1,500 NTD per board (WARN above), owner-approved ceiling 1,700 NTD (FAIL above; set on 24 Sep 2026 with the third oscillator) → WARNING, 34 NTD under the ceiling. Four items are counted conservatively: if JLCPCB does not charge the Extended fee for the through-hole jack (−19.5 NTD per board) and does not X-ray the three oscillators (−26.0 NTD per board each), the run is ≈ 1,569 NTD per board. The figure is a planning estimate; the gate becomes final with the JLCPCB quote from the real BOM/CPL files after layout, which also confirms X202’s class and whether X202 or X203 needs an assembly fixture (X202’s JLCPCB page says one may be needed). No cheaper 4.4 mm jack exists at LCSC (no SMD option).

Stock to reserve in the JLCPCB parts library before ordering (LCSC, Jun–Sep 2026): CH32V307RCT6 119, ME6216A13M3G ≈ 410, 4.4 mm jack 721, ES9018K2M 724, AGRV2KQ32 980.

Why the estimate changed: 1,540 NTD per board with one oscillator → 1,602 NTD with the 24.576 MHz oscillator (+62 NTD: the part with its decoupling capacitor 16.2, its Extended-part fee 19.5, its X-ray 26.0 and eight more SMT joints 0.4) → 1,666 NTD with the 22.5792 MHz oscillator (+64 NTD: the part with its capacitor and three resistors 17.5, its Extended-part fee 19.5, its X-ray 26.0 and twelve more SMT joints 0.6). Against Parts List v0.2, verified prices are mostly lower, but new parts were added (TPS2553, a second ME6211, the 100 MHz oscillator in place of the previous clock parts, nine 2N7002, sensing resistors) and three more Extended types are counted (the USB-C receptacle and the 3.5 mm jack confirmed Extended, the 4.4 mm jack now counted).

Reference only, not the budget basis: a 100-unit build on Standard PCBA comes to ≈ 801 NTD per board. The budget and the cost gate use the 5-board sample run above.

# 7. Design for JLCPCB (DFF/DFA/DFT rules) and tests

These rules apply to every revision and use JLCPCB’s PCBA price list; the 5-board sample run is built on Economic PCBA. The rules that can be checked automatically also run in CI (Section 8).

| ID | Type | Rule |
| --- | --- | --- |
| R01 | DFA | The sample run uses Economic PCBA: setup US$8.18, stencil US$1.53, 2–50 boards per order, one side only (all SMT parts on the top side). |
| R02 | DFA | Every fitted part has an LCSC number and a verified JLCPCB class (Parts List v0.3). Prefer Basic parts (no per-type fee): where the exact value is Extended, Basic E24 values or Basic combinations are used (100 k + 47 k, 1 k ∥ 5.1 k, 15 k + 1 k, 2 × 330 pF). Each Extended type costs US$3.07; this board has 19 Extended + 29 Basic types (the through-hole jack counted as Extended, conservatively, and the 24.576 MHz oscillator assumed Extended). |
| R03 | DFA | X-ray costs US$0.82 per leadless part, so prefer leaded packages where an equivalent exists. Leadless here (8 per board): AGRV2KQ32, ES9018K2M, 2 × OPA1622, LM27762 and the three oscillators (counted conservatively). The CH32V307RCT6 (LQFP-64), 2 × OPA2197 (SOIC-8) and TPS2553 (SOT-23-6) are leaded. |
| R04 | DFA | Joint fees: SMT US$0.0016 per joint (681 per board); through-hole US$0.0164 per joint + US$3.58 per order. Only the 4.4 mm jack is through-hole (7 joints); confirm that the USB-C receptacle’s shell pegs do not add a through-hole line. |
| R05 | DFA | Turn on “Confirm Parts Placement” (US$0.45) and check the 3D preview for rotation and polarity errors. Check in the quote whether X202 or X203 needs an assembly fixture (X202’s JLCPCB page notes that one may be needed). |
| R06 | DFA | Reserve low-stock parts in the JLCPCB parts library before ordering: CH32V307RCT6, ME6216A13M3G, 4.4 mm jack, ES9018K2M, AGRV2KQ32. |
| R07 | DFT | Test pad on every rail and switched domain (VBUS, 5V_ANA, 3V3M, 3V3D, 3V3A, 1.3 V DVDD, +3.78 V, −3.78 V, VREF) and on MCLK, the shared family-oscillator node (PLL input), OSC48_EN, OSC44_EN, BCLK, LRCLK, SDATA, the EN, PGOOD, FAULT and FLT nodes and the CC sense inputs. |
| R08 | DFT | Debug pads for the MCU (WCH-LinkE 2-wire: 3V3, SWDIO, SWCLK, GND) and the CPLD (JTAG: 3V3, TCK, TMS, TDI, TDO, GND); DNF 1 × 6 I²S header (MCLK, BCLK, LRCLK, SDATA, GND, 3V3 ref) with a firmware test mode that tristates the CPLD I²S outputs, so an external 3.3 V source (QA403, via series resistors) can drive the DAC. |
| R09 | DFT | Fiducials and polarity marks. |
| R10 | DFF | JLCPCB’s standard 4-layer impedance-controlled stack-up for the 90 Ω USB pair; solid ground plane; digital, charge-pump (2 MHz) and analog areas separated; the unbalanced jack sleeve gets a dedicated return with ≥ 4 vias (≤ 5.7 mΩ shared ground for −75 dB crosstalk at 32 Ω); both family oscillators and their shared node sit next to the CPLD PLL clock pin with short traces. |

Tests added in v0.5 and its amendment (automated where marked):

| Test | When | Pass criterion |
| --- | --- | --- |
| End-of-line calibration (automated, QA403 + scripts) | Every board | Full-scale level per leg → master trim to 1.75 Vrms ± 0.1 dB and per-channel fine trim, written to MCU flash |
| Offset screening (automated) | Every board | DC offset of every leg: ≤ ±10 mV unbalanced, ≤ ±20 mV balanced |
| THD+N and dynamic range (automated) | Every board | Section 2 targets at the calibrated level |
| Protection | Each board revision | DC fault injected by a digital DC test signal: every leg ≤ 0.1 V into 32 Ω within 100 ms of the fault onset and ≤ 1 mJ delivered after the trip |
| Overload (30 min each, High mode, maximum volume) | Each board revision | 4.4 mm 16 Ω, 4.4-to-3.5 mm adapter, 3.5 mm 16 Ω, 60 Ω balanced full scale: all parts within ratings and the board recovers |
| Plug short | Each board revision | 4.4 mm plug shorting leg+ to leg−: survival test |
| USB | Each board revision | Current in every state (USB power meter); attach inrush charge ≤ 50 µC; VBUS droop at each domain enable; Default and High mode on a Type-C 1.5 A source and on a USB-A port |
| Pop | Each board revision | ≤ 5 mV peak for controlled transitions (power-on after configuration, suspend/resume, rate and rate-family change, fault recovery); ≤ 20 mV peak at unplug or brownout (target); 32 Ω and 300 Ω |
| Jitter | Each board revision | J-test at the 48 kHz and 44.1 kHz families (both exact, from X202 and X203 through the CPLD PLL) and at 384 kHz; spectra recorded (no numeric limit yet) |
| Clock checks (firmware gate) | Proof of concept, before layout | The PLL relocks after every family change; no runt pulses on BCLK/LRCLK at family changes (scope); all eight rates bit-perfect; the CPLD meets timing at 100 MHz and at the PLL output (49.152 MHz) |
| Rails and sequencing | Bring-up | Negative-rail margin at VBUS 4.60 V and full load; supply order and DVDD ≤ DVCC on the scope |

# 8. Automation and verification plan

Every check runs automatically with an explicit pass/fail rule.

| Check | Runs when | Pass/fail rule |
| --- | --- | --- |
| ERC and DRC (KiCad command line) | Every commit (GitLab CI) | Zero errors |
| Manufacturing outputs: Gerbers, drill, BOM, pick-and-place, schematic PDF | Every commit | Files generated |
| BOM check | Every commit | Every fitted part has an LCSC number and a verified JLCPCB class (satisfied in Parts List v0.3) and stock ≥ the 5-board quantity; Extended types and leadless parts counted and reported |
| Cost gate | Every commit | 5-board run at a fixed 31.7 NTD/US$, including PCBs and Economic PCBA fees: WARN if > 1,500 NTD per board (target), FAIL if > 1,700 NTD per board (owner-approved ceiling); currently 1,666 NTD: WARN (planning estimate, final with the JLCPCB quote after layout). A 100-unit reference line (≈ 801 NTD) is reported but not gated |
| Python DFF/DFA/DFT and layout rules | Every commit | All rules pass: analog/charge-pump separation, 90 Ω USB pair, decoupling capacitor near every IC power pin, test point on every rail and domain, fiducials present, jack-ground rule |
| AI datasheet-review agent | Every merge request | Reads the DAC, amplifier and regulator datasheets, extracts requirements, checks the schematic and comments on the MR; seeded by the Schematic Design Notes checklist and scored on 10–20 planted errors (catch rate and false alarms) |
| Firmware unit tests (PC build) | Every commit (GitLab CI) | The pure-logic firmware parts (descriptor tables, feedback math, ring buffer, volume mapping) compile on a PC and every unit test passes |
| Firmware gate | Before layout: two-stage proof of concept (below) | Enumerates bus-powered with the new descriptors; every rate plays on Windows 10/11, macOS and Linux with working feedback; BCLK/LRCLK average rates correct for every rate (frequency counter) and the DPLL locks; host volume and mute work; all eight rates bit-perfect; the PLL relocks after every family change with no runt pulses; the CPLD meets timing at 100 MHz and at the PLL output (49.152 MHz) |
| Automated measurement, calibration and screening (Python + QuantAsylum QA403) | Every board | Calibration to 1.75 Vrms per leg, offset screening, THD+N, dynamic range, frequency response, crosstalk, output impedance and maximum power at 16–600 Ω on both jacks; auto-generated report compared against Section 2 |
| Automated THD-compensation tuning loop | Each board revision | Python sweeps the ES9018K2M THD-compensation registers 22–25 (2nd/3rd harmonic) while reading THD+N on the QA403; reports the best coefficients for the optional firmware setting |

Firmware (owner decision, 24 Sep 2026): our own USB Audio Class 2.0 implementation on the CH32V307 high-speed USB port, built on WCH’s register-level USBHS device examples; the chip’s EVT package has high-speed device examples for HID, CDC serial, mass storage and network adapters, but none for audio. TinyUSB (MIT) is an alternative only if its CH32V307 high-speed audio path proves reliable: a 2023 report showed UAC2 control requests failing on this chip, and its driver contributor had not verified isochronous transfers. The USBAudioToI2S firmware (Apache-2.0) stays a known-good comparison on the same hardware and a reference; license notices are kept if any code is copied. The CPLD logic is ours too.

The firmware gate runs as a two-stage proof of concept before layout. Stage 1 is a desk setup of a WCH CH32V307V-EVT-R1 board, an AGM 2K-LUT CPLD board and an ES9018K2M I²S-input DAC board (≈ US$48–57 for the three at listing prices, outside the board budget), brought up in order: enumeration and controls without audio, then streaming into a dummy sink with timer feedback, then the full chain with the CPLD and the DAC. Stage 2 is optional: a digital proof-of-concept board with the real chips and pin map (≈ US$59, about 1,860 NTD, for two assembled boards; estimate). The MCU–CPLD interface is ours to define; its specification and the pin map are frozen before that board and the schematic. Modules, all part of the gate:

| ID | Module |
| --- | --- |
| (1) | USB core on the high-speed port: enumeration, descriptors, standard requests, suspend/resume. Bus-powered descriptors (bmAttributes 0x80, bMaxPower 0xFA = 500 mA); test VID/PID 1209:0001 during development (pid.codes reserves it for private testing only) and a real PID before publishing; serial number from the CH32V307 96-bit unique ID |
| (2) | UAC2 controls: clock source with the advertised rates (CUR/RANGE) and clock valid; Feature Unit mute and volume (CUR/RANGE) mapped to the ES9018K2M attenuator over I²C, plus MCU fine scaling for channel trim; alternate settings for 16, 24 and 32 bit |
| (3) | Streaming: isochronous OUT → ring buffer → DMA to the CPLD |
| (4) | Feedback: an MCU timer counts the CPLD’s LRCLK against USB (micro)frames, plus a small FIFO-level correction, sent in 16.16 format. At 384 kHz, 32-bit stereo: 384 bytes per 125 µs microframe, up to 392 with one extra sample; feedback 48.0 samples per microframe = 0x300000 |
| (5) | Application: ES9018K2M driver (asynchronous mode, DPLL bandwidth, 32-bit I²S, soft start, mute, lock status, master trim); rate-family change sequence (mute, PLL reset, oscillator enables, relock, unmute); power-state machine and sequencing, CC-based power mode, ILIM_HI control, VBUS watchdog; fault and overload policies; calibration storage and test modes (DC fault injection by a digital DC signal; I²S-header tristate) |
| (6) | CPLD logic: FIFO; I²S output engine clocked from the PLL (×2 on the selected family oscillator: 49.152 MHz or 45.1584 MHz) with exact integer dividers; the oscillator enables (OSC48_EN, OSC44_EN) and the PLL reset for the family change; SPI registers, input side and FIFO write side on the 100 MHz system clock. No clock multiplexer in the fabric; the MCLK pin is unused (MCLK goes straight from X201 to the DAC) |
| (7) | MCU–CPLD interface, ours to define (for example samples by DMA over I²S or SPI, FIFO level over SPI, LRCLK back to a timer-capable MCU pin): written as a specification and frozen with the pin map before the proof-of-concept board and the schematic |

# 9. Risks and open decisions

## 9.1 Risks

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Own UAC2 firmware and CPLD logic: top risk (host compatibility of high-speed isochronous streaming with feedback) | No working USB audio; schedule | Two-stage proof of concept before layout (Section 8); USBAudioToI2S as a known-good comparison on the same hardware; PC-side unit tests in CI |
| CPLD timing at 100 MHz and at the PLL output (49.152 MHz), and PLL relock after each family change | Wrong rates, runt pulses or DPLL unlock | Part of the firmware gate; scope check at family changes |
| X203 disabled output must be high-impedance (shared node with X202) | Contention on the PLL input; corrupted bit clock | Datasheet check before the schematic; fallback: a small 2:1 logic multiplexer in front of the PLL input |
| Shared family-oscillator node routing | Reflections or crosstalk at the PLL input; bit-clock jitter | Both family oscillators and the node next to the CPLD PLL clock pin, short traces, each through its own 33 Ω; scope check at bring-up |
| 100 MHz MCLK at the ES9018K2M maximum | DAC timing margin | Verify; look for a stocked 80–98 MHz oscillator |
| MCU and CPLD currents are assumptions (oscillators at datasheet maxima) | Unconfigured state above 100 mA; Default-mode budget | Measure the unconfigured state; lower the MCU clock before configuration if needed |
| USB Default-mode worst case 0.473 A (27 mA below 0.5 A) | Default-mode budget | Measure in every state at bring-up; both family oscillators are counted at their maxima, so the real figure is lower |
| USB suspend ≤ 2.5 mA not guaranteed in v1 | USB 2.0 suspend compliance | Open item; audio domains off and MCU in low-power mode during suspend |
| DAC pin-to-pin DC mismatch (assumed ≤ 1 mV) and null point (assumed ±20 mV) | Output DC offset | Measure; 100 % offset screening |
| OPA1622 swing and current capability at ±3.78 V are extrapolated from typical ±18 V curves | Output power table | Bench-verify the power table |
| LM27762 charge-pump resistance at 4.5 V input (3.0 Ω assumed) | Negative-rail margin (0.17 V at VBUS 4.60 V) | Measure the negative-rail margin at 4.60 V |
| Stock of CH32V307, ES9018K2M, AGRV2K, ME6216 and the 4.4 mm jack | Schedule and cost | Reserve parts in the JLCPCB library before ordering |
| Cost 34 NTD under the ceiling (conservative planning estimate) | Budget | JLCPCB quote from the real BOM/CPL after layout; ≈ 1,569 NTD if the four conservative fees do not apply |
| Family oscillators X202 and X203: X202’s JLCPCB class not shown (assumed Extended), and its JLCPCB page notes that an assembly fixture may be needed | Cost and assembly | Confirm the classes and fixtures in the JLCPCB quote |
| Unplug pop ordering not guaranteed | Pop at unplug | Measure; the VBUS watchdog drops AMP_EN first |
| 4.4 mm plug shorted leg+ to leg− is outside TI’s one-amplifier short rating | OPA1622 stress (≈ 0.55 W per package) | Bench survival test |
| OPA2197 stability with the DAC pin capacitance and the 10 Ω + 660 pF network | Oscillation or peaking | SPICE before layout; bench check |
| Both jacks live; a 4.4-to-3.5 mm adapter shorts leg− | One amplifier per package shorted (≈ 0.45 W) | Warning in the manual |

## 9.2 Decisions (closed for v0.5; verification items open)

| Decision | Status | Resolution |
| --- | --- | --- |
| USB bridge | Closed | WCH CH32V307RCT6 + AGM AGRV2KQ32 CPLD |
| USB firmware and CPLD logic | Closed (owner decision, 24 Sep 2026) | Our own UAC2 firmware on WCH’s register-level USBHS device examples and our own CPLD logic (Section 8); USBAudioToI2S kept only as a known-good comparison and reference; TinyUSB only if its CH32V307 high-speed audio path proves reliable |
| DAC and output mode | Closed | ESS ES9018K2M in differential current mode with one I/V converter per pin (ESS application circuit); the newer ES9039Q2M / ES9020Q are a future option |
| Clock architecture | Closed (changed in the v0.5 amendment) | DAC MCLK 100 MHz from X201 (asynchronous mode), which also clocks the CPLD system logic; family oscillators X202 24.576 MHz and X203 22.5792 MHz selected by their enable pins in front of the CPLD PLL (×2); all eight rates exact with a 50:50 bit clock; no clock multiplexer in the fabric |
| I/V op-amp | Closed (changed in v0.5) | OPA2197 (Vos ±0.25 mV max), feedback 836 Ω with 10 Ω + 660 pF |
| Output stage | Closed | 2 × OPA1622 difference amplifiers fed by the I/V outputs, one amplifier per leg; clean envelope ≤ 80 mA peak per leg |
| Headphone rails | Closed (changed in v0.5) | ±3.78 V from the LM27762 (215 kΩ/100 kΩ and 210 kΩ/100 kΩ from Basic values); both rails budgeted at VBUS 4.60 V |
| Level and calibration | Closed (changed in v0.5) | Every unit calibrated to 1.75 Vrms per leg with the master trim at the end-of-line test; −3.5 dB before calibration |
| Power modes | Closed (changed in v0.5) | Default (≤ 500 mA, hardware-limited to ≤ 0.473 A) and High (only with a ≥ 1.5 A Type-C advertisement read on CC by the MCU ADC); bus-powered descriptors 0x80 / 0xFA |
| Analog current limit | Closed (changed in v0.5) | TPS2553 with two limits (Default 0.157–0.213 A, High 0.393–0.498 A); overload → PGOOD kill → firmware volume ceiling |
| Power domains | Closed (changed in v0.5) | 3V3M always on; 3V3D (ME6211 #2 CE: CPLD, oscillators, DVCC), 3V3A (LP5907 EN) and the rails (TPS2553 EN) switched; fixed 1.3 V ME6216A13M3G DVDD fed from 3V3D |
| Offset limit | Closed (changed in v0.5) | ±10 mV unbalanced and ±20 mV balanced (was ±15 mV), with 100 % screening |
| Protection | Closed (changed in v0.5) | LM393 DC detect; N-MOSFET latch with a blanked set path; PGOOD hardware kill of AMP_EN; rails first on, last off; no relays |
| Connectors | Closed | 3.5 mm unbalanced + 4.4 mm balanced; one headphone at a time |
| Volume | Closed | Host volume (UAC2 Feature Unit) in the ES9018K2M attenuator; hardware knob is a v2 option |
| Budget | Closed (owner decision, 24 Sep 2026) | Target 1,500 NTD per board; approved ceiling raised from 1,550 to 1,650 NTD with the second oscillator and to 1,700 NTD with the third (currently 1,666 NTD: WARNING) |
| Firmware gate | Open (gate before layout) | Two-stage proof of concept: Stage 1 desk setup, optional Stage 2 digital board; pass criteria in Section 8 |
| MCU–CPLD interface and pin map | Open (before the proof-of-concept board) | Write the interface specification and freeze the pin map before the proof-of-concept board and the schematic |
| X203 disabled output | Open (datasheet check) | Confirm in the SCTF datasheet that the disabled output is high-impedance (shared node with X202); fallback: a small 2:1 logic multiplexer in front of the CPLD PLL input |
| 100 MHz MCLK margin | Open (verification) | Look for a stocked 80–98 MHz oscillator |
| Unconfigured-state current ≤ 100 mA; Default-mode worst case 0.473 A | Open (verification) | Measure at bring-up (Default mode is 27 mA below 0.5 A) |
| USB suspend ≤ 2.5 mA | Open | Not guaranteed in v1 |
| DAC pin mismatch and zero-current point | Open (verification) | Measure; 100 % offset screening |
| OPA2197 stability with the DAC pin capacitance | Open (verification) | SPICE before layout |
| OPA1622 swing at ±3.78 V and negative-rail margin at 4.60 V | Open (verification) | Bench measurement |
| Final cost | Open (after layout) | JLCPCB quote from the real BOM/CPL files; confirm the classes and fixture notes of X202 and X203 |

# 10. Next steps

| # | Action | Done |
| --- | --- | --- |
| 1 | Define the MCU–CPLD interface (sample transfer, FIFO level, LRCLK capture, SPI registers, oscillator enables) and freeze the pin map before the proof-of-concept board and the schematic; confirm in the SCTF datasheet that X203’s disabled output is high-impedance. | ☐ |
| 2 | Stage 1 proof of concept (firmware gate, Section 8): enumeration and controls, streaming with timer feedback, then the full chain with the CPLD and the DAC; every rate on Windows, macOS and Linux, all eight rates bit-perfect, PLL relock after every family change without runt pulses, CPLD timing at 100 MHz and at the PLL output (49.152 MHz). Stage 2 digital board only if needed. | ☐ |
| 3 | SPICE the I/V converters (OPA2197 with 10 Ω + 660 pF and the DAC pin capacitance) and the OPA1622 legs with TI models; confirm stability, level and frequency response. | ☐ |
| 4 | Draw the schematic from Schematic Design Notes v0.4; run the cost gate. | ☐ |
| 5 | Get a schematic review. | ☐ |
| 6 | Layout. | ☐ |
| 7 | Get the JLCPCB quote from the real BOM/CPL files (final cost gate); check the JLCPCB classes and fixture notes of X202 and X203. | ☐ |
| 8 | Order the 5-board run. | ☐ |
| 9 | Bring-up, calibration and automated measurement against Section 2. | ☐ |

# Appendix A. Reference documents

| Ref | Document | Source |
| --- | --- | --- |
| [1] | ES9018K2M datasheet v3.7 (22 Apr 2021) | ESS Technology |
| [2] | OPA1622 datasheet (SBOS727B) | Texas Instruments |
| [3] | OPA2197 datasheet | Texas Instruments |
| [4] | LM27762 datasheet (SNVSAF7C) | Texas Instruments |
| [5] | TPS2553 datasheet (SLVS841F) | Texas Instruments |
| [6] | CH32V307 datasheet and EVT package (high-speed USB device examples) | WCH |
| [7] | AGRV2K CPLD datasheet | AGM |
| [8] | USBAudioToI2S: UAC2.0-to-I²S firmware for MCU + CPLD (Apache-2.0); known-good comparison and reference | GitHub, LoveLonelyTime/USBAudioToI2S |
| [9] | USB Type-C specification (current-advertisement voltage bands) | USB-IF |
| [10] | JLCPCB PCBA price list | JLCPCB |
| [11] | Schematic Design Notes v0.4 (companion document: block-level circuit guidance and chosen parts) | This project |
| [12] | Parts List v0.3 (source of truth for cost) | This project |

# Appendix B. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Draft for review |
| v0.2 | 23 Sep 2026 | Diego Yang | Budget raised to ≤ 1,500 NTD; outputs changed to 3.5 mm unbalanced + 4.4 mm balanced; parts and targets verified against the XU316-1024-QF60A, AK4493S, AKD4493S-A and OPA1622 datasheets; USB chip changed to XU316-1024-QF60A (1.8 V I/O); OPA1612 stage-1 and LT3045 added; CH224K replaced by CH224Q; JLCPCB fees updated to the 8 Sep 2026 price list; most open decisions closed. |
| v0.3 | 23 Sep 2026 | Diego Yang | Remaining parts chosen from stocked LCSC/Digi-Key parts; CH224A replaces CH224Q; TPS7A4901 replaces LT3045 to stay within 1,500 NTD; protection defined (LM339 + 2 × HFD4 relays with hardware amplifier shutdown); cost re-estimated with real prices at ≈ 1,490 NTD; −14 V rail set to −14.7 V; stage-1 count corrected to one dual OPA1612. |
| v0.4 | 23–24 Sep 2026 | Diego Yang | Cost-driven redesign for the 5-board budget: single USB-C bus-powered design; CH32V307 + AGRV2K CPLD USB bridge (open-source USBAudioToI2S); ES9018K2M DAC; 2 × OPA1622 as four difference amplifiers; LM27762 charge-pump rails; relays replaced by a 3.3 V hardware fault latch; three design reviews dispositioned. Revised on 24 Sep 2026: DAC output moved to current mode with I/V converters (owner decision, option B) and an owner-approved cost ceiling of 1,550 NTD per board. |
| v0.5 | 24 Sep 2026 | Diego Yang | Fourth review round (R4-A, R4-B, R4-C) dispositioned (Appendix C): headphone rails ±3.78 V; per-unit calibration to 1.75 Vrms per leg (Rf 836 Ω); power rated at 1 % THD+N or calibrated 0 dBFS, with High and Default modes; TPS2553 current limit and switched power domains per USB state; OPA2197 I/V converters; one 100 MHz oscillator with the DAC in asynchronous mode and fractional BCLK/LRCLK in the CPLD; protection completed (PGOOD kill, blanked latch, energy bound at the terminals); fixed 1.3 V DVDD LDO; balanced offset limit ±20 mV with 100 % screening; Parts List v0.3 with verified LCSC numbers and classes; cost 1,540 NTD per board (WARNING, under the 1,550 NTD ceiling); pre-layout firmware gate. |
| v0.5 amendment | 24 Sep 2026 | Diego Yang | Amendment for the owner decisions of 24 Sep 2026: family oscillators for exact bit clocks, because the ES9018K2M requires a bit-clock duty cycle of 45:55 to 55:45 and one 100 MHz clock for both families gave 40:60 at 352.8/384 kHz. First X202, 24.576 MHz (YXC OT322524.576MJBA4SL), for the 48 kHz family; then, at the owner’s request, X203, 22.5792 MHz (SCTF SX2M22.5792M20F30TNN), for the 44.1 kHz family, which replaced the interim both-edge fractional divider and its 352.8 kHz condition. Final clock architecture: DAC MCLK 100 MHz unchanged, family oscillator selected by its enable pin in front of the CPLD PLL (×2), all eight rates exact, family change with the PLL held in reset, no clock multiplexer in the fabric. Own UAC2 firmware and CPLD logic instead of modifying USBAudioToI2S (kept as a known-good comparison), with the firmware gate run as a two-stage proof of concept. Currents updated (Default ≤ 0.473 A, High ≤ 0.758 A); cost 1,540 → 1,602 → 1,666 NTD per board (WARNING), approved ceiling raised from 1,550 to 1,650 and then 1,700 NTD; Figures 1 and 2 updated. |

# Appendix C. Design review disposition

Status values: Closed = fixed in this revision; Closed – design, verify = design change made, confirmed by a bench or firmware-gate check; Closed – verify = resolved, confirmed at the firmware gate or at bring-up; Closed – N/A = not applicable after the v0.4 redesign; Open = tracked item with an owner and a gate (Section 9.2).

## C.1 Fourth review round (reviews R4-A, R4-B and R4-C of Spec v0.4)

Finding IDs: A- = R4-A, B- = R4-B, C- = R4-C.

| ID | Finding | Disposition in v0.5 | Status |
| --- | --- | --- | --- |
| A-H1 | FB101 mis-specified (GZ1608D601TF = 200 mA / 450 mΩ, C1002; C1015 is GZ2012D101TF) | Power ferrite now Murata BLM18PG121SN1D (C14709, Basic, 2 A, 50 mΩ) in the 5V_ANA path; GZ1608D601TF (C1002) only on DVCC (≈ 14 mA); drops recomputed (≈ 15–25 mV) | Closed |
| A-H2 | Offset used OPA1678 typical as maximum | Confirmed (±0.5 typ / ±2 mV max); I/V op-amp now OPA2197 (±0.25 mV max); budget 7.9 / 15.9 mV worst, balanced limit ±20 mV, 100 % screening; DAC-mismatch assumption Open (measure) | Closed |
| A-H3 | Safe disconnected output not established | Full sequence and current paths, rails first on/last off, PGOOD hardware kill, back-powering bounded (≤ 1.7 mA/pin) and removed in 10 ms, energy bound ≤ 16.5 mJ before / ≤ 0.22 mJ after the trip, protection defined at the headphone terminals | Closed – design, verify |
| A-H4 | Negative-rail headroom | Rails ±3.78 V; both rails budgeted at 4.60 V and maximum load with a 3.0 Ω charge-pump resistance (margin 0.17 V) | Closed |
| A-H5 | 100 mA ceiling not enforced | Clean envelope (80 mA/leg, swing curve) separated from overload; TPS2553 enforces the analog supply current; PGOOD kill; firmware ceiling steps; overload dissipation bounded; overload tests replace the 30-min 32 Ω balanced test | Closed – design, verify |
| A-H6 | USB power states and descriptors | State-by-state budget, MCU-only before configuration (≤ 83 mA estimate), descriptors 0x80/0xFA, CC detection by ADC for High mode, suspend policy (2.5 mA Open); verified in the firmware gate | Closed – design, verify |
| A-M1 | Mixed purchasing lines and placeholder ranges | One part per line, every value specified, 48 purchasing lines by LCSC number | Closed |
| A-M2 | 13.8 NTD ceiling margin; one Basic→Extended flip fails it | Every class now verified on the JLCPCB page (X202 assumed Extended); Extended count conservative (19, incl. THT jack and X202); 34 NTD under the 1,700 NTD ceiling; still a planning estimate until the JLCPCB quote | Closed – verify |
| A-M3 | Gain tolerance defeats the minimum power targets | Per-unit calibration to 1.75 Vrms/leg; targets defined at the calibrated level | Closed |
| A-M4 | Two LDOs do not guarantee sequencing | Separate enables (LP5907 EN, ME6211 CE) in a firmware sequence; DVDD LDO fed from 3V3D so DVDD ≤ DVCC by construction; scope check at bring-up | Closed – design, verify |
| A-M5 | Clock adaptation needs an earlier gate | Three oscillators: 100 MHz straight to the DAC MCLK (asynchronous mode) and the CPLD system clock; 24.576 MHz or 22.5792 MHz selected in front of the CPLD PLL (×2) for the two families (exact ratios); pre-layout firmware gate as a two-stage proof of concept | Closed – design, verify |
| B-S1 | −4.5 V rail cannot regulate from USB | As A-H4 | Closed |
| B-S2 | 2.0 Vrms/leg does not fit the supply; +1 dB units clip | Calibrated 1.75 Vrms/leg; swing model from OPA1622 Fig. 38/39; pre-calibration trim −3.5 dB | Closed |
| B-S3 | Power targets fail on low-spread units; rating method | As A-M3: “1 % THD+N or calibrated 0 dBFS, whichever first” | Closed |
| B-S4 | Firmware needs much more than the spec said | Own UAC2 firmware and CPLD logic, modules listed (Section 8); two-stage proof of concept before layout; 352.8 kHz from the 22.5792 MHz oscillator through the CPLD PLL (exact) | Closed – verify |
| B-S5 | 14 NTD margin will be used up (odd values, 1.5 nF C0G, crystal, AMS1117, THT jack fee) | Basic E24 values or Basic combinations used (100 k + 47 k, 1 k ∥ 5.1 k, 15 k + 1 k, 2 × 330 pF), Basic 5032 crystal, AMS1117 removed, THT fee counted; no cheaper 4.4 mm jack exists at LCSC (SMD option not available) | Closed – verify |
| B-M1 | Pop at unplug has no hardware control | PGOOD gates AMP_EN in hardware; VBUS watchdog; separate unplug pop target (≤ 20 mV) | Closed – design, verify |
| B-M2 | Fault latch re-trips on clear | Set path blanked ≈ 60 ms after PGOOD good, without clearing the latch | Closed |
| B-M3 | PLL jitter goes to the DAC in sync mode | DAC still in asynchronous mode with no PLL in the MCLK path (MCLK straight from the 100 MHz oscillator); the CPLD PLL is only in the bit-clock path of both families, whose jitter the DAC’s DPLL rejects; J-test stays in the plan | Closed |
| B-M4 | > 100 mA before enumeration; phones limit OTG | As A-H6; Default mode ≤ 0.473 A covers phones | Closed |
| B-M5 | AMS1117-ADJ needs tantalum-type capacitor and minimum load | Fixed 1.3 V ME6216A13M3G (±1 %, 300 mA, ceramic-stable) | Closed |
| B-minor | Duplicate “v0.4” rows in Appendix B; U301 “voltage-mode” text; HSE 8 MHz; VREF risk overstated | History cleaned (this is v0.5); U301 text corrected; HSE 8 MHz confirmed; VREF risk downgraded and quantified (1.17 mV) | Closed |
| C-B1 | USB power before configuration | As A-H6 | Closed – verify |
| C-B2 | Reference descriptors wrong (self-powered, 100 mA) | 0x80 / 0xFA in the firmware gate | Closed – verify |
| C-B3 | 352.8 kHz MCLK not supported by the reference CPLD | Confirmed in source; replaced by our own CPLD logic with the family oscillator selected before the PLL, as the reviewer suggested (X202 24.576 MHz or X203 22.5792 MHz, PLL ×2); all eight rates exact; MCLK 100 MHz straight to the DAC (asynchronous mode) | Closed – design, verify |
| C-B4 | Clock-family switching can glitch | MCLK never switches; the family changes by the oscillator enables while the I²S output is stopped and the PLL is held in reset; no clock multiplexer in the fabric; scope check for runt pulses | Closed – design, verify |
| C-B5 | Host volume not implemented | UAC2 volume/mute with GET/SET CUR/RANGE and I²C mapping, tested on three OSes in the gate | Closed – verify |
| C-B6 | “100 mA design current” not implemented | As A-H5 | Closed – design, verify |
| C-H1 | 500 mA budget has no margin | Default mode ≤ 0.473 A enforced (27 mA margin, measured at bring-up); High mode only with a ≥ 1.5 A Type-C advertisement | Closed |
| C-H2 | +4.5 V corner | Rails ±3.78 V, both rails budgeted | Closed |
| C-H3 | 7.7 µF table is not an inrush proof | Attach charge 38 µC including 3V3M output capacitance; staggered, soft-started domains | Closed – verify |
| C-H4 | Cost provisional; “Assign LCSC no.” left; “every part is LCSC” inaccurate | Every line has an LCSC number and verified class | Closed |
| C-H5 | Offset margin | As A-H2 | Closed |
| C-H6 | ≤ 5 mV pop ambitious without relays | Split targets, full sequence incl. unplug/brownout validated on the bench | Closed – verify |
| C-H7 | USB identity (WCH VID, PID 0) | Use the pid.codes VID 0x1209 with an assigned PID; serial number from the MCU unique ID | Closed – verify |

## C.2 Earlier review rounds (three reviews of Spec v0.3)

These findings were dispositioned in v0.4; dispositions that v0.5 supersedes are updated to the current design, with a pointer to the fourth-round finding where it applies.

| ID | Finding | Disposition (current design) | Status |
| --- | --- | --- | --- |
| R1-B1 | VBUS_PD capacitance ≈ 130 µF vs the USB-C limit of 10 µF | No PD port; single 5 V port; attach charge ≈ 38 µC (see C-H3) | Closed – N/A |
| R1-B2 | LMR33630 + SMAJ18A surge stacking ≈ 44 V | No inverting converter and no 15 V input; the LM27762 runs from 5 V | Closed – N/A |
| R1-B3 | LP5907 on 1V8_D over 10 µF output capacitance | No 1.8 V rails; the only LP5907 (DAC 3.3 V) is kept within its 10 µF output-capacitance limit | Closed – N/A |
| R1-B4 | Paralleling topology left open | No paralleling; one amplifier per leg with a clean envelope of ≤ 80 mA peak (see A-H5) | Closed |
| R1-B5 | LM339 at −12 V pulling 1.8 V logic | The comparator outputs reach the 3.3 V domain only through a 10 kΩ / 47 kΩ node clamped by a 1N4148WS; the latch uses N-MOSFETs only, and 3.3 V nodes never go below 0 V | Closed |
| R1-B6 | Offset target ±5 mV vs an 11 mV worst case | Worst case recomputed with datasheet maxima (7.9 / 15.9 mV); limits ±10 / ±20 mV with 100 % screening (see A-H2) | Closed |
| R1-H | AMS1117 dropout and noise | ME6211 for digital 3.3 V; LP5907 for DAC analog 3.3 V; fixed 1.3 V ME6216A13M3G for DVDD (see B-M5) | Closed |
| R1-H | XU316 VOH vs AK4493 VIH | All logic is 3.3 V | Closed – N/A |
| R1-H | Both jacks live; gain-switch pop | Gain switch removed; both jacks live documented (one headphone at a time) | Closed |
| R1-H | OPA1622 stock 197 vs 400 needed | The 5-board sample run needs 10 (2 per board) | Closed |
| R1-H | LM317/LM337 cost trade | LM27762 low-noise LDOs | Closed – N/A |
| R1-H | CI rule “every part has an LCSC number” vs Digi-Key/Mouser parts | Every fitted part has an LCSC number and a verified JLCPCB class (Parts List v0.3, see C-H4) | Closed |
| R1-H | Firmware port map before layout | Pre-layout firmware gate as a two-stage proof of concept (Section 8, see B-S4); the MCU–CPLD interface and the pin map are frozen before the proof-of-concept board and the schematic | Closed |
| R1 consistency | v0.2 wording, cost source of truth, THD+N conditions, Zout including contacts, DSD pins, grounding/ESD/CC/silkscreen | Fixed in the spec: the single USB-C port removes the silkscreen confusion; CC = 5.1 kΩ Rd; jack ESD LESD5D5.0CT1G; grounding rules added; DSD not in v1 | Closed |
| R2-S1 | TVS does not protect the −14.7 V converter; no OVP | No 15 V input and no inverting converter | Closed – N/A |
| R2-S2 | VBUS bulk before the PD contract; analog rails start at 5 V | No PD contract; single 5 V port | Closed – N/A |
| R2-S3 | Balanced output impedance with option B | The OPA1622 legs drive the jacks directly, no ballast (< 0.1 Ω + contacts) | Closed |
| R2-S4 | K1000 1 W with no margin | Closed by requirement change (budget redesign): ≈ 102 mW and ≈ 94 dB SPL into the K1000 documented | Closed |
| R2-S5 | Offset target contradicts the design | As R1-B6 | Closed |
| R2-M1 | Board ≈ 7 W in a PLA case | Analog input power bounded by the TPS2553 limit; board ≈ 3.8 W worst case (High mode, overload); overload tests (Section 7) | Closed |
| R2-M2 | LM317 near dropout; LM337 LC filter | Blocks removed (LM27762) | Closed – N/A |
| R2-M3 | ±0.05 dB trim cannot use 0.5 dB DAC steps | Optional digital scaling in the MCU firmware | Closed |
| R2-M4 | Maximum-power criterion | Rated at 1 kHz, both channels, at 1 % THD+N or calibrated 0 dBFS, whichever comes first (see B-S3) | Closed |
| R2-M5 | Cost margin 10 NTD; exchange rate | Gate uses a fixed 31.7 NTD/US$: target 1,500 NTD per board (WARN above) and owner-approved ceiling 1,700 NTD (FAIL above); currently 1,666: warning | Closed |
| R2-M6 | 4.4-to-3.5 mm adapter shorts leg− | One amplifier per package shorted (TI continuous-short rating), ≈ 0.45 W per package; risk and manual warning | Closed |
| R2-M7 | Unbalanced crosstalk needs ≤ 6 mΩ shared ground | Layout rule (≤ 5.7 mΩ) | Closed |
| R2 minor | Stale versions; KC2520Z “(assumed)”; through-hole count; pop requirement; unverified CH224A/AMS1117 claims; NVIDIA paragraph | Versions fixed; the three oscillators are counted as leadless conservatively; only the 4.4 mm jack is through-hole (confirm USB-C shell pegs); pop ≤ 5 mV controlled and ≤ 20 mV at unplug; CH224A removed; unverified regulator claims removed (see B-M5); job-application text moved to the repository README | Closed |
| R3-1 | DC fault latch and recovery | Hardware 3.3 V latch; firmware recovery policy (Section 5.3); status to the MCU | Closed |
| R3-2 | TVS/converter envelope | No 15 V input and no converter | Closed – N/A |
| R3-3 | Stage-1 gain 0.9009 with the input network | The gain includes the DAC’s 806 Ω source (current mode, gain = 2 × Rf/806 per leg); v0.5 uses Rf = 836 Ω and per-unit calibration to 1.75 Vrms per leg (see A-M3) | Closed |
| R3-4 | QA403 cannot drive 1.8 V I²S | I²S is now 3.3 V; header, firmware test mode and sequencing specified | Closed |
| R3-5 | 200 mA “limit” not implemented; thermal at 16 Ω balanced | Clean envelope ≤ 80 mA peak per leg; rated power at 1 % THD+N or calibrated 0 dBFS; overload bounded by the TPS2553 (see A-H5) | Closed |
| R3 consistency | Rails-good qualification; offset/trim; workbook external links; cost/sourcing reconciliation | Rails-good: LM27762 PGOOD gates AMP_EN in hardware and arms the latch ≈ 60 ms after PGOOD good. Offset/trim: as R1-B6 and R2-M3. Workbook links: the delivered v0.1 file references only its own sheets (links appear when sheets are copied into another workbook); v0.2 adds a self-check. Cost/sourcing: single source of truth (Parts List v0.3), all LCSC | Closed |