# USB DAC + Balanced Headphone Amplifier — Schematic Design Notes v0.4

Companion to Design Specification v0.5: datasheet-derived requirements and starting component values for the KiCad schematic after the fourth review round

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 24 Sep 2026 | v0.4 | Schematic design notes (companion to Design Specification v0.5) |

# 0. Purpose and references

This document collects the datasheet-derived requirements and starting component values for drawing the KiCad schematic of the USB DAC + balanced headphone amplifier. It is the companion to Design Specification v0.5 and Parts List v0.3 and replaces Notes v0.3, which stays unchanged as history. This revision answers the fourth review round (reviewers R4-A, R4-B and R4-C, Section 7.2): lower headphone rails, a calibrated output level, a hardware-enforced analog current limit, switched USB power domains, the DAC in asynchronous mode, completed protection and a one-part-per-line BOM. The amendment of 24 Sep 2026 adds two owner decisions: family oscillators X202 (24.576 MHz) and X203 (22.5792 MHz), selected in front of the CPLD PLL, with a new clock architecture, and our own USB firmware and CPLD logic instead of modifying the open-source USBAudioToI2S project (Sections 1, 3.3 and 5).

The designer owns and verifies every value. Circuit values are starting values unless marked otherwise, to be checked in SPICE and on the bench. Every number is computed from the stated values or taken from the datasheet named next to it. Labels are kept as written: “assumption” marks a value not yet confirmed, “to verify” marks a check still to be done, and “per datasheet — to verify” marks a value that must come from a datasheet not yet read. All costs use a fixed 31.7 NTD per US$.

| Ref | Document | Publisher | Document no. / note |
| --- | --- | --- | --- |
| R1 | CH32V307 datasheet and reference manual; CH32V307 EVT package (high-speed USB device examples: HID, CDC serial, mass storage, network adapter; no audio example) | WCH | — |
| R2 | AGRV2K datasheet | AGM | — |
| R3 | ES9018K2M datasheet | ESS | v3.7 (22 Apr 2021); pages 8, 27 and 31 used for the output stage, power-up and MCLK limits; page 30 for the audio-interface timing |
| R4 | OPA1622 datasheet | TI | SBOS727B; Fig. 38/39 (output swing) |
| R5 | LM27762 datasheet | TI | SNVSAF7C |
| R6 | LP5907 datasheet | TI | — |
| R7 | USBAudioToI2S firmware project | GitHub, LoveLonelyTime | Apache-2.0; known-good comparison and reference only (the firmware and CPLD logic are our own); source (audio_cpld.v) reviewed for the clock path |
| R8 | JLCPCB PCBA price list | JLCPCB | Economic PCBA fees used for the 5-board cost gate |
| R9 | TPS2553 datasheet | TI | SLVS841F (current-limit equations, Figure 24) |
| R10 | OPA2197 datasheet | TI | — |
| R11 | USB Type-C specification | USB-IF | CC voltage bands for the current advertisement |
| R12 | LCSC and JLCPCB part pages | LCSC / JLCPCB | Classes, prices and stock, Jun–Sep 2026 (sourcing evidence for Parts List v0.3) |

# 1. Key design decisions

The decisions below fix the v0.4 schematic. The Basis column records why each choice was made and which review finding it answers (finding IDs in Section 7.2). The clock, firmware and cost rows follow the owner decisions of 24 Sep 2026.

| Decision | Choice | Basis |
| --- | --- | --- |
| Cost target | ≤ 1,500 NTD per board for the 5-board sample run (incl. bare PCBs and JLCPCB assembly fees; excl. shipping, import tax, programmers and enclosure); owner-approved ceiling 1,700 NTD per board | Owner decisions (5-board budget; ceiling raised on 24 Sep 2026 from 1,550 to 1,650 and then 1,700 NTD per board with the second and third oscillators) |
| Cost status | 1,666 NTD per board (8,330 NTD for 5 boards) | WARNING: 34 NTD under the 1,700 NTD ceiling; ≈ 1,569 NTD per board without the conservative fees (Section 8). Parts List v0.3 is the source of truth; final with the JLCPCB quote after layout |
| USB audio bridge | WCH CH32V307RCT6 + AGM AGRV2KQ32 CPLD; our own USB Audio Class 2.0 firmware and our own CPLD logic (Section 5) | Owner decision 24 Sep 2026; USBAudioToI2S (R7) is kept only as a known-good comparison. Two-stage proof of concept before layout (R4-B S4; R4-C B3–B5, H7) |
| Clock | DAC MCLK 100 MHz from X201 (asynchronous mode, never switched), also the CPLD system clock; family oscillators X202 24.576 MHz and X203 22.5792 MHz share the CPLD PLL input and are enabled one at a time; PLL ×2 gives exact 50:50 bit clocks for all 8 rates, 352.8 kHz included (3.3) | The ES9018K2M needs a DATA_CLK duty cycle of 45:55 to 55:45 (R3 p. 30); one 100 MHz clock for both families gave 40:60 at 352.8/384 kHz. Owner decisions 24 Sep 2026 (+62 and +64 NTD per board); the oscillator is selected before the PLL, as R4-C suggested (R4-A M5; R4-B S4, M3; R4-C B3, B4) |
| DAC | ESS ES9018K2M, I²C register control; differential current-mode outputs into I/V converters (ESS application circuit, R3 p. 27) | DNR 127 dB-A and THD+N −120 dB per datasheet, specified in differential current mode and asynchronous mode |
| DAC core supply | Fixed 1.3 V LDO Microne ME6216A13M3G (±1 %, 300 mA, ceramic-stable) fed from the switched 3V3D domain | External DVDD is required for FSR > 192 kHz (R3). Fed from 3V3D, DVDD cannot exceed DVCC during ramps; the previous adjustable LDO needed a tantalum-type capacitor and a 5–10 mA minimum load (R4-B M5; R4-A M4) |
| Headphone rails | ±3.78 V from TI LM27762: FB+ 215 kΩ / 100 kΩ → +3.780 V; FB− 210 kΩ / 100 kΩ → −3.782 V; each top resistor built from Basic values (100 kΩ + 100 kΩ + 15 kΩ or 10 kΩ) | The negative rail (charge pump + negative LDO) keeps regulation at the USB supply corner: 0.17 V margin at VBUS 4.60 V and maximum load (2.3) (R4-A H4; R4-B S1; R4-C H2) |
| Output level | Every unit calibrated to 1.75 Vrms per leg at 0 dBFS (3.5 mm 1.75 Vrms, 4.4 mm 3.5 Vrms) with the ES9018K2M 32-bit master trim, stored in MCU flash at the end-of-line test; −3.5 dB before calibration; Rf = 1 kΩ ∥ 5.1 kΩ = 836 Ω | Untrimmed unit spread 1.955–2.589 Vrms per leg; the calibrated level fits the ±3.78 V supply. 836 Ω uses Basic values (R4-B S2, S3; R4-A M3) |
| Power rating method | Rated at 1 % THD+N or calibrated 0 dBFS, whichever comes first (both channels, 1 kHz, calibrated level); the clean output envelope (≤ 80 mA peak per leg and the OPA1622 swing curve) is kept separate from overload behaviour | R4-A H5, M3; R4-B S3; R4-C B6 |
| Analog current limit | TI TPS2553 current-limited switch feeds the LM27762 with two limits: Default 0.157–0.213 A (RILIM 147 kΩ) and High 0.393–0.498 A (59.5 kΩ) | Overload → rails droop → LM27762 PGOOD → amplifiers disabled in hardware → firmware lowers the volume ceiling (R4-A H5, H6; R4-B M4; R4-C B6, H1) |
| USB power states | Before configuration only the MCU domain 3V3M is powered (≤ 83 mA estimate; assumption on MCU current). Audio domains are switched: ME6211 #2 CE (3V3D), LP5907 EN (3V3A), TPS2553 EN (5V_ANA). CC1/CC2 are read by the MCU ADC through 10 kΩ to select Default (≤ 500 mA, hardware-limited to ≤ 0.473 A) or High mode (≥ 1.5 A advertised). Descriptors: bus-powered, bmAttributes 0x80, bMaxPower 0xFA (500 mA) | R4-A H6; R4-C B1, B2, H1 |
| I/V op-amp | TI OPA2197 (Vos ±0.25 mV max, 5.5 nV/√Hz, rail-to-rail I/O, 1 mA per channel; LCSC C139363) | The offset budget now uses datasheet maxima; the previous I/V part’s ±2 mV maximum alone would give 8.7 mV per leg (R4-A H2) |
| Output legs | 2 × TI OPA1622, one amplifier per leg; each leg a 4-resistor difference amplifier (2.00 kΩ 0.1 %) fed by two I/V outputs | U401 = L+/L−, U402 = R+/R−, so an adapter that shorts leg− loads only one amplifier per package (TI continuous-short rating, R4) |
| Protection | Rails first on / last off; PGOOD hardware kill of the amplifier enable; fault-latch set path blanked ≈ 60 ms after PGOOD good without clearing the latch; latch built from 2N7002 N-MOSFETs only; energy bounded at the headphone terminals; back-powering bounded and removed by firmware within 10 ms | R4-A H3; R4-B M1, M2; R4-C H6. The PNP transistor is no longer a JLCPCB Basic part, so the latch uses Basic 2N7002 (C8545) only |
| Input ferrite | Murata BLM18PG121SN1D (C14709, Basic, 2 A, 50 mΩ) in the 5V_ANA path (≤ 0.50 A); Sunlord GZ1608D601TF (C1002) only on the DAC DVCC branch (≈ 14 mA) | GZ1608D601TF is a 200 mA / 450 mΩ part (C1002; C1015 is GZ2012D101TF), so it cannot carry the power path (R4-A H1) |
| DC offset requirement | ≤ ±10 mV unbalanced, ≤ ±20 mV balanced; 100 % offset screening in the automated test | Worst case 7.9 / 15.9 mV with datasheet maxima; 20 mV into 16 Ω balanced = 25 µW DC (R4-A H2; R4-C H5) |
| BOM | One part per line; every line has an LCSC number and a JLCPCB class verified on the JLCPCB page (X202: not shown, assumed Extended) | R4-A M1; R4-C H4 |
| Firmware | Our own USB Audio Class 2.0 firmware on the CH32V307 high-speed port, built on WCH’s USBHS device examples (TinyUSB only if proven reliable), and our own CPLD logic; two-stage proof of concept before layout (firmware gate, Section 5) | Owner decision 24 Sep 2026. WCH’s high-speed examples cover HID, CDC, mass storage and network, not audio; USBAudioToI2S (R7) stays a known-good comparison (R4-B S4; R4-C B3–B5, H7) |
| Kept | 3.5 mm unbalanced + 4.4 mm balanced jacks (both live); ≥ 384 kHz; single USB-C port, bus-powered; JLCPCB Economic PCBA; CI, AI review and measurement automation | Unchanged requirements |

# 2. Power tree and current budget

One USB-C port supplies the board. VBUS (5V_SYS) feeds four regulators directly: an always-on 3.3 V LDO for the MCU domain, a switched 3.3 V LDO for the audio-digital domain, the DAC’s analog LDO and the TPS2553 current-limited switch for the headphone rails. The MCU enables the audio domains after USB configuration. Currents are estimates unless marked as datasheet figures.

Figure 1. Power domains and enables (5V_SYS, 3V3M, 3V3D with the CPLD and the three oscillators, 1V3, 3V3A, 5V_ANA, ±3.78 V).

## 2.1 Power domains

| Domain | Regulator and parts | Enable | Loads |
| --- | --- | --- | --- |
| 5V_SYS | VBUS from J101 (no switch); U101 USBLC6-2SC6; C102 1 µF | Always | Inputs of U502, U504, U302 and U503 (5 × 1 µF on VBUS in total) |
| 3V3M | U502 ME6211C33M5G-N; C501, C502 1 µF | Always on | CH32V307 (C211 2.2 µF bulk), fault latch and its pull-ups, LEDs, PGOOD and FAULT pull-ups (R505, R512) |
| 3V3D | U504 ME6211C33M5G-N; C503, C504 1 µF; CE pull-down R508 100 kΩ | AUD_EN | AGRV2KQ32, oscillators X201 (100 MHz), X202 (24.576 MHz) and X203 (22.5792 MHz) with enable resistors R212, R213, DAC DVCC via FB301, 1.3 V LDO input, I²C pull-ups (R207, R208) |
| 1V3 | U303 ME6216A13M3G from 3V3D; C311, C312 1 µF; C310 2.2 µF at the DAC pin | Follows 3V3D | ES9018K2M external DVDD |
| 3V3A | U302 LP5907MFX-3.3 from 5V_SYS; C301, C302 1 µF; EN pull-down R303 100 kΩ | AVCC_EN | VCCA, AVCC_L and AVCC_R (via 2.2 Ω), VREF divider |
| 5V_ANA | U503 TPS2553DBVR; C505, C506 1 µF; EN pull-down R513 100 kΩ; RILIM R509 + R514 = 147 kΩ, R510 in parallel via Q501 for High mode; FB501 to the LM27762 VIN | ANA_EN, ILIM_HI | LM27762 (C507 10 µF at VIN) |
| ±3.78 V | U501 LM27762DSSR from 5V_ANA; C508 1 µF flying, C509 10 µF CP, C510 10 µF OUT+, C511 10 µF OUT−; EN+/EN− = RAIL_EN (R506 pull-down, R507 10 kΩ); PGOOD → R505 to 3V3M | RAIL_EN | 2 × OPA1622, 2 × OPA2197, LM393 |

## 2.2 Rail budget by USB state

Digital loads at 5 V (typical / maximum):

| Load | Typ / max | Basis |
| --- | --- | --- |
| MCU CH32V307 incl. USB HS PHY | 60 / 80 mA | Assumption: the datasheet electrical tables were not obtained — to confirm |
| LEDs, latch, pull-ups | 2 / 3 mA | Estimate |
| CPLD AGRV2KQ32 | 15 / 25 mA | Assumption |
| Oscillator X201, 100 MHz | 12 / 35 mA | YXC YSO110TR datasheet: up to 35 mA max for 55–100 MHz; typical assumed |
| Oscillator X202, 24.576 MHz | 3 / 5 mA | YXC YSO110TR datasheet: 5 mA max; typical assumed |
| Oscillator X203, 22.5792 MHz | 5 / 10 mA | SCTF datasheet: 10 mA max; typical assumed. Counted with X202 although only one runs (conservative) |
| DAC DVCC | 13.5 / 14.2 mA | Datasheet (R3) |
| DAC external DVDD 1.3 V | 62.5 / 70 mA | Datasheet: 50 mA measured at 80 MHz MCLK, scaled to 100 MHz (assumption) |
| AVCC_L + AVCC_R + VCCA + VREF divider | 17 / 17.5 mA | Datasheet and divider |

Current by USB state (5 V at the receptacle):

| State | Domains on | Current (typ / max) | Notes |
| --- | --- | --- | --- |
| Attached, not configured | 3V3M only (MCU, latch, LEDs) | 62 / 83 mA | ≤ 100 mA unit load; rests on the MCU current assumption (measure; mitigation: lower MCU clock before configuration) |
| Configured, idle | All domains, outputs muted | 225 / 302 mA | Digital 190 / 260 mA plus analog quiescent 35 / 42 mA |
| Playing, Default mode | All | ≤ 0.473 A worst case | 0.260 A digital + 0.213 A TPS2553 maximum Default limit (27 mA below 0.5 A); covers USB 2.0 500 mA hosts and phones that limit OTG to ≤ 500 mA |
| Playing, High mode | All | ≤ 0.758 A worst case | Only when CC advertises ≥ 1.5 A (0.260 A digital + TPS2553 High limit ≤ 0.498 A) |
| Suspend | 3V3M, MCU low-power | Target ≤ 2.5 mA | USB 2.0 suspend limit is a target, not guaranteed in v1 (open item) |

Quiescent current per rail (max): OPA1622 13.2 mA, OPA2197 4.8 mA, LM393 1 mA, plus 3.4 mA DC from the I/V outputs into the leg networks (positive rail). Analog current per load is in 3.6.2.

## 2.3 Supply headroom

LM27762 (R5): VOUT+ = 1.2 V × (R1 + R2)/R2 and VOUT− = −1.22 V × (R3 + R4)/R4, with R2 and R4 ≥ 50 kΩ; VFB+ 1.182–1.218 V, VFB− −1.202 to −1.238 V. The dividers give +3.780 V (3.723–3.837 V) and −3.782 V (−3.726 to −3.838 V). Supply chain at the design corner: VBUS 4.60 V at the receptacle (host minimum 4.75 V minus 150 mV cable and connector drop at ≤ 0.5 A — assumption); TPS2553 rDS(on) 135 mΩ max; ferrite 50 mΩ (60 mΩ used); traces 20 mΩ; charge-pump resistance 3.0 Ω (assumption: 2.5 Ω is specified only at 5.5 V). Worst clean load: 4.4 mm, 32 Ω, 80 mA per leg (average analog input 249 mA, negative-rail peak 179 mA).

| VBUS at the receptacle | LM27762 VIN | Negative-rail capability | Negative margin | Positive-rail capability | Positive margin |
| --- | --- | --- | --- | --- | --- |
| 4.60 V (design corner) | 4.546 V | −3.956 V | 0.17 V | 4.466 V | 0.69 V |
| 4.40 V (stress corner) | 4.346 V | −3.756 V | −26 mV at current peaks | 4.266 V | 0.49 V |

At the 4.40 V stress corner the negative rail is 26 mV short at current peaks (brief dropout); the 95 % PGOOD threshold (3.59 V) is not reached. The earlier headroom check covered only the positive LDO and its fallback is dropped.

## 2.4 Inrush

| Item | Value | Basis |
| --- | --- | --- |
| Capacitance on VBUS before any switch | 5 × 1 µF: C102, C501, C503, C301, C505 | Connector and the inputs of U502, U504, U302 and U503 |
| 3V3M capacitance charged at attach | 4.0 µF: 1 µF LDO output + 8 × 100 nF + 2.2 µF MCU bulk | Only 3V3M starts at attach; C211 reduced to 2.2 µF for this budget |
| Attach charge | ≈ 5 µF × 5 V + 4.0 µF × 3.3 V = 38 µC | USB-IF limit 50 µC; nominal values (DC-bias derating makes the real figure lower) |
| Later domain enables | Staggered by firmware; the headphone rails charge through the TPS2553 soft start and current limit | VBUS droop at each enable is measured at bring-up |

## 2.5 Power-up and power-down sequence

Pull-downs on AUD_EN, AVCC_EN, ANA_EN, RAIL_EN, AMP_EN and RESETB and the latch set at power-up hold everything off until firmware runs the sequence below. It meets the ES9018K2M recommended power-up (R3 p. 27): VCCA first; AVCC_L/AVCC_R and DVCC at the same time or later; external DVDD at the same time as DVCC or later; RESETB asserted until ≥ 1 ms after all supplies and MCLK are stable; later resets ≥ 10 ns. X201 and X202 start with 3V3D (X203 stays disabled), and RESETB is released only when both clocks are stable.

| # | Step | Hardware and timing | ESS p. 27 / basis |
| --- | --- | --- | --- |
| 1 | Headphone supply | After USB configuration: ANA_EN high → TPS2553 soft start → 5V_ANA | Rails first on |
| 2 | Headphone rails | +5 ms: FLT_CLR pulse, then RAIL_EN high → ±3.78 V → PGOOD good; hardware arming (blanking ends) ≈ 60 ms later | RAIL_EN only after 5V_ANA (EN must not exceed VIN) |
| 3 | DAC analog | AVCC_EN high → LP5907: VCCA and AVCC together; VREF settles (τ ≈ 77 ms) | VCCA first; AVCC at the same time or later |
| 4 | DAC digital | +5 ms: AUD_EN high → 3V3D (DVCC, CPLD, X201 and X202 start together; X202 is enabled by default through R212, X203 stays off through R213); the 1.3 V DVDD LDO is fed from 3V3D, so DVDD ≤ DVCC during the ramp | DVCC at the same time as VCCA or later; DVDD at the same time as DVCC or later |
| 5 | Reset and init | MCLK (X201) and the family clock (X202, start-up ≤ 3 ms) running and the CPLD PLL locked → RESETB released ≥ 1 ms after all supplies and both clocks are stable → I²C init (asynchronous mode, 32-bit I²S, mute, master trim from flash) | RESETB ≥ 1 ms after stable supplies and MCLK (here both clocks) |
| 6 | Outputs on | When PGOOD has been good ≥ 100 ms, ≥ 400 ms after step 3 and the latch is clear → AMP_EN high → soft unmute | Pop ≤ 5 mV target |
| 7 | Power-down / suspend | Reverse order: soft mute, AMP_EN low, DAC reset, AUD_EN and AVCC_EN low, then RAIL_EN and ANA_EN low | Rails last off |
| 8 | Unplug / brownout | VBUS analog watchdog at 4.30 V drops AMP_EN first (target < 5 µs); the PGOOD kill is the hardware backstop. The +3.78 V LDO loses regulation near VIN ≈ 3.8 V while 3V3M holds to ≈ 3.5 V; ordering at a fast VBUS collapse is not guaranteed → measured | Unplug pop ≤ 20 mV target |
| 9 | Domain interfaces | MCU outputs toward the CPLD stay low or high-impedance while 3V3D is off; I²C pull-ups sit on 3V3D | No back-powering of the audio-digital domain |

# 3. Block notes

Entries with a reference are datasheet requirements; all other values are starting values. Designators, values and LCSC numbers follow Parts List v0.3.

Figure 2. Signal chain: 100 MHz oscillator (DAC MCLK and CPLD system clock), enable-selected 24.576 or 22.5792 MHz family oscillator into the CPLD PLL, DAC in current mode, OPA2197 I/V converters and OPA1622 legs.

## 3.1 USB-C input and sensing

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| Connector | J101 SHOU HAN TYPE-C 16PIN 2MD(073) (C2765186, Extended): USB 2.0 data + 5 V, sink | No Basic 16-pin USB-C receptacle exists; only the 4.4 mm jack is through-hole |
| CC pull-downs | R101, R102 5.1 kΩ Rd to GND | USB-C sink |
| CC sensing | CC1, CC2 → R103, R104 10 kΩ → MCU ADC; the ADC does not load CC | Bands (R11): Default 0.25–0.61 V, 1.5 A 0.70–1.16 V, 3.0 A 1.31–2.04 V. A USB-A-to-C cable reads as Default |
| Power mode | High mode only when ≥ 1.5 A is advertised; otherwise Default mode | Many phones limit OTG to ≤ 500 mA and run in Default mode |
| VBUS sense | R105 100 kΩ / R106 47 kΩ → MCU ADC (5.0 V → 1.60 V); analog-watchdog interrupt at 4.30 V | Unplug / brownout early warning → AMP_EN low, target < 5 µs |
| ESD | U101 ST USBLC6-2SC6 on D+, D− and VBUS at the connector | — |
| VBUS capacitance | C102 1 µF at the connector; 5 × 1 µF on VBUS in total | Inrush in 2.4 |
| Shield | R107 1 MΩ and C101 4.7 nF from shield to GND | Bleed and RF path |
| USB data | D+/D− to the CH32V307 USB HS pins as a 90 Ω differential pair | Layout per WCH reference |

## 3.2 CH32V307RCT6 (USB MCU)

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| Device | U201, LQFP-64 10 × 10 mm, 144 MHz RISC-V, on 3V3M (always on) | R1; C2988083. LCSC stock 119: reserve before ordering |
| HSE crystal | Y201 YXC X50328MSB2GI, 8 MHz, 5.0 × 3.2 mm, ±10 ppm, CL 20 pF (C115962, Basic); C201, C202 33 pF C0G | Load capacitors 2 × (20 pF − ≈ 3.5 pF stray) ≈ 33 pF. Firmware sets the USB HS PLL reference to 4 MHz as HSE ÷ 2, so 8 MHz is confirmed. No Basic 3.2 × 2.5 mm 8 MHz crystal exists |
| Decoupling | C203–C210 100 nF, one per VDD/VDDA/VIO pin; C211 2.2 µF bulk | 2.2 µF keeps the attach charge ≈ 38 µC (2.4) |
| Reset | R201 10 kΩ NRST pull-up; C212 100 nF | — |
| Boot | R202 10 kΩ BOOT0 pull-down | — |
| USB HS | D+/D− as a 90 Ω differential pair | Layout per WCH reference |
| Debug | J201 2-wire pads for WCH-LinkE: 3V3, SWDIO, SWCLK, GND | Programming |
| Pin map | Set by our MCU–CPLD interface specification (Section 5, open item); new enables and sensing inputs on free pins (Section 4) | Frozen before the proof-of-concept board and the schematic |
| ADC inputs | CC1, CC2 (through 10 kΩ) and VBUS_SENSE (analog watchdog) | 3.1 |
| Outputs toward the CPLD | Low or high-impedance while 3V3D is off | No back-powering through the CPLD pins |
| I²C | SCL, SDA to the DAC; R207, R208 2.2 kΩ pull-ups to 3V3D | Pull-ups switch off with the DAC domain |

## 3.3 CPLD and clock

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| CPLD | U202 AGM AGRV2KQ32, 2K LUTs, QFN-32 4 × 4 mm (leadless), on 3V3D; our own logic (Section 5): FIFO, I²S output engine, PLL ×2 on the selected family oscillator with exact division to BCLK/LRCLK for all 8 rates, oscillator enables OSC48_EN/OSC44_EN, SPI registers | R2; C18208899, LCSC stock 980 |
| Decoupling | C213–C216 100 nF | — |
| Oscillator X201 | YXC OT3225100MJBA4SL, 100 MHz 3.3 V CMOS, 3.2 × 2.5 mm, ±10 ppm tolerance, ±20 ppm stability (C4353707, Extended), on 3V3D; C217 100 nF; enable pin tied high | DAC MCLK (never switched) and the CPLD system clock (SPI registers, input side, FIFO write side) |
| Oscillator X202 | YXC OT322524.576MJBA4SL, 24.576 MHz 3.3 V CMOS, 3.2 × 2.5 mm, ±10 ppm tolerance, ±20 ppm stability (C2831388), on 3V3D; C218 100 nF; R210 33 Ω into the shared node; OSC48_EN with R212 100 kΩ pull-up (on by default) | 48 kHz family. YXC YSO110TR datasheet: pin 1 tri-state (enable ≥ 70 % VDD or floating, disable ≤ 30 % VDD), duty 45–55 %, start-up ≤ 3 ms, 5 mA max. JLCPCB class not shown, assumed Extended; the page notes an assembly fixture may be needed (check in the quote) |
| Oscillator X203 | SCTF SX2M22.5792M20F30TNN, 22.5792 MHz 1.62–3.63 V CMOS, 2.5 × 2.0 mm, ±20 ppm tolerance, ±30 ppm stability (C5452663, Extended), on 3V3D; C219 100 nF; R211 33 Ω into the shared node; OSC44_EN with R213 100 kΩ pull-down (off by default) | 44.1 kHz family. SCTF datasheet: enable/disable on pin 1, 10 mA max; LCSC stock 1,426 (Aug 2026). Open item: confirm that the disabled output is high-impedance (fallback: a 2:1 logic multiplexer in front of the PLL input) |
| Clock routing | X201 → 33 Ω → DAC MCLK (direct, short) and X201 → R209 33 Ω → CPLD system-clock input; X202 → R210 and X203 → R211 into one node at the CPLD’s dedicated PLL clock input; R203–R206 33 Ω are the series resistors on MCLK (to the DAC), BCLK, LRCLK and SDATA | Both family oscillators and the shared node next to the CPLD PLL clock pin, short traces |
| Oscillator enables | The CPLD drives OSC48_EN (X202) and OSC44_EN (X203); exactly one family oscillator is enabled at a time; X202 is on by default (R212), X203 off (R213) | Same 3V3D domain as the oscillators, so no back-powering when 3V3D is off; oscillator selected before the PLL, as R4-C suggested |
| DAC MCLK limits | Normal (asynchronous) mode: MCLK > 192 × FSR (73.7 MHz at 384 kHz); up to 100 MHz allowed with DVCC 3.3 V and an external 1.3 V DVDD (R3 note 2) | The ±10 ppm oscillator sits at that limit, the usual ES9018K2M arrangement; open item: a stocked 80–98 MHz oscillator would add margin |
| DAC audio-interface timing | DATA_CLK (BCLK) duty cycle 45:55 to 55:45; pulse width high and low ≥ 4.5 ns; cycle ≥ 10 ns; DATA setup 4.1 ns and hold 2 ns to the DATA_CLK rising edge | R3 p. 30. One 100 MHz clock for both families gave 40:60 at 352.8/384 kHz, hence the family oscillators |
| Bit clocks | CPLD PLL ×2 → 49.152 MHz (X202) or 45.1584 MHz (X203) → BCLK = PLL ÷ 2/4/8/16 → 384/192/96/48 kHz or 352.8/176.4/88.2/44.1 kHz; LRCLK = BCLK/64 | All 8 rates are exact integer ratios with a 50:50 bit clock (table below); the reference project uses the same PLL arrangement on this CPLD |
| Family change | DAC soft mute → I²S output stopped (held low) → PLL held in reset → enables switched → wait ≥ 10 ms (oscillator start-up) → PLL released, wait for lock → output restarts → DAC re-locks (lock_status, register 64) → unmute. MCLK is never touched | No clock multiplexer in the CPLD fabric; answers R4-C B4; scope check for runt pulses in the proof of concept |
| Clock path | No PLL in the DAC MCLK path; the CPLD PLL only makes the bit clock (both families), whose jitter the DAC’s DPLL rejects | Answers the PLL-jitter concern (R4-B M3) |
| JTAG | J202 pads: 3V3, TCK, TMS, TDI, TDO, GND | CPLD programming |
| Timing | CPLD timing at 100 MHz (system clock) and at the PLL output (49.152 MHz), including the FIFO crossing between the two clock domains | Part of the proof of concept (Section 5) |

Bit clock by sample rate (BCLK = 64 × fs = PLL output ÷ 2/4/8/16; every rate is an exact integer ratio; the DAC requires a duty cycle of 45:55 to 55:45):

| Sample rate | BCLK | Source | Worst duty | Within 45:55? |
| --- | --- | --- | --- | --- |
| 48 kHz | 3.072 MHz | 49.152 MHz (24.576 MHz, CPLD PLL ×2) ÷ 16 | 50.0 % | Yes, exact |
| 96 kHz | 6.144 MHz | 49.152 MHz ÷ 8 | 50.0 % | Yes, exact |
| 192 kHz | 12.288 MHz | 49.152 MHz ÷ 4 | 50.0 % | Yes, exact |
| 384 kHz | 24.576 MHz | 49.152 MHz ÷ 2 | 50.0 % | Yes, exact |
| 44.1 kHz | 2.8224 MHz | 45.1584 MHz (22.5792 MHz, CPLD PLL ×2) ÷ 16 | 50.0 % | Yes, exact |
| 88.2 kHz | 5.6448 MHz | 45.1584 MHz ÷ 8 | 50.0 % | Yes, exact |
| 176.4 kHz | 11.2896 MHz | 45.1584 MHz ÷ 4 | 50.0 % | Yes, exact |
| 352.8 kHz | 22.5792 MHz | 45.1584 MHz ÷ 2 | 50.0 % | Yes, exact |

## 3.4 DAC and its supplies (ES9018K2M)

| Pin / function | Connection | Requirement (R3) / note |
| --- | --- | --- |
| Mode | U301 ES9018K2M (C2688377, LCSC stock 724) in normal (asynchronous) mode: DPLL / jitter eliminator | Datasheet DNR/THD figures are specified in this mode (test condition MCLK 27 MHz, asynchronous) |
| DVCC | 3V3D through FB301 Sunlord GZ1608D601TF (C1002: 600 Ω at 100 MHz, 200 mA, 0.45 Ω) + 100 nF | +1.8 V or +3.3 V ±5 %; 13–14.2 mA |
| VCCA | 3V3A (LP5907) + 100 nF | +3.3 V ±5 %; 0.8 mA |
| AVCC_L, AVCC_R | 3V3A via R301, R302 2.2 Ω on separate branches (≈ 18 mV drop at 8 mA); C303, C304 2.2 µF + 100 nF each | +3.3 V ±5 %; 8 mA |
| DVDD | External 1.3 V from U303 Microne ME6216A13M3G (C236662: ±1 %, 300 mA, ceramic-stable, 6 µA Iq) fed from 3V3D; C311 1 µF in, C312 1 µF out; C310 2.2 µF + 100 nF at the pin | External +1.3 V ±5 % must be used for PCM when MCLK > 50 MHz or FSR > 192 kHz; ≥ 1 µF effective (2.2 µF X5R recommended). DVDD ≤ DVCC during ramps. LCSC stock ≈ 410: reserve |
| LP5907 | U302 LP5907MFX-3.3 (C80670) from 5V_SYS; EN = AVCC_EN with R303 100 kΩ pull-down; C301, C302 1 µF | Total output capacitance 5.7 µF (C302 + C303/C304 + 3 × 100 nF) ≤ 10 µF (R6) |
| MCLK | 100 MHz from X201 through 33 Ω, direct; never switches (both rate families) | > 192 × FSR; ≤ 100 MHz with DVCC 3.3 V and external DVDD (note 2) |
| BCLK, LRCLK, SDATA | From the CPLD through 33 Ω; 3.3 V logic; held low while the rate family changes | 32-bit I²S; DATA_CLK duty cycle 45:55 to 55:45, pulse width ≥ 4.5 ns, setup 4.1 ns, hold 2 ns (R3 p. 30; 3.3) |
| I²C | SCL, SDA to the MCU; pull-ups on 3V3D; ADDR strap R306 10 kΩ | Address 0x90 |
| RESETB | R305 100 kΩ pull-down; MCU GPIO | Held low until ≥ 1 ms after all supplies and both clocks are stable |
| Outputs | DACL, DACLB, DACR, DACRB in differential current mode, each only to its I/V inverting input (3.5.1) | ESS application circuit (R3 p. 27) |
| Output model | Each pin is a voltage source swinging 3.05 Vpp (0.924 × AVCC) around 1.65 V behind 806 Ω ±11 %; unloaded differential full scale 2.156 Vrms | R3 p. 31, note 1 |
| Master trim | 32-bit master trim sets the calibrated level (3.5.4) | Value stored in MCU flash |
| Lock status | DPLL lock on GPIO / register 64 (lock_status), checked before unmute after a rate or family change | Section 5 |
| Exposed pad | DGND | — |

## 3.5 Output stage: I/V converters and legs (2 × OPA2197 + 2 × OPA1622)

Each DAC output pin drives its own I/V converter: one OPA2197 amplifier holds the pin at VREF and turns its current into a voltage. Each leg is one OPA1622 amplifier wired as a 4-resistor difference amplifier fed by two I/V outputs: leg+ = I/V(DAC−) − I/V(DAC+), leg− the swap. U403 serves DACL and DACLB, U404 serves DACR and DACRB; U401 carries L+ and L−, U402 carries R+ and R−.

### 3.5.1 I/V converters (U403, U404)

| Item | Value | Basis / notes |
| --- | --- | --- |
| Converters | U403, U404 TI OPA2197IDR (C139363) on ±3.78 V, one amplifier per DAC pin | Vos ±0.25 mV max, 5.5 nV/√Hz, rail-to-rail I/O, 1 mA per channel; 10 mA input-current limit |
| Inputs | Inverting input = the DAC pin (its only load, short trace); non-inverting input = VREF | ESS application circuit (R3 p. 27) |
| Feedback resistor | Rf = R423–R426 1 kΩ ∥ R434–R437 5.1 kΩ = 836 Ω, 1 % | Basic values. Per leg L = (Rf/Rs) × 2.156 Vrms = 2.24 Vrms nominal before calibration |
| Feedback capacitor | R427–R430 10 Ω in series with C417–C420 ∥ C427–C430 (two 330 pF C0G = 660 pF per converter) | ≈ 288 kHz with 836 Ω. No Basic C0G above 330 pF; X7R is unsuitable in the signal path |
| VREF | R431 15 kΩ over R432 15 kΩ + R433 1 kΩ from 3V3A → 1.703 V; one shared node: C421 10 µF at the divider, C422 100 nF between the two OPA2197s, short traces to all four non-inverting inputs (τ ≈ 77 ms) | At the DAC zero-DC-current point (≈ 1.702 V). VREF errors are mostly common mode; the differential part is 1.17 mV per leg (3.5.6) |
| Decoupling | C423–C426 100 nF on V+ and V− of U403 and U404 | — |
| Output swing | At the calibrated level ±1.24 V peak around VREF (0.47 … 2.94 V); untrimmed highest-gain unit ±1.83 V (−0.13 … 3.53 V) | Computed from the level spread (3.5.4); inside the ±3.78 V rails |
| Back-powering | With the rails off and the DAC and VREF powered: ≤ 1.7 mA per DAC pin (806 Ω − 11 %) and ≤ 0.16 mA from VREF into the unpowered inputs | Below the OPA2197 10 mA input-current limit; firmware removes the source within 10 ms (3.7) |
| Stability | DAC pin capacitance on the inverting inputs with the 836 Ω ∥ (10 Ω + 660 pF) feedback | SPICE before layout (open item) |

### 3.5.2 Output legs (U401, U402)

| Item | Value | Basis / notes |
| --- | --- | --- |
| Amplifiers | U401, U402 TI OPA1622IDRCR (C2876559) on ±3.78 V; one amplifier per leg; thermal pad to V− copper | Isc +145/−130 mA typ (R4) |
| Resistors | R401–R416 YAGEO RT0603BRD072KL 2.00 kΩ 0.1 % 25 ppm thin film (C728588, Extended); gain 1 | Worst 4-resistor mismatch 0.2 % (common-mode leakage term in 3.5.6) |
| Filter | C401–C408 330 pF C0G across each feedback and ground-leg resistor | ≈ 241 kHz with 2 kΩ; 330 pF is the largest Basic C0G |
| Input mapping | leg+ = I/V(DAC−) − I/V(DAC+); leg− the swap | Each I/V inverts, so leg+ is in phase with DAC+ − DAC− |
| Input common mode | VREF ≈ 1.70 V → non-inverting node ≈ 0.85 V DC | Inside the OPA1622 range (V−)+1.5 V to (V+)−1 V, i.e. −2.28 V to +2.78 V on ±3.78 V |
| Output link | R417–R420 0 Ω per leg | Fit 0.47 Ω only if the capacitive-load test needs it |
| Enable | AMP_EN to both EN pins: R421 100 kΩ pull-down, R618 10 kΩ from the MCU; pulled low by Q608 (latch set) and Q609 (PGOOD not good) | EN threshold ≈ 0.82 V referred to the OPA1622 GND pin; the kill also keeps EN below the input limit whenever the rails are off |
| Supplies | C409–C412 100 nF and C413–C416 10 µF on V+ and V− | — |
| Swing model | Vpk ≤ (¦rail¦min − 0.9 V)/(1 + 8 Ω/Rleg); clean envelope ≤ 80 mA peak per leg | R4 Fig. 38/39, typical, 25 °C: rail − 0.8…0.9 V at light load, falling ≈ 8 Ω × Iout; current limit ≈ 110–120 mA at 85 °C die. Extrapolated to ±3.78 V — bench-verify |
| Jack ESD | D701–D704 GOODWORK LESD5D5.0CT1G (C41399463): Vrwm 5 V, 12 pF, one per jack leg | Legs swing ≤ ±3.8 V |
| Output impedance | < 0.1 Ω plus jack contacts (legs driven directly) | Target ≤ 0.5 Ω on both jacks (≤ 1.0 Ω balanced if the 0.47 Ω option is fitted) |
| Ground layout | Unbalanced crosstalk −75 dB at 32 Ω needs ≤ 5.7 mΩ shared ground | Dedicated sleeve return, ≥ 4 vias, measured at the jack |

### 3.5.3 Jack pinout

| Signal | 4.4 mm 5-pole (J701, G-Switch GT-3321667P-01, through-hole) | 3.5 mm (J702, HRO PJ-332A-6A, TRRS body used with TRS plugs) |
| --- | --- | --- |
| L+ | Tip | Tip |
| L− | Ring 1 | Not connected (leg+ only) |
| R+ | Ring 2 | Ring 1 |
| R− | Ring 3 | Not connected (leg+ only) |
| GND | Sleeve | Sleeve and ring 2 (tie both to GND) |

Both jacks are live; use one headphone at a time. JLCPCB marks J702 as assembly difficulty “high”.

### 3.5.4 Level and calibration

| Parameter | Value | Basis |
| --- | --- | --- |
| DAC source | 3.05 Vpp around 1.65 V behind 806 Ω ±11 % per pin; unloaded differential 2.156 Vrms | R3 p. 31, note 1 |
| Untrimmed level per leg | L = (Rf/Rs) × 2.156 Vrms = 2.236 Vrms nominal; unit spread 1.955–2.589 Vrms | Includes Rs ±11 %, Rf 1 % and AVCC (LP5907) ±2 % |
| Calibrated full scale | 1.75 Vrms per leg (2.475 V peak): 3.5 mm 1.75 Vrms, 4.4 mm 3.5 Vrms | Master trim −0.96 dB (lowest-gain unit) to −3.40 dB (highest), nominal −2.13 dB |
| Before calibration | Firmware applies −3.5 dB → ≤ 1.73 Vrms on any unit | Safe default for a new board |
| Channel trim | ±0.05 dB per channel by MCU digital scaling | Stored with the master trim |
| Calibration | Automated end-of-line test (QA403): master trim to 1.75 Vrms ± 0.1 dB and per-channel fine trim, written to MCU flash | 3.8, Section 5 |

### 3.5.5 Output power (High and Default mode)

Both channels driven, 1 kHz, at the calibrated level; rated at 1 % THD+N or calibrated 0 dBFS, whichever comes first. The limiting factor is in brackets. Estimates: the OPA1622 swing at ±3.78 V is extrapolated, so the table is bench-verified.

| Load | 3.5 mm unbalanced (both modes) | 4.4 mm balanced, High mode | 4.4 mm balanced, Default mode |
| --- | --- | --- | --- |
| 16 Ω | 51 mW (80 mA envelope) | — | — |
| 32 Ω | 80 mW (swing) | 102 mW (80 mA envelope) | ≈ 31 mW (TPS2553 Default limit) |
| 60 Ω | 51 mW (full scale) | 166 mW (swing) | ≈ 59 mW (TPS2553 Default limit) |
| 120 Ω | — | 102 mW (full scale) | 102 mW (full scale) |
| 300 Ω | 10 mW (full scale) | 41 mW (full scale) | 41 mW (full scale) |
| 600 Ω | — | 20 mW (full scale) | 20 mW (full scale) |

Default mode limits the analog current to the TPS2553 minimum of 0.157 A. Proposed power targets (both channels, same rating method): High mode — unbalanced ≥ 70 mW at 32 Ω; balanced ≥ 150 mW at 60 Ω, ≥ 90 mW at 120 Ω, ≥ 36 mW at 300 Ω. Default mode — balanced ≥ 50 mW at 60 Ω; others as High mode; 32 Ω balanced is not rated in Default mode.

SPL: HIFIMAN Susvara (60 Ω, 83 dB/mW) 166 mW → ≈ 105 dB SPL in High mode, 59 mW → ≈ 101 dB SPL in Default mode; AKG K1000 (120 Ω, 74 dB/mW) 102 mW → ≈ 94 dB SPL.

### 3.5.6 Offset, noise and frequency response

| Offset contribution per leg (25 °C, datasheet maxima) | Value |
| --- | --- |
| Common-mode leakage: VREF 1.703 V × 0.2 % (0.1 % resistors, gain 1) | 3.40 mV |
| OPA1622 Vos 0.5 mV × 2 | 1.00 mV |
| OPA1622 Ios × 1 kΩ × 2 | 0.10 mV |
| OPA2197 Vos 0.25 mV × noise gain up to 2.17, two amplifiers | 1.08 mV |
| DAC pin-to-pin DC mismatch ≤ 1 mV (assumption, measure) × Rf/Rs | 1.18 mV |
| VREF-to-null error (divider tolerance + assumed ±20 mV null-point uncertainty) × 3 % pair-gain mismatch | 1.17 mV |
| Unbalanced total; requirement ≤ ±10 mV | ≤ 7.9 mV worst (≈ 4.1 mV RSS); margin 2.1 mV |
| Balanced total; requirement ≤ ±20 mV | ≤ 15.9 mV worst (≈ 6.4 mV RSS); margin 4.1 mV |

Every board is measured by the automated test (100 % offset screening); the DAC-mismatch assumption stays open until measured.

| Parameter | Estimate | Basis |
| --- | --- | --- |
| I/V output noise | ≈ 12.4 nV/√Hz | OPA2197 5.5 nV/√Hz × 2.04 plus Rf and DAC-source thermal noise |
| Leg noise | ≈ 20.1 nV/√Hz → 2.84 µV (20 kHz) | Two I/V outputs and the leg network |
| Dynamic range, analog only | ≈ 115.8 dB unweighted, ≈ 117.7 dB(A) | Estimate |
| Dynamic range, system | ≈ 117.0 dB(A) unbalanced, ≈ 120.0 dB(A) balanced | With the DAC (127 dB(A) DNR, referred to the untrimmed level). Target ≥ 115 dB(A), minimum ≥ 112 dB(A) |
| I/V pole | 288 kHz | 836 Ω with 660 pF |
| Leg pole | 241 kHz | 2 kΩ with 330 pF |
| Frequency response | −0.05 dB at 20 kHz, −0.20 dB at 40 kHz | Both poles |

## 3.6 Power (ME6211 × 2, TPS2553, LM27762)

| Item | Value | Basis / notes |
| --- | --- | --- |
| 3V3M LDO | U502 Microne ME6211C33M5G-N (C82942), always on; C501, C502 1 µF | ±2 %, 100 mV dropout at 100 mA |
| 3V3D LDO | U504 ME6211C33M5G-N, CE = AUD_EN with R508 100 kΩ pull-down; C503, C504 1 µF | Same part type as U502 |
| 1.3 V LDO | U303 ME6216A13M3G from 3V3D | 3.4 |
| Current-limited switch | U503 TI TPS2553DBVR (C55266): EN = ANA_EN with R513 100 kΩ pull-down; FAULT → MCU with R512 100 kΩ pull-up to 3V3M; C505, C506 1 µF | rDS(on) 85 mΩ typ / 135 mΩ max; built-in soft start; FAULT open drain with 7.5 ms deglitch; constant-current limiting (R9) |
| Current-limit resistors | RILIM = R509 100 kΩ + R514 47 kΩ = 147 kΩ (Default); R510 100 kΩ switched in parallel by Q501 2N7002 (gate = ILIM_HI, R511 100 kΩ pull-down) → 59.5 kΩ (High) | R9: IOS max = 22980/R^0.94, nom = 23950/R^0.977, min = 25230/R^1.016 (mA, R in kΩ, 15–232 kΩ); two-level limit per datasheet Figure 24. Keep the RILIM trace short |
| Ferrite | FB501 Murata BLM18PG121SN1D (C14709, Basic): 120 Ω at 100 MHz, 2 A, 50 mΩ; 5V_ANA → LM27762 VIN | Drop ≈ 15–25 mV |
| Rail generator | U501 TI LM27762DSSR (C473398): C507 10 µF VIN, C508 1 µF flying, C509 10 µF CP, C510 10 µF OUT+, C511 10 µF OUT− | R5: charge-pump output resistance 2.5 Ω typ at VIN 5.5 V, 100 mA; LDO dropout 30 mV (negative) and 45 mV (positive) at 100 mA; input current limit 500 mA typ; UVLO 2.4 V falling / 2.6 V rising |
| FB+ divider | R501 100 kΩ + R515 100 kΩ + R516 15 kΩ = 215 kΩ over R502 100 kΩ → +3.780 V (3.723–3.837 V) | 1.2 V × 315/100; R2 ≥ 50 kΩ |
| FB− divider | R503 100 kΩ + R517 100 kΩ + R518 10 kΩ = 210 kΩ over R504 100 kΩ → −3.782 V (−3.726 to −3.838 V) | −1.22 V × 310/100; R4 ≥ 50 kΩ |
| Enable | EN+ and EN− = RAIL_EN: R506 100 kΩ pull-down, R507 10 kΩ from the MCU; pulled low by Q607 when the latch is set | EN high ≥ 1.2 V; EN must not exceed VIN, so RAIL_EN goes high only after 5V_ANA is up |
| PGOOD | Open drain, active low (low = both outputs ≥ 95 % of target); R505 100 kΩ pull-up to 3V3M → RAILS_OK_N to the MCU, Q609 gate and the blanking RC | In shutdown each output has a 50 kΩ pull-down (not relied on for discharge) |
| Rail capacitance | ≈ 30.6 µF per rail | Energy after a trip ≤ 0.22 mJ (3.7.1) |

### 3.6.1 Current limits

| Mode | RILIM | Limit min / nom / max | Use |
| --- | --- | --- | --- |
| Default | 147 kΩ | 157 / 183 / 213 mA | USB 2.0 hosts, USB-A ports and phones; total ≤ 0.473 A worst case |
| High | 59.5 kΩ | 393 / 442 / 498 mA | Only when CC advertises ≥ 1.5 A; total ≤ 0.758 A |

Limits include the 1 % resistor tolerance.

### 3.6.2 Analog supply current

| Jack and load | Analog current at the clean output (max quiescent) | Within the Default limit (157 mA min)? |
| --- | --- | --- |
| 3.5 mm 16 Ω | 146 mA | Yes |
| 3.5 mm 32 Ω | 134 mA | Yes |
| 3.5 mm 60 Ω | 96 mA | Yes |
| 3.5 mm 300 Ω | 53 mA | Yes |
| 4.4 mm 32 Ω | 249 mA | No: Default mode ≈ 31 mW |
| 4.4 mm 60 Ω | 235 mA | No: Default mode ≈ 59 mW |
| 4.4 mm 120 Ω | 149 mA | Yes |
| 4.4 mm 300 Ω | 85 mA | Yes |
| 4.4 mm 600 Ω | 64 mA | Yes |

## 3.7 Protection

Protection works without firmware: a DC window detector sets a fault latch, and the latch and the LM27762 PGOOD pull the amplifier and rail enables low through N-MOSFET kill switches. Firmware only clears the latch and applies the recovery policy (Section 5). All transistors are JSCJ 2N7002 (C8545, Basic).

| Item | Value | Basis / notes |
| --- | --- | --- |
| DC sensing | Each leg → R601–R604 100 kΩ + R619–R622 47 kΩ = 147 kΩ with C601–C604 1 µF (τ = 147 ms) → D601–D604 BAV99 into the MAX and MIN nodes (R605, R606 1 MΩ loads) | One filter per leg |
| Comparator | U601 onsemi LM393DR2G (C7955, Basic) on ±3.78 V; thresholds R607, R608 100 kΩ over R609, R610 12 kΩ from the rails → ±0.405 V | Trip at ≈ ±0.75 V DC on a leg |
| Node F | LM393 open-collector outputs (wired-OR) → R611 10 kΩ → node F; R612 47 kΩ pull-up to 3V3M; D605 1N4148WS clamp to GND | F is 3.3 V normally and ≈ −0.7 V on a fault (clamp current ≤ 0.31 mA); the 3.3 V domain never sees the −3.8 V comparator output |
| Set path | Q601 inverter (gate = F) drives node S (R613 100 kΩ pull-up): S high = set request; Q602 (gate = S) sets the latch by pulling node A low | — |
| Blanking | Q603 holds S low while its gate is high; gate = PGOOD through R617 1 MΩ with C606 100 nF → blanking ≈ 60 ms after PGOOD goes good | Blocks only the set request; it cannot clear an already-set latch (a clamp on the latch node would clear it and cause a hiccup) |
| Latch | Q604 and Q605 cross-coupled (drain A → gate B, drain B → gate A); R614, R615 100 kΩ pull-ups to 3V3M; FLT = node B (high = fault); C605 100 nF from 3V3M to node B → powers up latched | — |
| Clear | Q606 (gate = FLT_CLR, R616 100 kΩ pull-down) pulls node B low | A clear does not stick while a set request is present; the MCU reads FLT directly |
| Kill paths | Q607 pulls RAIL_EN low when the latch is set; Q608 pulls AMP_EN low when the latch is set; Q609 pulls AMP_EN low whenever PGOOD is high (not good, disabled or unpowered) | Hardware; no firmware needed |
| Detection time | ≈ 70 ms for 2 V DC; ≈ 46 ms for a leg stuck at the rail | Requirement: every leg ≤ 0.1 V into 32 Ω within 100 ms of the fault onset |
| No false trip | Full-scale 20 Hz gives 0.13 V after the filter, 5 Hz 0.52 V, 4 Hz 0.65 V | No false trip for full-scale content down to ≈ 4 Hz |

### 3.7.1 Energy at the headphone terminals and back-powering

| Item | Value | Basis / notes |
| --- | --- | --- |
| Before the trip | ≤ 16.5 mJ at 16 Ω (rail-stuck leg for 46 ms); 11.4 mJ at 32 Ω; 6.1 mJ at 60 Ω; 1.2 mJ at 300 Ω | Computed at the headphone terminals |
| After the trip | Rails disabled; ≈ 30.6 µF per rail → ≤ 0.22 mJ, the most a faulty output stage can deliver | The load discharges it; the LM27762 50 kΩ discharge is not relied on. OPA1622 shutdown is not treated as isolation (TI: “do not consider the amplifier output to be high-impedance”) |
| Back-powering | ≤ 1.7 mA per DAC pin and ≤ 0.16 mA from VREF into the unpowered I/V inputs | Below the OPA2197 10 mA input-current limit; firmware removes it (LP5907 off, DAC reset) within 10 ms of a fault; the normal sequence never has the DAC powered while the rails are off |
| Test criterion | Every leg ≤ 0.1 V into 32 Ω within 100 ms of the fault onset; ≤ 1 mJ after the trip | Digital DC test signal (3.8) |

### 3.7.2 Thermal and overload

| Case | Estimate | Basis / notes |
| --- | --- | --- |
| Analog input power | ≤ 1.06 W (Default), ≤ 2.49 W (High) | Bounded by the TPS2553 limit |
| Board worst case | ≈ 3.8 W | High mode, overload |
| 4.4-to-3.5 mm adapter (leg− shorted) | ≈ 0.35 W in the shorted amplifier + ≈ 0.1 W in the other → ≈ 0.45 W per package, +21 °C | θJA 47.6 °C/W; one amplifier per package, inside TI’s continuous-short rating |
| 4.4 mm plug shorted leg+ to leg− | ≈ 0.55 W per package (both amplifiers of one package) | Outside TI’s one-amplifier continuous-short rating → bench survival test |
| Overload response | Current limit → rails droop → PGOOD → AMP_EN low in hardware → firmware soft-mutes, lowers the volume ceiling 3 dB (down to −12 dB) and re-enables after 1 s; ceiling kept until replug | TPS2553 FAULT (7.5 ms deglitch) is also read |

## 3.8 Test and DFT

| Item | Provision | Notes |
| --- | --- | --- |
| Test pads | TP701–TP726 (26 copper pads): rails 5V_SYS, 5V_ANA, 3V3M, 3V3D, 3V3A, 1V3, ±3.8 V; clocks; I²S; EN/PG/FLT/FAULT nodes; 4 GND | — |
| Debug | J201 CH32V307 2-wire pads (WCH-LinkE); J202 CPLD JTAG pads | Section 5 |
| I²S header | J703 1 × 6, 2.54 mm, DNF: MCLK, BCLK, LRCLK, SDATA, GND, 3V3 reference | Firmware test mode tristates the CPLD I²S outputs; the QA403 digital output is 3.3 V-class: use series resistors, power the board first |
| Indicators | D705 KT-0805G green (status), D706 KT-0603R red (fault); R701, R702 2.2 kΩ (≈ 0.5–0.7 mA) from 3.3 V GPIO | No Basic 0603 green LED exists |
| Assembly | JLCPCB Economic PCBA, one side; all SMT on top except the 4.4 mm jack (7 through-hole joints); 3 fiducials, 4 × M3 holes; 4 layers, ≤ 80 × 60 mm | 8 leadless parts per board for X-ray: CPLD, DAC, 2 × OPA1622, LM27762, 3 oscillators |

End-of-line and bring-up tests:

| # | Test | Method and pass criterion |
| --- | --- | --- |
| 1 | End-of-line calibration | QA403 + scripts: full-scale level per leg → master trim to 1.75 Vrms ± 0.1 dB and per-channel fine trim, written to flash; DC offset of every leg (100 %); THD+N; dynamic range |
| 2 | Protection | DC fault injected by a digital DC test signal: every leg ≤ 0.1 V into 32 Ω within 100 ms of the fault onset and ≤ 1 mJ delivered after the trip |
| 3 | Overload (30 min each, High mode, maximum volume) | 4.4 mm 16 Ω; 4.4-to-3.5 mm adapter; 3.5 mm 16 Ω; 60 Ω balanced full scale — all parts within ratings, board recovers |
| 4 | USB | Current in every state (USB power meter); attach inrush charge; VBUS droop at each domain enable; Default and High mode on a Type-C 1.5 A source and on a USB-A port |
| 5 | Pop | ≤ 5 mV peak for controlled transitions (power-on after configuration, suspend/resume, rate change, fault recovery); ≤ 20 mV peak at unplug/brownout (target); 32 Ω and 300 Ω |
| 6 | Jitter | J-test for both families (exact bit clocks from the CPLD PLL), including 352.8 and 384 kHz; BCLK duty cycle measured at every rate; PLL relock and scope check for runt pulses on BCLK/LRCLK at every family change |
| 7 | Rails and power | Negative-rail margin at VBUS 4.60 V; output power table on the bench |
| 8 | Stability | Each leg with the 0 Ω links (fit 0.47 Ω only if the capacitive-load test needs it); each I/V output checked for oscillation |
| 9 | VREF null point | At idle the DC at each I/V output ≈ VREF, confirming the DAC zero-current point |

# 4. Signal plan

The MCU–CPLD interface is ours to define (Section 5): its pins are set by the MCU–CPLD interface specification, and the new control and sensing signals use free pins (“to assign”). The pin map is frozen before the proof-of-concept board and the schematic.

| Signal | From → to | Type / level | Pin / notes |
| --- | --- | --- | --- |
| USB D+, D− | J101 ↔ CH32V307 USB HS | USB 2.0 HS | Dedicated USB HS pins; 90 Ω pair |
| MCU ↔ CPLD audio interface | CH32V307 ↔ AGRV2KQ32: for example samples by DMA over I²S or SPI, FIFO level and registers over SPI, LRCLK back to a timer-capable MCU pin | 3.3 V LVCMOS | Per the MCU–CPLD interface specification (open item); MCU side low or Hi-Z while 3V3D is off |
| MCLK | X201 → 33 Ω → DAC; X201 → R209 33 Ω → CPLD system-clock input | 100 MHz CMOS | Never switches; X201 enable tied high; short route to the DAC |
| Family clock (shared node) | X202 → R210 and X203 → R211 → one node → CPLD dedicated PLL clock input | 24.576 or 22.5792 MHz CMOS | Only the enabled oscillator drives the node; both oscillators next to the CPLD PLL pin |
| BCLK, LRCLK, SDATA | CPLD → DAC through 33 Ω; J703 header | 3.3 V output | To assign (CPLD I/O); held low during rate-family changes; tristated in test mode |
| OSC48_EN | CPLD → X202 enable (pin 1) | 3.3 V output; R212 pull-up | To assign (CPLD I/O); X202 on by default |
| OSC44_EN | CPLD → X203 enable (pin 1) | 3.3 V output; R213 pull-down | To assign (CPLD I/O); X203 off by default |
| I²C SCL, SDA | CH32V307 ↔ ES9018K2M | Open drain; 2.2 kΩ to 3V3D | To assign; DAC address 0x90 |
| DAC RESETB | CH32V307 → DAC | 3.3 V output; R305 pull-down | To assign |
| CC1_ADC, CC2_ADC | CC1, CC2 → 10 kΩ → MCU ADC | Analog | To assign (ADC pins) |
| VBUS_SENSE | R105/R106 divider → MCU ADC | Analog; watchdog 4.30 V | To assign (ADC pin with analog watchdog) |
| AUD_EN | MCU → U504 CE | 3.3 V output; R508 pull-down | To assign |
| AVCC_EN | MCU → U302 EN | 3.3 V output; R303 pull-down | To assign |
| ANA_EN | MCU → U503 EN | 3.3 V output; R513 pull-down | To assign |
| ILIM_HI | MCU → Q501 gate | 3.3 V output; R511 pull-down | To assign; only with ≥ 1.5 A advertised |
| ANA_FAULT | U503 FAULT → MCU | Open drain; R512 pull-up to 3V3M | To assign |
| RAIL_EN | MCU → R507 10 kΩ → LM27762 EN+/EN−; Q607 kill | 3.3 V; R506 pull-down | To assign |
| RAILS_OK_N | LM27762 PGOOD → MCU, Q609 gate, blanking RC | Open drain; R505 pull-up to 3V3M | To assign |
| AMP_EN | MCU → R618 10 kΩ → both OPA1622 EN; Q608, Q609 kills | 3.3 V; R421 pull-down | To assign |
| FLT_CLR | MCU → Q606 gate | 3.3 V output; R616 pull-down | To assign |
| FLT | Latch node B → MCU | 3.3 V input | To assign |
| LED × 2 | MCU → D705, D706 through 2.2 kΩ | 3.3 V output | To assign |
| Debug SWDIO, SWCLK | J201 ↔ CH32V307 | 2-wire debug | Dedicated pins |
| CPLD JTAG TCK, TMS, TDI, TDO | J202 ↔ AGRV2KQ32 | JTAG | Dedicated pins |

# 5. Firmware requirements

The USB firmware and the CPLD logic are our own (owner decision, 24 Sep 2026). The firmware is our own USB Audio Class 2.0 implementation on the CH32V307 high-speed USB port, built on WCH’s register-level USBHS device examples; the chip’s EVT package has high-speed device examples for HID, CDC serial, mass storage and network adapters, but no audio example. TinyUSB (MIT) is an alternative only if its CH32V307 high-speed audio path proves reliable: a 2023 report showed UAC2 control requests failing on this chip, and the driver contributor had not verified isochronous transfers.

The open-source USBAudioToI2S firmware (R7, Apache-2.0) is kept as a known-good comparison on the same hardware and as a reference; licence notices are kept if any code is copied. Its source (audio_cpld.v) ties the MCLK output low, runs one PLL from the 48 kHz-family clock, has no 352.8 kHz path and a combinational clock select, and its descriptors report self-powered (0xC0), bMaxPower 0x32 (100 mA), Feature Unit bmaControls = 0 and VID 0x1A86 / PID 0. The plan below is proven in a two-stage proof of concept before layout (firmware gate).

## 5.1 Firmware plan and proof of concept

| # | Topic | Required behaviour |
| --- | --- | --- |
| 1 | USB core | High-speed port: enumeration, descriptors, standard requests, suspend/resume. Descriptors: bus-powered (bmAttributes 0x80), bMaxPower 0xFA (500 mA); test VID/PID 1209:0001 during development (pid.codes reserves it for private testing only), a real PID before publishing; serial number from the CH32V307 96-bit unique ID |
| 2 | UAC2 controls | Clock source with the rates (CUR/RANGE: all 8 rates from 44.1 to 384 kHz, 352.8 kHz included), clock valid, mute and volume (CUR/RANGE) mapped to the ES9018K2M attenuator over I²C plus MCU fine scaling for the channel trim, alternate settings for 16/24/32-bit |
| 3 | Streaming | Isochronous OUT → ring buffer → DMA to the CPLD. At 384 kHz, 32-bit stereo: 384 bytes per 125 µs microframe, up to 392 bytes with one extra sample |
| 4 | Feedback | The CPLD’s LRCLK counted by an MCU timer against USB (micro)frames, plus a small FIFO-level correction; sent in 16.16 format (at 384 kHz: 48.0 samples per microframe = 0x300000) |
| 5 | Application | ES9018K2M driver (asynchronous mode, DPLL bandwidth register 12, 32-bit I²S, soft start, mute, lock status, master trim); power states and sequencing (2.5); rate and family changes (5.2); CC and VBUS sensing, ILIM_HI control; fault and overload handling (5.2); calibration storage (master and channel trim); test modes (DC fault injection by a digital DC signal, I²S-header tristate) |
| 6 | MCU–CPLD interface | Ours to define, for example samples by DMA over I²S or SPI, FIFO level over SPI, LRCLK back to a timer-capable MCU pin. Write the interface specification and freeze the pin map before the proof-of-concept board and the schematic (open item) |
| 7 | CPLD logic | Our own: FIFO (written in the 100 MHz system-clock domain, read in the PLL bit-clock domain), I²S output engine on the PLL output with exact ÷2/4/8/16, PLL reset and lock handling, oscillator enables OSC48_EN/OSC44_EN, SPI registers (3.3). No clock multiplexer in the fabric |
| 8 | Proof of concept, stage 1 | Desk setup: WCH CH32V307V-EVT-R1 board, an AGM 2K-LUT CPLD board and an ES9018K2M I²S-input DAC board (≈ US$48–57 for the three boards at listing prices, outside the board budget). Order: enumeration and controls without audio → streaming into a dummy sink with timer feedback → full chain with CPLD and DAC |
| 9 | Proof of concept, stage 2 (optional) | Digital proof-of-concept board with the real chips and pin map (≈ US$59 / 1,860 NTD for two assembled boards, estimate) |
| 10 | Gate pass criteria | Enumerates bus-powered with the new descriptors; every rate plays on Windows 10/11, macOS and Linux; BCLK/LRCLK average rates correct for every rate (frequency counter) and the DPLL locks; host volume and mute work; PLL relock after every family change, no runt pulses, all 8 rates bit-perfect; CPLD meets timing at 100 MHz and at the PLL output (49.152 MHz) |
| 11 | Unit tests | Pure-logic parts (descriptor tables, feedback math, ring buffer, volume mapping) compile on a PC and run as unit tests in GitLab CI |
| 12 | Licences and reference | Keep the licence notices if any code is copied (USBAudioToI2S Apache-2.0, TinyUSB MIT). USBAudioToI2S stays a known-good comparison on the same hardware and a reference, not the base of our firmware |

## 5.2 State machine and policies

| State / event | Required behaviour |
| --- | --- |
| Attached | Only 3V3M is on; the MCU enumerates within 100 mA; the latch is set; all enables are low |
| Configured | Read CC1/CC2 → Default or High mode (ILIM_HI high only with ≥ 1.5 A advertised; a USB-A-to-C cable reads as Default); run the power-up sequence (2.5) |
| Playing | Monitor RAILS_OK_N, FLT, ANA_FAULT and the VBUS watchdog |
| Suspend | Power-down sequence (rails last off); MCU low-power; ≤ 2.5 mA is a target (open item) |
| DC fault | The hardware latch mutes (AMP_EN low, rails off) → firmware soft-mutes the DAC and removes the back-powering sources within 10 ms (LP5907 off, DAC reset), logs, waits 2 s and clears; if the fault returns within 10 s it retries; after 3 faults it stays latched until USB replug; the red LED blinks |
| Overload | PGOOD kill → soft mute, volume ceiling lowered 3 dB per event (down to −12 dB), re-enable after 1 s; the ceiling is kept until replug; TPS2553 FAULT (7.5 ms deglitch) is also read |
| VBUS watchdog | Analog watchdog at 4.30 V → AMP_EN low (target < 5 µs), then the power-down sequence |
| Rate change | MCLK never changes. Within a family: soft mute → CPLD changes the BCLK divider → wait for DPLL lock (lock_status, register 64) → soft unmute. Family change: soft mute → I²S output stopped (held low) → PLL held in reset → OSC48_EN/OSC44_EN switched → wait ≥ 10 ms (oscillator start-up) → PLL released, wait for lock → output restarts → DAC re-locks → soft unmute. AMP_EN stays high (outputs muted digitally); pop measured |
| Calibration | Before calibration apply −3.5 dB; the end-of-line test writes the master trim (1.75 Vrms ± 0.1 dB) and the channel trim (±0.05 dB) to flash |
| Test modes | DC fault injection by a digital DC test signal; I²S-header tristate (CPLD outputs Hi-Z, DAC kept configured over I²C) |
| Programming | CH32V307 through the WCH-LinkE pads; CPLD through the JTAG pads |

# 6. Schematic review checklist

These rules seed the AI datasheet-review agent, and each one can be turned into a planted-error test case. ERC is KiCad’s electrical rules check; a netlist script checks connections, values and counts in the exported netlist; the AI agent reviews rules that need datasheet or part-level interpretation.

| # | Rule | Source | Check method |
| --- | --- | --- | --- |
| 1 | VBUS: 5 × 1 µF before any switch (C102, C501, C503, C301, C505); attach charge ≤ 50 µC (38 µC estimate) | USB; 2.4 | Netlist script |
| 2 | CC1/CC2: 5.1 kΩ Rd each; CC → 10 kΩ (R103, R104) → MCU ADC, nothing else on CC | USB-C; 3.1 | Netlist script |
| 3 | VBUS divider R105 100 kΩ / R106 47 kΩ to an ADC pin that has the analog watchdog | 3.1 | Netlist script |
| 4 | USBLC6-2SC6 on D+, D− and VBUS at the connector; USB pair in a 90 Ω differential net class | R1; 3.1 | Netlist script; AI agent (stack-up) |
| 5 | Every IC supply pin decoupled: CH32V307 100 nF per pin + 2.2 µF; CPLD 4 × 100 nF; each oscillator 100 nF (C217 at X201, C218 at X202, C219 at X203); DAC per 3.4; OPA2197 100 nF per rail pin; OPA1622 100 nF + 10 µF per rail pin; LM27762 per 3.6 | R1–R5 | Netlist script |
| 6 | HSE crystal Y201 8 MHz, CL 20 pF, with 33 pF C0G load capacitors (C201, C202) | R1; 3.2 | Netlist script; AI agent |
| 7 | CH32V307 and CPLD pins match the frozen pin map of the MCU–CPLD interface specification | Section 5 | Netlist script (pin-map diff) |
| 8 | MCU pins toward the CPLD are low or Hi-Z while 3V3D is off (no MCU-side pull-ups into 3V3D); I²C pull-ups R207, R208 on 3V3D | 2.5 | Netlist script; AI agent |
| 9 | MCLK: X201 drives only the DAC MCLK pin (through 33 Ω) and the CPLD clock input (through R209); no other MCLK load | 3.3 | Netlist script |
| 10 | DAC in asynchronous mode with MCLK 100 MHz: allowed only with DVCC 3.3 V and the external 1.3 V DVDD | R3 note 2 | AI agent |
| 11 | External DVDD from ME6216A13M3G fed from 3V3D (DVDD ≤ DVCC); C310 2.2 µF + 100 nF at the pin | R3; 3.4 | Netlist script |
| 12 | AVCC_L and AVCC_R on separate 2.2 Ω branches from the LP5907; LP5907 EN = AVCC_EN with pull-down; LP5907 total output capacitance ≤ 10 µF (5.7 µF) | R3; R6 | Netlist script |
| 13 | DAC RESETB 100 kΩ pull-down and MCU drive; ADDR strap for 0x90 | R3 | Netlist script |
| 14 | Each DAC output pin connects only to its I/V inverting input: no other load, short trace | R3 p. 27 | Netlist script (pin fan-out); AI agent (trace length) |
| 15 | VREF from 3V3A through R431 / (R432 + R433); C421 at the divider and C422 between the two OPA2197s, short traces to all four non-inverting inputs; one shared node | 3.5.1 | Netlist script |
| 16 | Each I/V feedback: 1 kΩ ∥ 5.1 kΩ, with 10 Ω in series with 2 × 330 pF C0G across it | 3.5.1 | Netlist script |
| 17 | C0G only in the signal path (I/V and leg capacitors); no X5R/X7R there | 3.5 | Netlist script (dielectric field) |
| 18 | Four 2.00 kΩ 0.1 % resistors in every difference amplifier; leg+ = I/V(DAC−) − I/V(DAC+), leg− the swap; U401 = L+/L−, U402 = R+/R−; U403 = DACL/DACLB, U404 = DACR/DACRB | 3.5 | Netlist script |
| 19 | AMP_EN: R421 100 kΩ pull-down, R618 10 kΩ series from the MCU, kill FETs Q608 (latch) and Q609 (PGOOD) to ground | 3.5.2; 3.7 | Netlist script |
| 20 | OPA1622 thermal pad to V−; input common mode inside (V−)+1.5 V … (V+)−1 V (≈ 0.85 V on ±3.78 V) | R4 | Netlist script; AI agent |
| 21 | TPS2553: RILIM 100 kΩ + 47 kΩ; R510 through Q501 with gate pull-down R511; RILIM trace short; EN pull-down R513; FAULT pull-up R512 to 3V3M | R9; 3.6 | Netlist script; AI agent (layout) |
| 22 | LM27762 dividers: 100 k + 100 k + 15 k over 100 k (FB+) and 100 k + 100 k + 10 k over 100 k (FB−); bottom resistors ≥ 50 kΩ | R5 | Netlist script |
| 23 | LM27762 EN must not exceed VIN: RAIL_EN only after 5V_ANA (firmware order); RAIL_EN pull-down R506, series R507, kill FET Q607 to ground | R5; 2.5 | AI agent; netlist script |
| 24 | LM27762 PGOOD pulled up to 3V3M (R505) and routed to the MCU, the Q609 gate and the blanking RC | R5; 3.7 | Netlist script |
| 25 | Blanking RC R617 1 MΩ + C606 100 nF on the Q603 gate from PGOOD; blanking acts on node S only, never on the latch nodes | 3.7 | Netlist script; AI agent |
| 26 | Node F: R611 10 kΩ series, R612 47 kΩ pull-up to 3V3M, D605 clamp to GND; only the Q601 gate connects to F | 3.7 | Netlist script; AI agent |
| 27 | Latch: Q604/Q605 cross-coupled with 100 kΩ pull-ups to 3V3M; C605 from 3V3M to node B (powers up latched); Q606 clear from FLT_CLR with pull-down R616; FLT (node B) to an MCU input | 3.7 | Netlist script |
| 28 | DC detect: 147 kΩ + 1 µF per leg, BAV99 into MAX/MIN, 1 MΩ loads, thresholds 100 kΩ / 12 kΩ from the rails | 3.7 | Netlist script |
| 29 | Ferrites: BLM18PG121SN1D in the 5V_ANA path; GZ1608D601TF only on DVCC | 3.4; 3.6 | Netlist script |
| 30 | ESD diode (LESD5D5.0CT1G) on every jack leg | 3.5.2 | Netlist script |
| 31 | Test pad on every rail (5V_SYS, 5V_ANA, 3V3M, 3V3D, 3V3A, 1V3, +3.78 V, −3.78 V) | 3.8 | Netlist script |
| 32 | One part per BOM line: one value, one MPN, one LCSC number and a verified JLCPCB class per line; no mixed or range values | Parts List v0.3 | Netlist script (BOM) |
| 33 | X202 and X203: C218 and C219 100 nF at their supply pins; R210 and R211 33 Ω in series at each oscillator output, joined into one node at the CPLD’s dedicated PLL clock input | 3.3 | Netlist script; AI agent (placement) |
| 34 | Shared family-clock node routed next to the CPLD PLL clock pin: both family oscillators and the node beside the pin, short traces, no stubs; X201 reaches the CPLD system-clock input through R209 | R2; 3.3 | Netlist script (pin map); AI agent (layout) |
| 35 | Short 100 MHz MCLK route from X201 to the DAC MCLK pin; the CPLD branch leaves through R209 at the oscillator | 3.3 | AI agent (layout) |
| 36 | No clock trace (MCLK, family clock, BCLK, LRCLK) crosses the analog area (I/V converters, output legs, VREF) | 3.3; 3.5 | AI agent (layout) |
| 37 | I²S output (BCLK, LRCLK, SDATA) held low during rate-family changes, with the DAC soft-muted and the PLL held in reset while the enables switch | 3.3; 5.2 | AI agent (CPLD logic review); scope check in the proof of concept |
| 38 | Oscillator enables: OSC48_EN with R212 100 kΩ pull-up to 3V3D (X202 on by default) and OSC44_EN with R213 100 kΩ pull-down (X203 off by default) | 3.3 | Netlist script |
| 39 | X201 enable pin tied high (3V3D); X201 is never disabled | 3.3 | Netlist script |
| 40 | OSC48_EN and OSC44_EN driven from CPLD I/O in the same 3V3D domain as the oscillators (no drive from the always-on MCU domain, so no back-powering when 3V3D is off) | 2.5; 3.3 | Netlist script (power domain); AI agent |

# 7. Open items and review disposition

Open items are verification tasks with a closing gate: MCU–CPLD interface specification (before the proof-of-concept board), firmware gate (two-stage proof of concept on development hardware, before layout), SPICE before layout, schematic review, JLCPCB quote after layout, bring-up.

## 7.1 Open items

| # | Item | Why it matters | How to close | Gate |
| --- | --- | --- | --- | --- |
| 1 | Own UAC2 firmware and CPLD logic (top risk: host compatibility of high-speed isochronous streaming with feedback) | v1 depends on our own USB stack on the CH32V307 high-speed port; WCH has no high-speed audio example | Two-stage proof of concept on development hardware (5.1) | Firmware gate |
| 2 | MCU–CPLD interface specification | The link is ours to define and the pin map depends on it | Write the specification and freeze the pin map | Before the proof-of-concept board and the schematic |
| 3 | CPLD timing at 100 MHz and at the PLL output (49.152 MHz), including the FIFO crossing between the two clock domains | Every bit clock comes from the PLL; the system side runs at 100 MHz | Build and time the CPLD design; frequency counter and duty on every rate; PLL relock and runt-pulse check at family changes | Firmware gate |
| 4 | X203 disabled output must be high-impedance | X202 and X203 share one node at the CPLD PLL input | Confirm in the SCTF datasheet; fallback: a small 2:1 logic multiplexer in front of the PLL input | Before the schematic |
| 5 | Shared oscillator node routing | Two oscillator outputs and the PLL clock pin on one net | Both oscillators and the node next to the CPLD PLL clock pin, short traces; scope check of the clock | Layout review; bring-up |
| 6 | 100 MHz MCLK at the ES9018K2M maximum | The ±10 ppm oscillator sits at the datasheet limit | Verify lock and performance; look for a stocked 80–98 MHz oscillator | Bring-up |
| 7 | MCU and CPLD currents are assumptions; oscillator typicals assumed (datasheet maxima used) | The unconfigured state must stay ≤ 100 mA; the configured totals depend on them | Measure; mitigation: lower MCU clock before configuration | Bring-up |
| 8 | USB Default-mode worst case 0.473 A (27 mA margin) | Close to the 500 mA USB 2.0 limit; rests on the current estimates | Measure in every state with a USB power meter | Bring-up |
| 9 | USB suspend ≤ 2.5 mA | Not guaranteed in v1 | Measure; define the v1 behaviour | Bring-up |
| 10 | DAC pin-to-pin DC mismatch (≤ 1 mV assumed) and null point (±20 mV assumed) | Offset budget terms of 1.18 mV and 1.17 mV | Measure; 100 % offset screening | Bring-up |
| 11 | OPA1622 swing and current capability at ±3.78 V | Extrapolated from typical ±18 V curves | Bench-verify the power table | Bring-up |
| 12 | LM27762 charge-pump resistance at 4.5 V (3.0 Ω assumed) | Sets the 0.17 V negative-rail margin | Measure the negative-rail margin at VBUS 4.60 V | Bring-up |
| 13 | Stock: CH32V307 (119), ES9018K2M (724), AGRV2KQ32 (980), ME6216A13M3G (≈ 410), 4.4 mm jack (721) | Low LCSC stock (Jun–Sep 2026) | Reserve the parts in the JLCPCB parts library before ordering | Before ordering |
| 14 | Cost 34 NTD under the 1,700 NTD ceiling (conservative) | Conservative fees worth ≈ 97 NTD per board (through-hole Extended fee, X-ray of the three oscillators) | JLCPCB quote from the real BOM/CPL files after layout | JLCPCB quote |
| 15 | Oscillators X202 and X203: JLCPCB class and assembly fixture | X202’s class is not shown on its JLCPCB page (assumed Extended) and the page notes that an assembly fixture may be needed; X203 is Extended per its page | Confirm both in the JLCPCB quote | JLCPCB quote |
| 16 | Unplug pop ordering not guaranteed | Fast VBUS collapse | Measure; VBUS watchdog | Bring-up |
| 17 | 4.4 mm plug shorted leg+ to leg− | Outside TI’s one-amplifier short rating (≈ 0.55 W per package) | Survival test | Bring-up |
| 18 | OPA2197 stability with the DAC pin capacitance and the 10 Ω + 660 pF network | I/V converter loop | SPICE | Before layout |
| 19 | Both jacks live; 4.4-to-3.5 mm adapter | One headphone at a time | Warning stays in the manual | Manual |

## 7.2 Review disposition, fourth round (R4-A, R4-B, R4-C)

IDs: A = R4-A (review 1 of Spec v0.4), B = R4-B (review 2), C = R4-C (review 3). Status values: Closed (fixed in this revision); Closed – design, verify (design change made; a bench or firmware-gate check confirms it); Closed – verify (confirmed by a later check); Open (tracked item with owner and gate).

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| A-H1 | FB101 mis-specified (GZ1608D601TF = 200 mA / 450 mΩ, C1002; C1015 is GZ2012D101TF) | Power ferrite now Murata BLM18PG121SN1D (C14709, Basic, 2 A, 50 mΩ) in the 5V_ANA path; GZ1608D601TF (C1002) only on DVCC (≈ 14 mA); drops recomputed (≈ 15–25 mV) | Closed |
| A-H2 | Offset used OPA1678 typical as maximum | Confirmed (±0.5 mV typ / ±2 mV max); I/V op-amp now OPA2197 (±0.25 mV max); budget 7.9 / 15.9 mV worst, balanced limit ±20 mV, 100 % screening; DAC-mismatch assumption Open (measure) | Closed |
| A-H3 | Safe disconnected output not established | Full sequence and current paths; rails first on / last off; PGOOD hardware kill; back-powering bounded (≤ 1.7 mA per pin) and removed in 10 ms; energy ≤ 16.5 mJ before / ≤ 0.22 mJ after the trip; protection defined at the headphone terminals | Closed – design, verify |
| A-H4 | Negative-rail headroom | Rails ±3.78 V; both rails budgeted at 4.60 V and maximum load with a 3.0 Ω charge-pump resistance (margin 0.17 V) | Closed |
| A-H5 | 100 mA ceiling not enforced | Clean envelope (80 mA per leg, swing curve) separated from overload; TPS2553 enforces the analog supply current; PGOOD kill; firmware ceiling steps; overload dissipation bounded; overload tests replace the 30-min 32 Ω balanced test | Closed – design, verify |
| A-H6 | USB power states and descriptors | State-by-state budget; MCU-only before configuration (≤ 83 mA estimate); descriptors 0x80 / 0xFA; CC detection by ADC for High mode; suspend policy (2.5 mA Open) | Closed – design, verify (firmware gate) |
| A-M1 | Mixed purchasing lines and placeholder ranges | One part per line, every value specified, 48 purchasing lines by LCSC number | Closed |
| A-M2 | 13.8 NTD ceiling margin; one Basic→Extended flip fails it | Every class verified on the JLCPCB page except X202 (not shown; assumed Extended); Extended count conservative (19, incl. the through-hole jack, X202 and X203); still a planning estimate until the JLCPCB quote | Closed – verify |
| A-M3 | Gain tolerance defeats the minimum power targets | Per-unit calibration to 1.75 Vrms per leg; targets defined at the calibrated level | Closed |
| A-M4 | Two LDOs do not guarantee sequencing | Separate enables (LP5907 EN, ME6211 CE) in a firmware sequence; DVDD LDO fed from 3V3D so DVDD ≤ DVCC by construction; scope check at bring-up | Closed – design, verify |
| A-M5 | Clock adaptation needs an earlier gate | Three oscillators: X201 100 MHz for the DAC MCLK (asynchronous mode, no PLL in the MCLK path) and the CPLD system clock; X202 24.576 MHz and X203 22.5792 MHz, selected by their enables in front of the CPLD PLL ×2 (all 8 rates exact); two-stage proof of concept on development hardware before layout | Closed – design, verify |
| B-S1 | −4.5 V rail cannot regulate from USB | As A-H4 | Closed |
| B-S2 | 2.0 Vrms per leg does not fit the supply; +1 dB units clip | Calibrated 1.75 Vrms per leg; swing model from OPA1622 Fig. 38/39; pre-calibration trim −3.5 dB | Closed |
| B-S3 | Power targets fail on low-spread units; rating method | As A-M3: “1 % THD+N or calibrated 0 dBFS, whichever first” | Closed |
| B-S4 | Firmware needs much more than the spec said | Firmware scope listed module by module for our own UAC2 firmware and CPLD logic (Section 5); two-stage proof of concept before layout; 352.8 kHz comes from X203 through the same CPLD PLL | Closed – verify |
| B-S5 | 14 NTD margin will be used up (odd values, 1.5 nF C0G, crystal, AMS1117, through-hole jack fee) | Basic E24 values or Basic combinations (100 k + 47 k, 1 k ∥ 5.1 k, 15 k + 1 k, 2 × 330 pF); Basic 5032 crystal; AMS1117 removed; through-hole fee counted; no cheaper 4.4 mm jack exists at LCSC (no SMD option) | Closed – verify |
| B-M1 | Pop at unplug has no hardware control | PGOOD gates AMP_EN in hardware; VBUS watchdog; separate unplug pop target (≤ 20 mV) | Closed – design, verify |
| B-M2 | Fault latch re-trips on clear | Set path blanked ≈ 60 ms after PGOOD good, without clearing the latch | Closed |
| B-M3 | PLL jitter goes to the DAC in sync mode | Asynchronous mode, MCLK straight from the 100 MHz oscillator (no PLL in the DAC MCLK path); the CPLD PLL is only in the bit-clock path (both families), whose jitter the DAC’s DPLL rejects; J-test stays in the plan | Closed |
| B-M4 | > 100 mA before enumeration; phones limit OTG | As A-H6; Default mode ≤ 0.473 A covers phones | Closed |
| B-M5 | AMS1117-ADJ needs a tantalum-type capacitor and a minimum load | Fixed 1.3 V ME6216A13M3G (±1 %, 300 mA, ceramic-stable) | Closed |
| B-minor | Duplicate version rows in the Spec history; U301 “voltage-mode” text; HSE 8 MHz; VREF risk overstated | Spec history cleaned; U301 text fixed; HSE 8 MHz confirmed; VREF risk downgraded and quantified (1.17 mV) | Closed |
| C-B1 | USB power before configuration | As A-H6 | Closed – verify |
| C-B2 | Reference descriptors wrong (self-powered, 100 mA) | 0x80 / 0xFA in the firmware gate | Closed – verify |
| C-B3 | 352.8 kHz MCLK not supported by the reference CPLD | Confirmed in the source; replaced by our own CPLD logic: the family oscillator (X202 24.576 MHz or X203 22.5792 MHz) is selected before the PLL, as the reviewer suggested; all 8 rates exact, 352.8 kHz included | Closed – design, verify |
| C-B4 | Clock-family switching can glitch | MCLK never switches; the family changes by the oscillator enables while the output is stopped and the PLL is held in reset; no clock multiplexer in the CPLD fabric; scope check for runt pulses | Closed – design, verify |
| C-B5 | Host volume not implemented | UAC2 volume/mute with GET/SET CUR/RANGE and I²C mapping, tested on three OSes in the gate | Closed – verify |
| C-B6 | “100 mA design current” not implemented | As A-H5 | Closed – design, verify |
| C-H1 | 500 mA budget has no margin | Default mode ≤ 0.473 A (27 mA below 0.5 A) enforced; High mode only with a ≥ 1.5 A Type-C advertisement | Closed |
| C-H2 | +4.5 V corner | Rails ±3.78 V, both rails budgeted | Closed |
| C-H3 | 7.7 µF table is not an inrush proof | Attach charge 38 µC including the 3V3M output capacitance; staggered, soft-started domains | Closed – verify |
| C-H4 | Cost provisional; “Assign LCSC no.” left; “every part is LCSC” inaccurate | Every line has an LCSC number and a verified class (X202: assumed Extended until the JLCPCB quote) | Closed |
| C-H5 | Offset margin | As A-H2 | Closed |
| C-H6 | ≤ 5 mV pop ambitious without relays | Split targets; full sequence incl. unplug/brownout validated on the bench | Closed – verify |
| C-H7 | USB identity (WCH VID, PID 0) | pid.codes VID 0x1209: test PID 0x0001 during development, a real PID before publishing; UID-based serial | Closed – verify |

## 7.3 Review disposition, earlier rounds (R1–R3)

Findings of the first three review rounds as closed in Notes v0.3, kept unchanged for traceability. Values in this table describe the design at that time.

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| R1-B1 | VBUS_PD ≈ 130 µF vs USB-C 10 µF | No PD port; single 5 V port, VBUS budget 7.7 µF | Closed – N/A |
| R1-B2 | LMR33630 + SMAJ18A surge stacking ≈ 44 V | No inverting converter or 15 V input; LM27762 runs from 5 V | Closed – N/A |
| R1-B3 | LP5907 on 1V8_D over 10 µF | No 1.8 V rails; the only LP5907 (DAC 3.3 V) is budgeted ≤ 10 µF | Closed – N/A |
| R1-B4 | Paralleling open | No paralleling; one amplifier per leg (100 mA design current) | Closed |
| R1-B5 | LM339 at −12 V pulling 1.8 V logic | Comparator on ±4.5 V drives a PNP into a 3.3 V latch; 3.3 V nodes only see transistor/N-FET pull-downs, never below 0 V | Closed |
| R1-B6 | Offset ±5 mV vs 11 mV | Worst case recomputed (current mode: 7.4 / 14.7 mV); targets ±10 / ±15 mV | Closed |
| R1-H | AMS1117 dropout/noise | ME6211 (digital), LP5907 (DAC analog) | Closed |
| R1-H | XU316 VOH vs AK4493 VIH | All logic 3.3 V | Closed – N/A |
| R1-H | Both jacks live / gain-switch pop | Gain switch removed; both jacks live documented (one headphone at a time) | Closed |
| R1-H | OPA1622 stock 197 vs 400 | Sample run needs 10 (2 per board) | Closed |
| R1-H | LM317/LM337 cost trade | LM27762 low-noise LDOs | Closed – N/A |
| R1-H | CI “every part has an LCSC number” vs Digi-Key/Mouser parts | Every part is now an LCSC/JLCPCB part; the rule stands as written | Closed |
| R1-H | Firmware port map before layout | USBAudioToI2S build on the reference pin map before layout (open item 1) | Closed as a gate |
| R1 | Consistency: v0.2 wording, cost source of truth, THD+N conditions, Zout incl. contacts, DSD pins, grounding/ESD/CC/silkscreen | Fixed in Spec v0.4: single USB-C port removes the silkscreen confusion; CC = 5.1 kΩ Rd; jack ESD PESD15VL1BA; grounding rules added; DSD not in v1 | Closed |
| R2-S1 | TVS does not protect −14.7 V converter; no OVP | Block removed | Closed – N/A |
| R2-S2 | VBUS bulk before contract; analog rails start at 5 V | Block removed | Closed – N/A |
| R2-S3 | Balanced Zout with option B | Legs driven directly (< 0.1 Ω + contacts) | Closed |
| R2-S4 | K1000 1 W no margin | Requirement change (budget redesign); ≈ 0.13 W, 95 dB SPL documented | Closed |
| R2-S5 | Offset target contradicts design | As R1-B6 | Closed |
| R2-M1 | Board ≈ 7 W, PLA case | ≤ ≈ 2.5 W board, ≤ 0.54 W per OPA1622 | Closed |
| R2-M2 | LM317 near dropout; LM337 LC filter | Block removed | Closed – N/A |
| R2-M3 | ±0.05 dB trim cannot use 0.5 dB DAC steps | Optional digital scaling in the MCU firmware | Closed |
| R2-M4 | Max power criterion | Rated at 1 kHz, 1 % THD+N, both channels | Closed |
| R2-M5 | Cost margin 10 NTD / exchange rate | Gate uses fixed 31.7 NTD/US$: target 1,500 NTD per board (WARN above), approved ceiling 1,550 (FAIL above); current 1,536 → WARNING | Closed |
| R2-M6 | 4.4-to-3.5 mm adapter shorts leg− | One amplifier per package shorted (TI continuous-short rating), ≤ 0.6 W; risk + manual warning | Closed |
| R2-M7 | Unbalanced crosstalk needs ≤ 6 mΩ ground | Layout rule (≤ 5.7 mΩ) | Closed |
| R2 | Minor: stale versions; KC2520Z “(assumed)”; through-hole count; pop requirement; CH224A/AMS1117 unverified claims; NVIDIA paragraph | Versions fixed; oscillators counted leadless conservatively; only the 4.4 mm jack is through-hole (confirm USB-C shell pegs); pop ≤ 5 mV; CH224A removed; AMS1117-3.3 replaced by ME6211C33 (digital 3V3) and LP5907 (DAC analog); an AMS1117-ADJ is used only for the 1.30 V DAC core supply; job-application text moved to the repository README | Closed |
| R3-1 | DC fault latch and recovery | Hardware 3.3 V latch, firmware recovery policy (Section 5, item 6), status to MCU | Closed |
| R3-2 | TVS/converter envelope | Block removed | Closed – N/A |
| R3-3 | Stage-1 gain 0.9009 with the input network | No stage 1; the gain now includes the DAC’s 806 Ω source (current mode, gain = 2 × Rf/806 per leg) | Closed |
| R3-4 | QA403 cannot drive 1.8 V I²S | I²S is now 3.3 V; header + firmware test mode + sequencing specified | Closed |
| R3-5 | 200 mA “limit” not implemented; thermal at 16 Ω balanced | 100 mA/leg design current, rated power at 1 % THD+N, full load matrix; 0.54 W worst per package | Closed |
| R3 | Consistency: rails-good qualification; offset/trim; workbook external links; cost/sourcing reconciliation | LM27762 PG + latch + 50 ms; offset/trim closed; the delivered v0.1 workbook references only its own sheets (links appear when sheets are copied into another workbook; v0.2 adds a self-check); single source of truth, all LCSC | Closed |

# 8. Chosen parts

Parts for the 5-board sample run, one line per LCSC part number (48 purchasing lines); Qty is per board. Unit prices are the LCSC tier that matches the run quantity (1+ tier for one-per-board ICs, 10+ for two-per-board parts). Parts List v0.3 is the source of truth for cost; the figures below are copied from it. Every line has an LCSC number and a JLCPCB class checked on the JLCPCB part page or basic-parts list (Jun–Sep 2026); the class of X202 is not shown on its page and is assumed Extended until the JLCPCB quote.

| # | Part | MPN | LCSC no. | JLCPCB class | Qty | Unit US$ | Refs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Jack, 4.4 mm balanced | G-Switch GT-3321667P-01 | C2762984 | Extended | 1 | 6.8656 | J701 |
| 2 | Op-amp, output legs, VSON-10 3 × 3 mm | TI OPA1622IDRCR | C2876559 | Extended | 2 | 1.7871 | U401, U402 |
| 3 | DAC, QFN-28 5 × 5 mm | ESS ES9018K2M | C2688377 | Extended | 1 | 3.4678 | U301 |
| 4 | MCU, USB HS, LQFP-64 10 × 10 mm | WCH CH32V307RCT6 | C2988083 | Extended | 1 | 3.45 | U201 |
| 5 | Op-amp, I/V converters, SOIC-8 | TI OPA2197IDR | C139363 | Extended | 2 | 1.2164 | U403, U404 |
| 6 | CPLD, QFN-32 4 × 4 mm | AGM AGRV2KQ32 | C18208899 | Extended | 1 | 1.6503 | U202 |
| 7 | Charge pump + LDOs, ±3.78 V, WSON-12 2 × 3 mm | TI LM27762DSSR | C473398 | Extended | 1 | 1.1014 | U501 |
| 8 | Oscillator, 100 MHz | YXC OT3225100MJBA4SL | C4353707 | Extended | 1 | 1.0964 | X201 |
| 9 | Current-limited switch, SOT-23-6 | TI TPS2553DBVR | C55266 | Extended | 1 | 0.5966 | U503 |
| 10 | Oscillator, 22.5792 MHz | SCTF SX2M22.5792M20F30TNN | C5452663 | Extended | 1 | 0.5455 | X203 |
| 11 | Resistor, 2.00 kΩ, 0603, 0.1 %, 25 ppm thin film | YAGEO RT0603BRD072KL | C728588 | Extended | 16 | 0.034 | R401–R416 |
| 12 | Oscillator, 24.576 MHz | YXC OT322524.576MJBA4SL | C2831388 | Extended | 1 | 0.5075 | X202 |
| 13 | Jack, 3.5 mm | HRO PJ-332A-6A | C2848643 | Extended | 1 | 0.2921 | J702 |
| 14 | Crystal, 8 MHz | YXC X50328MSB2GI | C115962 | Basic | 1 | 0.2452 | Y201 |
| 15 | LDO 3.3 V (3V3A), SOT-23-5 | TI LP5907MFX-3.3/NOPB | C80670 | Extended | 1 | 0.2104 | U302 |
| 16 | USB ESD array, SOT-23-6 | ST USBLC6-2SC6 | C7519 | Extended | 1 | 0.1844 | U101 |
| 17 | Capacitor, 10 µF, 0805, 25 V X5R | Samsung CL21A106KAYNNNE | C15850 | Basic | 9 | 0.02 | C421, C413–C416, C507, C509, C510, C511 |
| 18 | N-MOSFET, 2N7002, SOT-23 | JSCJ 2N7002 | C8545 | Basic | 10 | 0.0164 | Q501, Q601–Q609 |
| 19 | LDO 3.3 V (3V3M, 3V3D), SOT-23-5 | Microne ME6211C33M5G-N | C82942 | Extended | 2 | 0.0603 | U502, U504 |
| 20 | USB-C receptacle | SHOU HAN TYPE-C 16PIN 2MD(073) | C2765186 | Extended | 1 | 0.1 | J101 |
| 21 | Capacitor, 1 µF, 0603, 25 V X5R | Samsung CL10A105KB8NNNC | C15849 | Basic | 16 | 0.006 | C102, C301, C302, C311, C312, C501, C502, C503, C504, C505, C506, C508, C601–C604 |
| 22 | Capacitor, 100 nF, 0402, 16 V X7R | Samsung CL05B104KO5NNNC | C1525 | Basic | 32 | 0.003 | C203–C210, C212, C213–C216, C217–C219, C305–C309, C422, C423–C426, C409–C412, C605, C606 |
| 23 | Comparator, SOIC-8 | onsemi LM393DR2G | C7955 | Basic | 1 | 0.09 | U601 |
| 24 | Diode, BAV99, SOT-23 | Nexperia BAV99,215 | C2500 | Basic | 4 | 0.0211 | D601–D604 |
| 25 | Capacitor, 330 pF, 0603, 50 V C0G | Samsung CL10C331JB8NNNC | C1664 | Basic | 16 | 0.005 | C417–C420, C427–C430, C401–C408 |
| 26 | Diode, 5 V, SOD-523 | GOODWORK LESD5D5.0CT1G | C41399463 | Extended | 4 | 0.02 | D701–D704 |
| 27 | Resistor, 100 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1003TCE | C25741 | Basic | 30 | 0.002 | R105, R212, R213, R303, R305, R421, R508, R509, R510, R511, R512, R513, R501, R515, R502, R503, R517, R504, R505, R506, R601–R604, R607, R608, R613, R614, R615, R616 |
| 28 | LDO 1.3 V (DVDD), SOT-23 | Microne ME6216A13M3G | C236662 | Extended | 1 | 0.06 | U303 |
| 29 | Capacitor, 2.2 µF, 0402, 6.3 V X5R | Samsung CL05A225MQ5NSNC | C12530 | Basic | 4 | 0.005 | C211, C303, C304, C310 |
| 30 | Resistor, 10 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1002TCE | C25744 | Basic | 9 | 0.002 | R103, R104, R201, R202, R306, R518, R507, R611, R618 |
| 31 | LED, Green, 0805 | Hubei KENTO KT-0805G | C2297 | Basic | 1 | 0.0163 | D705 |
| 32 | Diode, 1N4148WS, SOD-323 | JSCJ 1N4148WS | C2128 | Basic | 1 | 0.016 | D605 |
| 33 | Ferrite bead, 120 Ω @ 100 MHz, 0603 | Murata BLM18PG121SN1D | C14709 | Basic | 1 | 0.0157 | FB501 |
| 34 | Resistor, 47 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF4702TCE | C25792 | Basic | 7 | 0.002 | R106, R514, R619–R622, R612 |
| 35 | Resistor, 33 Ω, 0402, 1 % | UNI-ROYAL 0402WGF330JTCE | C25105 | Basic | 7 | 0.002 | R203–R206, R209, R210, R211 |
| 36 | Ferrite bead, 600 Ω @ 100 MHz, 0603 | Sunlord GZ1608D601TF | C1002 | Basic | 1 | 0.0127 | FB301 |
| 37 | Resistor, 5.1 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF5101TCE | C25905 | Basic | 6 | 0.002 | R101, R102, R434–R437 |
| 38 | Resistor, 1 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1001TCE | C11702 | Basic | 5 | 0.002 | R423–R426, R433 |
| 39 | Resistor, 1 MΩ, 0402, 1 % | UNI-ROYAL 0402WGF1004TCE | C26083 | Basic | 4 | 0.002 | R107, R605, R606, R617 |
| 40 | Resistor, 2.2 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF2201TCE | C25879 | Basic | 4 | 0.002 | R207, R208, R701, R702 |
| 41 | Resistor, 10 Ω, 0603, 1 % | UNI-ROYAL 0603WAF100JT5E | C22859 | Basic | 4 | 0.002 | R427–R430 |
| 42 | Resistor, 0 Ω, 0402, jumper | UNI-ROYAL 0402WGF0000TCE | C17168 | Basic | 4 | 0.002 | R417–R420 |
| 43 | LED, Red, 0603 | Hubei KENTO KT-0603R | C2286 | Basic | 1 | 0.0075 | D706 |
| 44 | Resistor, 2.2 Ω, 0805, 1 % | UNI-ROYAL 0805W8F220JT5E | C17521 | Basic | 2 | 0.003 | R301, R302 |
| 45 | Resistor, 15 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1502TCE | C25756 | Basic | 3 | 0.002 | R431, R432, R516 |
| 46 | Capacitor, 33 pF, 0402, 50 V C0G | FH 0402CG330J500NT | C1562 | Basic | 2 | 0.002 | C201, C202 |
| 47 | Resistor, 12 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1202TCE | C25752 | Basic | 2 | 0.002 | R609, R610 |
| 48 | Capacitor, 4.7 nF, 0402, 50 V X7R | FH 0402B472K500NT | C1538 | Basic | 1 | 0.002 | C101 |

Cost at this revision (fixed 31.7 NTD per US$, JLCPCB Economic PCBA): parts US$141.88 (4,497 NTD), five bare 4-layer PCBs US$10 (estimate), fees US$110.89 → US$262.77 = 8,330 NTD = 1,666 NTD per board. Fees: setup 8.18; stencil 1.53; 19 Extended part types × 3.07 = 58.33 (includes the through-hole 4.4 mm jack and X202, conservatively); SMT joints 681 per board × 0.0016 × 5 = 5.45; X-ray 8 leadless parts per board × 5 × 0.82 = 32.80; through-hole 4.15; Confirm Parts Placement 0.45.

Cost gate: target 1,500 NTD per board (WARN above), owner-approved ceiling 1,700 NTD per board (FAIL above; raised on 24 Sep 2026 from 1,550 to 1,650 NTD with the second oscillator and to 1,700 NTD with the third) → status WARNING, 34 NTD under the ceiling. If JLCPCB does not charge the Extended fee for the through-hole jack (−19.5 NTD per board) and does not X-ray the three oscillators (−26.0 NTD per board each), the run is ≈ 1,569 NTD per board. The estimate moved from 1,540 (one oscillator) to 1,602 (+24.576 MHz) and 1,666 NTD per board (+22.5792 MHz). The figure is a planning estimate; the gate becomes final with the JLCPCB quote from the real BOM/CPL files after layout.

Pricing rule: US$0.10 minimum per purchasing line for parts under US$0.05 (attrition and minimum-quantity allowance). By block (NTD per board): USB-C input and sensing 10; USB bridge 240; DAC and supplies 121; I/V and output legs 215; power 64; protection 13; outputs, indicators and board 230 (4.4 mm jack 218). Counts: 126 BOM lines (121 fitted), 48 purchasing lines, 227 placed parts per board, 19 Extended + 29 Basic part types, 681 SMT + 7 through-hole joints and 8 leadless parts per board; 100-unit reference ≈ 801 NTD per board. Excluded: shipping, import tax, programmers, enclosure.

# Appendix A. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Initial issue: datasheet-derived requirements and starting values for schematic capture; companion to Design Specification v0.2 |
| v0.2 | 23 Sep 2026 | Diego Yang | Remaining parts chosen; CH224A replaces CH224Q; TPS7A4901 replaces LT3045; protection circuit defined (LM339 + 2 × HFD4 relays); open items updated |
| v0.3 | 23 Sep 2026 | Diego Yang | Cost-driven redesign after three design reviews, companion to Design Specification v0.4: CH32V307 + AGRV2KQ32 USB bridge, ES9018K2M DAC, 2 × OPA1622 legs, LM27762 ±4.5 V, relay-less 3.3 V latch protection |
| v0.3 (revised) | 24 Sep 2026 | Diego Yang | Current-mode DAC outputs with OPA1678 I/V converters (option B); estimates, checklist, open items and cost updated |
| v0.4 | 24 Sep 2026 | Diego Yang | Fourth review round (R4-A, R4-B, R4-C), companion to Design Specification v0.5: ±3.78 V rails; calibrated 1.75 Vrms per leg; TPS2553 current limit with Default and High modes; switched USB power domains with CC sensing; OPA2197 I/V; single 100 MHz clock with the DAC in asynchronous mode; completed protection with an all-2N7002 latch; corrected ferrite; fixed 1.3 V LDO; ±10 / ±20 mV offset with screening; one-part-per-line BOM; firmware gate |
| v0.4 amendment | 24 Sep 2026 | Diego Yang | Owner decisions of 24 Sep 2026: family oscillators for both rate families, first X202 (24.576 MHz, 48 kHz family) and then X203 (22.5792 MHz, 44.1 kHz family), selected by their enables in front of the CPLD PLL ×2 so that all 8 rates have exact 50:50 bit clocks, because one 100 MHz clock gave a 40:60 bit-clock duty cycle at 352.8/384 kHz against the ES9018K2M 45:55 requirement; own USB firmware and CPLD logic instead of modifying USBAudioToI2S (kept as a known-good comparison). Currents, power-up sequence, block notes, firmware plan, checklist, open items, review rows, both figures, parts table (48 lines) and cost (1,666 NTD per board, ceiling 1,700 NTD) updated |