# USB DAC + Balanced Headphone Amplifier — Design Specification v0.4

Product requirements and architecture for a personal PCB project

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 24 Sep 2026 | v0.4 (draft for review) | Product requirements + architecture |

Companion documents: “Schematic Design Notes v0.3” holds the block-level circuit guidance, and “Parts List v0.2” (DAC_HPA_Parts_List_v0.2.xlsx) is the source of truth for cost. This specification states the requirements and architecture they implement.

# 1. Purpose and scope

This specification defines the requirements and architecture of a single USB-C, bus-powered Hi-Res DAC with one ESS ES9018K2M DAC chip and a balanced headphone amplifier. The outputs are a 3.5 mm unbalanced jack and a 4.4 mm balanced jack, for one headphone at a time.

v0.4 is a cost-driven redesign. Three independent design reviews of v0.3 found blockers (Appendix C), and the budget was then clarified. With every review fix applied, the v0.3 architecture (XMOS XU316 + AKM AK4493SEQ + 4 × OPA1622 + 15 V USB PD power) was estimated at ≈ 17,100 NTD for 5 boards (≈ 3,400 NTD per board). v0.4 therefore uses a cheaper USB bridge, DAC and amplifier, and accepts lower drive than K1000/Susvara-class amplifiers.

Goal: “best measured performance for the money”, proven by published measurements; not “beats every product”, which a hobby analyzer cannot prove (QuantAsylum itself says its QA403 won’t match a US$30,000 analyzer).

Budget: ≤ 1,500 NTD per board refers to the 5-board sample run (what the owner actually pays: ≈ 7,500 NTD for five assembled boards, including the bare PCBs and JLCPCB assembly fees, excluding shipping, import tax, programmers and enclosure). It is not a 100-unit average. Owner decision (24 Sep 2026): a small overrun of the 1,500 NTD target is accepted, with an approved ceiling of 1,550 NTD per board (Section 6).

Portfolio context is kept in the repository README.

| In scope for v1 | Out of scope for v1 |
| --- | --- |
| USB-C Hi-Res DAC with a single ES9018K2M DAC chip, bus-powered from one USB-C port (data + 5 V) | DSD in v1 (stretch goal: the ES9018K2M supports DSD; depends on firmware) |
| Balanced headphone amplifier: 4.4 mm balanced jack plus 3.5 mm unbalanced jack (one headphone at a time) | K1000-class 1 W drive (v0.4 gives ≈ 0.13 W into the K1000, Section 5) |
| PCM 44.1–384 kHz, 16–32 bit, USB Audio Class 2.0 High-Speed | USB PD power (the v0.3 15 V PD port is removed) |
| DC-fault protection: hardware fault latch that mutes the amplifiers and turns the analog rails off | Output relays |
| Host volume in the DAC’s digital volume control | Hardware volume knob |
| Design automation: CI checks, BOM cost gate, DFF/DFA/DFT rules, AI datasheet review, automated measurement | Enclosure design (3D-printed case for prototypes only) |

# 2. Key targets

All values below are proposed targets for v0.4, based on the ES9018K2M, OPA1678 and OPA1622 datasheets and the v0.4 design calculations; every board revision is measured against this table (Section 8). Design estimates and assumptions are labelled as such.

| Parameter | Target | Minimum | Conditions | How verified |
| --- | --- | --- | --- | --- |
| PCM sample rate | 44.1–384 kHz, 16–32 bit | 384 kHz | USB Audio Class 2.0, High-Speed | Host playback sweep at every rate |
| DSD | Not in v1 (stretch: the ES9018K2M supports DSD; depends on firmware) | — | — | — |
| Hosts | Windows 10/11 built-in UAC2 driver, macOS, Linux; Android over USB-C (High-Speed) | — | — | Host playback on each OS |
| THD+N at 1 kHz | ≤ −105 dB | ≤ −100 dB | 0 dBFS, 300 Ω, both jacks, 20 Hz–20 kHz (proposed) | QA403 via Python (Section 8) |
| Dynamic range | ≥ 115 dB(A) | ≥ 112 dB(A) | Both jacks (estimate ≈ 118–119 dB(A) with the I/V stage) | QA403 |
| Level at 0 dBFS | 2.0 Vrms unbalanced / 4.0 Vrms balanced | — | By design: 2 × 750/806 per leg (DAC current mode); unit-to-unit spread +1.0/−0.9 dB | QA403, 1 kHz, no load and 300 Ω |
| Output power (1 % THD+N, both channels) | Unbalanced ≥ 120 mW at 32 Ω; balanced ≥ 250 mW at 60 Ω, ≥ 50 mW at 300 Ω | — | 1 kHz; design estimates in Section 5.2 | QA403 + dummy loads |
| Output impedance | ≤ 0.5 Ω | — | 1 kHz, both jacks, including contacts | Load/no-load method, 32 Ω and 300 Ω |
| Frequency response | +0.1/−0.3 dB, 20 Hz–20 kHz | — | 44.1, 48 and 384 kHz sample rates; estimate −0.15 dB at 20 kHz (I/V and leg poles) | QA403 |
| Crosstalk at 1 kHz | Balanced ≤ −100 dB (300 Ω); unbalanced ≤ −75 dB (32 Ω, at the jack) | — | Layout rule for the shared ground (Section 7) | QA403 |
| Channel balance | ≤ 0.3 dB; optional firmware fine trim to ±0.05 dB by 32-bit digital scaling in the MCU (DAC volume steps are 0.5 dB) | — | — | QA403 |
| DC offset | ≤ ±10 mV unbalanced, ≤ ±15 mV balanced | — | Worst-case estimate 7.4 / 14.7 mV (balanced close to the limit) | Multimeter at the jack |
| Protection | Outputs muted and rails off within 100 ms of ≥ ±2 V DC on any leg (≈ 50–58 ms); ≈ 20 ms for a rail-stuck leg; starts latched at power-up | — | No false trip at 20 Hz full scale | Oscilloscope; DC-fault injection |
| Pop | Output transient ≤ 5 mV peak | — | Power-on, power-off (USB unplug), sample-rate change, fault recovery; 32 Ω and 300 Ω | Oscilloscope |
| Power | Bus-powered, ≤ 500 mA at 5 V; VBUS capacitance ≤ 10 µF | — | Estimate 0.49 A worst case | USB current at full scale into 60 Ω balanced; attach inrush |
| Thermal | All parts within ratings | — | 30 min at full scale into 32 Ω balanced | Thermocouple / IR camera |
| PCB | 4 layers, target ≤ 80 × 60 mm, all SMT parts on the top side except the 4.4 mm jack, 90 Ω USB pair | — | JLCPCB Economic PCBA | ERC/DRC and layout-rule checks in CI (Section 8) |
| Cost | ≤ 1,500 NTD per board for the 5-board run (≤ 7,500 NTD); approved ceiling 1,550 NTD per board (owner decision, 24 Sep 2026) | — | Including PCB and PCBA fees; excluding shipping, tax and tools | Automated BOM cost gate on every commit: WARN > 1,500, FAIL > 1,550 (Section 8) |

# 3. Architecture

## 3.1 Signal and control path

Figure 1. Signal and control path (DAC in current mode with I/V converters).

The single USB-C port carries USB 2.0 High-Speed audio and the 5 V bus. D+, D− and VBUS pass through a USBLC6-2SC6 ESD array; CC1 and CC2 have 5.1 kΩ Rd pull-downs.

USB bridge: a WCH CH32V307RCT6 RISC-V MCU with a built-in USB 2.0 High-Speed PHY runs USB Audio Class 2.0 with the open-source USBAudioToI2S firmware (Apache-2.0; MCU + CPLD asynchronous USB audio, up to 32-bit/384 kHz). The pin map follows the project’s schematic, so the firmware runs unmodified first; the 8 MHz HSE crystal frequency follows the WCH reference schematic (to verify).

An AGM AGRV2KQ32 CPLD generates the I²S clocks from two 3.3 V audio oscillators, 22.5792 MHz for the 44.1 kHz family and 24.576 MHz for the 48 kHz family (the project states both are necessary), and drives MCLK, BCLK, LRCLK and SDATA to the DAC through 33 Ω series resistors.

DAC MCLK plan (to verify): the CPLD outputs MCLK = 128 × fs for the ES9018K2M synchronous mode (sync_mode), from 5.6448/6.144 MHz at 44.1/48 kHz up to 45.1584/49.152 MHz at 352.8/384 kHz (oscillator × 2 via the CPLD PLL). This is a small change to the project’s Verilog. A dedicated ≥ 74 MHz oscillator for asynchronous mode (MCLK > 192 × 384 kHz = 73.7 MHz) is a DNF option.

The ESS ES9018K2M (32-bit, PCM to 384 kHz via I²S) is configured by the MCU over I²C (address 0x90) and runs in differential current mode. Each output pin is equivalent to a voltage source in series with 806 Ω ±11 % (datasheet p. 31), and ESS specifies DNR and THD+N only in differential current mode, so v0.4 follows the ESS application circuit (p. 27) with one I/V converter per pin. The MCU drives RESETB (100 kΩ pull-down) and reads the DPLL lock status.

Signal chain: DAC (current mode) → 2 × OPA1678 I/V converters (one amplifier per DAC pin, VREF ≈ 1.70 V) → 2 × OPA1622 difference-amplifier legs → jacks. U403 serves DACL/DACLB and U404 DACR/DACRB, on ±4.5 V; each pin drives only an inverting input (a virtual ground), and the non-inverting inputs sit at VREF = 1.706 V, the DAC’s zero-DC-current point. Each feedback is 750 Ω 1 % in parallel with 10 Ω + 1.5 nF C0G (≈ 141 kHz), so each I/V output swings ±1.42 V peak around VREF (0.28–3.12 V), inside the OPA1678 swing on ±4.5 V.

Output stage: 2 × TI OPA1622, one amplifier per leg, with no paralleling. Each of the four amplifiers is a 4-resistor 2.00 kΩ difference amplifier fed by the low-impedance I/V outputs: leg+ = I/V(DAC−) − I/V(DAC+), leg− = the swap (each I/V inverts). The leg input common mode is VREF (non-inverting node ≈ 0.85 V DC), inside the OPA1622 range, and VREF noise and error cancel as common mode. U401 drives L+ and L−, U402 drives R+ and R−.

Protection: an LM393 window comparator detects DC on any leg and sets a 3.3 V-domain hardware latch, which pulls both OPA1622 EN pins and the LM27762 EN+/EN− low (outputs muted, analog rails off). Firmware clears the latch; power-up starts latched. There are no relays.

The 4.4 mm balanced jack carries L+, L−, R+, R− and GND; the 3.5 mm unbalanced jack carries L+, R+ and GND. Both jacks are live, so only one headphone is connected at a time.

## 3.2 Power path

The single USB-C port supplies data and 5 V; the board is bus-powered within the USB 2.0 limit of 500 mA. A 600 Ω ferrite feeds VBUS to 5V_SYS.

VBUS capacitance budget: 7.7 µF ≤ 10 µF (1 µF at the connector + 4.7 µF LM27762 input + 1 µF ME6211 input + 1 µF LP5907 input). Capacitance behind the LDOs is charged through their current limit; attach inrush is verified at bring-up.

Analog rails: a TI LM27762 (inverting charge pump + low-noise LDOs, ±1.5 to ±5 V, ±250 mA, 2.7–5.5 V input, 2 MHz, separate EN+/EN−, power-good) makes ±4.5 V for the OPA1622 legs and the OPA1678 I/V converters, with a 1 µF flying capacitor and 10 µF outputs. Its positive LDO runs from VIN: at VBUS ≥ 4.65 V (4.75 V minus cable drop, before the ferrite drop) the +4.5 V output has ≥ 105 mV headroom against the 45 mV dropout at 100 mA; after the ferrite drop the headroom is small (risk in Section 9.1). PG goes to the MCU.

Digital 3.3 V: an ME6211C33 LDO supplies the MCU, CPLD, oscillators, the DAC DVCC (through a ferrite) and the fault latch.

DAC analog 3.3 V: an LP5907-3.3 from 5V_SYS supplies VCCA and AVCC_L/AVCC_R, with AVCC_L and AVCC_R on separate branches (1 Ω + 2.2 µF + 100 nF each), so the LP5907 output total is 5.7 µF (under its 10 µF limit). 3V3A also feeds the I/V reference divider: VREF = 3V3A × 10.7/(10.0 + 10.7) = 1.706 V, filtered by 10 µF + 100 nF (≈ 3 Hz), so VREF tracks AVCC.

DAC DVDD: an AMS1117-ADJ set to 1.30 V (240 Ω / 10 Ω: 1.25 × (1 + 10/240) = 1.302 V) supplies the external DVDD that the ES9018K2M requires for PCM when FSR > 192 kHz; the 5.2 mA divider current also meets the regulator’s minimum load. Fed from 3V3, it starts after DVCC, as the ESS power-up sequence requires (p. 27).

The fault latch runs from 3V3, so it keeps its state when the ±4.5 V rails turn off (no hiccup).

Figure 2. Power path (the ±4.5 V rails feed 2 × OPA1622 and 2 × OPA1678).

## 3.3 Volume and gain structure

Gain is fixed; the v0.3 gain switch is removed. Volume comes from the host via USB Audio Class and is applied in the ES9018K2M digital volume (per channel, 0 to −127.5 dB in 0.5 dB steps); a hardware knob is out of scope. Each I/V converter has a gain of 750/806 from the DAC’s internal source, and each leg is a unity-gain difference amplifier (2 kΩ/2 kΩ), so the level per leg is 2 × 750/806 × 1.078 Vrms = 2.0 Vrms by design.

| Jack | Level per leg at 0 dBFS | Full scale | Notes |
| --- | --- | --- | --- |
| 3.5 mm unbalanced | 2.0 Vrms (leg+) | 2.0 Vrms | By design: 2 × 750/806 × 1.078 Vrms = 2.0 Vrms per leg (DAC current mode); unit-to-unit spread +1.0/−0.9 dB from the 806 Ω ±11 % source (moves only the maximum level; L and R on one die track) |
| 4.4 mm balanced | 2.0 Vrms (leg+ and leg−) | 4.0 Vrms | 2.83 V peak per leg vs an estimated ±3.3 V usable swing at ≤ 100 mA on ±4.5 V rails: ≈ 1.3 dB headroom (verify on OPA1622 Fig. 38/39) |

Channel balance: an optional fine trim in the MCU firmware (32-bit digital scaling) sets the balance to ±0.05 dB, because the DAC volume steps are 0.5 dB.

# 4. Key parts

| Part | Role | Key specs | Package / assembly note | Unit price 1–9 pcs | Source |
| --- | --- | --- | --- | --- | --- |
| WCH CH32V307RCT6 | USB audio bridge (MCU) | 144 MHz RISC-V, 3.3 V; built-in USB 2.0 High-Speed PHY; runs the USBAudioToI2S firmware | LQFP-64, 10 × 10 mm; leaded, no X-ray fee; JLCPCB Extended | US$3.45 (10+: US$2.98) | LCSC C2988083 (119–261 in stock) |
| AGM AGRV2KQ32 | CPLD: I²S clocks and DAC MCLK | 2K LUTs; PLL (clock multiplication); 3.3 V LVCMOS; JTAG/ISP; instant-on non-volatile | QFN-32, 4 × 4 mm; leadless, X-ray fee; JLCPCB Extended | US$1.65 (10+: US$1.40) | LCSC C18208899 (980 in stock) |
| ESS ES9018K2M | DAC | 32-bit; PCM to 384 kHz via I²S; DSD; DNR 127 dB-A and THD+N −120 dB (datasheet, differential current mode only); each output pin ≈ voltage source + 806 Ω ±11 %, 3.783 mApp full-scale current per pin; I²C control (0x90 or 0x92); volume 0 to −127.5 dB in 0.5 dB steps; external DVDD +1.3 V ±5 % for PCM when MCLK > 50 MHz or FSR > 192 kHz; MCLK > 192 × FSR (normal mode) or = 128 × FSR (sync_mode) | QFN-28, 5 × 5 mm; leadless, X-ray fee; JLCPCB Extended | US$3.47 (10+: US$2.93) | LCSC C2688377 (724–756 in stock); ESS datasheet v3.7 |
| TI OPA1678IDR (× 2) | I/V converters, one amplifier per DAC pin | 4.5 nV/√Hz; THD 0.0001 % at 1 kHz; input bias 10 pA; rail-to-rail output within 800 mV at 2 kΩ; ±2.25 to ±18 V; 2 mA per channel | SOIC-8; leaded, no X-ray fee; JLCPCB Extended | US$1.07 (10+: US$0.97) | LCSC C192421 (10,592 in stock) |
| TI OPA1622 (× 2) | Output legs, one amplifier per leg | ±2 to ±18 V; Vos ±100 µV typ / ±500 µV max; Ib 1.2 µA typ / 2.0 µA max; Isc +145/−130 mA typ; swing 800 mV from rail (no load); Iq 2.6 mA typ per channel; EN threshold ≈ 0.82 V referenced to its GND pin; thermal shutdown 175 °C; continuous short rated for one amplifier per package | VSON-10, 3 × 3 mm; leadless, X-ray fee; thermal pad to V−; JLCPCB Extended | US$2.63 | LCSC C2876559 (197 in stock); TI SBOS727B |
| TI LM27762DSSR | ±4.5 V analog rails | ±1.5 to ±5 V adjustable, ±250 mA; 2.7–5.5 V input; 2 MHz inverting charge pump + negative LDO (30 mV dropout at 100 mA) + positive LDO from VIN (45 mV dropout at 100 mA); separate EN+/EN−; PG (active low) | WSON-12, 2 × 3 mm; leadless, X-ray fee; JLCPCB Extended | US$1.10 (10+: US$0.95) | LCSC C473398 (9,948 in stock) |

All other parts are chosen in Schematic Design Notes v0.3, Section 8 (part number, LCSC number, stock and price); Parts List v0.2 is the source of truth for cost.

# 5. Headphone drive

## 5.1 Load requirements

Current, not voltage, limits the low-impedance loads. The design current is 100 mA peak per leg, with one OPA1622 amplifier per leg and no paralleling. The OPA1622’s −130 mA figure is a typical short-circuit current, not a linear output rating, so it is not used as a design limit.

| Requirement | Value | Basis |
| --- | --- | --- |
| Loads | 16–600 Ω on both jacks | Load matrix in Section 5.2 |
| Design current | 100 mA peak per leg | One amplifier per leg (U401 = L+/L−, U402 = R+/R−) |
| OPA1622 output current | +145/−130 mA typical short-circuit current; not a linear rating | TI SBOS727B |
| Full-scale level | 2.0 Vrms per leg (2.83 V peak) by design: 2.0 Vrms unbalanced, 4.0 Vrms balanced; unit-to-unit spread +1.0/−0.9 dB | I/V gain 750/806 per pin from the DAC’s 806 Ω ±11 % source (datasheet p. 31) |
| Voltage headroom | ≈ 1.3 dB: 2.83 V peak per leg vs an estimated ±3.3 V usable swing at ≤ 100 mA on ±4.5 V | Estimate; verify on OPA1622 Fig. 38/39 |
| Output impedance | < 0.1 Ω plus jack contacts (the OPA1622 legs drive the jacks directly, no ballast); target ≤ 0.5 Ω on both jacks (≤ 1.0 Ω balanced if the 0.47 Ω option is fitted) | Estimate |
| Reference headphones | HIFIMAN Susvara (60 Ω, 83 dB/mW); AKG K1000 (120 Ω, 74 dB/mW) | Maker specifications; SPL in Section 5.2 |

## 5.2 Output power by jack

Rated power is defined at 1 kHz, 1 % THD+N, both channels driven, and is measured on the bench. The values below are design estimates at 0 dBFS, limited either by the 100 mA peak design current per leg or by the full-scale level (2.0 Vrms unbalanced, 4.0 Vrms balanced by design; unit-to-unit spread +1.0/−0.9 dB). They are unchanged by the move to current mode.

| Load | 3.5 mm unbalanced | 4.4 mm balanced |
| --- | --- | --- |
| 16 Ω | 80 mW (100 mA/leg) | 80 mW (100 mA/leg) |
| 32 Ω | 125 mW (2.0 Vrms) | 160 mW (100 mA/leg) |
| 60 Ω | 67 mW | 267 mW (4.0 Vrms) |
| 120 Ω | 33 mW | 133 mW |
| 300 Ω | 13 mW | 53 mW |
| 600 Ω | 6.7 mW | 27 mW |

SPL from the maker sensitivity, 4.4 mm balanced at 0 dBFS (estimates). The v0.3 target of 1 W into the K1000 is dropped.

| Headphone | Impedance, sensitivity | Power at 0 dBFS | Peak current per leg | SPL |
| --- | --- | --- | --- | --- |
| HIFIMAN Susvara | 60 Ω, 83 dB/mW | ≈ 0.27 W (4.0 Vrms) | 94 mA | ≈ 107 dB SPL |
| AKG K1000 | 120 Ω, 74 dB/mW | ≈ 0.13 W (4.0 Vrms) | 47 mA | ≈ 95 dB SPL |

## 5.3 Power, thermal and protection

USB current and board power are estimates, measured at bring-up:

| Load on VBUS | Current at 5 V | Note |
| --- | --- | --- |
| Digital: MCU with High-Speed USB, CPLD, 2 oscillators, DAC including the 50 mA external DVDD | ≈ 0.21 A | Estimate |
| Analog: full scale into 60 Ω balanced, both channels | ≈ 0.28 A | Estimate; includes 2 × OPA1678 (≈ 8 mA per rail quiescent) and the I/V output currents; the charge pump draws about its output current |
| Worst case, total | ≈ 0.49 A | vs the 0.50 A USB 2.0 limit: tight, measure at bring-up; fallback: read the Type-C current advertisement on CC |
| Idle | ≈ 0.25 A | Estimate |

Board dissipation is ≤ ≈ 2.4 W. OPA1622 dissipation is ≤ 0.54 W per package (16 Ω balanced at the 100 mA limit) and ≤ 0.32 W at 60 Ω balanced, so thermal is no longer a concern; each thermal pad is tied to V− copper.

Protection is required, because the target headphones are expensive and hard to replace. There are no relays; a DC fault mutes the amplifiers and turns the analog rails off through a hardware latch:

| Function | Implementation | Requirement |
| --- | --- | --- |
| DC-offset detect | Each leg → 100 kΩ + 1 µF (τ = 100 ms) → BAV99 into MAX and MIN nodes (1 MΩ loads); LM393 on ±4.5 V compares MAX > +0.48 V and MIN < −0.48 V (100 kΩ / 12 kΩ from ±4.5 V); trip ≈ ±0.78–0.88 V DC at a leg | Muted and rails off within 100 ms of ≥ ±2 V DC on any leg (≈ 50–58 ms at 2 V; ≈ 19–22 ms for a leg stuck at a rail); no false trip on a full-scale 20 Hz tone (0.22 V peak after the filter) |
| Fault latch | Open-collector outputs → FAULT_N (47 kΩ to +4.5 V) → PNP MMBT3906 level shifter → 3.3 V latch (MMBT3904 + MMBT3906 from 3V3) → two 2N7002 pull AMP_EN (both OPA1622 EN) and RAIL_EN (LM27762 EN+ and EN−) to 0 V | 3.3 V nodes never exceed 3.3 V or go below 0 V; the latch holds its state when the ±4.5 V rails turn off |
| Power-up | 100 nF sets the latch at power-up. The DAC supplies follow ESS p. 27: VCCA, AVCC and DVCC together from the 3.3 V LDOs, external 1.3 V DVDD after DVCC. Firmware releases RESETB ≥ 1 ms after all rails and MCLK are stable, clears the latch after USB enumeration and DAC configuration, then waits for LM27762 PG + 50 ms before AMP_EN goes high | Starts latched: rails off, amplifiers off, DAC in reset |
| Recovery | MCU FLT_CLR pulses an MMBT3904 to clear the latch and reads its state directly (3.3 V logic). Firmware soft-mutes the DAC, logs, waits 2 s and clears; if the fault returns within 10 s it retries; after 3 faults it stays latched until USB replug; the red LED blinks | Policy per Schematic Design Notes v0.3, Section 5 |
| Pop-free start and stop | OPA1622 EN pins (100 kΩ pull-downs); DAC soft start ramps the outputs to ½ AVCC; soft mute around sample-rate changes | Output transient ≤ 5 mV peak at power-on, power-off (USB unplug), sample-rate change and fault recovery (32 Ω and 300 Ω) |
| Adapter short | A 4.4 mm-to-3.5 mm adapter shorts leg−; with U401 = L+/L− and U402 = R+/R−, only one amplifier per package is shorted, which TI’s continuous-short rating covers (“Short-circuit to VS/2 ... one amplifier per package”) | ≤ 0.6 W per package; warning in the manual; shorted-plug test on the dummy-load bench |
| Jack ESD | 4 × PESD15VL1BA (15 V standoff, 16 pF), one per jack leg | — |
| Bring-up safety | Dummy resistors only | Never test on real headphones |

# 6. Cost budget

Budget for the 5-board sample run on JLCPCB Economic PCBA, at the fixed rate of 31.7 NTD per US$ that the cost gate uses. Part prices are 1–9-piece prices from Parts List v0.2, the source of truth for cost; quoted lines use distributor prices and the rest are estimates.

| Item | Basis | US$, 5 boards | NTD per board |
| --- | --- | --- | --- |
| USB-C input and protection | Quotes + estimates | 2.04 | 13 |
| USB bridge: CH32V307 + CPLD + crystal + 2 oscillators | Quotes + estimates | 31.68 | 201 |
| DAC: ES9018K2M + LDOs | Quotes + estimates | 19.37 | 123 |
| I/V converters and output legs: 2 × OPA1678 + 2 × OPA1622 + 0.1 % resistors | Quotes + estimates | 41.62 | 264 |
| Power: digital 3.3 V and analog ±4.5 V | Quotes + estimates | 6.40 | 41 |
| Protection and control | Estimates | 1.69 | 11 |
| Outputs, indicators, test (the 4.4 mm jack is 218 NTD of this) | Quotes + estimates | 37.15 | 236 |
| Minimum-spend allowance (US$0.10 per passive line) | Planning allowance | 1.74 | 11 |
| Subtotal, parts (4,491 NTD) | Parts List v0.2 | 141.68 | 898 |
| 5 bare 4-layer PCBs | Estimate | 10.00 | 63 |
| Economic PCBA setup | JLCPCB price list | 8.18 | 52 |
| Stencil | JLCPCB price list | 1.53 | 10 |
| Extended part types (14 × US$3.07) | JLCPCB price list | 42.98 | 272 |
| SMT joints (579 per board × US$0.0016 × 5) | JLCPCB price list | 4.63 | 29 |
| X-ray (7 leadless parts per board × 5 × US$0.82) | Counts both oscillators; −US$8.20 if JLCPCB does not X-ray them | 28.70 | 182 |
| Through-hole joints (4.4 mm jack, 7 joints × 5 × US$0.0164 + US$3.58) | JLCPCB price list | 4.15 | 26 |
| Confirm Parts Placement | JLCPCB price list | 0.45 | 3 |
| Subtotal, JLCPCB fees | JLCPCB price list | 90.63 | 575 |
| Total, 5-board run (7,681 NTD) | Parts + PCBs + fees | 242.30 | 1,536 |
| Cost-gate status: target 1,500 NTD per board; approved ceiling 1,550 NTD | WARN above 1,500, FAIL above 1,550 (owner decision, 24 Sep 2026); 36 NTD over the target, 14 NTD under the ceiling | — | WARNING |

Lines are rounded to whole NTD, so a subtotal can differ by up to 2 NTD from the sum of its lines.

Excluded: shipping, import tax, the WCH-LinkE and CPLD programming cable, and the enclosure.

Counts: 81 BOM lines (76 fitted), 54 purchasing lines, 181 placed parts per board, 14 Extended + 39 Basic part types, 579 SMT joints per board, 7 leadless parts per board and 1 through-hole part (the 4.4 mm jack).

Owner decision (24 Sep 2026): “a little bit off-budget is acceptable”. The target stays at 1,500 NTD per board and the approved ceiling is 1,550 NTD per board, so at 1,536 NTD the Parts List v0.2 cost gate shows WARNING (36 NTD over the target, 14 NTD under the ceiling). Levers: (1) if JLCPCB does not X-ray the two oscillators, −52 NTD per board → ≈ 1,484 NTD (back under the target); (2) a cheaper 4.4 mm jack (the current one is 218 NTD per board at 1–9 pcs; each US$1 saved = 31.7 NTD per board).

Reference only, not the budget basis: a 100-unit build on Standard PCBA comes to ≈ 772 NTD per board. The budget and the cost gate use the 5-board sample run above.

# 7. Design for JLCPCB (DFF/DFA/DFT rules)

These rules apply to every revision and use JLCPCB’s PCBA price list; the 5-board sample run is built on Economic PCBA. The rules that can be checked automatically also run in CI (Section 8).

| ID | Type | Rule |
| --- | --- | --- |
| R01 | DFA | The sample run uses Economic PCBA: setup US$8.18, stencil US$1.53, 2–50 boards per order, one side only (all SMT parts on the top side). |
| R02 | DFA | Prefer JLCPCB Basic parts (no per-type fee): all passives, transistors, LM393, ME6211, AMS1117, the ferrite and the crystal. Each Extended part type costs US$3.07 (14 here). Parts marked “Basic (likely)” are confirmed in the JLCPCB library before ordering. |
| R03 | DFA | X-ray costs US$0.82 per leadless part, so prefer leaded packages where an equivalent exists. Leadless here (7 per board): AGRV2KQ32 CPLD, ES9018K2M, 2 × OPA1622, LM27762 and the two oscillators (counted conservatively; JLCPCB may not X-ray them). The CH32V307RCT6 (LQFP-64) and the 2 × OPA1678 (SOIC-8) are leaded. |
| R04 | DFA | Joint fees: SMT US$0.0016 per joint (579 per board); through-hole US$0.0164 per joint + US$3.58 per order. Only the 4.4 mm jack is through-hole (7 joints); confirm that the USB-C receptacle’s shell pegs do not add a through-hole line. |
| R05 | DFA | Turn on “Confirm Parts Placement” (US$0.45) and check the 3D preview for rotation and polarity errors. |
| R06 | DFA | Every fitted part has an LCSC number and stock ≥ the 5-board quantity (checked in CI). |
| R07 | DFT | Test pad on every rail (5V_SYS, 3V3, 3V3A, 1V3, +4.5 V, −4.5 V) and on the clocks, I²S, EN/PG and fault nodes: 24 test pads including 4 GND. |
| R08 | DFT | Debug pads for the MCU (WCH-LinkE 2-wire: 3V3, SWDIO, SWCLK, GND) and the CPLD (JTAG: 3V3, TCK, TMS, TDI, TDO, GND); DNF 1 × 6 I²S header (MCLK, BCLK, LRCLK, SDATA, GND, 3V3 ref) with a firmware test mode that tristates the CPLD I²S outputs, so an external 3.3 V source (QA403, via series resistors) can drive the DAC. |
| R09 | DFT | Fiducials and polarity marks. |
| R10 | DFF | JLCPCB’s standard 4-layer impedance-controlled stack-up for the 90 Ω USB pair; solid ground plane; digital, charge-pump (2 MHz) and analog areas separated; the unbalanced jack sleeve gets a dedicated return with ≥ 4 vias (≤ 5.7 mΩ shared ground for −75 dB crosstalk at 32 Ω). |

# 8. Automation and verification plan

Every check runs automatically with an explicit pass/fail rule.

| Check | Runs when | Pass/fail rule |
| --- | --- | --- |
| ERC and DRC (KiCad command line) | Every commit (GitLab CI) | Zero errors |
| Manufacturing outputs: Gerbers, drill, BOM, pick-and-place, schematic PDF | Every commit | Files generated |
| BOM check | Every commit | Every fitted part has an LCSC number and stock ≥ 5-board quantity; Extended part types and leadless parts counted and reported |
| Cost gate | Every commit | 5-board run at a fixed 31.7 NTD/US$, including PCBs and Economic PCBA fees: WARN if > 1,500 NTD per board (target), FAIL if > 1,550 NTD per board (approved ceiling, owner decision 24 Sep 2026); currently 1,536 NTD: WARN. A 100-unit reference line (≈ 772 NTD) is reported but not gated |
| Python DFF/DFA/DFT and layout rules | Every commit | All rules pass: analog/charge-pump separation, 90 Ω USB pair, decoupling capacitor near every IC power pin, test point on every rail, fiducials present, jack-ground rule |
| AI datasheet-review agent | Every merge request | Reads the DAC, amplifier and regulator datasheets, extracts requirements, checks the schematic and comments on the MR; seeded by the Schematic Design Notes checklist and scored on 10–20 planted errors (catch rate and false alarms) |
| Firmware gate | Before layout | USBAudioToI2S builds and runs unmodified on the reference pin map (CH32V307: MounRiver Studio / WCH toolchain; CPLD: AGM toolchain); freezes the pin map |
| Automated measurement suite (Python + QuantAsylum QA403) | Each board revision | THD+N, dynamic range, frequency response, crosstalk, output impedance, DC offset (including the idle DC at the I/V outputs vs VREF) and maximum power at 16–600 Ω on both jacks; auto-generated report compared against Section 2 |
| Automated THD-compensation tuning loop | Each board revision | Python sweeps the ES9018K2M THD-compensation registers 22–25 (2nd/3rd harmonic) while reading THD+N on the QA403; reports the best coefficients for the optional firmware setting |

# 9. Risks and open decisions

## 9.1 Risks

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Firmware from a small open-source project (one contributor, one release): top risk | No working USB audio; schedule | Build USBAudioToI2S before layout (gate); keep the reference pin map; fallback is an XMOS-based v2 (budget permitting) |
| OPA1678 stability with the DAC pin capacitance at the I/V inverting input | Oscillation or peaking; distortion | SPICE the I/V stage (10 Ω + 1.5 nF feedback network, ≈ 141 kHz) with the DAC pin capacitance before layout; check on the bench |
| DAC zero-current point: VREF (1.706 V) must sit at the DAC’s zero-DC-current point (≈ 1.70 V) | DC at the I/V outputs; headroom and offset | At bring-up, measure the idle DC at the I/V outputs (should be ≈ VREF); VREF errors are common mode and removed by the leg difference amplifiers |
| Balanced DC offset close to its limit: worst case ≤ 14.7 mV vs ±15 mV (unbalanced ≤ 7.4 mV vs ±10 mV); DAC pin-to-pin offset mismatch assumed ≤ 1 mV | Output DC offset | Measure at the jack on every board; the largest term is common-mode leakage through the 0.1 % resistors (1.70 V × 0.20 % = 3.4 mV) |
| The datasheet says an external DVDD should be shut down in standby; the AMS1117-ADJ has no enable | DAC standby behaviour | Confirm the behaviour, or fit an enable-capable LDO |
| USB current 0.49 A worst case (estimate) vs the 0.5 A limit | USB 2.0 power budget | Measure at bring-up; fallback: read the Type-C current advertisement on CC |
| CH32V307 HSE frequency and USB High-Speed layout | USB enumeration | Follow the WCH reference schematic and layout; 90 Ω pair |
| Oscillator LCSC part numbers not yet assigned (3225, 3.3 V, 22.5792 / 24.576 MHz); whether JLCPCB X-rays oscillators | Cost and sourcing | Assign before ordering; −52 NTD per board if they are not X-rayed |
| “Basic (likely)” parts (ME6211C33, AMS1117-ADJ, LM393, GZ1608D601TF, 8 MHz crystal) not yet confirmed in the JLCPCB library | US$3.07 per part type if Extended | Confirm in the JLCPCB library before ordering |
| OPA1622 swing at 100 mA on ±4.5 V (Fig. 38/39) | Headroom (≈ 1.3 dB estimated) | Verify on the datasheet curves and on the bench |
| The +4.5 V positive LDO (LM27762, runs from VIN) has little headroom at minimum USB voltage after the ferrite drop | +4.5 V rail regulation and output swing | Measure at minimum VBUS and full load; fallback: ±4.3 V rails, which cost about 0.5 dB of output headroom (estimated swing 3.1 V vs 3.3 V peak; headroom at 0 dBFS drops from ≈ 1.3 dB to ≈ 0.8 dB) |
| Both jacks live; a 4.4 mm-to-3.5 mm adapter shorts leg− | Current limit in one amplifier per package (≤ 0.6 W) | One headphone at a time; warning in the manual; shorted-plug test on the dummy-load bench |
| Thermal and 2 MHz charge-pump noise coupling into the analog area | Noise floor | Layout separation (Section 7, R10) |
| Cost 1,536 NTD per board: over the 1,500 NTD target (WARNING), 14 NTD under the approved 1,550 NTD ceiling | Budget | Cost gate on every commit; levers in Section 6 (oscillators not X-rayed → ≈ 1,484 NTD) |

## 9.2 Decisions (all closed for v0.4; verification items open)

| Decision | Status | Resolution |
| --- | --- | --- |
| USB bridge | Closed (changed in v0.4) | WCH CH32V307RCT6 + AGM AGRV2KQ32 CPLD with the open-source USBAudioToI2S firmware (replaces the XMOS XU316) |
| DAC | Closed (changed in v0.4) | ESS ES9018K2M (replaces the AK4493SEQ); the newer ES9039Q2M / ES9020Q are a future option |
| DAC output mode | Closed (changed 24 Sep 2026) | Differential current mode with 2 × OPA1678 I/V converters (owner decision, option B), after the ESS application circuit; a direct 2 kΩ voltage-mode connection would give 1.19 Vrms per leg (−5.1 dB) with ±0.4 dB spread |
| Clock source | Closed (changed in v0.4) | Two 3.3 V audio oscillators (22.5792 / 24.576 MHz); the CPLD makes MCLK = 128 × fs (sync_mode) |
| Output stage | Closed (changed in v0.4) | 2 × OPA1622 as four difference amplifiers fed by the I/V outputs, one amplifier per leg; no paralleling, no DC servo |
| Power | Closed (changed in v0.4) | Single USB-C port, bus-powered; LM27762 ±4.5 V (OPA1622 and OPA1678); ME6211C33 3.3 V digital; LP5907-3.3 DAC analog and VREF; AMS1117-ADJ 1.30 V DVDD |
| Connectors | Closed | 3.5 mm unbalanced + 4.4 mm balanced; one headphone at a time |
| Volume and gain | Closed (changed in v0.4) | Fixed gain (2 × 750/806 per leg); host volume in the ES9018K2M digital volume; gain switch removed |
| Protection | Closed (changed in v0.4) | LM393 DC detect + 3.3 V hardware fault latch (amplifiers off, rails off); relays removed |
| Budget | Closed (owner decision, 24 Sep 2026) | Target 1,500 NTD per board; a small overrun is accepted with an approved ceiling of 1,550 NTD per board (currently 1,536 NTD: WARNING) |
| Remaining parts | Closed | Chosen in Schematic Design Notes v0.3, Section 8; Parts List v0.2 is the source of truth for cost |
| Volume knob | Closed (v2 option) | Out of scope for v1 |
| ES9018K2M output impedance and drive (v0.4 open item) | Closed (24 Sep 2026) | Datasheet p. 31: 806 Ω ±11 % per pin; moved to current mode, so each pin only drives an I/V virtual ground |
| DAC MCLK = 128 × fs in the CPLD Verilog | Open (verification) | Small change to the project’s Verilog; check DPLL lock at every rate; DNF ≥ 74 MHz oscillator as the fallback |
| DAC zero-current point (VREF) | Open (verification) | At bring-up, check that the I/V outputs sit at ≈ VREF (1.70 V) at idle |
| OPA1678 stability with the DAC pin capacitance | Open (verification) | SPICE the 10 Ω + 1.5 nF feedback network before layout; check on the bench |
| OPA1622 swing at 100 mA on ±4.5 V | Open (verification) | Datasheet Fig. 38/39; bench at bring-up |
| USB current and attach inrush | Open (verification) | Measure at bring-up (estimate 0.49 A worst case) |
| External DVDD in standby (AMS1117-ADJ has no enable) | Open (verification) | Confirm before layout, or fit an enable-capable LDO |
| CH32V307 HSE frequency | Open (verification) | Per the WCH reference schematic, before layout |
| Basic-part status, oscillator LCSC numbers, USB-C shell pegs | Open (verification) | Confirm in the JLCPCB library before ordering |

# 10. Next steps

| # | Action | Done |
| --- | --- | --- |
| 1 | Set up the GitLab repository and KiCad project skeleton with CI (ERC/DRC + outputs) before drawing anything. | ☐ |
| 2 | Build USBAudioToI2S on the reference hardware pin map (gate before layout). | ☐ |
| 3 | Simulate the I/V converters (OPA1678 with the 806 Ω DAC source and the DAC pin capacitance; 10 Ω + 1.5 nF feedback network) and the OPA1622 difference-amplifier legs with TI SPICE models before layout; confirm level, stability and frequency response. | ☐ |
| 4 | Draw the schematic from Schematic Design Notes v0.3; assign the remaining LCSC numbers; run the cost gate. | ☐ |
| 5 | Get a schematic review. | ☐ |
| 6 | Start layout once the schematic review is closed and the firmware gate has passed. | ☐ |
| 7 | Order the 5-board run. | ☐ |
| 8 | Bring-up and automated measurement against Section 2. | ☐ |

# Appendix A. Reference documents

| Ref | Document | Source |
| --- | --- | --- |
| [1] | ES9018K2M datasheet v3.7 (22 Apr 2021) | ESS Technology |
| [2] | OPA1622 datasheet (SBOS727B); OPA1678 datasheet | Texas Instruments |
| [3] | LM27762 datasheet | Texas Instruments |
| [4] | CH32V307 datasheet | WCH |
| [5] | AGRV2K CPLD datasheet | AGM |
| [6] | USBAudioToI2S: UAC2.0-to-I²S firmware for MCU + CPLD (Apache-2.0) | GitHub, LoveLonelyTime/USBAudioToI2S |
| [7] | JLCPCB PCBA price list | JLCPCB |
| [8] | Schematic Design Notes v0.3 (companion document: block-level circuit guidance and chosen parts) | This project |
| [9] | Parts List v0.2 (source of truth for cost) | This project |

# Appendix B. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Draft for review |
| v0.2 | 23 Sep 2026 | Diego Yang | Budget raised to ≤ 1,500 NTD; outputs changed to 3.5 mm unbalanced + 4.4 mm balanced; parts and targets verified against the XU316-1024-QF60A, AK4493S, AKD4493S-A and OPA1622 datasheets; USB chip changed to XU316-1024-QF60A (1.8 V I/O); OPA1612 stage-1 and LT3045 added; CH224K replaced by CH224Q; JLCPCB fees updated to the 8 Sep 2026 price list; most open decisions closed. |
| v0.3 | 23 Sep 2026 | Diego Yang | Remaining parts chosen from stocked LCSC/Digi-Key parts; CH224A replaces CH224Q; TPS7A4901 replaces LT3045 to stay within 1,500 NTD; protection defined (LM339 + 2 × HFD4 relays with hardware amplifier shutdown); cost re-estimated with real prices at ≈ 1,490 NTD; −14 V rail set to −14.7 V; stage-1 count corrected to one dual OPA1612. |
| v0.4 | 23 Sep 2026 | Diego Yang | Cost-driven redesign for the 5-board budget (≤ 1,500 NTD per board): single USB-C bus-powered design; CH32V307 + AGRV2K CPLD USB bridge (open-source USBAudioToI2S); ES9018K2M DAC; 2 × OPA1622 as four difference amplifiers; LM27762 ±4.5 V; relays replaced by a 3.3 V hardware fault latch; all three design reviews dispositioned (Appendix C). |
| v0.4 | 24 Sep 2026 | Diego Yang | DAC output changed to current mode (owner decision, option B): 2 × OPA1678 I/V converters (Rf 750 Ω, VREF ≈ 1.70 V) between the ES9018K2M and the OPA1622 legs, because the datasheet gives an 806 Ω ±11 % source per pin and specifies performance only in differential current mode; level 2.0 Vrms per leg by design; offset, noise and USB current re-estimated; cost 1,536 NTD per board against the approved 1,550 NTD ceiling (cost gate WARN > 1,500, FAIL > 1,550); Figures 1 and 2, key parts, risks, decisions and review disposition updated. |

# Appendix C. Design review disposition

Three independent reviews of Spec v0.3, Schematic Design Notes v0.2 and Parts List v0.1 are dispositioned below. Status: Closed = fixed in v0.4; Closed – N/A = not applicable after the redesign (the block no longer exists); Open = verification item with an owner and a gate (Section 9.2).

| ID | Finding | Disposition in v0.4 | Status |
| --- | --- | --- | --- |
| R1-B1 | VBUS_PD capacitance ≈ 130 µF vs the USB-C limit of 10 µF | No PD port; single 5 V port with a VBUS capacitance budget of 7.7 µF | Closed – N/A |
| R1-B2 | LMR33630 + SMAJ18A surge stacking ≈ 44 V | No inverting converter and no 15 V input; the LM27762 runs from 5 V | Closed – N/A |
| R1-B3 | LP5907 on 1V8_D over 10 µF output capacitance | No 1.8 V rails; the only LP5907 (DAC 3.3 V) totals 5.7 µF of output capacitance, under its 10 µF limit | Closed – N/A |
| R1-B4 | Paralleling topology left open | No paralleling; one amplifier per leg (100 mA design current) | Closed |
| R1-B5 | LM339 at −12 V pulling 1.8 V logic | Comparator on ±4.5 V drives a PNP into a 3.3 V latch; 3.3 V nodes only see transistor/N-FET pull-downs, never below 0 V | Closed |
| R1-B6 | Offset target ±5 mV vs an 11 mV worst case | Worst case recomputed for the current-mode stage (7.4 / 14.7 mV); targets ±10 / ±15 mV unchanged; balanced is close to its limit (Section 9.1) | Closed |
| R1-H | AMS1117 dropout and noise | ME6211 for digital 3.3 V; LP5907 for DAC analog 3.3 V | Closed |
| R1-H | XU316 VOH vs AK4493 VIH | All logic is 3.3 V | Closed – N/A |
| R1-H | Both jacks live; gain-switch pop | Gain switch removed; both jacks live documented (one headphone at a time) | Closed |
| R1-H | OPA1622 stock 197 vs 400 needed | The 5-board sample run needs 10 (2 per board) | Closed |
| R1-H | LM317/LM337 cost trade | LM27762 low-noise LDOs | Closed – N/A |
| R1-H | CI rule “every part has an LCSC number” vs Digi-Key/Mouser parts | Every part is now an LCSC/JLCPCB part; the rule stands as written | Closed |
| R1-H | Firmware port map before layout | Closed as a gate: USBAudioToI2S build on the reference pin map before layout | Closed |
| R1 consistency | v0.2 wording, cost source of truth, THD+N conditions, Zout including contacts, DSD pins, grounding/ESD/CC/silkscreen | Fixed in Spec v0.4: the single USB-C port removes the silkscreen confusion; CC = 5.1 kΩ Rd; jack ESD PESD15VL1BA; grounding rules added; DSD not in v1 | Closed |
| R2-S1 | TVS does not protect the −14.7 V converter; no OVP | No 15 V input and no inverting converter | Closed – N/A |
| R2-S2 | VBUS bulk before the PD contract; analog rails start at 5 V | No PD contract; single 5 V port | Closed – N/A |
| R2-S3 | Balanced output impedance with option B | The OPA1622 legs drive the jacks directly, no ballast (< 0.1 Ω + contacts) | Closed |
| R2-S4 | K1000 1 W with no margin | Closed by requirement change (budget redesign): ≈ 0.13 W and ≈ 95 dB SPL documented | Closed |
| R2-S5 | Offset target contradicts the design | As R1-B6 | Closed |
| R2-M1 | Board ≈ 7 W in a PLA case | ≤ 2.4 W board; ≤ 0.54 W per OPA1622 | Closed |
| R2-M2 | LM317 near dropout; LM337 LC filter | Blocks removed (LM27762) | Closed – N/A |
| R2-M3 | ±0.05 dB trim cannot use 0.5 dB DAC steps | Optional digital scaling in the MCU firmware | Closed |
| R2-M4 | Maximum-power criterion | Rated at 1 kHz, 1 % THD+N, both channels | Closed |
| R2-M5 | Cost margin 10 NTD; exchange rate | Gate uses a fixed 31.7 NTD/US$: target 1,500 NTD per board (WARN above) and approved ceiling 1,550 NTD (FAIL above), owner decision 24 Sep 2026; currently 1,536: warning | Closed |
| R2-M6 | 4.4-to-3.5 mm adapter shorts leg− | One amplifier per package shorted (TI continuous-short rating), ≤ 0.6 W; risk and manual warning | Closed |
| R2-M7 | Unbalanced crosstalk needs ≤ 6 mΩ shared ground | Layout rule (≤ 5.7 mΩ) | Closed |
| R2 minor | Stale versions; KC2520Z “(assumed)”; through-hole count; pop requirement; unverified CH224A/AMS1117 claims; NVIDIA paragraph | Versions fixed; oscillators counted as leadless conservatively; only the 4.4 mm jack is through-hole (confirm USB-C shell pegs); pop ≤ 5 mV; CH224A removed; AMS1117-3.3 replaced by ME6211C33 (digital 3V3) and LP5907 (DAC analog); an AMS1117-ADJ is used only for the 1.30 V DAC core supply; job-application text moved to the repository README | Closed |
| R3-1 | DC fault latch and recovery | Hardware 3.3 V latch; firmware recovery policy (Section 5.3); status to the MCU | Closed |
| R3-2 | TVS/converter envelope | No 15 V input and no converter | Closed – N/A |
| R3-3 | Stage-1 gain 0.9009 with the input network | The gain now includes the DAC’s 806 Ω source: current mode, gain = 2 × Rf/806 per leg (Rf = 750 Ω gives 2.0 Vrms per leg) | Closed |
| R3-4 | QA403 cannot drive 1.8 V I²S | I²S is now 3.3 V; header, firmware test mode and sequencing specified | Closed |
| R3-5 | 200 mA “limit” not implemented; thermal at 16 Ω balanced | 100 mA per leg design current; rated power at 1 % THD+N; full load matrix; 0.54 W worst case per package | Closed |
| R3 consistency | Rails-good qualification; offset/trim; workbook external links; cost/sourcing reconciliation | Rails-good: LM27762 PG + latch + 50 ms. Offset/trim: as R1-B6 and R2-M3. Workbook links: the delivered v0.1 file references only its own sheets (links appear when sheets are copied into another workbook); v0.2 adds a self-check. Cost/sourcing: single source of truth, all LCSC | Closed |