# USB DAC + Balanced Headphone Amplifier — Schematic Design Notes v0.3

Companion to Design Specification v0.4: datasheet-derived requirements and starting component values for the KiCad schematic of the cost-driven redesign

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 24 Sep 2026 | v0.3 | Schematic design notes (companion to Design Specification v0.4) |

# 0. Purpose and references

This document collects the datasheet-derived requirements and starting component values for drawing the KiCad schematic of the USB DAC + balanced headphone amplifier. It is the companion to Design Specification v0.4. The circuit content is new: after three design reviews the design was re-architected to meet a limit of 1,500 NTD per board for the 5-board sample run, so every block is described from scratch rather than as a change to Notes v0.2. The 24 Sep 2026 revision runs the DAC outputs in differential current mode into OPA1678 I/V converters, per the ESS application circuit (Section 1 and 3.5.1).

The designer owns and verifies every value. Circuit values are starting values unless marked otherwise, to be checked in SPICE (TI OPA1622 model) and on the bench. Labels are kept as written: “assumption” marks a value not yet confirmed, “to verify” marks a check still to be done, and “per datasheet — to verify” marks a value that must come from a datasheet not yet read. References are cited as R1 to R8.

| Ref | Document | Publisher | Document no. / note |
| --- | --- | --- | --- |
| R1 | CH32V307 datasheet and reference manual | WCH | — |
| R2 | AGRV2K datasheet | AGM | — |
| R3 | ES9018K2M datasheet | ESS | v3.7 (22 Apr 2021); pages 8, 27 and 31 used for the output stage |
| R4 | OPA1622 datasheet | TI | SBOS727B |
| R5 | LM27762 datasheet | TI | — |
| R6 | LP5907 datasheet | TI | — |
| R7 | USBAudioToI2S firmware project | GitHub, LoveLonelyTime | Apache-2.0; UAC2.0 to I²S, up to 32-bit 384 kHz |
| R8 | JLCPCB PCBA price list | JLCPCB | Economic PCBA fees used for the 5-board cost gate |

# 1. Key design decisions

The decisions below fix the v0.4 architecture. The Basis column records why each choice was made, so a later change can be checked against the original reason.

| Decision | Choice | Basis |
| --- | --- | --- |
| Cost target | Target ≤ 1,500 NTD per board for the 5-board sample run (≈ 7,500 NTD for five assembled boards, including bare PCBs and JLCPCB assembly fees; excluding shipping, import tax, programmers and enclosure); approved ceiling 1,550 NTD per board (owner, 24 Sep 2026) | Owner clarification after the reviews; not a 100-unit average. The v0.3 architecture with every review fix was ≈ 17,100 NTD for 5 boards (≈ 3,400 per board) |
| Cost status | 1,536 NTD per board (7,681 NTD for 5 boards) at a fixed 31.7 NTD per US$ | WARNING: 36 NTD over the 1,500 NTD target and 14 NTD under the 1,550 NTD ceiling (owner: “a little bit off-budget is acceptable”). Parts List v0.2 is the source of truth (Section 8) |
| USB audio bridge | WCH CH32V307RCT6 (RISC-V MCU with built-in USB 2.0 High-Speed PHY) + AGM AGRV2KQ32 CPLD + two 3.3 V audio oscillators (22.5792 MHz, 24.576 MHz) | Follows USBAudioToI2S (R7): MCU + CPLD asynchronous USB audio, UAC2.0 to I²S up to 32-bit 384 kHz; the project states both oscillators are necessary. One contributor and one release, so firmware bring-up is the top risk (Section 7) |
| DAC | ESS ES9018K2M, I²C register control; outputs in differential current mode into I/V converters, per the ESS application circuit (R3 p. 27) | 32-bit, PCM to 384 kHz via I²S, DSD, DNR up to 127 dB and THD+N −120 dB per datasheet (R3), specified only in differential current mode; QFN-28 5 × 5 mm. ESS lists a newer ES9039Q2M / ES9020Q (future option) |
| DAC core supply | External DVDD 1.30 V from an AMS1117-ADJ | R3: an external DVDD of +1.3 V ±5 % must be used for PCM when MCLK > 50 MHz or FSR > 192 kHz |
| DAC master clock | CPLD outputs MCLK = 128 × fs (synchronous, ES9018K2M sync_mode) | R3 allows MCLK = 128 × FSR with synchronous MCLK up to 384 kHz. Needs a small change to the project’s Verilog (to verify); a dedicated ≥ 74 MHz oscillator is the DNF alternative |
| Output stage | 2 × TI OPA1678 I/V converters (one amplifier per DAC pin) + 2 × TI OPA1622 output legs (one amplifier per leg); each leg is a 4-resistor difference amplifier fed by two I/V outputs | Why current mode (owner decision, option B, 24 Sep 2026): each DAC pin is a source behind 806 Ω ±11 % (R3 p. 31). The rejected direct connection into the 2 kΩ difference amplifiers gave 1.19 Vrms per leg (−5.1 dB), a ±0.4 dB unit spread and 0.59 mA DC + 0.84 mA peak per pin, and ESS specifies performance only in differential current mode. No paralleling; U401 = L+/L−, U402 = R+/R−, so an adapter that shorts leg− loads only one amplifier per package (TI continuous-short rating, R4) |
| Gain | Fixed, gain = 1; gain switch removed | Volume by DAC digital attenuation, 0 to −127.5 dB in 0.5 dB steps |
| Power input | Single USB-C port (data + 5 V), bus-powered within USB 2.0’s 500 mA | Estimate 0.49 A worst (Section 2) |
| Analog rails | TI LM27762, ±4.5 V | Inverting charge pump + low-noise LDOs: ±1.5 to ±5 V, ±250 mA, 2.7–5.5 V input, 2 MHz, separate EN+/EN−, power-good (R5) |
| Digital and DAC supplies | ME6211C33 (digital 3V3), LP5907-3.3 (DAC analog 3V3A), AMS1117-ADJ (1V3 DVDD) | Closes review finding R1-H (AMS1117 dropout and noise): low-dropout LDO for digital, low-noise LDO for the DAC analog pins |
| Protection | No relays. An LM393 window comparator detects DC on any leg and sets a 3.3 V-domain hardware latch that pulls both OPA1622 EN pins and LM27762 EN+/EN− low | Outputs muted and analog rails off; power-up starts latched and firmware clears the latch. The latch lives on 3V3, so it keeps its state when the ±4.5 V rails turn off |
| Kept | 3.5 mm unbalanced + 4.4 mm balanced jacks; ≥ 384 kHz; JLCPCB PCBA; CI, AI review and measurement automation | Unchanged requirements |
| Dropped | 15 V USB PD power port, CH224A, eFuse, inverting buck-boost, LM317/LM337, relays, gain switch, DC servo, paralleled halves, K1000 1 W target | Cost; the owner accepted lower drive than K1000/Susvara class |

# 2. Power tree and current budget

One USB-C port supplies the whole board. VBUS passes through a 600 Ω ferrite to 5V_SYS, which feeds the LM27762 and two LDOs; the 1.30 V DAC core rail is taken from digital 3V3. Currents are design estimates unless marked as datasheet figures.

## 2.1 Regulation chains

| Chain | Regulator | Rail | Loads |
| --- | --- | --- | --- |
| 5V_SYS → ±4.5 V | TI LM27762 (U501): inverting charge pump + positive LDO from VIN + negative LDO | +4.5 V, −4.5 V | 2 × OPA1678 (I/V converters); 2 × OPA1622; LM393 comparator; FAULT_N pull-up |
| 5V_SYS → 3V3 | Microne ME6211C33 (U502) | 3V3 (digital) | CH32V307, AGRV2KQ32, X201/X202, ES9018K2M DVCC (via ferrite), I²C pull-ups, fault latch, LEDs, AMS1117-ADJ input |
| 5V_SYS → 3V3A | TI LP5907-3.3 (U302) | 3V3A (DAC analog) | ES9018K2M VCCA, AVCC_L, AVCC_R; VREF divider (R431, R432) |
| 3V3 → 1V3 | AMS1117-ADJ (U303), 240 Ω / 10 Ω | 1V3 (1.302 V) | ES9018K2M external DVDD |

## 2.2 Rail budget

| Domain | Current (estimate) | Basis / notes |
| --- | --- | --- |
| Digital: 3V3, 1V3 and 3V3A | ≈ 0.21 A | MCU with HS USB, CPLD, 2 oscillators, DAC including the 50 mA external DVDD. DAC datasheet figures (R3): DVCC 13–14.2 mA; VCCA 0.8 mA; AVCC_L/AVCC_R 8 mA; external DVDD 50 mA measured at 192 kHz, MCLK 80 MHz. The 1V3 divider adds 5.2 mA |
| Analog: ±4.5 V from 5V_SYS | ≈ 0.26 A at full scale into 60 Ω balanced, both channels (≈ 0.13 A into 32 Ω unbalanced), plus the OPA1678 I/V pair: 8 mA quiescent per rail plus the I/V output currents | The charge pump draws ≈ its output current from 5 V; OPA1622 Iq 2.6 mA typ per channel (R4); OPA1678 2 mA per channel |
| 5V_SYS total, worst | ≈ 0.49 A (was 0.47 A before the I/V stage) | vs the 0.50 A USB 2.0 limit: tight, measure at bring-up. Fallback: read the Type-C current advertisement on CC |
| 5V_SYS total, idle | ≈ 0.25 A | — |
| Board dissipation | ≤ ≈ 2.5 W | Worst case, 0.49 A × 5 V; ≤ 0.54 W per OPA1622 package (3.5) |

## 2.3 VBUS capacitance budget

The table lists the capacitance seen directly on VBUS. Capacitance behind the LDOs is charged through their current limit and is not counted; verify the attach inrush at bring-up (3.8).

| Location | Designator | Capacitance |
| --- | --- | --- |
| USB-C connector | C102 | 1 µF |
| LM27762 input | C501 | 4.7 µF |
| ME6211C33 input | C506 | 1 µF |
| LP5907 input | C301 | 1 µF |
| Total on VBUS | — | 7.7 µF ≤ 10 µF |

## 2.4 Power sequencing

The hardware defaults hold the board in a safe state before firmware runs, and the rail structure follows the ES9018K2M recommended power-up order (R3 p. 27). Firmware then brings the analog side up in order (Section 5).

| # | Requirement | Implementation | Source |
| --- | --- | --- | --- |
| 1 | ES9018K2M supply order: VCCA first; AVCC_L/AVCC_R and DVCC at the same time as VCCA or later | VCCA and AVCC_L/AVCC_R (3V3A, LP5907) and DVCC (3V3, ME6211, via FB301) come up together from the two 3.3 V LDOs on 5V_SYS | R3 p. 27 |
| 2 | External DVDD at the same time as DVCC or later | The 1.3 V DVDD (AMS1117-ADJ) is fed from 3V3, so it starts after DVCC | R3 p. 27 |
| 3 | RESETB asserted until ≥ 1 ms after all supplies and MCLK are stable; later resets ≥ 10 ns | RESETB 100 kΩ pull-down holds the DAC in reset; firmware releases it ≥ 1 ms after all rails and MCLK are stable (after USB enumeration), then configures the DAC over I²C | R3 p. 27; Section 5 |
| 4 | Outputs muted and analog rails off from power-up | C605 (100 nF) sets the fault latch at power-up. The latch holds AMP_EN and RAIL_EN at 0 V, so both OPA1622 are disabled and LM27762 EN+/EN− are low. The OPA1622 EN pins also have 100 kΩ pull-downs | Design (3.7) |
| 5 | Analog rails up before the amplifiers are enabled | Firmware clears the latch → LM27762 starts → wait for PG (RAILS_OK_N) + 50 ms → AMP_EN high → soft unmute | R5; Section 5 |
| 6 | Fault state survives the rails turning off | The latch is powered from 3V3, not from ±4.5 V, so it cannot hiccup when RAIL_EN switches the rails off | Design (3.7) |
| 7 | External DVDD shut down in standby | The datasheet says an external DVDD should be shut down in standby; the AMS1117-ADJ has no enable. Open item: confirm behaviour or use an enable-capable LDO | R3; Section 7 |
| 8 | Pop ≤ 5 mV peak | Power-on, power-off (USB unplug), sample-rate change and fault recovery, measured on a scope at 32 Ω and 300 Ω | Spec v0.4; 3.8 |

# 3. Block notes

Entries with a reference are datasheet requirements; all other values are starting values. Designators follow Parts List v0.2.

## 3.1 USB-C input and ESD

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| Connector | J101 HRO TYPE-C-31-M-12 (LCSC C165948), USB 2.0 data + 5 V | Single port. Only the 4.4 mm jack is through-hole (confirm the USB-C shell pegs) |
| CC1, CC2 | 5.1 kΩ Rd to GND on each (R101, R102) | USB-C sink |
| ESD | ST USBLC6-2SC6 (U101) on D+, D− and VBUS, at the connector | — |
| VBUS filter | FB101 Sunlord GZ1608D601TF, 600 Ω (1.3 A, 0.15 Ω), VBUS → 5V_SYS | Keeps switching noise off the cable; JLCPCB Basic to verify |
| VBUS capacitance | C102 1 µF at the connector; total 7.7 µF ≤ 10 µF (2.3) | Verify attach inrush at bring-up |
| Shield | 1 MΩ (R103) and 4.7 nF (C101) from shield to GND | Bleed and RF path |
| USB data | D+/D− to the CH32V307 USB HS pins as a 90 Ω differential pair | Layout per WCH reference (Section 7) |

## 3.2 CH32V307RCT6 (USB MCU)

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| Device | U201, LQFP-64 10 × 10 mm (leaded), 144 MHz RISC-V, 3.3 V from 3V3 | R1; LCSC C2988083 |
| HSE crystal | Y201 8 MHz (frequency per datasheet — to verify), ±20 ppm, 3.2 × 2.5 mm; 20 pF load capacitors (C201, C202) | USB HS PHY reference; confirm against the WCH EVT / reference schematic |
| Decoupling | 100 nF per supply pin (C203–C210, one per VDD/VDDA/VIO pin) + 10 µF bulk (C211) | Place each 100 nF at its pin |
| Reset | NRST RC 10 kΩ / 100 nF (R201, C212) | — |
| Boot | BOOT0 10 kΩ pull-down (R202) | — |
| USB HS | D+/D− routed as a 90 Ω differential pair | Layout per WCH reference (Section 7) |
| Debug | J201 2-wire pads for WCH-LinkE: 3V3, SWDIO, SWCLK, GND | Programming (Section 5) |
| Pin map | Follow the USBAudioToI2S schematic so the firmware runs unmodified first | R7. The unmodified build on this pin map is a gate before layout |
| I²C | SCL/SDA to the DAC; 2.2 kΩ pull-ups to 3V3 (R207, R208) | Pins to assign (Section 4) |

## 3.3 CPLD and clocks

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| CPLD | U202 AGM AGRV2KQ32: 2K LUTs, PLL (clock multiplication), 3.3 V LVCMOS, JTAG/ISP, instant-on non-volatile; QFN-32 4 × 4 mm (leadless) | R2; LCSC C18208899, JLCPCB Extended |
| Decoupling | 4 × 100 nF (C213–C216) | — |
| Oscillators | X201 22.5792 MHz (44.1 kHz family) and X202 24.576 MHz (48 kHz family): 3.3 V CMOS, ±25 ppm, 3.2 × 2.5 mm; 100 nF each (C217, C218) | R7 states both are necessary. LCSC numbers still to assign (Section 7) |
| JTAG | J202 pads: 3V3, TCK, TMS, TDI, TDO, GND | CPLD programming |
| Series termination | 33 Ω (R203–R206) on MCLK, BCLK, LRCLK and SDATA toward the DAC | Source termination |
| DAC MCLK plan | CPLD outputs MCLK = 128 × fs (synchronous; ES9018K2M sync_mode): 5.6448/6.144 MHz at 44.1/48 kHz up to 45.1584/49.152 MHz at 352.8/384 kHz (oscillator × 2 via the CPLD PLL) | A small change to the project’s Verilog — to verify |
| Alternative (DNF) | Dedicated ≥ 74 MHz oscillator for asynchronous mode | MCLK > 192 × 384 kHz = 73.7 MHz |

MCLK per sample rate under the 128 × fs plan (intermediate rates computed from the same rule):

| Sample rate | Oscillator | MCLK = 128 × fs | From the oscillator |
| --- | --- | --- | --- |
| 44.1 kHz | 22.5792 MHz | 5.6448 MHz | ÷ 4 |
| 48 kHz | 24.576 MHz | 6.144 MHz | ÷ 4 |
| 88.2 kHz | 22.5792 MHz | 11.2896 MHz | ÷ 2 |
| 96 kHz | 24.576 MHz | 12.288 MHz | ÷ 2 |
| 176.4 kHz | 22.5792 MHz | 22.5792 MHz | × 1 |
| 192 kHz | 24.576 MHz | 24.576 MHz | × 1 |
| 352.8 kHz | 22.5792 MHz | 45.1584 MHz | × 2 (CPLD PLL) |
| 384 kHz | 24.576 MHz | 49.152 MHz | × 2 (CPLD PLL) |

## 3.4 DAC (ES9018K2M)

| Pin / function | Connection | Requirement (R3) / note |
| --- | --- | --- |
| DVCC | 3V3 through FB301 (600 Ω ferrite) + 100 nF | +1.8 V or +3.3 V ±5 %; 13–14.2 mA |
| VCCA | 3V3A (LP5907-3.3) + 100 nF | +3.3 V ±5 %; 0.8 mA (oscillator) |
| AVCC_L | 3V3A via 1 Ω (R301) + 2.2 µF (C303) + 100 nF | +3.3 V ±5 %; 8 mA. Own branch from the LP5907 |
| AVCC_R | 3V3A via 1 Ω (R302) + 2.2 µF (C304) + 100 nF | As AVCC_L, on a separate branch |
| DVDD | External 1.30 V from AMS1117-ADJ (U303): 240 Ω / 10 Ω (R303, R304) → 1.25 × (1 + 10/240) = 1.302 V; 2.2 µF (C310) + 100 nF at the pin; 10 µF in and out (C311, C312) | External +1.3 V ±5 % must be used for PCM when MCLK > 50 MHz or FSR > 192 kHz (external 50 mA measured at 192 kHz, MCLK 80 MHz). Decouple ≥ 1 µF effective (2.2 µF X5R recommended). The 5.2 mA divider current also meets the AMS1117 minimum load |
| LP5907 output capacitance | 1 µF (C302) + 2 × 2.2 µF (C303, C304) behind 1 Ω + 3 × 100 nF = 5.7 µF | LP5907 COUT 1–10 µF (R6). Design value 2.2 µF for C303/C304 gives 5.7 µF total, under the 10 µF limit |
| MCLK | From the CPLD via 33 Ω | > 192 × FSR (normal mode, FSR ≤ 384 kHz) or = 128 × FSR (synchronous MCLK, sync_mode quick lock, FSR ≤ 384 kHz). Plan: 128 × FSR (3.3) |
| BCLK, LRCLK, SDATA | From the CPLD via 33 Ω; 3.3 V logic | 32-bit I²S (Section 5) |
| I²C | SCL, SDA to the MCU; 2.2 kΩ pull-ups to 3V3 | Address 0x90 or 0x92, set by the ADDR pin |
| ADDR | Strap for 0x90 (R306, 10 kΩ) | Firmware register map uses 0x90 |
| RESETB | 100 kΩ pull-down (R305); driven by an MCU GPIO | DAC held in reset at power-up |
| Exposed pad | DGND | — |
| Outputs | DACL, DACLB, DACR, DACRB in differential current mode: each pin connects only to the inverting input of its own I/V converter (3.5.1), held at VREF = 1.706 V | ESS application circuit (R3 p. 27). Dynamic performance is specified only in differential current mode: DNR 127 dB-A (−60 dBFS) and THD+N −120 dB (0 dBFS) at AVCC = VCCA = DVCC = 3.3 V, internal DVDD, fs 44.1 kHz, MCLK 27 MHz (R3 p. 31) |
| Output model | Each pin is a voltage source swinging 3.05 Vpp (0.924 × AVCC) around 1.65 V (AVCC/2) behind 806 Ω ±11 %. Full-scale current into a virtual ground 3.783 mApp; DC current at virtual-ground voltage Vg = 2.112 − (1000 × Vg)/806 mA, zero at Vg ≈ 1.70 V | R3 p. 31, note *1. Unloaded differential full scale 2.16 Vrms. R3 p. 8 lists gain modulation of the output stage, as the DAC output impedance swings with the signal, as a distortion source (THD compensation registers) |
| Features used | Per-channel volume 0 to −127.5 dB in 0.5 dB steps; 32-bit master trim; THD compensation registers; DPLL lock status (reg 64 / GPIO) | Firmware (Section 5) |

## 3.5 Output stage: I/V converters and legs (2 × OPA1678 + 2 × OPA1622)

Each DAC output pin drives its own I/V converter: one OPA1678 amplifier holds the pin at VREF and turns its current into a voltage, in differential current mode per the ESS application circuit (R3 p. 27). Each leg is one OPA1622 amplifier wired as a 4-resistor difference amplifier fed by two I/V outputs. U403 serves DACL and DACLB, U404 serves DACR and DACRB; U401 carries L+ and L−, U402 carries R+ and R−.

### 3.5.1 I/V converters (U403, U404)

| Item | Value | Basis / notes |
| --- | --- | --- |
| Converters | U403 (DACL, DACLB) and U404 (DACR, DACRB): TI OPA1678IDR dual op-amp on ±4.5 V, one amplifier per DAC pin | LCSC C192421 (10,592 in stock), SOIC-8 leaded; 4.5 nV/√Hz, THD 0.0001 % at 1 kHz, input bias 10 pA, rail-to-rail output within 800 mV at 2 kΩ, ±2.25 to ±18 V, 2 mA per channel |
| Inputs | Inverting input = the DAC pin (its only load, short trace); non-inverting input = VREF | ESS application circuit (R3 p. 27) |
| Feedback | 750 Ω 1 % (R423–R426) in parallel with 10 Ω (R427–R430) in series with 1.5 nF C0G (C417–C420) | After the ESS topology (2.7 kΩ ∥ 10 Ω + 510 pF with LM4562); ≈ 141 kHz |
| VREF | 3V3A × 10.7/(10.0 + 10.7) = 1.706 V from R431 (10.0 kΩ) and R432 (10.7 kΩ), 1 %; one shared node: C421 10 µF at the divider, C422 100 nF between the two OPA1678s (≈ 3 Hz), short traces to all four non-inverting inputs | Sits at the DAC’s zero-DC-current point (≈ 1.70 V) and tracks AVCC; VREF noise and error are common-mode and are removed by the leg difference amplifiers |
| Decoupling | 100 nF at each rail pin: C423–C426 (V+ and V− of U403 and U404) | ±4.5 V from the LM27762 |
| Output swing | VREF ± 1.42 V peak at 0 dBFS: 0.28 … 3.12 V | Inside the OPA1678 swing on ±4.5 V (≈ ±3.7 V at 2 kΩ) |
| Pin current | 3.783 mApp full scale (±1.89 mA peak) per pin; ≈ 0 mA DC with the pin held at VREF | R3 p. 31 source model |
| Supply current | 8 mA quiescent per rail for the pair, plus the I/V output currents | Included in the 0.49 A USB estimate (2.2) |
| Stability | DAC pin capacitance on the inverting inputs with the 750 Ω ∥ (10 Ω + 1.5 nF) feedback | Starting values; check in SPICE with the OPA1678 model (Section 7) |

Design equations (starting values; verify in SPICE and on the bench):

| Quantity | Equation | Value |
| --- | --- | --- |
| DAC pin source (R3 p. 31) | Vsrc = 1.65 V ± 1.525 V peak (3.05 Vpp = 0.924 × AVCC) behind Rdac = 806 Ω ±11 % | 1.078 Vrms per pin at 0 dBFS; unloaded differential 2.16 Vrms |
| Pin current into a virtual ground | Idc = 2.112 − (1000 × Vg)/806 mA; Iac = 3.05 Vpp / 806 Ω | 0 mA DC at Vg ≈ 1.70 V; 3.783 mApp full scale |
| VREF | VREF = 3V3A × R432/(R431 + R432) = 3.3 V × 10.7/20.7 | 1.706 V |
| I/V output | Vout = VREF − (Rf/Rdac) × (Vsrc − VREF), Rf = 750 Ω | ±1.42 V peak around 1.70 V (0.28 … 3.12 V) |
| I/V feedback pole | f = 1/(2π × 750 Ω × 1.5 nF) | ≈ 141 kHz |
| Leg output | Vleg+ = I/V(DAC−) − I/V(DAC+) = (Rf/Rdac) × (Vsrc+ − Vsrc−) | 2 × 750/806 × 1.078 Vrms = 2.01 Vrms per leg; 4.0 Vrms balanced |
| Unit spread | Rdac = 806 Ω ±11 %; level ∝ 1/Rdac | +1.0/−0.9 dB in maximum level |
| Response at 20 kHz | I/V pole 141 kHz and leg pole 169 kHz | −0.15 dB |

### 3.5.2 Output legs (U401, U402)

| Item | Value | Basis / notes |
| --- | --- | --- |
| Resistors | Four 2.00 kΩ 0.1 % 25 ppm thin film per leg (R401–R416, 16 in total); gain = 1 (Rf/Rin = 2 k/2 k) | Matching sets the common-mode rejection; worst 4-resistor mismatch 0.20 % with 0.1 % parts |
| Filter | 470 pF C0G across the feedback and ground-leg resistors (C401–C408) | ≈ 169 kHz with 2 kΩ; with the 141 kHz I/V pole the response is −0.15 dB at 20 kHz |
| Output link | 0 Ω per leg (R417–R420) | Fit 0.47 Ω only if the capacitive-load test needs it |
| Enable | Both EN pins with 100 kΩ pull-downs (R421, R422); AMP_EN from the MCU, forced to 0 V by the latch | EN threshold ≈ 0.82 V referenced to the OPA1622 GND pin (R4) |
| Supplies | ±4.5 V; 100 nF + 10 µF per supply pin (C409–C416); thermal pad to V− copper | — |
| Input common-mode | Leg inputs sit at VREF ≈ 1.706 V (the I/V output DC) → non-inverting input DC ≈ 0.85 V | Inside the OPA1622 range (V−)+1.5 V to (V+)−1 V, i.e. −3.0 V to +3.5 V on ±4.5 V |
| Input mapping and gain | leg+ = I/V(DAC−) − I/V(DAC+); leg− = I/V(DAC+) − I/V(DAC−). Gain per leg = 2 × 750/806 ≈ 1.86, referred to one pin’s source voltage | Each I/V inverts, so the swap keeps leg+ in phase with DAC+ − DAC−. The legs are fed by the low-impedance I/V outputs and do not load the DAC |
| Jack ESD | PESD15VL1BA (D701–D704) on every jack leg | VRWM 15 V, 16 pF |
| Adapter short | A 4.4 mm-to-3.5 mm adapter shorts leg− | One amplifier per package shorted, covered by TI’s continuous-short rating, ≤ 0.6 W; warn in the manual |
| Ground layout | Unbalanced crosstalk −75 dB at 32 Ω needs ≤ 5.7 mΩ shared ground | Layout rule: dedicated sleeve return, ≥ 4 vias, measured at the jack |

### 3.5.3 Jack pinout

| Signal | 4.4 mm 5-pole (J701, G-Switch GT-3321667P-01, through-hole) | 3.5 mm (J702, HRO PJ-332A-6A, TRRS body used with TRS plugs) |
| --- | --- | --- |
| L+ | Tip | Tip |
| L− | Ring 1 | Not connected (leg+ only) |
| R+ | Ring 2 | Ring 1 |
| R− | Ring 3 | Not connected (leg+ only) |
| GND | Sleeve | Sleeve and ring 2 (tie both to GND) |

Both jacks are live; use one headphone at a time.

### 3.5.4 Computed estimates

| Parameter | Estimate | Basis |
| --- | --- | --- |
| Level at 0 dBFS | 2.01 Vrms per leg → 3.5 mm 2.0 Vrms, 4.4 mm 4.0 Vrms | 2 × 750/806 × 1.078 Vrms from the DAC source model (R3 p. 31). Unit-to-unit spread +1.0/−0.9 dB from 806 Ω ±11 %; it only moves the maximum level (volume is digital) and L/R on one die track. Confirm on the bench |
| Swing headroom | 2.84 V peak per leg vs an estimated ±3.3 V usable swing at ≤ 100 mA on ±4.5 V rails → ≈ 1.3 dB | Verify on OPA1622 Fig. 38/39 |
| Frequency response | −0.15 dB at 20 kHz | I/V pole 141 kHz + leg pole 169 kHz |
| Design current | 100 mA peak per leg | Rated power to be measured at 1 kHz, 1 % THD+N, both channels driven |
| OPA1622 dissipation | ≤ 0.54 W per package (16 Ω balanced at the 100 mA limit); ≤ 0.32 W at 60 Ω balanced | Thermal is no longer a concern |
| DC offset, worst case (25 °C, datasheet max) | Unbalanced ≤ 7.4 mV; balanced ≤ 14.7 mV (legs independent) | Common-mode leakage 1.70 V × 0.20 % = 3.4 mV; OPA1622 1.0 mV (plus Ios × 2 kΩ = 0.1 mV); I/V differential (two OPA1678 × 0.5 mV × noise gain 1.93) 1.9 mV; DAC pin-to-pin offset mismatch assumed ≤ 1 mV (assumption, measure) → 0.9 mV. Targets ≤ ±10 mV unbalanced, ≤ ±15 mV balanced; balanced is close to its limit; no servo |
| Noise and dynamic range | ≈ 117.6 dB unweighted and ≈ 119.6 dB-A for the analog stages; system estimate with the DAC ≈ 118–119 dB-A | I/V ≈ 10 nV/√Hz per side, ≈ 18.6 nV/√Hz at each leg → 2.64 µV in 20 kHz vs 2.0 Vrms; DAC 127 dB-A. Target ≥ 115 dB-A, minimum ≥ 112 dB-A |
| Output impedance | &lt; 0.1 Ω plus jack contacts | Legs driven directly (no ballast). Target ≤ 0.5 Ω on both jacks (≤ 1.0 Ω balanced if the 0.47 Ω option is fitted) |
| USB current | ≈ 0.49 A worst; ≈ 0.25 A idle | OPA1678 quiescent 8 mA per rail plus the I/V output currents; tight against the 0.5 A limit, measure at bring-up (Section 7) |

### 3.5.5 Output power at 0 dBFS

| Load | 3.5 mm unbalanced | OPA1622 per package | 4.4 mm balanced | OPA1622 per package |
| --- | --- | --- | --- | --- |
| 16 Ω | 80 mW (100 mA/leg) | 0.25 W | 80 mW (100 mA/leg) | 0.54 W |
| 32 Ω | 125 mW (2.0 Vrms) | 0.18 W | 160 mW (100 mA/leg) | 0.46 W |
| 60 Ω | 67 mW | 0.12 W | 267 mW (4.0 Vrms) | 0.32 W |
| 120 Ω | 33 mW | 0.08 W | 133 mW | 0.18 W |
| 300 Ω | 13 mW | 0.06 W | 53 mW | 0.10 W |
| 600 Ω | 6.7 mW | 0.05 W | 27 mW | 0.07 W |

Design current 100 mA peak per leg; the limiting condition is shown in brackets. SPL: HIFIMAN Susvara (60 Ω, 83 dB/mW) ≈ 0.27 W → ≈ 107 dB SPL; AKG K1000 (120 Ω, 74 dB/mW) ≈ 0.13 W → ≈ 95 dB SPL. The old 1 W K1000 target is dropped.

## 3.6 Power

| Item | Value | Basis / notes |
| --- | --- | --- |
| LM27762 (U501) | ±4.5 V at up to ±250 mA from 5V_SYS: inverting charge pump (2 MHz) + negative LDO (30 mV dropout at 100 mA) + positive LDO from VIN (45 mV dropout at 100 mA) | R5; WSON-12 2 × 3 mm (leadless) |
| Feedback dividers | R501–R504 for +4.5 V and −4.5 V | Values per datasheet — to verify (datasheet equations) |
| Capacitors | 4.7 µF input (C501, part of the VBUS budget); 1 µF flying capacitor (C502); 10 µF on the charge-pump output and LDO outputs (C503–C505) | Output capacitor placement per datasheet — to verify |
| Enable | EN+ and EN− driven only by RAIL_EN from the latch (Q605/Q606) | Rails off whenever the latch is set |
| Power-good | PG (active low) → RAILS_OK_N to the MCU; 100 kΩ pull-up to 3V3 (R505) | Firmware waits for PG + 50 ms before AMP_EN |
| Positive-rail headroom | At VBUS ≥ 4.65 V (4.75 V minus cable drop) the +4.5 V output has ≥ 105 mV headroom vs the 45 mV dropout at 100 mA | The positive LDO runs from VIN. The FB101 drop (0.15 Ω, ≈ 74 mV at 0.49 A) is not included — to verify |
| ME6211C33 (U502) | 3V3 digital; 1 µF in and out (C506, C507) | ≈ 100 mV dropout at 100 mA; JLCPCB Basic (likely), to verify |
| LP5907-3.3 (U302) | 3V3A from 5V_SYS; 1 µF in and out (C301, C302) | COUT 1–10 µF (R6); total output capacitance 5.7 µF (3.4) |
| AMS1117-ADJ (U303) | 1V3 from 3V3; 240 Ω / 10 Ω → 1.302 V; 10 µF in and out (C311, C312) | No enable pin (standby open item); JLCPCB Basic (likely), to verify |
| Layout | Keep the 2 MHz charge pump away from the analog area | Thermal and charge-pump noise coupling (Section 7) |

## 3.7 Protection

| Item | Value | Basis / notes |
| --- | --- | --- |
| DC sensing | Each leg → 100 kΩ (R601–R604) + 1 µF (C601–C604), τ = 100 ms → BAV99 (D601–D604) into a MAX and a MIN node (1 MΩ loads) | One filter per leg |
| Comparator | LM393 (U601) on ±4.5 V: trips when MAX > +0.48 V or MIN < −0.48 V | Thresholds 100 kΩ / 12 kΩ from ±4.5 V: 4.5 × 12/112 = 0.482 V |
| Trip level | ≈ ±0.78–0.88 V DC at a leg | Threshold plus the 0.3–0.4 V diode drop |
| Trip time | ≈ 50–58 ms at 2 V DC; ≈ 19–22 ms for a leg stuck at a rail | Requirement: within 100 ms of ≥ ±2 V DC on any leg |
| No false trip | A full-scale 20 Hz tone is 0.22 V peak after the filter | Below the 0.48 V threshold |
| Level shifter | Open-collector outputs → FAULT_N (47 kΩ, R611, to +4.5 V) → PNP MMBT3906 (Q601) → base of the 3.3 V latch | The collector current flows into a base, so the 3.3 V node cannot exceed 3.3 V or go negative |
| Latch | MMBT3904 + MMBT3906 pair (Q602, Q603) powered from 3V3; base resistors and pull-downs R612–R619 (10–100 kΩ) | Keeps its state when the ±4.5 V rails turn off (no hiccup) |
| Outputs | Two 2N7002 (Q605, Q606) pull AMP_EN (both OPA1622 EN) and RAIL_EN (LM27762 EN+ and EN−) to 0 V | Outputs muted and analog rails off |
| Power-up set | 100 nF (C605) sets the latch at power-up | The board starts latched until firmware clears it |
| Clear | MCU FLT_CLR pulses an MMBT3904 (Q604) | Recovery policy in Section 5 |
| Status | The MCU reads the latch state directly (LATCH, 3.3 V logic) | — |
| Adapter short | A 4.4 mm-to-3.5 mm adapter shorts leg− to GND. The leg sits near 0 V, so no DC trip is expected; the OPA1622 current limit (Isc +145/−130 mA typ) carries it, one amplifier per package | ≤ 0.6 W per package; warn in the manual |

## 3.8 Test and DFT

| Item | Provision | Notes |
| --- | --- | --- |
| Test pads | TP701–TP724, 24 copper pads: every rail (5V_SYS, 3V3, 3V3A, 1V3, +4.5 V, −4.5 V), clocks, I²S, EN/PG/fault nodes, 4 GND | — |
| Debug | J201 CH32V307 2-wire pads (WCH-LinkE); J202 CPLD JTAG pads | Section 5 |
| I²S header | J703 1 × 6, 2.54 mm, DNF: MCLK, BCLK, LRCLK, SDATA, GND, 3V3 reference | Firmware test mode tristates the CPLD I²S outputs. The QA403 digital output is 3.3 V-class (reviewer cites ≈ 3.2 V nominal): use series resistors; power the DUT first, connect the adapter last |
| Indicators | D705 (green, status) and D706 (red, fault) driven directly by 3.3 V GPIO through 1 kΩ (R701, R702) | — |
| Assembly | JLCPCB Economic PCBA, one side; all SMT on top except the 4.4 mm jack; 3 fiducials, 4 × M3 holes; 4 layers, target ≤ 80 × 60 mm | 7 leadless parts per board for X-ray: CPLD, ES9018K2M, 2 × OPA1622, LM27762, 2 oscillators |

Bring-up tests:

| # | Test | Method and pass criterion |
| --- | --- | --- |
| 1 | USB attach inrush | Scope the VBUS current at attach (7.7 µF on VBUS plus the capacitance behind the LDOs, charged through their current limit); record peak and duration |
| 2 | Current at full scale | 5 V current at idle and at full scale into 60 Ω balanced, both channels: ≤ 0.50 A (estimates 0.25 A idle, 0.49 A worst) |
| 3 | Rails and sequencing | 5V_SYS, 3V3, 3V3A, 1V3, ±4.5 V at the test pads; the board starts latched; PG asserts after the latch is cleared |
| 4 | DC-fault injection | Inject DC on each leg: trip within 100 ms at ≥ ±2 V (estimate ≈ 50–58 ms) and ≈ 20 ms for a rail-stuck leg; the latch holds with the rails off; recovery via FLT_CLR |
| 5 | Pop measurement | ≤ 5 mV peak at power-on, power-off (USB unplug), sample-rate change and fault recovery; scope, 32 Ω and 300 Ω |
| 6 | Capacitive-load test | Check each leg for stability with the 0 Ω links, fit 0.47 Ω only if needed; check each I/V output for oscillation with the DAC connected |
| 7 | Level, offset and VREF null point | Confirm the 0 dBFS level (2.01 Vrms per leg computed) and the DC offset (≤ ±10 mV unbalanced, ≤ ±15 mV balanced; worst case 7.4 / 14.7 mV). At idle the DC at each I/V output should equal VREF (≈ 1.706 V), confirming the DAC zero-current point |
| 8 | Thermal | 30 min at full scale into 32 Ω balanced; all parts within ratings |

# 4. Signal plan

Pins marked “per reference project” follow the USBAudioToI2S schematic so the firmware runs unmodified first; the build on that map is the gate before layout. Pins marked “to assign” are added for this board and use pins the reference project leaves free.

| Signal | From → to | Type / level | Pin / notes |
| --- | --- | --- | --- |
| USB D+, D− | J101 ↔ CH32V307 USB HS | USB 2.0 HS | Pin per reference project; 90 Ω pair; USBLC6-2SC6 at the connector |
| MCU ↔ CPLD audio interface | CH32V307 ↔ AGRV2KQ32 | 3.3 V LVCMOS | Pins per reference project |
| Oscillator clocks | X201, X202 → CPLD | 3.3 V CMOS | Pins per reference project |
| I²S MCLK, BCLK, LRCLK, SDATA | CPLD → ES9018K2M via 33 Ω (R203–R206); J703 header | 3.3 V output | Pins per reference project; MCLK = 128 × fs; tristated in test mode |
| I²C SCL, SDA | CH32V307 ↔ ES9018K2M | Open-drain; 2.2 kΩ to 3V3 | To assign; DAC address 0x90 |
| DAC RESETB | CH32V307 → ES9018K2M | 3.3 V output; 100 kΩ pull-down | To assign |
| AMP_EN | CH32V307 → both OPA1622 EN; forced low by the latch (Q605/Q606) | 3.3 V output; 100 kΩ pull-downs | To assign. Drive through a series resistor so the latch FET can override the GPIO (value to set in the schematic) |
| RAIL_EN | Latch → LM27762 EN+ and EN− | 3.3 V, latch only | Not an MCU pin |
| FLT_CLR | CH32V307 → Q604 base | 3.3 V output (pulse) | To assign |
| LATCH (status) | Latch → CH32V307 | 3.3 V input | To assign |
| RAILS_OK_N | LM27762 PG → CH32V307 | Input, active low; 100 kΩ pull-up to 3V3 | To assign |
| LED × 2 | CH32V307 → D705 (status), D706 (fault) via 1 kΩ | 3.3 V output | To assign |
| Debug SWDIO, SWCLK | J201 ↔ CH32V307 | 2-wire debug | Dedicated pins |
| CPLD JTAG TCK, TMS, TDI, TDO | J202 ↔ AGRV2KQ32 | JTAG | Dedicated pins |

# 5. Firmware requirements

The hardware defaults (latch set, EN and RESETB pulled down) hold the board safe until firmware takes over. Firmware then meets the requirements below.

| # | Topic | Required behaviour |
| --- | --- | --- |
| 1 | Reference build | Build USBAudioToI2S unmodified on the reference pin map first (CH32V307: MounRiver Studio / WCH toolchain; CPLD: AGM toolchain). This build is a gate before layout and freezes the pin map |
| 2 | Additions | ES9018K2M I²C driver (32-bit I²S, sync_mode, soft start, volume/mute, lock-status read, optional THD-compensation coefficients); UAC2 volume/mute mapping; sample-rate family switching; protection handling; LEDs |
| 3 | CPLD change | DAC MCLK = 128 × fs (× 2 PLL at 352.8/384 kHz) |
| 4 | Power-up | Latch set, rails off, amps off, DAC RESETB low → USB enumerate → release RESETB ≥ 1 ms after all rails and MCLK are stable (R3 p. 27), configure DAC → clear latch → wait for LM27762 PG + 50 ms → AMP_EN high → soft unmute |
| 5 | Sample-rate family change | Soft mute → CPLD clock switch → DAC relock → unmute |
| 6 | Fault | The hardware latch mutes (amps off, rails off) → firmware soft-mutes the DAC, logs, waits 2 s, clears; if the fault returns within 10 s, retry; after 3 faults stay latched until USB replug; the red LED blinks |
| 7 | Volume and balance | DAC volume 0 to −127.5 dB in 0.5 dB steps; optional channel-balance fine trim to ±0.05 dB by 32-bit digital scaling in the MCU |
| 8 | I²S test mode | Tristate the CPLD I²S outputs and keep the DAC configured over I²C, so an external 3.3 V source can drive the DAC through the J703 header (series resistors; power the DUT first, connect the adapter last) |
| 9 | Programming | CH32V307 via WCH-LinkE 2-wire pads; CPLD via JTAG pads |

# 6. Schematic review checklist

These rules seed the AI datasheet-review agent, and each one can be turned into a planted-error test case. ERC is KiCad’s electrical rules check; a netlist script checks connections, values and counts in the exported netlist; the AI agent reviews rules that need datasheet or part-level interpretation.

| # | Rule | Source | Check method |
| --- | --- | --- | --- |
| 1 | VBUS total capacitance ≤ 10 µF (budget 7.7 µF: C102, C501, C506, C301) | USB; 2.3 | Netlist script |
| 2 | CC1 and CC2 each 5.1 kΩ to GND; USBLC6-2SC6 on D+, D− and VBUS at the connector | USB-C; 3.1 | Netlist script |
| 3 | USB D+/D− in a 90 Ω differential net class | R1 | Netlist script; AI agent (stack-up) |
| 4 | Every IC supply pin decoupled: CH32V307 100 nF per pin + 10 µF; CPLD 4 × 100 nF; each oscillator 100 nF; DAC per 3.4; OPA1622 100 nF + 10 µF per supply pin | R1–R4 | Netlist script |
| 5 | CH32V307 HSE crystal frequency and load capacitors match the WCH reference | R1 | AI agent |
| 6 | CH32V307 and CPLD pin map identical to the USBAudioToI2S schematic | R7 | Netlist script (pin-map diff) |
| 7 | ES9018K2M external DVDD (1.30 V) fitted, as required for PCM with FSR &gt; 192 kHz or MCLK &gt; 50 MHz | R3 | Netlist script; AI agent |
| 8 | DVDD decoupling ≥ 1 µF effective (2.2 µF X5R + 100 nF at the pin) | R3 | AI agent (DC-bias derating) |
| 9 | AVCC_L and AVCC_R on separate branches from the LP5907 (1 Ω + 2.2 µF + 100 nF each) | R3 | Netlist script |
| 10 | LP5907 total output capacitance ≤ 10 µF (design 5.7 µF: C302 1 µF + C303/C304 2 × 2.2 µF + 3 × 100 nF) | R6 | Netlist script |
| 11 | ES9018K2M RESETB 100 kΩ pull-down and MCU drive; ADDR strap for 0x90; I²C pull-ups to 3V3 | R3 | Netlist script |
| 12 | DAC MCLK = 128 × FSR (sync_mode) or &gt; 192 × FSR; 33 Ω series on MCLK, BCLK, LRCLK, SDATA | R3 | AI agent; netlist script |
| 13 | Each DAC output pin (DACL, DACLB, DACR, DACRB) connects only to its I/V inverting input: no other load, short trace | R3 p. 27, 31 | Netlist script (pin fan-out); AI agent (trace length) |
| 14 | VREF comes from 3V3A through R431/R432. Place C421 (10 µF) at the divider and C422 (100 nF) between the two OPA1678s, with short traces to all four non-inverting inputs. VREF is common mode to the output legs, so one shared node is enough | 3.5.1 | Netlist script |
| 15 | 10 Ω + 1.5 nF C0G in series across each 750 Ω I/V feedback resistor (R427–R430, C417–C420) | R3 p. 27; 3.5.1 | Netlist script |
| 16 | OPA1678 decoupling: 100 nF at each rail pin (C423–C426) | 3.5.1 | Netlist script |
| 17 | Leg mapping as specified: leg+ = I/V(DAC−) − I/V(DAC+), leg− = the swap; U403 = DACL/DACLB, U404 = DACR/DACRB | 3.5 | Netlist script |
| 18 | OPA1622 EN pins pulled down (100 kΩ), never floating | R4 | ERC; netlist script |
| 19 | OPA1622 input common-mode inside (V−)+1.5 V … (V+)−1 V (≈ 0.85 V on ±4.5 V) | R4 | AI agent |
| 20 | Four equal 0.1 % resistors in every difference amplifier (R401–R416) | 3.5 | Netlist script |
| 21 | One amplifier per leg; package assignment U401 = L+/L−, U402 = R+/R− | R4 | Netlist script |
| 22 | OPA1622 thermal pad connected to V− | R4 | Netlist script; AI agent (footprint) |
| 23 | Fault latch (Q602, Q603) powered from 3V3, with the power-up set capacitor C605 | 3.7 | Netlist script |
| 24 | No 3.3 V-domain node driven below 0 V or above 3.3 V; the comparator reaches the latch only through the PNP level shifter | 3.7 | AI agent |
| 25 | LM27762 EN+ and EN− driven only by the latch (RAIL_EN); PG pulled up to 3V3 and routed to the MCU; feedback dividers per datasheet | R5; 3.6 | Netlist script; AI agent |
| 26 | ESD diode (PESD15VL1BA) on every jack leg | 3.5 | Netlist script |
| 27 | Test pad on every rail (5V_SYS, 3V3, 3V3A, 1V3, +4.5 V, −4.5 V) | 3.8 | Netlist script |
| 28 | Every fitted part has an LCSC number and stock ≥ the 5-board quantity | CI BOM rule | Netlist script (BOM) |

# 7. Open items and review disposition

Open items are verification tasks with a closing gate. Gates follow the next-step order in Spec v0.4: reference firmware build, DAC output check and SPICE, schematic capture, schematic review, layout, order, bring-up.

## 7.1 Open items

| # | Item | Why it matters | How to close | Gate |
| --- | --- | --- | --- | --- |
| 1 | Firmware from a small open-source project (top risk) | USBAudioToI2S has one contributor and one release | Build it unmodified on the reference pin map; keep that pin map. Fallback: XMOS-based v2 (budget permitting) | Before layout |
| 2 | ES9018K2M output impedance and drive | v0.4 question on how the DAC pins could be loaded | Closed by the datasheet (806 Ω ±11 % per pin, R3 p. 31) and the move to current mode (3.5.1) | Closed |
| 3 | DAC pin-to-pin DC offset mismatch at idle | Assumed ≤ 1 mV (0.9 mV at the leg) in the 7.4 / 14.7 mV offset budget | Measure | Bring-up |
| 4 | External DVDD in standby | The datasheet says an external DVDD should be shut down in standby; the AMS1117-ADJ has no enable | Confirm behaviour, or add an enable-capable LDO | Schematic review |
| 5 | USB current | 0.49 A worst vs the 0.5 A limit (the OPA1678 pair adds ≈ 20 mA) | Measure at full scale; fallback: read the Type-C current advertisement on CC | Bring-up |
| 6 | CH32V307 HSE frequency and USB HS layout | USB HS PHY reference and 90 Ω pair | Per datasheet — to verify (WCH reference / EVT) | Schematic review (HSE); layout (USB) |
| 7 | Oscillator LCSC part numbers; X-ray | X201/X202 still “Assign”; the X-ray fee counts both oscillators (− US$8.20 for the run, ≈ −52 NTD per board, giving ≈ 1,484 NTD per board, if JLCPCB does not X-ray them) | Assign 3225, 3.3 V, 22.5792 / 24.576 MHz parts; ask JLCPCB | Cost gate |
| 8 | “Basic (likely)” parts | ME6211C33, AMS1117-ADJ, LM393, GZ1608D601TF, 8 MHz crystal; an Extended part type costs US$3.07 | Confirm in the JLCPCB library | Cost gate |
| 9 | OPA1622 swing at 100 mA on ±4.5 V | Sets the ≈ 1.3 dB headroom estimate | OPA1622 Fig. 38/39; bench | Schematic review |
| 10 | Both jacks live; adapter shorts leg− | One headphone at a time; current limit, one amplifier per package | Warn in the manual | Bring-up / manual |
| 11 | Thermal and 2 MHz charge-pump noise coupling into the analog area | Noise and heat near the DAC and output stage | Layout separation | Layout |
| 12 | LM27762 feedback resistors and output capacitors | Values not yet taken from the datasheet | Per datasheet — to verify | Schematic capture |
| 13 | LP5907 output capacitance | With 4.7 µF on C303/C304 the nominal count was 10.7 µF vs the 10 µF limit | Closed: C303/C304 set to 2.2 µF; total 5.7 µF | Closed |
| 14 | CPLD MCLK = 128 × fs | Verilog change to the reference project (to verify) | Build and measure MCLK at every rate; the DNF ≥ 74 MHz oscillator is the fallback | Bring-up |
| 15 | +4.5 V headroom including the FB101 drop | ≈ 74 mV across 0.15 Ω at 0.49 A is not in the ≥ 105 mV figure | Measure at minimum VBUS and full load. Fallback: set the LM27762 outputs to ±4.3 V if dropout appears at bring-up (costs about 0.5 dB of output headroom: estimated swing 3.1 V vs 3.3 V peak; headroom at 0 dBFS drops from ≈ 1.3 dB to ≈ 0.8 dB) | Bring-up |
| 16 | VREF null point | VREF (1.706 V) must sit at the DAC’s zero-DC-current point (≈ 1.70 V, R3 p. 31); an error shows up as DC at the I/V outputs | At idle, measure the DC at each I/V output: ≈ VREF confirms the null point | Bring-up |
| 17 | OPA1678 stability with the DAC pin capacitance | The pin capacitance sits at each I/V inverting input with the 750 Ω ∥ (10 Ω + 1.5 nF) feedback | SPICE with the OPA1678 model; check the I/V outputs for oscillation at bring-up | Schematic review |
| 18 | Balanced DC offset close to its limit | Worst case 14.7 mV vs the ±15 mV target (datasheet max, 25 °C) | Measure on every board; record the spread | Bring-up |

## 7.2 Review disposition

Status values: Closed (fixed in v0.4); Closed – N/A (the block no longer exists after the redesign); Open (verification item with owner and gate, Section 7.1).

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| R1-B1 | VBUS_PD ≈ 130 µF vs USB-C 10 µF | No PD port; single 5 V port, VBUS budget 7.7 µF | Closed – N/A |
| R1-B2 | LMR33630 + SMAJ18A surge stacking ≈ 44 V | No inverting converter or 15 V input; LM27762 runs from 5 V | Closed – N/A |
| R1-B3 | LP5907 on 1V8_D over 10 µF | No 1.8 V rails; the only LP5907 (DAC 3.3 V) is budgeted ≤ 10 µF | Closed – N/A |
| R1-B4 | Paralleling open | No paralleling; one amplifier per leg (100 mA design current) | Closed |
| R1-B5 | LM339 at −12 V pulling 1.8 V logic | Comparator on ±4.5 V drives a PNP into a 3.3 V latch; 3.3 V nodes only see transistor/N-FET pull-downs, never below 0 V | Closed |
| R1-B6 | Offset ±5 mV vs 11 mV | Worst case recomputed (current mode: 7.4 / 14.7 mV); targets ±10 / ±15 mV | Closed |
| R1-H | AMS1117 dropout/noise | ME6211 (digital), LP5907 (DAC analog) | Closed |
| R1-H | XU316 VOH vs AK4493 VIH | All logic 3.3 V | Closed – N/A |
| R1-H | Both jacks live / gain-switch pop | Gain switch removed; both jacks live documented (one headphone at a time) | Closed |
| R1-H | OPA1622 stock 197 vs 400 | Sample run needs 10 (2 per board) | Closed |
| R1-H | LM317/LM337 cost trade | LM27762 low-noise LDOs | Closed – N/A |
| R1-H | CI “every part has an LCSC number” vs Digi-Key/Mouser parts | Every part is now an LCSC/JLCPCB part; the rule stands as written | Closed |
| R1-H | Firmware port map before layout | USBAudioToI2S build on the reference pin map before layout (open item 1) | Closed as a gate |
| R1 | Consistency: v0.2 wording, cost source of truth, THD+N conditions, Zout incl. contacts, DSD pins, grounding/ESD/CC/silkscreen | Fixed in Spec v0.4: single USB-C port removes the silkscreen confusion; CC = 5.1 kΩ Rd; jack ESD PESD15VL1BA; grounding rules added; DSD not in v1 | Closed |
| R2-S1 | TVS does not protect −14.7 V converter; no OVP | Block removed | Closed – N/A |
| R2-S2 | VBUS bulk before contract; analog rails start at 5 V | Block removed | Closed – N/A |
| R2-S3 | Balanced Zout with option B | Legs driven directly (< 0.1 Ω + contacts) | Closed |
| R2-S4 | K1000 1 W no margin | Requirement change (budget redesign); ≈ 0.13 W, 95 dB SPL documented | Closed |
| R2-S5 | Offset target contradicts design | As R1-B6 | Closed |
| R2-M1 | Board ≈ 7 W, PLA case | ≤ ≈ 2.5 W board, ≤ 0.54 W per OPA1622 | Closed |
| R2-M2 | LM317 near dropout; LM337 LC filter | Block removed | Closed – N/A |
| R2-M3 | ±0.05 dB trim cannot use 0.5 dB DAC steps | Optional digital scaling in the MCU firmware | Closed |
| R2-M4 | Max power criterion | Rated at 1 kHz, 1 % THD+N, both channels | Closed |
| R2-M5 | Cost margin 10 NTD / exchange rate | Gate uses fixed 31.7 NTD/US$: target 1,500 NTD per board (WARN above), approved ceiling 1,550 (FAIL above); current 1,536 → WARNING | Closed |
| R2-M6 | 4.4-to-3.5 mm adapter shorts leg− | One amplifier per package shorted (TI continuous-short rating), ≤ 0.6 W; risk + manual warning | Closed |
| R2-M7 | Unbalanced crosstalk needs ≤ 6 mΩ ground | Layout rule (≤ 5.7 mΩ) | Closed |
| R2 | Minor: stale versions; KC2520Z “(assumed)”; through-hole count; pop requirement; CH224A/AMS1117 unverified claims; NVIDIA paragraph | Versions fixed; oscillators counted leadless conservatively; only the 4.4 mm jack is through-hole (confirm USB-C shell pegs); pop ≤ 5 mV; CH224A removed; AMS1117-3.3 replaced by ME6211C33 (digital 3V3) and LP5907 (DAC analog); an AMS1117-ADJ is used only for the 1.30 V DAC core supply; job-application text moved to the repository README | Closed |
| R3-1 | DC fault latch and recovery | Hardware 3.3 V latch, firmware recovery policy (Section 5, item 6), status to MCU | Closed |
| R3-2 | TVS/converter envelope | Block removed | Closed – N/A |
| R3-3 | Stage-1 gain 0.9009 with the input network | No stage 1; the gain now includes the DAC’s 806 Ω source (current mode, gain = 2 × Rf/806 per leg) | Closed |
| R3-4 | QA403 cannot drive 1.8 V I²S | I²S is now 3.3 V; header + firmware test mode + sequencing specified | Closed |
| R3-5 | 200 mA “limit” not implemented; thermal at 16 Ω balanced | 100 mA/leg design current, rated power at 1 % THD+N, full load matrix; 0.54 W worst per package | Closed |
| R3 | Consistency: rails-good qualification; offset/trim; workbook external links; cost/sourcing reconciliation | LM27762 PG + latch + 50 ms; offset/trim closed; the delivered v0.1 workbook references only its own sheets (links appear when sheets are copied into another workbook; v0.2 adds a self-check); single source of truth, all LCSC | Closed |

# 8. Chosen parts

Parts for the 5-board sample run, one line per part type; Qty is per board and prices are at the 1–9-piece tier, except the OPA1678 at its 10+ price (the run buys 10 pieces). DAC_HPA_Parts_List_v0.2.xlsx is the source of truth for cost; the figures below are copied from it. “Basic (likely)” and “Assign” entries are still to be confirmed in the JLCPCB library.

| # | Part type | MPN | LCSC no. | JLCPCB type | Qty | Unit US$ (1–9 pcs) | Basis |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Jack | G-Switch GT-3321667P-01 | C2762984 | Extended | 1 | 6.8656 | Quote |
| 2 | Op-amp | TI OPA1622IDRCR | C2876559 | Extended | 2 | 2.6271 | Quote |
| 3 | DAC | ESS ES9018K2M | C2688377 | Extended | 1 | 3.4678 | Quote |
| 4 | MCU | WCH CH32V307RCT6 | C2988083 | Extended | 1 | 3.45 | Quote |
| 5 | Op-amp | TI OPA1678IDR | C192421 | Extended | 2 | 0.97 | Quote |
| 6 | CPLD | AGM AGRV2KQ32 | C18208899 | Extended | 1 | 1.6503 | Quote |
| 7 | Charge pump + low-noise LDOs, ±4.5 V | TI LM27762DSSR | C473398 | Extended | 1 | 1.1014 | Quote |
| 8 | Jack | HRO PJ-332A-6A | C2848643 | Extended | 1 | 0.2921 | Quote |
| 9 | ESD diode | Nexperia PESD15VL1BA,115 | C85378 | Extended | 4 | 0.06 | Quote |
| 10 | USB-C receptacle | HRO TYPE-C-31-M-12 | C165948 | Extended | 1 | 0.1879 | Quote |
| 11 | ESD array | ST USBLC6-2SC6 | C7519 | Extended | 1 | 0.1844 | Quote |
| 12 | LDO, 3.3 V, 250 mA | TI LP5907MFX-3.3/NOPB | Assign | Extended | 1 | 0.1746 | Estimate |
| 13 | LDO, 1.3 V, ≥ 50 mA | AMS1117-ADJ | Assign | Basic (likely) | 1 | 0.08 | Estimate |
| 14 | LDO, 3.3 V, 500 mA | Microne ME6211C33M5G-N | Assign | Basic (likely) | 1 | 0.06 | Estimate |
| 15 | Comparator | LM393DR (any Basic source) | Assign | Basic (likely) | 1 | 0.05 | Estimate |
| 16 | Ferrite bead, 600 Ω | Sunlord GZ1608D601TF | C1015 (verify Basic) | Basic (likely) | 2 | 0.01 | Estimate |
| 17 | Resistor, 2.00 kΩ, 0603, 0.1% 25 ppm thin film | Generic | Assign | Extended | 16 | 0.05 | Estimate |
| 18 | Oscillator, 22.5792 MHz, 3.2 × 2.5 mm, ±25 ppm | Generic | Assign | Extended | 1 | 0.5 | Estimate |
| 19 | Oscillator, 24.576 MHz, 3.2 × 2.5 mm, ±25 ppm | Generic | Assign | Extended | 1 | 0.5 | Estimate |
| 20 | Capacitor, 10 µF, 0805, 10 V X5R | Generic | Assign | Basic (likely) | 11 | 0.02 | Estimate |
| 21 | Capacitor, 100 nF, 0402, 16 V X7R | Generic | Assign | Basic (likely) | 29 | 0.005 | Estimate |
| 22 | Crystal, 8 MHz, 3.2 × 2.5 mm, ±20 ppm | Generic | Assign | Basic (likely) | 1 | 0.1 | Estimate |
| 23 | Capacitor, 470 pF, 0603, 50 V C0G | Generic | Assign | Basic (likely) | 8 | 0.01 | Estimate |
| 24 | Capacitor, 1 µF, 0603, 10 V X7R | Generic | Assign | Basic (likely) | 5 | 0.01 | Estimate |
| 25 | Diode pair, BAV99, SOT-23 | Generic | Assign | Basic (likely) | 4 | 0.012 | Estimate |
| 26 | Capacitor, 1 µF, 0402, 10 V X5R | Generic | Assign | Basic (likely) | 5 | 0.008 | Estimate |
| 27 | Capacitor, 1.5 nF, 0603, 50 V C0G | Generic | Assign | Basic (likely) | 4 | 0.01 | Estimate |
| 28 | NPN + PNP pair (3.3 V fault latch), MMBT3904 / MMBT3906, SOT-23, 40 V | Generic | Assign | Basic (likely) | 2 | 0.02 | Estimate |
| 29 | N-MOSFET, 2N7002, SOT-23, 60 V | Generic | Assign | Basic (likely) | 2 | 0.02 | Estimate |
| 30 | Capacitor, 2.2 µF, 0603, 6.3 V X5R (C303, C304, C310) | Generic | Assign | Basic (likely) | 3 | 0.012 | Estimate |
| 31 | Resistor, 100 kΩ, 0402, 1% | Generic | Assign | Basic (likely) | 8 | 0.004 | Estimate |
| 32 | Resistor, 10–100 kΩ, 0402, 1% | Generic | Assign | Basic (likely) | 8 | 0.004 | Estimate |
| 33 | Resistor, 1 MΩ / 100 kΩ / 12 kΩ, 0402, 1% | Generic | Assign | Basic (likely) | 6 | 0.004 | Estimate |
| 34 | LED, —, 0603 | Generic | Assign | Basic (likely) | 2 | 0.012 | Estimate |
| 35 | PNP, MMBT3906, SOT-23, 40 V | Generic | Assign | Basic (likely) | 1 | 0.02 | Estimate |
| 36 | NPN, MMBT3904, SOT-23, 40 V | Generic | Assign | Basic (likely) | 1 | 0.02 | Estimate |
| 37 | Resistor, 33 Ω, 0402, 1% | Generic | Assign | Basic (likely) | 4 | 0.004 | Estimate |
| 38 | Resistor, 750 Ω, 0603, 1% | Generic | Assign | Basic (likely) | 4 | 0.004 | Estimate |
| 39 | Resistor, 10 Ω, 0402, 1% | Generic | Assign | Basic (likely) | 4 | 0.004 | Estimate |
| 40 | Link, 0 Ω, 0603 | Generic | Assign | Basic (likely) | 4 | 0.004 | Estimate |
| 41 | Resistor, [per datasheet], 0402, 1% | Generic | Assign | Basic (likely) | 4 | 0.004 | Estimate |
| 42 | Capacitor, 4.7 µF, 0603, 10 V X5R (C501) | Generic | Assign | Basic (likely) | 1 | 0.012 | Estimate |
| 43 | Capacitor, 20 pF, 0402, 50 V C0G | Generic | Assign | Basic (likely) | 2 | 0.005 | Estimate |
| 44 | Resistor, 5.1 kΩ, 0402, 1% | Generic | Assign | Basic (likely) | 2 | 0.004 | Estimate |
| 45 | Reset RC (NRST), 10 kΩ / 100 nF, 0402 | Generic | Assign | Basic (likely) | 2 | 0.004 | Estimate |
| 46 | Resistor, 10 kΩ, 0402, 1% | Generic | Assign | Basic (likely) | 2 | 0.004 | Estimate |
| 47 | Resistor, 2.2 kΩ, 0402, 1% | Generic | Assign | Basic (likely) | 2 | 0.004 | Estimate |
| 48 | Resistor, 1 Ω, 0603, 1% | Generic | Assign | Basic (likely) | 2 | 0.004 | Estimate |
| 49 | Resistor, 240 Ω / 10 Ω, 0402, 1% | Generic | Assign | Basic (likely) | 2 | 0.004 | Estimate |
| 50 | Resistor, 10.0 kΩ / 10.7 kΩ, 0402, 1% | Generic | Assign | Basic (likely) | 2 | 0.004 | Estimate |
| 51 | Resistor, 1 kΩ, 0402, 1% | Generic | Assign | Basic (likely) | 2 | 0.004 | Estimate |
| 52 | Capacitor, 4.7 nF, 0402, 50 V X7R | Generic | Assign | Basic (likely) | 1 | 0.005 | Estimate |
| 53 | Resistor, 1 MΩ, 0603, 1% | Generic | Assign | Basic (likely) | 1 | 0.004 | Estimate |
| 54 | Resistor, 47 kΩ, 0402, 1% | Generic | Assign | Basic (likely) | 1 | 0.004 | Estimate |

Cost at this revision (fixed 31.7 NTD per US$): parts US$141.68 (4,491 NTD), five bare 4-layer PCBs US$10 (estimate), JLCPCB fees US$90.63 → US$242.30 = 7,681 NTD = 1,536 NTD per board. Cost gate: target 1,500 NTD per board (WARN above), approved ceiling 1,550 NTD per board (FAIL above; owner decision 24 Sep 2026) → status WARNING, 36 NTD over the target and 14 NTD under the ceiling.

By block (NTD per board, 1–9-piece prices): USB-C input 13; USB bridge 201; DAC 123; I/V converters and output legs 264; power 41; protection 11; outputs and indicators 236 (4.4 mm jack 218); plus US$0.10 minimum spend per passive line. Counts: 81 BOM lines (76 fitted), 54 purchasing lines, 14 Extended SMT + 39 Basic part types, 181 placed parts, 579 SMT joints, 7 leadless parts per board; 100-unit reference ≈ 772 NTD per board. Levers: no X-ray on the oscillators (−52 NTD → ≈ 1,484 NTD per board); a cheaper 4.4 mm jack. Excluded: shipping, import tax, programmers, enclosure.

# Appendix A. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Initial issue: datasheet-derived requirements and starting values for schematic capture; companion to Design Specification v0.2 |
| v0.2 | 23 Sep 2026 | Diego Yang | Remaining parts chosen; CH224A replaces CH224Q; TPS7A4901 replaces LT3045; protection circuit defined (LM339 + 2 × HFD4 relays); open items updated |
| v0.3 | 23 Sep 2026 | Diego Yang | Cost-driven redesign after three design reviews, companion to Design Specification v0.4: CH32V307 + AGRV2KQ32 USB bridge, ES9018K2M DAC, 2 × OPA1622 legs, LM27762 ±4.5 V, relay-less 3.3 V latch protection |
| v0.3 (revised) | 24 Sep 2026 | Diego Yang | Current-mode DAC outputs with OPA1678 I/V converters (option B); estimates, checklist, open items and cost updated |