# USB DAC + Balanced Headphone Amplifier — Design Specification v0.9

Product requirements and architecture for a personal PCB project

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 25 Sep 2026 | v0.9 (layout-ready) | Product requirements + architecture |

Companion documents: Schematic Design Notes v0.8 holds the block-level circuit guidance, and Parts List v0.7 is the source of truth for parts, quantities and cost. This specification states the requirements and architecture they implement.

# 1. Purpose and scope

This specification defines the requirements and architecture of a single USB-C, bus-powered Hi-Res DAC with one ESS ES9018K2M DAC chip and a balanced headphone amplifier. The outputs are a 3.5 mm unbalanced jack and a 4.4 mm balanced jack, for one headphone at a time.

v0.9 is the layout-ready specification. It answers the eighth review round, four external reviews of Spec v0.8 (R8-A…R8-D; Appendix C.1), and four independent design reviews of the resulting v0.9 package: power states and sequencing, signal integrity, and two independent pin-map reviews (Appendix C.2). Every finding has a disposition: fixed or closed, accepted, a documented limitation (L-1…L-11), an owner decision (OD-1, OD-2, both decided on 25 Sep 2026) or rejected with the reason.

What changed: the discrete fault latch is gone. Four Toshiba TLP3545A photorelays, one per leg, now disconnect the headphones, and their LEDs conduct only while the firmware enable RELAY_EN, the running DAC master clock and the hardware permit node H all allow it. Per-leg DC windows and an I/V common-mode window on TLV1704 comparators, plus a new DAC-supply and MCLK supervisor, drive H; the supervisor makes the ESS power order hardware-backed and removes frozen-DAC-code faults within 2.43 ms. The USB input gets a TPS25200 over-voltage clamp (5V_SYS ≤ 5.55 V) and an SMDJ12A TVS; the headphone rails are fed through a TPS259621 eFuse with switched Default/High limits, and the LM27762 enables are tied to its input. The I/V converters get their own hold-up supplies, so a hot unplug no longer takes the DAC pins outside their ratings. The two family oscillators (49.152 / 45.1584 MHz) reach the CPLD through a 2:1 multiplexer and the bit clock is divided without a PLL. The output legs use a T input network, because the v0.8 legs were unstable with 1 nF plus cable. Section 4 lists the circuits removed and Section 9.3 the decisions closed in v0.9.

Goal: a board that works flawlessly and meets every requirement in Section 2. Every requirement is met by worst-case calculation or is a documented limitation (Section 9.1); board 1 is measured against the full bring-up plan and boards 2–5 against the production-test subset (Section 8).

Budget and cost patch: on 25 Sep 2026 the owner decided that the budget is no longer a constraint; the goal is a board that works flawlessly and meets every requirement, and cost is reported for information only (Section 6), with no cost gate and no price ceiling. The same day, after the first v0.9 build, the owner decided on a least-cost patch that loses no requirement: no spare parts (the exact five-board quantity plus JLCPCB attrition), Economic PCBA only if it is the cheaper safe choice (it is not, Section 7), cheaper drop-in parts wherever every requirement is still met worst case, and the special, costly parts hand-soldered by the owner himself: the four photorelays, the 4.4 mm jack, the input TVS and the eFuse current-limit resistor. The patched design was re-verified, and every requirement still holds.

Portfolio context is kept in the project repository.

| In scope for v1 | Out of scope for v1 |
| --- | --- |
| USB-C Hi-Res DAC with a single ES9018K2M DAC chip, bus-powered from one USB-C port (data + 5 V) | DSD in v1 (stretch goal: the ES9018K2M supports DSD; depends on firmware) |
| Balanced headphone amplifier: 4.4 mm balanced jack plus 3.5 mm unbalanced jack (both live, one headphone at a time) | Using both jacks at the same time (one headphone at a time; silkscreen note) |
| PCM 44.1–384 kHz (including 352.8 kHz; all eight rates with exact integer clock ratios), 16–32 bit, USB Audio Class 2.0 High-Speed | K1000-class 1 W drive (v0.9 gives 71.9 mW into the K1000 in High mode, Section 5.2) |
| USB power modes: Default (≤ 500 mA; the analog branch is limited by the TPS259621 eFuse) and High (only with a ≥ 1.5 A Type-C advertisement); USB suspend ≤ 2.5 mA within 10 ms of the last bus activity | USB PD power |
| Output switching: four photorelay output switches (Toshiba TLP3545A), one per leg, open in every power transition and fault | Fast rail-proximity DC detector (v2 option; a rail-stuck failure in High mode stays within the v1 energy limits, Section 5.3) |
| DC-fault protection without a hardware latch: per-leg DC windows, an I/V common-mode window and a DAC-supply/MCLK supervisor on the non-latching permit node H, plus firmware DC tiers, pre-close checks, stream DC guards and a DAC lock-status check; after any trip the firmware keeps the switches open until the full pre-close sequence has passed | Hardware volume knob |
| Host volume in the DAC’s attenuator; per-unit level calibration at the end-of-line test | Enclosure design (3D-printed case for prototypes only) |
| Design automation: automated netlist/BOM consistency, component stress, JLCPCB assembly-class and worst-case calculation checks; automated measurement and calibration (Section 8) | Cost gate or price ceiling (cost is reported for information only, Section 6) |

# 2. Key targets

All values below are the v0.9 targets (the requirement list of Spec v0.8 with the budget removed) and the worst-case design values that meet them, from the v0.9 calculations; values based on typical data or on assumptions are labelled as such. Every board revision is measured against this table (Section 8).

| Parameter | Target | Minimum | Conditions and v0.9 design value | How verified |
| --- | --- | --- | --- | --- |
| PCM sample rate | 44.1–384 kHz, 16–32 bit: all eight rates (44.1, 48, 88.2, 96, 176.4, 192, 352.8 and 384 kHz) with exact integer clock ratios and a 50:50 bit clock | 384 kHz | USB Audio Class 2.0 High-Speed. Bit clock = 64 × fs, divided ÷2/4/8/16 from the 49.152 / 45.1584 MHz family clocks without a PLL: exact ratios and 50.0 % duty by construction; I²S setup / hold margins 14.0 / 16.1 ns at 384 kHz (worst-case calculation, 2.22 ns skew assumption) | Host playback sweep at every rate; bit-perfect word compare through the on-board I²S capture taps at every rate and format |
| DSD | Not in v1 (stretch: the ES9018K2M supports DSD; depends on firmware) | — | — | — |
| Hosts | Windows 10/11 built-in UAC2 driver, macOS, Linux; Android over USB-C (High-Speed) | — | — | Host playback on each OS |
| THD+N at 1 kHz | ≤ −105 dB | ≤ −100 dB | Calibrated 0 dBFS, 300 Ω, both jacks, 20 Hz–20 kHz. Estimate −112.9 dB (typical only; no maxima are published); photorelay contribution ≤ −164 dB (worst-case calculation) | QA403 via Python, at the leg and at the jack (through the switch) |
| Dynamic range | ≥ 115 dB(A) | ≥ 112 dB(A) | Both jacks. Estimate 117.8–119.5 dB(A) (typical only): noise 1.75 µVrms A-weighted unbalanced, 2.83 µVrms balanced | QA403 (results recorded with the analyzer loopback floor) |
| Level at 0 dBFS | 1.55 Vrms per leg after calibration (3.5 mm 1.55 Vrms, 4.4 mm 3.1 Vrms) ± 0.1 dB; −4.5 dB before calibration | — | Untrimmed 1.881 Vrms per leg nominal (1.653–2.165 Vrms: DAC source resistance ±11 %, Rf ±0.1 %, 3V3A ±2 %); master trim −1.68 dB nominal (−2.90…−0.56 dB); default master trim 0x4C3EA838 (−4.5 dB) gives 0.985–1.290 Vrms per leg; per-channel trim to ±0.05 dB (Section 3.3) | Automated end-of-line calibration with a 1 kHz 0 dBFS tone (Section 8) |
| Output power, High mode | 3.5 mm ≥ 50 mW at 32 Ω; 4.4 mm ≥ 105 mW at 60 Ω, ≥ 65 mW at 120 Ω, ≥ 30 mW at 300 Ω | — | Worst unit, both channels, 1 kHz, 1 % THD+N or calibrated 0 dBFS; rated at VBUS ≥ 4.60 V (L-3). Worst case 54.9 / 114.3 / 71.9 / 31.1 mW (margins +4.9 / +9.3 / +6.9 / +1.1 mW); 5V_ANA peaks ≤ 317 mA against the High limit minimum of 393.3 mA (Section 5.2) | QA403 + dummy loads at VBUS 4.40 V and 4.60 V |
| Output power, Default mode | 3.5 mm ≥ 38 mW at 32 Ω; 4.4 mm ≥ 17 mW at 60 Ω, ≥ 35 mW at 120 Ω, ≥ 30 mW at 300 Ω; 32 Ω balanced not rated | — | As High mode, within the TPS259621 Default limit (minimum 177.9 mA): 3.5 mm 32 Ω needs 176.97 mA (+0.97 mA), ≈ 38.6 mW; 4.4 mm 60 / 120 / 300 Ω need ≈ 167.2 / 173.9 / 137.9 mA; 300 Ω is full-scale limited (32.0 mW) (Section 5.2) | QA403 + dummy loads |
| Output impedance | ≤ 0.5 Ω | — | 1 kHz, both jacks, including contacts and the photorelays: 0.251 Ω (3.5 mm) / 0.383 Ω (4.4 mm) at 60 °C, margins 0.249 / 0.117 Ω; photorelay RON 0.081 Ω at 60 °C (datasheet limit 60 mΩ at 25 °C × 1.35, assumption) | Load/no-load method, 32 Ω and 300 Ω |
| Frequency response | +0.1/−0.3 dB, 20 Hz–20 kHz | — | 44.1, 48 and 384 kHz sample rates; estimate −0.067 dB at 20 kHz (typical only) | QA403 |
| Crosstalk at 1 kHz | Balanced ≤ −100 dB (300 Ω); unbalanced ≤ −75 dB (32 Ω, at the jack) | — | 3.5 mm −84 dB at the 2 mΩ shared-return layout target (−75 dB at 5.7 mΩ); switch to switch −166 dB (typical only); layout rule in Section 7 | QA403 |
| Channel balance | ≤ 0.3 dB; per-channel trim to ±0.05 dB at calibration | — | ≤ 0.094 dB after trim, including the photorelay RON mismatch | Automated end-of-line test |
| DC offset | ≤ ±10 mV unbalanced, ≤ ±20 mV balanced, 0–50 °C | — | Worst case 9.23 / 18.45 mV (margins 0.77 / 1.55 mV) with the 0.1 % / 25 ppm leg resistors, guaranteed by a 100 % production screen at 25 °C against ≤ 7.19 / 14.39 mV plus datasheet-bounded drift; a leg that fails the screen gets the Susumu 0.05 % resistors on the same lands (6.24 / 12.48 mV); the VREF trim footprints R446/R447 (not fitted) stay as the designed-in remedy | Automated 25 °C DC screen of every board with a DC-capable DMM (≤ 7.19 / 14.39 mV) |
| Protection, hardware | DC ≥ 2 V on any leg removed within 100 ms and ≥ 1 V within 250 ms, worst case, without firmware; ≤ 30 mJ at 16 Ω and ≤ 15 mJ at 32 Ω for a rail-stuck output (High mode) and for a balanced ±x failure; no false trip for full-scale content ≥ 20 Hz | — | Leg windows trip at ±0.276…±0.320 V behind a 115 kΩ / 2.2 µF filter (τ ≈ 253 ms): 2 V removed in 51.2 ms, 1 V in 111.0 ms; rail-stuck 25.6 / 12.8 mJ (hardware leg windows only; combined single-ended envelope with the CM window and firmware 25.5 / 12.8 mJ), stuck-pin envelope 27.2 / 13.6 mJ, common-mode path ≤ 12.4 / 7.4 mJ; a full-scale 20 Hz square at the aged-capacitor corner stays 6.2 % below the lowest trip (false-trip corners 12.0 Hz sine, 19.6 Hz full-scale square, 18.7 Hz aged) (Section 5.3) | Detector test on each leg with R417–R420 removed, no-trip tests, common-mode window and supervisor tests (Section 8) |
| Protection, supervisor | Frozen DAC code (MCLK dead, open MCLK, DVDD collapse) removed within 3 ms and ≤ 2 mJ | — | 3V3A-good, 1V3-good and MCLK-alive channels: X201 dead while enabled removed in ≤ 2.43 ms, ≤ 1.39 mJ (16 Ω) / ≤ 1.24 mJ (32 Ω); open MCLK ≤ 2.18 ms; DVDD collapse ≤ 2.02 ms; the MCLK-alive detector reads “dead” ≤ 0.452 ms after the clock stops. The DAC lock-status check (owner decision of 25 Sep 2026) adds lock loss ≤ 1.04 ms, a status or register mismatch or I²C no-acknowledge ≤ 16.45 ms and a stuck I²C bus ≤ 17.45 ms, every detected case ≤ 10.1 mJ; a hang that keeps lock and answers I²C with the written registers is limitation L-11 (Section 9.1) | Boot self-test at every attach, including the DAC_LOCK check; slow-ramp threshold tests; production test |
| Protection, firmware | Tiers per leg and per balanced pair remove any DC ≥ 0.2 V across a driver within 3 s, every sub-hardware-trip case ≤ 30 / 15 mJ; residual < 0.2 V accepted by the owner | — | Guaranteed trips (leg / pair): T1 0.995 / 1.000 V within 208 ms, T2 0.311 / 0.306 V within 574 ms, T3 0.176 / 0.192 V within 2.25 s; pre-close ¦leg¦ ≤ 0.070 V, ¦pair¦ ≤ 0.090 V, closure refused above 129 / 155 mV; the stricter stream check (owner decision of 25 Sep 2026) removes full-scale digital DC within 23.6 ms: 13.8 mJ at 32 Ω and 25.3 mJ at 16 Ω balanced, 3.6 / 6.9 mJ single-ended at 32 / 16 Ω, within the limits; content boundary: full-scale sines ≥ 20 Hz never mute, full-scale squares at or below 22.7 Hz and steady offsets ≥ 0.15 FS mute (Section 5.3) | Tier boundary tests with the stream guard bypassed; pre-close tests; stream-check content tests (Section 8) |
| Output switch | Open in every power transition and fault; closed only while all three permits are present | — | Open ≤ 1.008 ms after RELAY_EN low, ≤ 1.010 ms at an MCU reset or IWDG reset, ≤ 1.002 ms after any MCLK stop, ≤ 1.98 ms after a comparator trip, ≤ 1.31 ms at a hardware unplug; hung MCU: open ≤ 40.4 ms, no re-close before 53 ms | Every switch opens and closes (production test, L-6); tON / tOFF at 0 / 50 °C (L-2) |
| Pop | ≤ 5 mV peak for controlled transitions (power-on after configuration, suspend/resume, rate and family change, fault recovery; 32 Ω and 300 Ω); ≤ 20 mV peak at unplug or brown-out (target) | — | Pop = transient beyond the settled DC step, 20 Hz–20 kHz, AC-coupled. Power-up ≤ 4.70 mV at 300 Ω (6 % margin on stacked assumptions; typical ≤ 0.27 mV), ≤ 4.11 mV with C441 fitted; controlled power-down 0.126 mV with the DAC ramp to ground (register 14); rate and family changes run with the switches open; the hot-unplug transient is limitation L-5 | Oscilloscope at the jack contacts, AC-coupled and band-limited, at 300 Ω and 600 Ω |
| USB current | Before configuration ≤ 100 mA; Default ≤ 500 mA; High only with a ≥ 1.5 A Type-C advertisement | — | Worst case per state (Section 5.3): S1 82.40 mA (+17.60 mA), S2 342.28 mA, S3 490.80 mA (+9.20 mA), S4 758.11 mA (limit 1500 mA), S5 350.43 mA; the MCU and CPLD currents are vendor-unpublished and taken as assumptions (L-1) | USB power meter in every state on every board |
| USB suspend | ≤ 2.5 mA within 10 ms of the last bus activity | — | S6 2.4965 mA with a legacy A-to-C cable (+3.5 µA) / 2.4021 mA C-to-C (+97.9 µA), reached ≤ 8.50 ms after the last SOF (margin ≥ 1.50 ms); the MCU Stop current is a planning assumption of 1.9 mA (vendor typical 0.11 mA, maximum unpublished; L-1): the budget holds while it stays ≤ 1.9035 mA, so it is measured on every board | Current against time from the last SOF, every board |
| USB attach | ≤ 50 µC (USB-IF Compliance Update #45 region method); D+ pull-up within 100 ms of VBUS valid (USB 2.0) | — | Region A 28.5 µC, region B 40.2 µC (19.6 % margin; 36.5 µC with the U102 current-limit credit); with R109 470 kΩ and C104 100 nF, U102 switches on 3.31–55.3 ms after attach, so the regions stay ≥ 1 ms apart; the D+ pull-up is enabled ≤ 79.2 ms after VBUS is valid (margin 20.8 ms with an assumed MCU boot ≤ 10 ms) | Charge at attach and D+ timing on board 1 |
| Thermal | All parts within ratings at 50 °C ambient and VBUS up to 5.5 V; the board recovers from 30 min overload tests | — | Worst junctions: TPS7A20 96.0 °C, TLV767 ≈ 88 °C (S5), LM27762 ≤ 115.4 °C (bound); ES9018K2M at 50 °C room plus local rise against TA ≤ 70 °C (measured at bring-up); TPS259621 125 °C only in a Default dead short, which the firmware ends by removing ANA_EN | Thermocouple / IR camera in the 30 min overload tests, including the leg-to-leg short survival test (L-9) |
| PCB | 4 layers, ≤ 80 × 60 mm, all SMT parts on the top side except the 4.4 mm jack, 90 Ω USB pair, JLCPCB rules | — | 80 × 60 mm, FR-4 1.6 mm (JLC04161H-7628), ENIG; exposed-pad thermal vias unfilled and tented; USB pair 90.0 Ω (fabrication corners 80.0–99.3 Ω); JLCPCB Standard PCBA, top side only (USB4105 stakes pin-in-paste); J701, K601–K604, D102 and R509 hand-soldered by the owner after delivery | Automated netlist/BOM and JLCPCB scans; release gates G-1…G-6 (Section 7) |
| Cost | For information only: no cost gate, no ceiling | — | Parts US$ 65.41 (NTD 2,075) per board (JLCPCB-placed 42.66 + owner-fitted 22.75); 5-board order US$ 758.82 = NTD 24,078 (JLCPCB Standard PCBA US$ 597.22 + owner purchase US$ 161.60), i.e. US$ 151.76 = NTD 4,816 per assembled board; US$ 314.05 (29.3 %) below the first v0.9 estimate of US$ 1,072.87 = NTD 34,042 (Section 6) | JLCPCB quote and the owner’s two orders recorded at order |

# 3. Architecture

## 3.1 Signal and control path

Figure 1. Signal and control path: USB-C (USB 2.0 High-Speed, UAC2, our firmware) → CH32V307 MCU → SPI link through the U208 buffer → AGRV2K CPLD (FIFO, I²S master; bit clock divided from the selected 49.152 / 45.1584 MHz family oscillator, no PLL) → ES9018K2M with its own 80 MHz MCLK, gated in hardware by U206 (MCLK_EN ∧ AVCC_EN ∧ MCLK_PERMIT) → 2 × OPA2210 I/V converters → 2 × OPA1622 difference-amplifier legs on +3.85 V / −3.72 V → four TLP3545A photorelays → 4.4 mm balanced and 3.5 mm unbalanced jacks. The MCU also captures the I²S words after the CPLD for the post-CPLD stream guard; the DC and common-mode windows, the supervisor and the MCU DC tiers form the switch permit.

USB and bridge: the single USB-C port (J101, GCT USB4105-GF-A) carries USB 2.0 High-Speed audio and the 5 V bus. D+ and D− pass through a USBLC6-2SC6 inline within 5 mm of J101 (its supply pin on 5V_SYS, behind the clamp); CC1 and CC2 carry the 5.1 kΩ Rd pull-downs, a TPD2E2U06 ESD array within 3 mm and 10 kΩ taps to MCU ADC inputs that classify the Type-C advertisement (Section 3.2). The WCH CH32V307RCT6 (built-in HS PHY, 144 MHz core, 8 MHz crystal) runs our own USB Audio Class 2.0 firmware; 46 GPIO are used and 3 are spare.

Link: the MCU sends audio to the AGM AGRV2KQ32 CPLD over SPI at 36 MHz (16-bit frames with DMA, 70.0 % worst-case utilisation at 384 kHz). There is no MISO data at 36 MHz: the FIFO level comes from LRCLK counts, and register reads run at 9 MHz (+36.6 ns margin). SCK, MOSI and FRAME pass through U208, a TI SN74LVC3G34 buffer with Ioff on 3V3D placed within 5 mm of the CPLD (36 MHz write setup margin +2.03 ns), so the MCU never drives the switched 3V3D domain while it is off. The CPLD holds a 1024 × 32 FIFO and is the I²S master (32-bit Philips format, 64 BCLK per frame). Its configuration time is unpublished, so the firmware polls the ID register (0xA6D2) every 1 ms with a 100 ms timeout and one CPLD_NRST retry before it reports a fault.

Clocks: the family oscillators X202 49.152 MHz and X203 45.1584 MHz (NDK NZ2520SDA, true stand-by with the oscillator stopped, both off by default) reach the CPLD’s only global clock pin through U205, a 74LVC1G157 2:1 multiplexer selected by OSC44_EN, so the two outputs never contend. BCLK = 64 × fs by binary division (÷2/4/8/16) without a CPLD PLL, so all eight rates are exact with a 50.0 % bit clock. The DAC runs in asynchronous mode from its own 80.000 MHz MCLK (Kyocera KC2520K80, X201) on a trace of at most 10 mm. MCLK gate: X201 runs only while U206 (74LVC1G11) output N2_MCLK_OE = MCLK_EN ∧ AVCC_EN ∧ MCLK_PERMIT is high, and the supervisor pulls the AVCC_EN input low while 3V3A is not good. N2_MCLK_OE is also the MCLK permit of the output switch: every MCLK stop opens the switches within 1.002 ms (1.204 ms with U206 unpowered), and at any MCU reset X201 stops within 0.39 µs.

DAC and post-CPLD stream guard: the ES9018K2M is configured over I²C at 100 kHz (address 0x48, 10 kΩ pull-ups). RESETB is driven by Q207, an open-drain AO3400A (PA8 low = DAC released); R240 holds the DAC in reset whenever the MCU is in reset, and R305 10 kΩ pulls RESETB up to DVCC, so nothing drives the DVCC domain while it is off. The MCU captures the I²S words after the CPLD on its I2S2 slave input (PB12, PB13, PB15) for the on-board bit-perfect test and for the post-CPLD stream guard (together with the USB-side guard ≤ 13.3 % CPU at 384 kHz): the audio was clean at USB, so a trigger on the captured words means the CPLD or link is faulty, and the guard opens the output switch at once (≤ 1.008 ms), zeroes the pair and logs it; re-closing needs the full pre-close sequence. The MCU also checks the DAC’s lock status (Section 5.3). At 352.8 and 384 kHz the guard is valid only if the I2S2 slave follows the 22.6 / 24.6 MHz bit clock, which WCH does not publish: a bring-up measurement (Section 8).

Analog chain: each DAC output pin drives one OPA2210 I/V converter (Rf 698 Ω 0.1 % ∥ Cf 820 pF C0G, +IN at VREF). The I/V converters run from their own hold-up supplies (Section 3.2), and the BAT54S clamps D405–D408 sit on the I/V outputs, not on the DAC pins, so nothing loads the virtual ground. Two OPA1622 form four difference-amplifier legs (leg+ = I/V(DAC−) − I/V(DAC+), leg− the swap) with a T input network (1.30 kΩ + 680 Ω with 1.8 nF C0G at the midpoint), 2.00 kΩ ∥ 10 pF feedback and a gain of 1.0101; all 24 leg resistors are 0.1 % / 25 ppm thin film (YAGEO 1.30 kΩ and 680 Ω, Panasonic 2.00 kΩ), with the Susumu RG1608N 0.05 % parts as the rework fallback on the same lands. The 0 Ω links R417–R420 join each amplifier output to its leg and are removed for the detector and failed-output tests. Each leg reaches its jack contact through one TLP3545A photorelay (K601–K604, hand-soldered by the owner; RON ≤ 60 mΩ, tON ≤ 5 ms and tOFF ≤ 0.5 ms at 25 °C); switching all four legs isolates 24 of 24 plug and adapter cases, where switching L+ and R+ only would fail 12 of 24.

Switch permit: per-leg DC windows, the I/V common-mode window and the supervisor channels (1V3 good, MCLK alive, 3V3A repeater) are wired-OR on node H, and the photorelay LED strings conduct only while RELAY_EN, N2_MCLK_OE and H all permit. H is not a latch: it recovers 53–135 ms after the last trip clears, while the firmware keeps RELAY_EN low after any trip until the full pre-close sequence has passed (Section 5.3).

Outputs: J701 (4.4 mm balanced: L+, L−, R+, R−, GND) and J702 (3.5 mm unbalanced: L+, R+, GND) are both live, for one headphone at a time; bidirectional LESD5D5.0 ESD diodes sit within 3 mm of the contacts. The jack pin numbers are provisional until the two-sample continuity test (release gate G-2, Section 7). J701 is hand-soldered by the owner as the last part; J702 is placed by JLCPCB.

## 3.2 Power path

Figure 2. Power path: VBUS → SMDJ12A TVS and TPS25200 input clamp (5V_SYS ≤ 5.55 V; disconnects above its 6.8–8.45 V OVLO) → TPS7A20 3V3M (always on: MCU, supervisor) · TLV767 3V3D (AUD_EN: CPLD, oscillators, DAC DVCC) → TLV758P 1V3 (DAC DVDD) · LP5907 3V3A (AVCC_EN: DAC VCCA/AVCC, VREF, DC-sense networks) · TPS259621 eFuse (ANA_EN; Default 178–220 mA, High 393–487 mA) → LM27762 +3.85 / −3.72 V with its enables tied to its input → 2 × OPA1622, 4 × TLV1704 and, through the I/V hold-up supplies, 2 × OPA2210. BAT54 reverse clamps across the regulators, the 3V3A-good supervisor on 3V3M and the switched discharge of 3V3D, 1V3 and 3V3A.

Input: the single USB-C port supplies data and 5 V; the board is bus-powered. D102, a Littelfuse SMDJ12A TVS at J101 (hand-soldered by the owner before the first USB plug-in), keeps the U102 input ≤ 16.99 V in a hard short behind U102 (absolute maximum 20 V), and C102 4.7 µF absorbs the cable-inductance kick. U102, a TPS25200 over-voltage clamp, holds 5V_SYS at 5.25–5.55 V for any VBUS below its 6.8–8.45 V OVLO and disconnects above it; vSafe5V (≤ 5.50 V) is never disconnected, and 5V_SYS ≤ 5.55 V stays 0.25 V below the lowest downstream absolute maximum (LM27762 VIN 5.8 V). With R109 470 kΩ and C104 100 nF on its enable, U102 switches on 3.31–55.3 ms after attach, which keeps the two USB-IF inrush regions ≥ 1 ms apart, and the D+ pull-up follows ≤ 79.2 ms after VBUS is valid (USB 2.0 limit 100 ms). R108 100 kΩ sets its current limit to 870 / 970 / 1077 mA, above the 758.1 mA High-mode maximum (+112 mA). The 5V_SYS minimum is 4.311 V at VBUS 4.40 V.

Domains and order: before SET_CONFIGURATION only 3V3M runs (S1 ≤ 100 mA). Afterwards the firmware enables ANA_EN (eFuse and rails), then AVCC_EN (3V3A) and, ≥ 1 ms later, AUD_EN (3V3D and 1V3). The supervisor U605 on 3V3M holds AUD_EN and the MCLK gate input low until 3V3A is good (trip 3.006–3.125 V), so the ESS order (VCCA/AVCC before DVCC, DVDD with or after DVCC, MCLK only with VCCA good) is hardware-backed. U303 takes its input and its enable from 3V3D, so DVDD never exceeds DVCC (≤ +5 mV at a hard unplug).

Headphone rails: the TPS259621 eFuse (U503) feeds 5V_ANA with a Default limit of 177.9 / 198.9 / 220.0 mA (R509 4.81 kΩ 0.1 %, hand-soldered by the owner before the first power-up; without it U503 stays off, because an open ILM pin means shutdown) and a High limit of 393.3 / 440.4 / 487.1 mA (R510 3.74 kΩ switched in parallel by the TMUX1101 U505, whose 0.2 nA off-leakage shifts the limit by only 2 ppm). ILIM_HI floats at reset, so the board always starts in Default. The Type-C class is read from 1 ms CC averages (thresholds 0.20 V, 0.66 V and 1.23 V; above 2.3 V invalid, treated as Default); High mode is used only when a ≥ 1.5 A class has been stable for ≥ 100 ms and the switches are open, and a change to a lower class forces a live downgrade within tSinkAdj. The LM27762 (U501) makes VPOS 3.789 / 3.852 / 3.915 V and VNEG −3.661 / −3.721 / −3.781 V (0.1 % dividers); its enables are tied to its input, so the rails start with the eFuse’s dVdt ramp and are good ≤ 18.8 ms after ANA_EN, and the firmware waits T_START = 40 ms. PGOOD reaches the MCU as RAILS_OK_N (valid only with ANA_EN high for ≥ T_START), the eFuse fault flag as ANA_FAULT_N, and ANA_SENSE (5V_ANA_F × 0.3197, R535 100 kΩ / R536 47 kΩ, the same divider as VBUS_SENSE) gives the supply-fault rule: 5V_ANA_F < 4.0 V (1.279 V = 1587 ADC counts) for more than 5 ms after T_START removes ANA_EN in every state.

Hold-up, clamps and discharge: the OPA2210 I/V converters run from N4_VPOS_IV / N4_VNEG_IV, fed from the rails through PMEG2010AEH Schottky diodes (D411/D412) into 220 µF AVX TAJD reservoirs (C442/C443) with BAT54 reverse clamps (D413/D414). At a hard unplug they stay powered and inside their common-mode range until 3V3A < 0.3 V in all 1,944 analysed corners (re-checked with the AVX parts at 176 µF, −20 %), so the DAC pins stay between GND and AVCC. BAT54 clamps D103–D105 across U502, U302 and U503 limit the reverse bias at an input collapse to 0.40–0.46 V for tens of µs (L-8). Every switched domain has a guaranteed discharge FET driven from its enable: 3V3A and both AVCC pins < 0.1 V in ≤ 0.385 ms, 3V3D in ≤ 0.63 ms and 1V3 in ≤ 0.134 ms; a regulator never fights its discharge, and every domain discharges at boot and at any MCU reset.

Brown-out: the MCU watches VBUS_SENSE (5V_SYS × 0.3197, C103 100 pF C0G) with the ADC analog watchdog at 4.00 V (2-sample debounce); its interrupt drives RELAY_EN, AVCC_EN and AUD_EN low 0.049–0.118 ms after the source is lost, before the MCU reset in 96 of 96 corners, and turns MCLK off, holds the DAC in reset and drops ANA_EN after the switches are open.

| Rail | Regulator | Input / enable | Supplies | Discharge |
| --- | --- | --- | --- | --- |
| 5V_SYS | U102 TPS25200 (over-voltage clamp) | VBUS; D102 SMDJ12A at J101 (owner-fitted); EN via R109 470 kΩ + C104 100 nF; ILIM R108 100 kΩ | All regulator inputs; photorelay LED strings | — |
| 3V3M | U502 TPS7A2033 | 5V_SYS; EN tied to IN | MCU, supervisor U605, U505, pull-ups; always on (IQ ≤ 15 µA for suspend) | none (always on) |
| 3V3D | U504 TLV76733 | 5V_SYS; EN = AUD_EN (R508 10 kΩ) | CPLD, oscillators, U206/U208, DAC DVCC, I²C pull-ups | Q505 + R529 10 Ω: ≤ 0.63 ms |
| 1V3 | U303 TLV75801P (R533 / R534 0.1 %) | 3V3D; EN tied to 3V3D | DAC DVDD: 1.2716–1.3248 V (ESS limits 1.235–1.365 V) | Q506 + R530 4.7 Ω: ≤ 0.134 ms |
| 3V3A | U302 LP5907-3.3 | 5V_SYS; EN = AVCC_EN (R303 100 kΩ) | DAC VCCA and AVCC, VREF, DC-sense and reference networks (20.90 mA) | Q503 + R527 10 Ω: ≤ 0.385 ms |
| 5V_ANA | U503 TPS259621 eFuse | 5V_SYS; EN = ANA_EN (R513 100 kΩ); dVdt C514 10 nF | LM27762 through FB501; Default / High limit (R509 owner-fitted) | — |
| VPOS / VNEG | U501 LM27762 | 5V_ANA_F; EN± tied to VIN | OPA1622 legs, comparators, node H: +3.852 / −3.721 V nominal | internal 50 kΩ (datasheet) |
| N4_VPOS_IV / N4_VNEG_IV | D411/D412 PMEG2010AEH + C442/C443 220 µF | VPOS / VNEG | OPA2210 I/V converters (hold-up; D413/D414 reverse clamps) | — |

## 3.3 Volume, gain and level

Gain is fixed. Volume comes from the host through USB Audio Class volume and mute and is applied in the ES9018K2M digital attenuator; the MCU applies the per-channel trim. A hardware knob is out of scope.

| Parameter | Value | Basis |
| --- | --- | --- |
| DAC output | Differential current into each I/V converter; DAC source resistance 806 Ω ±11 % | ES9018K2M datasheet |
| I/V converter | Rf 698 Ω 0.1 % ∥ Cf 820 pF C0G; VREF = 0.50607 × 3V3A (1.634–1.706 V, the middle of the ESS null range 1.65–1.702 V) | R431 9.76 kΩ / R432 10.0 kΩ 0.1 %, C421 33 pF |
| Leg gain | 1.0101 (R2/R1 = 2.00 kΩ / 1.98 kΩ; T input network 1.30 kΩ + 680 Ω with 1.8 nF C0G) | 0.1 % / 25 ppm thin film, all 24 leg resistors (Susumu 0.05 % as the rework fallback) |
| Untrimmed level | 1.881 Vrms per leg nominal, 1.653–2.165 Vrms | DAC source ±11 %, Rf ±0.1 %, 3V3A ±2 % (worst-case calculation) |
| Calibrated level | 1.55 Vrms per leg (3.1 Vrms balanced) at 0 dBFS | Master trim −1.68 dB nominal (−2.90…−0.56 dB); the lowest-gain unit keeps +0.56 dB spare |
| Pre-calibration default | −4.5 dB = master trim 0x4C3EA838 (registers 20…17 = 4C 3E A8 38): 0.985–1.290 Vrms per leg (≤ 1.55 Vrms) | Written at every initialisation until the unit is calibrated |
| Channel trim and balance | Per channel to ±0.05 dB at production calibration (1 kHz 0 dBFS tone at J702 / J701); balance ≤ 0.094 dB after trim | Includes the photorelay RON mismatch (requirement ≤ 0.3 dB) |
| I/V headroom | Calibrated full scale +0.087 V against the 25 °C 600 Ω datasheet swing row (typical +0.441 V); −4.5 dB default +0.280 V; OPA2210 common-mode margin 0.283 V | Measured at bring-up; fallback R501 221 kΩ → 232 kΩ (VPOS +0.13 V, same footprint) |
| Negative-rail floor | 4.4 mm 300 Ω full scale needs ¦VNEG¦ min ≥ 3.625 V; 3.661 V available (+36 mV) | Worst-case calculation |

# 4. Key parts

Unit prices are the indicative prices used in the cost report (Section 6); Parts List v0.7 holds every line. “Buy” is the lean quantity for the five boards: the exact need plus JLCPCB attrition, with no spares (Section 10). Owner-fitted parts are bought by the owner and hand-soldered after delivery (Section 7).

| Part | Role | Key specs | Package / assembly note | Unit price (US$) | Source |
| --- | --- | --- | --- | --- | --- |
| U201 WCH CH32V307RCT6 | USB bridge MCU: UAC2 firmware, sequencing, DC tiers, stream guards | RISC-V, 144 MHz, built-in USB 2.0 HS PHY; 46 GPIO used, 3 spare | LQFP-64 10 × 10; MSL 3 (bake) | 3.47 | LCSC C2988083; single source, buy 6 |
| U202 AGM AGRV2KQ32 | CPLD: 1024 × 32 FIFO, I²S master, clock division | 13 of 24 I/O used; programmed through J202 only | QFN-32 4 × 4, 0.4 mm pitch, exposed pad to GND (gate G-3, X-ray); marking AG32VF in the order remarks | 1.6503 | LCSC C18208899; single source, buy 6 |
| U301 ESS ES9018K2M | DAC, asynchronous mode, differential current output | 32-bit, up to 384 kHz; external 1.3 V DVDD (MCLK 80 MHz > 50 MHz, FSR > 192 kHz) | QFN-28 + exposed pad: solder-mask-defined pad, 5 thermal vias unfilled and tented, 50–70 % windowpane paste; X-ray | 3.4678 | LCSC C2688377; single source, buy 6 |
| X201 Kyocera KC2520K80.0000C1GE00 | 80.000 MHz DAC MCLK | ±50 ppm; ≤ 8.0 mA; stand-by ≤ 5 µA, disable ≤ 200 ns; symmetry 45–55 % (equal to the ESS limit) | 2520 land only; X-ray | 1.39 | Global sourcing; buy 6 (32-week factory lead) |
| X202, X203 NDK NZ2520SDA 49.152 / 45.1584 MHz | Family clocks for the 48 kHz and 44.1 kHz families | ±50 ppm; ≤ 15 mA running, ≤ 35 µA stand-by (output Hi-Z, oscillator stopped); ≤ 4 ms start-up | 2520 land only; X-ray | 2.00 | Global sourcing; buy 6 each |
| U205 74LVC1G157, U206 74LVC1G11, U208 TI SN74LVC3G34DCUR | Family-clock mux, MCLK gate, link buffer | Ioff inputs: safe while 3V3D is off | SOT-363, SOT-363 (6-pin), VSSOP-8 | 0.2106, 0.0768, 0.2012 | LCSC; U208 buy 10 |
| U403, U404 TI OPA2210IDR | I/V converters, one amplifier per DAC pin | Vos 35 µV, IQ 2.5 mA per channel max, input common mode (V−) + 1.5 V to (V+) − 1.5 V; no compliant pin-compatible alternative | SOIC-8; MSL 2 | 2.6708 | LCSC C1849415; buy 11 now (LCSC 16 in stock) |
| U401, U402 TI OPA1622IDRCR | Four difference-amplifier legs | IQ 3.3 mA per channel max (25 °C); EN tied to VPOS | VSON-10 3 × 3, thermal pad on VNEG; X-ray | 1.7871 | LCSC C2876559 |
| R401–R416, R438–R445 YAGEO RT0603BRD, Panasonic ERA-3AEB | Leg networks (1.30 kΩ, 2.00 kΩ, 680 Ω) | 0.1 %, 25 ppm/K thin film; offset worst case 9.23 / 18.45 mV with the 25 °C screen | 0603; Susumu RG1608N 0.05 % as the rework fallback on the same lands | 0.0259, 0.0697, 0.0256 | LCSC C705738, C193242, C861519; buy 60 / 50 / 60 (the 1.30 kΩ pre-ordered) |
| K601–K604 Toshiba TLP3545A(TP1,F) | Output switch, one photorelay per leg | 60 V, 4 A; RON ≤ 60 mΩ at IF 5 mA, 25 °C; tON ≤ 5 ms, tOFF ≤ 0.5 ms; IFT ≤ 3 mA | DIP6 gull-wing SMD; owner-fitted on hand-solder lands, after the protection self-tests | 3.684 | Owner order (DigiKey), 20 |
| U601–U603, U606 TI TLV1704AIPWR | Leg DC windows, common-mode window, supervisor on VPOS/VNEG | VOS ±5.5 mV, IB ≤ 20 nA, rail-to-rail inputs, ≤ 100 µA per channel | TSSOP-14; MSL 2 | 1.2502 | LCSC C181596 if ≥ 25 are in stock at order time, otherwise global sourcing; buy 21 |
| U605 TI TLV3402IDR | 3V3A-good supervisor on 3V3M (two channels) | IQ ≤ 0.95 µA per channel; delay typical only (×3 bound); lifecycle note | SOIC-8 | 0.6771 | LCSC C91242; buy 6 |
| U607 Nexperia 74AUP1G17GW | MCLK-alive detector buffer | Schmitt-trigger input, Ioff, on 3V3D | SOT-353 | 0.10 | Global sourcing; buy 6 |
| U604 TI OPA2333AIDR | Photorelay LED current sinks (two strings) | VOS ≤ 10 µV; IF 5.315–6.599 mA per string | SOIC-8 | 0.6555 | LCSC C38732 |
| U102 TI TPS25200DRVR | USB input over-voltage clamp | 5V_SYS 5.25–5.55 V; OVLO 6.8–8.45 V; IN 20 V absolute maximum; IOS 870 / 970 / 1077 mA (R108 100 kΩ) | WSON-6 2 × 2 + pad; X-ray | 0.44 | LCSC C128401 |
| D102 Littelfuse SMDJ12A | VBUS TVS | VRWM 12 V, 3000 W; U102 input ≤ 16.99 V at a hard short | DO-214AB (SMC); owner-fitted before the first USB plug-in; polarity gate G-4 | 1.01 | Owner order (DigiKey), 5 |
| U503 TI TPS259621DDAR, U505 TI TMUX1101DBVR, R509 KOA RN73R1JTTD4811B25 | Analog-branch eFuse, its Default/High limit switch and the Default limit resistor (4.81 kΩ 0.1 %) | ILIM = 903/R + 11.2 mA, ±10.4 %; FLT only for thermal shutdown or an ILM short; switch off-leakage ≤ 0.2 nA | HSOIC-8 PowerPAD (MSL 2, bake; void check), SOT-23-5, 0603 (R509 owner-fitted before the first power-up) | 1.30, 0.45, 0.10 | LCSC, buy 6 each; R509 owner order (DigiKey), 5; any alternate must also be 4.81 kΩ 0.1 % |
| U501 TI LM27762DSSR | ±3.8 V charge-pump rails | VIN 5.8 V absolute maximum; ±250 mA | WSON-12 2 × 3 + pad; X-ray | 1.0174 | LCSC C473398 |
| U502 TPS7A2033, U504 TLV76733, U302 LP5907-3.3, U303 TLV75801P | 3V3M, 3V3D, 3V3A and 1V3 regulators | IQ ≤ 15 µA; VDO ≤ 0.31 V at 221 mA; 6.5 µVrms; full datasheet limits over temperature | SOT-23-5, WSON-6 (X-ray), SOT-23-5, SOT-23-5 | 0.22, 0.27, 0.2104, 0.18 | LCSC |
| C442, C443 AVX TAJD227K010RNJ; D411, D412 Nexperia PMEG2010AEH | I/V hold-up reservoirs and diodes | 220 µF 10 V tantalum (hold-up re-checked at 176 µF: the I/V supply outlives 3V3A in all 1,944 corners); VF ≤ 0.30 V at ≤ 30 mA | 7343-31 D case with polarity band, SOD-123F | 0.5257, 0.3209 | LCSC C8024, C110921; buy 12 / 11 |
| D405–D408 Nexperia BAT54S,215 | I/V-output clamps | DAC pins held within −0.42 V…AVCC + 0.42 V in faults, ≤ 0.61 mA | SOT-23 | 0.0245 | LCSC C47546 |
| J101 GCT USB4105-GF-A | USB-C receptacle | 48 V, VBUS 5 A; 40 mΩ max per contact; 255 °C / 5 s peak | SMD, shell stakes pin-in-paste; pin map gate G-1 | 1.09 | Global sourcing; buy 6 (plus 2 samples in the owner’s DigiKey order) |
| J701 G-Switch GT-3321667P-01, J702 HRO PJ-332A-6A | 4.4 mm balanced and 3.5 mm jacks | Pin maps provisional until the continuity test (gate G-2) | THT, owner-fitted last (hand-soldered from L4); SMD | 6.906, 0.2921 | J701 owner order (LCSC C2762984), 5; J702 LCSC C2848643 |
| U101 USBLC6-2SC6, U103 TPD2E2U06, D701–D704 LESD5D5.0CT1G | USB data, CC and jack ESD | IRM ≤ 150 nA at 5.25 V; CL ≤ 1.9 pF; bidirectional, VRWM 5 V | SOT-23-6, SOT-5X3, SOD-523 | 0.1844, 0.22, 0.0155 | LCSC |

Circuits removed since v0.8 (the functions no longer exist):

| Removed | Replaced by |
| --- | --- |
| Hardware fault latch, kill FETs, blanking and interlock networks (Q601–Q611, R611–R617, R623, C605–C607; 22 parts) | Four-leg photorelay switch with the non-latching permit node H |
| TPS2553 FAULT to latch path (D605, D606, R638) | eFuse FLT to the MCU only |
| AMP_EN amplifier-enable control (R421 pull-down, R618 kill resistor) | OPA1622 EN tied to VPOS |
| DNF VBUS TVS D102 (SMF6.0A) | U102 TPS25200 clamp and the fitted SMDJ12A (designator D102 re-used) |
| 3V3D dropper R519/R520, constant bleeders R524/R525, enable series resistors R307/R521 | TLV76733 dissipates ≤ 0.5 W itself; switched, guaranteed discharge |
| Q501 2N7002 current-limit switch, R514, LM27762 feedback combinations R515–R518/R523 | U505 TMUX1101; single 0.1 % feedback resistors R501/R503 |
| RAIL_EN control (R506, R507, net RAIL_EN) | LM27762 EN± tied to its input |
| C503/C504 (TLV767 input and output 1 µF) | C501 shared, C512 10 µF |
| I/V parallel Rf/Cf parts (C427–C430, R434–R437), Cf series 10 Ω (R427–R430), VREF option R433, ADDR strap R306 | Single Rf 698 Ω 0.1 % ∥ Cf 820 pF C0G; ADDR tied to GND |
| Shared family-clock node pull-down R214; C216 | U205 2:1 mux; C213–C215 cover the three CPLD supply pins |
| RESETB series resistor R308 (and its successor R233) | Q207 open-drain RESETB driver |
| Threshold series parts R636/R637 | Single E96 0.1 % dividers R607–R610 |
| 3225 oscillator land option | 2520-only lands for X201–X203 |

Parts replaced under the same designator: U503 TPS2553 → TPS259621DDAR; U303 ME6216A13 → TLV75801PDBVR; U502 and U504 ME6211C33 → TPS7A2033 and TLV76733; J101 → GCT USB4105-GF-A; U601/U602 LM339 → TLV1704AIPWR (plus U603 and U606); X201 SCTF SX3M80 → Kyocera KC2520K80; X202/X203 → NDK NZ2520SDA 49.152 / 45.1584 MHz; D405–D408 → Nexperia BAT54S on the I/V outputs; leg resistors → the T network (1.30 kΩ + 680 Ω, 2.00 kΩ); C103 → 100 pF C0G; R305 RESETB pull-down → 10 kΩ pull-up to DVCC; R509/R510 → 4.81 kΩ / 3.74 kΩ 0.1 %; C301/C302 → 2.2 µF 16 V. Compared with Parts List v0.6, 165 designators were added, 63 removed and 106 changed; retired designators are never reused (67 entries, checked automatically). Cost patch of 25 Sep 2026 (same designators, same function, checks re-run): leg resistors → YAGEO RT0603BRD 1.30 kΩ / 680 Ω and Panasonic ERA-3AEB 2.00 kΩ, 0.1 % / 25 ppm (Susumu 0.05 % as the rework fallback); C442/C443 → AVX TAJD227K010RNJ; R641/R642 → YAGEO RT0402BRD0724R9L (24.9 Ω 0.1 %); U208 → TI SN74LVC3G34DCUR; D610 → BAS70-04 (LCSC C389351, the D609 type); R668/R669 → 1.00 MΩ / 100 kΩ 1 %; R108 → 100 kΩ; R109 → 470 kΩ; R535/R536 → 100 kΩ / 47 kΩ; D103–D105 → BAT54,215 (LCSC C85084); R510 → an LCSC-stocked YAGEO part. D102, R509, K601–K604 and J701 are now owner-fitted (D102, R509 and K601–K604 from DigiKey, J701 from LCSC); U605 TLV3402IDR is LCSC C91242, and U607 74AUP1G17GW is not at LCSC (global sourcing).

# 5. Headphone drive and estimates

## 5.1 Load requirements

Current, not voltage, limits the low-impedance loads. In Default mode the eFuse limit of the whole analog branch (minimum 177.9 mA) sets the power; in High mode the 5V_ANA peaks stay ≤ 317 mA, below the 393.3 mA High minimum. The legs are stable with the switch closed into 0–1 nF plus cable and with the switch open.

| Requirement | Value | Basis |
| --- | --- | --- |
| Rated loads | 3.5 mm: 32 Ω. 4.4 mm: 60, 120 and 300 Ω (3.5 mm 16 Ω and 4.4 mm 32 Ω are not rated) | Section 2 |
| Default drive limit | eFuse Default minimum 177.9 mA for the analog branch; analog quiescent 71.61 mA (86.34 mA with the DAC in reset); 3.5 mm 32 Ω needs 176.97 mA (+0.97 mA) | Worst-case calculation |
| High drive limit | 5V_ANA peaks ≤ 317 mA for the rated targets against the High minimum of 393.3 mA | Worst-case calculation |
| Output impedance | 0.251 Ω (3.5 mm), 0.383 Ω (4.4 mm) at 60 °C, including one photorelay per leg | Requirement ≤ 0.5 Ω |
| Leg stability, switch closed | Phase margin ≥ 51.3°, gain margin ≥ 7.7 dB (RON 0.035–0.081 Ω, 5–15 nH, 0–1 nF and cable; 2 nF stress 47.0°) | 37 load cases × 4 OPA1622 models; fallbacks Clead 4.7–22 pF and Ct 1.5–2.2 nF on the same lands |
| Leg stability, switch open | Phase margin ≥ 59.9°, gain margin ≥ 10.1 dB (COFF 0.1–1 nF; 1.5 nF stress 57.7°) | Same analysis |
| I/V stability | Phase margin ≥ 67.5°, gain margin 5.6 dB (pessimistic corner) with the common-mode summing load; closed-loop peaking +2.1…2.8 dB near 1.5 MHz | 1,080 corners |
| Negative-rail headroom | At VBUS 4.60 V all rated targets stay in regulation for a charge-pump resistance R_NEG ≤ 3.66 Ω. At VBUS 4.40 V, High 3.5 mm 32 Ω / 4.4 mm 60 Ω need R_NEG ≤ 2.49 / 2.37 Ω for strict regulation (PGOOD-safe ≤ 3.26 / 3.10 Ω) and do not clip up to ≥ 6.85 Ω; Default 3.5 mm 32 Ω needs ≤ 3.43 Ω | R_NEG 2.5 Ω is typical only (L-3); measured at VIN 4.2 V, 150 mA |
| Leg-to-leg short (4.4 mm plug or cable fault) | An overload, not DC: both amplifiers of one OPA1622 current-limit (≈ 0.55 W per package, TJ ≈ +26 °C); not detected as DC in High mode; the firmware overload policy acts when the rails sag | L-9; bring-up survival test |

## 5.2 Output power and SPL

Rated power is defined at 1 kHz, both channels driven, at 1 % THD+N or calibrated 0 dBFS, whichever comes first, for the worst-case unit with the output series resistance included (0.251 / 0.383 Ω). High mode is rated at the VBUS 4.60 V design corner (R_NEG 3.5 Ω); Default mode at the eFuse Default limit. Worst-case estimates:

| Jack / load | High mode (mW) | Default mode (mW) | Target High / Default (mW) | Status |
| --- | --- | --- | --- | --- |
| 3.5 mm 32 Ω | 54.9 (swing-limited) | ≈ 38.6 at the 177.9 mA minimum limit (36.7 / 40.1 / 47.4 at 175 / 180 / 190 mA) | 50 / 38 | Met: High +4.9 mW; Default ≈ +0.6 mW (needs 176.97 mA, +0.97 mA) |
| 4.4 mm 60 Ω | 114.3 (swing-limited) | Target met at the 177.9 mA minimum limit (needs ≈ 167.2 mA) | 105 / 17 | Met: High +9.3 mW; Default met (≈ 167.2 mA needed ≤ 177.9 mA) |
| 4.4 mm 120 Ω | 71.9 (swing-limited) | Target met at the 177.9 mA minimum limit (needs ≈ 173.9 mA) | 65 / 35 | Met: High +6.9 mW; Default met (≈ 173.9 mA needed ≤ 177.9 mA) |
| 4.4 mm 300 Ω | 32.0 at full scale; guaranteed floor 31.1 | 32.0 at full scale; target met at the 177.9 mA minimum limit (needs ≈ 137.9 mA) | 30 / 30 | Met: +1.1 mW on the guaranteed floor |
| 3.5 mm 16 Ω / 4.4 mm 32 Ω | 67.3 / 97.7 | 22.4 / 12.1 | Not rated | — |

At VBUS 4.40 V the High column becomes 52.9 mW (3.5 mm 32 Ω) and 108.3 mW (4.4 mm 60 Ω) with the typical charge-pump model, so the targets are still met, but only while the LM27762 negative-rail resistance stays near its typical value: limitation L-3 rates High mode at VBUS ≥ 4.60 V and treats 4.40 V as a measured corner. There is no VBUS-dependent volume ceiling, because it would break the 300 Ω target; a firmware headroom-dip rule logs PGOOD dips instead (Section 5.3).

SPL from the maker sensitivity, 4.4 mm balanced at the worst-case power (estimates; SPL = sensitivity + 10·log10(P / 1 mW)):

| Headphone | Impedance, sensitivity | High mode: power, SPL | Default mode |
| --- | --- | --- | --- |
| HIFIMAN Susvara | 60 Ω, 83 dB/mW | 114.3 mW, ≈ 103.6 dB SPL | Meets the 17 mW target at the 177.9 mA minimum limit (needs ≈ 167.2 mA); SPL not estimated |
| AKG K1000 | 120 Ω, 74 dB/mW | 71.9 mW, ≈ 92.6 dB SPL | Meets the 35 mW target at the 177.9 mA minimum limit (needs ≈ 173.9 mA); SPL not estimated |

## 5.3 Power, offset, noise, clocks, thermal and protection

USB current per state, worst case. The MCU (80 mA), CPLD (40 mA) and MCU Stop (1.9 mA) currents are vendor-unpublished planning values (typical MCU 22.4 mA, Stop 0.11 mA); the suspend budget holds while the Stop current stays ≤ 1.9035 mA, and every board is measured in every state at production test (L-1).

| State | Content | Maximum (mA) | Limit (mA) | Margin |
| --- | --- | --- | --- | --- |
| S1 | Attached, unconfigured: 3V3M only, MCU at maximum, LEDs on, ≥ 5.2 ms on the internal oscillator after reset | 82.40 | 100 | +17.60 mA |
| S2 | Configured idle: all domains, analog quiescent, switch closed | 342.28 | 500 | +157.72 mA |
| S3 | Default mode playing, 5V_ANA at the eFuse maximum of 220.0 mA | 490.80 | 500 | +9.20 mA |
| S3f | Family change: both family oscillators running, muted, switch open | 357.62 | 500 | +142.38 mA |
| S4 | High mode (only with a ≥ 1.5 A Type-C advertisement) | 758.11 | 1500 | +741.89 mA |
| S5 | Programming mode, only after SET_CONFIGURATION: rails, 3V3A and 3V3D on, DAC in reset, MCLK off, switch open | 350.43 | 500 | +149.57 mA |
| S6 | Suspend: MCU Stop with an IWDG wake every 0.50–1.20 s (≈ 3.4 µA average) | 2.4965 (legacy A-to-C) / 2.4021 (C-to-C) | 2.5 | +3.5 µA / +97.9 µA |

The 10 kΩ I²C pull-ups lower the 3V3D static load, so the state figures (computed with 4.7 kΩ) are conservative by ≤ 0.75 mA. The cost patch adds no budget: the new ANA_SENSE divider draws 37.8 µA and the TI U208 adds 6 µA on 3V3D, and the other substitutions do not change the budget, so the state table stands. S3 leaves 9.2 mA to 500 mA; any further non-analog load now costs Default power, because no R509 value is left. Suspend: SOF loss (1.0 ms) starts a 1 ms CPLD fade to digital zero and the DAC soft mute; the firmware waits for the minimum-volume flag with a deadline 5.31 ms after the last SOF, then drives RELAY_EN low, runs the power-down order and enters Stop. The suspend current is reached ≤ 8.50 ms after the last SOF at every rate and volume (margin ≥ 1.50 ms), and the switch never opens on live audio.

DC offset, 0–50 °C, worst case. With the fitted 0.1 % / 25 ppm leg resistors it is guaranteed by a 100 % DC screen at 25 °C against ≤ 7.19 / 14.39 mV plus datasheet-bounded drift, because the DAC matching is not published; a leg that fails the screen gets the Susumu 0.05 % resistors on the same lands:

| Term | Unbalanced (mV) | Basis |
| --- | --- | --- |
| Total with the fitted 0.1 % / 25 ppm leg resistors, unbalanced / balanced | 9.23 / 18.45 (limits 10 / 20 mV; margins 0.77 / 1.55 mV) | Worst-case calculation; guaranteed by the 25 °C screen (≤ 7.19 / 14.39 mV) |
| Total with the Susumu 0.05 % rework resistors, unbalanced / balanced | 6.24 / 12.48 (margins 3.76 / 7.52 mV) | Worst-case calculation; terms below |
| Common-mode leakage | 2.56 | Worst-case calculation |
| OPA1622 Vos / Ios | 1.20 / 0.16 | Worst-case calculation |
| OPA2210 | 0.19 | Worst-case calculation |
| Pair-gain mismatch | 1.15 | Assumption: 3 % DAC mismatch |
| DAC null mismatch | 0.98 | Assumption |

Noise, dynamic range, THD+N, response and crosstalk are estimated from typical data only (no maxima are published) and are measured at bring-up: noise 1.75 µVrms A-weighted unbalanced and 2.83 µVrms balanced; dynamic range 117.8–119.5 dB(A); THD+N −112.9 dB at 1 kHz into 300 Ω (photorelay contribution ≤ −164 dB); response −0.067 dB at 20 kHz and within +0.1/−0.3 dB at 44.1, 48 and 384 kHz; crosstalk 3.5 mm −84 dB at the 2 mΩ shared-return layout target (−75 dB at 5.7 mΩ), switch to switch −166 dB.

Clocks, links and converters:

| Item | Value | Margin / note |
| --- | --- | --- |
| Family clocks X202 / X203 | 49.152 / 45.1584 MHz, ±50 ppm; exact integer ratios for all eight rates; 50.0 % BCLK by construction | ≤ 4 ms start-up; at a family change the new oscillator runs ≥ 10 ms before the mux switches |
| DAC MCLK X201 | 80.000 MHz, ±50 ppm; symmetry 45–55 %; tr/tf ≤ 4.5 ns; phase jitter 1.0 ps max (catalogue) | Symmetry has zero margin to the ESS limit: measured at bring-up, fallback R203 → 0 Ω; conditional second source NDK NSC5338D-80M |
| MCLK current | X201 + R227: 8.17 mA against an 8.2 mA budget | Datasheet limit + worst-case calculation |
| I²S | 32-bit Philips, 64 BCLK per frame; setup 14.0 ns, hold 16.1 ns at 384 kHz | Worst-case calculation with a 2.22 ns skew assumption |
| SPI link | Writes at 36 MHz: 70.0 % utilisation at 384 kHz (87.6 µs per 125 µs microframe), 2.7 ns skew bound through the TI U208, setup margin +2.03 ns; reads at 9 MHz, +36.6 ns | BER test of 10¹⁰ bits at bring-up; fallback: falling-edge latch in the CPLD logic |
| ADC | Dual simultaneous mode at 20.000 kHz, sample time 41.5 cycles, 9-slot scan 40.5 µs of the 50 µs period; ADC1 = VBUS_SENSE in all 9 slots (watchdog 4.00 V); ADC2 = CC1, CC2, V3A_MON (dummy, then measured), the four DC-sense legs, ANA_SENSE (trip 1.279 V = 1587 counts for 5V_ANA_F 4.00 V) | R242–R246 330 Ω add +0.42 mV per channel at 41.5 cycles |
| I²C | 100 kHz, 10 kΩ pull-ups: sink ≤ 0.34 mA, rise time ≤ 0.85 µs at 100 pF | Rise time and DAC ACK measured at bring-up (257 / 428 / 856 ns at 30 / 50 / 100 pF) |

Thermal, worst case at 50 °C ambient:

| Part | Worst case | Note |
| --- | --- | --- |
| TPS7A2033 (U502) | 96.0 °C junction | Hottest regulator |
| TLV76733 (U504) | ≈ 88 °C in S5 | Dissipates ≤ 0.5 W itself (no dropper) |
| LM27762 (U501) | ≤ 115.4 °C (bound) | — |
| TLV75801P (U303) | 81.6 °C (122 mW) | Basis of the 1V3 tolerance (TJ ≤ 85 °C) |
| TPS259621 (U503) | 125 °C in a Default dead short (1.24 W) | Fault only: the supply-fault rule removes ANA_EN; a High-mode short goes to thermal shutdown with auto-retry (95 ms) |
| ES9018K2M (U301) | 50 °C room + local rise against TA ≤ 70 °C | Board temperature measured at bring-up |
| AGRV2KQ32 (U202) | ≈ 56 °C (current assumption) against 85 °C | CPLD current measured (L-1) |
| OPA1622 in a leg-to-leg short | ≈ 0.55 W per package, TJ ≈ +26 °C | Survival test (L-9) |

Protection functions. The hardware works without firmware; every limit below is a worst-case value:

| Function | Implementation | Numbers |
| --- | --- | --- |
| Leg DC windows (U601 L+/L−, U602 R+/R−) | LEG_xx → R601–R604 115 kΩ → C601–C604 2.2 µF (τ ≈ 253 ms) → R649–R652 100 kΩ → window comparator; thresholds from single E96 0.1 % dividers on VPOS / VNEG | Trip ±0.298 V nominal, ±0.276…±0.320 V worst case (VOS 5.5 mV, IB 20 nA); input clamp current ≤ 45 µA at rail collapse; aged capacitor range 1.79–2.42 µF analysed |
| I/V common-mode window (U603) | Each I/V pair averaged through 100 kΩ 0.1 % with 10 nF (τ ≈ 0.5 ms); window from 3V3A plus R660 from VPOS | Trips above 2.151 V or below 1.245 V; normal band 1.512–1.816 V, no-false-trip margins 264 / 210 mV; catches a failed I/V amplifier, a stuck DAC pin (≥ 0.98 V per leg at silence), the DAC reset state (CM ≥ 2.889 V) and a missing 3V3A |
| Supervisor: 3V3A good (U605 TLV3402 on 3V3M, two channels) | Q619 pulls the MCLK gate input N2_AVCC_EN_G low, Q620 pulls AUD_EN low; both read back by the MCU | Trip 3.006–3.125 V, release 3.052–3.168 V (66 mV below the lowest operating 3V3A of 3.234 V); gate ≤ 1.04 ms after the threshold |
| Supervisor: 1V3 good, MCLK alive, 3V3A repeater (U606 TLV1704 on VPOS/VNEG, outputs on node H) | MCLK-alive detector: 1 kΩ tap at the DAC XI pin → 74AUP1G17 → 100 pF charge pump → BAS70-04 → 82 pF ∥ 1 MΩ | 1V3 trip 1.103–1.189 V (1V3 ≥ 1.283 V in operation); MCLK running ≥ 0.890 V against a 0.326–0.384 V threshold (2.32×, R668/R669 1.00 MΩ / 100 kΩ 1 %), reads dead ≤ 0.452 ms, fail-safe with 3V3D off; MCLK loading ≤ 1.29 pF |
| Node H and switch drive | Open-collector outputs of U601–U603 and U606 wired-OR, R647 100 kΩ to VPOS, C617 1 µF; Q617 (gate H) and Q616 (gate N2_MCLK_OE) in series with the LED strings; RELAY_EN sets the LED current reference | H ≥ 3.68 V when permitted; trip to 0.45 V ≤ 0.980 ms; recovery 53–135 ms after the last trip clears (no chatter, no latch); IF 5.315–6.599 mA per string; off-state IF ≤ 5 µA, below IFC 10 µA |
| Permit readback | Q618 pulls DC_PERMIT_N low while H permits | “Permitted” 61–205 ms after the last trip clears |

Protection timing and energy (limits 30 mJ at 16 Ω and 15 mJ at 32 Ω):

| Case | Removal (worst case) | Energy 16 Ω / 32 Ω (mJ) | Requirement |
| --- | --- | --- | --- |
| DC 2 V on a leg (hardware) | 51.2 ms | — | ≤ 100 ms |
| DC 1 V on a leg (hardware) | 111.0 ms | — | ≤ 250 ms |
| Full-scale digital DC (hardware windows) | 46.6 ms | — | — |
| Rail-stuck output, single-ended, High mode | Leg windows (hardware only) | 25.6 / 12.8 (15 % margin); combined single-ended envelope with the CM window and firmware 25.5 / 12.8 | 30 / 15 |
| Combined stuck-pin envelope | Leg and CM windows | 27.2 / 13.6 (9 % margin) | 30 / 15 |
| Opposite-sign legs (balanced ±x) | Leg windows | 14.7 / 7.4 | 30 / 15 |
| Common-mode path (stuck DAC pin, failed I/V) | CM window: ≥ 0.98 V per leg in ≤ 3.63 ms | ≤ 12.4 / 7.4 | 30 / 15 |
| I/V output stuck high (2.89 V), worst content | CM window trips at silence | 10.3 / 5.2 | 30 / 15 |
| Frozen DAC code: X201 dead while enabled | ≤ 2.43 ms (supervisor) | ≤ 1.39 / 1.24 | ≤ 3 ms, ≤ 2 mJ |
| Frozen DAC code: open MCLK | ≤ 2.18 ms | ≤ 1.25 / 1.11 | ≤ 3 ms, ≤ 2 mJ |
| Frozen DAC code: DVDD collapse | ≤ 2.02 ms | ≤ 1.16 / 1.03 | ≤ 3 ms, ≤ 2 mJ |
| Full-scale digital DC (content or digital-path fault), stricter stream check | Removed within 23.6 ms (worst case over every DC level and content) | Balanced 25.3 / 13.8; single-ended 6.9 / 3.6 | 30 / 15: met (OD-1, decided 25 Sep 2026) |
| CPLD frozen on a constant word ≥ 0.30 FS (post-CPLD guard) | Switch opened within 23.6 ms | Covered like full-scale digital DC (balanced ≤ 25.3 / 13.8) | 30 / 15: met |
| DAC lock lost while streaming (DAC_LOCK interrupt) | ≤ 1.04 ms | 0.60 / 0.60 (balanced) | 30 / 15: met (OD-2, decided 25 Sep 2026) |
| DAC status or register mismatch (10 ms poll: lock and automute bits, chip ID, DAC_LOCK consistency, registers 0–20) | ≤ 16.45 ms | ≤ 9.5 | 30 / 15: met |
| DAC I²C no-acknowledge (confirmed by one retry) | ≤ 16.45 ms | ≤ 9.5 | 30 / 15: met |
| I²C bus stuck low (1 ms timeout and bus recovery) | ≤ 17.45 ms | ≤ 10.1 | 30 / 15: met |
| DPLL ratio error (100 ms check) | ≤ 102.4 ms | Any DC with it is removed first by the leg windows | — |
| DAC-core hang that keeps lock, answers I²C, returns the written registers and keeps MCLK, 1V3 and 3V3A good | Hardware leg windows only: ±1.51 V for 69.1 ms (16 Ω), ±2.02 V for 50.7 ms (32 Ω) | 39.6 / 25.9 balanced (16.2 at 60 Ω, 3.4 at 300 Ω) | Above the limits: limitation L-11 |

Switch opening: RELAY_EN low ≤ 1.008 ms; MCU reset or IWDG reset ≤ 1.010 ms (R646 pull-down); any MCLK stop ≤ 1.002 ms (1.204 ms with U206 unpowered); node H trip ≤ 1.98 ms after the comparator; hardware unplug ≤ 1.31 ms (the transient during this time is limitation L-5). A hung MCU is reset by the IWDG ≤ 39.4 ms after a trip, so the switch is open ≤ 40.4 ms, while Q617 cannot re-close before 53 ms after the trip clears; there is no autonomous re-close, because RELAY_EN stays latched low after any trip until the full pre-close sequence has passed.

Firmware DC tiers, per leg and per balanced pair, with the stream guard bypassed in the boundary tests:

| Tier | Guaranteed trip, leg / pair | Removed within | Never-trip level, leg / pair |
| --- | --- | --- | --- |
| T1 | 0.995 / 1.000 V | 208 ms | 644 / 465 mV |
| T2 | 0.311 / 0.306 V | 574 ms | 115 / 41 mV |
| T3 | 0.176 / 0.192 V | 2.25 s | 32.3 / 22.1 mV |

The error budget is 60.30 mV per leg and 65.51 mV per pair plus 2.5 % gain; the R242–R246 330 Ω pin resistors raise the pin-side source to 2.819 kΩ (pin leakage 15.42 mV, sampling charge 8.31 mV). The T3 pair threshold is 0.120 V. Boundary tests, both signs, leg / pair: T1 at 1.015 / 1.020 V trips within 208 ms and 579 / 418 mV for 10 s does not; T2 at 0.317 / 0.312 V trips within 574 ms and 104 / 37 mV does not; T3 at 0.180 / 0.196 V trips within 2246 ms and 29 / 20 mV does not. DC below 0.176 / 0.192 V is never removed by firmware: the owner-accepted residual below 0.2 V (≤ 2.30 mW into 16 Ω). The owner rule, ≥ 0.2 V removed within 3 s, is met.

Pre-close checks before every closure: DAC lock and DPLL within ±0.1 %; V3A_MON inside 1.526–1.691 V for ≥ 150 ms; |leg| ≤ 0.070 V and |pair| ≤ 0.090 V (50 ms average; advisory: within 25 mV of the factory baseline, three consecutive failures refuse); DC_PERMIT_N low for ≥ 50 ms; rails good; MCLK_PERMIT = 1 and the U206 readback = 1. Then RELAY_EN goes high (≤ 440.8 ms after SET_CONFIGURATION) and the volume ramps ≥ 10 ms later. With the DAC muted the sensing error is 56.10 mV (leg) / 61.30 mV (pair) plus 2.5 % gain; a healthy board (offset ≤ 9.23 / 18.45 mV, guaranteed by the 25 °C screen) reads ≤ 65.6 / 80.2 mV, margins 4.4 / 9.8 mV (3.7 / 8.2 mV at the 10 / 20 mV offset limits), and closure is guaranteed refused for true DC above 129 mV (leg) / 155 mV (pair).

Stream DC guards: on 25 Sep 2026 the owner decided for a stricter stream check (OD-1). Both guards, the USB-side guard on the decoded audio and the post-CPLD guard on the I²S words the MCU captures, apply four rules per channel at every sample rate: P1, level persistence: every sample of the last 22.0 ms ≥ +0.30 FS, or every sample ≤ −0.30 FS; P2: two cascaded 50 ms averages, trigger ≥ 0.25 FS (was 0.30 FS); P3, flat content: a range ≤ 0.001 FS over 26 ms with |mean| ≥ 0.02 FS; P4: two cascaded 150 ms averages, trigger ≥ 0.15 FS. A USB-side trigger zeroes the channel pair with a 1 ms ramp and logs it; the pair is released after the average stays below 0.05 FS for 1 s. A post-CPLD trigger means the CPLD or link is faulty, because the audio was clean at USB: the output switch opens at once (≤ 1.008 ms), the pair is zeroed and logged, and re-closing needs the full pre-close sequence. A stalled capture (> 2 ms without DMA progress) is a fault. The decision latency is ≤ 1 ms (0.60 ms post-CPLD, 0.225 ms USB side); both guards together use ≤ 13.3 % CPU at 384 kHz and 12.5 KB RAM.

Results and content boundary of the stricter check: worst case over every DC level and programme content, full-scale digital DC is removed within 23.6 ms, with 13.8 mJ at 32 Ω balanced (limit 15 mJ), 25.3 mJ at 16 Ω balanced (limit 30 mJ) and 3.6 / 6.9 mJ single-ended at 32 / 16 Ω (previously 15.5–22.8 mJ at 32 Ω balanced, over the limit); a constant CPLD word ≥ 0.30 FS is covered the same way. The cost the owner accepted is a new no-false-trigger boundary: full-scale sines at any frequency ≥ 20 Hz, any phase or onset, never mute (margins 8 % on P1, 36 % on P2, 65 % on P4); full-scale square waves (≥ 0.30 FS) at or below 22.7 Hz now mute, above 22.7 Hz they play; a steady digital offset ≥ 0.15 FS (−16.5 dBFS) mutes after 0.17–0.3 s, and with a full-scale 20 Hz tone an offset ≥ 0.10 FS may mute, while real programme offsets are < 0.01 FS (−40 dBFS), so music does not mute; flat content ≥ 0.02 FS for 26 ms mutes, as before. The hardware DC windows are unchanged (false-trip corners 12.0 Hz sine, 19.6 Hz full-scale square, 18.7 Hz aged).

DAC lock-status check (OD-2, decided 25 Sep 2026): while streaming, the DAC’s lock output DAC_LOCK, already wired to the MCU, is an interrupt that opens the switch within 1.04 ms (0.60 mJ at 16 / 32 Ω balanced); it is masked during rate and family changes and at start-up, when the switch is open anyway. A status poll every 10 ms (lock and automute bits, chip ID, DAC_LOCK consistency, read-back of the configuration registers 0–20) opens the switch within 16.45 ms (≤ 9.5 mJ), as does a DAC I²C no-acknowledge confirmed by one retry; a stuck-low I²C bus (1 ms timeout and bus recovery) opens it within 17.45 ms (≤ 10.1 mJ); a DPLL ratio check every 100 ms catches a clocking error within 102.4 ms, and any DC with it is removed first by the leg windows. Every detected case stays ≤ 10.1 mJ at 16 and 32 Ω; the I²C bus load is 27.4 %. False alarms: lock loss is expected only during rate or family changes and at start-up (check masked, switch open), a spurious no-acknowledge is confirmed by the retry, every event is logged and counted, and recovery is the controlled restart. A hang that keeps lock and answers I²C with the written registers is limitation L-11 (Section 9.1).

Boot self-test at every attach, before the first closure, with the switches open and the guards bypassed: U206/R234 readback; digital DC of ±0.5 FS on L then R (DC_PERMIT_N “not permitted” 58–97 ms after the DC reaches the legs, pass window 45–120 ms, 95–210 ms before calibration); MCLK alive (0.06–1.28 ms, pass ≤ 2.0 ms; back ≤ 250 ms); 1V3 good (pass ≤ 2.0 ms); 3V3A-good readbacks; and a DAC_LOCK check (2.3 ms: the MCU input and interrupt must follow the DAC’s lock output). It adds ≤ 3.81 s to the first start, plus 2.3 ms for the DAC_LOCK check at every start; any failure means no RELAY_EN, a red LED and an error report over USB.

Fault policy: after a DC, common-mode or clock trip the firmware retries after 2 s through the full pre-close checks, and three trips within 10 s lock the output until replug. Overload (RAILS_OK_N high while ANA_EN has been high for ≥ T_START, or ANA_SENSE below 4.0 V) opens the switches, shuts down in order and lowers the volume ceiling by 3 dB per event down to −12 dB, with a retry after ≥ 1 s; PGOOD dips shorter than 25 ms with ANA_FAULT_N high are headroom events and are only logged. The IWDG timeout is 16.0–38.4 ms whenever RELAY_EN may be high or a switched domain is on (0.667–1.600 s with RTC wakes in suspend only).

Firmware sequencing rules (hardware backing named where it exists):

| State | Rule |
| --- | --- |
| Always | Link outputs low and PA8 high before AUD_EN falls; U208/U206 (Ioff) and Q207 (open drain) cannot drive the 3V3D/DVCC domain while it is off. The supervisor holds AUD_EN and the MCLK gate low until 3V3A is good; the firmware still sequences AVCC_EN, then AUD_EN ≥ 1 ms later. Any MCLK stop opens the switch (hardware). DC_PERMIT_N interrupt at the highest priority, interrupts never masked for > 1 ms; RELAY_EN low ≥ 1.1 ms before any analog-state change (except the brown-out interrupt); DAC volume ramp rate 0 (register 6 = 0x48) from init; DAC ramp to ground (register 14 = 0x03) at every power-down, retried once, otherwise the logged I²C-fault path. |
| Attach (≤ 100 mA) | Reset state from the pull-downs: all enables, RELAY_EN, ILIM_HI and MCLK_PERMIT low, DAC in reset, switch open, X201 stopped, Default limit. ≥ 5.2 ms at low clock; ADC calibration; Type-C classification; the programming strap is latched only here. The D+ pull-up is enabled ≤ 79.2 ms after VBUS is valid (U102 on 3.31–55.3 ms after attach; USB 2.0 limit 100 ms; the largest acceptable 5V_SYS-up delay is 71.1 ms). |
| Power-up after SET_CONFIGURATION | ILIM_HI only with a stable ≥ 1.5 A class and the switch open; ANA_EN → rails good (T_START 40 ms) → AVCC_EN → ≥ 1 ms → AUD_EN; CPLD ID poll; family oscillator; MCLK_EN and MCLK_PERMIT; DAC released ≥ 7 ms after MCLK_EN / MCLK_PERMIT; I²C init with the −4.5 dB default trim; boot self-test; pre-close checks → RELAY_EN → ≥ 10 ms → volume ramp. |
| Playing | Tiers T1–T3; the stricter stream check on both guards (USB side: zero the pair; post-CPLD: open the switch at once); the DAC lock-status check (DAC_LOCK interrupt, 10 ms status and register poll, 100 ms DPLL ratio check); one overload rule; supply-fault rule (5V_ANA_F < 4.0 V, ANA_SENSE 1.279 V = 1587 counts, for > 5 ms after T_START → ANA_EN low; ANA_FAULT_N low → ANA_EN low); live Default↔High downgrade on a lower CC class (mute or limit, ILIM_HI low within tSinkAdj). |
| Rate or family change | Production sequence with the switch open: soft mute → minimum-volume flag (timeout 1.5 × ramp) → RELAY_EN low → 1.1 ms → divider change (family: new oscillator on for ≥ 10 ms, mux, old one to stand-by) → FIFO prime → lock → full pre-close checks → RELAY_EN high → ≥ 10 ms → unmute; worst case 871 ms. MCLK is never stopped for a rate or family change; closed-switch changes only after measurements show ≤ 5 mV. The DAC_LOCK interrupt is masked during the change and at start-up, when the switch is open anyway. |
| Suspend | Fade and mute, then RELAY_EN low, power-down order and Stop; suspend current ≤ 8.50 ms after the last SOF; the switch never opens on live audio. |
| Resume | Stop exit and clocks ≤ 5.28 ms; the short IWDG timeout is restored before any domain is enabled; then the power-up sequence. |
| Brown-out / unplug | Watchdog below 4.00 V (2-sample debounce) → RELAY_EN and AVCC_EN low back to back, AUD_EN in the same write → MCLK off, PA8 high (never before the switch is open), ANA_EN low after 0.53 ms; the I/V hold-up supplies and D405–D408 are the hardware backing. |
| MCU reset / IWDG | Hardware: R646 opens the switch ≤ 1.01 ms, R234 stops X201 ≤ 0.39 µs, R240 and Q207 reset the DAC, the enables fall to their pull-downs and R511 selects the Default limit; a reset while playing is a fault transition (L-4). |
| Programming mode | Only after SET_CONFIGURATION (500 mA granted); until then 3V3M only with a slow red blink. Rails, 3V3A and 3V3D on, CPLD released, MCLK_PERMIT low, DAC in reset, RELAY_EN low; the supply-fault rule stays active (S5 350.43 mA). |

Pop, controlled transitions, 20 kHz band: power-up ≤ 4.70 mV at 300 Ω (6 % margin on stacked assumptions; typical ≤ 0.27 mV; ≤ 4.11 mV with C441 fitted); LED-to-output coupling ≤ 0.42 mV; controlled power-down with the register-14 ramp before RESETB 0.126 mV. The fallback order after a double I²C fault is a logged fault path (≤ 12.0 mV), not a controlled transition. A hot unplug is not a controlled transition either: the transient during the ≤ 1.31 ms opening is a measured target of ≤ 20 mV (L-5).

# 6. Cost (for information)

Cost is reported for information only: the budget is not a constraint, and no requirement was traded for cost. After the first v0.9 build the owner chose a least-cost patch (Section 1): no spares, owner-fitted special parts, cheaper drop-in parts and tented exposed-pad vias, with JLCPCB Standard PCBA kept. Unit prices are indicative distributor prices, and US$ are converted at 31.73 NTD per US$ (an assumption: the rate at payment will differ). The JLCPCB quote and the owner’s two orders replace these figures at order.

| Item | US$ | NTD | Basis |
| --- | --- | --- | --- |
| JLCPCB-placed parts per board (150 lines, 359 parts) | 42.66 | 1,354 | BOM unit prices |
| Owner-fitted parts per board (4 lines, 7 parts) | 22.75 | 722 | Distributor prices |
| All fitted parts per board | 65.41 | 2,075 | First v0.9 estimate US$ 82.02 |
| JLCPCB parts for 5 boards, exact need plus attrition | 248.20 | 7,876 | Exact need US$ 213.29 plus estimated attrition; no spares |
| JLCPCB Standard PCBA fees | 277.00 | 8,789 | Published fee table; includes the handling-fee top-up of US$ 23.56 |
| Global-sourcing extras | 24.02 | 762 | Partly assumptions (price difference, USB4105 inspection) |
| PCB, 5 panels (no epoxy via fill) | 23.00 | 730 | Assumption, not quoted |
| Shipping JLCPCB → Taiwan | 25.00 | 793 | Assumption |
| JLCPCB order (Standard PCBA) | 597.22 | 18,950 | JLCPCB parts, fees, global sourcing, PCB and shipping |
| Owner purchase, two orders: DigiKey 105.63 + LCSC 55.97 (owner-fitted parts and gate samples) | 161.60 | 5,128 | DigiKey including 5 % tax; LCSC including ≈ US$ 10 shipping (assumption) |
| Total per 5-board order | 758.82 | 24,078 | First v0.9 estimate US$ 1,072.87 = NTD 34,042 |
| Equivalent per assembled board (JLCPCB 119.44 + owner 32.32) | 151.76 | 4,816 | Order total / 5 |
| For information: total with Economic PCBA | 743.89 | 23,604 | −14.93 against Standard; not used (Section 7) |

Where the saving comes from: US$ 314.05 per order (NTD 9,965, 29.3 %; US$ 62.81 per assembled board) against the first v0.9 estimate:

| Cost element | First estimate (US$) | Patched (US$) | Change (US$) | Main cause |
| --- | --- | --- | --- | --- |
| JLCPCB parts (with spares or attrition) | 640.32 | 248.20 | −392.12 | No spares; LCSC substitutes (leg resistors, C442/C443, R641/R642, U208, R668/R669, D610); owner-fitted lines moved out |
| Assembly fees (Standard) | 271.90 | 277.00 | +5.10 | 100 placed part types instead of 109 and no THT part lower the Standard base to US$ 253.44, but the handling-fee rule lifts Standard to Economic + 14.93 |
| Global-sourcing extras | 82.65 | 24.02 | −58.63 | No photorelay duty (owner-bought); leg resistors, reservoirs, LED sense resistors, R509/R510 and D102 no longer global-sourced |
| PCB | 53.00 | 23.00 | −30.00 | Epoxy fill and cap of the exposed-pad vias dropped (tented) |
| Shipping | 25.00 | 25.00 | 0.00 | — |
| Owner purchase (two orders) | 0.00 | 161.60 | +161.60 | Owner-fitted K601–K604, J701, D102 and R509 at distributor prices plus the G-1…G-4 gate samples |
| Total | 1,072.87 | 758.82 | −314.05 | Saving 29.3 % |

# 7. Design for JLCPCB (DFF/DFA/DFT rules) and tests

These rules apply to every revision. The five boards are built on JLCPCB Standard PCBA, top side only; the layout engineer works from the layout package in Schematic Design Notes v0.8, summarised here.

| ID | Type | Rule |
| --- | --- | --- |
| R01 | DFA | Standard PCBA, one design, top side only, reflow 240 ± 5 °C; the USB4105 shell stakes are pin-in-paste in the reflow (peak ≤ 245 °C against the connector’s 255 °C / 5 s limit; JLCPCB to confirm the dwell in the order remarks). JLCPCB places 1036 SMT joints and the 4 stakes per board and no THT part. Economic PCBA would be US$ 14.93 cheaper per order but is not used: U202 (QFN-32, 0.40 mm) would sit at Economic’s minimum pitch without solder-paste inspection and cannot be hand-fitted, and J101 (255 °C / 5 s) and J702 (rating not verified) do not fit Economic’s fixed 255 ± 5 °C reflow. |
| R02 | DFA | X-ray inspection of the 10 leadless parts per board: U102, U202, U301, U401, U402, U501, U504, X201, X202 and X203; void check of the U503 PowerPAD. |
| R03 | DFA | MSL and bake: CH32V307 MSL 3; TPS2596, OPA2210 and TLV1704 MSL 2; bake notes for the new parts in the order remarks. Tall-part shadow rule; part rotation checked in the JLCPCB 3D preview (Confirm Parts Placement). |
| R04 | DFA | Buy-now parts go into the JLCPCB private parts library before the order, in the exact five-board quantity plus JLCPCB attrition, with no spares (Section 10); the owner-fitted D102, R509, K601–K604 and J701 are removed from the JLCPCB BOM and placement files (DNP); global-sourced lines carry the verified footprint and the maker drawing; the AG32VF marking of U202 is stated in the order remarks. NC pads are kept. |
| R05 | DFA | Cleaning: the 22 MΩ and 4.7 MΩ supervisor nodes are sensitive to surface leakage (1 nA gives 22 mV at 22 MΩ), so the order remarks ask for no-clean flux residue control and a clean board. |
| R06 | DFF | 4 layers FR-4 1.6 mm, JLC04161H-7628, 1 oz outer / 0.5 oz inner, ENIG, 90 Ω differential impedance control; board 80 × 60 mm with R1 corners and two 5 mm V-cut rails (80 × 70 mm panel); exposed-pad thermal vias 0.30 mm, unfilled and tented on L4 (epoxy fill and cap dropped in the cost patch; the ESS note allows plugging or tenting), with 50–70 % windowpane paste and the X-ray void check kept; every exposed-pad part stays below 125 °C (LM27762 ≤ 90 °C with +20 % θJA assumed for solder wicking); ENIG kept (HASL rejected for planarity on the 0.4–0.5 mm pitch parts). |
| R07 | DFF | Layer use: L1 all parts and every fast or critical signal; L2 solid GND with no splits, slots or traces; L3 power pours by zone and slow signals below 1 MHz; L4 GND pour stitched to L2 (≤ 5 mm grid) plus slow analog and control nets (DC-sense, CC_ADC, VBUS_SENSE, CM nodes, enables), no components, no clocks. |
| R08 | DFF | USB pair: W 0.235 / S 0.15 mm on L1 over L2 = 90.0 Ω (fabrication corners 80.0–99.3 Ω, inside 76.5–103.5 Ω); ≤ 30 mm, no vias, skew ≤ 0.15 mm, U101 inline. |
| R09 | DFF | Critical routing: MCLK X201 → R203 → U301 pin 7 ≤ 10 mm (absolute 20 mm) with a via fence; FAM_CLK ≤ 10 mm; I²S ≤ 25 mm, matched ±5 mm; DAC pins → OPA2210 −IN ≤ 5 mm, no vias, symmetric, Rf/Cf loop < 5 mm²; N2_MCLK_OE ≤ 5 mm, and R645 → Q616 gate (slow) on L3/L4 ≥ 2 mm from clocks and audio; LEG_xx → switch → JACK_xx ≤ 20 mm per net; L and R groups never interleaved (≥ 2 mm plus GND pour); jack ESD diodes ≤ 3 mm from the contacts; TPS2596 ILM ≤ 3 mm; charge-pump flying node ≤ 3 mm with no analog within 10 mm. |
| R10 | DFF | Grounding: one GND net, separation by placement (digital west, power south-west, analog east); headphone returns on a dedicated ≥ 3 mm L1 strip to the output-stage star with ≥ 4 vias per jack GND pin (shared return ≤ 5.7 mΩ for −75 dB, target 2 mΩ for −84 dB); ESD returns through L2 to the jack mounting pins. |
| R11 | DFF | Power distribution: VBUS / 5V_SYS ≥ 1.0 mm; 5V_ANA → FB501 → LM27762 ≥ 0.8 mm; VPOS / VNEG L3 pours ≥ 3 mm; ≥ 2 vias per 0.5 A transition; heat sources (U501, U504, U102, U503) ≥ 15 mm from U301, U202, X201, U601–U603 and J701. |
| R12 | DFF | Placement of the v0.9 parts: D102 ≤ 5 mm from J101; U605 with Q619/Q620 near U206, with 3V3A Kelvin taps at the DAC; MCLK tap R665 on the XI trace ≤ 1.5 mm from U301 pin 7; the U607/C624/D609/C625/R666 group within 8 mm; U606 beside U603; U208 ≤ 5 mm from U202 pins 9–11; Q207 ≤ 3 mm from RESETB; D411/D412/C442/C443 within 10 mm of U403/U404 but outside the DAC → I/V corridor; D103–D105 ≤ 3 mm from their regulators. |
| R13 | DFF | Lands: X201–X203 2520-only; U206 SOT363 6-pin; C442/C443 D case 7343-31 with polarity band; U301 exposed pad solder-mask-defined per the ESS land pattern, 5 thermal vias 0.30 mm, unfilled and tented, 50–70 % windowpane paste; MH1–MH4 (GND) and FID1–FID7 designators. Owner-fitted parts get hand-solder lands (below). |
| R14 | DFT | 52 test pads Ø1.0 mm on L1 (TP701–TP731, TP733–TP753; a GND pad within 5 mm of each clock and rail pad), all netlisted as one-pin pads, among them TP719 N2_AVCC_EN_G, TP746 DAC_RESETB, TP747 N6_MCK_RC, TP748/TP749 the U605 readbacks, TP750/TP751 the I/V hold-up rails, TP752 ANA_SENSE and TP753 VBUS at D102; TP732 is the PROG strap pad. |
| R15 | DFT | Debug pads: J201 for the MCU (3V3M sense only, via R241 0 Ω not fitted, so the WCH-LinkE 3V3 lead is never connected; SWDIO, SWCLK, NRST, GND, BOOT0, USART TX/RX) and J202 for the CPLD (3V3D sense only, JTCK, JTMS, CPLD_NRST, GND; J-Link or AGM DAP-Link); I²S capture taps; J703 I²S header and R703 MCLK monitor not fitted; fiducials 3 global, 2 local at U202 and 2 at U201; 4 × M3 plated holes to GND. |
| R16 | DFT | Firmware test hooks: stream-guard bypass (test mode), tier arming with a fault present, fault injection by removing R417–R420, PROG strap (TP732) for programming mode. |

Release gates: routing starts only after G-1…G-4 have passed, and the order is released only after G-5 and G-6. G-1…G-3 are limitation L-7 until they pass. The owner-fitted parts double as their own samples for these non-destructive checks, made before fitting; the other samples come with the owner’s two orders.

| Gate | Check | When |
| --- | --- | --- |
| G-1 | J101 pin map: two samples continuity-tested and overlaid 1:1 on the footprint | Before routing |
| G-2 | J701 and J702 pin maps, plug in and plug out (two samples each) | Before routing |
| G-3 | 1:1 overlays of every new part (TLP3545A, USB4105, X201 land, TPS2596 DDA, TPD2E2U06 DRL, TLV1704 PW and the other v0.9 parts) and of the U202 exposed pad size and net: the AGM package drawing if AGM or LCSC provide it, otherwise the LCSC footprint (EP 2.7 mm to GND) against a real part; the joint is X-rayed | Before routing |
| G-4 | Polarity: D705/D706 LEDs (DMM diode test of two samples against the tape pocket and body mark), BAT54S, D102, D103–D105, D411/D412, D609, C442/C443 | Before routing |
| G-5 | Netlist and BOM freeze: consistency check clean, stress table and PCBA scan re-run, value-locked parts (Rf, VREF divider and leg resistors, which set the common-mode window) unchanged | After layout |
| G-6 | Order release: order remarks (reflow dwell, cleaning, MSL and bake, AG32VF marking, X-ray list) and the JLCPCB quote | At order |

Tests: before any test the owner fits the owner-fitted parts in the fixed order below; until the photorelays are fitted, the headphone outputs are open. Board 1 runs the full bring-up plan of Section 8; boards 2–5 repeat the production-test subset (rails and currents in every state, eFuse limits and rail start, MCLK, LED string current, hardware protection, firmware tiers, calibration and the 25 °C DC screen at ≤ 7.19 / 14.39 mV, every photorelay opening and closing, and the U606 channel 3 and Q619 wrong-order tests).

Owner-fitted parts: D102, R509, K601–K604 and J701 are not placed by JLCPCB (DNP in the JLCPCB upload); the owner buys them in his two orders and hand-solders them after delivery. None is leadless or needs X-ray; their lands get toes extended by 0.3 mm and ≥ 1.5 mm soldering-iron clearance, and the JLCPCB order then contains no THT part. The fitting order is binding:

| Step | Ref and part (owner source) | Land and clearance | When and why |
| --- | --- | --- | --- |
| 1 | D102 Littelfuse SMDJ12A (DigiKey) | Maker SMC land, toes +0.3 mm; ≥ 1.5 mm iron clearance to the J101 pads and the U102 parts | Before the first USB plug-in: it is the VBUS surge path |
| 1 | R509 KOA RN73R1JTTD4811B25, 4.81 kΩ 0.1 %, 0603 (DigiKey) | 0603 land, toes +0.3 mm; ≥ 1.5 mm to the U503 pin 7 neighbours (R510, U505), short ILM loop | Before the first power-up: it sets the eFuse limit, and U503 must never run with the ILM pin open (without R509 it stays off) |
| 2 | K601–K604 Toshiba TLP3545A(TP1,F) (DigiKey) | Toshiba land, toes +0.3 mm; ≥ 1.5 mm iron clearance around each body | After the protection and permit self-tests with the outputs open; until then the headphone outputs are open |
| 3 | J701 G-Switch GT-3321667P-01 4.4 mm jack (LCSC; THT, 7 joints) | Holes = measured pin + 0.2 mm (gate G-2); no SMD part within 1.5 mm of a pin on L4 | Last, soldered from L4; J702 stays JLCPCB-placed |

# 8. Automation and verification plan

The design is verified by automated checks on the merged netlist, the BOM and the calculation set. Each has an explicit pass rule and is re-run after any change, at the latest for gate G-5.

| Check | What it checks | Result for v0.9 |
| --- | --- | --- |
| Netlist / BOM consistency | Every designator both ways between BOM and netlist, equal pin counts, no duplicate designator, no retired designator reused (67 entries), all interface nets present | 378 parts, 208 nets, 1111 component pins (+ 52 one-pin test pads = 1163 pin rows): 0 issues; 80 of 80 interface nets present |
| BOM totals | Fitted and DNF lines, joint counts and X-ray count for the assembly | 165 lines, 366 fitted parts (7 owner-fitted: K601–K604, J701, D102, R509), 104 unique MPNs (JLCPCB places 100 types: 34 Basic, 66 Extended or unverified), 9 DNF; 1064 SMT and 11 THT joints per board (JLCPCB: 1036 SMT and the 4 pin-in-paste stakes); 10 leadless parts per board |
| Component stress table | Worst-case voltage, current, power and temperature of every part against its rating | 137 current rows: 110 OK, 12 thin, 11 availability, 4 data gaps, 0 fails (thin items in Section 9.1) |
| JLCPCB class, pitch and reflow scan | Every BOM line against the Standard PCBA rules: reflow peak, pitch, sourcing and MSL | 166 lines: 0 Standard-PCBA blockers (reflow peak or pitch); J101 reflow dwell to be confirmed by JLCPCB; U202 0.4 mm pitch noted; global-sourced and MSL ≥ 2 lines flagged for ordering |
| Worst-case calculations per block | Power: states S1–S6, attach charge, rail start-up, discharge, unplug order (96 corners), thermal. Analog: level, power, swing, offset, stability (37 load cases × 4 models; I/V 1,080 corners), pop, hot unplug (1,944 corners). Protection: windows, energies, timing, supervisor, tiers, pre-close limits. Digital: clock and I²S timing, link, ADC settings, firmware sequences and currents. | Every Section 2 requirement is met by calculation or is a documented limitation (L-1…L-11) |
| USB pair impedance | Field-solver calculation of the 90 Ω pair on the JLC04161H-7628 stack-up | 90.0 Ω; fabrication corners 80.0–99.3 Ω inside 76.5–103.5 Ω |
| Traceability | Each requirement → calculation → measurement item, with a status | Every number labelled datasheet limit, typical only, worst-case calculation, assumption or measured at bring-up |

Bring-up and measurement plan. Safety first: the owner fits D102 and R509 and inspects them before any USB plug-in (a board without R509 keeps U503 off), runs the protection and permit self-tests with the outputs open, then fits K601–K604 and finally J701; until the photorelays are fitted, the headphone outputs are open. Board 1 is then brought up on a current-limited bench supply (5.0 V / 0.6 A), then on USB, in the order below; boards 2–5 repeat the production-test subset (✓). Every item has its fallback; none needs a board change unless noted.

| # | Measurement | Pass criterion | Designed-in fallback | Prod. |
| --- | --- | --- | --- | --- |
| 1 | Pre-layout gates G-1…G-4: J101/J701/J702 continuity (two samples each), 1:1 overlays of every new part and the U202 exposed pad, polarity including D705/D706 (the owner-fitted parts serve as their own samples) | Contact-to-pad map equals the netlist; overlays and polarity match | Netlist pin numbers edited before layout release |  |
| 2 | Rails and currents per state S1–S6 (L-1), with the USBLC6 leakage above 5.25 V inside the measured S6 current (L-10) | S1 ≤ 100 mA; S3 ≤ 490.80 mA at the eFuse maximum; S5 ≤ 350.43 mA (only after SET_CONFIGURATION); S6 ≤ 2.5 mA reached ≤ 8.50 ms after the last SOF with the IWDG wakes (≈ 3.4 µA); MCU Stop current ≤ 1.9035 mA (planning assumption 1.9 mA) | S6 margin is +3.5 µA: no hardware fallback, firmware Stop configuration only | ✓ |
| 3 | Attach inrush (USB-IF Compliance Update #45) and attach timing | ≤ 50 µC: region A 28.5 µC, region B 40.2 µC, separated by C104; U102 on 3.31–55.3 ms after plug-in; D+ pull-up ≤ 100 ms after VBUS valid (model ≤ 79.2 ms) | None needed; firmware low-clock rule |  |
| 4 | U102 input at a 5V_SYS hard short with D102 | ≤ 17 V (calculated ≤ 16.99 V; absolute maximum 20 V) | None designed in |  |
| 5 | eFuse limits and FLT; LM27762 start into the Default limit with the hold-up reservoirs | Default 177.9–220.0 mA, High 393.3–487.1 mA; rails good ≤ 18.8 ms after ANA_EN (T_START 40 ms), 5V_ANA_F ≥ 4.12 V; VPOS ≥ −0.2 V and VNEG ≤ +0.2 V while the LM27762 is off or starting; supply-fault trip at ANA_SENSE 1.279 V (1587 counts) | R510 value (same land; R509 stays 4.81 kΩ 0.1 %); longer RAILS_OK_N wait; D413/D414 clamp the reverse bias of C442/C443 | ✓ |
| 6 | 1V3 over 0 / 50 °C; discharge times and DVDD ≤ DVCC; PB4 low level | 1V3 1.235–1.365 V; 3V3A ≤ 0.385 ms, 3V3D ≤ 0.63 ms, 1V3 ≤ 0.134 ms | R533/R534; enable delays; R522/R532 values |  |
| 7 | MCLK: X201 symmetry, edges, start-up, disable; U206/R234 boot self-test | Symmetry 45–55 % at U301 pin 7 (TP711) | R203 → 0 Ω; NDK NSC5338D-80M second source (then R509 4.87 kΩ) | ✓ |
| 8 | Supervisor thresholds by a slow ramp; MCLK-alive with the slowest X201 edges | U605 3V3A trip 3.006–3.125 V; U606 1V3 trip 1.103–1.189 V; MCLK reads alive; dead detection ≤ 0.5 ms and self-test (b) ≤ 2.0 ms with the 1 % R668/R669 | R665 → 0 Ω | ✓ (U606 channel 3, Q619) |
| 9 | Boot self-test windows on every board | Leg windows 45–120 ms (95–210 ms uncalibrated); MCLK alive ≤ 2.0 ms / back ≤ 250 ms; 1V3 good ≤ 2.0 ms; 3V3A-good readbacks | None designed in | ✓ |
| 10 | CPLD configuration time and I/O state (unpublished); ID poll | Configured within the 100 ms timeout | Firmware timeout and retry |  |
| 11 | LED string current at 0 / 25 / 50 °C and 5V_SYS 4.30 V (TP743–TP745); TLP3545A VF at 5–6.6 mA; U604 step response; H at 50 °C (TP724) | IF ≥ 5.0 mA; H ≥ 3.68 V | R641/R642 24.9 → 22.1 Ω; R643/R644 → 1 kΩ; R647 → 47 kΩ with C617 → 2.2 µF | ✓ |
| 12 | Hardware protection with R417–R420 removed: DC of 0.33 / 1.0 / 2.0 / 3.8 V, both signs, on each leg; 0.27 V for 10 s; full-scale 20 Hz sine and square; CM window (an I/V output pulled to V± through 100 Ω); 3V3A off with the rails on; MCLK stop | Open within 1.0 s at 0.33 V, within 250 ms at 1.0 V (worst case 111.0 ms), within 100 ms at 2.0 V (worst case 51.2 ms) and within 25 ms at 3.8 V; no trip at 0.27 V or at full-scale 20 Hz; CM trip < 3 ms; DC_PERMIT_N high with 3V3A off; switch open on MCLK stop | Capacitor what-if: a worst-case unit may mute on a full-scale 20 Hz square (no hazard) | ✓ |
| 13 | Firmware tier boundary tests with the stream guard bypassed and R242–R246 fitted | Both signs, leg / pair: T1 1.015 / 1.020 V trip ≤ 208 ms, no trip at 579 / 418 mV for 10 s; T2 0.317 / 0.312 V trip ≤ 574 ms, no trip at 104 / 37 mV; T3 0.180 / 0.196 V trip ≤ 2246 ms, no trip at 29 / 20 mV; pre-close limits 0.070 / 0.090 V | Firmware constants | ✓ |
| 14 | Audio: level and calibration (1.55 Vrms ±0.05 dB), DC offset screen at 25 °C on every board (≤ 7.19 / 14.39 mV), THD+N at the leg and at the jack, dynamic range, response, crosstalk, J-test | Section 2 targets | Offset: a leg that fails the screen gets the Susumu 0.05 % resistors (same lands); stability: Clead 4.7–22 pF, Ct 1.5–2.2 nF | ✓ |
| 15 | I/V headroom with the hold-up supplies: swing at calibrated full scale; D411/D412 VF at 10–30 mA and 0 / 50 °C | Swing margin ≥ 0 (bound +0.087 V); VF ≤ 0.30 V | R501 221 kΩ → 232 kΩ (VPOS +0.13 V, same footprint) |  |
| 16 | Power at the rated loads at VBUS 4.40 / 4.60 V; R_NEG at VIN 4.2 V, 150 mA; PGOOD dips | Section 5.2 tables | Firmware headroom-dip rule; optional −1 dB adaptive ceiling |  |
| 17 | Pop: power-up, suspend and controlled power-down at 300 / 600 Ω; hot-unplug ordering; pop with music at the limit | ≤ 5 mV (power-up bound 4.70 mV) | Register-14 ramp (firmware); C441 only if the effective 3V3A total ≤ 10 µF is confirmed and the enable-to-ANA_EN wait is ≥ 0.75 ms; slower 3V3A ramp (option, not fitted) |  |
| 18 | Hot unplug: DAC pins and jack transient | DAC pins inside GND…AVCC; ≤ 20 mV at the jack (L-5) | None designed in (L-5) |  |
| 19 | Link BER through U208 | 10¹⁰ bits without error | Falling-edge latch in the CPLD logic |  |
| 20 | MCU I2S2 slave capture at 352.8 and 384 kHz (22.6 / 24.6 MHz BCLK, not published by WCH) | Bit-exact capture of known patterns on PB12/PB13/PB15 for ≥ 10⁹ frames at each rate | If it fails: the post-CPLD guard is used up to 192 kHz and the CPLD frozen word at 352.8/384 kHz falls under limitation L-11 |  |
| 21 | I²C with the 10 kΩ pull-ups: rise time and DAC ACK at 100 kHz | tr ≤ 1000 ns (calculated 257 / 428 / 856 ns at 30 / 50 / 100 pF) | None designed in |  |
| 22 | Photorelays: every switch opens and closes; tON / tOFF at 0 / 50 °C | Opens and closes (L-6); within the design values tON 10 ms, tOFF 1.0 ms (L-2) | None designed in (L-2, L-6) | ✓ |
| 23 | Survival test (L-9): leg-to-leg short of one channel at the 4.4 mm jack, High mode, full scale, 30 min | No damage; the firmware overload policy acts when the rails sag | None designed in (L-9) |  |
| 24 | VCCA current at 80 MHz; CPLD current; MCU current at 144 MHz | DMM per rail within the budgets | Budget margins (Section 5.3) |  |
| 25 | Stream check with the guards in test mode: full-scale sines 19.998–2000 Hz, 24 phases, onset included | Never trigger | Firmware constants |  |
| 26 | Stream check: full-scale square at 22.8 Hz and at 22.6 Hz | No trigger at 22.8 Hz; trigger in 22.0–23.6 ms at 22.6 Hz | Firmware constants |  |
| 27 | Stream check: flat digital DC of 1.0 FS; 0.65 FS DC plus a 0.35 FS 20 Hz tone | Trigger in ≤ 23.6 ms; trigger in ≤ 51 ms | Firmware constants |  |
| 28 | Stream check: DC offset of 0.15 FS; 0.08 FS offset for 10 s | Trigger in ≤ 0.3 s; no trigger | Firmware constants |  |
| 29 | Post-CPLD guard: a forced constant CPLD word | Switch open in ≤ 23.6 ms | None designed in |  |
| 30 | DAC_LOCK boot self-test: the DAC lock output forced low and high | The MCU input and the interrupt follow | None designed in |  |

# 9. Risks and owner decisions

## 9.1 Documented limitations, thin margins and sourcing risks

Every requirement of Section 2 is met by worst-case calculation or is one of the documented limitations below; none needs a hardware change.

| ID | Limitation | Handling |
| --- | --- | --- |
| L-1 | The MCU and CPLD currents are vendor-unpublished; the 100 mA, 500 mA and suspend budgets use conservative assumptions (MCU 80 mA against 22.4 mA typical; Stop current a planning assumption of 1.9 mA against 0.11 mA typical, maximum unpublished; CPLD 40 mA) | Every board is measured in every state at production test; margins S1 +17.60 mA, S3 +9.20 mA, S6 +3.5 µA (legacy cable) / +97.9 µA; the suspend budget holds while the Stop current stays ≤ 1.9035 mA |
| L-2 | Photorelay timing over temperature is extrapolated (×2) from the 25 °C maxima | Verified by measurement (tON / tOFF at 0 / 50 °C) |
| L-3 | High-mode rated powers at VBUS 4.40 V depend on the LM27762 charge-pump resistance, which is typical only (strict regulation needs R_NEG ≤ 2.37–2.49 Ω; no clipping up to ≥ 6.85 Ω) | Rated at VBUS ≥ 4.60 V; 4.40 V is a measured corner |
| L-4 | An MCU reset or IWDG reset while playing is a fault transition: the switch opens ≤ 1.01 ms and a click is possible | Energies far below the limits |
| L-5 | The hot-unplug transient through the switches during their ≤ 1.3 ms opening time | Measured target ≤ 20 mV |
| L-6 | A shorted photorelay is a latent fault | Production test verifies that every switch opens and closes |
| L-7 | Connector pin maps (J101, J701, J702) are not yet verified on parts | Pre-layout gates G-1…G-3: two samples of each continuity-tested and overlaid 1:1 before routing |
| L-8 | At a hard unplug OUT − IN of U502 / U302 / U503 reaches 0.40–0.46 V for tens of µs with the BAT54 clamps D103–D105 (a lower-VF Schottky would break the suspend budget by its leakage) | The external Schottky carries the reverse current |
| L-9 | A leg-to-leg short of one channel (4.4 mm plug or cable fault) is an overload, not DC: both amplifiers of one OPA1622 current-limit (≈ 0.55 W per package, TJ ≈ +26 °C), outside TI’s one-amplifier continuous-short rating, and it is not detected as DC in High mode | The firmware overload policy acts when the rails sag; bring-up survival test (Section 8, item 23) |
| L-10 | A source above 5.50 V (outside the USB specification) lets the U102 clamp hold 5V_SYS at 5.50–5.55 V: the LP5907 and LM27762 are then 0–50 mV above their recommended maximum but inside their absolute maxima | The USBLC6 leakage above its 5.25 V VRM is covered by the measured suspend current |
| L-11 | A DAC-core hang that keeps lock, answers I²C, returns the written registers and keeps MCLK, 1V3 and 3V3A good is not detected by any supervisor, guard or check; only the hardware leg windows remove it: 39.6 mJ at 16 Ω balanced (±1.51 V for 69.1 ms) and 25.9 mJ at 32 Ω balanced (±2.02 V for 50.7 ms), above the 30 / 15 mJ limits (16.2 mJ at 60 Ω, 3.4 mJ at 300 Ω). If the bring-up capture test of the post-CPLD guard at 352.8 / 384 kHz fails, a CPLD frozen word at those two rates falls under the same limitation | Replaces the open owner case of the DAC-core hang; ESS publishes no failure mode that produces it; if the capture test fails, the post-CPLD guard is kept for ≤ 192 kHz |

Single-component failures outside the supervisor (stated): a failed OPA2210 leaves the DAC pins unprotectable, and an OPA1622 rail short is clamped at the DAC pins to −0.42 V…AVCC + 0.42 V by D405–D408.

Thin margins:

| Item | Value | Handling |
| --- | --- | --- |
| DC offset with the 0.1 % leg resistors (thinnest) | 9.23 / 18.45 mV against 10 / 20 mV: +0.77 / +1.55 mV | Guaranteed by the 25 °C production screen (≤ 7.19 / 14.39 mV); Susumu 0.05 % rework fallback |
| Default USB total (S3) | 490.80 mA against 500 mA: +9.20 mA | No R509 value is left for further non-analog load; measured in production |
| eFuse Default minimum against the 3.5 mm 32 Ω need | 177.9 mA against 176.97 mA: +0.97 mA | Measured (Section 8, item 5) |
| Suspend, legacy A-to-C cable (S6) | 2.4965 mA against 2.5 mA: +3.5 µA | Measured on every board; no hardware fallback |
| I/V swing at calibrated full scale | +0.087 V against the 25 °C 600 Ω datasheet row (typical +0.441 V) | Measured; fallback R501 → 232 kΩ |
| Power-up pop | 4.70 mV against 5 mV (6 % on stacked assumptions) | Measured; C441 under its fit condition |
| T3 pair never-trip level | 22.1 mV against the 20 mV balanced offset limit: +2.1 mV (pair threshold 0.120 V) | Boundary tests (Section 8, item 13) |
| X201 MCLK symmetry | 45–55 %, equal to the ESS limit | Measured; fallback R203 → 0 Ω |
| MCLK-alive tap | With the slowest Kyocera edge (4.5 ns) the U607 input behind 1 kΩ does not provably reach VT+ max; the failure direction is safe (reads dead, no audio) | Checked at every boot and in production; fallback R665 → 0 Ω |
| 4.4 mm 300 Ω guaranteed floor | 31.1 mW against 30 mW: +1.1 mW | Measured |
| MCLK net loading | 14.4 pF against 15 pF | Layout rule R12 (tap ≤ 1.5 mm from U301 pin 7) |
| ANA_SENSE level at 5.55 V | ≤ 1.7987 V: 1.3 mV inside the 1.8 V design guideline | The ADC range is 0–3.3 V, so the electrical margin is ≥ 1.5 V |
| TLV1704 stock | LCSC stock must be ≥ 25 at order time (21 bought) | Otherwise JLCPCB global sourcing (≈ US$ 2.6 more) |

Sourcing: with no spares, a damaged or missing part at JLCPCB stops the order, so the thin lines are pre-ordered now in the lean quantity (Section 10): the YAGEO 1.30 kΩ leg resistor (LCSC 100 in stock for 60 needed), the OPA2210IDR (no compliant pin-compatible alternative, since the OPA1612 fails the Default power and offset targets; LCSC 16 in stock for 11 needed, US$ 2.6708, C1849415) and the thin-stock ES9018K2M, CH32V307 and AGRV2KQ32. The TLV1704AIPWR is bought from LCSC only if ≥ 25 are in stock at order time, otherwise through global sourcing (≈ US$ 2.6 more). Still global-sourced: J101 (GCT USB4105; a cheaper connector is not verified), X201 (Kyocera) and X202/X203 (NDK) with factory leads of up to 24–32 weeks, U607 (74AUP1G17), the 22 MΩ resistors and the 0.1 % 0402 values 121 k / 115 k / 5.1 k (LCSC out of stock) and 953 k / 330 k / 1 M (no LCSC listing). The owner’s two orders (DigiKey and LCSC) must arrive before the gates and the bring-up; the LCSC order stays tax-free only while its shipping is ≤ ≈ US$ 16, and the hand-soldering quality is the owner’s. Part data still to be confirmed at bring-up: the TLV3402 delay (typical only, ×3 bound; TI recommends an alternate for new designs), the 74AUP1G17 CPD and CI, the leakage of the SMDJ12A, BAT54, PMEG2010AEH and the tantalum reservoirs at 50 °C (25 °C data only), and the CH32V307 reset state of PA15/PB3/PB4 (the reference-manual text was not retrieved).

Other open items, each with its fallback in Section 8: the pre-layout gates G-1…G-4; VPOS ≥ −0.2 V and VNEG ≤ +0.2 V while the LM27762 is off or starting (the reverse bias of the polarised C442/C443 is clamped by D413/D414; to be confirmed by calculation and measured); the MCU I2S2 slave at 352.8 / 384 kHz.

## 9.2 Owner decisions (decided 25 Sep 2026)

Both owner decisions were decided on 25 Sep 2026, in firmware only: there is no hardware or parts change, and the design check is unchanged (378 parts, 208 nets, 1111 pins, 0 issues).

| # | Item | Numbers | Decision | Status |
| --- | --- | --- | --- | --- |
| OD-1 | Full-scale digital DC into 32 Ω balanced: program content, a digital-path fault, or a CPLD frozen word caught by the post-CPLD stream guard | Before: 15.5–22.8 mJ at 32 Ω balanced, over the 15 mJ limit. Decided: removed within 23.6 ms; 13.8 mJ at 32 Ω balanced (limit 15 mJ), 25.3 mJ at 16 Ω balanced (limit 30 mJ), 3.6 / 6.9 mJ single-ended at 32 / 16 Ω | Stricter stream check on both guards (rules P1–P4, Section 5.3). Accepted cost: full-scale squares at or below 22.7 Hz and steady offsets ≥ 0.15 FS mute; full-scale sines ≥ 20 Hz never mute | Decided 25 Sep 2026 (firmware only) |
| OD-2 | DAC-core hang with MCLK, 1V3 and 3V3A all good (not detected by the supervisor channels) | Before: 39.6 / 25.9 mJ at 16 / 32 Ω (leg windows only). Decided: lock loss removed ≤ 1.04 ms (0.60 mJ); status or register mismatch and I²C no-acknowledge ≤ 16.45 ms (≤ 9.5 mJ); stuck I²C bus ≤ 17.45 ms (≤ 10.1 mJ); DPLL ratio error ≤ 102.4 ms; every detected case ≤ 10.1 mJ at 16 and 32 Ω | Firmware check of the DAC’s lock status (DAC_LOCK interrupt, 10 ms status and register poll, I²C no-acknowledge and bus checks, 100 ms DPLL ratio check); the undetectable remainder is limitation L-11 | Decided 25 Sep 2026 (firmware only) |

## 9.3 Decisions closed in v0.9

| Decision | Resolution |
| --- | --- |
| Output disconnect | Four TLP3545A photorelays, one per leg (24 of 24 plug cases isolated), replace the hardware fault latch |
| Hardware permit | Non-latching node H (leg DC windows, CM window, supervisor) in series with RELAY_EN and the MCLK permit; the firmware keeps RELAY_EN low after any trip until the pre-close sequence passes |
| Frozen DAC code | DAC-supply and MCLK supervisor: removed in ≤ 2.43 ms, ≤ 1.39 mJ |
| Hot-unplug DAC-pin excursion | I/V hold-up supplies (PMEG2010AEH + 220 µF AVX TAJD): clamp exposure 0 ms |
| USB input | TPS25200 over-voltage clamp and SMDJ12A TVS; C104 separates the attach regions |
| Analog current limit | TPS259621 eFuse, R509 4.81 kΩ (Default) and R510 3.74 kΩ through the TMUX1101 (High); the internal 490 mA reserve rule is relaxed, leaving 9.2 mA to 500 mA in S3 |
| LM27762 enable | EN± tied to its input; RAIL_EN deleted |
| ESS power order and domain crossing | Supervisor gates AUD_EN and the MCLK gate; U208 (Ioff) link buffer, Q207 open-drain RESETB, U303 enable from 3V3D |
| Clocks | NDK 49.152 / 45.1584 MHz family clocks through the U205 mux without a CPLD PLL; Kyocera 80 MHz MCLK with a hardware gate; 2520-only lands |
| Output legs | T input network and 10 pF lead capacitor (the v0.8 legs were unstable with 1 nF plus cable); 0.1 % / 25 ppm resistors with a 25 °C offset screen, Susumu 0.05 % as the rework fallback |
| Leg DC window filter | R601–R604 115 kΩ: no false trip on a full-scale 20 Hz square at the aged-capacitor corner (6.2 % margin) |
| ADC pins and I²C | R242–R246 330 Ω limit the injection into an unpowered MCU to ≤ 3.0 mA per pin (≤ 15 mA total); I²C pull-ups 10 kΩ (the DAC VOL is specified only at 100 µA) |
| Firmware rules | Programming mode only after SET_CONFIGURATION; suspend fade and mute with a 5.31 ms deadline; IWDG 16.0–38.4 ms (RTC wakes in suspend); boot self-test at every attach; one overload rule; supply-fault rule in every state; switch open for rate and family changes |
| Budget | Not a constraint (owner, 25 Sep 2026); cost reported for information only. Least-cost patch of the same day without losing any requirement: US$ 758.82 per 5-board order; JLCPCB Standard PCBA kept (Sections 6 and 7) |
| OPA2210 sourcing | 11 pre-ordered from the 16 in LCSC stock (no spares) |
| Full-scale digital DC (OD-1) | Stricter stream check on both guards: removed within 23.6 ms, 13.8 mJ at 32 Ω and 25.3 mJ at 16 Ω balanced, within the limits; full-scale squares at or below 22.7 Hz and steady offsets ≥ 0.15 FS now mute (accepted by the owner) |
| DAC plausibility (OD-2) | Firmware DAC lock-status check: every detected case ≤ 10.1 mJ; the undetectable DAC-core hang is limitation L-11 |

# 10. Next steps

| # | Action | Done |
| --- | --- | --- |
| 1 | Pre-layout gates G-1…G-4 with the samples from the owner’s two orders (the owner-fitted parts serve as their own samples): continuity tests of J101, J701 and J702; 1:1 overlays of every new part and of the U202 exposed pad; polarity checks including D705/D706 (Section 7). | ☐ |
| 2 | Buy now into the JLCPCB private parts library, lean (exact five-board need plus attrition, no spares): OPA2210IDR 11 (LCSC C1849415); YAGEO RT0603BRD071K3L 60 (LCSC, thin stock), RT0603BRD07680RL 60, Panasonic ERA-3AEB202V 50 and RT0603BRD07698RL 40; TLV1704AIPWR 21 (LCSC if ≥ 25 are in stock, otherwise global sourcing); TLV3402IDR 6; SN74LVC3G34DCUR 10; PMEG2010AEH 11; AVX TAJD227K010RNJ 12; ES9018K2M, CH32V307RCT6 and AGRV2KQ32 6 each. Global sourcing: X201, X202, X203, J101 and U607 6 each, 22 MΩ 12, and the 0.1 % YAGEO 121 k / 115 k / 5.1 k and 953 k / 330 k / 1 M values (five boards plus attrition); the 237 k from LCSC. | ☐ |
| 3 | Owner purchase in two orders, US$ 161.60 = NTD 5,128: DigiKey US$ 105.63 including 5 % tax (TLP3545A × 20, SMDJ12A × 5, KOA 4.81 kΩ 0.1 % × 5 and two gate samples each of USB4105, KC2520K80, both NZ2520SDA, 74AUP1G17, TPS259621 and TLV1704) and LCSC US$ 55.97 including ≈ US$ 10 shipping, an assumption (GT-3321667P-01 × 5 and 14 sample types). Both orders must arrive before the gates and the bring-up. | ☐ |
| 4 | Owner decisions OD-1 and OD-2: decided 25 Sep 2026 (stricter stream check; DAC lock-status check), firmware only (Section 9.2). | ☑ |
| 5 | Layout from the layout package (Section 7, rules R06–R16), started only after G-1…G-4 have passed. | ☐ |
| 6 | Gate G-5: netlist and BOM freeze, with the consistency check, stress table and PCBA scan re-run. | ☐ |
| 7 | Gate G-6 and order: JLCPCB Standard PCBA, five boards, top side, with the owner-fitted parts removed from the BOM and placement files; order remarks; record the quote. | ☐ |
| 8 | Stage 2 bring-up on the JLCPCB-built boards: the owner first fits D102 and R509, runs the protection self-tests with the outputs open, then fits K601–K604 and J701; board 1 then goes through the full plan of Section 8 and boards 2–5 through the production-test subset with the 25 °C offset screen; publish the measurements against Section 2. | ☐ |

# Appendix A. Reference documents

| Ref | Document | Source |
| --- | --- | --- |
| [1] | ES9018K2M datasheet v3.7 (22 Apr 2021) | ESS Technology |
| [2] | OPA1622 datasheet (SBOS727B) | Texas Instruments |
| [3] | OPA210/OPA2210 datasheet (SBOS924H, Aug 2021): I/V converters U403 and U404 | Texas Instruments |
| [4] | LM27762 datasheet (SNVSAF7C) | Texas Instruments |
| [5] | TPS25200 datasheet (SLVSCJ0F): input over-voltage clamp U102 | Texas Instruments |
| [6] | TPS2596x eFuse datasheet (TPS259621, U503): current-limit equation and FLT behaviour | Texas Instruments |
| [7] | TMUX1101 datasheet (U505) | Texas Instruments |
| [8] | TPS7A20 datasheet (TPS7A2033, U502) | Texas Instruments |
| [9] | TLV767 datasheet (TLV76733, U504) | Texas Instruments |
| [10] | TLV758P datasheet (TLV75801P, U303) | Texas Instruments |
| [11] | LP5907 datasheet (U302) | Texas Instruments |
| [12] | TLV1704 datasheet (SBOS589D): window comparators U601–U603 and U606 | Texas Instruments |
| [13] | TLV3402 datasheet (U605) | Texas Instruments |
| [14] | OPA2333 datasheet (U604) | Texas Instruments |
| [15] | TPD2E2U06 and SN74LVC3G34 datasheets (U103, U208) | Texas Instruments |
| [16] | USBLC6-2SC6 datasheet (U101) | STMicroelectronics |
| [17] | TLP3545A photorelay datasheet (2023-05-24): output switches K601–K604 | Toshiba |
| [18] | KC2520K series oscillator datasheet (X201, KC2520K80.0000C1GE00) | Kyocera |
| [19] | NZ2520SDA oscillator datasheet (X202, X203; NSC5083D) | NDK |
| [20] | 74LVC1G157, 74LVC1G11 and 74AUP1G17 datasheets (U205, U206, U607) | Nexperia |
| [21] | PMEG2010AEH, BAT54S, BAT54 and BAS70-04 datasheets (D411/D412, D405–D408, D103–D105 and D413/D414, D609) | Nexperia |
| [22] | SMDJ series TVS datasheet (SMDJ12A, D102) | Littelfuse |
| [23] | TAJ series tantalum capacitor datasheet (TAJD227K010RNJ, C442/C443) | Kyocera AVX |
| [24] | USB4105-GF-A USB-C receptacle drawing and datasheet (J101) | GCT |
| [25] | GT-3321667P-01 4.4 mm jack (J701) and PJ-332A-6A 3.5 mm jack (J702) drawings | G-Switch; HRO |
| [26] | LESD5D5.0CT1G ESD diode listing (D701–D704) | GOODWORK |
| [27] | RT0603BRD and RT0402BRD (YAGEO), ERA-3AEB (Panasonic) and RN73R (KOA, R509) thin-film resistor datasheets; RG1608N (Susumu) for the rework fallback | YAGEO; Panasonic; KOA; Susumu |
| [28] | CH32V307 datasheet and EVT package (high-speed USB device examples) | WCH |
| [29] | AGRV2K CPLD datasheet | AGM |
| [30] | USBAudioToI2S: UAC2.0-to-I²S firmware for MCU + CPLD (Apache-2.0); known-good comparison and reference | GitHub, LoveLonelyTime/USBAudioToI2S |
| [31] | USB Type-C Cable and Connector Specification R2.0 (current-advertisement thresholds) | USB-IF |
| [32] | USB Type-C sink timing (tSinkAdj 60 ms max, tRpValueChange 10–20 ms), as quoted in the Zephyr USB-C stack and the Chromium EC documentation | USB-IF specification; Zephyr Project; Chromium OS EC |
| [33] | USB-IF Compliance Update #45 (inrush region method, 50 µC) | USB-IF |
| [34] | PCB Assembly Cost: What Does the Price Include? (JLCPCB fee table, updated 8 Sep 2026) | JLCPCB |
| [35] | Schematic Design Notes v0.8 (companion document: block-level circuit guidance and the layout package) | This project |
| [36] | Parts List v0.7 (parts, quantities, ordering plan and cost) | This project |

# Appendix B. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Draft for review |
| v0.2 | 23 Sep 2026 | Diego Yang | Budget raised to ≤ 1,500 NTD; outputs changed to 3.5 mm unbalanced + 4.4 mm balanced; parts and targets verified against the XU316-1024-QF60A, AK4493S, AKD4493S-A and OPA1622 datasheets; USB chip changed to XU316-1024-QF60A (1.8 V I/O); OPA1612 stage-1 and LT3045 added; CH224K replaced by CH224Q; JLCPCB fees updated to the 8 Sep 2026 price list; most open decisions closed. |
| v0.3 | 23 Sep 2026 | Diego Yang | Remaining parts chosen from stocked LCSC/Digi-Key parts; CH224A replaces CH224Q; TPS7A4901 replaces LT3045 to stay within 1,500 NTD; protection defined (LM339 + 2 × HFD4 relays with hardware amplifier shutdown); cost re-estimated with real prices at ≈ 1,490 NTD; −14 V rail set to −14.7 V; stage-1 count corrected to one dual OPA1612. |
| v0.4 | 23–24 Sep 2026 | Diego Yang | Cost-driven redesign for the 5-board budget: single USB-C bus-powered design; CH32V307 + AGRV2K CPLD USB bridge (open-source USBAudioToI2S); ES9018K2M DAC; 2 × OPA1622 as four difference amplifiers; LM27762 charge-pump rails; relays replaced by a 3.3 V hardware fault latch; three design reviews dispositioned. Revised on 24 Sep 2026: DAC output moved to current mode with I/V converters (owner decision, option B) and an owner-approved cost ceiling of 1,550 NTD per board. |
| v0.5 | 24 Sep 2026 | Diego Yang | Fourth review round (R4-A, R4-B, R4-C) dispositioned (Appendix C): headphone rails ±3.78 V; per-unit calibration to 1.75 Vrms per leg (Rf 836 Ω); power rated at 1 % THD+N or calibrated 0 dBFS, with High and Default modes; TPS2553 current limit and switched power domains per USB state; OPA2197 I/V converters; DAC in asynchronous mode from a 100 MHz MCLK; first issued with the interim both-edge divider for the 44.1 kHz family and amended the same day to three oscillators (next row); protection completed (PGOOD kill, blanked latch, energy bound at the terminals); fixed 1.3 V DVDD LDO; balanced offset limit ±20 mV with 100 % screening; Parts List v0.3 with verified LCSC numbers and classes; cost 1,540 NTD per board (WARNING, under the 1,550 NTD ceiling); pre-layout firmware gate. |
| v0.5 amendment | 24 Sep 2026 | Diego Yang | Amendment for the owner decisions of 24 Sep 2026: family oscillators for exact bit clocks, because the ES9018K2M requires a bit-clock duty cycle of 45:55 to 55:45 and one 100 MHz clock for both families gave 40:60 at 352.8/384 kHz. First X202, 24.576 MHz (YXC OT322524.576MJBA4SL), for the 48 kHz family; then, at the owner’s request, X203, 22.5792 MHz (SCTF SX2M22.5792M20F30TNN), for the 44.1 kHz family, which replaced the interim both-edge fractional divider and its 352.8 kHz condition. Final clock architecture: DAC MCLK 100 MHz unchanged, family oscillator selected by its enable pin in front of the CPLD PLL (×2), all eight rates exact, family change with the PLL held in reset, no clock multiplexer in the fabric. Own UAC2 firmware and CPLD logic instead of modifying USBAudioToI2S (kept as a known-good comparison), with the firmware gate run as a two-stage proof of concept. Currents updated (Default ≤ 0.473 A, High ≤ 0.758 A); cost 1,540 → 1,602 → 1,666 NTD per board (WARNING), approved ceiling raised from 1,550 to 1,650 and then 1,700 NTD; Figures 1 and 2 updated. |
| v0.6 | 24 Sep 2026 | Diego Yang | Fifth review round (R5-A, R5-B, R5-C of the amended Spec v0.5) dispositioned (Appendix C): 80 MHz MCLK and CPLD system clock (SCTF SX3M80.000B20F30TNN) below the DAC limit; family oscillators off by default with a break-before-make handover, a node pull-down and a DNF mux fallback (U205); MCU link captured in its own clock domain; OPA2210 I/V converters (single input pair); rails +3.78 V / −3.72 V including resistor tolerance (R518 5.1 kΩ); fault latch also set by PGOOD-bad (Q610) and a sustained TPS2553 overcurrent (D606); rails-first-off power-down, VBUS watchdog 4.10 V dropping AMP_EN and RAIL_EN, VREF clamp D401 and C421 100 nF; protection envelope by DC magnitude with recalculated terminal energies; Default-mode power rated from the instantaneous peak current; USB estimated maxima 0.469 A (Default) and 0.754 A (High); mandatory Stage 2 board with the USB current measurements as a layout gate; DC offset screening with a DC-capable DMM; J703 MCLK monitor-only; document-lint CI check; cost 1,647 NTD per board (WARNING, 53 NTD under the 1,700 NTD ceiling, recorded with the owner statement of 24 Sep 2026); Figures 1 and 2 updated; companion documents Schematic Design Notes v0.5 and Parts List v0.4. |
| v0.7 | 24 Sep 2026 | Diego Yang | Sixth review round (R6-A, R6-B, R6-C and R6-D of Spec v0.6) dispositioned (Appendix C): latch capture ≈ 35 µs with C605 on node F (power-on set, set-request stretch), 47 kΩ latch pull-ups, Q609 as a slow interlock and Q611 killing AVCC_EN; worst-case DC detector (R601–R604 100 kΩ, R609/R610 10 kΩ: 2 V ≤ 59 ms, 1 V ≤ 183 ms) and worst-case terminal energies (26.2 / 13.3 mJ at 16 / 32 Ω including ≤ 0.6 mJ after the trip); slow firmware DC tier through the MCU ADC (≥ 0.8 V in 1 s, ≥ 0.2 V in 3 s) setting the same latch, with the residual below 0.2 V as a pending owner decision; fault-cause capture at the interrupt and a 3-strike rule for DC faults; VREF tracks AVCC (C421 330 pF; D401 and C422 removed) and the DAC analog supply turns off first, so a controlled power-down takes ≤ 4 ms; USB suspend ≤ 2.5 mA within 10 ms as a v1 requirement; pop definition; regulator dissipation table with the R519/R520 dropper and C512; DAC centre-voltage alternative with the R433 option; OPA2210 output swing and the −3.5 dB safety default; SPI link with chip-select framing; SDATA/LRCLK on the falling BCLK edge; mux pull-downs R216/R217; 31 test pads; Stage 2 with the real regulators (≈ US$90 ≈ 2,860 NTD); cost 1,650 NTD per board (WARNING, 50 NTD under the 1,700 NTD ceiling, gate covering every JLCPCB line); Figure 1 updated; companion documents Schematic Design Notes v0.6 and Parts List v0.5. |
| v0.8 | 24 Sep 2026 | Diego Yang | Seventh review round (R7-A…R7-E, a full design check of Spec v0.7, Schematic Design Notes v0.6 and Parts List v0.5 in five areas) dispositioned (Appendix C): series resistors R307 (AVCC_EN) and R521 (AUD_EN); PGOOD pulled up to 5V_ANA through R505 10 kΩ with R522, R623 1 MΩ and C607 10 nF (genuine ≈ 35 µs capture, new blanking and arming times); TI LM393B detector with 147 kΩ / 10 kΩ thresholds, C612/C613 and R638 (leg trip 0.36–0.80 V; 1 V ≤ 179 ms, 2 V ≤ 58 ms; 25.6 / 13.1 mJ at 16 / 32 Ω); firmware start-up rules and FLT interrupt order; residual DC below 0.2 V accepted by the owner; calibrated level 1.55 Vrms per leg with the 1.3 V OPA1622 clean-swing model, new power targets and a 70 mA envelope at 4.4 mm 32 Ω; DR estimate 117.4 / 118.6 dB(A); V+ +3.90 V (R523) for the OPA1612 fallback; DAC reset state handled by the power-down order (MCLK_EN; pop at risk until measured); CPLD on 2-wire Compact-JTAG (J202), family clock on PIN_1, no 80 MHz input, MCLK_EN with R218, FB202 and C220; J201 with NRST, BOOT0 and UART pads; programming mode (TP732, 32 test pads); VBUS watchdog on ADC1; bleeders R524/R525; DNF TVS D102; R107 0 Ω and C101 DNF; LM393BIDR added to the key parts; USB estimated maxima 0.474 A / 0.759 A; suspend budget 2.31 mA, level ≈ 9.3 ms; Stage 1 optional and Stage 2 (with the ES9018K2M, ≈ US$93 ≈ 2,940 NTD) the first hardware; cost 1,691 NTD per board (WARNING, 9 NTD under the 1,700 NTD ceiling); Figures 1 and 2 updated; companion documents Schematic Design Notes v0.7 and Parts List v0.6. |
| v0.8 (amended, 24 Sep 2026) | 24 Sep 2026 | Diego Yang | Amended in place (same version) for three external reviews of the previous set (R7-F, R7-G, R7-H; Appendix C.2): hardware DC detector changed to two TI LM339DR quad comparators with one window per leg and no diode-OR (BAV99 D601–D604 and R605/R606 removed; R607/R608 47 kΩ, R636 15 kΩ, R637 10 kΩ; C612–C615): leg trip 0.43–0.67 V, 1 V ≤ 125 ms, 2 V ≤ 47 ms, rail-stuck 21.4 / 11.0 mJ at 16 / 32 Ω, balanced digital-DC note; slow DC-sense network redesigned (15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF, 3V3A monitor on PC4) with its error budget; four firmware DC tiers per leg and per balanced pair with the accepted envelope; DAC-pin analysis for a failed-output shutdown stated as a residual risk, C421 330 pF → 33 pF, DNF BAT54S clamps D405–D408 and DAC-pin tests; adaptive AMP_EN at power-down (suspend typically ≈ 8.2 ms, at worst 9.3 ms), LEDs off in suspend, resume ≈ 1.0 s, stream DC guard, option A′; USB Default 0.475 A, High 0.760 A, suspend budget ≈ 2.30 mA; cost 1,697 NTD per board (8,487 NTD for five; WARNING, 3 NTD under the ceiling); Figures 1 and 2 updated; companion documents Schematic Design Notes v0.7 and Parts List v0.6, both amended the same day; DAC-pin clamps D405–D408 fitted (owner decision). |
| v0.9 | 25 Sep 2026 | Diego Yang | Layout-ready specification. Eighth review round (R8-A…R8-D, four external reviews of Spec v0.8; Appendix C.1) and four independent design reviews of the v0.9 package (power states and sequencing, signal integrity, two pin-map reviews; Appendix C.2) dispositioned. The hardware fault latch is replaced by four TLP3545A photorelays, one per leg, with the non-latching permit node H (TLV1704 leg DC windows with R601–R604 115 kΩ, I/V common-mode window, DAC-supply and MCLK supervisor U605/U606/U607) in series with RELAY_EN and the MCLK permit; TPS25200 input clamp and SMDJ12A TVS; TPS259621 eFuse with the TMUX1101 Default/High limit (R509 4.81 kΩ, R510 3.74 kΩ); LM27762 enables tied to its input; TPS7A2033, TLV76733 and TLV75801P regulators; I/V hold-up supplies (PMEG2010AEH, 220 µF tantalum) and BAT54S clamps on the I/V outputs; NDK 49.152 / 45.1584 MHz family clocks through a 2:1 mux without a CPLD PLL and a Kyocera 80 MHz MCLK behind a hardware gate; U208 link buffer and Q207 open-drain RESETB; T-network output legs; R242–R246 ADC-pin limiters and 10 kΩ I²C pull-ups; firmware rules for suspend, IWDG, pre-close checks, tiers, stream guards and the boot self-test. Budget no longer a constraint (owner, 25 Sep 2026): cost for information only. Least-cost patch of the same day, every requirement re-verified: no spares; D102, R509, K601–K604 and J701 hand-soldered by the owner; 0.1 % / 25 ppm leg resistors with a 25 °C offset screen and other cheaper drop-in parts; exposed-pad vias tented; Standard PCBA kept. Parts US$ 65.41 per board, 5-board order US$ 758.82 = NTD 24,078 (US$ 314.05 or 29.3 % below the first estimate of US$ 1,072.87). Documented limitations L-1…L-11; owner decisions of the same day, firmware only: OD-1 stricter stream check (full-scale digital DC 13.8 mJ at 32 Ω balanced, within 15 mJ) and OD-2 DAC lock-status check (every detected case ≤ 10.1 mJ; the undetectable DAC-core hang is L-11); JLCPCB Standard PCBA, top side; release gates G-1…G-6. |

# Appendix C. Design review disposition

Appendix C records the disposition of every review finding. C.1 and C.2 are new in v0.9. C.3–C.8 are the review history of Spec v0.8 (its Sections C.1–C.6), carried over verbatim and kept as history: references to C.1–C.6 inside them use the v0.8 numbering (v0.8 Section C.n is C.n+2 here), and the values, rules and gates they quote (for example the former cost gate) are superseded by the body of this specification.

Disposition key for C.1 and C.2: fixed or closed (hardware, firmware or document change, with the section); accepted; limitation (documented, L-n, Section 9.1); owner decision (OD-n, decided 25 Sep 2026, Section 9.2); rejected (with the reason). Measurement items name their item in the bring-up plan of Section 8.

Status values used in C.3–C.8, as in v0.8:

Status values: Closed = fixed in this revision; Closed – design = design changed in this revision; Closed – design, verify = design change made, confirmed by a bench or firmware-gate check; Closed – firmware rule = firmware requirement added (Section 8); Closed – documentation = the documents corrected; Closed – verify = resolved, confirmed at the firmware gate, on the Stage 2 board or at bring-up; Closed – N/A = not applicable after the v0.4 redesign; Closed – owner decision = decided by the owner; Open – verify (on Stage 2) = to be confirmed on hardware; Open – owner action = waiting for an owner purchase or check (Section 10); Open – owner decision = awaiting the owner’s decision; Open – tracked = tracked item with an owner and a gate (Section 9.2); Accepted (v1) = accepted for v1 without change; Partly closed = some parts closed, the rest open with a gate; residual risk = a consequence of an already-failed part, stated and measured but not designed out; Not adopted = optional change not taken; Info = confirmation, no action.

## C.1 Eighth review round (R8-A…R8-D)

Four external reviews of Spec v0.8. Finding IDs are the reviewers’ own numbers with the review prefix: R8-A1…R8-A10, R8-B1…R8-B12, R8-C1…R8-C4 and R8-D1…R8-D10, plus each review’s general, verified or minor items. Where a later analysis in v0.9 changed a value, the row gives the v0.9 value.

| ID | Finding | Resolution in v0.9 | Section |
| --- | --- | --- | --- |
| R8-A1 | DNF TVS D102 cannot protect ≤ 6 V inputs | Closed: the DNF SMF6.0A is removed; the U102 TPS25200 over-voltage clamp holds 5V_SYS at 5.25–5.55 V (datasheet), 0.25 V below the lowest absolute maximum (5.8 V); the SMDJ12A (designator D102 re-used, hand-soldered by the owner) keeps U102 IN ≤ 16.99 V at a hard short | 3.2 |
| R8-A2 | FAULT → 1N4148 → node F threshold path | Closed: discrete latch and node F deleted; eFuse FLT → ANA_FAULT_N (MCU); the hardware permit is the wired-OR comparators on H; a dead short on 5V_ANA is caught by the firmware supply-fault rule (DC_PERMIT_N not permitted, ANA_SENSE below 4.0 V) | 3.2, 5.3 |
| R8-A3 | Stream guard mutes valid full-scale 20 Hz content | Closed: decided 25 Sep 2026, stricter stream check (OD-1): four rules on both guards remove full-scale digital DC within 23.6 ms (13.8 mJ at 32 Ω balanced, within 15 mJ); full-scale sines ≥ 20 Hz never mute, while full-scale squares at or below 22.7 Hz now mute (the cost the owner accepted) | 5.3, 9.2 |
| R8-A4 | Suspend relies on the typical LP5907 discharge | Closed: guaranteed enable-derived discharge Q502–Q506 (3V3A ≤ 0.385 ms); suspend ≤ 8.50 ms with the fade-and-mute sequence (margin ≥ 1.50 ms); measured (Section 8, item 2) | 3.2, 5.3 |
| R8-A5 | BAT54S nonlinear capacitance and leakage on the DAC node | Closed: clamps moved to the OPA2210 outputs (D405–D408), nothing on the virtual ground; THD+N measured (item 14) | 3.1 |
| R8-A6 | −0.4 V / AVCC + 0.4 V still possible | Closed for hot unplug: the I/V hold-up supplies make the exposure 0 ms; D405–D408 still bound OPA1622 rail-short faults (−0.42 V…AVCC + 0.42 V), a stated single-fault residual | 3.2, 9.1 |
| R8-A7 | Inrush ≈ 49 µC against 50 µC | Closed: region A 28.5 µC, region B 40.2 µC (C104 separates the regions), C503 deleted, capacitance budget rule (5V_SYS ≤ 3.3 µF, 3V3M ≤ 5.1 µF nominal); measured (item 3) | 3.2, 5.3 |
| R8-A8 | PB4 must stay 5 V tolerant | Closed: PB4 stays on a 5 V-tolerant pin; R532 470 kΩ gives 3.12–4.78 V ≤ 5.0 V | 3.2 |
| R8-A9 | OPA2210 sourcing dependency | Action, order now: 11 pieces pre-ordered from the 16 in LCSC stock (no spares, owner decision of 25 Sep 2026); no OPA1612 variant; price corrected to US$ 2.6708 | 9.1, 10 |
| R8-A10 | Connector pin maps | Limitation L-7: two-sample continuity tests of J101/J701/J702 before the footprint freeze (gates G-1/G-2; closed when measured) | 7, 9.1 |
| R8-A (verified items) | LM339 windows, timing, DC-sense, SPI, pin maps and the other items the reviewer verified | Re-verified for the changed parts: TLV1704 windows, new DC-sense networks, pin maps by the netlist check | 5.3, 8 |
| R8-B1 | 44.1 kHz feedback value 8× too large | Closed: all eight values generated from the rate table with assertions | Firmware (Notes v0.8) |
| R8-B2 | Start-up negative DC before the ADC bias exists | Closed: switch open until the pre-close checks; hardware leg windows armed from rail-up; the CM window trips without 3V3A; readings valid only after V3A_MON has been in its window for ≥ 150 ms | 5.3 |
| R8-B3 | Fault shutdown does not stop MCLK | Closed: N2_MCLK_OE = MCLK_EN ∧ AVCC_EN ∧ MCLK_PERMIT; an MCU reset stops X201 within 0.39 µs in hardware; R227 holds XI low | 3.1 |
| R8-B4 | VBUS watchdog: 32 kΩ source against 7.5 sample cycles | Closed: ADC1 single channel, sample time 41.5 cycles (43.5 kΩ allowed), watchdog 4.00 V with a 2-sample debounce; C103 100 pF gives the action 0.049–0.118 ms after source loss (96 of 96 corners) | 3.2, 5.3 |
| R8-B5 | Tier deadlines ignore the measurement error | Closed: guaranteed physical trip levels with a linear error budget (e_leg 60.30 mV; T3 0.176 / 0.192 V); boundary tests with worst-sign errors and initialised windows; measured (item 13) | 5.3 |
| R8-B6 | BAT54S leakage cancellation not guaranteed | Closed: clamps off the DAC node, so the offset has no leakage term (worst case 9.23 / 18.45 mV with the 0.1 % leg resistors and the 25 °C screen) | 5.3 |
| R8-B7 | BAT54S VF at 25 °C pulsed | Closed: clamp bound from the full VF range, −0.42 V…AVCC + 0.42 V; the I/V hold-up supplies remove the unplug exposure | 3.1, 3.2 |
| R8-B8 | Comparator input below V− at shutdown | Closed: 100 kΩ R649–R652 after the filter capacitor limit the input current to ≤ 45 µA at rail collapse (TLV1704) | 5.3 |
| R8-B9 | 6 V TVS cannot protect the 5.8 V LM27762 | Closed: U102 clamp ≤ 5.55 V | 3.2 |
| R8-B10 | Bit-perfect not proven by FFT | Design closed: I²S capture taps (R228–R230 → PB12/13/15) for an on-board word compare at every rate and format; measured at bring-up | 2, 3.1 |
| R8-B11 | LP5907 230 Ω typical; pop > 5 mV | Closed: discharge guaranteed by Q503; power-up pop bound 4.70 mV (6 % margin); the register-14 ramp at power-down; measured (item 17) | 3.2, 5.3 |
| R8-B12 | OPA1612 fallback not compliant | Closed: no fallback variant; OPA2210 only | 9.1 |
| R8-B (general) | Keep specified, calculated and measured values apart | Closed: every value labelled datasheet limit, typical only, worst-case calculation, assumption or measured at bring-up; traceability status per requirement | 2, 8 |
| R8-C1 | Negative-leg DC not caught for ≈ 0.2 s at start-up (94 mJ) | Closed: blanking deleted; windows armed whenever the rails are up; switch open until the pre-close checks; rail-stuck 25.6 / 12.8 mJ | 5.3 |
| R8-C2 | Reset-state saturation, pop on every power order | Closed: the four-leg output switch is the planned output mute, open in every sequence; reset saturation is harmless; power-up pop ≤ 4.70 mV; measured (item 17) | 5.3 |
| R8-C3 | Balanced hardware fault 32.8 mJ > 30 mJ | Closed: I/V common-mode window U603 trips at ≥ 0.98 V per leg in ≤ 3.63 ms, ≤ 12.4 / 7.4 mJ; envelope 27.2 / 13.6 mJ (9 % margin) | 5.3 |
| R8-C4 | RAILS_OK_N “good” with 5V_ANA off | Closed: validity rule, only with ANA_EN = 1 for ≥ T_START = 40 ms (RAIL_EN deleted) | 3.2, 5.3 |
| R8-C (minor) | MISO at 36 MHz; suspend 9.3 ms; High power after calibration | Closed: no MISO data at 36 MHz (FIFO level from LRCLK counts, reads at 9 MHz); suspend ≤ 8.50 ms; High targets met (Susvara 114.3 mW, K1000 71.9 mW) | 3.1, 5.2, 5.3 |
| R8-D1 | Power-down relies on an uncharacterised DAC state | Closed: switch opened first, then the DAC ramp to ground (retried once), MCLK off, RESETB, AVCC; the supervisor removes the frozen-code cases (X201 dead, open MCLK, DVDD collapse) in ≤ 2.43 ms; DAC-core hang: decided 25 Sep 2026, DAC lock-status check (OD-2); residual L-11 | 5.3, 9.2 |
| R8-D2 | Comparator current × charge-pump factor → 36.5 mW | Closed: quiescent recomputed (71.61 mA), need 176.97 mA, eFuse minimum 177.9 mA → ≈ 38.6 mW ≥ 38 mW | 5.1, 5.2 |
| R8-D3 | RAILS_OK_N good with 5V_ANA off; blanking; PB4 5.15 V | Closed: validity rule (R8-C4); blanking deleted; PB4 ≤ 4.78 V with R532; ANA_EN off in suspend (S6 budget) | 3.2, 5.3 |
| R8-D4 | AGRV2K configuration time unpublished | Closed by rule: ID-register poll every 1 ms, 100 ms timeout, one retry, then fault; measured (item 10) | 3.1 |
| R8-D5 | Reset state saturates the I/V converters | Closed: soft saturation in one corner only; the switch is open and the CM window tripped, so it is harmless | 5.3 |
| R8-D6 | VBUS_SENSE single channel, no capacitor on PA1 | Closed: single channel kept (ADC1 in all 9 slots); C103 = 100 pF C0G, a documented deviation (100 pF instead of none) | 3.2, 5.3 |
| R8-D7 | JTAG-default pins PA15/PB4 | Closed: the CH32V307 has 2-wire SDI only, and PA15/PB3/PB4 are plain floating GPIO at reset (datasheet pin table and SDK); ILIM_HI via U505 with R511 gives Default through reset; the reference-manual text was not retrieved, so this is a bring-up check | 3.2, 9.1 |
| R8-D8 | — | Not listed by the reviewers (numbering gap) | — |
| R8-D9 | BAT54S leakage against temperature (offset) | Closed: clamps off the DAC node, no offset term; the leakage at 50 °C is only a data gap for the I/V output node | 5.3, 8 |
| R8-D10 | ME6211 dropout at 160 mA not guaranteed | Closed: 3V3D = TLV76733, VDO ≤ 0.31 V at 221 mA → +0.772 V at 5V_SYS 4.311 V | 3.2, 4 |
| R8-D (minor) | T3 envelope 28.5 mJ within 1.5 mJ of 30 mJ | Closed: combined single-ended envelope (hardware, CM window and firmware) 25.5 / 12.8 mJ; rail stuck with the hardware leg windows only 25.6 / 12.8 mJ; stuck-pin envelope 27.2 / 13.6 mJ (9 % margin) | 5.3 |

## C.2 Independent design reviews of the v0.9 package

Four independent reviews of the v0.9 package: power states and sequencing (items #1–#34), signal integrity (F1–F12), pin-map review A (findings 1–4, written Pins-A 1…4) and pin-map review B (B-01…B-18). Every finding has a disposition; none is left open.

| ID | Finding | Severity | Resolution | Section |
| --- | --- | --- | --- | --- |
| #1 | Unpowered non-5 V-tolerant ADC pins (PA1–PA3, PC0–PC4) driven at attach and unplug | Blocking | Fixed: WCH allows exceeding the input maximum when the injected current is limited externally (±5 mA per pin, ±25 mA total, datasheet); PA1 ≤ 55 µA, PA2/PA3 ≤ 0.35 mA and PA0 ≤ 66 µA were already limited; R242–R246 330 Ω 1 % added between the DC-sense and V3A_MON nodes and PC0–PC4 → ≤ 3.0 mA per pin, ≤ 15 mA total; protection error budget re-run (e_leg 60.30 mV) | 3.1, 5.3 |
| #2 | LM27762 EN above VIN | Blocking | Fixed: EN± tied to VIN; RAIL_EN, R506 and R507 deleted | 3.2 |
| #3 | Attach regions separated only typically | Blocking | Fixed: C104 delays U102 (3.31–55.3 ms after attach with R109 470 kΩ), keeping the regions ≥ 1 ms apart; region A 28.5 µC, region B 40.2 µC | 3.2, 5.3 |
| #4 | 100 / 500 mA budgets not worst case (unpublished MCU and CPLD currents) | Blocking | Limitation L-1, with production test in every state | 5.3, 9.1 |
| #5 | Suspend margin rests on a planning value | Blocking | Limitation L-1: the MCU Stop current is a planning assumption of 1.9 mA (vendor typical 0.11 mA, maximum unpublished); S6 2.4965 mA (+3.5 µA) holds while it stays ≤ 1.9035 mA, so it is measured on every board | 5.3, 9.1 |
| #6 | Hardware trip can re-close before the IWDG reset | High | Fixed (firmware rule): IWDG 16.0–38.4 ms, below the 53 ms permit recovery; RELAY_EN latched low after a trip | 5.3 |
| #7 | Pre-close allows closing onto large DC | High | Fixed (firmware rule): 0.070 V per leg, 0.090 V per pair; closure refused above 129 / 155 mV | 5.3 |
| #8 | ≤ 5 mV pop not guaranteed | High | Fixed in part: C441 footprint (not fitted, conditional fit); bound 4.70 mV (6 % margin); measured (item 17) | 5.3, 9.1 |
| #9 | Rate or family change through lock loss with the switch closed | High | Fixed (firmware rule): switch open is the production sequence, worst case 871 ms | 5.3 |
| #10 | Every hot unplug violates the DAC-output absolute maximum | Blocking | Fixed: I/V hold-up supplies D411/D412 + C442/C443, exposure 0 ms | 3.2 |
| #11 | High mode firmware-only, no Default↔High transition | Blocking | Fixed (firmware rule): live downgrade sequence, vRd thresholds from the Type-C specification (0.66 / 1.23 V); U505/R511 give Default through reset (hardware) | 3.2, 5.3 |
| #12 | C103 100 pF only in the merged release | Medium | Fixed: 100 pF C0G in every block schematic; the automated check guards it | 3.2 |
| #13 | Full-scale 20 Hz square can false-trip at the aged capacitance | Medium | Fixed: R601–R604 115 kΩ, no trip at the aged corner (6.2 % margin) | 5.3 |
| #14 | Relay timing extrapolated | High | Limitation L-2 | 5.3, 9.1 |
| #15 | Frozen-code energies exceed the limits | Blocking | Fixed: supervisor removes them in ≤ 2.43 ms, ≤ 1.39 / 1.24 mJ; the remaining hang case: decided 25 Sep 2026, DAC lock-status check (OD-2); residual L-11 | 5.3, 9.2 |
| #16 | 5V_ANA short undetectable in programming mode | High | Fixed: ANA_SENSE supply-fault rule in every state | 3.2, 5.3 |
| #17 | MCU outputs drive the collapsed 3V3D/DVCC domain | Blocking | Fixed: U208 (Ioff) link buffer, Q207 open-drain RESETB, R305 pull-up to DVCC, R233 retired | 3.1 |
| #18 | MCLK interlock monitors the AVCC command, not VCCA | High | Fixed: U605A → Q619 on N2_AVCC_EN_G | 3.1, 5.3 |
| #19 | 3V3D fault leaves DVDD above DVCC | Blocking | Fixed: U303 EN tied to 3V3D; DVDD − DVCC ≤ +5 mV in every unplug corner | 3.2 |
| #20 | Programming mode from an unconfigured port | Blocking | Fixed (firmware rule): programming mode only after SET_CONFIGURATION | 5.3 |
| #21 | IWDG resets repeatedly in suspend | Blocking | Fixed: suspend IWDG 0.667–1.6 s with RTC wakes (≈ 3.4 µA) | 5.3 |
| #22 | Input collapse reverse-biases the regulators and the eFuse | Blocking | Fixed: D103–D105 BAT54, with the residual as limitation L-8 (0.40–0.46 V for tens of µs) | 3.2, 9.1 |
| #23 | Suspend opens the switches on live audio | High | Fixed: CPLD fade and mute with a 5.31 ms deadline; suspend ≤ 8.50 ms | 5.3 |
| #24 | Analog supply changed before the switches are open at brown-out | High | Fixed: the I/V hold-up keeps the DAC pins inside GND…AVCC; the transient during the ≤ 1.3 ms opening is limitation L-5 (measured target) | 3.2, 9.1 |
| #25 | Full-scale digital DC exceeds 15 mJ at 32 Ω | High | Decided 25 Sep 2026: stricter stream check (OD-1); 13.8 mJ at 32 Ω balanced, within the 15 mJ limit | 9.2 |
| #26 | MCU reset changes rails before the switches open | High | Limitation L-4 (fault transition; Q207 now resets the DAC at an MCU reset) | 5.3, 9.1 |
| #27 | ESS VCCA-before-DVCC mislabelled as hardware-backed | High | Fixed: U605B → Q620 on AUD_EN | 3.2, 5.3 |
| #28 | TPS25200 IN above 20 V at a hard short | Blocking | Fixed: D102 SMDJ12A, ≤ 16.99 V without current-limit credit | 3.2 |
| #29 | High-mode power not guaranteed at VBUS 4.40 V | High | Limitation L-3 (rated at ≥ 4.60 V) | 5.2, 9.1 |
| #30 | AUD_EN not guaranteed low at reset (TLV767 pull-up) | Medium | Fixed: R508 10 kΩ (≤ 0.11 V at 10 µA) | 3.2 |
| #31 | CPLD JTAG pins float | Medium | Fixed: R238 JTMS pull-up, R239 JTCK pull-down | 7 |
| #32 | Programming header back-powers 3V3M | Medium | Fixed: J201 pin 1 via R241 0 Ω, not fitted | 7 |
| #33 | USB-C receptacle pin map open | Blocking | Limitation L-7 (pre-layout gate G-1) | 7, 9.1 |
| #34 | Shorted photorelay latent | Medium | Limitation L-6 (production test) | 9.1 |
| F1 | Frozen DAC code with the MCLK run signal high | High | Fixed as #15; the remaining hang case: decided 25 Sep 2026, DAC lock-status check (OD-2); residual L-11 | 5.3, 9.2 |
| F2 | Power-up pop margin on assumed bounds, no fallback | Medium | Fixed in part: C441 footprint as a conditional fallback; measured (item 17) | 5.3 |
| F3 | Shorted photorelay latent | Medium | Limitation L-6 | 9.1 |
| F4 | Offset “guarantee” rests on assumed DAC matching | Medium | Fixed: the statement is now “guaranteed by 100 % test at 25 °C plus datasheet-bounded drift”; VREF trims R446/R447 (not fitted) are the remedy, change-controlled with the CM window | 2, 5.3 |
| F5 | Jack ESD does not protect the OPA1622 with the switch closed | Medium | Fixed (alternate): BLM15PX330SN1D bead approved for R417–R420 (ESD current ≤ 0.46 A); 0 Ω fitted by default | 3.1 |
| F6 | No diagnostic coverage of latent DC-path faults | Medium | Fixed (firmware rule): boot self-test at every attach | 5.3 |
| F7 | High-mode power at 4.40 V typical only | Medium | Limitation L-3 | 9.1 |
| F8 | RAILS_OK_N rule specified three ways | Medium | Fixed (firmware rule): one overload rule | 5.3 |
| F9 | Full-scale digital DC into 32 Ω balanced | Low | Decided 25 Sep 2026: stricter stream check (OD-1); 13.8 mJ at 32 Ω balanced, within the 15 mJ limit | 9.2 |
| F10 | Every hot unplug outside the DAC absolute maximum | Low | Fixed: I/V hold-up supplies | 3.2 |
| F11 | Leg-to-leg plug or cable short of one channel not detected in High mode | Low | Limitation L-9: an overload, not DC (≈ 0.55 W per package, TJ ≈ +26 °C, outside TI’s one-amplifier continuous-short rating); bring-up survival test; firmware overload policy when the rails sag | 5.1, 8, 9.1 |
| F12 | Simplification: LED-drive precision parts not needed | Low | Rejected: the 0.1 % reference and sense parts carry the IF ≥ 5.0 mA guarantee at 5V_SYS 4.17 V (+6.3 %) | 5.3 |
| Pins-A 1 | LM27762 EN from a 3.3 V MCU output against VIN | High | Fixed: EN± tied to VIN | 3.2 |
| Pins-A 2 | U102 clamp maximum 5.55 V against the LP5907/LM27762 5.5 V recommended maximum | Medium | Limitation L-10: 5.50–5.55 V only for a source above vSafe5V (> 5.50 V, outside the USB specification); the LP5907/LM27762 are then 0–50 mV above the recommended maximum, inside the absolute maxima | 9.1 |
| Pins-A 3 | USBLC6 VRM 5.25 V against 5V_SYS 5.55 V | Low | Limitation L-10: the leakage above 5.25 V is covered by the measured suspend current (production test, L-1); VBR ≥ 6.0 V | 9.1 |
| Pins-A 4 | U303 EN before its IN | Medium | Fixed: U303 EN = 3V3D | 3.2 |
| B-01 | C103 two parts | Medium | Fixed: one part, 100 pF C0G | 3.2 |
| B-02 | Test pads only in the merged netlist; PROG note | Low | Closed: 52 pads netlisted (TP746–TP753 added); pad symbols are drawn at schematic capture | 7 |
| B-03 | J201/J202/TP732 BOM pin count 0 against netlisted pads | Low | Accepted as a convention: copper-pad designators (no part), counted separately by the consistency check | 8 |
| B-04 | J701/J702 pin maps unverified | High | Limitation L-7 (gate G-2) | 7 |
| B-05 | JTMS/JTCK undefined | Medium | Fixed: R238/R239 | 7 |
| B-06 | U202 exposed pad and marking undocumented by AGM | Low | Closed: EP to GND per the AGM reference; gate G-3 checks the EP size and net (AGM drawing if available, otherwise the LCSC footprint, EP 2.7 mm to GND, against a real part) and the joint is X-rayed; the AG32VF marking remark stays in G-6 | 7 |
| B-07 | DVCC/VCCA LC filters under-damped | Medium | Fixed: C439/C440 2.2 µF (Q ≤ 7.3) | Notes v0.8 |
| B-08 | Oversized 2520/3225 land | Medium | Fixed: 2520-only lands | 4, 7 |
| B-09 | LED polarity not confirmed | Medium | Closed: pre-layout gate G-4 (DMM diode test of two samples against the tape pocket and body mark before routing) | 7 |
| B-10 | Inputs floating in some states | Low | Accepted: firmware-defined; listed for completeness by the reviewer | Notes v0.8 |
| B-11 | OPA2210 BOM price ≈ 2.7× too low; 16 in stock | Low | Closed: unit price corrected to US$ 2.6708 (LCSC C1849415, 10+ tier, 25 Sep 2026; 1+ US$ 3.1151); 11 pieces pre-ordered now from the 16 in LCSC stock (10 for five boards plus attrition, no spares); no compliant pin-compatible alternative exists, so the sourcing risk is mitigated by buying now | 6, 9.1, 10 |
| B-12 | J201 pin 1 named 3V3M_SENSE, but the WCH-LinkE 3V3 is an output | Low | Fixed: R241 not fitted, net N2_J201_3V3 | 7 |
| B-13 | Standard PCBA residual actions | Low | Accepted: order remarks in the layout package | 7 |
| B-14 | ES9018K2M VOL specified only at 100 µA against 0.72 mA pull-ups | Low | Closed: I²C pull-ups R207/R208 10 kΩ (sink ≤ 0.34 mA; tr ≤ 0.85 µs at 100 pF, standard mode) | 3.1, 5.3 |
| B-15 | U301 exposed-pad solder-mask rule missing from the layout package | Low | Closed: exposed pad solder-mask-defined per the ESS land pattern, 50–70 % windowpane paste; the 0.30 mm thermal vias are unfilled and tented since the cost patch (the ESS note allows plugging or tenting) | 7 |
| B-16 | U206 6-pin package not in every list | Medium | Fixed: 6-pin SOT363 in every list | 7 |
| B-17 | Stale U202 pin-21 table in a supporting design report | Low | Closed: text only; the netlist is correct | — |
| B-18 | Mounting holes without designators | Low | Closed in the layout package: MH1–MH4 (GND) and FID1–FID7 designators; symbols drawn at schematic capture | 7 |

## C.3 Seventh review round (R7-A…R7-E, full design check of Spec v0.7)

Historical text: the cost gate quoted here was retired on 25 Sep 2026; cost is now reported for information.

Finding IDs (the reviewers’ own numbers are kept, written for example as R7-C PR-1): R7-A = power (P-1…P-22), R7-B = analog (AN-1…AN-13), R7-C = protection (PR-1…PR-19), R7-D = digital (DG-1…DG-21), R7-E = parts and cost (BM-1…BM-15). The five independent reviews covered Spec v0.7, Schematic Design Notes v0.6 and Parts List v0.5 before any hardware is bought, because development and test boards are hard to get in Taiwan. They raised three blockers (PR-1 = P-1 = DG-3, the AVCC_EN contention; DG-1, the CPLD programming port; BM-1, the unverified OPA2210 stock) and 13 high findings; findings raised by more than one reviewer are cross-referenced. The last row closes R6-B2 of the sixth round.

| ID | Finding | Disposition in v0.8 | Status |
| --- | --- | --- | --- |
| R7-A P-1 | AVCC_EN has no series resistor, so the Q611 latch kill fights the MCU output (= PR-1, DG-3) | New R307 10 kΩ from PC11 to the LP5907 EN node; Q611 pulls against 0.33 mA | Closed – design |
| R7-A P-2 | RAILS_OK_N loaded by R623/C607 and R617/C606 (step to ≈ 1.57 V); capture timing not real; PGOOD pulled up while the LM27762 is unpowered (= PR-2) | R505 → 10 kΩ to 5V_ANA; R522 10 kΩ into PB4; R623 → 1 MΩ, C607 → 10 nF; rise < 1 µs; new blanking and arming times | Closed – design, verify |
| R7-A P-3 | 2.2 Ω MPN and LCSC number disagree (0805W8F220JT5E is 22 Ω) (= AN-8, BM-3) | MPN corrected to UNI-ROYAL 0805W8F220KT5E (C17521) | Closed – documentation |
| R7-A P-4 | U504 CE can exceed VIN + 0.3 V at unplug under load | New R521 10 kΩ from AUD_EN (PC10) into U504 CE; clamp current ≤ 0.2 mA | Closed – design |
| R7-A P-5 | Suspend budget leaves out the D+ pull-up and uses a low LDO Iq | Budget 2.31 mA (D+ pull-up 0.20 mA, ME6211 60 µA typ; margin 0.19 mA) | Closed – documentation |
| R7-A P-6 | Negative-rail headroom mixed average and peak currents and used a low cable drop | Peak-current method with 0.3 Ω at the real current (host 4.75 V): +0.05 V worst case at 70 mA; 70 mA envelope at 4.4 mm 32 Ω | Closed – design |
| R7-A P-7 | VBUS has ceramic capacitors only and no surge clamp | DNF D102 (TVS at J101, VRWM 6.0 V); hot-plug test on board 1; fitted only if the peak exceeds 5.8 V | Closed – design, verify |
| R7-A P-8 | 3V3D and 1V3 have no discharge path | Bleeders R524 2.2 kΩ (3V3D) and R525 1 kΩ (1V3); AUD_EN off ≥ 100 ms before re-enabling | Closed – design |
| R7-A P-9 | ME6216A13M3G pinout not recorded; some claims differ from the datasheet (= BM-4) | SOT23-3: pin 1 VSS, pin 2 VOUT, pin 3 VIN; 250 mA max, ±15 mV, Pd 0.54 W; stock ≈ 3,100 | Closed – documentation |
| R7-A P-10 | USB-C J101 pin functions not checked against the drawing | A6/B6 and A7/B7 tied at the connector; drawing and sample check before layout | Open – verify |
| R7-A P-11 | Minimum delay from ANA_EN to RAIL_EN not specified | RAIL_EN ≥ 10 ms after ANA_EN | Closed – firmware rule |
| R7-A P-12 | The rails start inside the TPS2553 limit; the inrush can look like a fault | Firmware ignores its ANA_FAULT classification for 20 ms after RAIL_EN; bench check in Default mode at 4.40 V | Closed – firmware rule, verify |
| R7-A P-13 | PGOOD pulled up to 3V3M while the LM27762 VIN is 0 V (= PR-9) | Moot: PGOOD now pulled up to 5V_ANA (P-2) | Closed – design (moot) |
| R7-A P-14 | LM27762 dissipation quoted at the design corner only | 0.39 W at 5.25 V (≈ +24 °C), 0.46 W at 5.5 V | Closed – documentation |
| R7-A P-15 | U504 evaluated only up to 5.25 V | 0.20 W at 5.25 V, 0.24 W at 5.5 V (300 mW rating; 160 mA including the bleeders) | Closed – documentation |
| R7-A P-16 | 3V3A discharge time used the typical LP5907 discharge | 5.5 ms worst case used in the power-down timing | Closed – documentation |
| R7-A P-17 | CC firmware thresholds not stated | 0.66 V and 1.23 V with hysteresis; only the CC pin with Rp is active | Closed – firmware rule |
| R7-A P-18 | VBUS_SENSE source impedance vs ADC sampling for the < 5 µs watchdog (= DG-5) | ADC1 continuous on VBUS_SENSE with the analog watchdog, 7.5-cycle sampling at 12 MHz: ≈ 3.8 µs worst-case latency | Closed – design |
| R7-A P-19 | Shell capacitor C101 overstressed by ESD; CC lines without ESD devices | R107 0 Ω (shell to GND), C101 DNF; CC1/CC2 rely on the 10 kΩ series resistors | Closed – design |
| R7-A P-20 | BOM and documentation details (C15849 rating, ME6211 dropout, LM27762 symbol pins) | C15849 is 50 V; ME6211 dropout 120 mV typ at 100 mA; LM27762 PGOOD = pin 1, FB− = pin 7 | Closed – documentation |
| R7-A P-21 | Start-up into the ME6211 and ME6216 foldback current limits | AUD_EN cycled 100 times at 4.40 V and 5.25 V on Stage 2 and at bring-up; MCLK off until DVDD is up | Open – verify |
| R7-A P-22 | FB501 + C507 LC resonance (≈ 80–100 kHz) | LM27762 VIN ripple checked at light load; 0.47–1 Ω + 1 µF damping if needed | Open – verify |
| R7-B AN-1 | Jack pin numbers not documented or verified | Pin-number placeholders in the Notes; buy 2 samples of each jack and continuity-test every pin before layout; checklist rule | Open – owner action |
| R7-B AN-2 | OPA1622 clean swing ends ≈ 1.3 V from the rail, not 0.9 V | Swing model (¦rail¦min − 1.3 V)/(1 + 8 Ω/Rleg); calibrated 1.55 Vrms per leg; −4.5 dB before calibration; new power targets | Closed – design |
| R7-B AN-3 | DAC reset state not modelled (outputs driven to ground) | Power-down keeps RESETB high and stops MCLK (MCLK_EN) before AVCC goes off; AVCC-on step ≈ 18–35 mV measured; option B kept; pop at risk until measured | Closed – design, verify |
| R7-B AN-4 | Pin-level connections of the OPA1622 and the ES9018K2M not documented | OPA1622 VSON-10 and ES9018K2M QFN-28 pin lists in the Notes; OPA1622 pin 3 to GND | Closed – documentation |
| R7-B AN-5 | Leg-network noise underestimated | ≈ 11.5 nV/√Hz (13.0 with the OPA1622); DR 117.4 / 118.6 dB(A) at 1.55 Vrms | Closed – documentation |
| R7-B AN-6 | Default-mode unbalanced peak rule left out the charge-pump factor | 53 mA + 2 × 1.03 × Ipk in the power table | Closed – documentation |
| R7-B AN-7 | The 0.47 Ω fallback cannot fix capacitive-load instability | 0 Ω links kept; 1 nF and long-cable test; 2.2–4.7 Ω only as a last resort | Closed – documentation, verify |
| R7-B AN-8 | 2.2 Ω MPN decodes to 22 Ω (= P-3, BM-3) | UNI-ROYAL 0805W8F220KT5E (C17521) | Closed – documentation |
| R7-B AN-9 | Disabled-leg DC path and DC-tier gating | AVCC_EN only while AMP_EN is high and the rails are good; slow tier armed whenever AVCC_EN is high | Closed – firmware rule |
| R7-B AN-10 | DC-sense accuracy depends on unspecified MCU pin leakage | Superseded by the redesigned DC-sense network (R7-F F3, Section C.2): leakage error ≤ 15 mV at the leg inside the full error budget | Closed – design, verify |
| R7-B AN-11 | ESS application-circuit figure (p. 27) could not be compared | Owner compares it with the design by eye before the schematic | Open – owner action |
| R7-B AN-12 | Small numeric inconsistencies (VREF time constant) | τ = 7.74 kΩ × 330 pF = 2.55 µs | Closed – documentation |
| R7-B AN-13 | J701 JLCPCB stock is 21, not 721 (= BM-2) | Buy 7 (5 + 2) into the JLCPCB library | Open – owner action |
| R7-C PR-1 | AVCC_EN / Q611 contention (blocker; = P-1, DG-3) | R307 10 kΩ; every enable the latch kills now has a series resistor (R507, R618, R307) | Closed – design |
| R7-C PR-2 | The capture claim ignored the loading of RAILS_OK_N (= P-2) | R505 10 kΩ to 5V_ANA, R522, R623 1 MΩ, C607 10 nF; ≈ 35 µs capture genuine | Closed – design, verify |
| R7-C PR-3 | LM393 input bias current (up to 400 nA) into the 1 MΩ loads can false-trip | Superseded by the LM339 windows (R7-G G2, Section C.2); the interim fix was U601 → TI LM393BIDR with 147 kΩ / 10 kΩ thresholds | Closed – design |
| R7-C PR-4 | Blanking hides faults present at power-up | PGOOD timeout 20 ms, start-up DC watch at 1 kHz (> 0.5 V drops RAIL_EN), first power-up without headphones | Closed – firmware rule |
| R7-C PR-5 | Arming time assumes a fully charged blanking capacitor | FLT_CLR ≥ 600 ms after ANA_EN or rails off; second FLT_CLR at PGOOD good + 20 ms; AMP_EN ≥ 250 ms after PGOOD good | Closed – firmware rule |
| R7-C PR-6 | The FLT interrupt leaves the MCU driving the enables against the kill FETs | ISR drives AMP_EN, RAIL_EN, AVCC_EN, AUD_EN and RESETB low within 50 µs, then captures the cause; IWDG enabled | Closed – firmware rule |
| R7-C PR-7 | MCU pin configuration on the latch nodes not specified (= DG-9) | FLT, RAILS_OK_N and ANA_FAULT floating inputs without internal pulls; unit test on the GPIO table | Closed – firmware rule |
| R7-C PR-8 | Cause classification used instantaneous readings | Pre-trip 0.5-s average of each DC-sense channel (ring buffer) | Closed – firmware rule |
| R7-C PR-9 | PGOOD pull-up to 3V3M while VIN = 0 (= P-13) | Moot: pull-up on 5V_ANA | Closed – design (moot) |
| R7-C PR-10 | No U601 supply decoupling | C612, C613 100 nF | Closed – design |
| R7-C PR-11 | DC-tier ADC network source impedance vs sampling | Superseded by the redesign (R7-F F3, Section C.2): 4.7 kΩ source, dithered 1 kHz sampling, auto-zero, 3V3A monitor | Closed – design, verify |
| R7-C PR-12 | DC-sense node below 0 V on a negative leg | Fixed by the redesign (R7-F F3, Section C.2): the node stays within 0.29–2.85 V at every corner | Closed – design |
| R7-C PR-13 | Timing numbers in the documents | New timing: stretch 1.5–7.7 ms, Q609 on 1.9–9.4 ms, re-blank 19–94 ms, arming 51–186 ms | Closed – documentation |
| R7-C PR-14 | Documentation and netlist ambiguities | Netlist details in the Notes (BAV99 pins, comparator inputs, U601 pins, D605/D606, Q607/Q608/Q611 gates, OPA1622 pin 3) | Closed – documentation |
| R7-C PR-15 | LM393DR2G ESD rating only 250 V HBM | LM339DR ESD ±2 kV HBM (TI datasheet SLCS006Z); the LM339BIDR (lower input bias) is preferred if JLCPCB stocks it (R7-G G2 topology) | Closed – design |
| R7-C PR-16 | C605 discharges directly into the MCU pin or the TPS2553 FAULT output | R638 1 kΩ in series with D606 | Closed – design |
| R7-C PR-17 | Q609 slow ramp near the EN threshold | Moot with the PR-2 fix (the latch acts first) | Closed – design (moot) |
| R7-C PR-18 | A quick replug may start with the latch cleared | Harmless: the enables are pulled down | Accepted (v1) |
| R7-C PR-19 | Unplug wording; PGOOD latch backstop ineffective | Wording corrected; PGOOD goes bad at VBUS ≈ 3.70 V while 5V_ANA is ≈ 3.6 V, so Q610 turns on | Closed – documentation |
| R7-D DG-1 | CPLD programming port specified as 4-wire JTAG; the AGRV2KQ32 has only JTMS and JTCK (blocker) | J202 = 3V3D (sense), JTCK (25), JTMS (24), NRST (4), GND for 2-wire Compact-JTAG (AGM Supra with a J-Link or AGM DAP-Link); the owner buys the probe | Closed – design; Open – owner action |
| R7-D DG-2 | The clock plan needed three global clock inputs; only PIN_1 is a dedicated clock pin | Family clock on PIN_1 → PLL; 80 MHz input removed (R209 deleted); SCK through ALTCLKCTRL; FIFO written in the SPI domain | Closed – design |
| R7-D DG-3 | MCU output fights the Q611 kill FET (= P-1, PR-1) | R307 10 kΩ | Closed – design |
| R7-D DG-4 | No defined way to power the CPLD for programming | Programming mode: PROG pad (TP732, PC13) at reset powers 3V3D without USB configuration; the probe senses 3V3D only | Closed – design |
| R7-D DG-5 | The < 5 µs watchdog needs a dedicated ADC (= P-18) | ADC1 continuous on VBUS_SENSE with the analog watchdog; ISR on a fast vector | Closed – firmware rule |
| R7-D DG-6 | MCLK can back-power VCCA after a latch trip | ISR drops AUD_EN and RESETB first; MCLK_EN stops X201 before AVCC goes off | Closed – firmware rule + design |
| R7-D DG-7 | Missing DAC register stop_div | Register 10 stop_div = 0 above 96 kHz; register 1 = 0x80; both in the unit tests | Closed – firmware rule |
| R7-D DG-8 | Idle levels of the MCU → CPLD link conflict with the 3V3D-off rule | LINK_FRAME active-high (idle low), CPOL = 0, outputs low or input while AUD_EN is low; R224/R225 33 Ω; CPLD weak pull-downs | Closed – design |
| R7-D DG-9 | MCU internal pulls on the latch-node inputs (= PR-7) | Floating inputs, also in low-power code; ANA_FAULT open-drain low only | Closed – firmware rule |
| R7-D DG-10 | Suspend path unproven on the CH32V307 | Stage 2 gate: Stop with LPDS, PHY suspended, wake on resume, HSE/PLL restart within 10 ms; fallback EXTI wake | Open – verify on Stage 2 |
| R7-D DG-11 | One PLL must lock at 22.5792 and 24.576 MHz | Stage 2 gate; fallback BCLK ÷ 1 from X202/X203 | Open – verify on Stage 2 |
| R7-D DG-12 | SPI clock quantised; 36 MHz is the only workable setting | 36 MHz (68 % utilised at 384 kHz); reads at ≤ 18 MHz; second lane reserved | Open – verify on Stage 2 |
| R7-D DG-13 | CPLD supply isolation | FB202 branch for VDD33/VDDA33; C213–C216 plus C220 1 µF at VDDA33 | Closed – design |
| R7-D DG-14 | J201 has no NRST or BOOT0 pad | J201 = 3V3 (sense), SWDIO, SWCLK, NRST, GND; BOOT0 and USART1 TX pads | Closed – design |
| R7-D DG-15 | X201 load and duty margin | CPLD branch removed; X201 drives only the DAC XI (≈ 10 pF + trace, ≤ 20 mm); duty measured at XI | Closed – design |
| R7-D DG-16 | DAC pin details not written down | ADDR to DGND through R306 (0x48, written 0x90), XO open, DATA_CLK/DATA1/DATA2 = BCLK/LRCLK/SDATA, GPIO2 → DAC_LOCK | Closed – documentation |
| R7-D DG-17 | RESETB injection at unplug or brownout | R308 1 kΩ in series; RESETB dropped first in the ISRs | Closed – design |
| R7-D DG-18 | RCT6 supply-pin count; VBAT not tied | VBAT to 3V3M; one 100 nF per supply pin plus extras at VDD_2/VDD_4; C221 1 µF at VDDA | Closed – design |
| R7-D DG-19 | Link word order | 8-bit frames or a halfword swap in the CPLD; known-pattern test; MISO returns the FIFO level | Closed – documentation |
| R7-D DG-20 | Descriptor details not stated | Feedback wMaxPacketSize 4; isochronous OUT ≥ 392; non-integer 44.1 k-family feedback; bcdDevice bumps; VID/PID 1209:0001 for private testing only | Closed – documentation |
| R7-D DG-21 | The CPLD is AG32 silicon in pure-CPLD mode; the toolchain is Windows-centred | Dynamic current measured on Stage 2; CPLD CI builds on a Windows runner or Supra on Linux | Open – verify |
| R7-E BM-1 | OPA2210 supply and price rest on a 12-month-old JLCPCB stock figure (blocker until verified) | Owner checks the JLCPCB stock now (buy 12 if ≥ 12); OPA1612AIDR fallback designed in (V+ 3.90 V); cost sensitivities stated | Open – owner action |
| R7-E BM-2 | J701 JLCPCB stock is 21, not 721 (= AN-13) | Buy 7 into the JLCPCB library | Open – owner action |
| R7-E BM-3 | Wrong MPN on the 2.2 Ω line (= P-3, AN-8) | 0805W8F220KT5E | Closed – documentation |
| R7-E BM-4 | ME6216A13M3G package and pinout (= P-9) | SOT23-3 (“M3”), pinout and 250 mA recorded | Closed – documentation |
| R7-E BM-5 | Cost-model prices stale and too low for some passives | Re-priced at LCSC on 24 Sep 2026; 1,691 NTD per board | Closed – documentation |
| R7-E BM-6 | J702 flagged “assembly difficulty high” by JLCPCB | Allowed in Economic PCBA; PJ-332-6A (C962626) not qualified; kept and noted | Accepted (v1) |
| R7-E BM-7 | Mixed price basis (LCSC and JLCPCB) | Noted; JLCPCB-vs-LCSC differences ≈ −24 NTD per board (indicative) | Info |
| R7-E BM-8 | C15849 is rated 50 V, not 25 V | Corrected | Closed – documentation |
| R7-E BM-9 | 2.2 µF 0402 6.3 V at 3.3 V loses capacitance | Optional 2.2 µF 16 V 0603 (C23630) for C303/C304 not adopted | Not adopted (optional) |
| R7-E BM-10 | Green LED current about 3× lower than stated | R701 2.2 kΩ → 1 kΩ (R702 stays 2.2 kΩ) | Closed – design |
| R7-E BM-11 | No input-voltage margin on the LP5907 and LM27762 above 5.5 V | Accepted for v1; VBUS measured on the chargers used | Accepted (v1) |
| R7-E BM-12 | Counts and stale figures in the documents | Counts corrected (152 BOM lines, 263 placed parts, 20 Extended types) and stock figures refreshed | Closed – documentation |
| R7-E BM-13 | Fee rules vs JLCPCB’s Economic PCBA table (08 Sep 2026) | All match | Info |
| R7-E BM-14 | Economic PCBA limits | All met (single-sided placement, 2–50 pcs, standard stack-up) | Info |
| R7-E BM-15 | Part ratings versus use | All check out | Info |
| R6-B2 (sixth round) | Residual DC blind region below the detection tiers is a product-safety decision | Residual below 0.2 V per leg (≤ 2.5 mW into 16 Ω, ≤ 1.25 mW into 32 Ω) accepted by the owner on 24 Sep 2026 | Closed – owner decision |

## C.4 Seventh round, external reviews (R7-F, R7-G, R7-H)

Historical text: the cost gate quoted here was retired on 25 Sep 2026; cost is now reported for information.

Finding IDs: R7-F = external review 1 (F1–F6), R7-G = external review 2 (G1 negative ADC range, G2 BAV99 threshold, G3 owner decision, G4 release conditions), R7-H = external review 3 (H1–H8 plus minors). The three reviews covered the previous set (Spec v0.7, Schematic Design Notes v0.6 and Parts List v0.5), so many items were already fixed in the R7-A…R7-E round (Section C.1); the new design changes of the amendment of 24 Sep 2026 are the LM339 window detector, the redesigned slow DC-sense network, the firmware tiers per leg and per balanced pair, C421 33 pF with the DAC-pin analysis (and, by owner decision of the same day, the fitted DAC-pin clamps D405–D408), the adaptive AMP_EN at power-down and the stream DC guard. The C.1 rows they supersede (R7-B AN-10, R7-C PR-3, PR-11, PR-12 and PR-15) are marked there.

| ID | Finding | Disposition in the v0.8 amendment | Status |
| --- | --- | --- | --- |
| R7-F F1 / R7-H H1 | R623/C607 and R617/C606 load RAILS_OK_N (R505 100 kΩ): the node steps to ≈ 1.57 V, Q610 capture up to 15–19 ms, not 35 µs | Already fixed in R7-A P-2 / R7-C PR-2, using both of R7-H’s options: R505 10 kΩ to 5V_ANA (TI: “connect a 10-kΩ pullup resistor (RPU) to VIN”), R623 1 MΩ, C607 10 nF. With the real loads RAILS_OK_N steps to 4.31–5.15 V (0.98 × 5V_ANA) and crosses 2.5 V in ≈ 1.0–1.3 µs; the ≈ 35 µs capture holds. New rule: RAILS_OK_N carries only ≥ 1 MΩ RC branches and high-impedance gate/MCU inputs (R522). The pulse test emulates the real open-drain release, never a low-impedance generator | Closed – design, verify |
| R7-F F2 / R7-H 2 | AVCC_EN has no series resistor | Already fixed: R307 10 kΩ (R7-A P-1) | Closed – design |
| R7-F F3 / R7-G G1 / R7-H 3 | Slow DC-sense network: 333 kΩ source vs the 50 kΩ ADC limit and 1 µA pin leakage (1 V leg error); the node reaches −0.18 V, outside the ADC range | Redesigned: 15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF, source 4.7 kΩ, node 0.29–2.85 V at every corner, leakage error ≤ 15 mV at the leg, full error budget (≤ 44 mV per leg, ≤ 88 mV per pair), auto-zero, 3V3A monitor | Closed – design, verify |
| R7-F F4 | VREF tracking does not prove DAC-pin safety in a loaded failed-output shutdown; the test measures VREF, not the DAC pins; “below junction conduction” is not an ESS allowance | Analysed: with a stuck leg the I/V positive common-mode margin is gone 0.11–0.39 ms after the kill while 3V3A needs 3.2–5.6 ms to fall below 0.3 V, so the DAC pins are uncontrolled for up to ≈ 5.5 ms (≈ −0.6 V with ≤ 3.9 mA, or ≈ 0.33 V above AVCC); stated as a residual risk of an already-failed output stage. Unplug excursion reduced from ≤ 0.13 V to ≤ 15 mV (C421 330 pF → 33 pF). BAT54S clamps D405–D408 (DNF footprints at first; fitted by owner decision of 24 Sep 2026): each DAC pin held within ≈ −0.40 V…AVCC + 0.40 V (≤ 4.4 mA per pin). Tests probe the four DAC pins (R423–R426 DAC-side pads) against AVCC_L/AVCC_R during failed-output emulation and unplug (pass: within −0.4 V…AVCC + 0.4 V) | Closed – design (clamps fitted, owner decision 24 Sep 2026), verify; small residual: ≤ 0.4 V beyond the rails for ≤ 5.5 ms after an output-stage failure |
| R7-F F5 | Residual-DC acceptance used 0.2 V across the headphone, but the thresholds are per leg: opposite-sign legs give 0.4 V across a balanced driver (4× the power) | Firmware tiers now act on every leg and on each balanced pair difference (L+ − L−, R+ − R−); T4 0.11 V nominal, worst-case trip ≤ 0.198 V differential after the full error budget, so the accepted residual ≤ 2.5 mW (16 Ω) / 1.25 mW (32 Ω) holds for balanced headphones too | Closed – firmware rule, verify |
| R7-F F6 | The classifier can mistake bass for DC (instantaneous 33 ms-filtered reading) | Already changed (R7-C PR-8) to the pre-trip rolling 0.5-s average plus event history; added: events that match neither pattern are logged as “ambiguous” and counted toward the DC three-strike limit (conservative, deliberate) | Closed – firmware rule |
| R7-G G2 | The hardware worst case relies on an extrapolated BAV99 VF at ≈ 0.4 µA (the datasheet guarantees only 1 mA) | Topology changed: two TI LM339DR quad comparators, one window per leg directly on its filter node, no diode-OR, so the threshold is set only by the divider, the comparator offset and the bias current, all datasheet-guaranteed. Leg trip 0.43–0.67 V worst case; 1 V ≤ 125 ms, 2 V ≤ 47 ms; rail-stuck energy ≤ 21.4 mJ (16 Ω) / 11.0 mJ (32 Ω) | Closed – design |
| R7-G G3 | Owner decision on the blind region; suggests testing 0.1 V / 5 s | Owner accepted (24 Sep 2026). T4 is 0.11 V nominal (≤ 0.198 V guaranteed, 2-s window, ≤ 3 s), effectively the suggested 0.1 V; the worst-case band is stated | Closed – owner decision |
| R7-G G4 | Release conditions: ME6216/LM27762 thermal data, OPA2210 SPICE, Stage 1/2 gates | ME6216 Pd 0.54 W (datasheet) vs 0.12 W and LM27762 RθJA 62.2 °C/W (datasheet, ≈ +24 °C at 0.39 W): closed. OPA2210 stability SPICE: open, before layout. Stage 1 is optional (owner constraint); its gates run on Stage 2 | Partly closed |
| R7-H H2 | Legs live while the DAC powers up or down: unbounded DAC transients reach the jack | Covered by R7-B AN-3: the DAC-reset step is measured; firmware options A (legs first), A′ (AUD_EN together with AVCC_EN, allowed by ESS: DVCC with or after VCCA) and B (DAC first, legs last), chosen by the pop measured on main board 1 (Stage 2 has no output stage) | Open – verify |
| R7-H H3 | VBUS watchdog latency in a 7-channel scan | Already fixed (R7-D DG-5): ADC1 converts VBUS_SENSE continuously; the scanned channels are on ADC2 | Closed – design |
| R7-H H4 | LM27762 PGOOD state while unpowered | Moot: PGOOD now pulled up to 5V_ANA (TI recommendation) | Closed – design |
| R7-H minor (power-down) | Power-down pop: AMP_EN goes low while 3V3A ≈ 0.24 V | AMP_EN now drops when the 3V3A monitor reads < 0.1 V (typically t0 + 4.7 ms, pop ≈ 4.5 mV at 300 Ω including the sense-bias path), with a deadline at t0 + 5.8 ms (worst-case discharge: 3V3A ≈ 0.28 V, pop ≈ 12.7 mV at 300 Ω, measured); suspend level typically ≈ 8.2 ms, at worst 9.3 ms after the last bus activity | Closed – design, verify |
| R7-H H5 | Step 4 assumes pair-symmetric DAC pins with DVCC off | Same as H2 / AN-3; test added: I/V outputs and legs scoped during the AVCC ramp with RESETB low; fallback A′ | Open – verify |
| R7-H H6 | The suspend budget omits the LEDs | Firmware rule: both LEDs off in suspend (0 mA in the budget) | Closed – firmware rule |
| R7-H H7 | Step 2 wording; resume latency | Wording already “≥ 600 ms after ANA_EN or after the rails last went off”; resume-to-audio ≈ 1.0 s (0.6 s blanking charge, rails, 250 ms, DAC init, 105 ms soft start) stated | Closed – documentation |
| R7-H H8 | DC in the program material trips the board | New firmware stream DC guard in the MCU before the DAC: no zero crossing for 25 ms and ¦mean¦ ≥ 0.25 FS → output zeroed at once; 0.2-s mean ≥ −30 dBFS → soft mute, unmute after 1 s below −40 dBFS. DC in the program material therefore mutes (reported to the host log) instead of latching | Closed – firmware rule |
| R7-H envelope | The owner decision should cover the whole sub-hardware-trip envelope | Envelope stated per tier (Section 5.3), all ≤ 28.2 mJ at 16 Ω and ≤ 14.1 mJ at 32 Ω | Closed – documentation |
| R7-H minors | C211 note “≈ 38 µC”; X201 “208 ×”; C605 discharge through Q610 | C211 note ≈ 42.5 µC; “≈ 208 ×”; C605 discharges through Q610 in a sub-µs ≈ 0.5 A pulse, fine for a 2N7002, so no series resistor is added there | Closed – documentation |
| R7-F cost | Workbook consistent at 8,248 NTD (v0.5) | 8,487 NTD / 1,697 NTD per board after these changes; now 8,594 NTD / 1,719 NTD per board with the DAC-pin clamps fitted (Section 6) | Info |

## C.5 Sixth review round (R6-A, R6-B, R6-C and R6-D of Spec v0.6)

Historical text: the cost gate quoted here was retired on 25 Sep 2026; cost is now reported for information.

Finding IDs: R6-A = review 1 (latch timing, power-down, suspend, fault test, cause capture, cost gate), R6-B = review 2 (worst-case protection, blind region, suspend wording, fixture in the gate), R6-C = review 3 (3-strike lockout, mux inputs, OPA2210 single source, minors), R6-D = review 4 (pop vs offset, latch on controlled power-down, LDO dissipation, DAC centre voltage, OPA2210 output swing, watchdog timing, rail floating, 3V3D bulk, minors). The reviews covered Spec v0.6, Schematic Design Notes v0.5 and Parts List v0.4; findings raised by more than one reviewer are dispositioned together. Kept as history: the dispositions as issued in v0.7; R6-B2 is closed in Section C.1 (residual below 0.2 V accepted by the owner on 24 Sep 2026), and where the seventh round changed a value, Sections C.1 and C.2 and the body of this specification are current.

| ID | Finding | Disposition in v0.7 | Status |
| --- | --- | --- | --- |
| R6-A1 | Latch can miss short PGOOD faults; latch delay missing from the energy figures | C605 moved to node F (power-on set and ≥ 1.5 ms set-request stretch); 47 kΩ latch pull-ups; capture ≈ 35 µs (budget 0.1 ms); Q609 slowed to a static interlock (6–32 ms), so a PGOOD glitch either latches or does nothing; energy computed through the rail-disable instant | Closed – design, verify |
| R6-A2 | Power-down does not establish safe DAC-pin voltage or the post-trip energy bound | VREF tracks AVCC (C421 330 pF; D401, C422 removed); DAC analog off first with the I/V and legs powered, rails last; latch also kills AVCC_EN (Q611); after-trip bound 0.51 mJ (both rails, C +10 %, maximum rails) + ≤ 1 µJ back-power → ≤ 0.6 mJ; unplug case bench-checked | Closed – design, verify |
| R6-A3 | ≥ 50 ms shutdown conflicts with USB suspend timing | Power-down ≤ 4 ms after the suspend interrupt, suspend level ≈ 7 ms after the last bus activity; suspend is a v1 requirement; Stage 2 carries the real regulators with dummy loads and measures current vs time | Closed – design, verify |
| R6-A4 | Hardware fault test tests detection, not removal | Detector test kept; failed-output emulation from the board’s own rail through an external switch with dummy loads, V²/R integrated; FLT high; no +20 % allowance | Closed |
| R6-A5 | PGOOD read after the trip cannot identify the cause | Per-leg DC readings (independent ADC path) and the firmware-trip flag stored first at the interrupt; ≥ 0.3 V → DC fault; unknown → DC fault | Closed – design, verify |
| R6-A (cost) / R6-B4 | Fixture fee outside the ceiling | Gate covers every JLCPCB line incl. any fixture; gated subtotal and total payable recorded; the fixture sentence is standard page text | Closed |
| R6-B1 | Protection limits are nominal | Full worst case (R, C incl. DC bias, LM393 offset, BAV99 VF, rails); R601–R604 100 kΩ, R609/R610 10 kΩ → 2 V ≤ 59 ms (41 % margin), 1 V ≤ 183 ms (27 %); energy 26.2 / 13.3 mJ worst ≤ 30 / 15 mJ | Closed – design, verify |
| R6-B2 | 0.755 V blind region is a product-safety decision | Firmware slow tier on every leg (≥ 0.8 V in 1 s, ≥ 0.2 V in 3 s) sets the same latch; residual < 0.2 V (≤ 2.5 mW into 16 Ω) | Open – owner decision |
| R6-B3 | Suspend wording contradicts the gate | Suspend ≤ 2.5 mA is a v1 requirement everywhere | Closed |
| R6-B (cost) / R6-C3 | OPA2210 single source decides the ceiling | No qualified second source (OPA2209AIDR stock 0; OPA1612 fails the common-mode range at the minimum rail); reserve 12; fallback LCSC price | Open – tracked |
| R6-C1 | Default-mode overloads can lock the board off | Overloads count toward the 3-strike stop only at the −12 dB ceiling; DC faults counted | Closed |
| R6-C2 | Mux inputs float with U205 fitted | DNF R216, R217 10 kΩ pull-downs fitted with U205 | Closed |
| R6-C (minor) | Clipping before a trip near the Default limit; link framing | Scope test added; SPI with chip-select framing | Closed |
| R6-D1 | Pop target conflicts with the offset limit | Pop defined as the band-limited transient beyond the settled DC step; legs enabled/disabled while VREF = 0; SPICE item for the disabled-state output | Closed – verify |
| R6-D2 | Every controlled power-down sets the latch | Expected and masked by firmware (not logged, counted or applied) | Closed |
| R6-D3 | LDO dissipation not analysed | Dissipation table; R519 + R520 dropper keeps U504 ≤ 0.20 W; copper and IR check | Closed – verify |
| R6-D4 | Two DAC centre voltages | Both ESS values stated; offset 8.6 mV worst if 1.65 V; R433 0 Ω option | Closed – verify |
| R6-D5 | OPA2210 output swing not quantified | ≈ (V+) − 0.3 V at ≈ 3 mA; 0.43 V margin calibrated; −3.5 dB pre-calibration default is a safety setting; SPICE | Closed – verify |
| R6-D6 | VBUS watchdog timing margin | Quantified (≈ 1 µs margin at 0.47 A); PGOOD latch as backstop; latency measured on Stage 2 | Closed – verify |
| R6-D7 | Positive rail can float during the rails-off window | Window removed by the new order; the latch trip collapses AVCC in hardware | Closed |
| R6-D8 | 3V3D bulk capacitance | C512 10 µF at U504 | Closed |
| R6-D (minor) | ≈ 208 ×; attach charge; test-pad table; node F margin; BCLK edge; DR loopback floor; A-M2 bracket (Spec v0.6 Section C.2, now C.5) | All updated as listed in this round | Closed |

## C.6 Fifth review round (R5-A, R5-B and R5-C of the amended Spec v0.5)

Historical text: the cost gate quoted here was retired on 25 Sep 2026; cost is now reported for information.

Finding IDs: R5-A = review 1, R5-B = review 2, R5-C = review 3. Findings raised by more than one reviewer are dispositioned together. Kept as history: the dispositions as issued in v0.6; where the sixth round changed a value, the v0.7 value is added in brackets and Sections C.1 to C.3 apply.

| ID | Finding | Disposition in v0.6 | Status |
| --- | --- | --- | --- |
| R5-A1 / R5-B1 | Spec §4 Key-parts table showed placeholder tokens | The saved v0.5 table was repaired during the amendment (the review saw an interim copy); v0.6 table rebuilt and the saved file scanned for tokens; CI document-lint check added | Closed |
| R5-A2 / R5-B6 / R5-C1 | Shared oscillator outputs need break-before-make; X203 enabled when OE is open | Both enables 2.2 kΩ pull-downs (off at power-up and during CPLD configuration); node pull-down R214; enables mutually exclusive in CPLD logic; sequence with ≥ 1 ms both-off, ≥ 10 ms start-up, PLL held in reset; SCTF/YXC datasheet facts cited (no disable time specified → measured); DNF mux U205/R215 fitted on one Stage 2 board | Closed – design, verify |
| R5-A3 / R5-B2 | 100 MHz MCLK at or above the DAC limit; “worst duty” wording | 80 MHz SCTF SX3M80 (12.5 ns cycle, ≥ 5.6 ns pulse, ≈ 208 × 384 kHz); “ideal duty 50 %”; all 8 rates stay in the gate | Closed – design, verify |
| R5-A4 | 0.473 A is an estimate, not a whole-board limit; CC changes not monitored | “Estimated maximum with hardware-limited analog current”, now 0.469 A; Stage 2 measurements before layout; CC monitoring within tSinkAdj | Closed – design, verify |
| R5-A5 | OPA2197 input-pair transition region at VREF | OPA2210 (single input pair, 0.47 V common-mode margin, 35 µV, 2.2 nV/√Hz) | Closed |
| R5-A6 | Shutdown back-powering; AVCC can collapse faster than VREF | ES9018K2M abs. max cited; rails-first-off power-down; watchdog also drops RAIL_EN; VREF clamp D401 and C421 100 nF [v0.7: VREF tracks AVCC with C421 330 pF, D401 and C422 removed, DAC analog off first; see R6-A2] | Closed – design, verify |
| R5-A7 / R5-B3 | Overload restarts when PGOOD recovers; overload does not latch | Q610 (PGOOD bad) and D606 (TPS2553 FAULT) set the latch; firmware clear required; ≥ 1 s recovery, ceiling steps, 3-strike stop [v0.7: overloads count toward the stop only at the −12 dB ceiling; see R6-C1] | Closed – design, verify |
| R5-A8 / R5-B5 | Protection envelope and terminal energy | Envelope by DC magnitude (t(V)); energies recalculated as V²/R × t for digital DC and a rail-stuck failure; both rails 0.46 mJ; hardware fault-injection test added [v0.7: worst-case envelope, 26.2 / 13.3 mJ at 16 / 32 Ω including ≤ 0.6 mJ after the trip; failed-output emulation without allowance; see R6-B1, R6-A2 and R6-A4] | Closed – design, verify |
| R5-A9 | Divider resistor tolerance omitted | Included; R518 5.1 kΩ (−3.722 V); design-corner margin 0.11 V | Closed |
| R5-A10 / R5-C5 | Balanced DR advantage assumes uncorrelated noise | Correlated model: balanced ≈ 120.3 vs unbalanced ≈ 119.5 dB(A) (+0.85 dB) | Closed |
| R5-A11 | QA403 cannot measure DC offset | DC-capable DMM for the 100 % offset screening | Closed |
| R5-A12 | External MCLK contention at the test header | J703 MCLK is a monitor output via R703 1 kΩ; external source drives BCLK/LRCLK/SDATA only | Closed |
| R5-A13 / R5-B9 | 1,700 NTD called “owner-approved”; ceiling policy | Owner statement recorded (budget flexible as long as it does not surge); 1,700 kept as the hard order gate; the JLCPCB quote must pass; exclusions listed; estimate now 1,647 [v0.7: 1,650; the gate covers every JLCPCB line] | Closed |
| R5-A14 | “Every class verified” wording; fixture fee not included | X202 exception stated; possible fixture fee (X202 and OPA2210 pages) listed as excluded, to confirm in the quote [v0.7: the fixture sentence is standard page text; any fixture line counts toward the gate; see R6-A (cost) / R6-B4] | Closed |
| R5-B4 | USB compliance only at bring-up | Stage 2 mandatory; unconfigured, configured and suspend currents measured before layout; layout blocked unless all pass | Closed – design, verify |
| R5-B7 | “Exact sample rates” | “Exact integer clock ratios”; absolute rate within the oscillator tolerance, absorbed by the asynchronous feedback | Closed |
| R5-B8 | Stage 2 should be mandatory | Mandatory, ≈ US$79 ≈ 2,500 NTD for two boards (outside the board budget) [v0.7: ≈ US$90 ≈ 2,860 NTD with the real regulators and dummy loads] | Closed |
| R5-C2 / R5-C3 | Close the X203 datasheet item; “only one runs” wording; PLL ×2 justification | Datasheet facts cited; “only one drives the node”; idle-trace rule; PLL ×2 also fixes the 45–55 % oscillator duty | Closed |
| R5-C4 | Default-mode power from average current | Instantaneous-peak method; new table and targets | Closed |
| R5-C6 | Blanking 28–119 ms vs 100 ms wait | 25–133 ms incl. R/C tolerance; AMP_EN after PGOOD good ≥ 150 ms; FLT_CLR after ≥ 200 ms rails off | Closed |
| R5-C7 | 0.1 V margin on the 4.30 V VBUS watchdog | 4.10 V (3.95–4.25 V), ≥ 0.15 V margin to the 4.40 V stress corner | Closed |
| R5-C8 | K1000 swing margin ≈ 0.06 dB | With resistor tolerance the worst unit reaches the swing limit 0.27 dB below full scale at 120 Ω balanced; rated as swing-limited; bench-verified | Closed – verify |
| R5-C9 | Continuous CC monitoring | Firmware requirement (≤ 5 ms sampling, tRpValueChange debounce, tSinkAdj response) | Closed – design, verify |
| R5-C10 | MCU link sampled by the system clock | Link captured in its own clock domain with a word toggle synchronizer [v0.7: SPI with chip-select framing] | Closed – design, verify |
| R5-C11 | Versioning and stale rows | New files Spec v0.6 / Notes v0.5 / Parts List v0.4; Spec history notes that v0.5 first used the interim divider; Notes earlier-round rows R1-B1 (7.7 µF) and R2-M5 (1,550 / 1,536) marked as the design at that time with the current values [v0.7: Spec v0.7 / Notes v0.6 / Parts List v0.5] | Closed |

## C.7 Fourth review round (reviews R4-A, R4-B and R4-C of Spec v0.4)

Historical text: the cost gate quoted here was retired on 25 Sep 2026; cost is now reported for information.

Finding IDs: A- = R4-A, B- = R4-B, C- = R4-C. Kept as history: the dispositions as issued in v0.5. Where the fifth or sixth round changed the design (for example the MCLK, the I/V op-amp, the rails, the USB estimate, the protection and the cost), Sections C.1 to C.4 and the body of this specification are current; where a row still quotes an older number, the current value is added in brackets.

| ID | Finding | Disposition in v0.5 | Status |
| --- | --- | --- | --- |
| A-H1 | FB101 mis-specified (GZ1608D601TF = 200 mA / 450 mΩ, C1002; C1015 is GZ2012D101TF) | Power ferrite now Murata BLM18PG121SN1D (C14709, Basic, 2 A, 50 mΩ) in the 5V_ANA path; GZ1608D601TF (C1002) only on DVCC (≈ 14 mA); drops recomputed (≈ 15–25 mV) | Closed |
| A-H2 | Offset used OPA1678 typical as maximum | Confirmed (±0.5 typ / ±2 mV max); I/V op-amp now OPA2197 (±0.25 mV max); budget 7.9 / 15.9 mV worst, balanced limit ±20 mV, 100 % screening; DAC-mismatch assumption Open (measure) [v0.7: OPA2210, 7.0 / 14.0 mV worst] | Closed |
| A-H3 | Safe disconnected output not established | Full sequence and current paths, rails first on/last off, PGOOD hardware kill, back-powering bounded (≤ 1.7 mA/pin) and removed in 10 ms, energy bound ≤ 16.5 mJ before / ≤ 0.22 mJ after the trip, protection defined at the headphone terminals [v0.7: legs on before AVCC, DAC analog off first; ≤ 30 / 15 mJ worst case at 16 / 32 Ω including ≤ 0.6 mJ after the trip] | Closed – design, verify |
| A-H4 | Negative-rail headroom | Rails ±3.78 V; both rails budgeted at 4.60 V and maximum load with a 3.0 Ω charge-pump resistance (margin 0.17 V) [v0.7: +3.78 V / −3.72 V, margin 0.11 V] | Closed |
| A-H5 | 100 mA ceiling not enforced | Clean envelope (80 mA/leg, swing curve) separated from overload; TPS2553 enforces the analog supply current; PGOOD kill; firmware ceiling steps; overload dissipation bounded; overload tests replace the 30-min 32 Ω balanced test | Closed – design, verify |
| A-H6 | USB power states and descriptors | State-by-state budget, MCU-only before configuration (≤ 83 mA estimate), descriptors 0x80/0xFA, CC detection by ADC for High mode, suspend policy (2.5 mA Open); verified in the firmware gate [v0.7: suspend ≤ 2.5 mA within 10 ms is a v1 requirement] | Closed – design, verify |
| A-M1 | Mixed purchasing lines and placeholder ranges | One part per line, every value specified, 48 purchasing lines by LCSC number [v0.7: 47 purchasing lines] | Closed |
| A-M2 | 13.8 NTD ceiling margin; one Basic→Extended flip fails it | Every class now verified on the JLCPCB page (X202 assumed Extended); Extended count conservative (19, incl. THT jack and X202); 34 NTD under the 1,700 NTD ceiling [v0.7: 50 NTD under]; still a planning estimate until the JLCPCB quote | Closed – verify |
| A-M3 | Gain tolerance defeats the minimum power targets | Per-unit calibration to 1.75 Vrms/leg; targets defined at the calibrated level | Closed |
| A-M4 | Two LDOs do not guarantee sequencing | Separate enables (LP5907 EN, ME6211 CE) in a firmware sequence; DVDD LDO fed from 3V3D so DVDD ≤ DVCC by construction; scope check at bring-up | Closed – design, verify |
| A-M5 | Clock adaptation needs an earlier gate | Three oscillators: 100 MHz straight to the DAC MCLK (asynchronous mode) and the CPLD system clock [v0.7: 80 MHz]; 24.576 MHz or 22.5792 MHz selected in front of the CPLD PLL (×2) for the two families (exact ratios); pre-layout firmware gate as a two-stage proof of concept | Closed – design, verify |
| B-S1 | −4.5 V rail cannot regulate from USB | As A-H4 | Closed |
| B-S2 | 2.0 Vrms/leg does not fit the supply; +1 dB units clip | Calibrated 1.75 Vrms/leg; swing model from OPA1622 Fig. 38/39; pre-calibration trim −3.5 dB | Closed |
| B-S3 | Power targets fail on low-spread units; rating method | As A-M3: “1 % THD+N or calibrated 0 dBFS, whichever first” | Closed |
| B-S4 | Firmware needs much more than the spec said | Own UAC2 firmware and CPLD logic, modules listed (Section 8); two-stage proof of concept before layout; 352.8 kHz from the 22.5792 MHz oscillator through the CPLD PLL (exact) | Closed – verify |
| B-S5 | 14 NTD margin will be used up (odd values, 1.5 nF C0G, crystal, AMS1117, THT jack fee) | Basic E24 values or Basic combinations used (100 k + 47 k, 1 k ∥ 5.1 k, 15 k + 1 k, 2 × 330 pF), Basic 5032 crystal, AMS1117 removed, THT fee counted; no cheaper 4.4 mm jack exists at LCSC (SMD option not available) | Closed – verify |
| B-M1 | Pop at unplug has no hardware control | PGOOD gates AMP_EN in hardware; VBUS watchdog; separate unplug pop target (≤ 20 mV) | Closed – design, verify |
| B-M2 | Fault latch re-trips on clear | Set path blanked ≈ 60 ms after PGOOD good, without clearing the latch [v0.7: 25–133 ms] | Closed |
| B-M3 | PLL jitter goes to the DAC in sync mode | DAC still in asynchronous mode with no PLL in the MCLK path (MCLK straight from the 100 MHz oscillator [v0.7: 80 MHz]); the CPLD PLL is only in the bit-clock path of both families, whose jitter the DAC’s DPLL rejects; J-test stays in the plan | Closed |
| B-M4 | > 100 mA before enumeration; phones limit OTG | As A-H6; Default mode ≤ 0.473 A covers phones [v0.7: 0.469 A] | Closed |
| B-M5 | AMS1117-ADJ needs tantalum-type capacitor and minimum load | Fixed 1.3 V ME6216A13M3G (±1 %, 300 mA, ceramic-stable) | Closed |
| B-minor | Duplicate “v0.4” rows in Appendix B; U301 “voltage-mode” text; HSE 8 MHz; VREF risk overstated | History cleaned (this is v0.5); U301 text corrected; HSE 8 MHz confirmed; VREF risk downgraded and quantified (1.17 mV) | Closed |
| C-B1 | USB power before configuration | As A-H6 | Closed – verify |
| C-B2 | Reference descriptors wrong (self-powered, 100 mA) | 0x80 / 0xFA in the firmware gate | Closed – verify |
| C-B3 | 352.8 kHz MCLK not supported by the reference CPLD | Confirmed in source; replaced by our own CPLD logic with the family oscillator selected before the PLL, as the reviewer suggested (X202 24.576 MHz or X203 22.5792 MHz, PLL ×2); all eight rates exact; MCLK 100 MHz straight to the DAC (asynchronous mode) [v0.7: 80 MHz] | Closed – design, verify |
| C-B4 | Clock-family switching can glitch | MCLK never switches; the family changes by the oscillator enables while the I²S output is stopped and the PLL is held in reset; no clock multiplexer in the fabric; scope check for runt pulses | Closed – design, verify |
| C-B5 | Host volume not implemented | UAC2 volume/mute with GET/SET CUR/RANGE and I²C mapping, tested on three OSes in the gate | Closed – verify |
| C-B6 | “100 mA design current” not implemented | As A-H5 | Closed – design, verify |
| C-H1 | 500 mA budget has no margin | Default mode ≤ 0.473 A enforced (27 mA margin, measured at bring-up) [v0.7: 0.469 A estimated maximum, 31 mA margin]; High mode only with a ≥ 1.5 A Type-C advertisement | Closed |
| C-H2 | +4.5 V corner | Rails ±3.78 V, both rails budgeted | Closed |
| C-H3 | 7.7 µF table is not an inrush proof | Attach charge 38 µC including 3V3M output capacitance [v0.7: ≈ 40 µC]; staggered, soft-started domains | Closed – verify |
| C-H4 | Cost provisional; “Assign LCSC no.” left; “every part is LCSC” inaccurate | Every line has an LCSC number and verified class | Closed |
| C-H5 | Offset margin | As A-H2 | Closed |
| C-H6 | ≤ 5 mV pop ambitious without relays | Split targets, full sequence incl. unplug/brownout validated on the bench | Closed – verify |
| C-H7 | USB identity (WCH VID, PID 0) | Use the pid.codes VID 0x1209 with an assigned PID; serial number from the MCU unique ID | Closed – verify |

## C.8 Earlier review rounds (three reviews of Spec v0.3)

Historical text: the cost gate quoted here was retired on 25 Sep 2026; cost is now reported for information.

These findings were dispositioned in v0.4 and updated to the design of v0.5; they are kept as history. Where a disposition quotes a value that v0.6 or v0.7 changed, the current value is added in brackets, and the later findings in Sections C.1 to C.5 apply.

| ID | Finding | Disposition (design at v0.5; later values in brackets) | Status |
| --- | --- | --- | --- |
| R1-B1 | VBUS_PD capacitance ≈ 130 µF vs the USB-C limit of 10 µF | No PD port; single 5 V port; attach charge ≈ 38 µC (see C-H3) (v0.7: ≈ 40 µC) | Closed – N/A |
| R1-B2 | LMR33630 + SMAJ18A surge stacking ≈ 44 V | No inverting converter and no 15 V input; the LM27762 runs from 5 V | Closed – N/A |
| R1-B3 | LP5907 on 1V8_D over 10 µF output capacitance | No 1.8 V rails; the only LP5907 (DAC 3.3 V) is kept within its 10 µF output-capacitance limit | Closed – N/A |
| R1-B4 | Paralleling topology left open | No paralleling; one amplifier per leg with a clean envelope of ≤ 80 mA peak (see A-H5) | Closed |
| R1-B5 | LM339 at −12 V pulling 1.8 V logic | The comparator outputs reach the 3.3 V domain only through a 10 kΩ / 47 kΩ node clamped by a 1N4148WS; the latch uses N-MOSFETs only, and 3.3 V nodes never go below 0 V | Closed |
| R1-B6 | Offset target ±5 mV vs an 11 mV worst case | Worst case recomputed with datasheet maxima (7.9 / 15.9 mV at v0.5; v0.6: 7.1 / 14.3 mV; v0.7: 7.0 / 14.0 mV); limits ±10 / ±20 mV with 100 % screening (see A-H2) | Closed |
| R1-H | AMS1117 dropout and noise | ME6211 for digital 3.3 V; LP5907 for DAC analog 3.3 V; fixed 1.3 V ME6216A13M3G for DVDD (see B-M5) | Closed |
| R1-H | XU316 VOH vs AK4493 VIH | All logic is 3.3 V | Closed – N/A |
| R1-H | Both jacks live; gain-switch pop | Gain switch removed; both jacks live documented (one headphone at a time) | Closed |
| R1-H | OPA1622 stock 197 vs 400 needed | The 5-board sample run needs 10 (2 per board) | Closed |
| R1-H | LM317/LM337 cost trade | LM27762 low-noise LDOs | Closed – N/A |
| R1-H | CI rule “every part has an LCSC number” vs Digi-Key/Mouser parts | Every fitted part has an LCSC number and a JLCPCB class (Parts List v0.3, see C-H4) (v0.6: every class from its JLCPCB part page or the basic-parts list, except X202, assumed Extended; see R5-A14) | Closed |
| R1-H | Firmware port map before layout | Pre-layout firmware gate as a two-stage proof of concept (Section 8, see B-S4); the MCU–CPLD interface and the pin map are frozen before the proof-of-concept board and the schematic | Closed |
| R1 consistency | v0.2 wording, cost source of truth, THD+N conditions, Zout including contacts, DSD pins, grounding/ESD/CC/silkscreen | Fixed in the spec: the single USB-C port removes the silkscreen confusion; CC = 5.1 kΩ Rd; jack ESD LESD5D5.0CT1G; grounding rules added; DSD not in v1 | Closed |
| R2-S1 | TVS does not protect the −14.7 V converter; no OVP | No 15 V input and no inverting converter | Closed – N/A |
| R2-S2 | VBUS bulk before the PD contract; analog rails start at 5 V | No PD contract; single 5 V port | Closed – N/A |
| R2-S3 | Balanced output impedance with option B | The OPA1622 legs drive the jacks directly, no ballast (< 0.1 Ω + contacts) | Closed |
| R2-S4 | K1000 1 W with no margin | Closed by requirement change (budget redesign): ≈ 102 mW and ≈ 94 dB SPL into the K1000 documented at v0.5 (v0.6: 95.9 mW and ≈ 93.8 dB SPL in High mode, 39.2 mW and ≈ 89.9 dB SPL in Default mode) | Closed |
| R2-S5 | Offset target contradicts the design | As R1-B6 | Closed |
| R2-M1 | Board ≈ 7 W in a PLA case | Analog input power bounded by the TPS2553 limit; board ≈ 3.8 W worst case (High mode, overload); overload tests (Section 7) | Closed |
| R2-M2 | LM317 near dropout; LM337 LC filter | Blocks removed (LM27762) | Closed – N/A |
| R2-M3 | ±0.05 dB trim cannot use 0.5 dB DAC steps | Optional digital scaling in the MCU firmware | Closed |
| R2-M4 | Maximum-power criterion | Rated at 1 kHz, both channels, at 1 % THD+N or calibrated 0 dBFS, whichever comes first (see B-S3) | Closed |
| R2-M5 | Cost margin 10 NTD; exchange rate | Gate uses a fixed 31.7 NTD/US$: target 1,500 NTD per board (WARN above) and a 1,700 NTD ceiling (FAIL above); 1,666 at v0.5 (v0.6: 1,647 NTD; v0.7: 1,650 NTD, WARNING; ceiling recorded with the owner statement of 24 Sep 2026, see R5-A13) | Closed |
| R2-M6 | 4.4-to-3.5 mm adapter shorts leg− | One amplifier per package shorted (TI continuous-short rating), ≈ 0.45 W per package; risk and manual warning | Closed |
| R2-M7 | Unbalanced crosstalk needs ≤ 6 mΩ shared ground | Layout rule (≤ 5.7 mΩ) | Closed |
| R2 minor | Stale versions; KC2520Z “(assumed)”; through-hole count; pop requirement; unverified CH224A/AMS1117 claims; NVIDIA paragraph | Versions fixed; the three oscillators are counted as leadless conservatively; only the 4.4 mm jack is through-hole (confirm USB-C shell pegs); pop ≤ 5 mV controlled and ≤ 20 mV at unplug; CH224A removed; unverified regulator claims removed (see B-M5); job-application text moved to the repository README | Closed |
| R3-1 | DC fault latch and recovery | Hardware 3.3 V latch; firmware recovery policy (Section 5.3); status to the MCU | Closed |
| R3-2 | TVS/converter envelope | No 15 V input and no converter | Closed – N/A |
| R3-3 | Stage-1 gain 0.9009 with the input network | The gain includes the DAC’s 806 Ω source (current mode, gain = 2 × Rf/806 per leg); v0.5 uses Rf = 836 Ω and per-unit calibration to 1.75 Vrms per leg (see A-M3) | Closed |
| R3-4 | QA403 cannot drive 1.8 V I²S | I²S is now 3.3 V; header, firmware test mode and sequencing specified | Closed |
| R3-5 | 200 mA “limit” not implemented; thermal at 16 Ω balanced | Clean envelope ≤ 80 mA peak per leg; rated power at 1 % THD+N or calibrated 0 dBFS; overload bounded by the TPS2553 (see A-H5) | Closed |
| R3 consistency | Rails-good qualification; offset/trim; workbook external links; cost/sourcing reconciliation | Rails-good: LM27762 PGOOD gates AMP_EN in hardware and arms the latch ≈ 60 ms after PGOOD good at v0.5 (v0.6: 25–133 ms, see R5-C6). Offset/trim: as R1-B6 and R2-M3. Workbook links: the delivered v0.1 file references only its own sheets (links appear when sheets are copied into another workbook); v0.2 adds a self-check. Cost/sourcing: single source of truth (Parts List v0.3 at v0.5; now Parts List v0.5), all LCSC | Closed |