# USB DAC + Balanced Headphone Amplifier — Design Specification v0.8

Product requirements and architecture for a personal PCB project

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 24 Sep 2026 | v0.8 (draft for review) | Product requirements + architecture |

Companion documents: “Schematic Design Notes v0.7” holds the block-level circuit guidance, and “Parts List v0.6” (DAC_HPA_Parts_List_v0.6.xlsx) is the source of truth for cost. This specification states the requirements and architecture they implement.

# 1. Purpose and scope

This specification defines the requirements and architecture of a single USB-C, bus-powered Hi-Res DAC with one ESS ES9018K2M DAC chip and a balanced headphone amplifier. The outputs are a 3.5 mm unbalanced jack and a 4.4 mm balanced jack, for one headphone at a time.

v0.8 answers the seventh review round (R7-A…R7-E, a full design check in five areas: power, analog, protection, digital, and parts and cost; Appendix C), run before any hardware is bought because development and test boards are hard to get in Taiwan. The architecture stays: CH32V307 + AGRV2K USB bridge with our own USB firmware and CPLD logic, ES9018K2M in differential current mode with one OPA2210 I/V converter per pin, 2 × OPA1622 difference-amplifier legs, LM27762 charge-pump rails and a hardware fault latch. What changed: every enable the latch can pull low now has a series resistor (new R307 on AVCC_EN, which removes the AVCC_EN contention blocker, and R521 on AUD_EN); PGOOD is pulled up to 5V_ANA through 10 kΩ (R505, with R522 into the MCU), so the ≈ 35 µs latch capture is genuine; mandatory firmware start-up rules cover faults present at power-up; the calibrated level drops to 1.55 Vrms per leg because the OPA1622 clean swing ends ≈ 1.3 V from the rail, with new power targets and a 70 mA envelope at 4.4 mm 32 Ω; V+ rises to +3.90 V so that the OPA1612 fallback for the OPA2210 fits its common-mode range; the CPLD is programmed over 2-wire Compact-JTAG, takes its family clock on PIN_1 and no longer receives the 80 MHz clock, which now drives only the DAC MCLK and is enabled by the CPLD (MCLK_EN); the power-down order handles the DAC reset state (MCLK stopped and RESETB kept high until AVCC is off); 3V3D and 1V3 get bleeders; and the suspend budget is ≈ 2.30 mA. Stage 1 becomes optional, and the JLCPCB-built Stage 2 board, now with the ES9018K2M and its real supplies, is the first hardware and the layout gate. The amendment of 24 Sep 2026 (same version) answers three external reviews (R7-F, R7-G, R7-H; Appendix C.2): the hardware DC detector becomes two TI LM339DR quad comparators with one window per leg and no diode-OR (leg trip 0.43–0.67 V; 1 V removed within 125 ms and 2 V within 47 ms, worst case); the slow DC-sense network is redesigned (15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF, with a 3V3A monitor) and four firmware DC tiers act on every leg and on each balanced pair; C421 becomes 33 pF, and the DAC-pin behaviour after an output-stage failure is analysed and, by the owner’s decision of the same day, handled by the fitted DAC-pin clamps D405–D408 (BAT54S), which hold the pins within ≈ −0.40 V…AVCC + 0.40 V and leave a small residual risk; AMP_EN drops adaptively at power-down; a stream DC guard mutes DC in the program material; and the cost is 1,719 NTD per board, 2 NTD under the order gate of 1,721 NTD (the 1,700 NTD ceiling plus the owner-approved 21 NTD for the clamps).

Goal: “best measured performance for the money”, proven by published measurements; not “beats every product”, which a hobby analyzer cannot prove (QuantAsylum itself says its QA403 won’t match a US$30,000 analyzer).

Budget: the target is ≤ 1,500 NTD per board for the 5-board sample run (what the owner actually pays: ≈ 7,500 NTD for five assembled boards at the target, including the bare PCBs and JLCPCB assembly fees); it is not a 100-unit average. Ceiling 1,700 NTD per board, set when the third oscillator was added, plus the owner-approved 21 NTD for the DAC-pin clamps (24 Sep 2026) → order gate 1,721 NTD per board. On 24 Sep 2026 the owner confirmed the budget is flexible as long as it does not surge, and on the same day he decided to fit the DAC-pin clamps D405–D408, knowing that they add ≈ 21 NTD per board and take the estimate over the 1,700 NTD ceiling. The order gate is hard and covers every JLCPCB line of the real quote: PCBs, PCBA fees (including any fixture or other assembly line that appears in the quote) and parts; when the quote is approved, both the gated subtotal and the total payable (gated subtotal + shipping + tax from the quote) are recorded. Shipping, import tax, programmers, enclosure and proof-of-concept hardware are outside the order gate. The sentence “A PCB assembly fixture is needed to protect and support this part during the assembly process” appears on every JLCPCB part page, including unrelated parts (checked on the OPA2209AIDR page) and an empty part-page template, so it is standard page text and not a part-specific requirement for X202 or the OPA2210; no fixture fee is expected for Economic PCBA. The current estimate is 1,719 NTD per board (8,594 NTD for five boards), a conservative planning figure: WARNING (above the 1,500 NTD target) and only 2 NTD under the order gate, so the real JLCPCB quote decides (Section 6). Owner decision of 24 Sep 2026: the residual DC region below 0.2 V per leg (≤ 2.5 mW into 16 Ω, ≤ 1.25 mW into 32 Ω; Section 5.3) is accepted.

Portfolio context is kept in the repository README.

| In scope for v1 | Out of scope for v1 |
| --- | --- |
| USB-C Hi-Res DAC with a single ES9018K2M DAC chip, bus-powered from one USB-C port (data + 5 V) | DSD in v1 (stretch goal: the ES9018K2M supports DSD; depends on firmware) |
| Balanced headphone amplifier: 4.4 mm balanced jack plus 3.5 mm unbalanced jack (one headphone at a time) | K1000-class 1 W drive (v0.8 gives ≈ 69.7 mW into the K1000 in High mode, Section 5) |
| PCM 44.1–384 kHz (including 352.8 kHz; all eight rates with exact integer clock ratios), 16–32 bit, USB Audio Class 2.0 High-Speed | USB PD power |
| USB power modes: Default (≤ 500 mA, hardware-limited) and High (only with a ≥ 1.5 A Type-C advertisement); USB suspend ≤ 2.5 mA within 10 ms of the last bus activity | Fast rail-proximity DC detector (v2 option; a rail-stuck failure in High mode stays within the v1 energy limits) |
| DC-fault protection: hardware fault latch (DC detect, firmware slow DC tier, PGOOD loss, sustained overcurrent) that disables the amplifiers and turns the headphone rails and the DAC analog supply off | Output relays |
| Host volume in the DAC’s attenuator; per-unit level calibration at the end-of-line test | Hardware volume knob |
| Design automation: CI checks, BOM cost gate, DFF/DFA/DFT rules, AI datasheet review, automated measurement and calibration | Enclosure design (3D-printed case for prototypes only) |

# 2. Key targets

All values below are proposed targets for v0.8, based on the ES9018K2M, OPA2210, OPA1622, LM27762 and TPS2553 datasheets and the v0.8 design calculations; every board revision is measured against this table (Section 8). Design estimates and assumptions are labelled as such; the protection limits are worst-case values.

| Parameter | Target | Minimum | Conditions | How verified |
| --- | --- | --- | --- | --- |
| PCM sample rate | 44.1–384 kHz, 16–32 bit: all eight rates (44.1, 48, 88.2, 96, 176.4, 192, 352.8 and 384 kHz) with exact integer clock ratios and a 50:50 bit clock | 384 kHz | USB Audio Class 2.0, High-Speed; DAC bit-clock duty 45:55 to 55:45 (Section 5.3); absolute rate within the oscillator tolerance (X203: ±20 ppm tolerance + ±30 ppm stability), absorbed by the asynchronous USB feedback | Host playback sweep at every rate; all eight rates bit-perfect on the Stage 2 board (layout gate) |
| DSD | Not in v1 (stretch: the ES9018K2M supports DSD; depends on firmware) | — | — | — |
| Hosts | Windows 10/11 built-in UAC2 driver, macOS, Linux; Android over USB-C (High-Speed) | — | — | Host playback on each OS |
| THD+N at 1 kHz | ≤ −105 dB | ≤ −100 dB | Calibrated 0 dBFS, 300 Ω, both jacks, 20 Hz–20 kHz (proposed) | QA403 via Python (Section 8) |
| Dynamic range | ≥ 115 dB(A) | ≥ 112 dB(A) | Both jacks (estimate ≈ 117.4 dB(A) unbalanced, ≈ 118.6 dB(A) balanced at 1.55 Vrms per leg; correlated I/V noise and the corrected leg-network noise, Section 5.3) | QA403 (results recorded with the analyzer loopback floor) |
| Level at 0 dBFS | 1.55 Vrms unbalanced / 3.1 Vrms balanced after calibration (± 0.1 dB) | — | Per-unit master trim stored in MCU flash; before calibration −4.5 dB (≤ 1.54 Vrms on the highest-gain unit), a safety setting that keeps an untrimmed unit within the calibrated full scale and the OPA1622 clean swing | Automated end-of-line test (Section 7) |
| Output power, High mode | 3.5 mm ≥ 50 mW at 32 Ω; 4.4 mm ≥ 105 mW at 60 Ω, ≥ 65 mW at 120 Ω, ≥ 30 mW at 300 Ω (worst unit, both channels) | — | 1 kHz, both channels, worst-case rail; rated at 1 % THD+N or calibrated 0 dBFS, whichever comes first; clean swing ≈ 1.3 V from the rail and a 70 mA envelope at 4.4 mm 32 Ω; estimates in Section 5.2 | QA403 + dummy loads |
| Output power, Default mode | 3.5 mm ≥ 38 mW at 32 Ω; 4.4 mm ≥ 17 mW at 60 Ω, ≥ 35 mW at 120 Ω, ≥ 30 mW at 300 Ω; 32 Ω balanced not rated (worst unit, both channels) | — | As High mode; rated by the instantaneous peak analog current within the TPS2553 Default minimum (157 mA); estimates in Section 5.2 (3.5 mm 32 Ω: 38.6 mW, small margin) | QA403 + dummy loads |
| Output impedance | ≤ 0.5 Ω | — | 1 kHz, both jacks, including contacts | Load/no-load method, 32 Ω and 300 Ω |
| Frequency response | +0.1/−0.3 dB, 20 Hz–20 kHz | — | 44.1, 48 and 384 kHz sample rates; estimate −0.05 dB at 20 kHz | QA403 |
| Crosstalk at 1 kHz | Balanced ≤ −100 dB (300 Ω); unbalanced ≤ −75 dB (32 Ω, at the jack) | — | Layout rule for the shared ground (Section 7) | QA403 |
| Channel balance | ≤ 0.3 dB; per-channel fine trim to ±0.05 dB by MCU digital scaling at calibration | — | — | Automated end-of-line test |
| DC offset | ≤ ±10 mV unbalanced, ≤ ±20 mV balanced | — | Worst-case estimate 7.7 / 15.4 mV (unbalanced ≈ 4.0 mV RSS), including ≤ 0.7 / 1.4 mV leakage of the DAC-pin clamps D405–D408; ≤ 9.3 / 18.6 mV if the DAC null is 1.65 V (R433 option, Section 5.3); 9.7 / 19.4 mV with the OPA1612 fallback (at the edge); 100 % screening | Automated test on every board with a DC-capable DMM |
| Protection | Hardware detector, worst case: a DC fault ≥ 2 V on any leg is removed within 100 ms and ≥ 1 V within 250 ms; firmware tiers (same latch, per leg and per balanced pair): ≥ 1.0 V within 0.25 s, ≥ 0.5 V within 0.35 s, ≥ 0.3 V within 1.1 s, ≥ 0.2 V within 3 s; every sub-hardware-trip case ≤ 30 mJ at 16 Ω and ≤ 15 mJ at 32 Ω; terminal energy ≤ 30 mJ at 16 Ω and ≤ 15 mJ at 32 Ω for a rail-stuck failure in High mode, worst case, including the after-trip energy (≤ 0.6 mJ); starts latched at power-up; residual below the last tier (≤ 0.198 V across a driver): accepted by the owner on 24 Sep 2026 | — | Defined at the headphone terminals; removed = RAIL_EN, AMP_EN and AVCC_EN pulled low; worst case 2 V → 47 ms, 1 V → 125 ms (two LM339 windows per leg, leg trip 0.43–0.67 V, τ 84–111 ms); rail-stuck energy 21.4 / 11.0 mJ worst case; no false trip for full-scale content above 9.5 Hz (hardware, worst case) and 4.7–9.3 Hz (firmware tiers); balanced digital DC is limited by the stream DC guard; firmware start-up rules for faults present at power-up (Section 5.3) | Digital DC test signal, firmware-tier test, detector test, failed-output emulation with dummy loads (worst-case envelope, no allowance), open-drain short-PGOOD-pulse test, balanced opposite-sign DC test, stream-guard test, DAC-pin probing and SPICE of a failed output stage (Section 7) |
| Pop | ≤ 5 mV peak for controlled transitions, at risk until measured (the DAC reset state gives ≈ 18–35 mV at AVCC-on); ≤ 20 mV peak at unplug or brownout (target) | — | Pop = peak of the transient at the jack beyond the settled DC step, band-limited 20 Hz–20 kHz (AC-coupled analyzer input); the DC before and after each transition is bounded separately by the offset limits (±10 / ±20 mV). Controlled: power-on after configuration, suspend/resume, rate and rate-family change, fault recovery; 32 Ω and 300 Ω; the AVCC-on step with RESETB low is measured on the first boards, and power-up order A (legs first), A′ (AUD_EN with AVCC_EN) or B (DAC first) is chosen by the pop measured on main board 1; power-down pop at 300 Ω ≈ 4.5 mV typical, ≈ 12.7 mV at the worst-case discharge (measured; Section 5.3) | Oscilloscope with an AC-coupled, band-limited analyzer input |
| USB power and inrush | Before configuration ≤ 100 mA (estimate ≤ 83 mA); Default mode ≤ 500 mA, estimated maximum 0.475 A with hardware-limited analog current; High mode only with a ≥ 1.5 A Type-C advertisement (estimated maximum 0.760 A); attach charge ≤ 50 µC (estimate ≈ 43 µC: 42.5 µC at 5.0 V, 43.8 µC at 5.25 V) | — | Bus-powered descriptors (bmAttributes 0x80, bMaxPower 0xFA); the TPS2553 limits only the analog branch, the MCU, CPLD and DAC-core currents are estimates or datasheet values; attach charge at nominal capacitance, domains staggered and soft-started | Measured on the Stage 2 board before layout (gate) and on every main board: USB power meter in every state; Type-C 1.5 A source and USB-A port; charge at attach and VBUS droop at each domain enable |
| USB suspend | ≤ 2.5 mA within 10 ms of the last bus activity (v1 requirement) | — | Controlled power-down: MCLK stopped and AVCC off first, AMP_EN when the 3V3A monitor reads < 0.1 V, the rails last; suspend level typically ≈ 8.2 ms and at worst 9.3 ms after the last bus activity; budget ≈ 2.30 mA (margin 0.20 mA, LEDs off), of which the MCU Stop current with the USBHS PHY suspended (≤ 1.9 mA) is an assumption and a Stage 2 gate | Current vs time from the last bus activity, on the Stage 2 board (layout gate) and on every main board |
| Thermal | All parts within ratings; the board recovers | — | Overload tests, 30 min each, High mode, maximum volume (Section 7); regulator dissipation table (Section 5.3): U504 0.20 W at 5.25 V with the input dropper, LM27762 0.39 W at 5.25 V (≈ +24 °C) | Thermocouple / IR camera, including the regulators at bring-up |
| PCB | 4 layers, target ≤ 80 × 60 mm, all SMT parts on the top side except the 4.4 mm jack, 90 Ω USB pair | — | JLCPCB Economic PCBA | ERC/DRC and layout-rule checks in CI (Section 8) |
| Cost | ≤ 1,500 NTD per board for the 5-board run (target); ceiling 1,700 NTD per board plus the owner-approved 21 NTD for the DAC-pin clamps (24 Sep 2026) → order gate 1,721 NTD per board, the hard gate, with the owner statement of 24 Sep 2026 (estimate 1,719 NTD: WARNING, 2 NTD under the order gate) | — | Every JLCPCB line of the real quote: PCBs, PCBA fees (including any fixture or other assembly line) and parts; excluding shipping, import tax, programmers, enclosure and proof-of-concept hardware; sensitivities: OPA1612 fallback ≈ 1,725 NTD (LCSC price, US$1.0942) to ≈ 1,740 NTD (JLCPCB price, US$1.33), above the order gate (owner decision only if the OPA2210 cannot be bought), OPA2210 at the LCSC price ≈ 1,825 NTD (then the fallback is used), LM339BIDR instead of LM339DR ≈ 1,715 NTD, JLCPCB-vs-LCSC price differences ≈ −24 NTD (indicative), lower-fee case ≈ 1,621 NTD; the margin is small and the real JLCPCB quote decides | Cost gate: WARN > 1,500, FAIL > 1,721 (Section 8); the JLCPCB quote from the real BOM/CPL files must pass before ordering; gated subtotal and total payable recorded at quote approval |

# 3. Architecture

## 3.1 Signal and control path

Figure 1. Signal and control path (80 MHz oscillator for the DAC MCLK only, enabled by the CPLD (MCLK_EN); 24.576 MHz and 22.5792 MHz family oscillators, one at a time, into the CPLD PLL; OPA2210 I/V converters; +3.90 V / −3.72 V output legs; the two LM339 window comparators (DC detect), the MCU slow DC check, PGOOD and overload into the fault latch).

The single USB-C port carries USB 2.0 High-Speed audio and the 5 V bus. D+, D− and VBUS pass through a USBLC6-2SC6 ESD array. CC1 and CC2 keep their 5.1 kΩ Rd pull-downs and also go through 10 kΩ to MCU ADC inputs, which read the Type-C current advertisement (Section 3.2). A6/B6 (D+) and A7/B7 (D−) are tied at the connector (checked against the drawing and a sample before layout), and the shell is tied to GND through R107 0 Ω (C101 DNF).

USB bridge: a WCH CH32V307RCT6 RISC-V MCU with a built-in USB 2.0 High-Speed PHY runs our own USB Audio Class 2.0 firmware, built on WCH’s register-level high-speed USB device examples; the CPLD logic is ours as well. The open-source USBAudioToI2S project (Apache-2.0; MCU + CPLD asynchronous USB audio) is kept only as a known-good comparison on the same hardware. v1 depends on a pre-layout firmware gate on the Stage 2 board, the first hardware (Section 8; the Stage 1 desk setup is optional). The MCU pin map (40 of 51 GPIO, including optional ones) is proposed in Schematic Design Notes v0.7 and frozen before the Stage 2 board. The 8 MHz HSE crystal is confirmed (the firmware derives the 4 MHz USB HS PLL reference as HSE ÷ 2); the Basic 5.0 × 3.2 mm YXC X50328MSB2GI is used with 33 pF load capacitors.

Clock: three 3.3 V oscillators on 3V3D. X201, 80 MHz (SCTF SX3M80.000B20F30TNN; ±20 ppm tolerance, ±30 ppm stability), drives only the DAC MCLK (XI) through 33 Ω; the CPLD no longer receives it (R209 deleted), so its load is ≈ 10 pF plus the trace (XI trace ≤ 20 mm, duty measured at XI). Its tri-state pin 1 is driven by MCLK_EN from CPLD pin 21 with the R218 2.2 kΩ pull-down, so X201 is off by default and during CPLD configuration; the MCU sets MCLK_EN over SPI only after DVDD is up and clears it before AVCC is switched off (Section 5.3). It replaces the 100 MHz part, which sat at the ES9018K2M limit (MCLK cycle time ≥ 10 ns, pulse width high and low ≥ 4.5 ns, duty 45:55 to 55:45; datasheet “XI / MCLK Timing”). At 80 MHz the cycle is 12.5 ns and the pulse width ≥ 5.6 ns at 45:55, and 80 MHz ≈ 208 × 384 kHz (208.3; asynchronous mode needs no integer ratio), above the 192 × FSR minimum for PCM normal (asynchronous) mode (73.7 MHz at 384 kHz). The ES9018K2M runs in that mode, the one its datasheet performance is specified in; the external 1.3 V DVDD is still required (FSR > 192 kHz).

An AGM AGRV2KQ32 CPLD makes BCLK = 64 × fs and LRCLK, and changes SDATA and LRCLK on the falling BCLK edge (≥ 20 ns half period at 24.576 MHz BCLK against the DAC’s 4.1 ns setup and 2 ns hold). X202, 24.576 MHz (YXC OT322524.576MJBA4SL), for the 48 kHz family and X203, 22.5792 MHz (SCTF SX2M22.5792M20F30TNN), for the 44.1 kHz family share one node at the CPLD’s only dedicated clock pin, PIN_1 (IO_GB), which feeds the PLL, each through its own 33 Ω source termination (R210, R211). Both are off by default: R212 and R213 are 2.2 kΩ pull-downs on OSC48_EN and OSC44_EN, so neither oscillator is enabled at power-up, during CPLD configuration or reprogramming, or whenever the CPLD pins are high-impedance, and R214 (10 kΩ) holds the shared node low while both are off. Only one family oscillator drives the node at a time (a disabled output-enable oscillator may keep running internally; both are counted in the current budget). The CPLD’s single PLL doubles it to 49.152 MHz or 45.1584 MHz, divided by 2/4/8/16 for 384/192/96/48 kHz or 352.8/176.4/88.2/44.1 kHz: all eight rates have exact integer clock ratios, and the PLL ×2 also turns the oscillators’ 45–55 % duty into a 50:50 bit clock (Section 5.3). There is no PLL in the DAC MCLK path. The CPLD has no free-running system clock: its global clocks are the PLL output and the SPI clock (SCK, routed to the global network through ALTCLKCTRL, as in the USBAudioToI2S reference). It is programmed over AGM’s 2-wire Compact-JTAG (JTMS pin 24, JTCK pin 25) from the Supra tool with a J-Link or an AGM DAP-Link through J202 (3V3D sense only, JTCK, JTMS, NRST, GND); a generic CMSIS-DAP probe such as the WCH-LinkE in DAP mode is not confirmed. For first-time and end-of-line programming, a PROG pad (TP732) makes the firmware power 3V3D without USB configuration (Section 7).

Rate-family change, break-before-make, sequenced by the MCU through CPLD SPI registers, with the two enables mutually exclusive in the CPLD logic and never high in the same cycle: (1) DAC soft mute; (2) I²S output stopped (held low); (3) PLL held in reset; (4) old enable low; (5) wait ≥ 1 ms with both off; (6) new enable high; (7) wait ≥ 10 ms (start-up ≤ 3 ms X202, ≤ 5 ms X203); (8) PLL released, wait for lock; (9) output restarts, and the MCU checks the LRCLK rate with its feedback timer (within ±0.1 % of the target; otherwise it stays muted, retries once, then reports a fault); (10) DAC re-locks (lock_status, register 64) → soft unmute. After configuration the CPLD starts with both enables low and the PLL in reset, and the MCU enables X202 through SPI after ≥ 1 ms. A family change never touches MCLK (X201), and both enable bits never change at the same time. The single PLL must lock at both 22.5792 and 24.576 MHz input (the AGM lock range is not published): a Stage 2 gate, with BCLK taken directly from X202/X203 as the fallback (duty 45–55 %, the DAC limit, zero margin).

Oscillator datasheets: the SCTF 2CM series datasheet (X203) gives pin 1 “Tri-State”, enabled when OE is logic 1 or open and disabled at logic 0 (≤ 0.3 VDD), start-up ≤ 5 ms, 10 mA max and duty 45–55 %, but no disable or enable time, internal pull-up value or disabled-output leakage; the YXC YSO110TR datasheet (X202) gives enable ≥ 70 % VDD or floating, disable ≤ 30 % VDD, start-up ≤ 3 ms, 5 mA max and duty 45–55 %, but no disable time. Both enable when pin 1 is open, so each has an internal pull-up of unknown strength: the 2.2 kΩ pull-downs keep pin 1 below 0.3 VDD against any combined pull-up ≥ 5.1 kΩ (oscillator and CPLD weak pull-up during configuration), and each enable draws 1.5 mA when driven high. Typical disable times are well under 1 µs, so the 1 ms gap is a large margin; the disable time is measured on the Stage 2 boards. Fallback: the DNF 2:1 multiplexer U205 (74LVC1G157GW, select = OSC44_EN) with R215 33 Ω at the PLL pin and the DNF R216, R217 10 kΩ pull-downs on both mux inputs; to use it, fit U205, R215, R216 and R217 and remove R210 and R211 (with R210/R211 removed, a disabled oscillator output is high-impedance, so a mux input would otherwise float). One of the two Stage 2 boards is built with it.

MCU → CPLD audio link: SPI with chip-select framing (MCU SPI1 master with DMA at 36 MHz, 72 MHz ÷ 2, the only workable setting: 68 % utilised at 384 kHz; register reads at ≤ 18 MHz; one CS frame per block of samples). LINK_FRAME is active-high (idle low), CPOL = 0, and R224/R225 33 Ω sit at the MCU on SCK and MOSI; SCK, MOSI, FRAME and RESETB are driven low or set as inputs whenever AUD_EN is low. The CPLD writes the FIFO directly in the SPI clock domain (SCK on the global network through ALTCLKCTRL) and reads it in the PLL bit-clock domain, with Gray-coded pointers (1024 × 32 bit in 4 × M9K, ≈ 1.33 ms at 384 kHz); each SPI frame starts with a 16-bit header so that the pointer synchronisers see SCK edges, the control registers live in the SPI domain, and MISO returns the FIFO level during the frame. I²S is not used for the link, because its frame rate is not locked to the audio rate. The payload at 384 kHz / 32-bit stereo is 24.576 Mbit/s; a second SPI lane (SPI2, PB12–PB15, two spare CPLD pins) is reserved, and the 36 MHz capture is a Stage 2 gate.

The ESS ES9018K2M (32-bit, PCM to 384 kHz via I²S) is configured by the MCU over I²C (7-bit address 0x48, written 0x90; ADDR to DGND through R306) and runs in differential current mode. Each output pin is equivalent to a voltage source (3.05 Vpp around 1.65 V) in series with 806 Ω ±11 % (datasheet p. 31), and ESS specifies DNR and THD+N only in differential current mode, so the design follows the ESS application circuit (p. 27) with one I/V converter per pin. The MCU drives RESETB (100 kΩ pull-down) through the new R308 1 kΩ, which limits injection to < 3 mA, and reads the DPLL lock status (register 64, and GPIO2 as DAC_LOCK). In reset the DAC outputs are driven to ground (datasheet pin table), which the power sequence takes into account (Section 3.2).

Signal chain: DAC (current mode) → 2 × OPA2210 I/V converters (one amplifier per DAC pin, VREF ≈ 1.70 V) → 2 × OPA1622 difference-amplifier legs → jacks, all on +3.90 V / −3.72 V. U403 serves DACL/DACLB and U404 DACR/DACRB. Each feedback is 836 Ω (1 kΩ ∥ 5.1 kΩ, 1 %) with 10 Ω + 660 pF (two 330 pF C0G in parallel), a 288 kHz pole. VREF = 1.703 V = 0.516 × 3V3A from a 15 kΩ over 15 kΩ + 1 kΩ divider (7.74 kΩ source) with C421 33 pF C0G only (τ ≈ 0.29 µs with ≈ 5 pF stray), so VREF tracks AVCC in every ramp; its in-band noise is unchanged (common mode, far below the 620 kHz pole). The DAC-pin clamps D405–D408 (JSMSEMI BAT54S, C916425, SOT-23, one per DAC pin) are fitted (owner decision, 24 Sep 2026): pin 3 (common) to the DAC pin (DACL and DACLB with AVCC_L, DACR and DACRB with AVCC_R), pin 1 (lower anode) to AGND at the DAC’s AGND pins and pin 2 (upper cathode) to AVCC_L/AVCC_R at their decoupling capacitors, within 5 mm of the DAC pins (pinout confirmed on the JSMSEMI drawing before layout). In normal operation both diodes are reverse biased by ≈ 1.6–1.7 V, and they never conduct in controlled transitions (VREF tracks AVCC), at unplug (VREF lag ≤ 15 mV with C421 33 pF) or during the DAC-reset step (the pins stay between 0 V and VREF). Their net leakage, ≤ 0.4 µA per pin (assumption: the two diodes of a package see almost equal reverse voltage, so their leakages largely cancel), adds ≤ 0.35 mV offset per pin, and their ≤ 20 pF per DAC-pin node raises the I/V high-frequency noise gain only to 1 + 20 pF/660 pF ≈ 1.03, negligible for stability and noise (included in the OPA2210 stability SPICE). VREF sits at the DAC’s zero-DC-current point from the current-output formula (2.112 mA − 1000·Vg/806 = 0 at Vg ≈ 1.702 V); the ESS voltage-source model gives 1.65 V (AVCC/2), and R433 1 kΩ → 0 Ω sets VREF = 1.650 V if the null test at bring-up shows that value (Section 5.3). The OPA2210 (single input pair, common-mode range (V−) + 1.5 V to (V+) − 1.5 V, Vos ±35 µV max, 2.2 nV/√Hz, 18 MHz) replaces the OPA2197, whose input-pair transition region, typically (V+) − 3 V to (V+) − 1.5 V (TI SBOS737C), contains VREF; VREF (≤ 1.754 V worst case) sits ≈ 0.53 V inside the common-mode range at the minimum +rail of 3.789 V. Its output swing at the ≈ 3 mA I/V load is estimated at (V+) − 0.3 V (≈ 3.49 V at the minimum rail), above the I/V output peak at the calibrated level. The OPA2210’s JLCPCB stock is unverified, so the OPA1612AIDR (same dual pinout, common-mode limit (V+) − 2 V) is designed in as the fallback: V+ = +3.90 V keeps the worst-case VREF (≤ 1.754 V, tracking 3V3A +2 %) ≥ 35 mV inside its range at the minimum rail. With the fallback the quiescent current rises to up to 4.5 mA per channel (instead of 2.5 mA), an analog quiescent current of ≈ 72 mA, so the Default-mode targets are not met: 3.5 mm 32 Ω ≈ 27 mW (target ≥ 38 mW), 3.5 mm 16 Ω ≈ 14 mW, 4.4 mm 60 Ω ≈ 13 mW (target ≥ 17 mW) and 4.4 mm 120 Ω ≈ 26 mW (target ≥ 35 mW); High mode is unchanged; the worst-case offset becomes 9.7 / 19.4 mV (inside the limits, at the edge) and the I/V noise is lower (1.1 nV/√Hz).

Output stage: 2 × TI OPA1622, one amplifier per leg, with no paralleling. Each amplifier is a 4-resistor 2.00 kΩ (0.1 %) difference amplifier with 330 pF C0G (a 241 kHz pole), fed by the I/V outputs: leg+ = I/V(DAC−) − I/V(DAC+), leg− = the swap. The leg input common mode is VREF, so VREF errors are mostly common mode. U401 drives L+ and L−, U402 drives R+ and R−. The clean output envelope is ≤ 80 mA peak per leg (70 mA at 4.4 mm 32 Ω, set by the negative-rail headroom) within the OPA1622 clean swing, which ends ≈ 1.3 V from the rail (Section 5). Each OPA1622 GND pin (pin 3, the EN reference) is tied to GND.

Protection: two TI LM339DR quad comparators (U601, U602) form one window per leg directly on its filter node, with no diode-OR, so the threshold depends only on the divider, the comparator offset and its bias current, all datasheet-guaranteed (worst-case leg trip 0.43–0.67 V, τ 84–111 ms); their eight open-collector outputs are wired-OR into node F and set an N-MOSFET fault latch on the always-on 3V3M domain; PGOOD-bad (Q610) and a sustained TPS2553 overcurrent (FAULT through D606 and R638, after its 5–10 ms deglitch) set it too. C605 sits on node F, so the latch captures a PGOOD fault in ≈ 35 µs (budget 0.1 ms; RAILS_OK_N crosses 2.5 V in ≈ 1.0–1.3 µs through R505 10 kΩ to 5V_ANA and carries only ≥ 1 MΩ RC branches and high-impedance inputs), every set request is stretched to 1.5–7.7 ms, and the board powers up latched. The latch pulls RAIL_EN (LM27762 EN+/EN−), AMP_EN (both OPA1622 EN pins) and, through Q611, AVCC_EN (LP5907) low, each against a 10 kΩ series resistor from the MCU (R507, R618, R307), and stays set until the firmware clears it (FLT_CLR). Q609 is a slow static interlock (on 1.9–9.4 ms after PGOOD goes bad) that keeps AMP_EN low whenever the rails are off, so a short PGOOD glitch either latches or does nothing. Four firmware DC tiers read every leg through its own low-impedance DC-sense network (15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF) on an MCU ADC pin, act on each leg and on each balanced pair difference (≥ 1.0 V within 0.25 s, ≥ 0.5 V within 0.35 s, ≥ 0.3 V within 1.1 s, ≥ 0.2 V within 3 s) and set the same latch through ANA_FAULT and D606; a stream DC guard in the MCU mutes DC in the program material before the DAC instead of latching; at the FLT interrupt the firmware first drives the enables low, then stores the leg readings and the fault flags, which identify the cause. The set path is armed 51–186 ms after PGOOD goes good, without clearing the latch; mandatory firmware start-up rules cover faults present at power-up; there are no relays (Section 5.3).

The 4.4 mm balanced jack carries L+, L−, R+, R− and GND; the 3.5 mm unbalanced jack carries L+, R+ and GND. Both jacks are live, so only one headphone is connected at a time.

## 3.2 Power path

The single USB-C port supplies data and 5 V; the board is bus-powered (bmAttributes 0x80, bMaxPower 0xFA = 500 mA). Only 5 × 1 µF sit on VBUS before any switch (connector and the inputs of ME6211 #1, ME6211 #2, LP5907 and TPS2553). At attach only 3V3M starts (5.3 µF behind it, including C212, C221, C605, C606 and C607; C607 is now 10 nF and C221 1 µF is added), so the attach charge is ≈ 42.5 µC at 5.0 V and 43.8 µC at 5.25 V against the 50 µC USB-IF limit (nominal values; DC-bias derating lowers the real figure). VBUS has ceramic capacitors only, and the LP5907 and LM27762 are rated 6.0 V and 5.8 V absolute maximum: a DNF footprint D102 (unidirectional TVS at J101, VRWM 6.0 V, for example MDD SMF6.0A, LCSC C123790) is fitted only if the hot-plug test on board 1 shows a VBUS peak above 5.8 V, after its leakage at 5.25 V has been measured (parts with a 5.0 V VRWM leak hundreds of µA at 5 V).

3V3M, always on: ME6211C33 #1 (U502) powers the MCU (including the DC-sense ADC, whose networks are biased from 3V3A), the fault latch and the LEDs (both off in suspend). Before USB configuration only this domain runs (62 / 83 mA typical / maximum estimate; assumption on the MCU current).

3V3D, switched: ME6211C33 #2 (U504, CE = AUD_EN through the new R521 10 kΩ, which keeps the CE clamp current ≤ 0.2 mA when CE would exceed VIN + 0.3 V at unplug under load) is fed through R519 + R520 (2 × 2.2 Ω 0805 in series, 4.4 Ω), which move ≈ 0.11 W out of the LDO (U504 0.20 W at VBUS 5.25 V and 0.24 W at 5.5 V against the ME6211’s 300 mW SOT-23 rating, at 160 mA including the bleeders; LDO input ≥ 3.70 V at VBUS 4.40 V), and C512 10 µF is its bulk capacitor. It powers the CPLD (through the new FB202 ferrite branch, with C220 1 µF at VDDA33), the three oscillators (80 MHz, 24.576 MHz and 22.5792 MHz), the I²C pull-ups and the DAC DVCC (through the GZ1608D601TF ferrite, ≈ 14 mA). A fixed, ceramic-stable 1.3 V LDO (ME6216A13M3G, SOT23-3, ±1 %, 250 mA) fed from 3V3D makes the external DVDD, so DVDD cannot exceed DVCC during the ramps. New bleeders discharge both after AUD_EN goes low: R524 2.2 kΩ on 3V3D (1.5 mA; 3V3D below 0.3 V ≈ 70 ms after AUD_EN low) and R525 1 kΩ on 1V3 (1.3 mA, so DVDD collapses before DVCC); the firmware keeps AUD_EN off for ≥ 100 ms before re-enabling it.

3V3A, switched: the LP5907-3.3 (EN = AVCC_EN from the MCU through the new R307 10 kΩ, also pulled low by the latch through Q611, which now pulls the node low against 0.33 mA instead of fighting the GPIO) supplies VCCA, AVCC_L/AVCC_R, the VREF divider, the DC-sense bias (R628–R631, 0.70 mA) and the 3V3A monitor (R639/R640 10 kΩ/10 kΩ with C616 100 nF to MCU PC4, 0.17 mA). VREF = 0.516 × 3V3A with only C421 33 pF (τ ≈ 0.29 µs with ≈ 5 pF stray), so VREF follows AVCC: the lag is ≤ 4 mV in controlled ramps (3V3A falls through the LP5907’s 230 Ω typical output auto-discharge: below 0.3 V within 5.5 ms worst case, with +50 % discharge resistance and 6.7 µF) and ≤ 15 mV at the fastest unplug slope (0.1 V/µs), so a DAC pin can exceed AVCC only while AVCC is below ≈ 31 mV. D401 and C422 are removed.

Headphone rails: a TI TPS2553 current-limited switch (EN = ANA_EN; built-in soft start; FAULT open drain with a 5/7.5/10 ms deglitch) feeds 5V_ANA through the Murata BLM18PG121SN1D ferrite (2 A, 50 mΩ) to the LM27762, which makes +3.900 V (3.789–4.014 V) and −3.722 V (3.618–3.828 V magnitude), both ranges including VFB and the 1 % divider resistors, with 225 kΩ/100 kΩ and 205.1 kΩ/100 kΩ feedback. Each top resistor is built from Basic values: 100 kΩ + 100 kΩ + 15 kΩ + 10 kΩ (the new R523), and 100 kΩ + 100 kΩ + 5.1 kΩ (R518). V+ is set this high so that the OPA1612AIDR fallback for the I/V converters keeps VREF inside its common-mode range (Section 3.1). EN = RAIL_EN (≥ 10 ms after ANA_EN); PGOOD (95 % thresholds +3.71 V / −3.54 V) is pulled up to 5V_ANA through R505 10 kΩ (the LM27762 VIN side, as TI specifies; no PGOOD current in suspend) and goes, as RAILS_OK_N, to the MCU through R522 10 kΩ, to the fault latch (Q610) and to the Q609 interlock (R623 1 MΩ, C607 10 nF). LM27762 dissipation is 0.39 W at 5.25 V (θJA 62.2 °C/W, ≈ +24 °C) and 0.46 W at 5.5 V.

Current limit: RILIM = 100 kΩ + 47 kΩ = 147 kΩ gives the Default limit (157 / 183 / 213 mA minimum / nominal / maximum, including 1 % resistors); a 2N7002 switches 100 kΩ in parallel for High mode (59.5 kΩ: 393 / 442 / 498 mA). The TPS2553 acts on the instantaneous current (short-circuit response ≈ 2 µs, then constant-current limiting) and limits only the analog branch, so the board figures are estimated maxima with hardware-limited analog current: 0.475 A in Default mode (25 mA below 0.5 A) and 0.760 A in High mode. High mode is used only when the Type-C advertisement read on CC1/CC2 is ≥ 1.5 A (bands: Default 0.25–0.61 V, 1.5 A 0.70–1.16 V, 3.0 A 1.31–2.04 V). A USB-A-to-C cable reads as Default, and many phones limit OTG to ≤ 500 mA and run in Default mode.

CC monitoring (firmware requirement): while configured, the MCU samples CC1/CC2 (ADC2) at least every 5 ms, with firmware thresholds of 0.66 V (Default ↔ 1.5 A) and 1.23 V (1.5 A ↔ 3.0 A) with hysteresis on the CC pin that sees Rp, and treats a change as valid after tRpValueChange (10–20 ms). On a drop from ≥ 1.5 A to Default it soft-mutes, sets ILIM_HI low and continues in Default mode within tSinkAdj (60 ms max) of the change; it switches up to High only after the new advertisement is stable (USB Type-C timing, as quoted in the Zephyr USB-C stack and the Chromium EC documentation).

Supply headroom, computed with the peak currents and a 0.3 Ω cable-and-connector resistance at the real total current (host VBUS 4.75 V), with the TPS2553 (135 mΩ max), the ferrite (60 mΩ) and traces (20 mΩ) in the path and a 3.0 Ω or 3.5 Ω charge-pump resistance (assumption; 2.5 Ω is specified only at 5.5 V): at the worst clean load (4.4 mm 32 Ω, High mode) the negative-rail capability exceeds the maximum |V−| of 3.828 V by +0.13 V (3.0 Ω) / +0.05 V (3.5 Ω) at 70 mA per leg; at 80 mA it would be +0.04 / −0.05 V, hence the 70 mA envelope for this load. 3.5 mm 16 Ω at 80 mA keeps +0.13 / +0.03 V, and at a host VBUS of 5.0 V every case keeps ≥ +0.20 V. On the positive side the LM27762 VIN is ≈ 4.50 V against a maximum V+ of 4.01 V. The negative-rail headroom is measured at VBUS 4.60 V and 4.40 V at bring-up.

VBUS sense: 100 kΩ / 47 kΩ into ADC1 (5.0 V → 1.60 V), converting continuously on this single channel (VBUS_SENSE, PA1) with the analog watchdog at 4.10 V nominal (3.95–4.25 V with the ADC reference 3V3M ±2 %, the divider ±1.4 % and the ADC error: ≥ 0.15 V below the 4.40 V stress corner and ≥ 0.35 V below the 4.60 V design corner). Sampling is 7.5 cycles at fADC = 72 MHz ÷ 6 = 12 MHz (PCLK2 must be 72 MHz; 18 MHz would exceed the 14 MHz limit): 1.67 µs per conversion and ≈ 3.8 µs worst-case latency, with the ISR on a fast (VTF) vector; ADC2 scans seven channels: CC1, CC2, the four DC-sense channels and the 3V3A monitor (PC4). On unplug or brownout the ISR drops AMP_EN and RAIL_EN at once (target < 5 µs), then RESETB, AUD_EN, AVCC_EN and ANA_EN; the PGOOD latch is the backstop. At 0.47 A with ≈ 4.5 µF on VBUS, VBUS falls at up to ≈ 0.1 V/µs: 4.10 V → 3.46 V (3.3 V LDO dropout) in ≈ 6 µs, so the DAC-first order is impossible at unplug; PGOOD goes bad at VBUS ≈ 3.70 V while 5V_ANA is still ≈ 3.6 V, so Q610 turns on and the latch sets. At full load the logic rails may droop before the sequence completes, so the unplug pop stays a measured target (≤ 20 mV peak), and the watchdog-to-GPIO latency is measured on the Stage 2 board. At a sudden unplug VREF tracks AVCC within ≤ 15 mV (C421 33 pF), so a DAC pin can exceed AVCC only while AVCC is below ≈ 31 mV. ESS specifies only GND < Vout < AVCC, with no margin and no injection rating, so no junction-conduction argument is used: the excursion is measured on the four DAC pins (at the R423–R426 DAC-side pads, against AVCC_L/AVCC_R) at hot unplug under full load, where they must stay within −0.4 V…AVCC + 0.4 V (the clamps D405–D408 do not conduct at unplug).

Power sequence (steps in Section 5.3): power-up enables the legs while AVCC is still off (VREF = 0 V), then AVCC_EN, so VREF rises in common mode while the legs hold the jack at ≈ 0 V; MCLK starts only after DVDD is up. Because the ES9018K2M drives its outputs to ground while RESETB is low, the AVCC-on step leaves ≈ 18–35 mV at the leg until RESETB is released and the ESS soft start ramps it out; this is measured on the first boards, and firmware options A′ (AUD_EN together with AVCC_EN, allowed by ESS: DVCC with or after VCCA) and B (DAC first, legs last) are kept, chosen by the pop measured on main board 1. Controlled power-down, including USB suspend: at t0 (the suspend interrupt, 3 ms after the last bus activity, or a host or fault command) soft mute; at t0 + 0.1 ms MCLK_EN low (MCLK stops, RESETB stays high, DAC muted) and AVCC_EN low, so the DAC turns off first while the I/V converters and legs stay powered and hold ≈ 0 V; 3V3A falls and VREF and the DC-sense bias follow. AMP_EN goes low as soon as the 3V3A monitor reads < 0.1 V (typically t0 + 4.7 ms), at the latest at t0 + 5.8 ms (worst-case discharge, 3V3A ≈ 0.28 V); then RESETB, AUD_EN, RAIL_EN and ANA_EN go low (the rails decay on their own, and nothing is left that could back-power the I/V stage) and the MCU enters low-power mode with both LEDs off, so the VBUS current reaches the suspend level typically ≈ 8.2 ms and at worst 9.3 ms after the last bus activity (USB limit 10 ms). The power-down pop at 300 Ω, including the sense-bias path, is ≈ 4.5 mV typical and ≈ 12.7 mV at the worst-case discharge (measured; target ≤ 5 mV). Resume to audio takes ≈ 1.0 s (0.6 s blanking charge, the rails, 250 ms, DAC init and the 105 ms soft start). No “rails off, DAC powered” window remains in a controlled sequence; the latch sets as the rails go off, which the firmware expects and masks.

Fault shutdown: when the latch sets, Q607, Q608 and Q611 pull RAIL_EN, AMP_EN and AVCC_EN low within ≈ 35 µs, each against its 10 kΩ series resistor. Within ≤ 50 µs the FLT interrupt drives AMP_EN, RAIL_EN and AVCC_EN low as well (mirroring the latch), then AUD_EN and RESETB (which stops MCLK back-powering VCCA through XI and any RESETB injection), captures the cause and finally sets ANA_EN low; the IWDG is enabled, and an MCU reset floats all outputs so that the pull-downs switch every domain off. 3V3A needs 3.2–5.6 ms to fall below 0.3 V. In a failed-output shutdown (a stuck leg into 16 or 32 Ω, rail capacitance ±10 %, LP5907 discharge typical and +50 %) the OPA2210 positive common-mode limit (V+ − 1.5 V) drops below VREF 0.11–0.39 ms after the kill, so the I/V converters may not hold the DAC pins for up to ≈ 5.5 ms: a pin sits between the DAC’s internal source (806 Ω) and the I/V output (836 Ω). Unclamped it would be pulled toward −1.76 V, stopped only by the DAC’s internal protection conducting at ≈ −0.6 V with ≤ 3.9 mA, or to ≈ 0.33 V above AVCC for a negative-stuck leg; the fitted clamps D405–D408 hold every DAC pin within ≈ −0.40 V…AVCC + 0.40 V (BAT54 VF ≤ 400 mV at 10 mA; ≤ 4.4 mA per pin) for this window. ESS specifies only GND < Vout < AVCC, with no margin, so a small residual risk remains: up to 0.4 V beyond the rails for ≤ 5.5 ms, and only after an output stage has already failed; it is measured on main board 1 (Section 7). The upper diodes can briefly back-feed 3V3A from the collapsing I/V rails (≤ 4.4 mA per pin, for about the I/V rail decay, ≈ 1 ms with a stuck load); the 3V3A decay is measured with the clamps fitted. Normal operation and every controlled transition keep the pins inside GND…AVCC, with both diodes of each clamp reverse biased.

Figure 2. Power path (switched power domains with the three oscillators on 3V3D, TPS2553 current limit, LM27762 +3.90 V / −3.72 V rails for the OPA2210 × 2, the OPA1622 × 2 and the two LM339 window comparators; PGOOD and the TPS2553 overcurrent flag set the fault latch).

## 3.3 Volume, gain and level

Gain is fixed. Volume comes from the host via USB Audio Class (Feature Unit volume and mute) and is applied in the ES9018K2M attenuator (0 to −127.5 dB in 0.5 dB steps), with MCU fine scaling for channel trim; a hardware knob is out of scope. Per leg, L = (Rf/Rs) × 2.156 Vrms with Rf = 836 Ω and Rs = 806 Ω ±11 %: nominal 2.236 Vrms, unit spread 1.955–2.589 Vrms including Rf 1 % and AVCC ±2 %. Every unit is therefore calibrated to 1.55 Vrms per leg at 0 dBFS (the OPA1622 clean swing ends ≈ 1.3 V from the rail) with the ES9018K2M 32-bit master trim, stored in MCU flash at the automated end-of-line test.

| Parameter | Value | Basis |
| --- | --- | --- |
| Calibrated full scale | 1.55 Vrms per leg (2.19 V peak): 3.5 mm 1.55 Vrms, 4.4 mm 3.1 Vrms | End-of-line calibration to ± 0.1 dB |
| Master trim | −2.02 dB (lowest-gain unit) to −4.46 dB (highest-gain unit); nominal −3.18 dB | ES9018K2M 32-bit master trim, stored in MCU flash |
| Before calibration | −4.5 dB, so ≤ 1.54 Vrms on any unit | Firmware safety default, active until a calibration is stored: an untrimmed highest-gain unit at 0 dBFS would exceed the calibrated full scale and the OPA1622 clean swing |
| Channel trim | ±0.05 dB | MCU digital scaling |
| Output envelope | ≤ 80 mA peak per leg (70 mA at 4.4 mm 32 Ω); Vpk ≤ (¦rail¦min − 1.3 V)/(1 + 8 Ω/Rleg) | OPA1622 clean swing: distortion starts ≈ 1.3 V from the rail (TI data at ±5 V: 600 Ω → 11.3 mW, 32 Ω → 150 mW); bench-verify at +3.90 V / −3.72 V |

# 4. Key parts

| Part | Role | Key specs | Package / assembly note | Unit price (run tier) | Source |
| --- | --- | --- | --- | --- | --- |
| WCH CH32V307RCT6 | USB audio bridge (MCU) | 144 MHz RISC-V, 3.3 V; built-in USB 2.0 High-Speed PHY; runs our own UAC2 firmware; sends the samples to the CPLD over SPI at 36 MHz with chip-select framing (MCU master, DMA); ADC1 on VBUS with the analog watchdog, ADC2 on seven channels: CC1, CC2, the four slow DC-sense channels and the 3V3A monitor (PC4), with pin leakage (1 µA × 15 kΩ) inside the DC-sense error budget; VBAT tied to 3V3M | LQFP-64, 10 × 10 mm; leaded; JLCPCB Extended | US$3.47 | LCSC C2988083 (LCSC 193 / JLCPCB 364 in stock; reserve) |
| AGM AGRV2KQ32 | CPLD: FIFO, I²S master, oscillator enables and MCLK_EN, bit clocks for all eight rates | 2K LUTs (AG32 silicon in pure-CPLD mode); one PLL, fed from the only dedicated clock pin PIN_1 (×2 on the selected family oscillator: 49.152 or 45.1584 MHz); no 80 MHz input and no free-running system clock; SPI SCK on the global network through ALTCLKCTRL, FIFO written in the SPI clock domain; 3.3 V LVCMOS; programmed over 2-wire Compact-JTAG (JTMS pin 24, JTCK pin 25; J-Link or AGM DAP-Link with AGM Supra); instant-on non-volatile | QFN-32, 4 × 4 mm; leadless, X-ray fee; Extended | US$1.65 | LCSC C18208899 (980 in stock) |
| SCTF SX3M80.000B20F30TNN (X201) | 80 MHz: DAC MCLK only, enabled by the CPLD (MCLK_EN) | 3.3 V CMOS; ±20 ppm tolerance, ±30 ppm stability; 40 mA max (SCTF 3C datasheet, 80–160 MHz); duty 45–55 %; tri-state pin 1 driven by MCLK_EN (CPLD pin 21) with the R218 2.2 kΩ pull-down, off by default and during CPLD configuration; load ≈ 10 pF + trace (DAC XI only); 12.5 ns cycle and ≥ 5.6 ns pulse width at 45:55 against the DAC limits of ≥ 10 ns and ≥ 4.5 ns | 3.2 × 2.5 mm, 4-pad; leadless, X-ray fee (counted conservatively); JLCPCB Extended | US$0.8916 (1+) | LCSC C5179978 (2,503 in stock); replaces the 100 MHz YXC part (C4353707) |
| YXC OT322524.576MJBA4SL (X202) | 24.576 MHz: 48 kHz family, OSC48_EN (off by default through the R212 2.2 kΩ pull-down; PLL ×2 = 49.152 MHz) | 1.8–3.3 V CMOS; tri-state pin 1 (YXC YSO110TR datasheet: enable ≥ 70 % VDD or floating, disable ≤ 30 % VDD; no disable time specified); start-up ≤ 3 ms; 5 mA max; duty 45–55 %; ±10 ppm tolerance, ±20 ppm stability; 33 Ω (R210) into the shared PLL input node | 3.2 × 2.5 mm, 4-pad; leadless, X-ray fee (counted conservatively); Extended assumed (class not shown on the JLCPCB page; its fixture sentence is standard page text) | US$0.51 | LCSC C2831388 (1,755 in stock) |
| SCTF SX2M22.5792M20F30TNN (X203) | 22.5792 MHz: 44.1 kHz family, OSC44_EN (off by default through the R213 2.2 kΩ pull-down; PLL ×2 = 45.1584 MHz) | 1.62–3.63 V CMOS; pin 1 “Tri-State” (SCTF 2CM datasheet: enabled when OE is logic 1 or open, disabled at logic 0, ≤ 0.3 VDD; no disable or enable time, internal pull-up value or disabled-output leakage specified); start-up ≤ 5 ms; 10 mA max; duty 45–55 %; ±20 ppm tolerance, ±30 ppm stability; 33 Ω (R211) into the shared PLL input node | 2.5 × 2.0 mm, 4-pad; leadless, X-ray fee (counted conservatively); Extended (JLCPCB part page) | US$0.55 | LCSC C5452663 (1,426 in stock) |
| ESS ES9018K2M | DAC | 32-bit; PCM to 384 kHz; DNR 127 dB-A and THD+N −120 dB (datasheet, current mode); each pin ≈ 3.05 Vpp source behind 806 Ω ±11 %; asynchronous (PCM normal) mode needs MCLK > 192 × FSR, MCLK cycle ≥ 10 ns and pulse width ≥ 4.5 ns; external 1.3 V DVDD above 192 kHz (50 mA at MCLK 80 MHz and 192 kHz); output pins GND < Vout < AVCC (absolute maximum); outputs driven to ground in reset (≈ 2.1 mA per pin with AVCC up); I²C 7-bit address 0x48 (written 0x90) with ADDR to DGND; attenuator 0 to −127.5 dB in 0.5 dB steps; 32-bit master trim | QFN-28, 5 × 5 mm; leadless, X-ray fee; Extended | US$3.47 | LCSC C2688377 (724 in stock; reserve); ESS datasheet v3.7 |
| TI OPA2210IDR (× 2, U403 and U404) | I/V converters, one amplifier per DAC pin | Single input pair, common-mode range (V−) + 1.5 V to (V+) − 1.5 V (VREF ≤ 1.754 V worst case, ≈ 0.53 V inside at the minimum +rail of 3.789 V); Vos ±35 µV max; 2.2 nV/√Hz; 18 MHz; 2.2 / 2.5 mA per channel; ±2.25 V to ±18 V; output swing ≈ (V+) − 0.3 V at the ≈ 3 mA I/V load (estimate; TI: (V+) − 0.2 V at 10 kΩ, (V+) − 0.6 V at 600 Ω); input current 10 mA (absolute maximum); same dual pinout as the OPA2197 it replaces and as the OPA1612AIDR fallback | SOIC-8; leaded; Extended; the fixture sentence on its JLCPCB page is standard text on every part page | US$1.0035 (JLCPCB 10+; 1+ US$1.2180, 100+ US$0.7530) | LCSC C1849415; JLCPCB stock unverified (the 76 figure is 12 months old): check the JLCPCB Parts Manager and buy 12 (10 + 2 spares) if ≥ 12, otherwise switch to the OPA1612AIDR fallback (C94590, Extended; LCSC 10+ US$1.0942, stock 9,754; JLCPCB US$1.33, 31,254 available, page of 28 Aug 2026; VREF ≥ 35 mV inside its (V+) − 2 V limit at +3.90 V); the OPA2210 at the LCSC price (US$2.67 at 10+) would add ≈ 106 NTD per board, over the ceiling; TI SBOS924H |
| TI OPA1622IDRCR (× 2) | Output legs, one amplifier per leg | Vos ±0.5 mV max; Isc +145/−130 mA typ; clean (low-distortion) swing ends ≈ 1.3 V from the rail (TI data at ±5 V: 600 Ω → 11.3 mW, 32 Ω → 150 mW), falling ≈ 8 Ω × Iout; current limit ≈ 110–120 mA at 85 °C die; EN threshold ≈ 0.82 V referenced to its GND pin (pin 3, tied to GND); continuous short rated for one amplifier per package; θJA 47.6 °C/W | VSON-10, 3 × 3 mm; leadless, X-ray fee; thermal pad to V−; Extended | US$1.79 (10+ tier) | LCSC C2876559; TI SBOS727B |
| TI LM27762DSSR | +3.90 V / −3.72 V headphone rails | VOUT+ = 1.2 V × (R1 + R2)/R2, VOUT− = −1.22 V × (R3 + R4)/R4: +3.900 V (3.789–4.014 V) and −3.722 V (3.618–3.828 V magnitude) including VFB and the 1 % resistors; ±250 mA; negative LDO dropout 30 mV and positive 45 mV at 100 mA; input current limit 500 mA typ; PGOOD open drain pulled up to 5V_ANA (R505 10 kΩ), low = both outputs ≥ 95 % (+3.71 V, −3.54 V), sets the latch (Q610) and drives the Q609 interlock; 0.39 W at 5.25 V (θJA 62.2 °C/W, ≈ +24 °C), 0.46 W at 5.5 V; VIN rated to 5.5 V (no margin above it, accepted for v1); 50 kΩ output pull-down in shutdown (not relied on) | WSON-12, 2 × 3 mm; leadless, X-ray fee; Extended | US$1.0174 | LCSC C473398; TI SNVSAF7C |
| TI TPS2553DBVR | Current-limited switch for 5V_ANA (Default and High limits) | Limit set by RILIM (15–232 kΩ), acting on the instantaneous current (short-circuit response ≈ 2 µs, then constant-current limiting); rDS(on) 85 mΩ typ / 135 mΩ max; 9 mW in normal use, ≤ 2.6 W into a short for ≤ 10 ms before the latch or FAULT path acts (thermal shutdown protects); soft start; FAULT open drain, 5/7.5/10 ms deglitch, also sets the fault latch through D606 and R638; auto-retry version kept (the latch-off TPS2553-1 is listed as not recommended for new designs) | SOT-23-6; leaded; Extended | US$0.60 | LCSC C55266; TI SLVS841F |
| TI LM339DR (× 2, U601 and U602) | DC-detect window comparators, one window per leg (replace the LM393BIDR) | Quad, open collector, on the +3.90 V / −3.72 V rails; supply current 0.8 mA typical, 2 mA maximum per package; each leg filter node drives a positive (IN− = node) and a negative (IN+ = node) comparator directly, no diode-OR; offset 5 mV at 25 °C, 9 mV over temperature; input bias 250 nA maximum at 25 °C, 400 nA over 0–70 °C per input, flowing out (node shift 0…+81 mV through 100 kΩ, inside the worst case); TI: one input can exceed VCC and the comparator provides a proper output state as long as the other input remains in the common-mode range (TI LM339 datasheet, SLCS006Z), so a rail-stuck leg still trips; no input goes more than 0.3 V below the negative supply (all nodes stay within the rails); ESD ±2 kV HBM; eight outputs wired-OR into node F; C612–C615 4 × 100 nF supply decoupling | SOIC-14; leaded; Extended (replaces the LM393BIDR, so the Extended-type count is unchanged) | US$0.18 (JLCPCB); LCSC 5+ US$0.1666 | LCSC C7948 (JLCPCB available 19,325); drop-in alternative LM339BIDR (C22416291, IB ≤ 50 nA, 2 kV ESD), preferred if JLCPCB stocks it (≈ −3.6 NTD per board) |
| Microne ME6216A13M3G | 1.3 V external DVDD (from 3V3D) | Fixed 1.3 V, ±1 % (±15 mV), 250 mA max, ceramic-stable, 6 µA Iq; Pd 0.54 W; pin 1 VSS, pin 2 VOUT, pin 3 VIN; short-circuit foldback 50–70 mA | SOT23-3 (“M3”); leaded; Extended | US$0.06 | LCSC C236662 (≈ 3,100 in stock) |
| Murata BLM18PG121SN1D | 5V_ANA power ferrite | 120 Ω at 100 MHz; 2 A; 50 mΩ | 0603; JLCPCB Basic | US$0.016 | LCSC C14709 |

Not in the table: U205, a Nexperia 74LVC1G157GW,125 2:1 logic multiplexer (LCSC C135822, SOT-363, Extended), with R215 (33 Ω) and the R216/R217 10 kΩ input pull-downs, forms a DNF fallback footprint at the CPLD PLL clock input (select = OSC44_EN). To use it, fit U205, R215, R216 and R217 and remove R210 and R211; one of the two Stage 2 boards is built with it fitted.

All other parts are chosen in Schematic Design Notes v0.7 (part number, LCSC number, stock and price). Parts List v0.6 is the source of truth for cost; every line’s JLCPCB class is taken from its JLCPCB part page or the basic-parts list, except X202, whose class is not shown (assumed Extended).

# 5. Headphone drive and estimates

## 5.1 Load requirements

Current limits the low-impedance loads. The clean output envelope is ≤ 80 mA peak per leg within the OPA1622 clean swing, and 70 mA at 4.4 mm 32 Ω, where 80 mA would leave negative headroom on the negative rail with a 3.5 Ω charge-pump resistance (Section 3.2); overload behaviour is handled separately by the TPS2553 current limit and the fault latch (Section 5.3). The OPA1622’s +145/−130 mA figure is a typical short-circuit current and its current limit is ≈ 110–120 mA at an 85 °C die, so neither is a linear output rating.

| Requirement | Value | Basis |
| --- | --- | --- |
| Loads | 16–600 Ω on both jacks | Load matrix in Section 5.2 |
| Clean envelope | ≤ 80 mA peak per leg; 70 mA at 4.4 mm 32 Ω | One amplifier per leg (U401 = L+/L−, U402 = R+/R−); 70 mA from the negative-rail headroom (Section 3.2) |
| Swing model | Vpk ≤ (¦rail¦min − 1.3 V)/(1 + 8 Ω/Rleg) | Clean swing: distortion starts ≈ 1.3 V from the rail (TI data at ±5 V: 600 Ω → 11.3 mW, 32 Ω → 150 mW); worst-case rail ¦rail¦min = 3.618 V (bench-verify) |
| Rails | +3.900 V (3.789–4.014 V), −3.722 V (−3.618 to −3.828 V) | LM27762 feedback reference (VFB) and the 1 % divider resistors |
| Full-scale level | 1.55 Vrms per leg (2.19 V peak) after calibration | Section 3.3 |
| Output impedance | < 0.1 Ω plus jack contacts (the OPA1622 legs drive the jacks directly, no ballast); target ≤ 0.5 Ω on both jacks | Estimate |
| Reference headphones | HIFIMAN Susvara (60 Ω, 83 dB/mW); AKG K1000 (120 Ω, 74 dB/mW) | Maker specifications; SPL in Section 5.2 |

## 5.2 Output power and SPL

Rated power is defined at 1 kHz, both channels driven, at 1 % THD+N or calibrated 0 dBFS, whichever comes first, for the worst-case unit on the worst-case rail (|rail|min = 3.618 V; clean-swing model Vpk ≤ (|rail|min − 1.3 V)/(1 + 8 Ω/Rleg); 80 mA clean envelope, 70 mA at 4.4 mm 32 Ω; calibrated full scale 1.55 Vrms per leg, 2.19 V peak). Default mode is rated by the instantaneous peak current, because the TPS2553 limits instantaneous current: quiescent (≈ 56 mA max, incl. two LM339 at 2 mA each) + 2 × 1.03 × Ipk (unbalanced: both channels on one rail, including the charge-pump factor) or + 2 × Ipk × 2.03 (balanced, both rails) must stay ≤ 157 mA, the Default minimum. High-mode peaks stay below the 393 mA High minimum (worst 340 mA at 4.4 mm 32 Ω). Estimates, with the limiting factor in brackets:

| Load | High mode (limit) | Default mode (limit) | Analog input current (High mode), average / instantaneous peak |
| --- | --- | --- | --- |
| 3.5 mm 16 Ω | 51.2 mW (80 mA envelope) | 19.3 mW (Default limit) | 156 / 220 mA |
| 3.5 mm 32 Ω | 53.8 mW (swing) | 38.6 mW (Default limit) | 128 / 175 mA |
| 3.5 mm 60 Ω | 34.9 mW (swing) | 34.9 mW (swing) | 97 / 126 mA |
| 3.5 mm 300 Ω | 8.0 mW (full scale) | 8.0 mW (full scale) | 62 / 71 mA |
| 4.4 mm 32 Ω | 78.4 mW (70 mA envelope) | 9.9 mW (Default limit) | 234 / 340 mA |
| 4.4 mm 60 Ω | 111.7 mW (swing) | 18.6 mW (Default limit) | 211 / 303 mA |
| 4.4 mm 120 Ω | 69.7 mW (swing) | 37.3 mW (Default limit) | 141 / 194 mA |
| 4.4 mm 300 Ω | 32.0 mW (full scale) | 32.0 mW (full scale) | 91 / 115 mA |
| 4.4 mm 600 Ω | 16.0 mW (full scale) | 16.0 mW (full scale) | 72 / 85 mA |

SPL from the maker sensitivity, 4.4 mm balanced at the rated power (estimates). At full scale the worst-case unit reaches the clean-swing limit below full scale on several loads: 3.5 mm 16 / 32 / 60 Ω at 3.0 / 1.5 / 0.6 dB below, 4.4 mm 60 / 120 Ω at 1.6 / 0.6 dB below, and 300 Ω balanced only just inside (+0.04 dB), so these loads are rated as swing-limited and bench-verified, because the swing model rests on typical data.

| Headphone | Impedance, sensitivity | High mode: power, SPL | Default mode: power, SPL | Limit (High / Default) |
| --- | --- | --- | --- | --- |
| HIFIMAN Susvara | 60 Ω, 83 dB/mW | 111.7 mW, ≈ 103.5 dB SPL | 18.6 mW, ≈ 95.7 dB SPL | Swing / Default limit |
| AKG K1000 | 120 Ω, 74 dB/mW | 69.7 mW, ≈ 92.4 dB SPL | 37.3 mW, ≈ 89.7 dB SPL | Swing / Default limit |

## 5.3 Power, offset, noise, bit clock, thermal and protection

USB current per state (estimates at 5 V, typical / maximum). The TPS2553 limits only the analog branch; the MCU, CPLD and DAC-core currents are estimates or datasheet values, so the Default- and High-mode figures are estimated maxima with hardware-limited analog current. Digital loads: MCU 60 / 80 mA (assumption); LEDs, latch and pull-ups 2 / 3 mA; CPLD 15 / 25 mA (assumption; AG32 silicon, dynamic current measured on Stage 2); X201 80 MHz 15 / 40 mA (SCTF 3C datasheet: 40 mA max for 80–160 MHz; typical assumed); X202 3 / 5 mA; X203 5 / 10 mA (both family oscillators counted); DAC DVCC 13.5 / 14.2 mA; DAC external DVDD 50 / 60 mA (datasheet 50 mA at MCLK 80 MHz and 192 kHz; +20 % for 384 kHz and spread); AVCC_L + AVCC_R + VCCA + VREF divider 17 / 17.5 mA; DC-sense bias and 3V3A monitor 0.87 mA; enable and node pull-downs 1.6 / 1.7 mA; I²C pull-ups 1.5 mA; bleeders R524 and R525 2.8 mA; digital total 187 / 262 mA:

| State | Powered domains | Current | Limit / note |
| --- | --- | --- | --- |
| Attached, not configured | 3V3M only (MCU, latch, LEDs) | 62 / 83 mA | ≤ 100 mA unit load; measured on the Stage 2 board before layout (assumption on the MCU current) |
| Configured, idle | 3V3M, 3V3D, 3V3A and the headphone rails | 226 / 311 mA | Digital 187 / 262 mA plus the analog quiescent current (the OPA2210 adds ≈ 10 mA) |
| Playing, Default mode | All; analog current ≤ TPS2553 Default limit | Estimated maximum 0.475 A (0.262 + 0.213 A) | ≤ 500 mA with hardware-limited analog current (25 mA below 0.5 A); Stage 2 measurement is a layout gate |
| Playing, High mode | All; analog current ≤ TPS2553 High limit | Estimated maximum 0.760 A | Only with a ≥ 1.5 A Type-C advertisement; CC monitored while configured |
| Suspend | 3V3M only: MCU in Stop mode with the USBHS PHY suspended, both LEDs off; audio domains, DAC analog supply (and with it the DC-sense bias) and rails off | Requirement ≤ 2.5 mA within 10 ms of the last bus activity; budget ≈ 2.30 mA (margin 0.20 mA) | v1 requirement. Budget (mA): MCU Stop with the USBHS PHY suspended ≤ 1.9 (assumption, Stage 2 gate); D+ 1.5 kΩ pull-up into the host’s 15 kΩ 0.20; ME6211 3V3M quiescent 0.06 (60 µA typ); latch pull-up 0.07; node S pull-up 0.033; VBUS divider 0.034; LEDs 0 (off in suspend); DC-sense networks 0 (biased from 3V3A, which is off); regulators off 0.003; PGOOD pull-up 0 (now on 5V_ANA, which is off); node F sits high (no current). Current vs time measured on the Stage 2 board (gate) and on every main board |

Analog input current per load, average / instantaneous peak, is listed in the power table (Section 5.2); the worst clean load is 4.4 mm 32 Ω in High mode (234 / 340 mA), which is why 32 Ω balanced is not rated in Default mode. The analog quiescent current is 45 / ≈ 56 mA (maximum including two LM339 at 2 mA each) (the OPA2210 adds ≈ 10 mA), and the analog input power is ≤ 1.06 W in Default mode and ≤ 2.49 W in High mode.

DC offset per leg, worst case at 25 °C with datasheet maxima (VREF at the design value 1.703 V):

| Term | Per leg | Basis |
| --- | --- | --- |
| Common-mode leakage | 3.40 mV | VREF 1.703 V × 0.2 % (0.1 % resistors, gain 1) |
| OPA1622 Vos and Ios | 1.10 mV | Vos 0.5 mV max × 2 (1.00 mV); Ios × 1 kΩ × 2 (0.10 mV) |
| OPA2210 Vos | 0.15 mV | 35 µV max × noise gain ≤ 2.17 × 2 (two amplifiers) |
| DAC pin-to-pin DC mismatch | 1.18 mV | ≤ 1 mV (assumption, measure) × Rf/Rs |
| VREF-to-null error | 1.17 mV | Divider tolerance and an assumed ±20 mV null-point uncertainty, × 3 % pair-gain mismatch |
| DAC-pin clamp leakage (D405–D408) | 0.70 mV | ≤ 0.4 µA net per pin (assumption: the two diodes of a package see almost equal reverse voltage, 1.70 V to AGND and 1.60 V to AVCC, so their leakages largely cancel; datasheet maximum 2 µA at 25 V, 25 °C) × 836 Ω = ≤ 0.35 mV per pin, two DAC pins per leg |
| Total, unbalanced | ≤ 7.7 mV worst (≈ 4.0 mV RSS) | Limit ±10 mV (margin 2.3 mV); ≤ 9.3 mV if the DAC null is 1.65 V; 9.7 mV with the OPA1612 fallback (at the edge) |
| Total, balanced | ≤ 15.4 mV worst | Limit ±20 mV (margin 4.6 mV; ≤ 18.6 mV if the DAC null is 1.65 V; 19.4 mV with the OPA1612 fallback, at the edge): 20 mV into 16 Ω balanced = 25 µW DC |

Every board’s offset is measured by the automated test with a DC-capable DMM (≥ 4½ digits, SCPI over USB), because the QA403 inputs are AC-coupled (100 % screening); the DAC-mismatch assumption stays open until measured. DAC centre voltage: the ES9018K2M datasheet gives two values, 1.702 V (current-output formula 2.112 mA − 1000·Vg/806 = 0) and 1.65 V (voltage-source model, AVCC/2), and the design uses 1.703 V. If the null is 1.65 V, the VREF-to-null term becomes 2.8 mV and the worst case ≤ 9.3 mV unbalanced and ≤ 18.6 mV balanced, including the clamp leakage (limits ±10 / ±20 mV); R433 1 kΩ → 0 Ω then sets VREF = 1.650 V, decided after the VREF null test at bring-up of the first main boards.

Noise (estimates): the I/V output is 6.95 nV/√Hz (OPA2210 2.2 nV/√Hz × 2.04 plus the Rf and DAC-source thermal noise); the leg network (four 2 kΩ resistors in a unity-gain difference amplifier) adds ≈ 11.5 nV/√Hz of its own, ≈ 13.0 nV/√Hz with the OPA1622, so an unbalanced leg is ≈ 16.3 nV/√Hz. Balanced, both legs share the same two I/V converters (leg+ = I/V(DAC−) − I/V(DAC+), leg− the swap), so the I/V noise doubles with the signal and only the leg-network noise adds as √2: ≈ 26.9 nV/√Hz against a signal ×2, a balanced advantage of ≈ 1.2 dB, not 3 dB. VREF noise (LP5907 < 6.5 µVrms × 0.516) is common mode; the residual through the 3 % pair-gain mismatch is ≈ 0.2 µVrms per leg, negligible. Dynamic range at the calibrated 1.55 Vrms per leg (A-weighted, with the DAC’s 127 dB(A), which is also common to both legs): ≈ 117.4 dB(A) unbalanced and ≈ 118.6 dB(A) balanced. Target ≥ 115 dB(A), minimum ≥ 112 dB(A), unchanged; measured results are recorded with the analyzer loopback floor. The I/V pole (288 kHz) and the leg pole (241 kHz) give −0.05 dB at 20 kHz and −0.20 dB at 40 kHz.

Bit-clock duty cycle (design calculation per rate). The ES9018K2M requires a duty cycle of 45:55 to 55:45, high and low times ≥ 4.5 ns and a cycle ≥ 10 ns, with data setup 4.1 ns and hold 2 ns to the rising edge (datasheet v3.7, Audio Interface Timing); the CPLD changes SDATA and LRCLK on the falling BCLK edge, ≥ 20 ns before the next rising edge at 24.576 MHz BCLK. With the selected family oscillator doubled by the CPLD PLL, every rate is an exact integer division, and the PLL ×2 also turns the oscillators’ 45–55 % duty into a 50:50 bit clock. Integer division sets the nominal duty, so the table gives the ideal duty; PLL jitter and output timing are verified on the bench:

| Rate | Bit clock (64 × fs) from | Ideal duty | Against 45:55 to 55:45 |
| --- | --- | --- | --- |
| 48 kHz | X202 24.576 MHz × 2 (CPLD PLL) = 49.152 MHz ÷ 16 | 50 % | Inside (exact integer ratio) |
| 96 kHz | 49.152 MHz ÷ 8 | 50 % | Inside (exact integer ratio) |
| 192 kHz | 49.152 MHz ÷ 4 | 50 % | Inside (exact integer ratio) |
| 384 kHz | 49.152 MHz ÷ 2 | 50 % | Inside (exact integer ratio) |
| 44.1 kHz | X203 22.5792 MHz × 2 (CPLD PLL) = 45.1584 MHz ÷ 16 | 50 % | Inside (exact integer ratio) |
| 88.2 kHz | 45.1584 MHz ÷ 8 | 50 % | Inside (exact integer ratio) |
| 176.4 kHz | 45.1584 MHz ÷ 4 | 50 % | Inside (exact integer ratio) |
| 352.8 kHz | 45.1584 MHz ÷ 2 | 50 % | Inside (exact integer ratio) |

Thermal: the analog input power is bounded by the TPS2553 limit (≤ 1.06 W Default, ≤ 2.49 W High); the board worst case is ≈ 3.8 W (3.77 W, High mode, overload). A 4.4-to-3.5 mm adapter shorts leg−, one amplifier per package: ≈ 0.35 W in the shorted amplifier + ≈ 0.1 W in the other, ≈ 0.45 W per package, +21 °C (θJA 47.6 °C/W). A 4.4 mm plug shorting leg+ to leg− loads both amplifiers of one package with ≈ 0.55 W per package, outside TI’s one-amplifier continuous-short rating, so it gets a bench survival test. Regulator and power-part dissipation (estimates; 3V3D maximum load: CPLD 25, X201 40, X202 5, X203 10, DVCC 14.2, DVDD 60 through U303, enables and nodes 1.7, I²C 1.5, bleeders 2.8 mA, 160 mA in total; layout rule: copper pours on the LDO VIN, GND and VOUT pins; IR or thermocouple check at bring-up):

| Part | Load | Dissipation | Note |
| --- | --- | --- | --- |
| U504 3V3D ME6211, without dropper | 160 mA max | 0.27 W at 5.0 V, 0.31 W at 5.25 V | ME6211 SOT-23 power dissipation rating 300 mW (datasheet absolute maximum) → too close |
| U504 with R519 + R520 (2 × 2.2 Ω 0805 in series, 4.4 Ω) | 160 mA max (including the bleeders) | 0.20 W at 5.25 V, 0.24 W at 5.5 V (vSafe5V maximum); ≈ 56 mW per resistor (0.125 W rating) | LDO input ≥ 3.70 V at VBUS 4.40 V (≈ 0.25 V dropout needed, estimate) |
| U502 3V3M ME6211 | 84 mA | 0.16 W at 5.25 V | — |
| U303 1.3 V ME6216 | 60 mA | 0.12 W | Pd rating 0.54 W (SOT23-3): inside |
| U302 LP5907 | 17.5 mA | 0.03 W | — |
| U501 LM27762 | Worst clean load (4.4 mm 32 Ω, High) | 0.39 W at 5.25 V, 0.46 W at 5.5 V | θJA 62.2 °C/W → ≈ +24 °C at 5.25 V; copper pours |
| U503 TPS2553 | 0.26 A | 9 mW normal; ≤ 2.6 W into a short for ≤ 10 ms before the latch/FAULT path | Its thermal shutdown protects |

Protection is required, because the target headphones are expensive and hard to replace. There are no relays; the outputs are kept safe by the power sequence, the hardware fault latch with its fast capture, the slow firmware DC tier and the Q609 interlock:

| Function | Implementation | Requirement |
| --- | --- | --- |
| DC detect (hardware) | Each leg → R601–R604 100 kΩ + C601–C604 1 µF (τ = 100 ms; 84–111 ms with 1 % R and C −15 %/+10 % incl. DC bias) → two comparators of U601/U602 (TI LM339DR) directly on the node, no diode-OR: positive (IN− = node, IN+ = VTH+) and negative (IN+ = node, IN− = VTH−); VTH+ from V+ through R607 47 kΩ + R636 15 kΩ over R609 10 kΩ (+0.517…+0.567 V), VTH− from V− through R608 47 kΩ + R637 10 kΩ over R610 10 kΩ (−0.531…−0.581 V); C612–C615 100 nF decoupling → leg trip +0.43…+0.58 V and −0.52…−0.67 V, 0.43–0.67 V worst case (nominal ≈ 0.55 V), including the comparator offset (≤ 9 mV) and bias current (≤ 400 nA per input: 0…+81 mV at the node), all datasheet-guaranteed; idle margin ≥ 0.43 V | Worst case: 1.0 V → 125 ms (≤ 250 ms), 2.0 V → 47 ms (≤ 100 ms), including node F and the latch capture (35 µs); no false trip for full-scale content above 9.5 Hz worst case (6.2 Hz nominal); opposite-sign legs below the trip (up to 1.15 V across a balanced driver) are left to the firmware tiers |
| Slow DC tier (firmware) | Per leg, from the jack side of R417–R420: R624–R627 15 kΩ (series), R628–R631 10 kΩ (to 3V3A), R632–R635 22 kΩ (to GND), C608–C611 2.2 µF → MCU ADC2 node = 1.556 V + 0.314 × leg, 0.29–2.85 V at every corner, source 4.7 kΩ, τ 6–10 ms, leg load ≈ 104 µA (driven output); 3V3A monitor R639/R640 10 kΩ/10 kΩ + C616 100 nF on PC4 for the ratiometric correction; 1 kHz dithered sampling, factory zero plus a power-up auto-zero (±30 mV acceptance); error ≤ 44 mV per leg and ≤ 88 mV per pair difference, worst case. The firmware sets the latch by switching ANA_FAULT to open-drain low → D606 → R638 → node F | Four tiers on max(¦each leg¦, ¦L+ − L−¦, ¦R+ − R−¦): ≥ 1.0 V within 0.25 s, ≥ 0.5 V within 0.35 s, ≥ 0.3 V within 1.1 s, ≥ 0.2 V within 3 s (T4 0.11 V nominal, worst-case trip 0.022–0.198 V on a pair); no false trip for full-scale content above 4.7–9.3 Hz (per tier); armed whenever AVCC_EN is high; every sub-hardware-trip case ≤ 28.2 mJ at 16 Ω and ≤ 14.1 mJ at 32 Ω; residual ≤ 0.198 V across a driver (≤ 2.45 mW into 16 Ω, ≤ 1.23 mW into 32 Ω): accepted by the owner on 24 Sep 2026 |
| Hardware kill paths | When the latch is set, Q607 pulls RAIL_EN (LM27762 EN+ and EN−), Q608 pulls AMP_EN (both OPA1622 EN) and Q611 pulls AVCC_EN (LP5907) low; each enable has a 100 kΩ pull-down and is driven by the MCU through a 10 kΩ series resistor (R507, R618 and the new R307), so a kill FET never fights a GPIO (Q611 pulls against 0.33 mA). Q609 (gate from RAILS_OK_N through R623 1 MΩ, C607 10 nF to GND) is a slow static interlock: on 1.9–9.4 ms after PGOOD goes bad, off 5–19 ms after it goes good, so it keeps AMP_EN low whenever the rails are off, also after a firmware clear | No firmware needed; ≈ 35 µs from PGOOD bad to RAIL_EN, AMP_EN and AVCC_EN low; a short PGOOD glitch either latches or does nothing (never an un-latched AMP_EN dropout); EN stays below the OPA1622 input limit whenever the rails are off |
| Fault latch | N-MOSFETs only (11 × 2N7002, Q601–Q611): the eight LM339 open-collector outputs, wired-OR → R611 10 kΩ → node F (47 kΩ to 3V3M, C605 100 nF to GND, 1N4148WS clamp D605: ≈ −0.7 V on a fault, ≤ 0.31 mA) → Q601 inverter → set request S → Q602 sets the Q604/Q605 cross-coupled latch (R614/R615 47 kΩ pull-ups, no latch capacitor; FLT high = fault). Q610 (gate RAILS_OK_N: R505 10 kΩ to 5V_ANA; the node carries only ≥ 1 MΩ RC branches and high-impedance gate and MCU inputs) pulls node F low when PGOOD is bad; D606 with R638 1 kΩ (node F to ANA_FAULT) lets a sustained TPS2553 overcurrent or the firmware set it; Q606 (MCU FLT_CLR) clears it; C605 starts node F at 0 V, so the latch sets at attach independent of the unpowered LM27762 PGOOD pin; the MCU reads FLT, ANA_FAULT and RAILS_OK_N (through R522 10 kΩ) as floating inputs without internal pulls | Capture ≈ 35 µs, now genuine (with the real loads RAILS_OK_N steps to 4.31–5.15 V, 0.98 × 5V_ANA, and crosses 2.5 V in ≈ 1.0–1.3 µs; budget 0.1 ms, measured); node F stays low 1.5–7.7 ms after any PGOOD or fault pulse, so every pulse that turns Q610 on is latched; with ANA_FAULT low, F sits at 0.5–0.7 V against the Q601 VGS(th) minimum of 1.0 V (0.3–0.5 V margin, smaller at low temperature); a clear does not stick while a set request is present; C605 discharges through Q610 in a sub-µs ≈ 0.5 A pulse, fine for a 2N7002, so no series resistor is added there |
| Blanking | Q603 holds S low after PGOOD goes good (R617 1 MΩ + C606 100 nF from PGOOD; 5V_ANA 4.40–5.25 V, 2N7002 VGS(th) 1.0–2.5 V, R 1 %, C ±10 %): the set path is armed 51–186 ms after PGOOD goes good (from a fully charged blanking capacitor, i.e. ≥ 600 ms of rails off) and re-blanked 19–94 ms after PGOOD goes bad | Blocks only the set request and never clears a set latch (no hiccup); an overload that drops PGOOD sets the latch at once, while the PGOOD-bad request at start-up is blanked; firmware: FLT_CLR only ≥ 600 ms after ANA_EN or after the rails went off, a second FLT_CLR at PGOOD good + 20 ms, AMP_EN only ≥ 250 ms after PGOOD good (after the latest arming time, 186 ms); start-up rules for faults present at power-up (Section 5.3) |
| Energy at the terminals | E = V²/R × (detector + node F + latch time) + the energy after the trip (tables below): a rail-stuck failure in High mode (4.01 V maximum rail) 21.4 mJ at 16 Ω and 11.0 mJ at 32 Ω worst case; digital DC with a healthy output stage ≤ 10.8 mJ at 16 Ω (unbalanced). After the trip both rails’ capacitors (C +10 %, maximum rails) hold 0.52 mJ and DAC back-power into a failed stage adds ≤ 1 µJ → ≤ 0.6 mJ; the LM27762 50 kΩ discharge is not relied on | ≤ 30 mJ at 16 Ω and ≤ 15 mJ at 32 Ω, worst case, including the after-trip energy; balanced digital DC on a healthy stage is not covered by this requirement (hardware alone 32.8 / 21.7 mJ) and is limited by the stream DC guard (≈ 10 / 12 mJ); OPA1622 shutdown is not treated as isolation (TI: the output is not high-impedance) |
| Power sequence and DAC pins | VREF tracks AVCC (C421 33 pF, τ ≈ 0.29 µs), and in normal operation the I/V converters hold the DAC pins at VREF. Power-up enables the legs before AVCC; controlled power-down stops MCLK (MCLK_EN low) and turns AVCC_EN off first, with RESETB still high (DAC muted) and the I/V converters and legs powered, then AMP_EN (adaptive, 3V3A monitor < 0.1 V), RESETB, AUD_EN and the rails. In reset the DAC outputs are driven to ground (≈ 2.1 mA per pin with AVCC up), so the AVCC-on step with RESETB low leaves ≈ 18–35 mV at the leg until the soft start (measured). In a failed-output shutdown the OPA2210 positive common-mode limit drops below VREF 0.11–0.39 ms after the kill, while 3V3A needs 3.2–5.6 ms to fall below 0.3 V, so the I/V converters lose control of the pins for up to ≈ 5.5 ms (unclamped: toward −1.76 V, stopped only by the DAC’s internal protection at ≈ −0.6 V with ≤ 3.9 mA, or ≈ 0.33 V above AVCC); the fitted BAT54S clamps D405–D408 (pin 3 to the DAC pin, pin 1 to AGND, pin 2 to AVCC_L/AVCC_R, within 5 mm) hold each pin within ≈ −0.40 V…AVCC + 0.40 V at ≤ 4.4 mA and are reverse biased by ≈ 1.6–1.7 V in normal operation | ES9018K2M absolute maximum GND < Vout < AVCC on DACL, DACR, DACLB and DACRB (no margin, no injection rating): inside in every controlled transition (the clamps never conduct); at a sudden unplug above AVCC only while AVCC < ≈ 31 mV (≤ 15 mV VREF lag); after an output-stage failure within ≈ −0.40 V…AVCC + 0.40 V for ≤ 5.5 ms (small residual risk of an already-failed board); measured on the four DAC pins at the R423–R426 DAC-side pads against AVCC_L/AVCC_R |
| Overload, cause and recovery | Analog current reaches the TPS2553 limit → rails droop → PGOOD bad (Q610) or, after the FAULT deglitch, D606 → latch set → rails, amplifiers and AVCC off. At the FLT interrupt the firmware first (≤ 50 µs) drives AMP_EN, RAIL_EN and AVCC_EN low, then AUD_EN and RESETB, and then stores the pre-trip ring-buffer averages of the four legs and the two pair differences, ANA_FAULT, RAILS_OK_N and whether it set the latch itself. The cause is classified from the pre-trip rolling 0.5-s averages plus the event history: a DC fault if they show DC on a leg or a pair or the firmware tripped, an overload if they show none while the overload flags are set; events that match neither pattern are logged as “ambiguous” and counted toward the DC three-strike limit (conservative, deliberate); unavailable readings count as a DC fault. FLT during a controlled power-down or before the first FLT_CLR after power-up is expected: not logged, not counted, no ceiling change | Overload: soft mute, volume ceiling −3 dB per event (down to −12 dB), clear after ≥ 1 s (FLT_CLR only after the rails have been off ≥ 600 ms), ceiling kept until replug; overloads count toward the 3-strike stop only once the ceiling has reached −12 dB. DC or ambiguous fault: clear after 2 s; 3 such faults within 10 s → latched until USB replug; red LED blinks. Overload tests in Section 7 |
| Unplug and brownout | The VBUS analog watchdog (ADC1 continuous on VBUS_SENSE, 4.10 V nominal, 3.95–4.25 V, ≈ 3.8 µs worst-case latency) drops AMP_EN and RAIL_EN at once (target < 5 µs), then RESETB, AUD_EN, AVCC_EN and ANA_EN; VBUS falls at up to ≈ 0.1 V/µs at 0.47 A; PGOOD goes bad at VBUS ≈ 3.70 V while 5V_ANA is still ≈ 3.6 V, so Q610 sets the latch as the hardware backstop | Pop ≤ 20 mV peak (target, measured); VREF lags AVCC by ≤ 15 mV (C421 33 pF), DAC pins measured at hot unplug under full load; watchdog-to-GPIO latency measured on Stage 2 |
| Adapter short | A 4.4-to-3.5 mm adapter shorts leg−; with U401 = L+/L− and U402 = R+/R−, only one amplifier per package is shorted (TI continuous-short rating) | ≈ 0.45 W per package; warning in the manual |
| Jack ESD and bring-up safety | 4 × LESD5D5.0CT1G (5 V standoff, 12 pF), one per jack leg; bring-up with dummy resistors only; every new board is first powered without headphones, each leg checked by DMM | Never test on real headphones |

Protection envelope, worst case, for the hardware detector (removed = RAIL_EN, AMP_EN and AVCC_EN pulled low): a DC fault ≥ 2 V on any leg is removed within 100 ms and ≥ 1 V within 250 ms. Each leg filter node (R601–R604 100 kΩ + C601–C604 1 µF) drives two LM339 comparators directly, a positive one (IN− = node, IN+ = VTH+) and a negative one (IN+ = node, IN− = VTH−); VTH+ = V+ × 10 kΩ/(R607 47 kΩ + R636 15 kΩ + R609 10 kΩ) = +0.517…+0.567 V and VTH− = V− × 10 kΩ/(R608 47 kΩ + R637 10 kΩ + R610 10 kΩ) = −0.531…−0.581 V. The worst case combines the rails (3.789–4.014 V / 3.618–3.828 V), 1 % resistors, the comparator offset (5 mV at 25 °C, ≤ 9 mV over 0–70 °C), its input bias current (250 nA maximum at 25 °C, ≤ 400 nA per input over 0–70 °C, flowing out; two inputs per node through 100 kΩ shift the node by 0…+81 mV), C601–C604 −15 %/+10 % (tolerance and DC bias), the node F delay through R611 into C605 and the latch capture (35 µs): leg trip +0.43…+0.58 V and −0.52…−0.67 V, so 0.43–0.67 V worst case (nominal ≈ 0.55 V), τ 84–111 ms, idle margin ≥ 0.43 V. In a rail-stuck fault one input of a comparator can lie above its common-mode range; TI states that one input can exceed VCC and the comparator provides a proper output state as long as the other input remains in the common-mode range (TI LM339 datasheet, SLCS006Z), which the threshold input always does, so the output stays valid. No input goes more than 0.3 V below the comparator’s negative supply, because every node stays within the rails. No false trip for full-scale content above 9.5 Hz worst case (6.2 Hz nominal). Opposite-sign legs below the hardware trip can put up to 1.15 V across a balanced driver without a hardware trip; the firmware tiers cover that case:

| DC on a leg | Removal time, nominal | Removal time, worst case | Requirement |
| --- | --- | --- | --- |
| 1.0 V | 80 ms | 125 ms | ≤ 250 ms (50 % margin) |
| 1.5 V | 46 ms | 67 ms | — |
| 2.0 V | 33 ms | 47 ms | ≤ 100 ms (53 % margin) |
| 2.19 V (full-scale digital DC at 1.55 Vrms) | 30 ms | 42 ms | — |
| 3.83–4.01 V (rail-stuck) | — | 19–23 ms | — |

Terminal energy E = V²/R × (detector + node F + latch time) + the energy after the trip, recalculated with the worst-case LM339 window detector (amended 24 Sep 2026) (the rail-stuck column includes the after-trip energy; nominal values in brackets):

| Load | Rail-stuck failure, High mode, worst case incl. after-trip (nominal) | Digital DC on a healthy output stage, worst case (V at the leg, removal time, energy; balanced ±x not covered by the rail-stuck limit) |
| --- | --- | --- |
| 16 Ω | 21.4 mJ (15.5 mJ); limit 30 mJ | 1.81 V (swing-limited), 52.8 ms, 10.8 mJ; balanced ±x, hardware only: 2.32 V across the driver (current-limited), 97 ms, 32.8 mJ (stream DC guard ≈ 10 mJ) |
| 32 Ω | 11.0 mJ (8.0 mJ); limit 15 mJ | 2.17 V (swing-limited), 42.4 ms, 6.3 mJ; balanced ±x, hardware only: 3.62 V across the driver, 52.8 ms, 21.7 mJ (stream DC guard ≈ 12 mJ) |
| 60 Ω | 6.1 mJ (4.5 mJ) | 2.19 V (full scale), 42.0 ms, 3.4 mJ |
| 300 Ω | 1.6 mJ (1.2 mJ) | 2.19 V (full scale), 42.0 ms, 0.7 mJ |

After the trip both rails’ capacitors, with C +10 % at the maximum rail voltage, hold 0.52 mJ, and DAC back-power into a failed stage adds ≤ 1 µJ, so the after-trip bound is ≤ 0.6 mJ (the LM27762 50 kΩ discharge is not relied on). Requirement (hard, worst case): ≤ 30 mJ at 16 Ω and ≤ 15 mJ at 32 Ω for a rail-stuck failure in High mode, including the after-trip energy; the worst-case estimates are 21.4 / 11.0 mJ. Balanced digital DC (the digital path driving the legs to ±x on a healthy stage) is larger and is not covered by the rail-stuck requirement: with the hardware detector alone it reaches 32.8 mJ at 16 Ω (current-limited, 2.32 V across the driver, 97 ms) and 21.7 mJ at 32 Ω (3.62 V, 52.8 ms), while the stream DC guard zeroes a large digital DC within ≈ 30 ms (≈ 10 mJ at 16 Ω, ≈ 12 mJ at 32 Ω). In Default mode a rail-stuck fault into 16–32 Ω exceeds the TPS2553 limit, so the PGOOD/overcurrent path latches first; the table is the High-mode worst case. Risk kept: a rail-stuck failure in High mode can put up to ≈ 21.4 mJ into a 16 Ω IEM; v2 option: a fast “leg within 0.5 V of a rail” detector (≈ 1 ms).

Slow DC-sense network and firmware DC tiers (redesigned in the amendment of 24 Sep 2026): per leg, R624–R627 15 kΩ from the jack side of R417–R420 to the ADC node, R628–R631 10 kΩ from the node to 3V3A (the bias exists only while AVCC is on: no suspend current, no back-feed into unpowered legs, and it has collapsed before AMP_EN drops at power-down), R632–R635 22 kΩ to GND and C608–C611 2.2 µF (≈ 1.3 µF effective at 1.6 V). The node is 1.556 V + 0.314 × leg and stays within 0.29–2.85 V at every rail, 3V3A ±2 % and resistor corner (≥ 0.29 V from 0 V and ≥ 0.38 V from the 3.234 V minimum ADC reference); the source impedance is 4.7 kΩ, inside the 50 kΩ CH32V307 limit, and τ is 6–10 ms (1 kHz attenuated ≈ 32 dB). ADC2 samples each channel at 1 kHz with ±20 % dithered timing in a fixed scan order (CC1, CC2, the four DC-sense channels and the 3V3A monitor on PC4), with 239.5-cycle sampling and the first conversion after each channel switch discarded; every reading is corrected ratiometrically with the 3V3A monitor, and each leg’s zero is re-read at every power-up while muted and accepted only within ±30 mV of the stored factory zero (otherwise a fault is reported). Error budget per leg, worst case after the factory zero and the power-up auto-zero: pin leakage 1 µA × 15 kΩ = 15 mV, ADC 2 LSB 5.1 mV, in-session drift (±15 °C, 0.3 % of the bias) 14.8 mV, 3V3A/3V3M ratio after the monitor correction 4.8 mV, sampling charge 0.1 mV and aliasing residual with dithered sampling 4 mV, so ≤ 44 mV per leg (23 mV RSS) and ≤ 88 mV on a pair difference (32 mV RSS), with no clipping inside the rail range. With AMP_EN low and AVCC on (option B or a fault) the bias adds 3.3 mV (32 Ω), 31 mV (300 Ω) and 61 mV (600 Ω) at the jack on top of the VREF path; programming mode (AVCC on, rails off) back-feeds ≤ 0.41 mA into the unpowered output stage (bench only). Four firmware tiers act on max(|each leg|, |L+ − L−|, |R+ − R−|) of the calibrated readings and set the latch through ANA_FAULT, D606 and R638 into node F, the same hardware kill path. They are armed whenever AVCC_EN is high, and AVCC_EN is set only while AMP_EN is high and the rails are good (default power order), because a disabled leg with VREF present shows VREF·RL/(4 kΩ + RL) at the jack (13.5 mV at 32 Ω, 119 mV at 300 Ω, 222 mV at 600 Ω, 1.70 V open-circuit):

| Tier | Window mean | Threshold | Removal within | No false trip for full-scale content above |
| --- | --- | --- | --- | --- |
| T1 | 0.2 s | 1.0 V | 0.25 s | 7.0 Hz |
| T2 | 0.3 s | 0.5 V | 0.35 s | 9.3 Hz |
| T3 | 1 s | 0.3 V | 1.1 s | 4.7 Hz |
| T4 | 2 s | 0.11 V (worst-case trip 0.022–0.198 V differential, 0.066–0.154 V per leg) | 3 s | 5.8 Hz |

Accepted envelope below the hardware trip, worst case including the 88 mV pair error: up to 1.15 V across a balanced driver for ≤ 0.25 s (20.7 mJ at 16 Ω, 10.4 mJ at 32 Ω); just below T1 (1.09 V) for ≤ 0.35 s, 25.9 / 12.9 mJ; just below T2 (0.59 V) for ≤ 1.1 s, 23.8 / 11.9 mJ; just below T3 (0.39 V) for ≤ 3 s, 28.2 / 14.1 mJ. Every sub-hardware-trip case therefore stays ≤ 30 mJ at 16 Ω and ≤ 15 mJ at 32 Ω. The residual below T4, ≤ 0.198 V across the driver (≤ 2.45 mW into 16 Ω, ≤ 1.23 mW into 32 Ω, continuous), was accepted by the owner on 24 Sep 2026 and now holds for balanced headphones too. A stream DC guard in the MCU before the DAC keeps DC in the program material from tripping the board: no zero crossing for 25 ms with |mean| ≥ 0.25 FS zeroes the output at once, and a 0.2-s mean ≥ −30 dBFS soft-mutes it, with unmute after 1 s below −40 dBFS; the mute is reported to the host log instead of latching. Because the blanking also hides faults present at power-up, three start-up rules are mandatory: a start-up fault if PGOOD is not good within 20 ms of RAIL_EN (RAIL_EN and ANA_EN low); from RAIL_EN until the set path is armed, the four DC-sense channels are sampled at 1 kHz and RAIL_EN is dropped directly if any leg exceeds 0.5 V (a stuck leg is bounded to ≈ 10 mJ); and every new board is first powered without headphones, with each leg checked by DMM. FLT, RAILS_OK_N and ANA_FAULT are always floating inputs without internal pulls (ANA_FAULT is only ever driven open-drain low), checked by a unit test on the GPIO configuration table. The recovery rule counts DC faults (3 within 10 s latch the board off until USB replug); events that match neither the DC nor the overload pattern are logged as “ambiguous” and counted toward the DC limit (conservative, deliberate); overloads count only once the volume ceiling has reached −12 dB, because Default mode at full volume into 60 Ω balanced needs three −3 dB steps and the v0.6 rule could lock the board off within ≈ 5 s.

Power-up sequence after USB configuration, order A (default; controlled power-down and fault shutdown in Section 3.2). VREF tracks AVCC, so the DAC pins stay inside GND…AVCC in every step. Enabling the legs while VREF = 0, and disabling them after VREF has collapsed, keeps the disabled-state path through the difference-amplifier network (VREF through ≈ 4 kΩ into the load: 13.5 mV at 32 Ω and 119 mV at 300 Ω) out of every enable and disable. With RESETB low the ES9018K2M drives its outputs to ground (≈ 2.1 mA per pin), so the AVCC-on step leaves ≈ 18–35 mV at the leg (1–2 % I/V gain mismatch) until RESETB is released and the ESS soft start ramps it out over ≈ 105 ms (register 14); this is measured on main board 1 together with the alternatives A′ (AUD_EN together with AVCC_EN) and B (DAC first, legs last; last row), and the pop target stays at risk until then:

| Step | Action | Note |
| --- | --- | --- |
| 1 | ANA_EN high (ILIM_HI per CC) → TPS2553 soft start → 5V_ANA | Headphone supply first; the blanking capacitor charges from RAILS_OK_N (PGOOD bad) through R617 |
| 2 | ≥ 600 ms after ANA_EN (or after the rails were last turned off): FLT_CLR, confirm FLT low | Clears the power-on latch set; the blanking capacitor is fully charged |
| 3 | RAIL_EN high (≥ 10 ms after ANA_EN) → +3.90 V / −3.72 V → PGOOD good | Start-up fault if PGOOD is not good within 20 ms; firmware DC watch at 1 kHz until the set path is armed (RAIL_EN dropped if a leg exceeds 0.5 V); the firmware ignores its ANA_FAULT classification for 20 ms (rail inrush) |
| 4 | PGOOD good + 20 ms: second FLT_CLR | Clears a spurious set from the rail inrush (TPS2553 FAULT); set path armed 51–186 ms after PGOOD good |
| 5 | ≥ 250 ms after PGOOD good: AMP_EN high while AVCC is still off (VREF = 0 V, I/V outputs at 0 V) | The legs start at ≈ 0 V |
| 6 | AVCC_EN high → VCCA, AVCC_L/R and VREF rise together (τ ≈ 2.55 µs); slow DC tier armed | Common mode: the active legs hold the jack at ≈ 0 V ± 3.4 mV common-mode leakage; RESETB is still low, so the ≈ 18–35 mV step of the DAC reset state remains until step 8 (measured); ESS: VCCA first, AVCC with it |
| 7 | +5 ms: AUD_EN high → 3V3D (DVCC, CPLD, oscillators powered, X201 disabled by MCLK_EN low) and 1V3 (DVDD); ≥ 2 ms after 1V3 is up: MCLK_EN high (SPI register) → MCLK | ESS: DVCC later, DVDD with or after DVCC (DVDD ≤ DVCC during the ramp); MCLK only after DVDD is up |
| 8 | RESETB high ≥ 1 ms after MCLK is stable → I²C init: register 1 = 0x80 (32-bit I²S, input auto-select off), register 10 = 0x00 (slave, asynchronous, stop_div = 0, required above 96 kHz), mute, master trim → ESS soft start ramps the outputs to ½ AVCC → soft unmute when streaming | The CPLD starts with both family enables low and the PLL in reset; the MCU enables X202 through SPI after ≥ 1 ms → lock. MCU outputs toward the CPLD stay low or high-impedance while 3V3D is off; I²C pull-ups on 3V3D. Rate change: MCLK never changes; a family change runs the break-before-make sequence (Section 3.1) with the PLL held in reset; AMP_EN stays high |
| A′, B | Firmware alternatives, chosen only if their pop measured on main board 1 is lower: A′ runs steps 1–5, then AVCC_EN and AUD_EN together (allowed by ESS: DVCC with or after VCCA), then steps 7–8; B runs steps 1–4, then AVCC_EN, AUD_EN, MCLK_EN, RESETB and soft start with AMP_EN low, then AMP_EN | B exposes the jack to the disabled-leg path (VREF through ≈ 4 kΩ, plus the DC-sense bias: 3.3 mV at 32 Ω, 31 mV at 300 Ω) for ≈ 0.2 s |

# 6. Cost budget

Budget for the 5-board sample run on JLCPCB Economic PCBA, at the fixed rate of 31.7 NTD per US$ that the cost gate uses. Pricing rule: the LCSC tier that matches the run quantity (1+ tier for one-per-board ICs, 10+ for two-per-board parts), with a US$0.10 minimum per purchasing line for parts under US$0.05 (attrition and minimum-quantity allowance); the OPA2210 is priced from its JLCPCB part page, because the PCBA order is charged at JLCPCB prices (its JLCPCB stock is unverified). Prices were refreshed to LCSC on 24 Sep 2026. Parts List v0.6 is the source of truth for cost.

| Item | Basis | US$, 5 boards | NTD per board |
| --- | --- | --- | --- |
| USB-C input and sensing | LCSC prices | 1.40 | 9 |
| USB bridge: CH32V307 + CPLD + crystal + three oscillators | LCSC prices | 37.24 | 236 |
| DAC (ES9018K2M) and its supplies | LCSC prices | 19.15 | 121 |
| I/V converters and output legs: 2 × OPA2210 + 2 × OPA1622 + 0.1 % resistors + 4 × BAT54S clamps | LCSC prices; OPA2210 at the JLCPCB price | 33.47 | 212 |
| Power: domains, current limit and +3.90 V / −3.72 V rails | LCSC prices | 11.52 | 73 |
| Protection and control | LCSC prices (2 × LM339DR, Extended) | 3.57 | 23 |
| Outputs, indicators, test, board (the 4.4 mm jack is 218 NTD of this) | LCSC prices | 36.24 | 230 |
| Minimum-spend allowance (US$0.10 per purchasing line under US$0.05) | Pricing rule | 0.69 | 4 |
| Subtotal, parts (4,542 NTD) | Parts List v0.6 (amended) | 143.27 | 908 |
| 5 bare 4-layer PCBs | Estimate | 10.00 | 63 |
| Economic PCBA setup | JLCPCB price list | 8.18 | 52 |
| Stencil | JLCPCB price list | 1.53 | 10 |
| Extended part types (21 × US$3.07) | Includes the through-hole 4.4 mm jack (conservative), X202 (class assumed), the LM339DR and the BAT54S (D405–D408) | 64.47 | 409 |
| SMT joints (781 per board × US$0.0016 × 5) | JLCPCB price list | 6.25 | 40 |
| X-ray (8 leadless parts per board × 5 × US$0.82) | CPLD, DAC, 2 × OPA1622, LM27762, three oscillators (oscillators conservative) | 32.80 | 208 |
| Through-hole joints (4.4 mm jack, 7 joints × 5 × US$0.0164 + US$3.58) | JLCPCB price list | 4.15 | 26 |
| Confirm Parts Placement | JLCPCB price list | 0.45 | 3 |
| Subtotal, JLCPCB fees | JLCPCB price list; any fixture or other assembly line in the real quote counts toward the gate | 117.83 | 747 |
| Total, 5-board run (8,594 NTD) | Parts + PCBs + fees | 271.11 | 1,719 |
| Cost-gate status: target 1,500 NTD per board; order gate 1,721 NTD per board (1,700 NTD ceiling + 21 NTD owner-approved for the DAC-pin clamps) | WARN above 1,500, FAIL above 1,721; 219 NTD over the target, only 2 NTD under the order gate | — | WARNING |

Lines are rounded to whole NTD, so a subtotal can differ by 1 NTD from the sum of its lines.

Outside the ceiling: shipping, import tax, the WCH-LinkE and the CPLD probe (J-Link or AGM DAP-Link), the jack samples, the enclosure and the Stage 2 boards for the firmware gate (Section 8). Inside the ceiling: every JLCPCB line of the real quote, including any fixture or other assembly line. The fixture sentence on the X202 and OPA2210 part pages is standard text on every JLCPCB part page, so no fixture fee is expected for Economic PCBA. At quote approval both the gated subtotal and the total payable (gated subtotal + shipping + tax from the quote) are recorded.

Counts: 154 BOM lines (143 fitted; DNF: C101, D102, U205, R215, R216/R217, R703, J703 and the no-part pad lines), 48 purchasing lines, 267 placed parts per board, 21 Extended + 27 Basic part types, 781 SMT joints + 7 through-hole joints per board, 8 leadless parts per board (X-ray).

Extended types (21): CH32V307RCT6, AGRV2KQ32, ES9018K2M, OPA1622IDRCR, OPA2210IDR, LM27762DSSR, TPS2553DBVR, LM339DR (two fitted, one type), LP5907MFX-3.3, ME6211C33M5G-N (two fitted, one type), ME6216A13M3G, USBLC6-2SC6, the three oscillators X201 (SCTF SX3M80.000B20F30TNN, C5179978), X202 (YXC OT322524.576MJBA4SL, C2831388) and X203 (SCTF SX2M22.5792M20F30TNN, C5452663), RT0603BRD072KL (2.00 kΩ 0.1 %), the USB-C receptacle, the 3.5 mm jack, the 4.4 mm jack, LESD5D5.0CT1G and the BAT54S clamps D405–D408 (JSMSEMI, C916425; JLCPCB available 628,982); the DNF mux U205 and the DNF TVS D102 are not counted. Every line’s class is taken from its JLCPCB part page or the basic-parts list, except X202, whose class is not shown (assumed Extended); everything else is Basic.

Cost gate: target 1,500 NTD per board (WARN above) → WARN, 219 NTD over; order gate 1,721 NTD per board (FAIL above) → WARNING, only 2 NTD under the order gate, so the margin is small and the real JLCPCB quote decides. Ceiling 1,700 NTD per board, set when the third oscillator was added, plus the owner-approved 21 NTD for the DAC-pin clamps (24 Sep 2026) → order gate 1,721 NTD per board; on 24 Sep 2026 the owner confirmed the budget is flexible as long as it does not surge and decided to fit the clamps D405–D408, knowing that they take the estimate over the 1,700 NTD ceiling; the gate covers every JLCPCB line of the real quote, and the actual JLCPCB quote from the real BOM/CPL files must pass the order gate before ordering. Sensitivities: the OPA1612AIDR fallback instead of the OPA2210 gives ≈ 1,725 NTD per board at the LCSC 10+ price (US$1.0942) and up to ≈ 1,740 NTD at the JLCPCB part-page price (US$1.33, tier not shown; page of 28 Aug 2026), above the order gate, so it is an owner decision, taken only if the OPA2210 cannot be bought; the OPA2210 bought at the LCSC price (US$2.67) adds ≈ 106 NTD per board (≈ 1,825 NTD), over the order gate, so the OPA1612 fallback is used instead in that case; the drop-in LM339BIDR instead of the LM339DR saves ≈ 3.6 NTD per board (≈ 1,715 NTD); the JLCPCB-vs-LCSC price differences are ≈ −24 NTD per board (indicative); and the lower-fee case (no Extended fee on the through-hole jack, no X-ray on the three oscillators) is ≈ −98 NTD per board (≈ 1,621 NTD). No cheaper 4.4 mm jack exists at LCSC (no SMD option).

Stock and library purchases before ordering (checked Jun–Sep 2026): CH32V307RCT6 (LCSC 193 / JLCPCB 364), ES9018K2M 724 and AGRV2KQ32 980 are reserved in the JLCPCB parts library; the 4.4 mm jack J701 (C2762984) has only 21 at JLCPCB (LCSC 721), so 7 (5 + 2) are bought into the library; the OPA2210IDR JLCPCB stock is unverified (the 76 figure is 12 months old): check it in the JLCPCB Parts Manager now and buy 12 (10 + 2 spares) if ≥ 12, otherwise switch U403/U404 to the OPA1612AIDR (C94590); ME6216A13M3G ≈ 3,100 and X201 2,503 in stock at LCSC.

Why the estimate changed: 1,540 NTD per board with one oscillator → 1,602 NTD with the 24.576 MHz oscillator (+62 NTD: the part with its decoupling capacitor 16.2, its Extended-part fee 19.5, its X-ray 26.0 and eight more SMT joints 0.4) → 1,666 NTD with the 22.5792 MHz oscillator (+64 NTD: the part with its capacitor and three resistors 17.5, its Extended-part fee 19.5, its X-ray 26.0 and twelve more SMT joints 0.6) → 1,647 NTD per board in v0.6 (80 MHz MCLK, OPA2210 at the JLCPCB price, protection parts) → 1,650 NTD per board in v0.7 (+3 NTD: protection and power fixes) → 1,691 NTD per board in v0.8 (+41 NTD: prices refreshed to LCSC on 24 Sep 2026, +≈ 103 NTD per run, mainly the 10 µF 0805 at US$0.085; the LM393B’s Extended-part fee, +19.5 NTD per board; 32 more SMT joints and 16 more placed parts) → 1,697 NTD per board in the amendment of 24 Sep 2026 (+6 NTD: the two LM339DR and their four decoupling capacitors in place of the LM393BIDR and the BAV99 diodes, the new DC-sense network values and the 3V3A monitor; 14 more SMT joints) → 1,719 NTD per board with the DAC-pin clamps D405–D408 fitted (owner decision of 24 Sep 2026; +21 NTD: one more Extended type 19.5, the four BAT54S 1.3 and twelve more SMT joints 0.6).

Reference only, not the budget basis: a 100-unit build on Standard PCBA comes to ≈ 809 NTD per board. The budget and the cost gate use the 5-board sample run above.

# 7. Design for JLCPCB (DFF/DFA/DFT rules) and tests

These rules apply to every revision and use JLCPCB’s PCBA price list; the 5-board sample run is built on Economic PCBA. The rules that can be checked automatically also run in CI (Section 8).

| ID | Type | Rule |
| --- | --- | --- |
| R01 | DFA | The sample run uses Economic PCBA: setup US$8.18, stencil US$1.53, 2–50 boards per order, one side only (all SMT parts on the top side). |
| R02 | DFA | Every fitted part has an LCSC number and a JLCPCB class taken from its JLCPCB part page or the basic-parts list, except X202, whose class is not shown (assumed Extended) (Parts List v0.6). Prefer Basic parts (no per-type fee): where the exact value is Extended, Basic E24 values or Basic combinations are used (100 k + 47 k, 1 k ∥ 5.1 k, 15 k + 1 k, 100 k + 100 k + 5.1 k, 2 × 330 pF, 2 × 2.2 Ω). Each Extended type costs US$3.07; this board has 21 Extended + 27 Basic types (the through-hole jack counted as Extended, conservatively, and X202 assumed Extended). |
| R03 | DFA | X-ray costs US$0.82 per leadless part, so prefer leaded packages where an equivalent exists. Leadless here (8 per board): AGRV2KQ32, ES9018K2M, 2 × OPA1622, LM27762 and the three oscillators (counted conservatively). The CH32V307RCT6 (LQFP-64), 2 × OPA2210 (SOIC-8), 2 × LM339DR (SOIC-14), TPS2553 (SOT-23-6) and the DNF mux U205 (SOT-363) are leaded. |
| R04 | DFA | Joint fees: SMT US$0.0016 per joint (781 per board); through-hole US$0.0164 per joint + US$3.58 per order. Only the 4.4 mm jack is through-hole (7 joints); confirm that the USB-C receptacle’s shell pegs do not add a through-hole line. |
| R05 | DFA | Turn on “Confirm Parts Placement” (US$0.45) and check the 3D preview for rotation and polarity errors. The fixture sentence on the X202 and OPA2210 JLCPCB pages is standard text on every part page; any fixture or other assembly line in the real quote counts toward the 1,721 NTD order gate. |
| R06 | DFA | Reserve low-stock parts in the JLCPCB parts library before ordering: CH32V307RCT6, ES9018K2M and AGRV2KQ32; buy 7 × 4.4 mm jacks (J701, JLCPCB stock 21) into the library, and 12 × OPA2210IDR (10 + 2 spares) if the JLCPCB Parts Manager shows ≥ 12, otherwise switch U403/U404 to the OPA1612AIDR. |
| R07 | DFT | Test pad on every rail and switched domain and on the clock, I²S, enable, protection and CC nodes; 32 pads, TP701–TP732: TP701 5V_SYS, TP702 5V_ANA, TP703 3V3M, TP704 3V3D, TP705 3V3A, TP706 1V3, TP707 +3.90 V, TP708 −3.72 V, TP709 VREF, TP710 AVCC_L, TP711 MCLK, TP712 family-clock node, TP713 OSC48_EN, TP714 OSC44_EN, TP715 BCLK, TP716 LRCLK, TP717 SDATA, TP718 AMP_EN, TP719 RAIL_EN, TP720 AVCC_EN, TP721 RAILS_OK_N, TP722 FLT, TP723 ANA_FAULT, TP724 node F, TP725 VBUS_SENSE, TP726 CC1, TP727 CC2, TP728–TP731 GND, TP732 PROG (MCU PC13; shorted to GND at reset for the programming mode). |
| R08 | DFT | Debug pads for the MCU (J201 for the WCH-LinkE: 3V3 sense only, SWDIO, SWCLK, NRST, GND, plus a BOOT0-to-3V3 pad and a USART1 TX pad, PA9, with an optional RX pad, PA10) and the CPLD (J202, 2-wire Compact-JTAG for a J-Link or AGM DAP-Link: 3V3D sense only, JTCK, JTMS, NRST, GND; the probe never powers 3V3D); DNF 1 × 6 I²S test header J703 (MCLK, BCLK, LRCLK, SDATA, GND, 3V3 ref) with a firmware test mode that tristates the CPLD I²S outputs. The J703 MCLK pin is a monitor output only, through R703 1 kΩ (DNF, fitted with J703): in test mode an external 3.3 V source (QA403, via series resistors) drives only BCLK, LRCLK and SDATA, and the DAC stays in asynchronous mode on X201. |
| R09 | DFT | Fiducials and polarity marks. |
| R10 | DFF | JLCPCB’s standard 4-layer impedance-controlled stack-up for the 90 Ω USB pair; solid ground plane; digital, charge-pump (2 MHz) and analog areas separated; the unbalanced jack sleeve gets a dedicated return with ≥ 4 vias (≤ 5.7 mΩ shared ground for −75 dB crosstalk at 32 Ω); both family oscillators and their shared node sit next to the CPLD PLL clock pin (PIN_1) with short traces, the X201 trace to the DAC XI is ≤ 20 mm, and the idle oscillator’s trace is kept short (a disabled output-enable oscillator may keep running internally); copper pours on the LDO VIN, GND and VOUT pins (dissipation table, Section 5.3); USB-C A6/B6 and A7/B7 tied at J101; jack and USB-C pin numbers confirmed by continuity tests on bought samples before layout. |

Tests added or changed in v0.5, its amendment, v0.6, v0.7 and v0.8 (automated where marked):

| Test | When | Pass criterion |
| --- | --- | --- |
| End-of-line calibration (automated, QA403 + scripts) | Every board | Full-scale level per leg → master trim to 1.55 Vrms ± 0.1 dB and per-channel fine trim, written to MCU flash |
| Offset screening (automated, DC-capable DMM) | Every board | DC offset of every leg with a DC-capable DMM (≥ 4½ digits, SCPI over USB), because the QA403 inputs are AC-coupled: ≤ ±10 mV unbalanced, ≤ ±20 mV balanced; the QA403 stays for level, THD+N and dynamic range |
| THD+N and dynamic range (automated) | Every board | Section 2 targets at the calibrated level; dynamic-range results recorded with the analyzer loopback floor |
| Protection: removal and energy | Each board revision | (a) Detector test: an external lab source on a jack contact (its link R417–R420 removed) at 1.0 V, 2.0 V and 3.8 V; tests detection and the control signals only. (b) Failed-output emulation: the board’s own +3.90 V (TP707) or −3.72 V (TP708) switched onto a jack contact (link removed) by an external MOSFET switch, bypassing the amplifier enable, into 16 Ω and 32 Ω dummy loads; the terminal voltage is scoped and V²/R integrated until it stays below 0.1 V. Pass: FLT high, RAIL_EN, AMP_EN and AVCC_EN low within the worst-case envelope (no allowance beyond the hard 100 ms / 250 ms / energy limits), total energy ≤ 30 mJ (16 Ω) / ≤ 15 mJ (32 Ω); on main board 1 the four DAC pins (at the R423–R426 DAC-side pads) are probed against AVCC_L/AVCC_R during this emulation (pass: with the clamps D405–D408 fitted every pin stays within −0.4 V…AVCC + 0.4 V; the 3V3A decay is recorded with the clamps fitted). (c) Digital DC test signal (healthy output stage), including opposite-sign DC on a balanced pair. (d) SPICE of a failed output stage |
| Overload and plug short (30 min each, High mode, maximum volume) | Each board revision | 4.4 mm 16 Ω, 4.4-to-3.5 mm adapter, 3.5 mm 16 Ω, 60 Ω balanced full scale: the latch sets, recovery needs the firmware clear, all parts stay within ratings and the board recovers; near the Default-mode limit, rails and output scoped for clipping before a trip; 4.4 mm plug shorting leg+ to leg−: survival test |
| Latch timing and firmware DC tier | Each board revision | Short PGOOD pulses emulating the real open-drain release (OUT+ pulled below 95 % with a switched load, or the node released through an open-drain FET; never forced with a low-impedance generator) for 5 µs, 20 µs, 100 µs and 1 ms while the MCU keeps requesting AMP_EN → either nothing happens or the latch sets, never an un-latched AMP_EN dropout. RAILS_OK_N rise (≈ 1.0–1.3 µs to 2.5 V) and latch capture; blanking and arming times (re-blank 19–94 ms, arming 51–186 ms). Start-up rules: PGOOD timeout and start-up DC watch (> 0.5 V drops RAIL_EN). Firmware DC tiers: single-leg and opposite-sign balanced DC from the digital path at each tier threshold → latch within 0.25 / 0.35 / 1.1 / 3 s; DC-sense auto-zero, dithered sampling and ratiometric correction with the 3V3A monitor checked; stream DC guard with a DC test file → mute, no latch |
| USB | Stage 2 board before layout (gate) and each board revision | Current in every state (USB power meter): attached-not-configured ≤ 100 mA, configured digital ≤ 262 mA; suspend current vs time from the last bus activity, ≤ 2.5 mA by 10 ms, including the DAC-first power-down with the adaptive AMP_EN and the real LP5907, both LEDs off, on Stage 2 and on the main board; attach inrush charge ≤ 50 µC; VBUS droop at each domain enable; Default and High mode on a Type-C 1.5 A source and on a USB-A port; CC change handled within tSinkAdj; watchdog-to-GPIO latency < 5 µs (Stage 2); VBUS hot-plug ring with a USB-A-to-C cable into a live port (scope C102; D102 fitted only if the peak exceeds 5.8 V) |
| Pop | Each board revision | Pop (peak of the transient beyond the settled DC step, 20 Hz–20 kHz, AC-coupled): ≤ 5 mV peak for controlled transitions (power-on after configuration, suspend/resume, rate and rate-family change, fault recovery); ≤ 20 mV peak at unplug or brownout (target); 32 Ω and 300 Ω; AVCC-on / RESETB pop with orders A, A′ and B on main board 1, I/V outputs and legs scoped during the AVCC ramp with RESETB low (at risk until measured); power-down pop with the adaptive AMP_EN |
| Jitter | Each board revision | J-test at the 48 kHz and 44.1 kHz families (both exact integer ratios, from X202 and X203 through the CPLD PLL) and at 384 kHz; spectra recorded (no numeric limit yet) |
| Clock checks and programming (Stage 2 boards, firmware gate) | Stage 2, before layout | Family change: scope both enables and the shared node: never both enabled, node quiet for ≥ 1 ms before the new oscillator starts, disable time measured, no runt pulses, PLL relock; PLL lock at both family clocks; all eight rates bit-perfect to the DAC (lock status, FFT of the check output); on the shared-node board and on the mux board; the CPLD meets timing at 49.152 MHz (PLL output) and on the 36 MHz SPI clock; MCLK duty at the DAC XI; programming mode (PROG pad) and CPLD programming through J202 |
| Rails, power and unplug | Bring-up | Negative-rail headroom at VBUS 4.60 V and 4.40 V and full load (predicted ≥ +0.05 V worst case at 70 mA per leg, host 4.75 V); supply order and DVDD ≤ DVCC on the scope; AUD_EN cycled 100 times at VBUS 4.40 V and 5.25 V (3V3D and 1V3 reach regulation) and the 3V3D/1V3 decay with the bleeders; the four DAC pins (R423–R426 DAC-side pads) against AVCC_L/AVCC_R at power-up, power-down, latch trip, failed-output emulation and hot unplug at full load (within −0.4 V…AVCC + 0.4 V in the failed-output emulation and at hot unplug, with D405–D408 fitted); capacitive-load test with 1 nF and long cables; LM27762 VIN ripple at light load; OPA1622 clean swing at ±3.8 V; regulator temperatures (IR or thermocouple); power tables in both modes (High and Default) |

# 8. Automation and verification plan

Every check runs automatically with an explicit pass/fail rule.

| Check | Runs when | Pass/fail rule |
| --- | --- | --- |
| ERC and DRC (KiCad command line) | Every commit (GitLab CI) | Zero errors |
| Manufacturing outputs: Gerbers, drill, BOM, pick-and-place, schematic PDF | Every commit | Files generated |
| BOM check | Every commit | Every fitted part has an LCSC number and a JLCPCB class from its part page or the basic-parts list (X202 assumed Extended; Parts List v0.6) and stock ≥ the 5-board quantity; Extended types and leadless parts counted and reported |
| Cost gate | Every commit | 5-board run at a fixed 31.7 NTD/US$, covering every JLCPCB line (PCBs, PCBA fees including any fixture or other assembly line, parts): WARN if > 1,500 NTD per board (target), FAIL if > 1,721 NTD per board (order gate: the 1,700 NTD ceiling plus the owner-approved 21 NTD for the DAC-pin clamps); currently 1,719 NTD: WARN, 2 NTD under the order gate (planning estimate; the JLCPCB quote from the real BOM/CPL files must pass the order gate before ordering, and the gated subtotal and total payable are recorded at quote approval). A 100-unit reference line (≈ 809 NTD) is reported but not gated |
| Python DFF/DFA/DFT and layout rules | Every commit | All rules pass: analog/charge-pump separation, 90 Ω USB pair, decoupling capacitor near every IC power pin, test point on every rail and domain, fiducials present, jack-ground rule, short family-oscillator traces at the PLL pin |
| Document lint | Every documentation merge request | Generated DOCX/XLSX files contain no unresolved template tokens (for example ZQ-…-QZ), no empty cells in required tables and no internal file paths |
| AI datasheet-review agent | Every merge request | Reads the DAC, amplifier and regulator datasheets, extracts requirements, checks the schematic and comments on the MR; seeded by the Schematic Design Notes checklist and scored on 10–20 planted errors (catch rate and false alarms) |
| Firmware unit tests (PC build) | Every commit (GitLab CI) | The pure-logic firmware parts (descriptor tables, feedback math, ring buffer, volume mapping) compile on a PC and every unit test passes |
| Firmware gate | Before layout: the Stage 2 board, the first hardware (Section 8) | Enumerates bus-powered with the new descriptors; every rate plays on Windows 10/11, macOS and Linux with working feedback; BCLK/LRCLK average rates correct for every rate (frequency counter) and the DPLL locks; host volume and mute work; all eight rates bit-perfect to the DAC; the break-before-make family change passes on the shared-node and the mux board (never both enabled, ≥ 1 ms quiet node, no runt pulses, PLL relock) and the PLL locks at both family clocks; the CPLD meets timing at 49.152 MHz and the 36 MHz SPI link works with the real CPLD capture; the Stage 2 USB currents pass (unconfigured ≤ 100 mA, configured digital ≤ 262 mA, suspend ≤ 2.5 mA within 10 ms of the last bus activity, measured as current vs time) and the watchdog-to-GPIO latency is < 5 µs (layout gate) |
| Automated measurement, calibration and screening (Python + QuantAsylum QA403 + DC-capable DMM) | Every board | Calibration to 1.55 Vrms per leg, offset screening with the DMM, THD+N, dynamic range (with the analyzer loopback floor), frequency response, crosstalk, output impedance and maximum power at 16–600 Ω on both jacks in both modes; auto-generated report compared against Section 2 |
| Automated THD-compensation tuning loop | Each board revision | Python sweeps the ES9018K2M THD-compensation registers 22–25 (2nd/3rd harmonic) while reading THD+N on the QA403; reports the best coefficients for the optional firmware setting |

Firmware (owner decision, 24 Sep 2026): our own USB Audio Class 2.0 implementation on the CH32V307 high-speed USB port, built on WCH’s register-level USBHS device examples; the chip’s EVT package has high-speed device examples for HID, CDC serial, mass storage and network adapters, but none for audio. TinyUSB (MIT) is an alternative only if its CH32V307 high-speed audio path proves reliable: a 2023 report showed UAC2 control requests failing on this chip, and its driver contributor had not verified isochronous transfers. The USBAudioToI2S firmware (Apache-2.0) stays a known-good comparison on the same hardware and a reference; license notices are kept if any code is copied. The CPLD logic is ours too.

The firmware gate runs before layout on the Stage 2 board, the first hardware. Because development and test boards are hard to get in Taiwan (owner constraint, 24 Sep 2026), Stage 1 (a desk setup of a WCH CH32V307V-EVT-R1 board, an AGM 2K-LUT CPLD board and an ES9018K2M I²S-input DAC board) is optional: it is used only if the owner can get the boards, nothing gates on it, and everything it was to prove moves to the Stage 2 gates. Stage 2 is mandatory: two JLCPCB-built boards with the frozen pin map, carrying USB-C and ESD, the 3V3M/3V3D LDOs with the 3V3D dropper and the bleeders, the CH32V307 with its crystal, the AGRV2KQ32 (with FB202), X201, X202 and X203 with R210–R218, the TPS2553 and LM27762 with the real latch and protection section on dummy loads, and now also the ES9018K2M with its real supplies (LP5907 3V3A, ME6216 1V3, DVCC ferrite) instead of the 3V3A and 1V3 dummy loads. A passive check output (each DAC pin to a test pad with 10 kΩ to GND) allows a voltage-mode check of the lock and of a sine by FFT; the boards also carry the I²S/MCLK test header, J201/J202 and current-measurement points on 3V3M and 3V3D. One board is built with the shared oscillator node and one with the mux U205 fitted: ≈ US$93 ≈ 2,940 NTD for the two assembled boards (was ≈ US$90 ≈ 2,860 NTD without the DAC), outside the board budget.

USB compliance is part of the pre-layout gate. With the real CPLD design loaded, streaming 384 kHz at VBUS 4.60 V and 5.25 V and across rate and family changes, the Stage 2 board must show: USB enumeration and UAC2 streaming at all 8 rates with feedback; a bit-perfect path to the DAC (lock status, FFT of the check output); the 36 MHz SPI link with the real CPLD capture; PLL lock at both family clocks; the break-before-make clock handover; an attached-not-configured current ≤ 100 mA and a configured digital current ≤ 262 mA; a suspend current ≤ 2.5 mA within 10 ms of the last bus activity, including the DAC-first power-down with the real LP5907 (current vs time; Stop with LPDS, USBHS PHY suspended, wake on resume and HSE/PHY PLL restart within the 10 ms resume recovery, with EXTI on PB6/PB7 as the fallback wake); a watchdog-to-GPIO latency < 5 µs; AUD_EN cycled 100 times at 4.40 V and 5.25 V; the 3V3D/1V3 decay with the bleeders; the RAILS_OK_N rise and latch capture; the blanking and arming times; the programming mode and CPLD programming through J202; and the DC-sense auto-zero, dithered sampling and ratiometric correction with the 3V3A monitor. Layout does not start unless all pass; the main boards repeat the full-board measurement at bring-up. The MCU–CPLD interface (SPI with chip-select framing) is ours to define; its specification and both pin maps are frozen before the Stage 2 board and the schematic. Modules, all part of the gate:

| ID | Module |
| --- | --- |
| (1) | USB core on the high-speed port: enumeration, descriptors, standard requests, suspend/resume. Bus-powered descriptors (bmAttributes 0x80, bMaxPower 0xFA = 500 mA); test VID/PID 1209:0001 during development (pid.codes reserves it for private testing only) and a real PID before publishing; serial number from the CH32V307 96-bit unique ID; HS feedback endpoint wMaxPacketSize 4, isochronous OUT wMaxPacketSize ≥ 392, bcdDevice bumped on every descriptor change |
| (2) | UAC2 controls: clock source with the advertised rates (CUR/RANGE) and clock valid; Feature Unit mute and volume (CUR/RANGE) mapped to the ES9018K2M attenuator over I²C, plus MCU fine scaling for channel trim; alternate settings for 16, 24 and 32 bit |
| (3) | Streaming: isochronous OUT → ring buffer → DMA to the CPLD over SPI at 36 MHz with chip-select framing (MCU master; one CS frame per block of samples, each starting with a 16-bit header; 8-bit frames or a halfword swap in the CPLD set the word order, tested with a known pattern) |
| (4) | Feedback: an MCU timer counts the CPLD’s LRCLK against USB (micro)frames, plus a small FIFO-level correction, sent in 16.16 format. At 384 kHz, 32-bit stereo: 384 bytes per 125 µs microframe, up to 392 with one extra sample; feedback 48.0 samples per microframe = 0x300000. The same timer checks the LRCLK rate after every family change (within ±0.1 % of the target); the 44.1 kHz-family feedback value is non-integer (44.1 kHz = 0x002C1999 in 16.16) |
| (5) | Application: ES9018K2M driver (register 1 = 0x80, register 10 = 0x00 with stop_div = 0 above 96 kHz, both in the unit tests; asynchronous mode, DPLL bandwidth, soft start, mute, lock status, master trim); break-before-make family change through the CPLD SPI registers (soft mute, I²S stopped, PLL reset, old enable low, ≥ 1 ms both off, new enable, ≥ 10 ms start-up, PLL lock, LRCLK check, DAC relock, unmute); power-state machine (order A: AMP_EN before AVCC_EN at power-up, options A′ and B selectable; FLT_CLR ≥ 600 ms after ANA_EN and again at PGOOD good + 20 ms; AMP_EN ≥ 250 ms after PGOOD good; controlled power-down: mute, MCLK_EN and AVCC_EN low at + 0.1 ms, AMP_EN low when the 3V3A monitor reads < 0.1 V (deadline + 5.8 ms), then RESETB, AUD_EN, RAIL_EN and ANA_EN, low-power mode with both LEDs off; AUD_EN off ≥ 100 ms before re-enabling; programming mode on the PROG pad), CC monitoring every ≤ 5 ms with the tRpValueChange debounce, the 0.66 V / 1.23 V thresholds and the tSinkAdj response, ILIM_HI control, VBUS watchdog at 4.10 V on ADC1; DC-sense readings with dithered 1 kHz sampling, ratiometric correction by the 3V3A monitor and a power-up auto-zero; four firmware DC tiers on every leg and each balanced pair (≥ 1.0 V within 0.25 s, ≥ 0.5 V within 0.35 s, ≥ 0.3 V within 1.1 s, ≥ 0.2 V within 3 s, latch set through ANA_FAULT, armed whenever AVCC_EN is high); stream DC guard before the DAC (zero at once after 25 ms without a zero crossing at ¦mean¦ ≥ 0.25 FS; soft mute at a 0.2-s mean ≥ −30 dBFS, unmute after 1 s below −40 dBFS); start-up rules (PGOOD timeout 20 ms, start-up DC watch at 1 kHz); fault handling (FLT ISR drives the enables low first, then captures the cause from the 0.5-s averages and the event history, with an “ambiguous” class counted as DC; IWDG; FLT during a controlled power-down or before the first FLT_CLR masked; volume ceiling; 3-strike stop for DC faults and for overloads at the −12 dB ceiling); GPIO table with FLT, RAILS_OK_N and ANA_FAULT as floating inputs (unit-tested); calibration storage and test modes (DC fault injection by a digital DC signal; I²S-header tristate) |
| (6) | CPLD logic: dual-clock FIFO with Gray-coded pointers (1024 × 32 bit in 4 × M9K, written in the SPI clock domain, read in the PLL bit-clock domain; no free-running system clock); I²S output engine clocked from the PLL (fed from PIN_1; ×2 on the selected family oscillator: 49.152 MHz or 45.1584 MHz) with exact integer dividers, SDATA and LRCLK changing on the falling BCLK edge; SPI link receiver in the SPI clock domain (SCK on the global network through ALTCLKCTRL; chip-select framed, 16-bit frame header); the oscillator enables (OSC48_EN, OSC44_EN), mutually exclusive and never changed in the same cycle, MCLK_EN for X201 and the PLL reset for the family change, all set by the MCU through registers in the SPI domain. No clock multiplexer in the fabric; the MCLK pin is unused (MCLK goes straight from X201 to the DAC). Built with AGM Supra (Windows-centred: CPLD builds on a Windows CI runner, or Supra on Linux if it proves usable) |
| (7) | MCU–CPLD interface, ours to define: samples by DMA over SPI with chip-select framing (MCU master at 36 MHz; SCK to the CPLD global network through ALTCLKCTRL; 24.576 Mbit/s at 384 kHz / 32-bit stereo; second lane on SPI2 reserved; I²S is not used, because its frame rate is not locked to the audio rate), FIFO level returned on MISO during each frame, LRCLK back to a timer-capable MCU pin (PD2, TIM3_ETR); written as a specification and frozen with both pin maps before the Stage 2 board and the schematic |

# 9. Risks and open decisions

## 9.1 Risks

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Own UAC2 firmware and CPLD logic: top risk (host compatibility of high-speed isochronous streaming with feedback) | No working USB audio; schedule | Stage 2 board, the first hardware, before layout (Section 8; the Stage 1 desk setup is optional); USBAudioToI2S as a known-good comparison on the same hardware; PC-side unit tests in CI |
| MCU–CPLD interface specification (SPI with chip-select framing) not yet written | Rework of the pin maps or the CPLD logic | Write the specification and freeze both pin maps (proposed in Schematic Design Notes v0.7) before the Stage 2 board and the schematic |
| One CPLD PLL must lock at both family clocks (22.5792 and 24.576 MHz input; AGM lock range not published) and relock after each family change | Wrong rates, runt pulses or DPLL unlock | Stage 2 gate with a scope check at family changes; fallback: BCLK ÷ 1 directly from X202/X203 (duty 45–55 %, the DAC limit, zero margin) |
| CPLD programming: only 2-wire Compact-JTAG (JTMS, JTCK) through AGM Supra with a J-Link or AGM DAP-Link; a generic CMSIS-DAP probe (WCH-LinkE in DAP mode) is not confirmed | The CPLD cannot be programmed | J202 = 3V3D sense, JTCK, JTMS, NRST, GND; the owner buys a supported probe (confirm the models Supra accepts); programming mode through the PROG pad; Stage 2 gate |
| Oscillator disable time and disabled-output state are not specified in the SCTF/YXC datasheets | Contention or runt pulses on the shared PLL input node | Break-before-make with ≥ 1 ms both off; measure on Stage 2 (scope on the shared node); fallback: mux fitted (U205), already built on one Stage 2 board; idle oscillator trace kept short. Gate: Stage 2 |
| AGRV2K I/O state and weak pull-up strength during configuration | A family oscillator or X201 enabled while the CPLD configures | The 2.2 kΩ pull-downs (R212, R213, R218) hold against ≥ 5.1 kΩ; datasheet check and Stage 2 scope check |
| SPI clock quantised: 36 MHz (72 MHz ÷ 2) is the only workable setting, 68 % utilised at 384 kHz | Link too slow or marginal for 384 kHz / 32-bit stereo | Stage 2 gate with the real CPLD capture; register reads at ≤ 18 MHz; second lane reserved (SPI2, PB12–PB15, two spare CPLD pins) |
| MCU and CPLD currents are assumptions (oscillators at datasheet maxima; the CPLD is AG32 silicon with unpublished dynamic current) | Unconfigured state above 100 mA; Default-mode budget | Measure on the Stage 2 board before layout (gate); lower the MCU clock before configuration if needed |
| USB Default-mode estimated maximum 0.475 A (25 mA margin) | Default-mode budget | Stage 2 measurement (gate), then the main boards |
| USB suspend: the USBHS suspend path is unproven on the CH32V307 (the WCH example has only a placeholder; Stop turns off the HSE and PLL); MCU Stop current ≤ 1.9 mA assumed in the ≈ 2.30 mA budget (0.20 mA margin, LEDs off); suspend level typically ≈ 8.2 ms, at worst 9.3 ms after the last bus activity | USB 2.0 suspend compliance (v1 requirement) | Stage 2 gate: Stop with LPDS, PHY suspended, wake on resume, HSE/PHY PLL restart within the 10 ms resume recovery, current vs time with the real LP5907 and DAC; fallback wake: EXTI on PB6/PB7; again on the main boards |
| Pop at AVCC-on: in reset the ES9018K2M drives its outputs to ground, so the AVCC-on step with RESETB low gives ≈ 18–35 mV at the leg (1–2 % I/V gain mismatch) | Controlled pop target (≤ 5 mV) at risk | Power-down keeps RESETB high and stops MCLK before AVCC goes off; pop measured on main board 1 with orders A, A′ and B, and the lowest one is used; an injection resistor (rejected) and 0.1 % Rf (cost) are not adopted |
| DAC pin-to-pin DC mismatch (assumed ≤ 1 mV) and null point (1.702 V from the current-output formula or 1.65 V from the voltage-source model; ±20 mV uncertainty assumed) | Output DC offset (≤ 9.3 / 18.6 mV worst case if the null is 1.65 V) | VREF null test at bring-up; R433 0 Ω option (VREF 1.650 V); 100 % offset screening with a DC-capable DMM |
| OPA1622 clean swing (≈ 1.3 V from the rail, from TI data at ±5 V) extrapolated to +3.90 V / −3.72 V; small full-scale margins (3.5 mm 60 Ω and 4.4 mm 120 Ω 0.6 dB, 300 Ω balanced +0.04 dB) | Output power table | Bench-verify the clean swing at ±3.8 V and the power table in both modes |
| LM27762 charge-pump resistance at 4.5 V input (3.0–3.5 Ω assumed): negative-rail headroom +0.05 V worst case at 70 mA per leg (4.4 mm 32 Ω, High mode, host 4.75 V, 3.5 Ω) | Short clipping on current peaks from a weak host | 70 mA envelope at 4.4 mm 32 Ω; measure the negative-rail headroom at VBUS 4.60 V and 4.40 V on the bench |
| DAC pins after an output-stage failure: with a stuck leg the I/V converters lose control 0.11–0.39 ms after the kill while 3V3A needs 3.2–5.6 ms to fall; the fitted clamps D405–D408 hold each pin within ≈ −0.40 V…AVCC + 0.40 V (≤ 4.4 mA per pin) for up to ≈ 5.5 ms (unclamped: toward −1.76 V, the DAC’s own protection conducting at ≈ −0.6 V with ≤ 3.9 mA); ESS gives no allowance beyond GND < Vout < AVCC | Small residual risk to the DAC on a board whose output stage has already failed: ≤ 0.4 V beyond the rails for ≤ 5.5 ms; brief 3V3A back-feed through the upper diodes (≤ 4.4 mA per pin, ≈ 1 ms) | Designed: BAT54S clamps D405–D408 fitted (owner decision, 24 Sep 2026) within 5 mm of the DAC pins; the four DAC pins probed on main board 1 during failed-output emulation and hot unplug (pass: within −0.4 V…AVCC + 0.4 V); 3V3A decay recorded with the clamps fitted |
| Balanced digital DC on a healthy stage (legs driven ±x): larger than the rail-stuck case and not covered by its requirement (hardware alone 32.8 mJ at 16 Ω, 21.7 mJ at 32 Ω) | Energy into a balanced headphone from DC in the digital path | Stream DC guard zeroes a large digital DC within ≈ 30 ms (≈ 10 mJ at 16 Ω, ≈ 12 mJ at 32 Ω) and soft-mutes smaller DC; firmware tiers act on each balanced pair; stream-guard test with a DC test file and a balanced opposite-sign DC test |
| Jack pin numbers not verified (drawings image-only) | Wrong footprint wiring on the first boards | Buy 2 samples of each jack (LCSC, ships to Taiwan) and continuity-test every pin before layout; checklist rule |
| Low-stock parts: CH32V307 (LCSC 193 / JLCPCB 364), ES9018K2M 724, AGRV2KQ32 980, 4.4 mm jack J701 21 at JLCPCB (LCSC 721), ME6216A13M3G ≈ 3,100, X201 2,503 | Schedule and cost | Reserve parts in the JLCPCB library before ordering; buy 7 × J701 into the library |
| OPA2210 JLCPCB stock unverified (the 76 figure is 12 months old); JLCPCB US$1.00 vs LCSC US$2.67 at 10+ | ≈ +106 NTD per board at the LCSC price (≈ 1,825 NTD), over the order gate; with the OPA1612 fallback the Default-mode targets are not met | Check the JLCPCB Parts Manager now and buy 12 (10 + 2 spares) if ≥ 12; otherwise switch U403/U404 to the OPA1612AIDR fallback designed in with V+ = +3.90 V (VREF ≥ 35 mV inside its range; offset 9.7 / 19.4 mV, at the edge; with the fallback the Default-mode targets are not met: 3.5 mm 32 Ω ≈ 27 mW, 4.4 mm 60 Ω ≈ 13 mW, 120 Ω ≈ 26 mW; High mode unchanged; ≈ 1,725–1,740 NTD per board, above the 1,721 NTD order gate: an owner decision) |
| Cost only 2 NTD under the 1,721 NTD order gate (conservative planning estimate) | Budget: a small price or class change fails the gate | The gate (the 1,700 NTD ceiling plus the owner-approved 21 NTD for the DAC-pin clamps) covers every JLCPCB line of the real quote, which must pass it before ordering; gated subtotal and total payable recorded; ≈ 1,621 NTD if the conservative fees do not apply; OPA1612 fallback ≈ 1,725–1,740 NTD, above the gate; LM339BIDR ≈ 1,715 NTD; JLCPCB-vs-LCSC price differences ≈ −24 NTD (indicative) |
| Residual DC region below 0.2 V per leg (below both detection tiers) | Up to 2.5 mW into 16 Ω and 1.25 mW into 32 Ω without a trip | Accepted by the owner on 24 Sep 2026 |
| A rail-stuck hardware failure in High mode delivers up to 21.4 mJ into 16 Ω (worst case, including the after-trip energy) | Energy into a 16 Ω IEM | Within the ≤ 30 mJ limit for v1; v2 option: a fast “leg within 0.5 V of a rail” detector (≈ 1 ms) |
| Unplug pop and the VBUS watchdog margin (≈ 3.8 µs ADC latency against ≈ 6 µs of hold-up; the DAC-first order is impossible at unplug) | Pop at unplug | Measure at bring-up, and the watchdog-to-GPIO latency on Stage 2; the PGOOD latch is the backstop (effective: 5V_ANA is ≈ 3.6 V when PGOOD goes bad); DAC-pin safety does not depend on this timing (VREF tracks AVCC) |
| VBUS hot-plug ring (ceramic capacitors only; LP5907 absolute maximum 6.0 V, LM27762 5.8 V; no VIN margin above 5.5 V) | Regulator overstress | Hot-plug test on board 1 (scope C102); DNF TVS D102 fitted only if the peak exceeds 5.8 V, after measuring its leakage at 5.25 V; VBUS measured on the chargers used (5.5 V corner accepted for v1) |
| 4.4 mm plug shorted leg+ to leg− (outside TI’s one-amplifier short rating); both jacks live, so a 4.4-to-3.5 mm adapter shorts leg− | OPA1622 stress (≈ 0.55 W per package; ≈ 0.45 W with the adapter) | Bench survival test; warning in the manual |
| OPA2210 stability with the DAC pin capacitance (plus ≤ 20 pF per node from the clamps D405–D408, noise gain ≈ 1.03) and the 10 Ω + 660 pF network; OPA2210 output swing (≈ (V+) − 0.3 V estimated); OPA1622 capacitive-load stability and disabled-state output with the network | Oscillation, peaking, clipping or pop | SPICE before layout; bench check with 1 nF and long cables (0 Ω output links; 2.2–4.7 Ω only as a last resort, which breaks the ≤ 0.5 Ω output-impedance target) |
| LM27762 dissipation 0.39 W at 5.25 V (0.46 W at 5.5 V; θJA 62.2 °C/W → ≈ +24 °C) and U504 0.20 W at 5.25 V (0.24 W at 5.5 V against its 300 mW rating) | Regulator temperature | Copper pours on the LDO and charge-pump pins; IR check at bring-up |

## 9.2 Decisions (closed for v0.8; verification items and owner actions open)

| Decision | Status | Resolution |
| --- | --- | --- |
| USB bridge | Closed | WCH CH32V307RCT6 + AGM AGRV2KQ32 CPLD |
| USB firmware and CPLD logic | Closed (owner decision, 24 Sep 2026) | Our own UAC2 firmware on WCH’s register-level USBHS device examples and our own CPLD logic (Section 8); USBAudioToI2S kept only as a known-good comparison and reference; TinyUSB only if its CH32V307 high-speed audio path proves reliable |
| DAC and output mode | Closed | ESS ES9018K2M in differential current mode with one I/V converter per pin (ESS application circuit); the newer ES9039Q2M / ES9020Q are a future option |
| Clock architecture | Closed (changed in v0.8) | DAC MCLK 80 MHz from X201 (SCTF SX3M80.000B20F30TNN: 12.5 ns cycle, ≥ 5.6 ns pulse width, ≈ 208 × 384 kHz) only, enabled by the CPLD (MCLK_EN, R218 pull-down), DAC in asynchronous mode; the CPLD has only the family clock on PIN_1 (PLL) and the SPI clock (ALTCLKCTRL); family oscillators X202 24.576 MHz and X203 22.5792 MHz, both off by default (2.2 kΩ pull-downs), selected break-before-make in front of the CPLD PLL (×2); all eight rates with exact integer clock ratios and a 50:50 bit clock; DNF mux U205 with R216/R217 input pull-downs as the fallback; no clock multiplexer in the fabric |
| I/V op-amp | Closed (fallback designed in v0.8) | OPA2210 (single input pair; VREF ≈ 0.53 V inside the common-mode range at +3.90 V (VREF ≤ 1.754 V worst case, minimum rail 3.789 V); Vos ±35 µV max; 2.2 nV/√Hz; output swing ≈ (V+) − 0.3 V at ≈ 3 mA) replaces the OPA2197, whose input-pair transition region contains VREF; feedback 836 Ω with 10 Ω + 660 pF; JLCPCB stock unverified, so the OPA1612AIDR (C94590) is designed in as the fallback, with V+ raised to +3.90 V (VREF ≥ 35 mV inside its range) |
| Output stage | Closed | 2 × OPA1622 difference amplifiers fed by the I/V outputs, one amplifier per leg; clean envelope ≤ 80 mA peak per leg (70 mA at 4.4 mm 32 Ω) |
| Headphone rails | Closed (changed in v0.8) | +3.90 V / −3.72 V from the LM27762 (225 kΩ/100 kΩ with the new R523 10 kΩ, and 205.1 kΩ/100 kΩ with R518 5.1 kΩ, from Basic values); tolerance ranges include VFB and the 1 % resistors; headroom by the peak-current method at host VBUS 4.75 V with 0.3 Ω of cable and connector (negative-rail margin +0.05 V worst case at 70 mA) |
| Level and calibration | Closed (changed in v0.8) | Every unit calibrated to 1.55 Vrms per leg with the master trim at the end-of-line test (OPA1622 clean swing ≈ 1.3 V from the rail); −4.5 dB before calibration, a safety setting |
| Power modes | Closed (changed in v0.6) | Default (≤ 500 mA; estimated maximum 0.475 A with hardware-limited analog current) and High (only with a ≥ 1.5 A Type-C advertisement read on CC by the MCU ADC; estimated maximum 0.760 A); CC monitored while configured (tSinkAdj response); bus-powered descriptors 0x80 / 0xFA |
| Analog current limit | Closed (changed in v0.6) | TPS2553 with two limits (Default 0.157–0.213 A, High 0.393–0.498 A), acting on the instantaneous current; overload → PGOOD bad or a sustained FAULT → fault latch → firmware recovery with the volume ceiling |
| Power domains | Closed (changed in v0.8) | 3V3M always on; 3V3D (ME6211 #2 CE through R521, behind the R519/R520 dropper with C512 bulk and the R524 bleeder: CPLD on its FB202 branch, oscillators, DVCC), 3V3A (LP5907 EN through R307, also killed by the latch) and the rails (TPS2553 EN) switched; fixed 1.3 V ME6216A13M3G DVDD fed from 3V3D, with the R525 bleeder |
| Offset limit | Closed (changed in v0.5) | ±10 mV unbalanced and ±20 mV balanced (was ±15 mV), with 100 % screening by a DC-capable DMM |
| Protection | Closed (changed in the v0.8 amendment) | DC detect by two LM339DR with one window per leg, specified by its worst case (R601–R604 100 kΩ; thresholds 62 kΩ / 10 kΩ and 57 kΩ / 10 kΩ: 2 V ≤ 47 ms, 1 V ≤ 125 ms); redesigned DC-sense network (15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF, 3V3A monitor) and four firmware DC tiers per leg and per balanced pair (≥ 1.0 V in 0.25 s … ≥ 0.2 V in 3 s) setting the same latch; stream DC guard; N-MOSFET latch with ≈ 35 µs capture (RAILS_OK_N pulled up to 5V_ANA through 10 kΩ) and C605 on node F, set by PGOOD-bad (Q610), a sustained TPS2553 overcurrent (D606, R638) and the firmware, armed 51–186 ms after PGOOD good and cleared only by the firmware; the latch kills RAIL_EN, AMP_EN and AVCC_EN (Q611), each behind a 10 kΩ series resistor; Q609 slow interlock; firmware start-up rules; enables dropped first, then the cause captured at the interrupt; VREF tracks AVCC; VBUS watchdog 4.10 V; no relays |
| Connectors | Closed (pin numbers to verify) | 3.5 mm unbalanced + 4.4 mm balanced; one headphone at a time; jack and USB-C pin numbers confirmed by continuity tests on bought samples before layout |
| Volume | Closed | Host volume (UAC2 Feature Unit) in the ES9018K2M attenuator; hardware knob is a v2 option |
| Budget | Closed (owner statement and clamp decision, 24 Sep 2026) | Target 1,500 NTD per board (WARN above). Ceiling 1,700 NTD per board, set when the third oscillator was added, plus the owner-approved 21 NTD for the DAC-pin clamps (24 Sep 2026) → order gate 1,721 NTD per board (FAIL above); on 24 Sep 2026 the owner confirmed the budget is flexible as long as it does not surge and decided to fit the clamps D405–D408; the gate covers every JLCPCB line of the real quote, which must pass the order gate before ordering, with the gated subtotal and total payable recorded (currently 1,719 NTD: WARNING, 2 NTD under the order gate) |
| Firmware gate | Open (gate before layout) | Stage 2 board (two boards: shared node and mux, with the real regulators, the real latch and protection on dummy loads, and the ES9018K2M with its real supplies) is the first hardware and the layout gate; the Stage 1 desk setup is optional (owner constraint, 24 Sep 2026: development boards are hard to get in Taiwan); pass criteria in Section 8 |
| MCU–CPLD interface and pin map | Open (before the Stage 2 board) | Write the interface specification (SPI at 36 MHz with chip-select framing) and freeze both pin maps (proposed in Schematic Design Notes v0.7) before the Stage 2 board and the schematic; J202 is 2-wire Compact-JTAG (JTCK, JTMS, NRST) |
| Oscillator disable time and disabled-output state | Open (Stage 2 measurement) | Not specified in the SCTF/YXC datasheets: measured with a scope on the shared node; fallback: mux U205 fitted (one Stage 2 board is built with it) |
| MCU → CPLD audio link | Closed (changed in v0.8) | SPI with chip-select framing (MCU master at 36 MHz, DMA; one CS frame per block of samples with a 16-bit header), written into the FIFO in the SPI clock domain and read in the PLL bit-clock domain (no CPLD system clock); I²S is not used for the link; the 36 MHz capture and the reserved second lane are checked on Stage 2 |
| USB currents: unconfigured ≤ 100 mA, Default-mode estimated maximum 0.475 A, suspend ≤ 2.5 mA within 10 ms (v1 requirement) | Open (Stage 2 gate) | Measured on the Stage 2 board before layout (Default mode is 25 mA below 0.5 A; suspend budget ≈ 2.30 mA, level typically ≈ 8.2 ms and at worst 9.3 ms after the last bus activity, as current vs time) and again on the main boards |
| Residual DC below the last firmware tier | Closed – owner decision (24 Sep 2026) | Below the T4 tier (0.11 V nominal, ≤ 0.198 V worst case across a driver, per leg and per balanced pair) nothing trips (≤ 2.45 mW into 16 Ω, ≤ 1.23 mW into 32 Ω, now also for balanced headphones): accepted by the owner on 24 Sep 2026 |
| DAC pin mismatch and zero-current point | Open (verification at bring-up) | Null 1.702 V (current-output formula) or 1.65 V (voltage-source model): VREF null test at bring-up of the first main boards, R433 0 Ω option; 100 % offset screening with a DC-capable DMM |
| OPA2210 stability and output swing; OPA1622 capacitive-load stability and disabled-state output with the network | Open (verification) | SPICE before layout; 1 nF and long-cable test at bring-up |
| OPA1622 clean swing at +3.90 V / −3.72 V and negative-rail margin at 4.60 V and 4.40 V | Open (verification) | Bench measurement |
| Final cost | Open (after layout) | JLCPCB quote from the real BOM/CPL files must pass the 1,721 NTD order gate (the 1,700 NTD ceiling plus the owner-approved 21 NTD for the DAC-pin clamps) with every JLCPCB line included (any fixture line counts; the estimate is 2 NTD under it); confirm the class of X202; check the OPA2210 stock and buy 12 or switch to the OPA1612 fallback; buy 7 × J701; record the gated subtotal and the total payable |

# 10. Next steps

| # | Action | Done |
| --- | --- | --- |
| 1 | Owner: check the OPA2210IDR stock in the JLCPCB Parts Manager now; if ≥ 12, buy 12 into the private library, otherwise switch U403/U404 to the OPA1612AIDR (C94590). | ☐ |
| 2 | Owner: buy 7 × 4.4 mm jacks (C2762984) into the JLCPCB library (JLCPCB stock 21). | ☐ |
| 3 | Owner: reserve the CH32V307RCT6, ES9018K2M and AGRV2KQ32. | ☐ |
| 4 | Owner: buy a J-Link or AGM DAP-Link for the CPLD (confirm which models Supra accepts) and a WCH-LinkE for the MCU. | ☐ |
| 5 | Owner: buy 2 samples of each jack (and the USB-C receptacle if possible) and continuity-test the pin numbers before layout. | ☐ |
| 6 | Owner: compare the ESS application-circuit figure (datasheet p. 27) with the I/V design by eye. | ☐ |
| 7 | MCU–CPLD interface specification (SPI at 36 MHz with CS framing) and freeze of both pin maps (before Stage 2). | ☐ |
| 8 | Stage 2 boards, the first hardware (shared-node and mux builds, with the real regulators, latch and protection on dummy loads and the ES9018K2M with its real supplies): USB and UAC2 at all 8 rates, bit-perfect path to the DAC, 36 MHz SPI link, PLL lock, clock handover, USB currents incl. suspend current vs time, watchdog latency, CPLD programming through J202 — layout gate. The Stage 1 desk setup only if development boards can be obtained. | ☐ |
| 9 | SPICE before layout: OPA2210 I/V stability including the ≤ 20 pF of the clamps D405–D408 (open release condition), OPA2210 output swing, OPA1622 capacitive load and disabled-state output with the network, protection timing, failed output stage. | ☐ |
| 10 | Schematic capture and review against the checklist (AI agent + netlist script); layout; JLCPCB quote (gated subtotal ≤ 1,721 NTD per board, the order gate; total payable recorded). | ☐ |
| 11 | Bring-up and end-of-line tests (first power-up without headphones, calibration, DMM offset screening, protection incl. failed-output emulation and short PGOOD pulses, firmware DC tier, USB and suspend, AVCC-on pop, hot-plug ring, DAC-pin excursion measured on the four DAC pins with the clamps D405–D408 fitted during failed-output emulation and hot unplug (within −0.4 V…AVCC + 0.4 V) and the 3V3A decay, jitter/clock, rails, power and dissipation). | ☐ |

# Appendix A. Reference documents

| Ref | Document | Source |
| --- | --- | --- |
| [1] | ES9018K2M datasheet v3.7 (22 Apr 2021) | ESS Technology |
| [2] | OPA1622 datasheet (SBOS727B) | Texas Instruments |
| [3] | OPA197/OPA2197 datasheet (SBOS737C): input-pair transition region, table 6.8 and section 7.3.6 (the fifth-round finding against the former I/V op-amp) | Texas Instruments |
| [4] | LM27762 datasheet (SNVSAF7C) | Texas Instruments |
| [5] | TPS2553 datasheet (SLVS841F) | Texas Instruments |
| [6] | CH32V307 datasheet and EVT package (high-speed USB device examples) | WCH |
| [7] | AGRV2K CPLD datasheet | AGM |
| [8] | USBAudioToI2S: UAC2.0-to-I²S firmware for MCU + CPLD (Apache-2.0); known-good comparison and reference | GitHub, LoveLonelyTime/USBAudioToI2S |
| [9] | USB Type-C specification (current-advertisement voltage bands) | USB-IF |
| [10] | JLCPCB PCBA price list | JLCPCB |
| [11] | Schematic Design Notes v0.7 (companion document: block-level circuit guidance and chosen parts) | This project |
| [12] | Parts List v0.6 (source of truth for cost) | This project |
| [13] | OPA210/OPA2210 datasheet (SBOS924H, Aug 2021): I/V converters U403 and U404; output swing, absolute maximum input current and section 7.3.5 | Texas Instruments |
| [14] | 3C series oscillator datasheet (X201, SX3M80.000B20F30TNN: 40 mA max for 80–160 MHz) | SCTF |
| [15] | 2CM series oscillator datasheet, manufacturer file “2CM-Series-2024-11-04” (X203: tri-state pin 1, start-up ≤ 5 ms) | SCTF |
| [16] | YSO110TR oscillator datasheet (X202: tri-state pin 1, start-up ≤ 3 ms) | YXC |
| [17] | USB Type-C sink timing (tSinkAdj 60 ms max, tRpValueChange 10–20 ms), as quoted in the Zephyr USB-C stack and the Chromium EC documentation | USB-IF specification; Zephyr Project; Chromium OS EC |
| [18] | LP5907 datasheet: 230 Ω output auto-discharge (3V3A fall time and VREF tracking) | Texas Instruments |
| [19] | ME6211 datasheet: 300 mW power dissipation rating in SOT-23 (absolute maximum; U504 dissipation) | Microne |

# Appendix B. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Draft for review |
| v0.2 | 23 Sep 2026 | Diego Yang | Budget raised to ≤ 1,500 NTD; outputs changed to 3.5 mm unbalanced + 4.4 mm balanced; parts and targets verified against the XU316-1024-QF60A, AK4493S, AKD4493S-A and OPA1622 datasheets; USB chip changed to XU316-1024-QF60A (1.8 V I/O); OPA1612 stage-1 and LT3045 added; CH224K replaced by CH224Q; JLCPCB fees updated to the 8 Sep 2026 price list; most open decisions closed. |
| v0.3 | 23 Sep 2026 | Diego Yang | Remaining parts chosen from stocked LCSC/Digi-Key parts; CH224A replaces CH224Q; TPS7A4901 replaces LT3045 to stay within 1,500 NTD; protection defined (LM339 + 2 × HFD4 relays with hardware amplifier shutdown); cost re-estimated with real prices at ≈ 1,490 NTD; −14 V rail set to −14.7 V; stage-1 count corrected to one dual OPA1612. |
| v0.4 | 23–24 Sep 2026 | Diego Yang | Cost-driven redesign for the 5-board budget: single USB-C bus-powered design; CH32V307 + AGRV2K CPLD USB bridge (open-source USBAudioToI2S); ES9018K2M DAC; 2 × OPA1622 as four difference amplifiers; LM27762 charge-pump rails; relays replaced by a 3.3 V hardware fault latch; three design reviews dispositioned. Revised on 24 Sep 2026: DAC output moved to current mode with I/V converters (owner decision, option B) and an owner-approved cost ceiling of 1,550 NTD per board. |
| v0.5 | 24 Sep 2026 | Diego Yang | Fourth review round (R4-A, R4-B, R4-C) dispositioned (Appendix C): headphone rails ±3.78 V; per-unit calibration to 1.75 Vrms per leg (Rf 836 Ω); power rated at 1 % THD+N or calibrated 0 dBFS, with High and Default modes; TPS2553 current limit and switched power domains per USB state; OPA2197 I/V converters; DAC in asynchronous mode from a 100 MHz MCLK; first issued with the interim both-edge divider for the 44.1 kHz family and amended the same day to three oscillators (next row); protection completed (PGOOD kill, blanked latch, energy bound at the terminals); fixed 1.3 V DVDD LDO; balanced offset limit ±20 mV with 100 % screening; Parts List v0.3 with verified LCSC numbers and classes; cost 1,540 NTD per board (WARNING, under the 1,550 NTD ceiling); pre-layout firmware gate. |
| v0.5 amendment | 24 Sep 2026 | Diego Yang | Amendment for the owner decisions of 24 Sep 2026: family oscillators for exact bit clocks, because the ES9018K2M requires a bit-clock duty cycle of 45:55 to 55:45 and one 100 MHz clock for both families gave 40:60 at 352.8/384 kHz. First X202, 24.576 MHz (YXC OT322524.576MJBA4SL), for the 48 kHz family; then, at the owner’s request, X203, 22.5792 MHz (SCTF SX2M22.5792M20F30TNN), for the 44.1 kHz family, which replaced the interim both-edge fractional divider and its 352.8 kHz condition. Final clock architecture: DAC MCLK 100 MHz unchanged, family oscillator selected by its enable pin in front of the CPLD PLL (×2), all eight rates exact, family change with the PLL held in reset, no clock multiplexer in the fabric. Own UAC2 firmware and CPLD logic instead of modifying USBAudioToI2S (kept as a known-good comparison), with the firmware gate run as a two-stage proof of concept. Currents updated (Default ≤ 0.473 A, High ≤ 0.758 A); cost 1,540 → 1,602 → 1,666 NTD per board (WARNING), approved ceiling raised from 1,550 to 1,650 and then 1,700 NTD; Figures 1 and 2 updated. |
| v0.6 | 24 Sep 2026 | Diego Yang | Fifth review round (R5-A, R5-B, R5-C of the amended Spec v0.5) dispositioned (Appendix C): 80 MHz MCLK and CPLD system clock (SCTF SX3M80.000B20F30TNN) below the DAC limit; family oscillators off by default with a break-before-make handover, a node pull-down and a DNF mux fallback (U205); MCU link captured in its own clock domain; OPA2210 I/V converters (single input pair); rails +3.78 V / −3.72 V including resistor tolerance (R518 5.1 kΩ); fault latch also set by PGOOD-bad (Q610) and a sustained TPS2553 overcurrent (D606); rails-first-off power-down, VBUS watchdog 4.10 V dropping AMP_EN and RAIL_EN, VREF clamp D401 and C421 100 nF; protection envelope by DC magnitude with recalculated terminal energies; Default-mode power rated from the instantaneous peak current; USB estimated maxima 0.469 A (Default) and 0.754 A (High); mandatory Stage 2 board with the USB current measurements as a layout gate; DC offset screening with a DC-capable DMM; J703 MCLK monitor-only; document-lint CI check; cost 1,647 NTD per board (WARNING, 53 NTD under the 1,700 NTD ceiling, recorded with the owner statement of 24 Sep 2026); Figures 1 and 2 updated; companion documents Schematic Design Notes v0.5 and Parts List v0.4. |
| v0.7 | 24 Sep 2026 | Diego Yang | Sixth review round (R6-A, R6-B, R6-C and R6-D of Spec v0.6) dispositioned (Appendix C): latch capture ≈ 35 µs with C605 on node F (power-on set, set-request stretch), 47 kΩ latch pull-ups, Q609 as a slow interlock and Q611 killing AVCC_EN; worst-case DC detector (R601–R604 100 kΩ, R609/R610 10 kΩ: 2 V ≤ 59 ms, 1 V ≤ 183 ms) and worst-case terminal energies (26.2 / 13.3 mJ at 16 / 32 Ω including ≤ 0.6 mJ after the trip); slow firmware DC tier through the MCU ADC (≥ 0.8 V in 1 s, ≥ 0.2 V in 3 s) setting the same latch, with the residual below 0.2 V as a pending owner decision; fault-cause capture at the interrupt and a 3-strike rule for DC faults; VREF tracks AVCC (C421 330 pF; D401 and C422 removed) and the DAC analog supply turns off first, so a controlled power-down takes ≤ 4 ms; USB suspend ≤ 2.5 mA within 10 ms as a v1 requirement; pop definition; regulator dissipation table with the R519/R520 dropper and C512; DAC centre-voltage alternative with the R433 option; OPA2210 output swing and the −3.5 dB safety default; SPI link with chip-select framing; SDATA/LRCLK on the falling BCLK edge; mux pull-downs R216/R217; 31 test pads; Stage 2 with the real regulators (≈ US$90 ≈ 2,860 NTD); cost 1,650 NTD per board (WARNING, 50 NTD under the 1,700 NTD ceiling, gate covering every JLCPCB line); Figure 1 updated; companion documents Schematic Design Notes v0.6 and Parts List v0.5. |
| v0.8 | 24 Sep 2026 | Diego Yang | Seventh review round (R7-A…R7-E, a full design check of Spec v0.7, Schematic Design Notes v0.6 and Parts List v0.5 in five areas) dispositioned (Appendix C): series resistors R307 (AVCC_EN) and R521 (AUD_EN); PGOOD pulled up to 5V_ANA through R505 10 kΩ with R522, R623 1 MΩ and C607 10 nF (genuine ≈ 35 µs capture, new blanking and arming times); TI LM393B detector with 147 kΩ / 10 kΩ thresholds, C612/C613 and R638 (leg trip 0.36–0.80 V; 1 V ≤ 179 ms, 2 V ≤ 58 ms; 25.6 / 13.1 mJ at 16 / 32 Ω); firmware start-up rules and FLT interrupt order; residual DC below 0.2 V accepted by the owner; calibrated level 1.55 Vrms per leg with the 1.3 V OPA1622 clean-swing model, new power targets and a 70 mA envelope at 4.4 mm 32 Ω; DR estimate 117.4 / 118.6 dB(A); V+ +3.90 V (R523) for the OPA1612 fallback; DAC reset state handled by the power-down order (MCLK_EN; pop at risk until measured); CPLD on 2-wire Compact-JTAG (J202), family clock on PIN_1, no 80 MHz input, MCLK_EN with R218, FB202 and C220; J201 with NRST, BOOT0 and UART pads; programming mode (TP732, 32 test pads); VBUS watchdog on ADC1; bleeders R524/R525; DNF TVS D102; R107 0 Ω and C101 DNF; LM393BIDR added to the key parts; USB estimated maxima 0.474 A / 0.759 A; suspend budget 2.31 mA, level ≈ 9.3 ms; Stage 1 optional and Stage 2 (with the ES9018K2M, ≈ US$93 ≈ 2,940 NTD) the first hardware; cost 1,691 NTD per board (WARNING, 9 NTD under the 1,700 NTD ceiling); Figures 1 and 2 updated; companion documents Schematic Design Notes v0.7 and Parts List v0.6. |
| v0.8 (amended, 24 Sep 2026) | 24 Sep 2026 | Diego Yang | Amended in place (same version) for three external reviews of the previous set (R7-F, R7-G, R7-H; Appendix C.2): hardware DC detector changed to two TI LM339DR quad comparators with one window per leg and no diode-OR (BAV99 D601–D604 and R605/R606 removed; R607/R608 47 kΩ, R636 15 kΩ, R637 10 kΩ; C612–C615): leg trip 0.43–0.67 V, 1 V ≤ 125 ms, 2 V ≤ 47 ms, rail-stuck 21.4 / 11.0 mJ at 16 / 32 Ω, balanced digital-DC note; slow DC-sense network redesigned (15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF, 3V3A monitor on PC4) with its error budget; four firmware DC tiers per leg and per balanced pair with the accepted envelope; DAC-pin analysis for a failed-output shutdown stated as a residual risk, C421 330 pF → 33 pF, DNF BAT54S clamps D405–D408 and DAC-pin tests; adaptive AMP_EN at power-down (suspend typically ≈ 8.2 ms, at worst 9.3 ms), LEDs off in suspend, resume ≈ 1.0 s, stream DC guard, option A′; USB Default 0.475 A, High 0.760 A, suspend budget ≈ 2.30 mA; cost 1,697 NTD per board (8,487 NTD for five; WARNING, 3 NTD under the ceiling); Figures 1 and 2 updated; companion documents Schematic Design Notes v0.7 and Parts List v0.6, both amended the same day; DAC-pin clamps D405–D408 fitted (owner decision). |

# Appendix C. Design review disposition

Status values: Closed = fixed in this revision; Closed – design = design changed in this revision; Closed – design, verify = design change made, confirmed by a bench or firmware-gate check; Closed – firmware rule = firmware requirement added (Section 8); Closed – documentation = the documents corrected; Closed – verify = resolved, confirmed at the firmware gate, on the Stage 2 board or at bring-up; Closed – N/A = not applicable after the v0.4 redesign; Closed – owner decision = decided by the owner; Open – verify (on Stage 2) = to be confirmed on hardware; Open – owner action = waiting for an owner purchase or check (Section 10); Open – owner decision = awaiting the owner’s decision; Open – tracked = tracked item with an owner and a gate (Section 9.2); Accepted (v1) = accepted for v1 without change; Partly closed = some parts closed, the rest open with a gate; residual risk = a consequence of an already-failed part, stated and measured but not designed out; Not adopted = optional change not taken; Info = confirmation, no action.

## C.1 Seventh review round (R7-A…R7-E, full design check of Spec v0.7)

Finding IDs (the reviewers’ own numbers are kept, written for example as R7-C PR-1): R7-A = power (P-1…P-22), R7-B = analog (AN-1…AN-13), R7-C = protection (PR-1…PR-19), R7-D = digital (DG-1…DG-21), R7-E = parts and cost (BM-1…BM-15). The five independent reviews covered Spec v0.7, Schematic Design Notes v0.6 and Parts List v0.5 before any hardware is bought, because development and test boards are hard to get in Taiwan. They raised three blockers (PR-1 = P-1 = DG-3, the AVCC_EN contention; DG-1, the CPLD programming port; BM-1, the unverified OPA2210 stock) and 13 high findings; findings raised by more than one reviewer are cross-referenced. The last row closes R6-B2 of the sixth round.

| ID | Finding | Disposition in v0.8 | Status |
| --- | --- | --- | --- |
| R7-A P-1 | AVCC_EN has no series resistor, so the Q611 latch kill fights the MCU output (= PR-1, DG-3) | New R307 10 kΩ from PC11 to the LP5907 EN node; Q611 pulls against 0.33 mA | Closed – design |
| R7-A P-2 | RAILS_OK_N loaded by R623/C607 and R617/C606 (step to ≈ 1.57 V); capture timing not real; PGOOD pulled up while the LM27762 is unpowered (= PR-2) | R505 → 10 kΩ to 5V_ANA; R522 10 kΩ into PB4; R623 → 1 MΩ, C607 → 10 nF; rise < 1 µs; new blanking and arming times | Closed – design, verify |
| R7-A P-3 | 2.2 Ω MPN and LCSC number disagree (0805W8F220JT5E is 22 Ω) (= AN-8, BM-3) | MPN corrected to UNI-ROYAL 0805W8F220KT5E (C17521) | Closed – documentation |
| R7-A P-4 | U504 CE can exceed VIN + 0.3 V at unplug under load | New R521 10 kΩ from AUD_EN (PC10) into U504 CE; clamp current ≤ 0.2 mA | Closed – design |
| R7-A P-5 | Suspend budget leaves out the D+ pull-up and uses a low LDO Iq | Budget 2.31 mA (D+ pull-up 0.20 mA, ME6211 60 µA typ; margin 0.19 mA) | Closed – documentation |
| R7-A P-6 | Negative-rail headroom mixed average and peak currents and used a low cable drop | Peak-current method with 0.3 Ω at the real current (host 4.75 V): +0.05 V worst case at 70 mA; 70 mA envelope at 4.4 mm 32 Ω | Closed – design |
| R7-A P-7 | VBUS has ceramic capacitors only and no surge clamp | DNF D102 (TVS at J101, VRWM 6.0 V); hot-plug test on board 1; fitted only if the peak exceeds 5.8 V | Closed – design, verify |
| R7-A P-8 | 3V3D and 1V3 have no discharge path | Bleeders R524 2.2 kΩ (3V3D) and R525 1 kΩ (1V3); AUD_EN off ≥ 100 ms before re-enabling | Closed – design |
| R7-A P-9 | ME6216A13M3G pinout not recorded; some claims differ from the datasheet (= BM-4) | SOT23-3: pin 1 VSS, pin 2 VOUT, pin 3 VIN; 250 mA max, ±15 mV, Pd 0.54 W; stock ≈ 3,100 | Closed – documentation |
| R7-A P-10 | USB-C J101 pin functions not checked against the drawing | A6/B6 and A7/B7 tied at the connector; drawing and sample check before layout | Open – verify |
| R7-A P-11 | Minimum delay from ANA_EN to RAIL_EN not specified | RAIL_EN ≥ 10 ms after ANA_EN | Closed – firmware rule |
| R7-A P-12 | The rails start inside the TPS2553 limit; the inrush can look like a fault | Firmware ignores its ANA_FAULT classification for 20 ms after RAIL_EN; bench check in Default mode at 4.40 V | Closed – firmware rule, verify |
| R7-A P-13 | PGOOD pulled up to 3V3M while the LM27762 VIN is 0 V (= PR-9) | Moot: PGOOD now pulled up to 5V_ANA (P-2) | Closed – design (moot) |
| R7-A P-14 | LM27762 dissipation quoted at the design corner only | 0.39 W at 5.25 V (≈ +24 °C), 0.46 W at 5.5 V | Closed – documentation |
| R7-A P-15 | U504 evaluated only up to 5.25 V | 0.20 W at 5.25 V, 0.24 W at 5.5 V (300 mW rating; 160 mA including the bleeders) | Closed – documentation |
| R7-A P-16 | 3V3A discharge time used the typical LP5907 discharge | 5.5 ms worst case used in the power-down timing | Closed – documentation |
| R7-A P-17 | CC firmware thresholds not stated | 0.66 V and 1.23 V with hysteresis; only the CC pin with Rp is active | Closed – firmware rule |
| R7-A P-18 | VBUS_SENSE source impedance vs ADC sampling for the < 5 µs watchdog (= DG-5) | ADC1 continuous on VBUS_SENSE with the analog watchdog, 7.5-cycle sampling at 12 MHz: ≈ 3.8 µs worst-case latency | Closed – design |
| R7-A P-19 | Shell capacitor C101 overstressed by ESD; CC lines without ESD devices | R107 0 Ω (shell to GND), C101 DNF; CC1/CC2 rely on the 10 kΩ series resistors | Closed – design |
| R7-A P-20 | BOM and documentation details (C15849 rating, ME6211 dropout, LM27762 symbol pins) | C15849 is 50 V; ME6211 dropout 120 mV typ at 100 mA; LM27762 PGOOD = pin 1, FB− = pin 7 | Closed – documentation |
| R7-A P-21 | Start-up into the ME6211 and ME6216 foldback current limits | AUD_EN cycled 100 times at 4.40 V and 5.25 V on Stage 2 and at bring-up; MCLK off until DVDD is up | Open – verify |
| R7-A P-22 | FB501 + C507 LC resonance (≈ 80–100 kHz) | LM27762 VIN ripple checked at light load; 0.47–1 Ω + 1 µF damping if needed | Open – verify |
| R7-B AN-1 | Jack pin numbers not documented or verified | Pin-number placeholders in the Notes; buy 2 samples of each jack and continuity-test every pin before layout; checklist rule | Open – owner action |
| R7-B AN-2 | OPA1622 clean swing ends ≈ 1.3 V from the rail, not 0.9 V | Swing model (¦rail¦min − 1.3 V)/(1 + 8 Ω/Rleg); calibrated 1.55 Vrms per leg; −4.5 dB before calibration; new power targets | Closed – design |
| R7-B AN-3 | DAC reset state not modelled (outputs driven to ground) | Power-down keeps RESETB high and stops MCLK (MCLK_EN) before AVCC goes off; AVCC-on step ≈ 18–35 mV measured; option B kept; pop at risk until measured | Closed – design, verify |
| R7-B AN-4 | Pin-level connections of the OPA1622 and the ES9018K2M not documented | OPA1622 VSON-10 and ES9018K2M QFN-28 pin lists in the Notes; OPA1622 pin 3 to GND | Closed – documentation |
| R7-B AN-5 | Leg-network noise underestimated | ≈ 11.5 nV/√Hz (13.0 with the OPA1622); DR 117.4 / 118.6 dB(A) at 1.55 Vrms | Closed – documentation |
| R7-B AN-6 | Default-mode unbalanced peak rule left out the charge-pump factor | 53 mA + 2 × 1.03 × Ipk in the power table | Closed – documentation |
| R7-B AN-7 | The 0.47 Ω fallback cannot fix capacitive-load instability | 0 Ω links kept; 1 nF and long-cable test; 2.2–4.7 Ω only as a last resort | Closed – documentation, verify |
| R7-B AN-8 | 2.2 Ω MPN decodes to 22 Ω (= P-3, BM-3) | UNI-ROYAL 0805W8F220KT5E (C17521) | Closed – documentation |
| R7-B AN-9 | Disabled-leg DC path and DC-tier gating | AVCC_EN only while AMP_EN is high and the rails are good; slow tier armed whenever AVCC_EN is high | Closed – firmware rule |
| R7-B AN-10 | DC-sense accuracy depends on unspecified MCU pin leakage | Superseded by the redesigned DC-sense network (R7-F F3, Section C.2): leakage error ≤ 15 mV at the leg inside the full error budget | Closed – design, verify |
| R7-B AN-11 | ESS application-circuit figure (p. 27) could not be compared | Owner compares it with the design by eye before the schematic | Open – owner action |
| R7-B AN-12 | Small numeric inconsistencies (VREF time constant) | τ = 7.74 kΩ × 330 pF = 2.55 µs | Closed – documentation |
| R7-B AN-13 | J701 JLCPCB stock is 21, not 721 (= BM-2) | Buy 7 (5 + 2) into the JLCPCB library | Open – owner action |
| R7-C PR-1 | AVCC_EN / Q611 contention (blocker; = P-1, DG-3) | R307 10 kΩ; every enable the latch kills now has a series resistor (R507, R618, R307) | Closed – design |
| R7-C PR-2 | The capture claim ignored the loading of RAILS_OK_N (= P-2) | R505 10 kΩ to 5V_ANA, R522, R623 1 MΩ, C607 10 nF; ≈ 35 µs capture genuine | Closed – design, verify |
| R7-C PR-3 | LM393 input bias current (up to 400 nA) into the 1 MΩ loads can false-trip | Superseded by the LM339 windows (R7-G G2, Section C.2); the interim fix was U601 → TI LM393BIDR with 147 kΩ / 10 kΩ thresholds | Closed – design |
| R7-C PR-4 | Blanking hides faults present at power-up | PGOOD timeout 20 ms, start-up DC watch at 1 kHz (> 0.5 V drops RAIL_EN), first power-up without headphones | Closed – firmware rule |
| R7-C PR-5 | Arming time assumes a fully charged blanking capacitor | FLT_CLR ≥ 600 ms after ANA_EN or rails off; second FLT_CLR at PGOOD good + 20 ms; AMP_EN ≥ 250 ms after PGOOD good | Closed – firmware rule |
| R7-C PR-6 | The FLT interrupt leaves the MCU driving the enables against the kill FETs | ISR drives AMP_EN, RAIL_EN, AVCC_EN, AUD_EN and RESETB low within 50 µs, then captures the cause; IWDG enabled | Closed – firmware rule |
| R7-C PR-7 | MCU pin configuration on the latch nodes not specified (= DG-9) | FLT, RAILS_OK_N and ANA_FAULT floating inputs without internal pulls; unit test on the GPIO table | Closed – firmware rule |
| R7-C PR-8 | Cause classification used instantaneous readings | Pre-trip 0.5-s average of each DC-sense channel (ring buffer) | Closed – firmware rule |
| R7-C PR-9 | PGOOD pull-up to 3V3M while VIN = 0 (= P-13) | Moot: pull-up on 5V_ANA | Closed – design (moot) |
| R7-C PR-10 | No U601 supply decoupling | C612, C613 100 nF | Closed – design |
| R7-C PR-11 | DC-tier ADC network source impedance vs sampling | Superseded by the redesign (R7-F F3, Section C.2): 4.7 kΩ source, dithered 1 kHz sampling, auto-zero, 3V3A monitor | Closed – design, verify |
| R7-C PR-12 | DC-sense node below 0 V on a negative leg | Fixed by the redesign (R7-F F3, Section C.2): the node stays within 0.29–2.85 V at every corner | Closed – design |
| R7-C PR-13 | Timing numbers in the documents | New timing: stretch 1.5–7.7 ms, Q609 on 1.9–9.4 ms, re-blank 19–94 ms, arming 51–186 ms | Closed – documentation |
| R7-C PR-14 | Documentation and netlist ambiguities | Netlist details in the Notes (BAV99 pins, comparator inputs, U601 pins, D605/D606, Q607/Q608/Q611 gates, OPA1622 pin 3) | Closed – documentation |
| R7-C PR-15 | LM393DR2G ESD rating only 250 V HBM | LM339DR ESD ±2 kV HBM (TI datasheet SLCS006Z); the LM339BIDR (lower input bias) is preferred if JLCPCB stocks it (R7-G G2 topology) | Closed – design |
| R7-C PR-16 | C605 discharges directly into the MCU pin or the TPS2553 FAULT output | R638 1 kΩ in series with D606 | Closed – design |
| R7-C PR-17 | Q609 slow ramp near the EN threshold | Moot with the PR-2 fix (the latch acts first) | Closed – design (moot) |
| R7-C PR-18 | A quick replug may start with the latch cleared | Harmless: the enables are pulled down | Accepted (v1) |
| R7-C PR-19 | Unplug wording; PGOOD latch backstop ineffective | Wording corrected; PGOOD goes bad at VBUS ≈ 3.70 V while 5V_ANA is ≈ 3.6 V, so Q610 turns on | Closed – documentation |
| R7-D DG-1 | CPLD programming port specified as 4-wire JTAG; the AGRV2KQ32 has only JTMS and JTCK (blocker) | J202 = 3V3D (sense), JTCK (25), JTMS (24), NRST (4), GND for 2-wire Compact-JTAG (AGM Supra with a J-Link or AGM DAP-Link); the owner buys the probe | Closed – design; Open – owner action |
| R7-D DG-2 | The clock plan needed three global clock inputs; only PIN_1 is a dedicated clock pin | Family clock on PIN_1 → PLL; 80 MHz input removed (R209 deleted); SCK through ALTCLKCTRL; FIFO written in the SPI domain | Closed – design |
| R7-D DG-3 | MCU output fights the Q611 kill FET (= P-1, PR-1) | R307 10 kΩ | Closed – design |
| R7-D DG-4 | No defined way to power the CPLD for programming | Programming mode: PROG pad (TP732, PC13) at reset powers 3V3D without USB configuration; the probe senses 3V3D only | Closed – design |
| R7-D DG-5 | The < 5 µs watchdog needs a dedicated ADC (= P-18) | ADC1 continuous on VBUS_SENSE with the analog watchdog; ISR on a fast vector | Closed – firmware rule |
| R7-D DG-6 | MCLK can back-power VCCA after a latch trip | ISR drops AUD_EN and RESETB first; MCLK_EN stops X201 before AVCC goes off | Closed – firmware rule + design |
| R7-D DG-7 | Missing DAC register stop_div | Register 10 stop_div = 0 above 96 kHz; register 1 = 0x80; both in the unit tests | Closed – firmware rule |
| R7-D DG-8 | Idle levels of the MCU → CPLD link conflict with the 3V3D-off rule | LINK_FRAME active-high (idle low), CPOL = 0, outputs low or input while AUD_EN is low; R224/R225 33 Ω; CPLD weak pull-downs | Closed – design |
| R7-D DG-9 | MCU internal pulls on the latch-node inputs (= PR-7) | Floating inputs, also in low-power code; ANA_FAULT open-drain low only | Closed – firmware rule |
| R7-D DG-10 | Suspend path unproven on the CH32V307 | Stage 2 gate: Stop with LPDS, PHY suspended, wake on resume, HSE/PLL restart within 10 ms; fallback EXTI wake | Open – verify on Stage 2 |
| R7-D DG-11 | One PLL must lock at 22.5792 and 24.576 MHz | Stage 2 gate; fallback BCLK ÷ 1 from X202/X203 | Open – verify on Stage 2 |
| R7-D DG-12 | SPI clock quantised; 36 MHz is the only workable setting | 36 MHz (68 % utilised at 384 kHz); reads at ≤ 18 MHz; second lane reserved | Open – verify on Stage 2 |
| R7-D DG-13 | CPLD supply isolation | FB202 branch for VDD33/VDDA33; C213–C216 plus C220 1 µF at VDDA33 | Closed – design |
| R7-D DG-14 | J201 has no NRST or BOOT0 pad | J201 = 3V3 (sense), SWDIO, SWCLK, NRST, GND; BOOT0 and USART1 TX pads | Closed – design |
| R7-D DG-15 | X201 load and duty margin | CPLD branch removed; X201 drives only the DAC XI (≈ 10 pF + trace, ≤ 20 mm); duty measured at XI | Closed – design |
| R7-D DG-16 | DAC pin details not written down | ADDR to DGND through R306 (0x48, written 0x90), XO open, DATA_CLK/DATA1/DATA2 = BCLK/LRCLK/SDATA, GPIO2 → DAC_LOCK | Closed – documentation |
| R7-D DG-17 | RESETB injection at unplug or brownout | R308 1 kΩ in series; RESETB dropped first in the ISRs | Closed – design |
| R7-D DG-18 | RCT6 supply-pin count; VBAT not tied | VBAT to 3V3M; one 100 nF per supply pin plus extras at VDD_2/VDD_4; C221 1 µF at VDDA | Closed – design |
| R7-D DG-19 | Link word order | 8-bit frames or a halfword swap in the CPLD; known-pattern test; MISO returns the FIFO level | Closed – documentation |
| R7-D DG-20 | Descriptor details not stated | Feedback wMaxPacketSize 4; isochronous OUT ≥ 392; non-integer 44.1 k-family feedback; bcdDevice bumps; VID/PID 1209:0001 for private testing only | Closed – documentation |
| R7-D DG-21 | The CPLD is AG32 silicon in pure-CPLD mode; the toolchain is Windows-centred | Dynamic current measured on Stage 2; CPLD CI builds on a Windows runner or Supra on Linux | Open – verify |
| R7-E BM-1 | OPA2210 supply and price rest on a 12-month-old JLCPCB stock figure (blocker until verified) | Owner checks the JLCPCB stock now (buy 12 if ≥ 12); OPA1612AIDR fallback designed in (V+ 3.90 V); cost sensitivities stated | Open – owner action |
| R7-E BM-2 | J701 JLCPCB stock is 21, not 721 (= AN-13) | Buy 7 into the JLCPCB library | Open – owner action |
| R7-E BM-3 | Wrong MPN on the 2.2 Ω line (= P-3, AN-8) | 0805W8F220KT5E | Closed – documentation |
| R7-E BM-4 | ME6216A13M3G package and pinout (= P-9) | SOT23-3 (“M3”), pinout and 250 mA recorded | Closed – documentation |
| R7-E BM-5 | Cost-model prices stale and too low for some passives | Re-priced at LCSC on 24 Sep 2026; 1,691 NTD per board | Closed – documentation |
| R7-E BM-6 | J702 flagged “assembly difficulty high” by JLCPCB | Allowed in Economic PCBA; PJ-332-6A (C962626) not qualified; kept and noted | Accepted (v1) |
| R7-E BM-7 | Mixed price basis (LCSC and JLCPCB) | Noted; JLCPCB-vs-LCSC differences ≈ −24 NTD per board (indicative) | Info |
| R7-E BM-8 | C15849 is rated 50 V, not 25 V | Corrected | Closed – documentation |
| R7-E BM-9 | 2.2 µF 0402 6.3 V at 3.3 V loses capacitance | Optional 2.2 µF 16 V 0603 (C23630) for C303/C304 not adopted | Not adopted (optional) |
| R7-E BM-10 | Green LED current about 3× lower than stated | R701 2.2 kΩ → 1 kΩ (R702 stays 2.2 kΩ) | Closed – design |
| R7-E BM-11 | No input-voltage margin on the LP5907 and LM27762 above 5.5 V | Accepted for v1; VBUS measured on the chargers used | Accepted (v1) |
| R7-E BM-12 | Counts and stale figures in the documents | Counts corrected (152 BOM lines, 263 placed parts, 20 Extended types) and stock figures refreshed | Closed – documentation |
| R7-E BM-13 | Fee rules vs JLCPCB’s Economic PCBA table (08 Sep 2026) | All match | Info |
| R7-E BM-14 | Economic PCBA limits | All met (single-sided placement, 2–50 pcs, standard stack-up) | Info |
| R7-E BM-15 | Part ratings versus use | All check out | Info |
| R6-B2 (sixth round) | Residual DC blind region below the detection tiers is a product-safety decision | Residual below 0.2 V per leg (≤ 2.5 mW into 16 Ω, ≤ 1.25 mW into 32 Ω) accepted by the owner on 24 Sep 2026 | Closed – owner decision |

## C.2 Seventh round, external reviews (R7-F, R7-G, R7-H)

Finding IDs: R7-F = external review 1 (F1–F6), R7-G = external review 2 (G1 negative ADC range, G2 BAV99 threshold, G3 owner decision, G4 release conditions), R7-H = external review 3 (H1–H8 plus minors). The three reviews covered the previous set (Spec v0.7, Schematic Design Notes v0.6 and Parts List v0.5), so many items were already fixed in the R7-A…R7-E round (Section C.1); the new design changes of the amendment of 24 Sep 2026 are the LM339 window detector, the redesigned slow DC-sense network, the firmware tiers per leg and per balanced pair, C421 33 pF with the DAC-pin analysis (and, by owner decision of the same day, the fitted DAC-pin clamps D405–D408), the adaptive AMP_EN at power-down and the stream DC guard. The C.1 rows they supersede (R7-B AN-10, R7-C PR-3, PR-11, PR-12 and PR-15) are marked there.

| ID | Finding | Disposition in the v0.8 amendment | Status |
| --- | --- | --- | --- |
| R7-F F1 / R7-H H1 | R623/C607 and R617/C606 load RAILS_OK_N (R505 100 kΩ): the node steps to ≈ 1.57 V, Q610 capture up to 15–19 ms, not 35 µs | Already fixed in R7-A P-2 / R7-C PR-2, using both of R7-H’s options: R505 10 kΩ to 5V_ANA (TI: “connect a 10-kΩ pullup resistor (RPU) to VIN”), R623 1 MΩ, C607 10 nF. With the real loads RAILS_OK_N steps to 4.31–5.15 V (0.98 × 5V_ANA) and crosses 2.5 V in ≈ 1.0–1.3 µs; the ≈ 35 µs capture holds. New rule: RAILS_OK_N carries only ≥ 1 MΩ RC branches and high-impedance gate/MCU inputs (R522). The pulse test emulates the real open-drain release, never a low-impedance generator | Closed – design, verify |
| R7-F F2 / R7-H 2 | AVCC_EN has no series resistor | Already fixed: R307 10 kΩ (R7-A P-1) | Closed – design |
| R7-F F3 / R7-G G1 / R7-H 3 | Slow DC-sense network: 333 kΩ source vs the 50 kΩ ADC limit and 1 µA pin leakage (1 V leg error); the node reaches −0.18 V, outside the ADC range | Redesigned: 15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF, source 4.7 kΩ, node 0.29–2.85 V at every corner, leakage error ≤ 15 mV at the leg, full error budget (≤ 44 mV per leg, ≤ 88 mV per pair), auto-zero, 3V3A monitor | Closed – design, verify |
| R7-F F4 | VREF tracking does not prove DAC-pin safety in a loaded failed-output shutdown; the test measures VREF, not the DAC pins; “below junction conduction” is not an ESS allowance | Analysed: with a stuck leg the I/V positive common-mode margin is gone 0.11–0.39 ms after the kill while 3V3A needs 3.2–5.6 ms to fall below 0.3 V, so the DAC pins are uncontrolled for up to ≈ 5.5 ms (≈ −0.6 V with ≤ 3.9 mA, or ≈ 0.33 V above AVCC); stated as a residual risk of an already-failed output stage. Unplug excursion reduced from ≤ 0.13 V to ≤ 15 mV (C421 330 pF → 33 pF). BAT54S clamps D405–D408 (DNF footprints at first; fitted by owner decision of 24 Sep 2026): each DAC pin held within ≈ −0.40 V…AVCC + 0.40 V (≤ 4.4 mA per pin). Tests probe the four DAC pins (R423–R426 DAC-side pads) against AVCC_L/AVCC_R during failed-output emulation and unplug (pass: within −0.4 V…AVCC + 0.4 V) | Closed – design (clamps fitted, owner decision 24 Sep 2026), verify; small residual: ≤ 0.4 V beyond the rails for ≤ 5.5 ms after an output-stage failure |
| R7-F F5 | Residual-DC acceptance used 0.2 V across the headphone, but the thresholds are per leg: opposite-sign legs give 0.4 V across a balanced driver (4× the power) | Firmware tiers now act on every leg and on each balanced pair difference (L+ − L−, R+ − R−); T4 0.11 V nominal, worst-case trip ≤ 0.198 V differential after the full error budget, so the accepted residual ≤ 2.5 mW (16 Ω) / 1.25 mW (32 Ω) holds for balanced headphones too | Closed – firmware rule, verify |
| R7-F F6 | The classifier can mistake bass for DC (instantaneous 33 ms-filtered reading) | Already changed (R7-C PR-8) to the pre-trip rolling 0.5-s average plus event history; added: events that match neither pattern are logged as “ambiguous” and counted toward the DC three-strike limit (conservative, deliberate) | Closed – firmware rule |
| R7-G G2 | The hardware worst case relies on an extrapolated BAV99 VF at ≈ 0.4 µA (the datasheet guarantees only 1 mA) | Topology changed: two TI LM339DR quad comparators, one window per leg directly on its filter node, no diode-OR, so the threshold is set only by the divider, the comparator offset and the bias current, all datasheet-guaranteed. Leg trip 0.43–0.67 V worst case; 1 V ≤ 125 ms, 2 V ≤ 47 ms; rail-stuck energy ≤ 21.4 mJ (16 Ω) / 11.0 mJ (32 Ω) | Closed – design |
| R7-G G3 | Owner decision on the blind region; suggests testing 0.1 V / 5 s | Owner accepted (24 Sep 2026). T4 is 0.11 V nominal (≤ 0.198 V guaranteed, 2-s window, ≤ 3 s), effectively the suggested 0.1 V; the worst-case band is stated | Closed – owner decision |
| R7-G G4 | Release conditions: ME6216/LM27762 thermal data, OPA2210 SPICE, Stage 1/2 gates | ME6216 Pd 0.54 W (datasheet) vs 0.12 W and LM27762 RθJA 62.2 °C/W (datasheet, ≈ +24 °C at 0.39 W): closed. OPA2210 stability SPICE: open, before layout. Stage 1 is optional (owner constraint); its gates run on Stage 2 | Partly closed |
| R7-H H2 | Legs live while the DAC powers up or down: unbounded DAC transients reach the jack | Covered by R7-B AN-3: the DAC-reset step is measured; firmware options A (legs first), A′ (AUD_EN together with AVCC_EN, allowed by ESS: DVCC with or after VCCA) and B (DAC first, legs last), chosen by the pop measured on main board 1 (Stage 2 has no output stage) | Open – verify |
| R7-H H3 | VBUS watchdog latency in a 7-channel scan | Already fixed (R7-D DG-5): ADC1 converts VBUS_SENSE continuously; the scanned channels are on ADC2 | Closed – design |
| R7-H H4 | LM27762 PGOOD state while unpowered | Moot: PGOOD now pulled up to 5V_ANA (TI recommendation) | Closed – design |
| R7-H minor (power-down) | Power-down pop: AMP_EN goes low while 3V3A ≈ 0.24 V | AMP_EN now drops when the 3V3A monitor reads < 0.1 V (typically t0 + 4.7 ms, pop ≈ 4.5 mV at 300 Ω including the sense-bias path), with a deadline at t0 + 5.8 ms (worst-case discharge: 3V3A ≈ 0.28 V, pop ≈ 12.7 mV at 300 Ω, measured); suspend level typically ≈ 8.2 ms, at worst 9.3 ms after the last bus activity | Closed – design, verify |
| R7-H H5 | Step 4 assumes pair-symmetric DAC pins with DVCC off | Same as H2 / AN-3; test added: I/V outputs and legs scoped during the AVCC ramp with RESETB low; fallback A′ | Open – verify |
| R7-H H6 | The suspend budget omits the LEDs | Firmware rule: both LEDs off in suspend (0 mA in the budget) | Closed – firmware rule |
| R7-H H7 | Step 2 wording; resume latency | Wording already “≥ 600 ms after ANA_EN or after the rails last went off”; resume-to-audio ≈ 1.0 s (0.6 s blanking charge, rails, 250 ms, DAC init, 105 ms soft start) stated | Closed – documentation |
| R7-H H8 | DC in the program material trips the board | New firmware stream DC guard in the MCU before the DAC: no zero crossing for 25 ms and ¦mean¦ ≥ 0.25 FS → output zeroed at once; 0.2-s mean ≥ −30 dBFS → soft mute, unmute after 1 s below −40 dBFS. DC in the program material therefore mutes (reported to the host log) instead of latching | Closed – firmware rule |
| R7-H envelope | The owner decision should cover the whole sub-hardware-trip envelope | Envelope stated per tier (Section 5.3), all ≤ 28.2 mJ at 16 Ω and ≤ 14.1 mJ at 32 Ω | Closed – documentation |
| R7-H minors | C211 note “≈ 38 µC”; X201 “208 ×”; C605 discharge through Q610 | C211 note ≈ 42.5 µC; “≈ 208 ×”; C605 discharges through Q610 in a sub-µs ≈ 0.5 A pulse, fine for a 2N7002, so no series resistor is added there | Closed – documentation |
| R7-F cost | Workbook consistent at 8,248 NTD (v0.5) | 8,487 NTD / 1,697 NTD per board after these changes; now 8,594 NTD / 1,719 NTD per board with the DAC-pin clamps fitted (Section 6) | Info |

## C.3 Sixth review round (R6-A, R6-B, R6-C and R6-D of Spec v0.6)

Finding IDs: R6-A = review 1 (latch timing, power-down, suspend, fault test, cause capture, cost gate), R6-B = review 2 (worst-case protection, blind region, suspend wording, fixture in the gate), R6-C = review 3 (3-strike lockout, mux inputs, OPA2210 single source, minors), R6-D = review 4 (pop vs offset, latch on controlled power-down, LDO dissipation, DAC centre voltage, OPA2210 output swing, watchdog timing, rail floating, 3V3D bulk, minors). The reviews covered Spec v0.6, Schematic Design Notes v0.5 and Parts List v0.4; findings raised by more than one reviewer are dispositioned together. Kept as history: the dispositions as issued in v0.7; R6-B2 is closed in Section C.1 (residual below 0.2 V accepted by the owner on 24 Sep 2026), and where the seventh round changed a value, Sections C.1 and C.2 and the body of this specification are current.

| ID | Finding | Disposition in v0.7 | Status |
| --- | --- | --- | --- |
| R6-A1 | Latch can miss short PGOOD faults; latch delay missing from the energy figures | C605 moved to node F (power-on set and ≥ 1.5 ms set-request stretch); 47 kΩ latch pull-ups; capture ≈ 35 µs (budget 0.1 ms); Q609 slowed to a static interlock (6–32 ms), so a PGOOD glitch either latches or does nothing; energy computed through the rail-disable instant | Closed – design, verify |
| R6-A2 | Power-down does not establish safe DAC-pin voltage or the post-trip energy bound | VREF tracks AVCC (C421 330 pF; D401, C422 removed); DAC analog off first with the I/V and legs powered, rails last; latch also kills AVCC_EN (Q611); after-trip bound 0.51 mJ (both rails, C +10 %, maximum rails) + ≤ 1 µJ back-power → ≤ 0.6 mJ; unplug case bench-checked | Closed – design, verify |
| R6-A3 | ≥ 50 ms shutdown conflicts with USB suspend timing | Power-down ≤ 4 ms after the suspend interrupt, suspend level ≈ 7 ms after the last bus activity; suspend is a v1 requirement; Stage 2 carries the real regulators with dummy loads and measures current vs time | Closed – design, verify |
| R6-A4 | Hardware fault test tests detection, not removal | Detector test kept; failed-output emulation from the board’s own rail through an external switch with dummy loads, V²/R integrated; FLT high; no +20 % allowance | Closed |
| R6-A5 | PGOOD read after the trip cannot identify the cause | Per-leg DC readings (independent ADC path) and the firmware-trip flag stored first at the interrupt; ≥ 0.3 V → DC fault; unknown → DC fault | Closed – design, verify |
| R6-A (cost) / R6-B4 | Fixture fee outside the ceiling | Gate covers every JLCPCB line incl. any fixture; gated subtotal and total payable recorded; the fixture sentence is standard page text | Closed |
| R6-B1 | Protection limits are nominal | Full worst case (R, C incl. DC bias, LM393 offset, BAV99 VF, rails); R601–R604 100 kΩ, R609/R610 10 kΩ → 2 V ≤ 59 ms (41 % margin), 1 V ≤ 183 ms (27 %); energy 26.2 / 13.3 mJ worst ≤ 30 / 15 mJ | Closed – design, verify |
| R6-B2 | 0.755 V blind region is a product-safety decision | Firmware slow tier on every leg (≥ 0.8 V in 1 s, ≥ 0.2 V in 3 s) sets the same latch; residual < 0.2 V (≤ 2.5 mW into 16 Ω) | Open – owner decision |
| R6-B3 | Suspend wording contradicts the gate | Suspend ≤ 2.5 mA is a v1 requirement everywhere | Closed |
| R6-B (cost) / R6-C3 | OPA2210 single source decides the ceiling | No qualified second source (OPA2209AIDR stock 0; OPA1612 fails the common-mode range at the minimum rail); reserve 12; fallback LCSC price | Open – tracked |
| R6-C1 | Default-mode overloads can lock the board off | Overloads count toward the 3-strike stop only at the −12 dB ceiling; DC faults counted | Closed |
| R6-C2 | Mux inputs float with U205 fitted | DNF R216, R217 10 kΩ pull-downs fitted with U205 | Closed |
| R6-C (minor) | Clipping before a trip near the Default limit; link framing | Scope test added; SPI with chip-select framing | Closed |
| R6-D1 | Pop target conflicts with the offset limit | Pop defined as the band-limited transient beyond the settled DC step; legs enabled/disabled while VREF = 0; SPICE item for the disabled-state output | Closed – verify |
| R6-D2 | Every controlled power-down sets the latch | Expected and masked by firmware (not logged, counted or applied) | Closed |
| R6-D3 | LDO dissipation not analysed | Dissipation table; R519 + R520 dropper keeps U504 ≤ 0.20 W; copper and IR check | Closed – verify |
| R6-D4 | Two DAC centre voltages | Both ESS values stated; offset 8.6 mV worst if 1.65 V; R433 0 Ω option | Closed – verify |
| R6-D5 | OPA2210 output swing not quantified | ≈ (V+) − 0.3 V at ≈ 3 mA; 0.43 V margin calibrated; −3.5 dB pre-calibration default is a safety setting; SPICE | Closed – verify |
| R6-D6 | VBUS watchdog timing margin | Quantified (≈ 1 µs margin at 0.47 A); PGOOD latch as backstop; latency measured on Stage 2 | Closed – verify |
| R6-D7 | Positive rail can float during the rails-off window | Window removed by the new order; the latch trip collapses AVCC in hardware | Closed |
| R6-D8 | 3V3D bulk capacitance | C512 10 µF at U504 | Closed |
| R6-D (minor) | ≈ 208 ×; attach charge; test-pad table; node F margin; BCLK edge; DR loopback floor; A-M2 bracket (Spec v0.6 Section C.2, now C.5) | All updated as listed in this round | Closed |

## C.4 Fifth review round (R5-A, R5-B and R5-C of the amended Spec v0.5)

Finding IDs: R5-A = review 1, R5-B = review 2, R5-C = review 3. Findings raised by more than one reviewer are dispositioned together. Kept as history: the dispositions as issued in v0.6; where the sixth round changed a value, the v0.7 value is added in brackets and Sections C.1 to C.3 apply.

| ID | Finding | Disposition in v0.6 | Status |
| --- | --- | --- | --- |
| R5-A1 / R5-B1 | Spec §4 Key-parts table showed placeholder tokens | The saved v0.5 table was repaired during the amendment (the review saw an interim copy); v0.6 table rebuilt and the saved file scanned for tokens; CI document-lint check added | Closed |
| R5-A2 / R5-B6 / R5-C1 | Shared oscillator outputs need break-before-make; X203 enabled when OE is open | Both enables 2.2 kΩ pull-downs (off at power-up and during CPLD configuration); node pull-down R214; enables mutually exclusive in CPLD logic; sequence with ≥ 1 ms both-off, ≥ 10 ms start-up, PLL held in reset; SCTF/YXC datasheet facts cited (no disable time specified → measured); DNF mux U205/R215 fitted on one Stage 2 board | Closed – design, verify |
| R5-A3 / R5-B2 | 100 MHz MCLK at or above the DAC limit; “worst duty” wording | 80 MHz SCTF SX3M80 (12.5 ns cycle, ≥ 5.6 ns pulse, ≈ 208 × 384 kHz); “ideal duty 50 %”; all 8 rates stay in the gate | Closed – design, verify |
| R5-A4 | 0.473 A is an estimate, not a whole-board limit; CC changes not monitored | “Estimated maximum with hardware-limited analog current”, now 0.469 A; Stage 2 measurements before layout; CC monitoring within tSinkAdj | Closed – design, verify |
| R5-A5 | OPA2197 input-pair transition region at VREF | OPA2210 (single input pair, 0.47 V common-mode margin, 35 µV, 2.2 nV/√Hz) | Closed |
| R5-A6 | Shutdown back-powering; AVCC can collapse faster than VREF | ES9018K2M abs. max cited; rails-first-off power-down; watchdog also drops RAIL_EN; VREF clamp D401 and C421 100 nF [v0.7: VREF tracks AVCC with C421 330 pF, D401 and C422 removed, DAC analog off first; see R6-A2] | Closed – design, verify |
| R5-A7 / R5-B3 | Overload restarts when PGOOD recovers; overload does not latch | Q610 (PGOOD bad) and D606 (TPS2553 FAULT) set the latch; firmware clear required; ≥ 1 s recovery, ceiling steps, 3-strike stop [v0.7: overloads count toward the stop only at the −12 dB ceiling; see R6-C1] | Closed – design, verify |
| R5-A8 / R5-B5 | Protection envelope and terminal energy | Envelope by DC magnitude (t(V)); energies recalculated as V²/R × t for digital DC and a rail-stuck failure; both rails 0.46 mJ; hardware fault-injection test added [v0.7: worst-case envelope, 26.2 / 13.3 mJ at 16 / 32 Ω including ≤ 0.6 mJ after the trip; failed-output emulation without allowance; see R6-B1, R6-A2 and R6-A4] | Closed – design, verify |
| R5-A9 | Divider resistor tolerance omitted | Included; R518 5.1 kΩ (−3.722 V); design-corner margin 0.11 V | Closed |
| R5-A10 / R5-C5 | Balanced DR advantage assumes uncorrelated noise | Correlated model: balanced ≈ 120.3 vs unbalanced ≈ 119.5 dB(A) (+0.85 dB) | Closed |
| R5-A11 | QA403 cannot measure DC offset | DC-capable DMM for the 100 % offset screening | Closed |
| R5-A12 | External MCLK contention at the test header | J703 MCLK is a monitor output via R703 1 kΩ; external source drives BCLK/LRCLK/SDATA only | Closed |
| R5-A13 / R5-B9 | 1,700 NTD called “owner-approved”; ceiling policy | Owner statement recorded (budget flexible as long as it does not surge); 1,700 kept as the hard order gate; the JLCPCB quote must pass; exclusions listed; estimate now 1,647 [v0.7: 1,650; the gate covers every JLCPCB line] | Closed |
| R5-A14 | “Every class verified” wording; fixture fee not included | X202 exception stated; possible fixture fee (X202 and OPA2210 pages) listed as excluded, to confirm in the quote [v0.7: the fixture sentence is standard page text; any fixture line counts toward the gate; see R6-A (cost) / R6-B4] | Closed |
| R5-B4 | USB compliance only at bring-up | Stage 2 mandatory; unconfigured, configured and suspend currents measured before layout; layout blocked unless all pass | Closed – design, verify |
| R5-B7 | “Exact sample rates” | “Exact integer clock ratios”; absolute rate within the oscillator tolerance, absorbed by the asynchronous feedback | Closed |
| R5-B8 | Stage 2 should be mandatory | Mandatory, ≈ US$79 ≈ 2,500 NTD for two boards (outside the board budget) [v0.7: ≈ US$90 ≈ 2,860 NTD with the real regulators and dummy loads] | Closed |
| R5-C2 / R5-C3 | Close the X203 datasheet item; “only one runs” wording; PLL ×2 justification | Datasheet facts cited; “only one drives the node”; idle-trace rule; PLL ×2 also fixes the 45–55 % oscillator duty | Closed |
| R5-C4 | Default-mode power from average current | Instantaneous-peak method; new table and targets | Closed |
| R5-C6 | Blanking 28–119 ms vs 100 ms wait | 25–133 ms incl. R/C tolerance; AMP_EN after PGOOD good ≥ 150 ms; FLT_CLR after ≥ 200 ms rails off | Closed |
| R5-C7 | 0.1 V margin on the 4.30 V VBUS watchdog | 4.10 V (3.95–4.25 V), ≥ 0.15 V margin to the 4.40 V stress corner | Closed |
| R5-C8 | K1000 swing margin ≈ 0.06 dB | With resistor tolerance the worst unit reaches the swing limit 0.27 dB below full scale at 120 Ω balanced; rated as swing-limited; bench-verified | Closed – verify |
| R5-C9 | Continuous CC monitoring | Firmware requirement (≤ 5 ms sampling, tRpValueChange debounce, tSinkAdj response) | Closed – design, verify |
| R5-C10 | MCU link sampled by the system clock | Link captured in its own clock domain with a word toggle synchronizer [v0.7: SPI with chip-select framing] | Closed – design, verify |
| R5-C11 | Versioning and stale rows | New files Spec v0.6 / Notes v0.5 / Parts List v0.4; Spec history notes that v0.5 first used the interim divider; Notes earlier-round rows R1-B1 (7.7 µF) and R2-M5 (1,550 / 1,536) marked as the design at that time with the current values [v0.7: Spec v0.7 / Notes v0.6 / Parts List v0.5] | Closed |

## C.5 Fourth review round (reviews R4-A, R4-B and R4-C of Spec v0.4)

Finding IDs: A- = R4-A, B- = R4-B, C- = R4-C. Kept as history: the dispositions as issued in v0.5. Where the fifth or sixth round changed the design (for example the MCLK, the I/V op-amp, the rails, the USB estimate, the protection and the cost), Sections C.1 to C.4 and the body of this specification are current; where a row still quotes an older number, the current value is added in brackets.

| ID | Finding | Disposition in v0.5 | Status |
| --- | --- | --- | --- |
| A-H1 | FB101 mis-specified (GZ1608D601TF = 200 mA / 450 mΩ, C1002; C1015 is GZ2012D101TF) | Power ferrite now Murata BLM18PG121SN1D (C14709, Basic, 2 A, 50 mΩ) in the 5V_ANA path; GZ1608D601TF (C1002) only on DVCC (≈ 14 mA); drops recomputed (≈ 15–25 mV) | Closed |
| A-H2 | Offset used OPA1678 typical as maximum | Confirmed (±0.5 typ / ±2 mV max); I/V op-amp now OPA2197 (±0.25 mV max); budget 7.9 / 15.9 mV worst, balanced limit ±20 mV, 100 % screening; DAC-mismatch assumption Open (measure) [v0.7: OPA2210, 7.0 / 14.0 mV worst] | Closed |
| A-H3 | Safe disconnected output not established | Full sequence and current paths, rails first on/last off, PGOOD hardware kill, back-powering bounded (≤ 1.7 mA/pin) and removed in 10 ms, energy bound ≤ 16.5 mJ before / ≤ 0.22 mJ after the trip, protection defined at the headphone terminals [v0.7: legs on before AVCC, DAC analog off first; ≤ 30 / 15 mJ worst case at 16 / 32 Ω including ≤ 0.6 mJ after the trip] | Closed – design, verify |
| A-H4 | Negative-rail headroom | Rails ±3.78 V; both rails budgeted at 4.60 V and maximum load with a 3.0 Ω charge-pump resistance (margin 0.17 V) [v0.7: +3.78 V / −3.72 V, margin 0.11 V] | Closed |
| A-H5 | 100 mA ceiling not enforced | Clean envelope (80 mA/leg, swing curve) separated from overload; TPS2553 enforces the analog supply current; PGOOD kill; firmware ceiling steps; overload dissipation bounded; overload tests replace the 30-min 32 Ω balanced test | Closed – design, verify |
| A-H6 | USB power states and descriptors | State-by-state budget, MCU-only before configuration (≤ 83 mA estimate), descriptors 0x80/0xFA, CC detection by ADC for High mode, suspend policy (2.5 mA Open); verified in the firmware gate [v0.7: suspend ≤ 2.5 mA within 10 ms is a v1 requirement] | Closed – design, verify |
| A-M1 | Mixed purchasing lines and placeholder ranges | One part per line, every value specified, 48 purchasing lines by LCSC number [v0.7: 47 purchasing lines] | Closed |
| A-M2 | 13.8 NTD ceiling margin; one Basic→Extended flip fails it | Every class now verified on the JLCPCB page (X202 assumed Extended); Extended count conservative (19, incl. THT jack and X202); 34 NTD under the 1,700 NTD ceiling [v0.7: 50 NTD under]; still a planning estimate until the JLCPCB quote | Closed – verify |
| A-M3 | Gain tolerance defeats the minimum power targets | Per-unit calibration to 1.75 Vrms/leg; targets defined at the calibrated level | Closed |
| A-M4 | Two LDOs do not guarantee sequencing | Separate enables (LP5907 EN, ME6211 CE) in a firmware sequence; DVDD LDO fed from 3V3D so DVDD ≤ DVCC by construction; scope check at bring-up | Closed – design, verify |
| A-M5 | Clock adaptation needs an earlier gate | Three oscillators: 100 MHz straight to the DAC MCLK (asynchronous mode) and the CPLD system clock [v0.7: 80 MHz]; 24.576 MHz or 22.5792 MHz selected in front of the CPLD PLL (×2) for the two families (exact ratios); pre-layout firmware gate as a two-stage proof of concept | Closed – design, verify |
| B-S1 | −4.5 V rail cannot regulate from USB | As A-H4 | Closed |
| B-S2 | 2.0 Vrms/leg does not fit the supply; +1 dB units clip | Calibrated 1.75 Vrms/leg; swing model from OPA1622 Fig. 38/39; pre-calibration trim −3.5 dB | Closed |
| B-S3 | Power targets fail on low-spread units; rating method | As A-M3: “1 % THD+N or calibrated 0 dBFS, whichever first” | Closed |
| B-S4 | Firmware needs much more than the spec said | Own UAC2 firmware and CPLD logic, modules listed (Section 8); two-stage proof of concept before layout; 352.8 kHz from the 22.5792 MHz oscillator through the CPLD PLL (exact) | Closed – verify |
| B-S5 | 14 NTD margin will be used up (odd values, 1.5 nF C0G, crystal, AMS1117, THT jack fee) | Basic E24 values or Basic combinations used (100 k + 47 k, 1 k ∥ 5.1 k, 15 k + 1 k, 2 × 330 pF), Basic 5032 crystal, AMS1117 removed, THT fee counted; no cheaper 4.4 mm jack exists at LCSC (SMD option not available) | Closed – verify |
| B-M1 | Pop at unplug has no hardware control | PGOOD gates AMP_EN in hardware; VBUS watchdog; separate unplug pop target (≤ 20 mV) | Closed – design, verify |
| B-M2 | Fault latch re-trips on clear | Set path blanked ≈ 60 ms after PGOOD good, without clearing the latch [v0.7: 25–133 ms] | Closed |
| B-M3 | PLL jitter goes to the DAC in sync mode | DAC still in asynchronous mode with no PLL in the MCLK path (MCLK straight from the 100 MHz oscillator [v0.7: 80 MHz]); the CPLD PLL is only in the bit-clock path of both families, whose jitter the DAC’s DPLL rejects; J-test stays in the plan | Closed |
| B-M4 | > 100 mA before enumeration; phones limit OTG | As A-H6; Default mode ≤ 0.473 A covers phones [v0.7: 0.469 A] | Closed |
| B-M5 | AMS1117-ADJ needs tantalum-type capacitor and minimum load | Fixed 1.3 V ME6216A13M3G (±1 %, 300 mA, ceramic-stable) | Closed |
| B-minor | Duplicate “v0.4” rows in Appendix B; U301 “voltage-mode” text; HSE 8 MHz; VREF risk overstated | History cleaned (this is v0.5); U301 text corrected; HSE 8 MHz confirmed; VREF risk downgraded and quantified (1.17 mV) | Closed |
| C-B1 | USB power before configuration | As A-H6 | Closed – verify |
| C-B2 | Reference descriptors wrong (self-powered, 100 mA) | 0x80 / 0xFA in the firmware gate | Closed – verify |
| C-B3 | 352.8 kHz MCLK not supported by the reference CPLD | Confirmed in source; replaced by our own CPLD logic with the family oscillator selected before the PLL, as the reviewer suggested (X202 24.576 MHz or X203 22.5792 MHz, PLL ×2); all eight rates exact; MCLK 100 MHz straight to the DAC (asynchronous mode) [v0.7: 80 MHz] | Closed – design, verify |
| C-B4 | Clock-family switching can glitch | MCLK never switches; the family changes by the oscillator enables while the I²S output is stopped and the PLL is held in reset; no clock multiplexer in the fabric; scope check for runt pulses | Closed – design, verify |
| C-B5 | Host volume not implemented | UAC2 volume/mute with GET/SET CUR/RANGE and I²C mapping, tested on three OSes in the gate | Closed – verify |
| C-B6 | “100 mA design current” not implemented | As A-H5 | Closed – design, verify |
| C-H1 | 500 mA budget has no margin | Default mode ≤ 0.473 A enforced (27 mA margin, measured at bring-up) [v0.7: 0.469 A estimated maximum, 31 mA margin]; High mode only with a ≥ 1.5 A Type-C advertisement | Closed |
| C-H2 | +4.5 V corner | Rails ±3.78 V, both rails budgeted | Closed |
| C-H3 | 7.7 µF table is not an inrush proof | Attach charge 38 µC including 3V3M output capacitance [v0.7: ≈ 40 µC]; staggered, soft-started domains | Closed – verify |
| C-H4 | Cost provisional; “Assign LCSC no.” left; “every part is LCSC” inaccurate | Every line has an LCSC number and verified class | Closed |
| C-H5 | Offset margin | As A-H2 | Closed |
| C-H6 | ≤ 5 mV pop ambitious without relays | Split targets, full sequence incl. unplug/brownout validated on the bench | Closed – verify |
| C-H7 | USB identity (WCH VID, PID 0) | Use the pid.codes VID 0x1209 with an assigned PID; serial number from the MCU unique ID | Closed – verify |

## C.6 Earlier review rounds (three reviews of Spec v0.3)

These findings were dispositioned in v0.4 and updated to the design of v0.5; they are kept as history. Where a disposition quotes a value that v0.6 or v0.7 changed, the current value is added in brackets, and the later findings in Sections C.1 to C.5 apply.

| ID | Finding | Disposition (design at v0.5; later values in brackets) | Status |
| --- | --- | --- | --- |
| R1-B1 | VBUS_PD capacitance ≈ 130 µF vs the USB-C limit of 10 µF | No PD port; single 5 V port; attach charge ≈ 38 µC (see C-H3) (v0.7: ≈ 40 µC) | Closed – N/A |
| R1-B2 | LMR33630 + SMAJ18A surge stacking ≈ 44 V | No inverting converter and no 15 V input; the LM27762 runs from 5 V | Closed – N/A |
| R1-B3 | LP5907 on 1V8_D over 10 µF output capacitance | No 1.8 V rails; the only LP5907 (DAC 3.3 V) is kept within its 10 µF output-capacitance limit | Closed – N/A |
| R1-B4 | Paralleling topology left open | No paralleling; one amplifier per leg with a clean envelope of ≤ 80 mA peak (see A-H5) | Closed |
| R1-B5 | LM339 at −12 V pulling 1.8 V logic | The comparator outputs reach the 3.3 V domain only through a 10 kΩ / 47 kΩ node clamped by a 1N4148WS; the latch uses N-MOSFETs only, and 3.3 V nodes never go below 0 V | Closed |
| R1-B6 | Offset target ±5 mV vs an 11 mV worst case | Worst case recomputed with datasheet maxima (7.9 / 15.9 mV at v0.5; v0.6: 7.1 / 14.3 mV; v0.7: 7.0 / 14.0 mV); limits ±10 / ±20 mV with 100 % screening (see A-H2) | Closed |
| R1-H | AMS1117 dropout and noise | ME6211 for digital 3.3 V; LP5907 for DAC analog 3.3 V; fixed 1.3 V ME6216A13M3G for DVDD (see B-M5) | Closed |
| R1-H | XU316 VOH vs AK4493 VIH | All logic is 3.3 V | Closed – N/A |
| R1-H | Both jacks live; gain-switch pop | Gain switch removed; both jacks live documented (one headphone at a time) | Closed |
| R1-H | OPA1622 stock 197 vs 400 needed | The 5-board sample run needs 10 (2 per board) | Closed |
| R1-H | LM317/LM337 cost trade | LM27762 low-noise LDOs | Closed – N/A |
| R1-H | CI rule “every part has an LCSC number” vs Digi-Key/Mouser parts | Every fitted part has an LCSC number and a JLCPCB class (Parts List v0.3, see C-H4) (v0.6: every class from its JLCPCB part page or the basic-parts list, except X202, assumed Extended; see R5-A14) | Closed |
| R1-H | Firmware port map before layout | Pre-layout firmware gate as a two-stage proof of concept (Section 8, see B-S4); the MCU–CPLD interface and the pin map are frozen before the proof-of-concept board and the schematic | Closed |
| R1 consistency | v0.2 wording, cost source of truth, THD+N conditions, Zout including contacts, DSD pins, grounding/ESD/CC/silkscreen | Fixed in the spec: the single USB-C port removes the silkscreen confusion; CC = 5.1 kΩ Rd; jack ESD LESD5D5.0CT1G; grounding rules added; DSD not in v1 | Closed |
| R2-S1 | TVS does not protect the −14.7 V converter; no OVP | No 15 V input and no inverting converter | Closed – N/A |
| R2-S2 | VBUS bulk before the PD contract; analog rails start at 5 V | No PD contract; single 5 V port | Closed – N/A |
| R2-S3 | Balanced output impedance with option B | The OPA1622 legs drive the jacks directly, no ballast (< 0.1 Ω + contacts) | Closed |
| R2-S4 | K1000 1 W with no margin | Closed by requirement change (budget redesign): ≈ 102 mW and ≈ 94 dB SPL into the K1000 documented at v0.5 (v0.6: 95.9 mW and ≈ 93.8 dB SPL in High mode, 39.2 mW and ≈ 89.9 dB SPL in Default mode) | Closed |
| R2-S5 | Offset target contradicts the design | As R1-B6 | Closed |
| R2-M1 | Board ≈ 7 W in a PLA case | Analog input power bounded by the TPS2553 limit; board ≈ 3.8 W worst case (High mode, overload); overload tests (Section 7) | Closed |
| R2-M2 | LM317 near dropout; LM337 LC filter | Blocks removed (LM27762) | Closed – N/A |
| R2-M3 | ±0.05 dB trim cannot use 0.5 dB DAC steps | Optional digital scaling in the MCU firmware | Closed |
| R2-M4 | Maximum-power criterion | Rated at 1 kHz, both channels, at 1 % THD+N or calibrated 0 dBFS, whichever comes first (see B-S3) | Closed |
| R2-M5 | Cost margin 10 NTD; exchange rate | Gate uses a fixed 31.7 NTD/US$: target 1,500 NTD per board (WARN above) and a 1,700 NTD ceiling (FAIL above); 1,666 at v0.5 (v0.6: 1,647 NTD; v0.7: 1,650 NTD, WARNING; ceiling recorded with the owner statement of 24 Sep 2026, see R5-A13) | Closed |
| R2-M6 | 4.4-to-3.5 mm adapter shorts leg− | One amplifier per package shorted (TI continuous-short rating), ≈ 0.45 W per package; risk and manual warning | Closed |
| R2-M7 | Unbalanced crosstalk needs ≤ 6 mΩ shared ground | Layout rule (≤ 5.7 mΩ) | Closed |
| R2 minor | Stale versions; KC2520Z “(assumed)”; through-hole count; pop requirement; unverified CH224A/AMS1117 claims; NVIDIA paragraph | Versions fixed; the three oscillators are counted as leadless conservatively; only the 4.4 mm jack is through-hole (confirm USB-C shell pegs); pop ≤ 5 mV controlled and ≤ 20 mV at unplug; CH224A removed; unverified regulator claims removed (see B-M5); job-application text moved to the repository README | Closed |
| R3-1 | DC fault latch and recovery | Hardware 3.3 V latch; firmware recovery policy (Section 5.3); status to the MCU | Closed |
| R3-2 | TVS/converter envelope | No 15 V input and no converter | Closed – N/A |
| R3-3 | Stage-1 gain 0.9009 with the input network | The gain includes the DAC’s 806 Ω source (current mode, gain = 2 × Rf/806 per leg); v0.5 uses Rf = 836 Ω and per-unit calibration to 1.75 Vrms per leg (see A-M3) | Closed |
| R3-4 | QA403 cannot drive 1.8 V I²S | I²S is now 3.3 V; header, firmware test mode and sequencing specified | Closed |
| R3-5 | 200 mA “limit” not implemented; thermal at 16 Ω balanced | Clean envelope ≤ 80 mA peak per leg; rated power at 1 % THD+N or calibrated 0 dBFS; overload bounded by the TPS2553 (see A-H5) | Closed |
| R3 consistency | Rails-good qualification; offset/trim; workbook external links; cost/sourcing reconciliation | Rails-good: LM27762 PGOOD gates AMP_EN in hardware and arms the latch ≈ 60 ms after PGOOD good at v0.5 (v0.6: 25–133 ms, see R5-C6). Offset/trim: as R1-B6 and R2-M3. Workbook links: the delivered v0.1 file references only its own sheets (links appear when sheets are copied into another workbook); v0.2 adds a self-check. Cost/sourcing: single source of truth (Parts List v0.3 at v0.5; now Parts List v0.5), all LCSC | Closed |