# USB DAC + Balanced Headphone Amplifier — Schematic Design Notes v0.7

Companion to Design Specification v0.8: datasheet-derived requirements and starting component values for the KiCad schematic after the seventh review round

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 24 Sep 2026 | v0.7 | Schematic design notes (companion to Design Specification v0.8 and Parts List v0.6) |

# 0. Purpose and references

This document collects the datasheet-derived requirements and starting component values for drawing the KiCad schematic of the USB DAC + balanced headphone amplifier. It is the companion to Design Specification v0.8 and Parts List v0.6 and replaces Notes v0.6, which stays unchanged as history. This revision answers the seventh review round, a full design check in five areas before any hardware is bought (R7-A power, R7-B analog, R7-C protection, R7-D digital, R7-E parts and cost; Section 7.2): series resistors on every enable the latch can kill (R307 on AVCC_EN, R521 on AUD_EN), a PGOOD pull-up on 5V_ANA so that the ≈ 35 µs latch capture is genuine, V+ raised to +3.90 V so that the OPA1612 fallback fits, a calibrated level of 1.55 Vrms per leg from the OPA1622 clean swing, a DAC power-down that stops MCLK before AVCC goes off, a 2-wire Compact-JTAG CPLD port with the family clock on PIN_1, proposed MCU and CPLD pin maps and bleeders on 3V3D and 1V3. It was amended on 24 Sep 2026 for three external reviews (R7-F, R7-G, R7-H; Section 7.3): the hardware DC detector is now two LM339 quad comparators with one window per leg directly on its filter node (no diode-OR), the slow DC-sense network is redesigned (15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF, with a 3V3A monitor), four firmware DC tiers act per leg and per balanced pair, the DAC pins in a failed-output shutdown are held near the rails by the BAT54S clamps D405–D408 (C421 now 33 pF), AMP_EN drops adaptively at power-down and the BOM is re-priced (1,719 NTD per board). Following the owner decisions of 24 Sep 2026, the residual DC below the lowest firmware tier is accepted, the DAC-pin clamps D405–D408 are fitted, Stage 1 is optional and the JLCPCB-built Stage 2 board, now carrying the ES9018K2M, is the first hardware. The sixth and earlier review rounds stay as history in Sections 7.4 to 7.7.

The designer owns and verifies every value. Circuit values are starting values unless marked otherwise, to be checked in SPICE and on the bench. Every number is computed from the stated values or taken from the datasheet named next to it. Labels are kept as written: “assumption” marks a value not yet confirmed, “to verify” marks a check still to be done, and “per datasheet — to verify” marks a value that must come from a datasheet not yet read. All costs use a fixed 31.7 NTD per US$.

| Ref | Document | Publisher | Document no. / note |
| --- | --- | --- | --- |
| R1 | CH32V307 datasheet (LQFP-64 pin table used for the proposed MCU pin map, 3.2; I/O leakage maximum not confirmed, 7.1) and reference manual; CH32V307 EVT package (high-speed USB device examples: HID, CDC serial, mass storage, network adapter; no audio example) | WCH | — |
| R2 | AGRV2K datasheet | AGM | Only PIN_1 (IO_GB) is a dedicated clock pin and the PLL input must be on it; 2-wire Compact-JTAG (JTMS pin 24, JTCK pin 25) with AGM’s Supra tool (3.3); I/O state and weak pull-up during configuration still to check (7.1) |
| R3 | ES9018K2M datasheet | ESS | v3.7 (22 Apr 2021); pages 8, 27 and 31 used for the output stage, power-up and MCLK limits; page 30 for the audio-interface timing; “XI / MCLK Timing” (cycle ≥ 10 ns, pulse width ≥ 4.5 ns, duty 45:55 to 55:45) and the absolute maximum “DAC Output voltage Range: GND < Vout < AVCC”; pin table (QFN-28 pin functions; outputs “driven to ground” in reset, 3.4) |
| R4 | OPA1622 datasheet | TI | SBOS727B; Fig. 38/39 (output swing) and the ±5 V distortion data (600 Ω → 11.3 mW, 32 Ω → 150 mW) behind the 1.3 V clean-swing headroom (3.5.2) |
| R5 | LM27762 datasheet | TI | SNVSAF7C; PGOOD pull-up to VIN; θJA 62.2 °C/W (3.6.3) |
| R6 | LP5907 datasheet | TI | Output automatic discharge 230 Ω typical when disabled (3V3A below 0.3 V within 5.5 ms worst case after AVCC_EN goes low, at +50 % and 6.7 µF, 2.5); output capacitance ≤ 10 µF; noise < 6.5 µVrms; absolute maximum VIN 6.0 V |
| R7 | USBAudioToI2S firmware project | GitHub, LoveLonelyTime | Apache-2.0; known-good comparison and reference only (the firmware and CPLD logic are our own); source (audio_cpld.v) reviewed for the clock path; its SPI clock reaches the global network through ALTCLKCTRL, as in 3.3 |
| R8 | JLCPCB PCBA price list | JLCPCB | Economic PCBA fees used for the 5-board cost gate |
| R9 | TPS2553 datasheet | TI | SLVS841F (current-limit equations, Figure 24) |
| R10 | OPA197 datasheet (OPA197, OPA2197, OPA4197) | TI | SBOS737C; table 6.8 and section 7.3.6: input-pair transition region typically (V+) − 3 V to (V+) − 1.5 V, VOS not specified for (V+) − 3.5 V < VCM < (V+) − 1.5 V; the reason the OPA2197 was replaced (3.5.1) |
| R11 | USB Type-C specification | USB-IF | CC voltage bands for the current advertisement |
| R12 | LCSC and JLCPCB part pages | LCSC / JLCPCB | Classes, prices and stock, Jun–Sep 2026, prices refreshed on 24 Sep 2026 (sourcing evidence for Parts List v0.6); the assembly-fixture sentence is standard text on every part page (Section 8) |
| R13 | SCTF 2CM series oscillator datasheet (X203) | SCTF | Manufacturer file “2CM-Series-2024-11-04”: pin 1 “Tri-State”, enabled when OE is logic 1 or open, disabled when logic 0 (≤ 0.3 VDD); start-up ≤ 5 ms; 10 mA max; duty 45–55 %; no disable/enable time, internal pull-up value or disabled-output leakage specified |
| R14 | SCTF 3C series oscillator datasheet (X201, 80 MHz) | SCTF | SX3M80.000B20F30TNN: 3.3 V CMOS, ±20 ppm tolerance / ±30 ppm stability, 40 mA max for 80–160 MHz, duty 45–55 %, tri-state pin 1 (driven by MCLK_EN, 3.3) |
| R15 | OPA210 datasheet (OPA2210) | TI | SBOS924H: single input pair, common-mode range (V−) + 1.5 V to (V+) − 1.5 V, Vos ±35 µV max, 2.2 nV/√Hz, 18 MHz, 2.2 / 2.5 mA per channel, ±2.25 V to ±18 V; output swing (V+) − 0.2 V at RL = 10 kΩ and (V+) − 0.6 V at 600 Ω (±15 V); input current 10 mA (absolute maximum ratings and section 7.3.5) |
| R16 | USB Type-C timing: tSinkAdj 60 ms max, tRpValueChange 10–20 ms | Zephyr Project; Chromium EC | USB Type-C specification timing as quoted in the Zephyr USB-C stack and the Chromium EC documentation; used for the CC monitoring (5.2) |
| R17 | ME6211 datasheet (ME6211C33M5G-N, U502 and U504) | Microne | SOT-23 power dissipation 300 mW (absolute maximum ratings); CE rated ≤ VIN + 0.3 V (R521, 3.6); no output discharge (bleeders R524/R525); used for the LDO dissipation check (3.6.3) |

# 1. Key design decisions

The decisions below fix the v0.7 schematic. The Basis column records why each choice was made and which review finding it answers (finding IDs in Sections 7.2 to 7.7). The clock, firmware, cost, protection, BOM and proof-of-concept rows follow the owner decisions of 24 Sep 2026, including the decision to fit the DAC-pin clamps D405–D408.

| Decision | Choice | Basis |
| --- | --- | --- |
| Cost target | ≤ 1,500 NTD per board target for the 5-board sample run (WARN above); ceiling 1,700 NTD per board plus the owner-approved 21 NTD for the DAC-pin clamps (24 Sep 2026) → order gate 1,721 NTD per board, the hard gate (FAIL above). The gate covers every JLCPCB line of the real quote: PCBs, PCBA fees (including any fixture or other assembly line that appears in the quote) and parts. Outside the gate: shipping, import tax, programmers, enclosure and proof-of-concept hardware | Ceiling 1,700 NTD per board, set when the third oscillator was added; on 24 Sep 2026 the owner confirmed the budget is flexible as long as it does not surge, and decided to fit the DAC-pin clamps D405–D408 knowing they add ≈ 21 NTD per board and take the estimate over the 1,700 NTD ceiling (order gate 1,721 NTD); the actual JLCPCB quote from the real BOM/CPL files must pass the order gate before ordering, and at approval both the gated subtotal and the total payable (gated subtotal + shipping + tax) are recorded (R5-A13, R5-B9, R6-A cost note, R6-B4) |
| Cost status | 1,719 NTD per board (8,594 NTD for 5 boards) | WARNING: 219 NTD over the 1,500 NTD target and 2 NTD under the 1,721 NTD order gate (1,700 NTD ceiling + the owner-approved 21 NTD for the DAC-pin clamps D405–D408), so the margin is very small and the real JLCPCB quote decides; ≈ 1,621 NTD per board without the conservative fees; OPA1612 fallback ≈ 1,725–1,740 NTD per board (above the order gate: owner decision only if the OPA2210 cannot be bought); OPA2210 at the LCSC price ≈ 1,825 NTD, also above the gate (Section 8). Parts List v0.6 (amended) is the source of truth; final with the JLCPCB quote after layout |
| USB audio bridge | WCH CH32V307RCT6 + AGM AGRV2KQ32 CPLD; our own USB Audio Class 2.0 firmware and our own CPLD logic (Section 5) | Owner decision 24 Sep 2026; USBAudioToI2S (R7) is kept only as a known-good comparison. Stage 1 (desk setup on bought development boards) is optional, because development boards are hard to get in Taiwan (owner, 24 Sep 2026); the JLCPCB-built Stage 2 board is the first hardware and gates the layout (R4-B S4; R4-C B3–B5, H7; R5-B4, R5-B8) |
| Clock | DAC MCLK 80 MHz from X201 (SCTF SX3M80.000B20F30TNN; asynchronous mode), to the DAC XI only, enabled by the CPLD through MCLK_EN (R218 2.2 kΩ pull-down: off by default, during CPLD configuration and until DVDD is up; stopped before AVCC goes off); family oscillators X202 24.576 MHz and X203 22.5792 MHz share the CPLD PIN_1 (IO_GB, the only dedicated clock pin) into the PLL, both off by default (2.2 kΩ enable pull-downs) and switched break-before-make, so only one drives the node at a time; PLL ×2 gives exact integer clock ratios with an ideal 50 % bit-clock duty for all 8 rates, 352.8 kHz included; the CPLD has no free-running system clock (3.3) | 80 MHz is below the ES9018K2M MCLK limit: 12.5 ns cycle (≥ 10 ns), ≥ 5.6 ns pulse width (≥ 4.5 ns), 80 MHz ≈ 208 × 384 kHz > 192 × FSR (asynchronous mode needs no integer ratio) (R5-A3, R5-B2). The DAC needs a DATA_CLK duty cycle of 45:55 to 55:45 (R3 p. 30), hence the family oscillators in front of the PLL (owner decisions 24 Sep 2026). Break-before-make answers R5-A2, R5-B6 and R5-C1; the 80 MHz feed to the CPLD is removed because the QFN-32 has one dedicated clock pin (R7-D DG-2); MCLK_EN answers R7-B AN-3 and R7-D DG-6 |
| DAC | ESS ES9018K2M, I²C register control; differential current-mode outputs into I/V converters (ESS application circuit, R3 p. 27) | DNR 127 dB-A and THD+N −120 dB per datasheet, specified in differential current mode and asynchronous mode |
| DAC core supply | Fixed 1.3 V LDO Microne ME6216A13M3G (SOT23-3: pin 1 VSS, pin 2 VOUT, pin 3 VIN; ±15 mV, 250 mA max, ceramic-stable) fed from the switched 3V3D domain; R525 1 kΩ bleeder on 1V3 | External DVDD is required for FSR > 192 kHz (R3). Fed from 3V3D, DVDD cannot exceed DVCC during ramps; the ME6216 has no output discharge, so R525 makes DVDD collapse before DVCC (R7-A P-8); the previous adjustable LDO needed a tantalum-type capacitor and a 5–10 mA minimum load (R4-B M5; R4-A M4; R7-A P-9) |
| Headphone rails | +3.90 V / −3.72 V from TI LM27762: FB+ 225 kΩ / 100 kΩ → +3.900 V (3.789–4.014 V); FB− 205.1 kΩ / 100 kΩ → −3.722 V (3.618–3.828 V magnitude); ranges incl. VFB and the 1 % resistors; each top resistor built from Basic values (100 kΩ + 100 kΩ + 15 kΩ + 10 kΩ, or 100 kΩ + 100 kΩ + 5.1 kΩ) | V+ is raised by R523 10 kΩ so that the OPA1612AIDR fallback (common-mode limit (V+) − 2 V) keeps VREF (≤ 1.754 V worst case) ≥ 35 mV inside its range at the minimum rail; OPA2210 margin ≥ 0.53 V (R7-E BM-1). The negative rail (charge pump + negative LDO) keeps regulation with peak currents and a 0.3 Ω cable and connector drop from a 4.75 V host: ≥ +0.05 V to the maximum ¦V−¦ with the 70 mA envelope at 4.4 mm 32 Ω (2.3; R7-A P-6). R518 5.1 kΩ includes the divider-resistor tolerance (R4-A H4; R4-B S1; R4-C H2; R5-A9) |
| Output level | Every unit calibrated to 1.55 Vrms per leg at 0 dBFS (2.19 V peak; 3.5 mm 1.55 Vrms, 4.4 mm 3.1 Vrms) with the ES9018K2M 32-bit master trim, stored in MCU flash at the end-of-line test; the −4.5 dB pre-calibration default is a safety setting, active until a calibration is stored (highest-gain unit 1.54 Vrms, 3.5.4); Rf = 1 kΩ ∥ 5.1 kΩ = 836 Ω | Untrimmed unit spread 1.955–2.589 Vrms per leg. The OPA1622 distortion starts ≈ 1.3 V from the rail (TI data at ±5 V), so the clean swing is Vpk ≤ (¦rail¦min − 1.3 V)/(1 + 8 Ω/Rleg) and the calibrated level is lowered to 1.55 Vrms (R7-B AN-2). 836 Ω uses Basic values (R4-B S2, S3; R4-A M3; R6-D5) |
| Power rating method | Rated at 1 % THD+N or calibrated 0 dBFS, whichever comes first (both channels, 1 kHz, calibrated level, worst-case rail); the clean output envelope (≤ 80 mA peak per leg, ≤ 70 mA at 4.4 mm 32 Ω, and the OPA1622 swing model) is kept separate from overload behaviour; Default mode is rated by the instantaneous peak current | R4-A H5, M3; R4-B S3; R4-C B6; R5-C4 (the TPS2553 limits instantaneous current); R7-A P-6; R7-B AN-2, AN-6 |
| Analog current limit | TI TPS2553 current-limited switch feeds the LM27762 with two limits: Default 0.157–0.213 A (RILIM 147 kΩ) and High 0.393–0.498 A (59.5 kΩ); the limit acts on instantaneous current | Overload → rails droop → LM27762 PGOOD bad → fault latch set in hardware within ≈ 35 µs (RAILS_OK_N rises in < 1 µs through R505 10 kΩ to 5V_ANA), which disables the amplifiers and turns the rails and the DAC analog supply off → firmware captures the cause, clears the latch and lowers the volume ceiling (R4-A H5, H6; R4-B M4; R4-C B6, H1; R5-A7, R5-B3; R6-A1; R7-A P-2, R7-C PR-2) |
| USB power states | Before configuration only the MCU domain 3V3M is powered (≤ 83 mA estimate; assumption on MCU current). Audio domains are switched: ME6211 #2 CE (3V3D, through R521), LP5907 EN (3V3A, through R307), TPS2553 EN (5V_ANA). CC1/CC2 are read by the MCU ADC through 10 kΩ (firmware thresholds 0.66 V and 1.23 V with hysteresis) to select Default (≤ 500 mA; estimated maximum 0.475 A with hardware-limited analog current) or High mode (≥ 1.5 A advertised; estimated maximum 0.760 A) and are monitored while configured. Descriptors: bus-powered, bmAttributes 0x80, bMaxPower 0xFA (500 mA). USB suspend is a v1 requirement: ≤ 2.5 mA within 10 ms of the last bus activity (budget ≈ 2.30 mA with both LEDs off; suspend level typically ≈ 8.2 ms, at worst 9.3 ms after the last bus activity, 2.5) | R4-A H6; R4-C B1, B2, H1; R5-A4, R5-C9; R6-A3, R6-B3; R7-A P-5, P-16, P-17. The TPS2553 limits only the analog branch; the MCU, CPLD and DAC currents are estimates or datasheet values, measured on the Stage 2 board (now with the DAC and its real supplies) before layout (R5-B4) |
| I/V op-amp | TI OPA2210 (single input pair, common-mode range (V−) + 1.5 V to (V+) − 1.5 V, Vos ±35 µV max, 2.2 nV/√Hz, 18 MHz, 2.5 mA per channel max; LCSC C1849415); output swing ≈ (V+) − 0.3 V at the ≈ 3 mA I/V load, ≈ 0.69 V above the calibrated 2.80 V peak (3.5.1); JLCPCB stock not confirmed, so the drop-in OPA1612AIDR (C94590) is designed in as the fallback | VREF 1.703 V sat inside the OPA2197 input-pair transition region, where its offset is not specified and its noise rises (R10); the OPA2210 has one input pair and ≥ 0.53 V common-mode margin at the minimum +rail (R5-A5, R6-D5). The OPA1612 fits only because V+ is now +3.90 V (≥ 35 mV margin); it raises the worst-case offset to 9.7 / 19.4 mV (at the edge) and the analog quiescent by 16 mA to ≈ 72 mA, so the Default-mode targets at 3.5 mm 32 Ω and 4.4 mm 60 Ω and 120 Ω would not be met; High mode is unaffected (R7-E BM-1; Section 8; 3.5.5) |
| Output legs | 2 × TI OPA1622, one amplifier per leg; each leg a 4-resistor difference amplifier (2.00 kΩ 0.1 %) fed by two I/V outputs | U401 = L+/L−, U402 = R+/R−, so an adapter that shorts leg− loads only one amplifier per package (TI continuous-short rating, R4); pin 3 (GND, the EN reference) is tied to GND (R7-B AN-4) |
| Protection | Fast hardware tier: two TI LM339DR quad comparators (U601, U602), one window per leg directly on its filter node (R601–R604 100 kΩ, C601–C604 1 µF; VTH+ from R607 47 kΩ + R636 15 kΩ over R609 10 kΩ, VTH− from R608 47 kΩ + R637 10 kΩ over R610 10 kΩ; C612–C615 decoupling; no diode-OR), PGOOD bad (Q610) and a sustained TPS2553 overcurrent (D606 with R638 1 kΩ) set the fault latch through node F (C605 at node F: power-on set and a 1.5–7.7 ms set-request stretch); 47 kΩ latch pull-ups, PGOOD pull-up R505 10 kΩ to 5V_ANA, capture ≈ 35 µs; the latch kills RAIL_EN, AMP_EN and AVCC_EN (Q607, Q608, Q611; each enable has a series resistor, R507, R618, R307) until firmware clears it; Q609 is a slow interlock (on 1.9–9.4 ms after PGOOD goes bad). Firmware tiers (same latch, per leg and per balanced pair, through the ANA_FAULT pin; armed whenever AVCC_EN is high): ≥ 1.0 V within 0.25 s, ≥ 0.5 V within 0.35 s, ≥ 0.3 V within 1.1 s, ≥ 0.2 V within 3 s, fed by the redesigned DC-sense network (15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF). Mandatory firmware start-up, fault-interrupt and stream-DC-guard rules (3.7); latch built from 2N7002 N-MOSFETs only | Leg trip 0.43–0.67 V worst case (≈ 0.55 V nominal), set only by datasheet-guaranteed divider, offset and bias-current values (R7-G G2); worst-case removal ≤ 47 ms at 2 V (limit 100 ms) and ≤ 125 ms at 1 V (limit 250 ms); terminal energy for a rail-stuck failure in High mode 21.4 / 11.0 mJ worst case at 16 / 32 Ω (limits 30 / 15 mJ, including the ≤ 0.6 mJ after the trip) (3.7); every sub-hardware-trip case ≤ 30 mJ at 16 Ω and ≤ 15 mJ at 32 Ω (worst 28.2 / 14.1 mJ). R4-A H3; R4-B M1, M2; R4-C H6; R5-A6, R5-A7, R5-B3; R6-A1, R6-A5, R6-B1, R6-B2; R7-C PR-1 to PR-19; R7-F F3, F5; R7-G G1–G3. The residual below the lowest tier (≤ 0.198 V across a driver: ≤ 2.45 mW into 16 Ω, ≤ 1.23 mW into 32 Ω, balanced headphones included) was accepted by the owner on 24 Sep 2026. The latch uses Basic 2N7002 (C8545) only; the LM339DR replaces the LM393BIDR as the one Extended protection type |
| Power sequencing | VREF tracks AVCC (C421 33 pF C0G, τ ≈ 0.29 µs; D401 and C422 removed). Power-up (order A, default): 5V_ANA, rails, then AMP_EN while AVCC is still off (VREF = 0, the legs start at ≈ 0 V), then AVCC_EN (VCCA, AVCC and VREF rise together), then AUD_EN, MCLK_EN and RESETB; order A′ (AUD_EN together with AVCC_EN) and option B (DAC first, legs last) are firmware alternatives, chosen by the measured pop. Controlled power-down (suspend, host command, fault): soft mute, MCLK stopped with RESETB still high (DAC muted) and AVCC_EN low, AMP_EN low once the 3V3A monitor reads < 0.1 V (typically t0 + 4.7 ms, at the latest t0 + 5.8 ms), then RESETB, AUD_EN, RAIL_EN and ANA_EN; suspend level typically ≈ 8.2 ms, at worst 9.3 ms after the last bus activity; resume-to-audio ≈ 1.0 s | The ES9018K2M outputs must stay between GND and AVCC (R3 absolute maximum, no margin given): the I/V converters hold the pins at VREF, which follows AVCC in every controlled ramp. After an output-stage failure the I/V converters can lose control of the pins for up to ≈ 5.5 ms; the BAT54S clamps D405–D408 (fitted, owner decision 24 Sep 2026) then hold them within ≈ −0.40 V … AVCC + 0.40 V, a small residual beyond the ESS rating (3.5.1; R7-F F4). In reset the DAC outputs are driven to ground, which gives ≈ 18–35 mV at the leg while AVCC is up and RESETB is low, so RESETB stays high until AVCC has gone and the AVCC-on step is measured on main board 1 (pop target at risk, R7-B AN-3, R7-H H2); MCLK stops before AVCC goes off, so it cannot back-power VCCA through XI (R7-D DG-6); the adaptive AMP_EN keeps the power-down pop at 300 Ω ≈ 4.5 mV typical, ≈ 12.7 mV worst case (measured; R7-H) (R6-A2, R6-A3, R6-D1, R6-D7; 2.5) |
| Input ferrite | Murata BLM18PG121SN1D (C14709, Basic, 2 A, 50 mΩ) in the 5V_ANA path (≤ 0.50 A); Sunlord GZ1608D601TF (C1002) on the DAC DVCC branch (≈ 14 mA) and, as FB202, on the CPLD VDD33/VDDA33 branch (≈ 25 mA) | GZ1608D601TF is a 200 mA / 450 mΩ part (C1002; C1015 is GZ2012D101TF), so it cannot carry the power path (R4-A H1); the bead-isolated CPLD feed follows AGM’s recommendation (R7-D DG-13) |
| DC offset requirement | ≤ ±10 mV unbalanced, ≤ ±20 mV balanced; 100 % offset screening in the automated test with a DC-capable DMM | Worst case 7.7 / 15.4 mV with datasheet maxima and the clamp leakage (≈ 4.0 mV RSS unbalanced); 9.7 / 19.4 mV with the OPA1612 fallback (at the edge); 9.3 / 18.6 mV if the DAC null is 1.65 V (R433 option, 3.4); 20 mV into 16 Ω balanced = 25 µW DC (R4-A H2; R4-C H5; R5-A11; R6-D4) |
| BOM | One part per line; every line has an LCSC number; every line’s JLCPCB class is taken from its JLCPCB part page or the basic-parts list, except X202, whose class is not shown (assumed Extended), and the DNF D102 (assumed Extended); clamps D405–D408 (JSMSEMI BAT54S, C916425, Extended) fitted by owner decision of 24 Sep 2026; LCSC prices refreshed on 24 Sep 2026 | R4-A M1; R4-C H4; R5-A14; R7-E BM-5, BM-7; R7-F F4 (D405–D408) |
| Firmware | Our own USB Audio Class 2.0 firmware on the CH32V307 high-speed port, built on WCH’s USBHS device examples (TinyUSB only if proven reliable), and our own CPLD logic; the Stage 1 desk setup is optional; the JLCPCB-built Stage 2 board with the real chips (DAC included) is the firmware gate and the layout gate (Section 5) | Owner decision 24 Sep 2026. WCH’s high-speed examples cover HID, CDC, mass storage and network, not audio; USBAudioToI2S (R7) stays a known-good comparison (R4-B S4; R4-C B3–B5, H7; R5-B8) |
| Kept | 3.5 mm unbalanced + 4.4 mm balanced jacks (both live); ≥ 384 kHz; single USB-C port, bus-powered; JLCPCB Economic PCBA; CI, AI review and measurement automation | Unchanged requirements |

# 2. Power tree and current budget

One USB-C port supplies the board. VBUS (5V_SYS) feeds four regulators: an always-on 3.3 V LDO for the MCU domain, a switched 3.3 V LDO for the audio-digital domain (through the R519/R520 dropper), the DAC’s analog LDO and the TPS2553 current-limited switch for the headphone rails. The MCU enables the audio domains after USB configuration. Currents are estimates unless marked as datasheet figures; the TPS2553 limits only the analog branch, so the totals are estimated maxima with hardware-limited analog current, measured on the Stage 2 board before layout.

Figure 1. Power domains and enables (5V_SYS, 3V3M, 3V3D with the CPLD and the three oscillators including the 80 MHz DAC MCLK oscillator enabled by the CPLD, 1V3, 3V3A, 5V_ANA, +3.90 V / −3.72 V rails for the OPA2210 I/V converters, the OPA1622 legs and the two LM339 window comparators; PGOOD to the overload latch).

## 2.1 Power domains

| Domain | Regulator and parts | Enable | Loads |
| --- | --- | --- | --- |
| 5V_SYS | VBUS from J101 (no switch); U101 USBLC6-2SC6; C102 1 µF; D102 TVS footprint (DNF, fitted only after the hot-plug test, 3.1) | Always | Inputs of U502, U504 (through R519/R520), U302 and U503 (5 × 1 µF on VBUS in total) |
| 3V3M | U502 ME6211C33M5G-N; C501, C502 1 µF | Always on | CH32V307 (C211 2.2 µF bulk, C221 1 µF at VDDA, VBAT tied to 3V3M), fault latch and its pull-ups, LEDs (off in suspend), FAULT pull-up (R512) |
| 3V3D | U504 ME6211C33M5G-N, fed through the dropper R519 + R520 (2 × 2.2 Ω 0805 in series, 4.4 Ω); C503 1 µF at the input, C504 1 µF and C512 10 µF at the output; CE pull-down R508 100 kΩ and CE series resistor R521 10 kΩ; bleeder R524 2.2 kΩ (1.5 mA; 3V3D < 0.3 V ≈ 70 ms after AUD_EN goes low) | AUD_EN (through R521) | AGRV2KQ32 (through FB202; C220 1 µF at VDDA33), oscillators X201 (80 MHz, enabled by MCLK_EN with the R218 2.2 kΩ pull-down), X202 (24.576 MHz) and X203 (22.5792 MHz) with the enable pull-downs R212, R213 (2.2 kΩ) and the node pull-down R214, DAC DVCC via FB301, 1.3 V LDO input, I²C pull-ups (R207, R208) |
| 1V3 | U303 ME6216A13M3G (SOT23-3, 250 mA) from 3V3D; C311, C312 1 µF; C310 2.2 µF at the DAC pin; bleeder R525 1 kΩ (1.3 mA; DVDD collapses before DVCC) | Follows 3V3D | ES9018K2M external DVDD |
| 3V3A | U302 LP5907MFX-3.3 from 5V_SYS; C301, C302 1 µF; EN pull-down R303 100 kΩ and EN series resistor R307 10 kΩ from the MCU; 230 Ω typical output discharge when disabled (R6) | AVCC_EN (through R307; pulled low by Q611 when the latch is set) | VCCA, AVCC_L and AVCC_R (via 2.2 Ω), the VREF divider (VREF = 0.516 × 3V3A, tracks it), the slow DC-sense bias R628–R631 (0.70 mA) and the 3V3A monitor R639/R640 (0.17 mA) |
| 5V_ANA | U503 TPS2553DBVR; C505, C506 1 µF; EN pull-down R513 100 kΩ; RILIM R509 + R514 = 147 kΩ, R510 in parallel via Q501 for High mode; FB501 to the LM27762 VIN | ANA_EN, ILIM_HI | LM27762 (C507 10 µF at VIN) and the PGOOD pull-up R505 |
| +3.90 V / −3.72 V | U501 LM27762DSSR from 5V_ANA; C508 1 µF flying, C509 10 µF CP, C510 10 µF OUT+, C511 10 µF OUT−; EN+/EN− = RAIL_EN (R506 pull-down, R507 10 kΩ); PGOOD → R505 10 kΩ to 5V_ANA and R522 10 kΩ into the MCU | RAIL_EN | 2 × OPA1622, 2 × OPA2210, 2 × LM339 (C612–C615) |

## 2.2 Rail budget by USB state

Digital loads at 5 V (typical / maximum):

| Load | Typ / max | Basis |
| --- | --- | --- |
| MCU CH32V307 incl. USB HS PHY | 60 / 80 mA | Assumption: the datasheet electrical tables were not obtained — to confirm |
| LEDs, latch, pull-ups | 2 / 3 mA | Estimate |
| CPLD AGRV2KQ32 | 15 / 25 mA | Assumption |
| Oscillator X201, 80 MHz (DAC MCLK, enabled by MCLK_EN) | 15 / 40 mA | SCTF 3C datasheet (R14): 40 mA max for 80–160 MHz; typical assumed |
| Oscillator X202, 24.576 MHz | 3 / 5 mA | YXC YSO110TR datasheet: 5 mA max; typical assumed |
| Oscillator X203, 22.5792 MHz | 5 / 10 mA | SCTF 2CM datasheet (R13): 10 mA max; typical assumed. Only one family oscillator drives the node at a time; a disabled output-enable oscillator may keep running internally, so both are counted |
| DAC DVCC | 13.5 / 14.2 mA | Datasheet (R3) |
| DAC external DVDD 1.3 V | 50 / 60 mA | Datasheet: 50 mA at MCLK 80 MHz and 192 kHz; +20 % for 384 kHz and spread |
| AVCC_L + AVCC_R + VCCA + VREF divider + DC-sense bias and 3V3A monitor | 17 / 17.5 mA + 0.87 mA | Datasheet and divider; the DC-sense bias (0.70 mA) and the 3V3A monitor (0.17 mA) are on 3V3A |
| Enable pull-downs (2 × 2.2 kΩ, one enable driven high) + node pull-down R214; I²C pull-ups R207/R208; bleeders R524 (3V3D) and R525 (1V3) | 1.6 / 1.7 mA + 1.5 mA + 2.8 mA | 1.5 mA per enable when driven high; the I²C pull-ups and the bleeders are counted (digital maximum 262 mA) |

Current by USB state (5 V at the receptacle):

| State | Domains on | Current (typ / max) | Notes |
| --- | --- | --- | --- |
| Attached, not configured | 3V3M only (MCU, latch, LEDs) | 62 / 83 mA | ≤ 100 mA unit load; rests on the MCU current assumption; measured on the Stage 2 board before layout (gate); mitigation: lower MCU clock before configuration |
| Configured, idle | All domains, outputs muted | 226 / 311 mA | Digital up to 262 mA plus the analog quiescent current with the outputs muted |
| Playing, Default mode | All | ≤ 0.475 A estimated maximum | Estimated maximum with hardware-limited analog current: 0.262 A digital + 0.213 A TPS2553 maximum Default limit (25 mA below 0.5 A); covers USB 2.0 500 mA hosts and phones that limit OTG to ≤ 500 mA |
| Playing, High mode | All | ≤ 0.760 A estimated maximum | Only when CC advertises ≥ 1.5 A (0.262 A digital + TPS2553 High limit ≤ 0.498 A) |
| Suspend | 3V3M, MCU Stop | ≤ 2.5 mA within 10 ms of the last bus activity (v1 requirement); budget ≈ 2.30 mA (margin 0.20 mA) | USB 2.0 suspend limit. Budget (mA): MCU Stop with the USBHS PHY suspended ≤ 1.9 (assumption, Stage 2 gate), D+ 1.5 kΩ pull-up into the host’s 15 kΩ 0.20, ME6211 3V3M quiescent 0.06 (60 µA typical), latch pull-up 0.07, node S pull-up 0.033, VBUS divider 0.034, regulators off 0.003; both LEDs off (0); slow DC-sense networks 0 (biased from 3V3A, which is off); PGOOD pull-up 0 (on 5V_ANA, which is off); node F sits high in suspend (no current). Suspend level typically ≈ 8.2 ms, at worst 9.3 ms after the last bus activity (2.5); measured on the Stage 2 board (real LP5907 and DAC) and on the main board |

Quiescent current per rail (max): OPA1622 13.2 mA, OPA2210 10 mA, 2 × LM339 0.8 mA typical, 2 mA maximum per package (TI SLCS006Z), plus 3.4 mA DC from the I/V outputs into the leg networks (positive rail); the analog quiescent is taken as ≈ 56 mA max (two LM339 at 2 mA each) in 3.5.5. With the OPA1612 fallback the I/V quiescent rises to 4.5 mA per channel (+16 mA, analog quiescent ≈ 72 mA), so the Default-mode targets at 3.5 mm 32 Ω and 4.4 mm 60 Ω and 120 Ω would not be met (3.5.5); High mode is unaffected. The slow DC-sense bias and the 3V3A monitor add 0.87 mA to 3V3A (0.70 + 0.17 mA). Analog current per load, average and instantaneous peak, is in 3.6.2.

## 2.3 Supply headroom

LM27762 (R5): VOUT+ = 1.2 V × (R1 + R2)/R2 and VOUT− = −1.22 V × (R3 + R4)/R4, with R2 and R4 ≥ 50 kΩ; VFB+ 1.182–1.218 V, VFB− −1.202 to −1.238 V. Including VFB and the 1 % divider resistors, the dividers give +3.900 V (3.789–4.014 V; R523 10 kΩ added to the FB+ top) and −3.722 V (3.618–3.828 V magnitude; R518 5.1 kΩ). The headroom is computed with peak currents and a 0.3 Ω cable and connector drop at the real total current, from a host VBUS of 4.75 V (R7-A P-6): TPS2553 rDS(on) 135 mΩ max; ferrite 50 mΩ (60 mΩ used); traces 20 mΩ; charge-pump resistance 3.0 Ω (assumption: 2.5 Ω is specified only at 5.5 V) and 3.5 Ω as the pessimistic case. Worst clean loads: 4.4 mm 32 Ω and 3.5 mm 16 Ω in High mode.

| Load (High mode, host VBUS 4.75 V, 0.3 Ω cable and connector) | Clean envelope per leg | Margin to the max ¦V−¦ (3.828 V), 3.0 Ω charge pump | Margin to the max ¦V−¦, 3.5 Ω charge pump | At 80 mA per leg (3.0 / 3.5 Ω) | Positive rail |
| --- | --- | --- | --- | --- | --- |
| 4.4 mm 32 Ω | 70 mA | +0.13 V | +0.05 V | +0.04 / −0.05 V (hence the 70 mA envelope) | LM27762 VIN ≈ 4.50 V against V+ max 4.01 V |
| 3.5 mm 16 Ω | 80 mA | +0.13 V | +0.03 V | Same as the envelope | LM27762 VIN ≈ 4.50 V against V+ max 4.01 V |

At 80 mA per leg the 4.4 mm 32 Ω case would be 0.05 V short of the maximum |V−| with a 3.5 Ω charge pump, so its clean envelope is 70 mA per leg (3.5.2). At a host VBUS of 5.0 V all cases keep ≥ +0.20 V. The 95 % PGOOD thresholds are +3.71 V and −3.54 V. The negative-rail headroom is measured at VBUS 4.60 V and 4.40 V on the main board (3.8).

## 2.4 Inrush

| Item | Value | Basis |
| --- | --- | --- |
| Capacitance on VBUS before any switch | 5 × 1 µF: C102, C501, C503, C301, C505 | Connector and the inputs of U502, U504 (after R519/R520), U302 and U503 |
| 3V3M capacitance charged at attach | 5.3 µF: 1 µF LDO output + 11 × 100 nF (C203–C210, C212, C605, C606) + C607 10 nF + C221 1 µF at VDDA + 2.2 µF MCU bulk | Only 3V3M starts at attach; C211 is 2.2 µF for this budget |
| Attach charge | ≈ 42.5 µC at 5.0 V, ≈ 43.8 µC at 5.25 V (≈ 5 µF on VBUS plus 5.3 µF on 3V3M) | USB-IF limit 50 µC; nominal values (DC-bias derating makes the real figure lower) |
| Later domain enables | Staggered by firmware; the headphone rails charge through the TPS2553 soft start and current limit | VBUS droop at each enable is measured at bring-up |

## 2.5 Power-up and power-down sequence

Pull-downs on AUD_EN, AVCC_EN, ANA_EN, RAIL_EN, AMP_EN, RESETB, OSC48_EN, OSC44_EN and MCLK_EN and the latch set at power-up (node F starts at 0 V through C605) hold everything off until firmware runs the sequences below. They meet the ES9018K2M recommended power-up (R3 p. 27): VCCA first; AVCC_L/AVCC_R and DVCC at the same time or later; external DVDD at the same time as DVCC or later; RESETB asserted until ≥ 1 ms after all supplies and MCLK are stable; later resets ≥ 10 ns. X201 stays off through R218 until the MCU sets MCLK_EN (a CPLD SPI register) ≥ 2 ms after 1V3 is up; X202 and X203 stay off through their 2.2 kΩ enable pull-downs until X202 is enabled, and RESETB is released only when the clocks are stable. VREF tracks AVCC (0.516 × 3V3A, τ ≈ 0.29 µs with C421 33 pF), so the I/V converters hold the DAC pins inside GND…AVCC in every controlled transition; after an output-stage failure the fitted clamps D405–D408 hold the pins within ≈ −0.40 V … AVCC + 0.40 V, a small residual (3.5.1). In reset the DAC outputs are driven to ground (R3 pin table): with AVCC up and RESETB low each pin sinks VREF/806 Ω ≈ 2.1 mA and the leg sees VREF·(Rf/Rs)·Δk, 18 mV at 1 % and 35 mV at 2 % I/V gain mismatch. The controlled power-down therefore keeps RESETB high (DAC muted) and stops MCLK before AVCC is switched off, and drops AMP_EN adaptively once the 3V3A monitor reads < 0.1 V (R7-B AN-3, R7-H). Power-up order A (legs first) is the default; order A′ (AUD_EN together with AVCC_EN, allowed by ESS: DVCC with or after VCCA) and option B (DAC first, legs last) are firmware alternatives, chosen by the pop measured on main board 1 (Stage 2 has no output stage). Resume-to-audio takes ≈ 1.0 s (0.6 s blanking charge + rails + 250 ms + DAC init + 105 ms soft start). Power-up after USB configuration (order A):

| # | Step | Hardware and timing | ESS p. 27 / basis |
| --- | --- | --- | --- |
| 1 | Headphone supply | After USB configuration: ANA_EN high (ILIM_HI per CC) → TPS2553 soft start → 5V_ANA; the blanking capacitor charges from RAILS_OK_N (PGOOD bad) through R617 | RAIL_EN only after 5V_ANA (EN must not exceed VIN); R505 pulls PGOOD up to 5V_ANA |
| 2 | Latch clear | ≥ 600 ms after ANA_EN (or after the rails were last turned off): FLT_CLR, confirm FLT low (clears the power-on set) | The blanking capacitor is fully charged, so the PGOOD-bad set request is blanked (R7-C PR-5) |
| 3 | Headphone rails | RAIL_EN high (≥ 10 ms after ANA_EN) → +3.90 V / −3.72 V; fault if PGOOD is not good within 20 ms (RAIL_EN and ANA_EN low, start-up fault reported); firmware DC watch at 1 kHz on the four DC-sense channels until the set path is armed (drop RAIL_EN if a leg exceeds 0.5 V) | Faults present at power-up are removed before the hardware set path is armed (R7-C PR-4, R7-A P-11); firmware ignores its own ANA_FAULT classification for 20 ms after RAIL_EN (P-12) |
| 4 | Second clear and arming | PGOOD good + 20 ms: second FLT_CLR (clears a spurious set from the rail inrush, TPS2553 FAULT); the set path is armed 51–186 ms after PGOOD good | Arming time from a fully charged blanking capacitor (≥ 600 ms of rails off) |
| 5 | Output legs | ≥ 250 ms after PGOOD good: AMP_EN high while AVCC is still off (VREF = 0 V, I/V outputs at 0 V), so the legs start at ≈ 0 V | After the latest arming time (186 ms); legs enabled while VREF = 0, so the disabled-leg path (R7-B AN-9) is not present at the jack (pop definition, 3.8) |
| 6 | DAC analog | AVCC_EN high → LP5907: VCCA, AVCC_L/R and VREF rise together (τ ≈ 0.29 µs); slow DC tier armed; the DC-sense bias comes up with 3V3A and the firmware takes the power-up zero reading of each leg while muted (accepted only within ±30 mV of the stored factory zero). RESETB is still low, so the leg sees the reset-state step of ≈ 18–35 mV until step 8 (measured on main board 1) | VCCA first, AVCC with it; AVCC_EN only while AMP_EN is high and the rails are good (AN-9 interlock); VREF lag ≤ 4 mV, so the DAC pins stay inside GND…AVCC; R7-B AN-3 |
| 7 | DAC digital and MCLK | +5 ms: AUD_EN high → 3V3D (DVCC, CPLD and oscillators powered; X201 disabled by MCLK_EN low) and 1V3 (DVDD ≤ DVCC); ≥ 2 ms after 1V3 is up: MCLK_EN high (SPI register) → MCLK | DVCC later than VCCA; DVDD with or after DVCC; MCLK only after DVDD is up (R7-A P-21) |
| 8 | Reset, init and unmute | RESETB high ≥ 1 ms after MCLK is stable → I²C init: register 1 = 0x80 (32-bit I²S, input auto-select off), register 10 = 0x00 (slave, asynchronous, stop_div = 0, required above 96 kHz), mute, master trim from flash; the ESS soft start ramps the outputs to ½ AVCC (≈ 105 ms at 80 MHz, register 14 soft_start); the CPLD starts with both family enables low and the PLL in reset, then the MCU enables X202 through SPI after ≥ 1 ms → lock → soft unmute when streaming | RESETB ≥ 1 ms after stable supplies and MCLK; register settings per R7-D DG-7 |
| 9 | Order A′ and option B (firmware alternatives) | A′: as order A, but AUD_EN together with AVCC_EN (allowed by ESS: DVCC with or after VCCA). B: steps 1–4, then AVCC_EN, AUD_EN, MCLK_EN, RESETB and soft start with AMP_EN low, then AMP_EN | Chosen only if the measured pop is lower (main board 1; Stage 2 has no output stage; R7-H H2, H5); B exposes the jack to the disabled-leg path (AN-9) plus the DC-sense bias (3.3 mV at 32 Ω, 31 mV at 300 Ω, 61 mV at 600 Ω) for ≈ 0.2 s |

Controlled power-down (including USB suspend), fault shutdown, unplug and programming mode:

| # | Step | Hardware and timing | Basis |
| --- | --- | --- | --- |
| 10 | Power-down, t0: mute | Controlled power-down (USB suspend, host command or fault command). t0 = suspend interrupt (3 ms after the last bus activity) or the command: soft mute (the FIFO is already empty at suspend) | The legs stay enabled and hold ≈ 0 V while the DAC is turned off |
| 11 | t0 + 0.1 ms: MCLK and AVCC off | MCLK_EN low (MCLK stops; RESETB stays high, DAC muted) and AVCC_EN low → 3V3A < 0.3 V by t0 + 5.6 ms worst case (LP5907 230 Ω typical discharge +50 %, 6.7 µF, R6); VREF follows AVCC; the DC-sense bias collapses with 3V3A | MCLK stopped first, so it cannot back-power VCCA through XI (R7-D DG-6); RESETB high avoids the reset-state DC step (R7-B AN-3); 5.5 ms worst case (R7-A P-16) |
| 12 | Adaptive AMP_EN, then reset and rails off | AMP_EN low as soon as the 3V3A monitor reads < 0.1 V (typically t0 + 4.7 ms), at the latest at t0 + 5.8 ms; then RESETB low, AUD_EN low, RAIL_EN and ANA_EN low; R524/R525 bleed 3V3D and 1V3 (3V3D < 0.3 V ≈ 70 ms after AUD_EN goes low); nothing is left that could back-power the I/V stage | Legs disabled after VREF has collapsed: pop at 300 Ω incl. the sense-bias path ≈ 4.5 mV typical, ≈ 12.7 mV at the worst-case discharge (measured; target ≤ 5 mV; R7-H); the latch sets as the rails go off and firmware masks it (5.2) |
| 13 | Suspend level | MCU low-power mode (Stop, USBHS PHY suspended), both LEDs off → suspend level typically ≈ 8.2 ms, at worst 9.3 ms after the last bus activity | USB suspend ≤ 2.5 mA within 10 ms of the last bus activity: ≥ 0.7 ms margin; budget ≈ 2.30 mA (2.2); measured on Stage 2 with the real LP5907 and DAC |
| 14 | Resume | Power-up sequence from step 1; AUD_EN only after ≥ 100 ms off; resume-to-audio ≈ 1.0 s (0.6 s blanking charge + rails + 250 ms + DAC init + 105 ms soft start) | 3V3D and 1V3 must have decayed before AUD_EN rises again (R7-A P-8); latency stated per R7-H H7 |
| 15 | Fault shutdown (latch trip) | Latch → Q607, Q608 and Q611 pull RAIL_EN, AMP_EN and AVCC_EN low within ≈ 35 µs; FLT interrupt (≤ 50 µs): AMP_EN, RAIL_EN and AVCC_EN low (mirroring the latch), AUD_EN and RESETB low, then cause capture, then ANA_EN low; 3V3A collapses within 5.5 ms worst case | Every enable the latch kills has a series resistor (R507, R618, R307), so no GPIO is shorted; AUD_EN and RESETB low stop MCLK back-powering VCCA and RESETB injection (R7-C PR-1, PR-6; R7-D DG-6); residual back-power ≤ 1.7 mA per DAC pin (806 Ω − 11 %), inside the OPA2210 10 mA input-current rating (R15) |
| 16 | Unplug / brownout | VBUS watchdog ISR (ADC1 continuous with the analog watchdog at 4.10 V, 3.95–4.25 V; ≈ 3.8 µs ADC latency; target < 5 µs to the GPIOs): AMP_EN and RAIL_EN low at once, then RESETB, AUD_EN, AVCC_EN and ANA_EN low. PGOOD goes bad at VBUS ≈ 3.70 V while 5V_ANA ≈ 3.6 V, so Q610 still turns on and the latch is the hardware backstop | The DAC-first order is impossible in the ≈ 6 µs hold-up, so the unplug pop stays a measured target (≤ 20 mV peak); with C421 33 pF VREF lags AVCC by ≤ 15 mV at the fastest unplug slope, so a DAC pin can exceed AVCC only while AVCC < ≈ 31 mV (ESS gives no allowance; the DAC pins are probed at hot unplug, 3.8) (R7-A P-18, R7-C PR-19, R7-F F4) |
| 17 | Programming mode | PROG pad (TP732, MCU PC13 with internal pull-up) shorted to GND at reset: the firmware runs AVCC_EN → +5 ms AUD_EN with RESETB low, MCLK_EN low and ANA_EN, RAIL_EN, AMP_EN low, without USB configuration (power from a USB charger or bench supply, since it can exceed 100 mA unconfigured); the CPLD is then programmed through J202, whose probe senses 3V3D only and never powers it | End-of-line order: MCU firmware (WCH-LinkE on J201) → CPLD (J-Link or AGM DAP-Link on J202) → calibration (R7-D DG-1, DG-4) |
| 18 | Domain interfaces | LINK_SCK, LINK_MOSI, LINK_FRAME and RESETB are GPIO-low or input whenever AUD_EN is low; I2C2 is disabled while 3V3D is off and recovered with 9 clocks after power-up; the I²C pull-ups sit on 3V3D; RESETB is driven through R308 1 kΩ | No back-powering of the audio-digital domain (R7-D DG-8, DG-17) |

# 3. Block notes

Entries with a reference are datasheet requirements; all other values are starting values. Designators, values and LCSC numbers follow Parts List v0.6.

Figure 2. Signal chain: 80 MHz oscillator for the DAC MCLK only (enabled by the CPLD), enable-selected 24.576 or 22.5792 MHz family oscillator (break-before-make, one on at a time) into the CPLD PLL on PIN_1, DAC in current mode, OPA2210 I/V converters and OPA1622 legs on the +3.90 V / −3.72 V rails; the two LM339 window comparators (one window per leg) and the MCU slow DC check, PGOOD and overload set the fault latch.

## 3.1 USB-C input and sensing

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| Connector | J101 SHOU HAN TYPE-C 16PIN 2MD(073) (C2765186, Extended): USB 2.0 data + 5 V, sink; A6/B6 (D+) and A7/B7 (D−) tied at the connector | No Basic 16-pin USB-C receptacle exists; only the 4.4 mm jack is through-hole; pin functions checked against the drawing before layout (R7-A P-10, 7.1) |
| CC pull-downs | R101, R102 5.1 kΩ Rd to GND | USB-C sink |
| CC sensing | CC1, CC2 → R103, R104 10 kΩ → MCU ADC2 (PA2, PA3); the ADC does not load CC; the 10 kΩ series resistors also protect the pins (no ESD device on CC) | Bands (R11): Default 0.25–0.61 V, 1.5 A 0.70–1.16 V, 3.0 A 1.31–2.04 V; firmware thresholds 0.66 V (Default ↔ 1.5 A) and 1.23 V (1.5 A ↔ 3.0 A) with hysteresis; only the CC pin with Rp is active (R7-A P-17). A USB-A-to-C cable reads as Default |
| Power mode | High mode only when ≥ 1.5 A is advertised; otherwise Default mode; CC1/CC2 monitored while configured (sampled at least every 5 ms; a drop to Default is handled within tSinkAdj, 5.2) | Many phones limit OTG to ≤ 500 mA and run in Default mode; USB Type-C timing per R16 |
| VBUS sense | R105 100 kΩ / R106 47 kΩ → MCU PA1 (5.0 V → 1.60 V), ADC1 in continuous single-channel conversion with the analog watchdog at 4.10 V nominal (3.95–4.25 V with the ADC reference, divider and ADC errors) | Unplug / brownout early warning → firmware power-down (target < 5 µs to the GPIOs; 7.5-cycle sampling at fADC = 12 MHz → 1.67 µs per conversion, ≈ 3.8 µs worst-case latency, 2.5); ≥ 0.15 V margin to the 4.40 V stress corner and ≥ 0.35 V to the 4.60 V design corner |
| ESD | U101 ST USBLC6-2SC6 on D+, D− and VBUS at the connector | — |
| VBUS capacitance | C102 1 µF at the connector; 5 × 1 µF on VBUS in total; D102 TVS footprint at J101, DNF (unidirectional, VRWM 6.0 V, e.g. MDD SMF6.0A, LCSC C123790, SOD-123FL, VBR ≥ 6.67 V; class assumed Extended) | Inrush in 2.4. VBUS has ceramics only; LP5907 absolute maximum 6.0 V, LM27762 5.8 V. Hot-plug test on board 1: scope C102 while plugging a USB-A-to-C cable into a live port; fit D102 only if the peak exceeds 5.8 V, after measuring its leakage at 5.25 V (the suspend margin is ≈ 0.20 mA); avoid 5.0 V-VRWM parts (hundreds of µA leakage at 5 V) (R7-A P-7). No VIN margin on the LP5907 and LM27762 above 5.5 V: accepted for v1, VBUS measured on the chargers used (R7-E BM-11) |
| Shield | R107 0 Ω (USB-C shell tied to GND); C101 4.7 nF DNF | Shell tied directly to GND for ESD; C101 only if R107 later becomes a bleed resistor; CC1/CC2 rely on the 10 kΩ series resistors (R7-A P-19) |
| USB data | D+/D− to the CH32V307 USBHS pins (PB7, PB6) as a 90 Ω differential pair | Layout per WCH reference |

## 3.2 CH32V307RCT6 (USB MCU)

MCU pin map (proposed, frozen with the CPLD pin map before the Stage 2 board; 41 of 51 GPIO including the optional ones):

| Pin (LQFP-64) | Signal | Function and notes |
| --- | --- | --- |
| PB7 (59), PB6 (58) | USBHS D+, D− | USB 2.0 high-speed pair (90 Ω) |
| PD0 (5), PD1 (6) | HSE | Y201 8 MHz crystal |
| NRST (7) | NRST | R201, C212; J201 |
| BOOT0 (60) | BOOT0 | R202 pull-down and BOOT0-to-3V3 pad |
| PA13 (46), PA14 (49) | SWDIO, SWCLK | J201 (WCH-LinkE) |
| PA1 (15) | VBUS_SENSE | ADC1, continuous, with the analog watchdog |
| PA2 (16), PA3 (17) | CC1, CC2 | ADC2 |
| PC0–PC3 (8–11); PC4 (24), ADC14 | DC_SENSE_L+, L−, R+, R−; 3V3A_MON | ADC2, each channel at 1 kHz with ±20 % dithered timing; 3V3A monitor through R639/R640 with C616 |
| PA4 (20) | LINK_FRAME | GPIO, active high (idle low) |
| PA5 (21) | LINK_SCK | SPI1 SCK, 36 MHz, through R224 33 Ω |
| PA6 (22) | LINK_MISO | SPI1 MISO (FIFO level) |
| PA7 (23) | LINK_MOSI | SPI1 MOSI, through R225 33 Ω |
| PD2 (54) | LRCLK_FB | TIM3_ETR, latched at SOF |
| PB10 (29), PB11 (30) | I2C2 SCL, SDA | R207/R208 2.2 kΩ to 3V3D |
| PA8 (41) | DAC_RESETB | Through R308 1 kΩ |
| PC10 (51) | AUD_EN | Through R521 10 kΩ |
| PC11 (52) | AVCC_EN | Through R307 10 kΩ |
| PC12 (53) | ANA_EN | U503 EN (R513 pull-down) |
| PA15 (50) | ILIM_HI | Q501 gate (R511 pull-down) |
| PB8 (61) | RAIL_EN | Through R507 10 kΩ |
| PB9 (62) | AMP_EN | Through R618 10 kΩ |
| PA11 (44) | FLT_CLR | Q606 gate (R616 pull-down) |
| PB3 (55) | FLT | Floating input, EXTI3, highest priority |
| PB4 (56) | RAILS_OK_N | Floating input through R522, EXTI4 on both edges |
| PB5 (57) | ANA_FAULT | Floating input or open-drain low, EXTI5 |
| PC6 (37), PC7 (38) | LED_G, LED_R | Through R701 1 kΩ and R702 2.2 kΩ; both off in suspend |
| PA9 (42), PA10 (43) | USART1 TX, RX | Pads (RX optional) |
| PC8 (39) | CPLD_IRQ | Optional |
| PC9 (40) | DAC_LOCK | DAC GPIO2 (lock status) |
| PC13 (2) | PROG | Internal pull-up; TP732 to GND at reset selects the programming mode |
| PB12–PB15 (33–36) | Reserved | SPI2 second lane to two spare CPLD pins |
| Supplies | VBAT 1, VSSA 12, VDDA 13, VSS_4 18, VDD_4 19, VSS_1 31, VIO_1 32, VSS_2 47, VDD_2 48, VSS_3 63, VIO_3 64 | VBAT tied to 3V3M; 100 nF at each supply pin plus one extra at VDD_2 and VDD_4; C221 1 µF at VDDA |

Requirements and values:

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| Device | U201, LQFP-64 10 × 10 mm, 144 MHz RISC-V, on 3V3M (always on) | R1; C2988083. LCSC stock 193 / JLCPCB 364 (Sep 2026): reserve before ordering |
| HSE crystal | Y201 YXC X50328MSB2GI, 8 MHz, 5.0 × 3.2 mm, ±10 ppm, CL 20 pF (C115962, Basic); C201, C202 33 pF C0G | Load capacitors 2 × (20 pF − ≈ 3.5 pF stray) ≈ 33 pF. Firmware sets the USB HS PLL reference to 4 MHz as HSE ÷ 2, so 8 MHz is confirmed. No Basic 3.2 × 2.5 mm 8 MHz crystal exists |
| Decoupling | C203–C210 100 nF: one at each of the 6 supply pins (VBAT 1, VDDA 13, VDD_4 19, VIO_1 32, VDD_2 48, VIO_3 64) plus one extra at VDD_2 and VDD_4 (USB PHY); C221 1 µF at VDDA; C211 2.2 µF bulk; VBAT tied to 3V3M | C211 and C221 give the attach charge ≈ 42.5 µC at 5.0 V (2.4); R7-D DG-18 |
| Reset | R201 10 kΩ NRST pull-up; C212 100 nF; NRST on J201 (R7-D DG-14) | — |
| Boot | R202 10 kΩ BOOT0 pull-down; BOOT0-to-3V3 pad for recovery (R7-D DG-14) | — |
| USB HS | D+/D− (PB7/PB6, pins 59/58) as a 90 Ω differential pair; D+ has the 1.5 kΩ pull-up inside the PHY (0.20 mA into the host’s 15 kΩ in suspend) | Layout per WCH reference. The suspend path is a Stage 2 gate: Stop with LPDS, PHY suspended, wake on resume, HSE/PHY PLL restart within the 10 ms resume recovery; fallback wake: EXTI on PB6/PB7 (R7-D DG-10) |
| Debug | J201 pads for WCH-LinkE: 3V3 (sense only; the probe must not power 3V3M), SWDIO (PA13), SWCLK (PA14), NRST, GND, plus a BOOT0-to-3V3 pad and a USART1 TX pad (PA9; RX PA10 optional pad) | Programming and recovery (R7-D DG-14) |
| Pin map | Proposed pin map in the table above (41 of 51 GPIO, optional pins included), frozen before the Stage 2 board together with the CPLD pin map (3.3). FLT (PB3), RAILS_OK_N (PB4) and ANA_FAULT (PB5) are always floating inputs, never with internal pulls, also in low-power code; ANA_FAULT may be driven open-drain low, never push-pull high (3.7) | R7-C PR-7, R7-D DG-9; unit test on the GPIO configuration table |
| ADC inputs | ADC1: VBUS_SENSE (PA1) in continuous single-channel conversion with the analog watchdog (4.10 V), 7.5-cycle sampling at fADC = 72 MHz ÷ 6 = 12 MHz (PCLK2 must be 72 MHz; 18 MHz exceeds the 14 MHz limit) → 1.67 µs per conversion. ADC2 scans seven channels in a fixed order: CC1, CC2 (PA2, PA3), the four slow DC-sense inputs DC_SENSE_L+, L−, R+, R− (PC0–PC3) and the 3V3A monitor on PC4 (24), ADC14; 239.5-cycle sampling, the first conversion after each channel switch discarded, each channel at 1 kHz with ±20 % dithered timing | 3.1; CC monitoring per 5.2; slow DC tier per 3.7 and 5.2 (every DC-sense reading corrected ratiometrically with the 3V3A monitor; zero of each channel calibrated at the end-of-line test and re-read at every power-up); source impedance 4.7 kΩ, inside the 50 kΩ limit; VBUS watchdog ISR on a fast (VTF) vector (R7-A P-18, R7-D DG-5, R7-F F3) |
| Audio link and outputs toward the CPLD | SPI1 master with DMA: LINK_SCK (PA5, through R224 33 Ω), LINK_MOSI (PA7, through R225 33 Ω), LINK_MISO (PA6, returns the FIFO level during the frame) and LINK_FRAME (PA4, active high, idle low); CPOL = 0; 36 MHz (72 MHz ÷ 2) is the only workable clock (68 % utilised at 384 kHz; register reads at ≤ 18 MHz); each frame starts with a 16-bit header; 8-bit frames or a halfword swap in the CPLD set the word order; a second lane is reserved (SPI2 on PB12–PB15). LINK_SCK, LINK_MOSI, LINK_FRAME and RESETB are GPIO-low or input whenever AUD_EN is low | The CPLD writes its FIFO in the SPI clock domain (SCK through ALTCLKCTRL) and reads it in the PLL bit-clock domain (3.3); I²S is not used for the link (its frame rate is not locked to the audio rate); no back-powering through the CPLD pins (R7-D DG-8, DG-12, DG-19) |
| I²C | I2C2 SCL (PB10), SDA (PB11) to the DAC; R207, R208 2.2 kΩ pull-ups to 3V3D; I2C2 disabled while 3V3D is off, 9-clock bus recovery after power-up | Pull-ups switch off with the DAC domain (R7-D DG-8) |

## 3.3 CPLD and clock

CPLD pin map (AGRV2KQ32, proposed, 13 of 24 I/O used; frozen with the MCU pin map before the Stage 2 board). FIFO: 4 × M9K → 1024 × 32 bit (≈ 1.33 ms at 384 kHz). Global clocks: PLL output and SCK; there is no free-running system clock.

| Pin | Signal | Connection and notes |
| --- | --- | --- |
| 1 | FAM_CLK | IO_GB, the only dedicated clock pin: shared X202/X203 node (R210/R211, R214) into the PLL |
| 2 | OSC48_EN | X202 enable (pin 1), R212 2.2 kΩ pull-down |
| 3 | OSC44_EN | X203 enable (pin 1), R213 2.2 kΩ pull-down; U205 select if the mux is fitted |
| 4 | NRST | J202 |
| 6 | VDDA33 | FB202 branch, 100 nF + C220 1 µF |
| 9 | LINK_SCK | From MCU PA5 through R224; onto the global network through ALTCLKCTRL; weak pull-down |
| 10 | LINK_MOSI | From MCU PA7 through R225; weak pull-down |
| 11 | LINK_FRAME | From MCU PA4 (active high); weak pull-down |
| 12 | LINK_MISO | To MCU PA6 (FIFO level during the frame) |
| 13 | LRCLK_FB | To MCU PD2 (TIM3_ETR) |
| 14 | CPLD_IRQ | To MCU PC8 (optional) |
| 16, 32 | VDD33 | FB202 branch, 100 nF at each pin |
| 17, 30 | GND | Ground; exposed pad also to GND |
| 18 | BCLK | → R204 33 Ω → DAC DATA_CLK (27) |
| 19 | LRCLK | → R205 33 Ω → DAC DATA1 (26) |
| 20 | SDATA | → R206 33 Ω → DAC DATA2 (25) |
| 21 | MCLK_EN | → X201 pin 1, R218 2.2 kΩ pull-down |
| 24 | JTMS | J202 (2-wire Compact-JTAG) |
| 25 | JTCK | J202 (2-wire Compact-JTAG) |
| 5, 7, 8, 15, 22, 23, 26–29, 31 | Spare | Two of them reserved for the SPI2 second lane |

Requirements and values:

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| CPLD | U202 AGM AGRV2KQ32, 2K LUTs, QFN-32 4 × 4 mm (leadless), on 3V3D through FB202; our own logic (Section 5): SPI link capture with FRAME framing, dual-clock FIFO (1024 × 32 bit in 4 × M9K, ≈ 1.33 ms at 384 kHz), I²S output engine (SDATA and LRCLK change on the falling BCLK edge), PLL ×2 on the selected family oscillator with integer division to BCLK/LRCLK for all 8 rates, oscillator enables OSC48_EN/OSC44_EN and MCLK_EN, SPI registers | R2; C18208899, LCSC stock 980. AG32 silicon in pure-CPLD mode; dynamic current measured on Stage 2 (R7-D DG-21). I/O state and weak pull-up during configuration to check (7.1) |
| Decoupling | C213–C216 100 nF, one at each CPLD supply pin; C220 1 µF at VDDA33 (PLL supply); FB202 GZ1608D601TF (C1002, Basic) from 3V3D feeds the VDD33/VDDA33 branch (≈ 25 mA), as AGM recommends a separate, bead-isolated 3.3 V feed (R7-D DG-13); exposed pad to GND | — |
| Oscillator X201 | SCTF SX3M80.000B20F30TNN, 80 MHz 3.3 V CMOS, 3.2 × 2.5 mm, ±20 ppm tolerance / ±30 ppm stability, 40 mA max, duty 45–55 % (C5179978, Extended, LCSC stock 2,503), on 3V3D; C217 100 nF; tri-state pin 1 = MCLK_EN from CPLD pin 21 with the R218 2.2 kΩ pull-down (off by default and during CPLD configuration) | DAC MCLK only (XI load ≈ 10 pF + trace; XI trace ≤ 20 mm; duty measured at XI, R7-D DG-15); MCLK stays off until DVDD is up and is stopped before AVCC goes off (R7-A P-21, R7-B AN-3, R7-D DG-6). SCTF 3C datasheet (R14). Replaces the 100 MHz part (C4353707), which sat at the ES9018K2M limit |
| Oscillator X202 | YXC OT322524.576MJBA4SL, 24.576 MHz 3.3 V CMOS, 3.2 × 2.5 mm, ±10 ppm tolerance, ±20 ppm stability (C2831388), on 3V3D; C218 100 nF; R210 33 Ω into the shared node; OSC48_EN with R212 2.2 kΩ pull-down (off by default) | 48 kHz family. YXC YSO110TR datasheet: pin 1 tri-state (enable ≥ 70 % VDD or floating, disable ≤ 30 % VDD), duty 45–55 %, start-up ≤ 3 ms, 5 mA max; no disable time specified. JLCPCB class not shown, assumed Extended; the assembly-fixture sentence on its JLCPCB page is standard text on every part page, not a part-specific requirement (Section 8) |
| Oscillator X203 | SCTF SX2M22.5792M20F30TNN, 22.5792 MHz 1.62–3.63 V CMOS, 2.5 × 2.0 mm, ±20 ppm tolerance, ±30 ppm stability (C5452663, Extended), on 3V3D; C219 100 nF; R211 33 Ω into the shared node; OSC44_EN with R213 2.2 kΩ pull-down (off by default) | 44.1 kHz family. SCTF 2CM datasheet (R13): pin 1 “Tri-State”, enabled when OE is logic 1 or open, disabled when logic 0 (≤ 0.3 VDD); start-up ≤ 5 ms; 10 mA max; duty 45–55 %; no disable/enable time, internal pull-up value or disabled-output leakage specified (measured on Stage 2) |
| Clock routing | X201 → R203 33 Ω → DAC XI (direct, short), no CPLD branch; X202 → R210 and X203 → R211 into one node at CPLD PIN_1 (IO_GB, the only dedicated clock pin, which the PLL input must use); R204–R206 33 Ω are the series resistors on BCLK, LRCLK and SDATA | The 33 Ω resistors are source terminations, not contention protection. Both family oscillators and the shared node next to CPLD pin 1, short traces; the idle oscillator’s trace kept short; the former 80 MHz feed to the CPLD is removed (R7-D DG-2) |
| Shared-node pull-down | R214 10 kΩ from the shared family-clock node to GND | Defines the node low while both family oscillators are off |
| Fallback mux (DNF) | U205 Nexperia 74LVC1G157GW,125 (C135822, SOT-363, Extended) 2:1 mux and R215 33 Ω at the PLL pin, both DNF; select = OSC44_EN; R216, R217 10 kΩ pull-downs on both U205 inputs (DNF, fitted with U205) | To use it: fit U205, R215, R216 and R217 and remove R210 and R211; with R210/R211 removed a disabled oscillator output is high-impedance, so each mux input needs its pull-down. One of the two Stage 2 boards is built with the mux fitted; its output goes to CPLD PIN_1 |
| Oscillator enables | The CPLD drives OSC48_EN (X202, pin 2), OSC44_EN (X203, pin 3) and MCLK_EN (X201, pin 21) from SPI registers written by the MCU; R212, R213 and R218 are 2.2 kΩ pull-downs, so every oscillator is off at power-up, during CPLD configuration or reprogramming and whenever the CPLD pins are high-impedance; the family enables are mutually exclusive and never high in the same cycle; after configuration the CPLD starts with both family enables low and the PLL in reset, then the MCU enables X202 after ≥ 1 ms | 2.2 kΩ keeps pin 1 below 0.3 VDD against any combined pull-up ≥ 5.1 kΩ (the oscillators’ internal pull-ups of unknown strength and the CPLD weak pull-up during configuration); 1.5 mA per enable when driven high. Same 3V3D domain as the oscillators, so no back-powering when 3V3D is off. Only one family oscillator drives the node at a time; a disabled output-enable oscillator may keep running internally (both counted in the current budget) |
| DAC MCLK limits | Normal (asynchronous) mode at 80 MHz: cycle 12.5 ns (≥ 10 ns); pulse width ≥ 5.6 ns at 45:55 (≥ 4.5 ns); duty 45–55 % (DAC 45:55 to 55:45); 80 MHz ≈ 208 × 384 kHz (208.3; asynchronous mode needs no integer ratio), above the 192 × FSR minimum (73.7 MHz at 384 kHz); external 1.3 V DVDD still required (FSR > 192 kHz) | R3 “XI / MCLK Timing”. The previous 100 MHz part sat at the cycle-time limit (R5-A3, R5-B2) |
| DAC audio-interface timing | DATA_CLK (BCLK) duty cycle 45:55 to 55:45; pulse width high and low ≥ 4.5 ns; cycle ≥ 10 ns; DATA setup 4.1 ns and hold 2 ns to the DATA_CLK rising edge; the CPLD changes SDATA and LRCLK on the falling BCLK edge (≥ 20 ns half period at 24.576 MHz BCLK against 4.1 ns setup / 2 ns hold) | R3 p. 30. One fixed MCLK cannot give 45:55 bit clocks for both families (the earlier single 100 MHz clock gave 40:60 at 352.8/384 kHz), hence the family oscillators |
| Bit clocks | CPLD PLL ×2 → 49.152 MHz (X202) or 45.1584 MHz (X203) → BCLK = PLL ÷ 2/4/8/16 → 384/192/96/48 kHz or 352.8/176.4/88.2/44.1 kHz; LRCLK = BCLK/64 | All 8 rates are exact integer clock ratios with an ideal 50 % bit-clock duty (table below); the PLL ×2 also turns the oscillators’ 45–55 % duty into a 50:50 bit clock. Absolute rates follow the oscillator tolerance (±20 ppm tol. + ±30 ppm stab. for X203), which the asynchronous USB feedback absorbs |
| Family change | Break-before-make, sequenced by the MCU through CPLD SPI registers: (1) DAC soft mute; (2) I²S output stopped (held low); (3) PLL held in reset; (4) old enable low; (5) wait ≥ 1 ms with both off; (6) new enable high; (7) wait ≥ 10 ms (start-up ≤ 3 ms X202, ≤ 5 ms X203); (8) release the PLL, wait for lock; (9) restart the output; the MCU checks the LRCLK rate with its feedback timer (within ±0.1 % of the target, otherwise stay muted, retry once, then report a fault); (10) DAC re-locks (lock_status, register 64) → soft unmute. Never change both enable bits at the same time; MCLK (X201) is not touched | The datasheets give no disable time; typical oscillator disable times are well under 1 µs, so 1 ms is a large margin; the disable time is measured on Stage 2. No clock multiplexer in the CPLD fabric (R5-A2, R5-B6, R5-C1) |
| Clock path | No PLL in the DAC MCLK path; the CPLD PLL only makes the bit clock (both families), whose jitter the DAC’s DPLL rejects | Answers the PLL-jitter concern (R4-B M3) |
| MCU link capture | The CPLD has no free-running system clock: it writes the FIFO in the SPI clock domain (LINK_SCK on CPLD pin 9, onto the global network through ALTCLKCTRL, as in the USBAudioToI2S reference) and reads it in the PLL bit-clock domain (Gray-coded pointers); each SPI frame starts with a 16-bit header so that the pointer synchronisers see SCK edges; the registers live in the SPI domain | Only PIN_1 is a dedicated clock pin (IO_GB) and the PLL input must be on it, so the CPLD no longer takes an 80 MHz clock input (R7-D DG-2); I²S is not used for the link, because its frame rate is not locked to the audio rate. Global clocks used: PLL output and SCK |
| Programming port | J202 pads, 2-wire Compact-JTAG: 3V3D (sense only), JTCK (pin 25), JTMS (pin 24), NRST (pin 4), GND | AGM Supra tool with a J-Link or an AGM DAP-Link only (a generic CMSIS-DAP such as the WCH-LinkE in DAP mode is not confirmed); the CPLD is powered by the firmware programming mode (PROG strap, 2.5), never by the probe (R7-D DG-1, DG-4) |
| Timing | CPLD timing at the SPI clock (36 MHz) and at the PLL output (49.152 MHz), including the FIFO crossing between the two clock domains | Stage 2 gate (Section 5) |

Bit clock by sample rate (BCLK = 64 × fs = PLL output ÷ 2/4/8/16; every rate is an exact integer clock ratio; integer division sets the nominal duty, while PLL jitter and output timing are verified on the bench; the DAC requires a duty cycle of 45:55 to 55:45):

| Sample rate | BCLK | Source | Ideal duty | Within 45:55? |
| --- | --- | --- | --- | --- |
| 48 kHz | 3.072 MHz | 49.152 MHz (24.576 MHz, CPLD PLL ×2) ÷ 16 | 50 % | Yes, integer division |
| 96 kHz | 6.144 MHz | 49.152 MHz ÷ 8 | 50 % | Yes, integer division |
| 192 kHz | 12.288 MHz | 49.152 MHz ÷ 4 | 50 % | Yes, integer division |
| 384 kHz | 24.576 MHz | 49.152 MHz ÷ 2 | 50 % | Yes, integer division |
| 44.1 kHz | 2.8224 MHz | 45.1584 MHz (22.5792 MHz, CPLD PLL ×2) ÷ 16 | 50 % | Yes, integer division |
| 88.2 kHz | 5.6448 MHz | 45.1584 MHz ÷ 8 | 50 % | Yes, integer division |
| 176.4 kHz | 11.2896 MHz | 45.1584 MHz ÷ 4 | 50 % | Yes, integer division |
| 352.8 kHz | 22.5792 MHz | 45.1584 MHz ÷ 2 | 50 % | Yes, integer division |

## 3.4 DAC and its supplies (ES9018K2M)

ES9018K2M QFN-28 pins (R3): 1 DGND, 2 N.C., 3 SCL, 4 SDA, 5 ADDR, 6 XO, 7 XI (MCLK), 8 AGND, 9 DACR, 10 DACRB, 11 AGND_R, 12 AGND_L, 13 DACL, 14 DACLB, 15 AVCC_L, 16 AVCC_R, 17 VCCA, 18 DVCC, 19 N.C., 20 DGND (exposed pad), 21 DVDD, 22 N.C., 23 GPIO2, 24 GPIO1, 25 DATA2, 26 DATA1, 27 DATA_CLK, 28 RESETB. Connections (R7-D DG-16): ADDR (5) to DGND through R306 10 kΩ (7-bit address 0x48, written 0x90); XO (6) open; the N.C. pins may be grounded; DATA_CLK (27) = BCLK, DATA1 (26) = LRCLK, DATA2 (25) = SDATA; GPIO2 (23) → MCU PC9 (DAC_LOCK, the default DPLL lock-status output).

| Pin / function | Connection | Requirement (R3) / note |
| --- | --- | --- |
| Mode | U301 ES9018K2M (C2688377, LCSC stock 724) in normal (asynchronous) mode: DPLL / jitter eliminator | Datasheet DNR/THD figures are specified in this mode (test condition MCLK 27 MHz, asynchronous) |
| DVCC | 3V3D through FB301 Sunlord GZ1608D601TF (C1002: 600 Ω at 100 MHz, 200 mA, 0.45 Ω) + 100 nF | +1.8 V or +3.3 V ±5 %; 13–14.2 mA |
| VCCA | 3V3A (LP5907) + 100 nF | +3.3 V ±5 %; 0.8 mA |
| AVCC_L, AVCC_R | 3V3A via R301, R302 2.2 Ω on separate branches (≈ 18 mV drop at 8 mA); C303, C304 2.2 µF + 100 nF each | +3.3 V ±5 %; 8 mA |
| DVDD | External 1.3 V from U303 Microne ME6216A13M3G (C236662: SOT23-3, pin 1 VSS, pin 2 VOUT, pin 3 VIN; ±15 mV, 250 mA max, ceramic-stable, 6 µA Iq, Pd 0.54 W) fed from 3V3D; C311 1 µF in, C312 1 µF out; C310 2.2 µF + 100 nF at the pin; R525 1 kΩ bleeder on 1V3 | External +1.3 V ±5 % must be used for PCM when MCLK > 50 MHz or FSR > 192 kHz; ≥ 1 µF effective (2.2 µF X5R recommended). DVDD ≤ DVCC during ramps; R525 makes DVDD collapse before DVCC. LCSC stock ≈ 3,100 |
| LP5907 | U302 LP5907MFX-3.3 (C80670) from 5V_SYS; EN = AVCC_EN through R307 10 kΩ from the MCU (PC11) with the R303 100 kΩ pull-down, pulled low by Q611 when the latch is set (against 0.33 mA); C301, C302 1 µF | Total output capacitance 5.7 µF (C302 + C303/C304 + 3 × 100 nF) ≤ 10 µF (R6); 230 Ω typical output automatic discharge when disabled: 3V3A below 0.3 V within 5.5 ms worst case (2.5); 3V3A also carries the DC-sense bias (0.70 mA) and the 3V3A monitor (0.17 mA), which collapse with it. Option not adopted: C303/C304 as 2.2 µF 16 V 0603 (C23630) for less DC-bias loss (R7-E BM-9) |
| MCLK | 80 MHz from X201 through R203 33 Ω to XI (pin 7), direct; X201 enabled by MCLK_EN (CPLD) only after DVDD is up and stopped before AVCC goes off; not switched between the rate families | “XI / MCLK Timing”: cycle ≥ 10 ns (12.5 ns), pulse width high and low ≥ 4.5 ns (≥ 5.6 ns at 45:55), duty 45:55 to 55:45; > 192 × FSR (80 MHz ≈ 208 × 384 kHz); external DVDD required above 192 kHz |
| BCLK, LRCLK, SDATA | From CPLD pins 18/19/20 through R204–R206 33 Ω to DATA_CLK (27), DATA1 (26) and DATA2 (25); 3.3 V logic; SDATA and LRCLK change on the falling BCLK edge; held low while the rate family changes | 32-bit I²S; DATA_CLK duty cycle 45:55 to 55:45, pulse width ≥ 4.5 ns, setup 4.1 ns, hold 2 ns (R3 p. 30; 3.3) |
| I²C | SCL (3), SDA (4) to the MCU I2C2 (PB10, PB11); pull-ups on 3V3D; ADDR (5) to DGND through R306 10 kΩ; XO (6) open | 7-bit address 0x48 (written 0x90); register 1 = 0x80 (32-bit I²S, input auto-select off); register 10 = 0x00 (slave, asynchronous, stop_div = 0, required above 96 kHz) (R7-D DG-7, DG-16) |
| RESETB | R305 100 kΩ pull-down; MCU PA8 through R308 1 kΩ (injection < 3 mA) | Held low until ≥ 1 ms after all supplies and MCLK are stable; in the controlled power-down kept high (DAC muted) until 3V3A < 0.3 V, then driven low; dropped first in the fault and unplug interrupts (R7-B AN-3, R7-D DG-17) |
| Outputs | DACL, DACLB, DACR, DACRB in differential current mode, each only to its I/V inverting input and to its BAT54S clamp (D405–D408, fitted; 3.5.1) | ESS application circuit (R3 p. 27; the figure is image-only and is compared with this design by eye, 7.1). Absolute maximum: “DAC Output voltage Range (DACL, DACR, DACLB, DACRB): GND < Vout < AVCC”, with no margin or injection rating given; the I/V converters hold the pins at VREF, which tracks AVCC (3.5.1), so the pins stay inside GND…AVCC in every controlled transition; after an output-stage failure the clamps D405–D408 hold the pins within ≈ −0.40 V … AVCC + 0.40 V, a small residual beyond this rating (3.5.1). In reset the outputs are “driven to ground” (R3 pin table): with AVCC up and RESETB low each pin sinks VREF/806 Ω ≈ 2.1 mA and the I/V outputs go to VREF·(1 + Rf/Rs) ≈ 3.47 V (3.82 V worst, past the swing limit) |
| Output model | Each pin is a voltage source swinging 3.05 Vpp (0.924 × AVCC) around its centre voltage behind 806 Ω ±11 %; unloaded differential full scale 2.156 Vrms. The datasheet gives two centre-voltage values: 1.702 V (current-output formula 2.112 mA − 1000·Vg/806 = 0) and 1.65 V (voltage-source model, AVCC/2) | R3 p. 31, note 1. The design uses VREF = 1.703 V; R433 1 kΩ → 0 Ω sets VREF = 1.650 V, decided after the VREF null test on the first boards (offset in 3.5.6) |
| Master trim | 32-bit master trim sets the calibrated level (3.5.4) | Value stored in MCU flash |
| Lock status | DPLL lock on GPIO2 (pin 23 → MCU PC9, DAC_LOCK; default lock-status output) and register 64 (lock_status), checked before unmute after a rate or family change | Section 5 |
| Exposed pad | DGND (pin 20 and the exposed pad); N.C. pins 2, 19 and 22 may be grounded (R3 pin table) | — |

## 3.5 Output stage: I/V converters and legs (2 × OPA2210 + 2 × OPA1622)

Each DAC output pin drives its own I/V converter: one OPA2210 amplifier holds the pin at VREF and turns its current into a voltage. Each leg is one OPA1622 amplifier wired as a 4-resistor difference amplifier fed by two I/V outputs: leg+ = I/V(DAC−) − I/V(DAC+), leg− the swap. U403 serves DACL and DACLB, U404 serves DACR and DACRB; U401 carries L+ and L−, U402 carries R+ and R−.

### 3.5.1 I/V converters (U403, U404)

| Item | Value | Basis / notes |
| --- | --- | --- |
| Converters | U403, U404 TI OPA2210IDR (C1849415, SOIC-8, same dual pinout) on +3.90 V / −3.72 V, one amplifier per DAC pin; drop-in fallback OPA1612AIDR (C94590) if the OPA2210 stock check fails (Section 8) | OPA210 datasheet (R15): single input pair, common-mode range (V−) + 1.5 V to (V+) − 1.5 V → upper limit 2.29 V at the minimum +rail 3.789 V, ≥ 0.53 V above VREF (≤ 1.754 V worst case, tracking 3V3A +2 %); Vos ±35 µV max, 2.2 nV/√Hz, 18 MHz, 2.2 / 2.5 mA per channel; input current 10 mA (absolute maximum ratings and section 7.3.5). Replaces the OPA2197, whose input-pair transition region contains VREF (R10). OPA1612 fallback: common-mode limit (V+) − 2 V, so V+ is raised to 3.90 V (R523) and VREF stays ≥ 35 mV inside it at the minimum rail; quiescent up to 4.5 mA per channel (+16 mA, analog quiescent ≈ 72 mA: the Default targets at 3.5 mm 32 Ω and 4.4 mm 60/120 Ω are not met, High mode is unaffected), offset 9.7 / 19.4 mV, noise 1.1 nV/√Hz (3.5.5, 3.5.6) |
| Inputs | Inverting input = the DAC pin (its only load apart from its clamp D405–D408, ≤ 20 pF per node; short trace); non-inverting input = VREF | ESS application circuit (R3 p. 27) |
| Feedback resistor | Rf = R423–R426 1 kΩ ∥ R434–R437 5.1 kΩ = 836 Ω, 1 % | Basic values. Per leg L = (Rf/Rs) × 2.156 Vrms = 2.24 Vrms nominal before calibration |
| Feedback capacitor | R427–R430 10 Ω in series with C417–C420 ∥ C427–C430 (two 330 pF C0G = 660 pF per converter) | ≈ 288 kHz with 836 Ω. No Basic C0G above 330 pF; X7R is unsuitable in the signal path |
| VREF | R431 15 kΩ over R432 15 kΩ + R433 1 kΩ from 3V3A → 1.703 V (0.516 × 3V3A through the 7.74 kΩ divider); C421 33 pF C0G at the divider (C1562; τ ≈ 0.29 µs with ≈ 5 pF stray); one shared node with short traces to all four non-inverting inputs; C422 and D401 removed; R433 0 Ω option → 1.650 V (3.4) | VREF tracks AVCC: lag ≤ 4 mV in controlled ramps (3V3A falls through the LP5907’s 230 Ω output discharge, R6) and ≤ 15 mV at the fastest unplug slope (0.1 V/µs), so a pin can exceed AVCC only while AVCC < ≈ 31 mV (was ≤ 0.13 V while AVCC < 0.28 V with 330 pF); ESS gives no allowance above AVCC, so the four DAC pins themselves are probed at unplug (3.8; R7-F F4). 1.703 V sits at the ESS current-output null (1.702 V). VREF noise (LP5907 < 6.5 µVrms × 0.516) is common mode and unchanged in band (pole ≈ 620 kHz); residual ≈ 0.2 µVrms per leg through the 3 % pair-gain mismatch, negligible |
| Decoupling | C423–C426 100 nF on V+ and V− of U403 and U404 | — |
| Output swing | At the calibrated level ±1.10 V peak around VREF (0.61 … 2.80 V); untrimmed highest-gain unit at 0 dBFS ±1.83 V (−0.13 … 3.53 V); 2.79 V peak with the −4.5 dB pre-calibration default | OPA210 datasheet (R15): (V+) − 0.2 V at RL = 10 kΩ and (V+) − 0.6 V at 600 Ω (±15 V). The I/V output load is ≈ 3 mA → estimate (V+) − 0.3 V → 3.49 V at the minimum rail 3.789 V: calibrated peak 2.80 V (≈ 0.69 V margin). An untrimmed highest-gain unit at 0 dBFS would reach 3.53 V and clip, so the −4.5 dB pre-calibration default is a safety setting, always active until a calibration is stored. In DAC reset with AVCC up the I/V outputs go to ≈ 3.47 V (3.82 V worst, past the swing limit; 3.4). SPICE item |
| Back-powering | Failed-output shutdown (stuck leg into 16/32 Ω, rail capacitance ±10 %, LP5907 discharge typical and +50 %): the OPA2210 positive common-mode limit (V+ − 1.5 V) drops below VREF 0.11–0.39 ms after the kill, while 3V3A falls below 0.3 V only after 3.2–5.6 ms, so the I/V converters may not hold the DAC pins for up to ≈ 5.5 ms. A pin then sits between the DAC’s internal source (806 Ω, within 0…AVCC) and the I/V output (836 Ω, within V−…V+). The fitted clamps D405–D408 hold it within ≈ −0.40 V … AVCC + 0.40 V (BAT54 VF ≤ 400 mV at 10 mA; ≤ 4.4 mA per pin) for this ≤ 5.5 ms window; unclamped it could reach −1.76 V (the DAC’s own protection conducting at ≈ −0.6 V with ≤ 3.9 mA) or ≈ 0.33 V above AVCC for a negative-stuck leg. Controlled sequences: MCLK stops and the DAC analog supply goes off before the rails, and VREF tracks AVCC, so the I/V converters keep the pins inside GND…AVCC | ESS specifies only “GND < Vout < AVCC” (no margin, no injection rating). Clamps fitted (owner decision 24 Sep 2026): D405–D408 JSMSEMI BAT54S (C916425, Extended, SOT-23; 30 V, IR ≤ 2 µA at 25 V, CD ≤ 10 pF at 1 V), one per DAC pin: pin 3 (common) to the DAC pin, pin 1 (lower anode) to AGND at the DAC’s AGND pins, pin 2 (upper cathode) to AVCC_L for DACL/DACLB or AVCC_R for DACR/DACRB at their decoupling capacitors, within 5 mm of the DAC pins (confirm the pinout on the JSMSEMI drawing before layout). In normal operation both diodes are reverse biased by ≈ 1.6–1.7 V; they never conduct in controlled transitions (VREF tracks AVCC), at unplug (VREF lag ≤ 15 mV with C421 33 pF) or during the DAC-reset step (pins between 0 V and VREF). A small residual remains: ≤ 0.4 V beyond the rails for ≤ 5.5 ms, only after an output stage has already failed (R7-F F4). The upper diodes can briefly back-feed 3V3A from the collapsing I/V rails (≤ 4.4 mA per pin for about the I/V rail decay, ≈ 1 ms with a stuck load); measured (3.8). Net leakage ≤ 0.4 µA per pin adds ≤ 0.35 mV offset per pin (3.5.6); ≤ 20 pF added per DAC-pin node (stability SPICE, 7.1). Inside the OPA2210 10 mA input-current rating (R15) |
| Stability | DAC pin capacitance plus ≤ 20 pF per node from the clamps D405–D408 (high-frequency noise gain 1 + 20 pF/660 pF ≈ 1.03, negligible for stability and noise) on the OPA2210 inverting inputs with the 836 Ω ∥ (10 Ω + 660 pF) feedback | SPICE before layout (open item) |

### 3.5.2 Output legs (U401, U402)

OPA1622 VSON-10 pins (R4): 1 +INA, 2 V+, 3 GND (EN reference; tie to GND), 4 V−, 5 +INB, 6 −INB, 7 OUTB, 8 EN, 9 OUTA, 10 −INA; thermal pad → V− (R7-B AN-4).

| Item | Value | Basis / notes |
| --- | --- | --- |
| Amplifiers | U401, U402 TI OPA1622IDRCR (C2876559) on +3.90 V / −3.72 V; one amplifier per leg; thermal pad to V− copper; pin 3 (GND, the EN reference) to GND | Isc +145/−130 mA typ (R4); pinout above (R7-B AN-4) |
| Resistors | R401–R416 YAGEO RT0603BRD072KL 2.00 kΩ 0.1 % 25 ppm thin film (C728588, Extended); gain 1 | Worst 4-resistor mismatch 0.2 % (common-mode leakage term in 3.5.6) |
| Filter | C401–C408 330 pF C0G across each feedback and ground-leg resistor | ≈ 241 kHz with 2 kΩ; 330 pF is the largest Basic C0G |
| Input mapping | leg+ = I/V(DAC−) − I/V(DAC+); leg− the swap | Each I/V inverts, so leg+ is in phase with DAC+ − DAC− |
| Input common mode | VREF ≈ 1.70 V → non-inverting node ≈ 0.85 V DC | Inside the OPA1622 range (V−)+1.5 V to (V+)−1 V, i.e. −2.22 V to +2.90 V on the nominal rails |
| Output link | R417–R420 0 Ω per leg; the hardware DC sense and the slow DC-sense network (3.7) are taken at the jack side of each link | An isolation resistor cannot fix capacitive-load instability (zero at 339 MHz): keep the 0 Ω links and test with 1 nF plus long cables; last resort 2.2–4.7 Ω, which breaks the ≤ 0.5 Ω output-impedance target (R7-B AN-7); removing one link allows the detector test and the failed-output emulation (3.8) |
| Enable | AMP_EN to both EN pins (pin 8): R421 100 kΩ pull-down, R618 10 kΩ from the MCU (PB9); pulled low by Q608 when the latch is set (≈ 35 µs) and by the slow interlock Q609 while the rails are off (on 1.9–9.4 ms after PGOOD goes bad) | EN threshold ≈ 0.82 V referred to the OPA1622 GND pin (pin 3, tied to board GND); Q609 also keeps EN below the input limit whenever the rails are off, including after a firmware clear |
| Supplies | C409–C412 100 nF and C413–C416 10 µF on V+ and V− | — |
| Swing model | Vpk ≤ (¦rail¦min − 1.3 V)/(1 + 8 Ω/Rleg) at the worst-case rail ¦rail¦min = 3.618 V; clean envelope ≤ 80 mA peak per leg, ≤ 70 mA at 4.4 mm 32 Ω (negative-rail headroom, 2.3) | Distortion starts ≈ 1.3 V from the rail (TI data at ±5 V: 600 Ω → 11.3 mW, 32 Ω → 150 mW), not 0.9 V (R7-B AN-2); the swing falls ≈ 8 Ω × Iout (R4 Fig. 38/39, typical, 25 °C); current limit ≈ 110–120 mA at 85 °C die. Modelled for the ±3.8 V rails: bench-verify |
| Jack ESD | D701–D704 GOODWORK LESD5D5.0CT1G (C41399463): Vrwm 5 V, 12 pF, one per jack leg | Legs swing ≤ ±3.8 V |
| Output impedance | < 0.1 Ω plus jack contacts (legs driven directly) | Target ≤ 0.5 Ω on both jacks (a 2.2–4.7 Ω last-resort link would break it, AN-7) |
| Ground layout | Unbalanced crosstalk −75 dB at 32 Ω needs ≤ 5.7 mΩ shared ground | Dedicated sleeve return, ≥ 4 vias, measured at the jack |

### 3.5.3 Jack pinout

| Signal | 4.4 mm 5-pole (J701, G-Switch GT-3321667P-01, through-hole): contact and pin | 3.5 mm (J702, HRO PJ-332A-6A, TRRS body used with TRS plugs): contact and pin |
| --- | --- | --- |
| L+ | Tip [pin numbers to be confirmed from samples] | Tip [pin numbers to be confirmed from samples] |
| L− | Ring 1 [pin numbers to be confirmed from samples] | Not connected (leg+ only) |
| R+ | Ring 2 [pin numbers to be confirmed from samples] | Ring 1 [pin numbers to be confirmed from samples] |
| R− | Ring 3 [pin numbers to be confirmed from samples] | Not connected (leg+ only) |
| GND | Sleeve [pin numbers to be confirmed from samples] | Sleeve and ring 2 [pin numbers to be confirmed from samples] (tie both to GND) |

Both jacks are live; use one headphone at a time. JLCPCB marks J702 as assembly difficulty “high” (meaning undocumented; allowed in Economic PCBA; kept, R7-E BM-6); the alternative PJ-332-6A (C962626) is not qualified because its pin map is unknown. Jack pin numbers are not verified (the drawings are image-only): buy 2 samples of each jack (LCSC, ships to Taiwan) and continuity-test every pin before layout (R7-B AN-1; checklist rule 73).

### 3.5.4 Level and calibration

| Parameter | Value | Basis |
| --- | --- | --- |
| DAC source | 3.05 Vpp around 1.65 V behind 806 Ω ±11 % per pin; unloaded differential 2.156 Vrms | R3 p. 31, note 1 |
| Untrimmed level per leg | L = (Rf/Rs) × 2.156 Vrms = 2.236 Vrms nominal; unit spread 1.955–2.589 Vrms | Includes Rs ±11 %, Rf 1 % and AVCC (LP5907) ±2 % |
| Calibrated full scale | 1.55 Vrms per leg (2.19 V peak): 3.5 mm 1.55 Vrms, 4.4 mm 3.1 Vrms | Master trim −2.02 dB (lowest-gain unit) to −4.46 dB (highest), nominal −3.18 dB; set by the OPA1622 clean swing at the minimum rail (1.3 V headroom, 3.5.2; R7-B AN-2) |
| Before calibration | Firmware applies −4.5 dB → ≤ 1.54 Vrms on any unit (highest-gain unit); 2.79 V peak at the I/V output of the highest-gain unit | Safety setting, always active until a calibration is stored: without it the highest-gain unit at 0 dBFS would reach 3.53 V at the I/V output and clip (limit ≈ 3.49 V, 3.5.1) |
| Channel trim | ±0.05 dB per channel by MCU digital scaling | Stored with the master trim |
| Calibration | Automated end-of-line test (QA403): master trim to 1.55 Vrms ± 0.1 dB and per-channel fine trim, written to MCU flash | 3.8, Section 5 |

### 3.5.5 Output power (High and Default mode)

Both channels driven, 1 kHz, at the calibrated level and the worst-case rail (|rail|min = 3.618 V); rated at 1 % THD+N or calibrated 0 dBFS, whichever comes first, inside the clean envelope (swing model with 1.3 V headroom; 80 mA per leg, 70 mA at 4.4 mm 32 Ω). The limiting factor is in brackets. Default mode is rated by the instantaneous peak current, because the TPS2553 limits instantaneous current: peak analog input = quiescent (≈ 56 mA max, two LM339 at 2 mA each) + 2 × 1.03 × Ipk (unbalanced, both channels on one rail, including the charge-pump factor, R7-B AN-6) or + 2 × Ipk × 2.03 (balanced, both rails) ≤ 157 mA (Default minimum). Estimates: the OPA1622 swing at the ±3.8 V rails is modelled from TI data, so the table is bench-verified.

| Load | High mode (limit) | Default mode (limit) |
| --- | --- | --- |
| 3.5 mm 16 Ω | 51.2 mW (80 mA envelope) | 19.3 mW (Default limit) |
| 3.5 mm 32 Ω | 53.8 mW (swing) | 38.6 mW (Default limit) |
| 3.5 mm 60 Ω | 34.9 mW (swing) | 34.9 mW (swing) |
| 3.5 mm 300 Ω | 8.0 mW (full scale) | 8.0 mW (full scale) |
| 4.4 mm 32 Ω | 78.4 mW (70 mA envelope) | 9.9 mW (Default limit) |
| 4.4 mm 60 Ω | 111.7 mW (swing) | 18.6 mW (Default limit) |
| 4.4 mm 120 Ω | 69.7 mW (swing) | 37.3 mW (Default limit) |
| 4.4 mm 300 Ω | 32.0 mW (full scale) | 32.0 mW (full scale) |
| 4.4 mm 600 Ω | 16.0 mW (full scale) | 16.0 mW (full scale) |

High-mode peaks stay below the 393 mA High minimum (worst 340 mA at 4.4 mm 32 Ω). Swing margin at full scale: 3.5 mm 16/32/60 Ω reach the clean limit 3.0/1.5/0.6 dB below full scale, 4.4 mm 60/120 Ω 1.6/0.6 dB below, and 300 Ω balanced is +0.04 dB (just inside), so the swing-limited loads are bench-verified. Power targets (worst unit, both channels, same rating method): High mode: 3.5 mm ≥ 50 mW at 32 Ω; 4.4 mm ≥ 105 mW at 60 Ω, ≥ 65 mW at 120 Ω, ≥ 30 mW at 300 Ω. Default mode: 3.5 mm ≥ 38 mW at 32 Ω; 4.4 mm ≥ 17 mW at 60 Ω, ≥ 35 mW at 120 Ω, ≥ 30 mW at 300 Ω; 32 Ω balanced is not rated in Default mode. All Default targets are still met with the ≈ 56 mA quiescent, but 3.5 mm 32 Ω (38.6 mW against ≥ 38 mW) has only a small margin. With the OPA1612 fallback (quiescent up to 4.5 mA per channel, analog quiescent ≈ 72 mA) the Default-mode estimates become 3.5 mm 16 Ω ≈ 14 mW, 3.5 mm 32 Ω ≈ 27 mW, 4.4 mm 60 Ω ≈ 13 mW and 4.4 mm 120 Ω ≈ 26 mW, so the Default targets at 3.5 mm 32 Ω (≥ 38 mW), 4.4 mm 60 Ω (≥ 17 mW) and 4.4 mm 120 Ω (≥ 35 mW) would NOT be met with the fallback; High mode is unaffected.

SPL (maker sensitivity): HIFIMAN Susvara (60 Ω, 83 dB/mW): High mode 111.7 mW ≈ 103.5 dB SPL, Default mode 18.6 mW ≈ 95.7 dB SPL; AKG K1000 (120 Ω, 74 dB/mW): High mode 69.7 mW ≈ 92.4 dB SPL, Default mode 37.3 mW ≈ 89.7 dB SPL.

### 3.5.6 Offset, noise and frequency response

| Offset contribution per leg (25 °C, datasheet maxima) | Value |
| --- | --- |
| Common-mode leakage: VREF 1.703 V × 0.2 % (0.1 % resistors, gain 1) | 3.40 mV |
| OPA1622 Vos 0.5 mV × 2 | 1.00 mV |
| OPA1622 Ios × 1 kΩ × 2 | 0.10 mV |
| OPA2210 Vos 35 µV × noise gain ≤ 2.17 × 2 (two amplifiers) | 0.15 mV |
| DAC pin-to-pin DC mismatch ≤ 1 mV (assumption, measure) × Rf/Rs | 1.18 mV |
| VREF-to-null error (divider tolerance and the assumed ±20 mV null-point uncertainty) × 3 % pair-gain mismatch | 1.17 mV |
| Clamp leakage D405–D408: net ≤ 0.4 µA per pin (assumption, 3.5.1) × Rf 836 Ω ≤ 0.35 mV, two pins per leg | 0.70 mV |
| Unbalanced total; requirement ≤ ±10 mV | ≤ 7.7 mV worst (≈ 4.0 mV RSS); margin 2.3 mV |
| Balanced total; requirement ≤ ±20 mV | ≤ 15.4 mV worst; margin 4.6 mV |

Every board is measured by the automated test (100 % offset screening with a DC-capable DMM, because the QA403 inputs are AC-coupled); the DAC-mismatch assumption stays open until measured. If the DAC null is 1.65 V (ESS voltage-source model, 3.4) instead of 1.702 V, the VREF-to-null term is 2.8 mV and the totals become ≤ 9.3 mV unbalanced and ≤ 18.6 mV balanced, still inside ±10 / ±20 mV; R433 0 Ω then moves VREF to 1.650 V. With the OPA1612 fallback the worst-case totals are 9.7 / 19.4 mV unbalanced / balanced, at the edge of the requirement. The clamp-leakage term assumes that the two diodes of each BAT54S see almost equal reverse voltage (1.70 V to AGND, 1.60 V to AVCC), so their leakages largely cancel (net ≤ 0.4 µA per pin; the datasheet maximum of 2 µA is at 25 V and 25 °C); it adds ≤ 0.7 mV per unbalanced leg and ≤ 1.4 mV balanced.

| Parameter | Estimate | Basis |
| --- | --- | --- |
| I/V output noise | ≈ 6.95 nV/√Hz | OPA2210 2.2 nV/√Hz × 2.04 plus Rf and DAC-source thermal noise (the OPA1612 fallback has 1.1 nV/√Hz) |
| Leg noise | ≈ 16.3 nV/√Hz unbalanced; ≈ 26.9 nV/√Hz balanced | Balanced: both legs share the same two I/V converters (leg+ = I/V(DAC−) − I/V(DAC+), leg− the swap), so the I/V noise doubles with the signal and only the leg-network noise adds as √2 |
| Leg-network noise | ≈ 11.5 nV/√Hz (four 2 kΩ in a unity-gain difference amplifier); ≈ 13.0 nV/√Hz with the OPA1622 | Corrected in this revision (R7-B AN-5) |
| Dynamic range, system | ≈ 117.4 dB(A) unbalanced, ≈ 118.6 dB(A) balanced at 1.55 Vrms | With the DAC’s 127 dB(A), which is also common to both legs (correlated model); lower than before because of the corrected leg-network noise and the lower level (R7-B AN-5). Target ≥ 115 dB(A), minimum ≥ 112 dB(A); measured results are recorded with the analyzer loopback floor |
| I/V pole | 288 kHz | 836 Ω with 660 pF |
| Leg pole | 241 kHz | 2 kΩ with 330 pF |
| Frequency response | −0.05 dB at 20 kHz, −0.20 dB at 40 kHz | Both poles |

## 3.6 Power (ME6211 × 2, TPS2553, LM27762)

| Item | Value | Basis / notes |
| --- | --- | --- |
| 3V3M LDO | U502 Microne ME6211C33M5G-N (C82942), always on; C501, C502 1 µF | ±2 %, 120 mV typical dropout at 100 mA; quiescent 60 µA typical (suspend budget, 2.2) |
| 3V3D LDO | U504 ME6211C33M5G-N, CE = AUD_EN through R521 10 kΩ with the R508 100 kΩ pull-down, fed from 5V_SYS through R519 + R520 (2 × 2.2 Ω 0805 in series, 4.4 Ω); C503 1 µF input; C504 1 µF and C512 10 µF bulk at the output; R524 2.2 kΩ bleeder | Same part type as U502; the dropper keeps U504 at 0.20 W at VBUS 5.25 V and 0.24 W at 5.5 V (160 mA incl. the bleeders; 300 mW SOT-23 rating, R17; 3.6.3); LDO input ≥ 3.70 V at VBUS 4.40 V. CE is rated ≤ VIN + 0.3 V and VIN sits behind the dropper, so R521 limits the CE clamp current at unplug to ≤ 0.2 mA (R7-A P-4); the ME6211 has no output discharge, so R524 takes 3V3D below 0.3 V ≈ 70 ms after AUD_EN goes low (P-8). C512 is the 3V3D bulk for the CPLD, the 80 MHz oscillator and the 1.3 V LDO input |
| 1.3 V LDO | U303 ME6216A13M3G (SOT23-3 “M3”: pin 1 VSS, pin 2 VOUT, pin 3 VIN) from 3V3D; R525 1 kΩ bleeder (1.3 mA) | 250 mA max, ±15 mV, Pd 0.54 W; LCSC stock ≈ 3,100 (3.4; R7-A P-9, R7-E BM-4); DVDD collapses before DVCC |
| Current-limited switch | U503 TI TPS2553DBVR (C55266): EN = ANA_EN with R513 100 kΩ pull-down; FAULT → MCU (ANA_FAULT, PB5) with R512 100 kΩ pull-up to 3V3M, and to the latch through D606 and R638 1 kΩ; firmware can drive ANA_FAULT low to set the latch (3.7); C505, C506 1 µF | rDS(on) 85 mΩ typ / 135 mΩ max; built-in soft start; FAULT open drain with 5/7.5/10 ms deglitch; the limit acts on instantaneous current (short-circuit response ≈ 2 µs, then constant-current limiting) (R9). The auto-retry version stays; the latch-off TPS2553-1 is listed “not recommended for new designs”, and our latch gives an ordered shutdown |
| Current-limit resistors | RILIM = R509 100 kΩ + R514 47 kΩ = 147 kΩ (Default); R510 100 kΩ switched in parallel by Q501 2N7002 (gate = ILIM_HI, R511 100 kΩ pull-down) → 59.5 kΩ (High) | R9: IOS max = 22980/R^0.94, nom = 23950/R^0.977, min = 25230/R^1.016 (mA, R in kΩ, 15–232 kΩ); two-level limit per datasheet Figure 24. Keep the RILIM trace short |
| Ferrite | FB501 Murata BLM18PG121SN1D (C14709, Basic): 120 Ω at 100 MHz, 2 A, 50 mΩ; 5V_ANA → LM27762 VIN | Drop ≈ 15–25 mV. FB501 + C507 resonate at ≈ 80–100 kHz: check the LM27762 VIN ripple at light load and add 0.47–1 Ω + 1 µF damping if needed (R7-A P-22) |
| Rail generator | U501 TI LM27762DSSR (C473398): C507 10 µF VIN, C508 1 µF flying, C509 10 µF CP, C510 10 µF OUT+, C511 10 µF OUT− | R5: charge-pump output resistance 2.5 Ω typ at VIN 5.5 V, 100 mA; LDO dropout 30 mV (negative) and 45 mV (positive) at 100 mA; input current limit 500 mA typ; UVLO 2.4 V falling / 2.6 V rising; absolute maximum VIN 5.8 V; symbol pins PGOOD = 1, FB− = 7 (R7-A P-20); no VIN margin above 5.5 V, accepted for v1 (R7-E BM-11) |
| FB+ divider | R501 100 kΩ + R515 100 kΩ + R516 15 kΩ + R523 10 kΩ = 225 kΩ over R502 100 kΩ → +3.900 V (3.789–4.014 V incl. VFB and 1 % resistors) | 1.2 V × 325/100; R2 ≥ 50 kΩ. R523 raises V+ by 0.12 V so that the OPA1612 fallback keeps VREF ≥ 35 mV inside its common-mode range at the minimum rail; PGOOD 95 % threshold +3.71 V |
| FB− divider | R503 100 kΩ + R517 100 kΩ + R518 5.1 kΩ = 205.1 kΩ over R504 100 kΩ → −3.722 V (3.618–3.828 V magnitude incl. VFB and 1 % resistors) | −1.22 V × 305.1/100; R4 ≥ 50 kΩ. R518 was 10 kΩ; 5.1 kΩ restores the charge-pump headroom once the resistor tolerance is included |
| Enable | EN+ and EN− = RAIL_EN: R506 100 kΩ pull-down, R507 10 kΩ from the MCU (PB8); pulled low by Q607 when the latch is set | EN high ≥ 1.2 V; EN must not exceed VIN, so RAIL_EN goes high only after 5V_ANA is up (≥ 10 ms after ANA_EN, R7-A P-11); in the controlled power-down RAIL_EN goes low after the DAC and AMP_EN (AMP_EN typically at t0 + 4.7 ms, at the latest t0 + 5.8 ms; 2.5) |
| PGOOD | Open drain, active low (low = both outputs ≥ 95 % of target: +3.71 V, −3.54 V); R505 10 kΩ pull-up to 5V_ANA (the LM27762 VIN, as TI specifies) → RAILS_OK_N to the MCU (PB4 through R522 10 kΩ, since the node swings to 5V_ANA ≤ 5.25 V), the Q610 gate, the Q609 gate through R623 1 MΩ with C607 10 nF, and the blanking RC | RAILS_OK_N steps to 4.31–5.15 V (0.98 × 5V_ANA) against the real loads and crosses 2.5 V in ≈ 1.0–1.3 µs, so the ≈ 35 µs latch capture holds (R7-A P-2, R7-C PR-2, R7-F F1, R7-H H1); the node may carry only ≥ 1 MΩ RC branches and high-impedance gate or MCU inputs (R522). No PGOOD current in suspend (5V_ANA off). In shutdown each output has a 50 kΩ pull-down (not relied on for discharge). At unplug PGOOD goes bad at VBUS ≈ 3.70 V while 5V_ANA ≈ 3.6 V, so Q610 still turns on (R7-C PR-19) |
| Rail capacitance | ≈ 30.6 µF per rail | After a trip both rails with C +10 % at the maximum rail voltage hold 0.52 mJ; with ≤ 1 µJ DAC back-power the after-trip bound is ≤ 0.6 mJ (3.7.1) |

### 3.6.1 Current limits

| Mode | RILIM | Limit min / nom / max | Use |
| --- | --- | --- | --- |
| Default | 147 kΩ | 157 / 183 / 213 mA | USB 2.0 hosts, USB-A ports and phones; estimated maximum total 0.475 A with hardware-limited analog current |
| High | 59.5 kΩ | 393 / 442 / 498 mA | Only when CC advertises ≥ 1.5 A; estimated maximum total 0.760 A |

Limits include the 1 % resistor tolerance. The limit acts on instantaneous current, so Default-mode power is rated by the instantaneous peak (3.5.5).

### 3.6.2 Analog supply current

| Jack and load | Analog input current, High mode, average (max quiescent ≈ 56 mA) | Instantaneous peak | Within the Default limit (157 mA min) at the peak? |
| --- | --- | --- | --- |
| 3.5 mm 16 Ω | 159 mA | 220 mA | No: Default mode 19.3 mW |
| 3.5 mm 32 Ω | 131 mA | 175 mA | No: Default mode 38.6 mW |
| 3.5 mm 60 Ω | 100 mA | 126 mA | Yes |
| 3.5 mm 300 Ω | 65 mA | 71 mA | Yes |
| 4.4 mm 32 Ω | 237 mA | 340 mA | No: Default mode 9.9 mW |
| 4.4 mm 60 Ω | 214 mA | 303 mA | No: Default mode 18.6 mW |
| 4.4 mm 120 Ω | 144 mA | 194 mA | No: Default mode 37.3 mW |
| 4.4 mm 300 Ω | 94 mA | 115 mA | Yes |
| 4.4 mm 600 Ω | 75 mA | 85 mA | Yes |

### 3.6.3 Regulator and power-part dissipation

Dissipation of every regulator and power part at its maximum load (R6-D3; R7-A P-14, P-15). The ME6211 SOT-23 power dissipation rating is 300 mW (R17), so U504 is fed through the R519 + R520 dropper; the ME6216 Pd (0.54 W, SOT23-3) and the LM27762 θJA (62.2 °C/W) are taken from their datasheets.

| Part | Load | Dissipation | Note |
| --- | --- | --- | --- |
| U504 3V3D ME6211, without dropper | 157 mA max | 0.27 W at 5.0 V, 0.31 W at 5.25 V | ME6211 SOT-23 power dissipation rating 300 mW (datasheet absolute maximum) → too close |
| U504 with R519 + R520 (2 × 2.2 Ω 0805 in series, 4.4 Ω) | 160 mA max (incl. the bleeders) | 0.20 W at 5.25 V, 0.24 W at 5.5 V; ≈ 56 mW per resistor (0.125 W rating) | LDO input ≥ 3.70 V at VBUS 4.40 V; inside the 300 mW rating up to 5.5 V (R7-A P-15) |
| U502 3V3M ME6211 | 84 mA | 0.16 W at 5.25 V | — |
| U303 1.3 V ME6216 | 60 mA | 0.12 W | Pd 0.54 W (SOT23-3) |
| U302 LP5907 | 17.5 mA | 0.03 W | — |
| U501 LM27762 | Worst clean load at VBUS 5.25 V and 5.5 V | 0.39 W at 5.25 V (θJA 62.2 °C/W → ≈ +24 °C), 0.46 W at 5.5 V | Was underestimated (R7-A P-14); copper pour under the exposed pad |
| U503 TPS2553 | 0.26 A | 9 mW normal; ≤ 2.6 W into a short for ≤ 10 ms before the latch/FAULT path | Its thermal shutdown protects |

3V3D load (maximum): CPLD 25, X201 40, X202 5, X203 10, DVCC 14.2, DVDD 60 (through U303), enables and node 1.7, I²C 1.5, bleeders 2.8 mA (R524 1.5 mA, R525 1.3 mA). C512 10 µF 0805 at the U504 output is the 3V3D bulk (R6-D8). Layout rule: copper pours on the LDO VIN, GND and VOUT pins; IR or thermocouple check at bring-up.

## 3.7 Protection

Protection has a fast hardware tier and a slow firmware tier, and both end in the same fault latch. The hardware tier works without firmware: the DC window detector (U601, U602 LM339DR, one window per leg directly on its filter node), a PGOOD-bad request (Q610) and a sustained TPS2553 overcurrent (D606) pull node F low, and the latch then pulls RAIL_EN, AMP_EN and AVCC_EN low through N-MOSFET kill switches (Q607, Q608, Q611) within ≈ 35 µs. The MCU drives each of these enables through a series resistor (R507, R618, R307), so a kill never shorts a GPIO. The firmware tier reads each leg through its own ADC network and sets the latch through the ANA_FAULT pin, acting on every leg and on each balanced pair difference; mandatory firmware rules cover faults present at power-up. The latch stays set until firmware clears it (FLT_CLR); firmware drops the enables, captures the cause and applies the recovery policy (3.7.2, Section 5). All transistors are JSCJ 2N7002 (C8545, Basic), Q601–Q611.

| Item | Value | Basis / notes |
| --- | --- | --- |
| DC sensing (hardware) | Each leg, taken at the jack side of its 0 Ω output link (R417–R420) → R601–R604 100 kΩ with C601–C604 1 µF to GND (τ = 100 ms; 84–111 ms with 1 % R and C −15 %/+10 % for tolerance and DC bias) → the filter node drives its own window directly: one comparator with IN− = node and IN+ = VTH+, one with IN+ = node and IN− = VTH− (no diode-OR, no MAX/MIN loads) | One filter and one window per leg; sensing at the jack side allows the detector test and the failed-output emulation (3.8); the trip level depends only on the divider, the comparator offset and the input bias current, all datasheet-guaranteed (R7-G G2) |
| Comparator | U601, U602 TI LM339DR (C7948, Extended, quad, open collector, SOIC-14) on the ±3.8 V rails, C612–C615 100 nF at V+ and V− of each package; eight comparators = four windows; all eight outputs wired-OR through R611 into node F; VTH+ = V+ × 10 kΩ/(R607 47 kΩ + R636 15 kΩ + R609 10 kΩ) = +0.517…+0.567 V (nominal 0.542 V); VTH− = V− × 10 kΩ/(R608 47 kΩ + R637 10 kΩ + R610 10 kΩ) = −0.531…−0.581 V (nominal 0.556 V) | Leg trip +0.43…+0.58 V and −0.52…−0.67 V → 0.43–0.67 V worst case (≈ 0.55 V nominal); the thresholds follow the rails; idle margin ≥ 0.43 V. Rail-stuck fault: TI states that “one input can exceed VCC, and the comparator provides a proper output state as long as the other input remains in the common-mode range” (SLCS006Z), which holds here because the threshold input of that comparator stays inside the range; no input goes more than 0.3 V below the comparator’s negative supply, because the nodes stay within the rails. LM339DR ESD ±2 kV HBM (SLCS006Z); drop-in LM339BIDR (C22416291, IB ≤ 50 nA, 2 kV ESD) if JLCPCB stocks it; the LM393BIDR of the R7-A…E round is replaced (R7-C PR-3, PR-10, PR-15; R7-G G2) |
| Worst-case inputs | Rails 3.789–4.014 V / 3.618–3.828 V; divider resistors 1 %; comparator offset ≤ 9 mV over temperature (5 mV at 25 °C); input bias ≤ 400 nA per input over 0–70 °C (250 nA at 25 °C), flowing out, two inputs per node through 100 kΩ → node shift 0…+81 mV; C601–C604 −15 %/+10 % (tolerance and DC bias); τ 84–111 ms; node F delay through R611 into C605 and latch capture (35 µs) | All values datasheet-guaranteed (TI SLCS006Z); no low-current diode forward voltage is involved (the BAV99 diode-OR of the R7-A…E round is removed) |
| Node F | LM339 open-collector outputs (eight, wired-OR) → R611 10 kΩ → node F; R612 47 kΩ pull-up to 3V3M; C605 100 nF from node F to GND; D605 1N4148WS clamp (anode GND, cathode F) | F is 3.3 V normally and ≈ −0.7 V on a DC fault (clamp current ≤ 0.31 mA); the 3.3 V domain never sees the −3.8 V comparator output. Power-on set: F starts at 0 V (C605), so the latch sets at attach, independent of the LM27762 PGOOD pin while that part is unpowered |
| Set-request stretch | After any PGOOD or fault pulse node F stays low 1.5–7.7 ms (R612 47 kΩ into C605) | Every PGOOD-bad pulse that turns Q610 on (≥ ≈ 10 µs) is latched |
| PGOOD set request | Q610 2N7002: gate = RAILS_OK_N (LM27762 PGOOD, high = not good), drain = node F, source = GND | PGOOD bad → RAILS_OK_N steps to 4.31–5.15 V and crosses 2.5 V in ≈ 1.0–1.3 µs (R505 10 kΩ to 5V_ANA, real loads included) → node F low → set request → the latch sets unless blanked, so an overload that drops PGOOD latches in ≈ 35 µs. C605 discharges through Q610 in a sub-µs ≈ 0.5 A pulse, fine for a 2N7002 (no series resistor there). A deep short that collapses 5V_ANA (so RAILS_OK_N cannot rise) is caught by the TPS2553 FAULT → D606 path within its 5–10 ms deglitch |
| Overcurrent and firmware set request | D606 1N4148WS: anode = node F, cathode → R638 1 kΩ → ANA_FAULT (TPS2553 FAULT, open drain, 5/7.5/10 ms deglitch; MCU PB5); the MCU ANA_FAULT pin is a floating input, and firmware sets the latch by switching it to open-drain output low (slow DC tier), never push-pull high | Same hardware kill path; R638 limits the C605 discharge into the MCU pin or the TPS2553 FAULT output (R7-C PR-16). In normal operation D606 has 0 V across it and ANA_FAULT still reads correctly at the MCU. With ANA_FAULT low, F sits at VF(D606) ≈ 0.5–0.7 V plus ≈ 55 mV across R638 (≈ 55 µA through R612) against the Q601 VGS(th) minimum of 1.0 V: ≈ 0.25–0.45 V margin, smaller at low temperature |
| Set path | Q601 inverter (gate = F) drives node S (R613 100 kΩ pull-up): S high = set request; Q602 (gate = S) sets the latch by pulling node A low | — |
| Blanking | Q603 holds S low while its gate is high; gate = PGOOD (RAILS_OK_N) through R617 1 MΩ with C606 100 nF: re-blanked 19–94 ms after PGOOD goes bad; the set path is armed 51–186 ms after PGOOD goes good, from a fully charged blanking capacitor (≥ 600 ms of rails off) | 5V_ANA 4.40–5.25 V, 2N7002 VGS(th) 1.0–2.5 V, R 1 %, C ±10 %. Blocks only the set request; it cannot clear an already-set latch (a clamp on the latch node would clear it and cause a hiccup). At start-up the PGOOD-bad request is blanked, so faults present at power-up are handled by the firmware start-up rules below (R7-C PR-4, PR-5) |
| Latch | Q604 and Q605 cross-coupled (drain A → gate B, drain B → gate A); R614, R615 47 kΩ pull-ups to 3V3M; no capacitor on the latch nodes; FLT = node B (goes high when the latch sets), also the gate of Q607, Q608 and Q611 | Capture with the 2N7002 capacitances at their maxima and VGS(th) up to 2.5 V: RAILS_OK_N rises in < 1 µs, then node S, node B and the FET transitions → ≈ 35 µs from PGOOD going bad to RAIL_EN, AMP_EN and AVCC_EN pulled low (budget 0.1 ms; measured) |
| Clear | Q606 (gate = FLT_CLR, R616 100 kΩ pull-down) pulls node B low | The latch stays set until firmware clears it; a clear does not stick while a set request is present, so FLT_CLR is given only ≥ 600 ms after ANA_EN or after the rails went off (blanking capacitor fully charged), and a second FLT_CLR at PGOOD good + 20 ms clears a spurious set from the rail inrush (R7-C PR-5); the MCU reads FLT (PB3, floating input) directly |
| Kill paths | While the latch is set Q607 pulls RAIL_EN low, Q608 pulls AMP_EN low and Q611 pulls AVCC_EN low (gates = node B, drains = the enable nodes, sources = GND), so the latch also turns off the DAC analog supply (LP5907); the MCU drives these nodes through R507, R618 and R307 (10 kΩ each), so Q611 pulls the AVCC_EN node low against 0.33 mA and no GPIO is shorted | Hardware; no firmware needed (R7-C PR-1 = R7-A P-1 = R7-D DG-3). At unplug the VBUS watchdog (4.10 V) also starts the shutdown from firmware (2.5) |
| Slow interlock | Q609 2N7002 pulls AMP_EN low; gate from RAILS_OK_N through R623 1 MΩ with C607 10 nF (C15195) to GND → on 1.9–9.4 ms after PGOOD goes bad, off 5–19 ms after it goes good | Keeps AMP_EN low whenever the rails are off (also after a firmware clear) but is not a fast kill: a short PGOOD glitch either latches or does nothing, so the amplifiers cannot be disabled and re-enabled without the latch; the slow gate ramp is moot because the latch acts first (R7-C PR-17) |
| Slow DC-sense network (firmware tier) | Per leg at the jack side of R417–R420 → MCU ADC: R624–R627 15 kΩ (leg series), R628–R631 10 kΩ to 3V3A, R632–R635 22 kΩ to GND, C608–C611 2.2 µF (≈ 1.3 µF effective at 1.6 V) → node = 1.556 V + 0.314 × leg (at 3V3A = 3.3 V), 0.29–2.85 V over every rail, 3V3A ±2 % and resistor corner; source impedance 4.7 kΩ; τ 6–10 ms (1 kHz attenuated ≈ 32 dB); leg DC load ≈ 104 µA (driven output, negligible); 3V3A monitor R639/R640 10 kΩ/10 kΩ with C616 100 nF to PC4 | 5 MCU ADC pins (PC0–PC4 on ADC2). The bias is on 3V3A, so it exists only while AVCC is on: no suspend current, no back-feed into unpowered legs, and it has collapsed before AMP_EN drops at power-down. The node stays ≥ 0.29 V from 0 V and ≥ 0.38 V from the 3.234 V minimum ADC reference, inside the 50 kΩ source limit; no clipping inside the rail range. With AMP_EN low and AVCC on (option B or a fault) the bias adds 3.3 mV (32 Ω) / 31 mV (300 Ω) / 61 mV (600 Ω) at the jack on top of the AN-9 VREF path; programming mode (AVCC on, rails off) back-feeds ≤ 0.41 mA into the unpowered output stage (bench only, acceptable). Error budget below (R7-F F3, R7-G G1, R7-H) |
| Slow DC tiers | Four tiers (table below) on max(¦each leg¦, ¦L+ − L−¦, ¦R+ − R−¦) of the calibrated readings: ≥ 1.0 V within 0.25 s, ≥ 0.5 V within 0.35 s, ≥ 0.3 V within 1.1 s, ≥ 0.2 V within 3 s; the latch is set through ANA_FAULT; armed whenever AVCC_EN is high, not only while AMP_EN is high (R7-B AN-9) | Acting on the pair differences covers opposite-sign legs, which give up to twice the leg voltage across a balanced driver (R7-F F5). Residual below T4 (≤ 0.198 V across the driver: ≤ 2.45 mW into 16 Ω, ≤ 1.23 mW into 32 Ω) accepted by the owner on 24 Sep 2026 (R7-G G3) |
| No false trip (hardware) | Full-scale content above 9.5 Hz worst case (6.2 Hz nominal) | With the LM339 windows and their worst case (R7-G G2) |

Worst-case detection envelope of the hardware tier (removal = RAIL_EN, AMP_EN and AVCC_EN pulled low; the detector, the node F delay and the latch capture are included):

| DC on a leg | Removal time, nominal | Removal time, worst case | Requirement |
| --- | --- | --- | --- |
| 1.0 V | 80 ms | 125 ms | ≤ 250 ms (50 % margin) |
| 1.5 V | 46 ms | 67 ms | — |
| 2.0 V | 33 ms | 47 ms | ≤ 100 ms (53 % margin) |
| 2.19 V (full-scale digital DC at 1.55 Vrms) | 30 ms | 42 ms | — |
| 3.83–4.01 V (rail-stuck) | — | 19–23 ms | — |

Requirement (hard, worst case): a DC fault ≥ 2 V on any leg is removed within 100 ms and ≥ 1 V within 250 ms. The hardware tier trips at 0.43–0.67 V on a leg (worst case; ≈ 0.55 V nominal); the firmware tiers cover DC down to 0.11 V nominal (≤ 0.198 V worst case across a balanced driver). Opposite-sign legs below the hardware trip can put up to 1.15 V across a balanced driver undetected by hardware; the firmware tiers cover this case.

Firmware protection rules (mandatory, zero cost): (1) FLT interrupt: within ≤ 50 µs drive AMP_EN, RAIL_EN and AVCC_EN low (mirroring the latch) and AUD_EN and RESETB low (this stops MCLK back-powering VCCA through XI and RESETB injection), then capture the cause from the pre-trip readings in the ring buffer, then ANA_EN low; the IWDG is enabled, so an MCU reset floats all outputs and the pull-downs switch every domain off (R7-C PR-6, R7-D DG-6). (2) Start-up: fault if PGOOD is not good within 20 ms of RAIL_EN (RAIL_EN and ANA_EN low, start-up fault reported); from RAIL_EN until the set path is armed, sample the four DC-sense channels at 1 kHz and drop RAIL_EN directly if any leg exceeds 0.5 V (bounds a stuck leg to ≈ 10 mJ); first power-up of every new board without headphones, and DMM each leg before connecting any (PR-4). (3) FLT_CLR only ≥ 600 ms after ANA_EN or after the rails went off; a second FLT_CLR at PGOOD good + 20 ms; AMP_EN ≥ 250 ms after PGOOD good, after the latest arming time of 186 ms (PR-5). (4) Cause classification from the pre-trip rolling 0.5-s averages of the DC-sense channels plus the event history; events that match neither the DC nor the overload pattern are logged as “ambiguous” and counted toward the DC three-strike limit (conservative, deliberate; R7-C PR-8, R7-F F6). (5) FLT (PB3), RAILS_OK_N (PB4) and ANA_FAULT (PB5) are always floating inputs, never with internal pulls, also in low-power code; ANA_FAULT may be driven open-drain low, never push-pull high; unit test on the GPIO configuration table (PR-7, DG-9). (6) AVCC_EN only while AMP_EN is high and the rails are good (R7-B AN-9). (7) Stream DC guard in the MCU before the DAC (R7-H H8): no zero crossing for 25 ms and |mean| ≥ 0.25 FS → output zeroed at once; 0.2-s mean ≥ −30 dBFS → soft mute; unmute after 1 s below −40 dBFS; DC in program material therefore mutes (reported to the host log) instead of latching. (8) Both LEDs off in suspend. A quick replug may start with the latch cleared, which is harmless because the enables are pulled down (PR-18).

Slow DC-sense network error budget per leg (worst case, after the factory zero and a power-up auto-zero; R7-F F3, R7-G G1): pin leakage 1 µA × 15 kΩ = 15 mV; ADC 2 LSB after auto-zero 5.1 mV; in-session drift (±15 °C, 0.3 % of the bias) 14.8 mV; 3V3A/3V3M ratio after the monitor correction 4.8 mV; sampling charge 0.1 mV; aliasing residual with dithered sampling 4 mV → ≤ 44 mV per leg worst case (23 mV RSS) and ≤ 88 mV on a pair difference (32 mV RSS); no clipping inside the rail range. Firmware rules: ADC2 scans CC1, CC2, the four DC-sense channels and the 3V3A monitor in a fixed order with 239.5-cycle sampling, discarding the first conversion after each channel switch; each channel is sampled at 1 kHz with ±20 % dithered timing; every reading is corrected ratiometrically with the 3V3A monitor; a zero reading of each leg is taken at every power-up while muted and accepted only within ±30 mV of the stored factory zero (otherwise a fault is reported). Side effects: 3V3A load +0.87 mA (bias 0.70 mA, monitor 0.17 mA); the suspend budget loses the network term (≈ 2.30 mA).

Firmware DC tiers (R7-F F5, R7-H): each tier acts on max(|each leg|, |L+ − L−|, |R+ − R−|) of the calibrated readings and sets the latch through ANA_FAULT (unchanged path); armed whenever AVCC_EN is high:

| Tier | Window mean | Threshold | Removal within | No false trip for full-scale content above |
| --- | --- | --- | --- | --- |
| T1 | 0.2 s | 1.0 V | 0.25 s | 7.0 Hz |
| T2 | 0.3 s | 0.5 V | 0.35 s | 9.3 Hz |
| T3 | 1 s | 0.3 V | 1.1 s | 4.7 Hz |
| T4 | 2 s | 0.11 V (worst-case trip 0.022–0.198 V differential, 0.066–0.154 V per leg) | 3 s | 5.8 Hz |

Accepted envelope below the hardware trip (worst case including the 88 mV error): up to 1.15 V for ≤ 0.25 s → 20.7 mJ (16 Ω) / 10.4 mJ (32 Ω); just below T1 (1.09 V) for ≤ 0.35 s → 25.9 / 12.9 mJ; just below T2 (0.59 V) for ≤ 1.1 s → 23.8 / 11.9 mJ; just below T3 (0.39 V) for ≤ 3 s → 28.2 / 14.1 mJ; residual below T4 ≤ 0.198 V across the driver → ≤ 2.45 mW (16 Ω) / 1.23 mW (32 Ω) continuous, accepted by the owner (R7-G G3). Every sub-hardware-trip case therefore stays ≤ 30 mJ at 16 Ω and ≤ 15 mJ at 32 Ω.

### 3.7.1 Energy at the headphone terminals and back-powering

Terminal energy E = V²/R × (detector + node F + latch time) + the energy after the trip, at the worst case of the envelope above, for a hardware rail-stuck failure in High mode and for an unbalanced digital DC signal on a healthy output stage (after-trip energy with the rail capacitors at +10 % and the maximum rail voltage 0.52 mJ; bound ≤ 0.6 mJ kept):

| Load | Rail-stuck failure, High mode, worst case incl. after-trip (nominal) | Digital DC on a healthy output stage, unbalanced, hardware only, worst case (V at the leg, removal time, energy) |
| --- | --- | --- |
| 16 Ω | 21.4 mJ (15.5 mJ); limit 30 mJ | 1.81 V (swing-limited), 52.8 ms, 10.8 mJ |
| 32 Ω | 11.0 mJ (8.0 mJ); limit 15 mJ | 2.17 V (swing-limited), 42.4 ms, 6.3 mJ |
| 60 Ω | 6.1 mJ (4.5 mJ) | 2.19 V (full scale), 42.0 ms, 3.4 mJ |
| 300 Ω | 1.6 mJ (1.2 mJ) | 2.19 V (full scale), 42.0 ms, 0.7 mJ |

Balanced digital DC (legs driven to ±x) is larger and is not covered by the rail-stuck requirement: if the hardware alone acts, 16 Ω (current-limited, 2.32 V across the driver) 97 ms, 32.8 mJ, and 32 Ω (3.62 V) 52.8 ms, 21.7 mJ. The stream DC guard (5.1) zeroes a large digital DC within ≈ 30 ms (≈ 10 mJ at 16 Ω, ≈ 12 mJ at 32 Ω); this is stated as a risk (7.1). After the trip, back-powering and the test criterion:

| Item | Value | Basis / notes |
| --- | --- | --- |
| Default mode | In Default mode a rail-stuck fault into 16–32 Ω exceeds the TPS2553 limit, so the PGOOD/overcurrent path latches first | The energy table above is the High-mode worst case |
| After the trip | Rails disabled; both rails’ capacitors (≈ 30.6 µF per rail) with C +10 % at the maximum rail voltage = 0.52 mJ, plus DAC back-power into a failed stage ≤ 1 µJ → after-trip bound ≤ 0.6 mJ: the most a faulty output stage can deliver after the rails are disabled | Included in the energy table. The load discharges it; the LM27762 50 kΩ discharge is not relied on. OPA1622 shutdown is not treated as isolation (TI: “do not consider the amplifier output to be high-impedance”) |
| Back-powering | Latch trip on a healthy stage: Q611 kills AVCC_EN with the rails and the FLT interrupt drops AUD_EN and RESETB, so MCLK cannot back-power VCCA through XI; 3V3A collapses within 5.5 ms worst case; residual back-power ≤ 1.7 mA per DAC pin (806 Ω − 11 %). After an output-stage failure the I/V converters can lose the DAC pins for up to ≈ 5.5 ms; the fitted clamps D405–D408 then hold them within ≈ −0.40 V … AVCC + 0.40 V (≤ 4.4 mA per pin), a small residual (3.5.1). Controlled power-down: MCLK stops and AVCC goes off first while RESETB stays high, and VREF follows AVCC, so the pins stay inside GND…AVCC; at a sudden unplug VREF lags by ≤ 15 mV (C421 33 pF) | Inside the OPA2210 10 mA input-current rating (R15: absolute maximum ratings and section 7.3.5); ES9018K2M absolute maximum GND < Vout < AVCC with no margin given; the four DAC pins are probed at the R423–R426 DAC-side pads against AVCC_L/AVCC_R and must stay within −0.4 V … AVCC + 0.4 V (3.8; R7-F F4) |
| Test criterion | Hard, worst case: a DC fault ≥ 2 V on any leg is removed within 100 ms and ≥ 1 V within 250 ms; energy at the headphone terminals ≤ 30 mJ at 16 Ω and ≤ 15 mJ at 32 Ω for a rail-stuck failure in High mode, including the after-trip energy (worst-case estimates 21.4 / 11.0 mJ), and for every sub-hardware-trip case of the firmware tiers (worst 28.2 / 14.1 mJ) | Coverage: detector test, failed-output emulation with dummy loads and V²/R integrated, digital DC test signals (unbalanced and balanced opposite-sign) and SPICE of a failed output stage (3.8). Risks kept: up to 21.4 mJ into a 16 Ω IEM in High mode; balanced digital DC up to 32.8 mJ at 16 Ω if the hardware alone acts (≈ 10 mJ with the stream DC guard); v2 option: a fast “leg within 0.5 V of a rail” detector (≈ 1 ms) |

### 3.7.2 Thermal and overload

| Case | Estimate | Basis / notes |
| --- | --- | --- |
| Analog input power | ≤ 1.06 W (Default), ≤ 2.49 W (High) | Bounded by the TPS2553 limit |
| Board worst case | ≈ 3.8 W (3.77 W) | High mode, overload |
| 4.4-to-3.5 mm adapter (leg− shorted) | ≈ 0.35 W in the shorted amplifier + ≈ 0.1 W in the other → ≈ 0.45 W per package, +21 °C | θJA 47.6 °C/W; one amplifier per package, inside TI’s continuous-short rating |
| 4.4 mm plug shorted leg+ to leg− | ≈ 0.55 W per package (both amplifiers of one package) | Outside TI’s one-amplifier continuous-short rating → bench survival test |
| Overload response and recovery | Current limit → rails droop → PGOOD bad → Q610 sets the latch (a sustained TPS2553 FAULT sets it through D606) → within ≈ 35 µs the latch kills RAIL_EN, AMP_EN and AVCC_EN and stays set. The FLT interrupt first drives AMP_EN, RAIL_EN, AVCC_EN, AUD_EN and RESETB low, then classifies the cause from the pre-trip rolling 0.5-s averages of the four DC-sense channels in the ring buffer plus the event history, ANA_FAULT, RAILS_OK_N and whether firmware itself set the latch: DC fault, overload, or “ambiguous” when neither pattern matches (counted toward the DC three-strike limit); unavailable readings count as a DC fault. Overload: soft mute, volume ceiling −3 dB per event (down to −12 dB), clear after ≥ 1 s (FLT_CLR only ≥ 600 ms after the rails went off, second clear at PGOOD good + 20 ms), ceiling kept until replug; overloads count toward the 3-strike stop only once the ceiling has reached −12 dB. DC or ambiguous fault: clear after 2 s; 3 within 10 s → latched until USB replug; the red LED blinks | Overload latches (no restart when PGOOD recovers) and needs the firmware clear (Section 5). Default mode at full volume into 60 Ω balanced needs three −3 dB steps; the previous rule could lock the board off within ≈ 5 s (R6-C1). FLT during a controlled power-down or before the first FLT_CLR is expected and masked (5.2). TPS2553 FAULT deglitch 5/7.5/10 ms |

## 3.8 Test and DFT

| Item | Provision | Notes |
| --- | --- | --- |
| Test pads | TP701–TP732 (32 copper pads), allocated in the table below: rails, VREF and AVCC_L, clocks and I²S, enables and protection nodes, USB sensing, 4 GND and the PROG strap | — |
| Debug | J201 CH32V307 pads (WCH-LinkE): 3V3 sense, SWDIO, SWCLK, NRST, GND, plus BOOT0-to-3V3 and USART1 TX pads; J202 CPLD 2-wire Compact-JTAG pads (J-Link or AGM DAP-Link): 3V3D sense, JTCK (pin 25), JTMS (pin 24), NRST (pin 4), GND | Programming order: MCU, then the CPLD in programming mode, then calibration (2.5, Section 5) |
| I²S header | J703 1 × 6, 2.54 mm, DNF: MCLK monitor output, BCLK, LRCLK, SDATA, GND, 3V3 reference; the MCLK pin only through R703 1 kΩ (DNF, fitted with J703) | In test mode an external source drives only BCLK, LRCLK and SDATA (CPLD I²S outputs tristated) and the DAC stays in asynchronous mode on X201; the QA403 digital output is 3.3 V-class: use series resistors, power the board first |
| Indicators | D705 KT-0805G green (status), D706 KT-0603R red (fault); R701 1 kΩ (green, ≈ 0.2–0.7 mA) and R702 2.2 kΩ (red, ≈ 0.4–0.7 mA) from 3.3 V GPIO (PC6, PC7) | No Basic 0603 green LED exists; R701 lowered for the KT-0805G Vf of 2.6–3.1 V (R7-E BM-10) |
| Assembly | JLCPCB Economic PCBA, one side; all SMT on top except the 4.4 mm jack (7 through-hole joints; 781 SMT joints and 267 placed parts per board); 3 fiducials, 4 × M3 holes; 4 layers, ≤ 80 × 60 mm | 8 leadless parts per board for X-ray: CPLD, DAC, 2 × OPA1622, LM27762, 3 oscillators. J702 is marked assembly difficulty “high” by JLCPCB (allowed in Economic PCBA; kept, R7-E BM-6) |

Test-pad allocation (TP701–TP732, 32 pads):

| Pads | Group | Nets (in pad order) | Used by |
| --- | --- | --- | --- |
| TP701–TP708 | Supplies | TP701 5V_SYS, TP702 5V_ANA, TP703 3V3M, TP704 3V3D, TP705 3V3A, TP706 1V3, TP707 +3.90 V, TP708 −3.72 V | Rails and power; TP707/TP708 are the sources for the failed-output emulation |
| TP709–TP710 | VREF and AVCC references | TP709 VREF, TP710 AVCC_L | VREF and AVCC_L references; the four DAC pins themselves are probed at the R423–R426 DAC-side pads (3.8) |
| TP711–TP717 | Clocks and I²S | TP711 MCLK, TP712 family-clock node, TP713 OSC48_EN, TP714 OSC44_EN, TP715 BCLK, TP716 LRCLK, TP717 SDATA | Jitter, clock handover |
| TP718–TP724 | Enables and protection | TP718 AMP_EN, TP719 RAIL_EN, TP720 AVCC_EN, TP721 RAILS_OK_N, TP722 FLT, TP723 ANA_FAULT, TP724 node F | Protection tests, short PGOOD pulses, power sequence |
| TP725–TP727 | USB sensing | TP725 VBUS_SENSE, TP726 CC1, TP727 CC2 | USB tests, watchdog latency |
| TP728–TP732 | Ground and PROG | TP728, TP729, TP730, TP731: GND; TP732 PROG (MCU PC13, internal pull-up; shorted to GND at reset for the programming mode) | Probe and DMM returns; programming mode (2.5) |

End-of-line and bring-up tests:

| # | Test | Method and pass criterion |
| --- | --- | --- |
| 1 | End-of-line calibration | Programming first: MCU firmware (WCH-LinkE on J201), then the CPLD in programming mode (J-Link or AGM DAP-Link on J202). Then QA403 + scripts: full-scale level per leg → master trim to 1.55 Vrms ± 0.1 dB and per-channel fine trim, written to flash; zero of each slow DC-sense channel calibrated and stored; THD+N; dynamic range, recorded with the analyzer loopback floor. DC offset of every leg (100 %) with a DC-capable DMM (≥ 4½ digits, SCPI over USB for the automated test), because the QA403 inputs are AC-coupled |
| 2 | Detector test | External lab source on one jack contact with its link (R417–R420) removed, at 1.0 V, 2.0 V and 3.8 V; tests detection and the control signals only: FLT high, RAIL_EN, AMP_EN and AVCC_EN low within the worst-case envelope |
| 3 | Failed-output emulation | The board’s own +3.90 V (TP707) or −3.72 V (TP708) is switched onto a jack contact (link removed) by an external MOSFET switch, bypassing the amplifier enable; dummy loads 16 Ω and 32 Ω; scope the terminal voltage and integrate V²/R until it stays below 0.1 V. Pass: FLT high, RAIL_EN, AMP_EN and AVCC_EN low within the worst-case envelope (no allowance beyond the hard 100 ms / 250 ms / energy limits); total energy ≤ 30 mJ (16 Ω) / ≤ 15 mJ (32 Ω). Also: digital DC test signal on a healthy output stage and SPICE of a failed output stage |
| 4 | Short PGOOD pulses | Emulate the real open-drain release: pull OUT+ below 95 % with a switched load, or release the RAILS_OK_N node through an open-drain FET, for 5 µs, 20 µs, 100 µs and 1 ms while the MCU keeps requesting AMP_EN; never force PGOOD with a low-impedance generator. Pass: either nothing happens or the latch sets; never an un-latched AMP_EN dropout (R7-F F1, R7-H H1) |
| 5 | Firmware DC tiers and stream DC guard | Test mode with the stream DC guard bypassed: DC per leg and opposite-sign DC on a balanced pair above each tier threshold (1.0 / 0.5 / 0.3 / 0.2 V) → latch within 0.25 / 0.35 / 1.1 / 3 s, recorded as a DC fault. Stream DC guard: a DC test file mutes the output (no latch) and the event is reported to the host log (R7-F F5, R7-H H8) |
| 6 | Overload (30 min each, High mode, maximum volume) | 4.4 mm 16 Ω; 4.4-to-3.5 mm adapter; 3.5 mm 16 Ω; 60 Ω balanced full scale — all parts within ratings; the latch sets and recovery needs the firmware clear (FLT_CLR); the board recovers. Near the Default-mode limit: scope the rails and the output for clipping before a trip |
| 7 | USB | Current in every state (USB power meter), repeating the Stage 2 limits on the full board: attached-not-configured ≤ 100 mA, configured digital ≤ 262 mA; suspend current against time from the last bus activity (≤ 2.5 mA by 10 ms, LEDs off); attach inrush charge (≈ 42.5 µC estimate); VBUS droop at each domain enable; Default and High mode on a Type-C 1.5 A source and on a USB-A port; a CC change from ≥ 1.5 A to Default handled within tSinkAdj |
| 8 | Pop | Pop = peak of the transient at the jack beyond the settled DC step, band-limited 20 Hz–20 kHz (AC-coupled analyzer input); the DC before and after each transition is bounded separately by the offset limits (±10 / ±20 mV). ≤ 5 mV peak for controlled transitions (power-on after configuration, suspend/resume, rate change, fault recovery); ≤ 20 mV peak at unplug/brownout (target); 32 Ω and 300 Ω. The AVCC-on / RESETB step is measured with orders A and A′ and option B, with the I/V outputs and legs scoped during the AVCC ramp with RESETB low; the power-down pop with the adaptive AMP_EN (≈ 4.5 mV typical, ≈ 12.7 mV worst case at 300 Ω) is measured; the ≤ 5 mV target is at risk until measured (R7-B AN-3, R7-H H2, H5) |
| 9 | Jitter | J-test for both families (exact integer clock ratios from the CPLD PLL), including 352.8 and 384 kHz; BCLK duty cycle measured at every rate |
| 10 | Clock handover | Scope both enables and the shared node at every family change: never both enabled, node quiet for ≥ 1 ms before the new oscillator starts, disable time measured, no runt pulses, PLL relock, all 8 rates bit-perfect; on the shared-node board and on the mux board (Stage 2), repeated on the main boards |
| 11 | DAC pins against AVCC | Probe DACL, DACLB, DACR and DACRB (at the R423–R426 DAC-side pads) against AVCC_L/AVCC_R during (a) the failed-output emulation with dummy loads on main board 1 and (b) hot unplug at full load, and at power-up, power-down and latch trip. Controlled transitions: inside GND…AVCC. With the clamps D405–D408 fitted, all four pins must stay within −0.4 V … AVCC + 0.4 V in (a) and (b); record the 3V3A decay with the clamps fitted; replaces the VREF-only check (R7-F F4) |
| 12 | Rails and power | Negative-rail headroom at VBUS 4.60 V and 4.40 V (predicted ≥ +0.05 V worst case from a 4.75 V host with the 70 mA envelope); 3V3A below 0.3 V within 5.5 ms of AVCC_EN low; 3V3D and 1V3 decay with the bleeders; LM27762 VIN ripple at light load (R7-A P-22); hot-plug VBUS ring on C102 with a USB-A-to-C cable into a live port (fit D102 only above 5.8 V, R7-A P-7); LDO temperatures by IR or thermocouple (3.6.3); output power tables in both modes on the bench, with the OPA1622 clean swing at ±3.8 V (R7-B AN-2) |
| 13 | Stability | Each leg with the 0 Ω links, tested with 1 nF plus long cables; an isolation resistor cannot fix capacitive-load instability, so 2.2–4.7 Ω is only a last resort that breaks the ≤ 0.5 Ω output-impedance target (R7-B AN-7); each I/V output checked for oscillation |
| 14 | VREF null point | At idle the DC at each I/V output ≈ VREF, confirming the DAC zero-current point; decides R433 on the first boards (1 kΩ for 1.703 V, 0 Ω for 1.650 V) |
| 15 | DC-sense network | Power-up zero reading within ±30 mV of the stored factory zero; ratiometric correction with the 3V3A monitor across 3V3A ±2 %; dithered 1 kHz sampling with a 1 kHz tone on the legs (no aliased offset); per-leg error ≤ 44 mV and pair error ≤ 88 mV worst case against a DMM (R7-F F3) |

Stage 2 boards (mandatory, the first hardware and the layout gate, 5.1): two JLCPCB-built boards with the frozen pin map: USB-C and ESD, 3V3M/3V3D LDOs with the dropper and bleeders, CH32V307 with its crystal, AGRV2KQ32 (with FB202), X201/X202/X203 with R210–R218 (shared node on one board, mux U205 fitted on the other), TPS2553 and LM27762 with the real latch and protection section on dummy loads, plus the ES9018K2M with its real supplies (LP5907 3V3A, ME6216 1V3, DVCC ferrite) in place of the 3V3A and 1V3 dummy loads, a passive check output (each DAC pin to a test pad with 10 kΩ to GND: voltage-mode check of lock and of a sine by FFT), the I²S/MCLK test header, J201/J202 and current-measurement points on 3V3M and 3V3D. Estimate ≈ US$93 ≈ 2,940 NTD for two boards (was ≈ US$90 ≈ 2,860 NTD without the DAC), outside the board budget. Stage 1 (desk setup on bought development boards) is optional: only if the owner can get the boards; nothing gates on it.

Stage 2 gates (all before layout): USB enumeration and UAC2 streaming at all 8 rates with feedback; bit-perfect path to the DAC (lock status, FFT of the check output); SPI 36 MHz link with the real CPLD capture (R7-D DG-12); PLL lock at both family clocks (DG-11); clock handover break-before-make; attached ≤ 100 mA and configured digital ≤ 262 mA; suspend ≤ 2.5 mA within 10 ms including the DAC-first power-down with the real LP5907 and the adaptive AMP_EN timing on the 3V3A monitor (DG-10, R7-A P-16); watchdog latency < 5 µs; AUD_EN cycled 100× at VBUS 4.40 V and 5.25 V (P-21); 3V3D and 1V3 decay with the bleeders; RAILS_OK_N rise and latch capture with the open-drain PGOOD pulse test; blanking and arming times; programming mode and CPLD programming through J202 (DG-1, DG-4); DC-sense auto-zero, dithered sampling and ratiometric correction with the 3V3A monitor (R7-F F3). Main-board bring-up adds: first power-up without headphones (PR-4); hot-plug VBUS ring test (P-7); negative-rail headroom at VBUS 4.60 V and 4.40 V; AVCC-on / RESETB pop with orders A and A′ and option B, with the I/V outputs and legs scoped during the AVCC ramp with RESETB low (AN-3, R7-H H2, H5); DAC pins probed during the failed-output emulation and hot unplug (R7-F F4); balanced opposite-sign DC and stream-guard tests; capacitive-load test with 1 nF and long cables (AN-7); LM27762 VIN ripple (P-22); OPA1622 clean swing at ±3.8 V (AN-2); jack continuity from samples before layout (AN-1).

# 4. Signal plan

The MCU–CPLD interface is ours to define (Section 5). Pin numbers follow the proposed MCU and CPLD pin maps (3.2, 3.3), which are frozen before the Stage 2 board and the schematic. Test pads follow the allocation table in 3.8.

| Signal | From → to | Type / level | Pin / notes |
| --- | --- | --- | --- |
| USB D+, D− | J101 ↔ CH32V307 USBHS | USB 2.0 HS | PB7 (59) D+, PB6 (58) D−; 90 Ω pair; A6/B6 and A7/B7 tied at J101 |
| MCU → CPLD audio link | CH32V307 SPI1 master (DMA) → AGRV2KQ32: LINK_SCK, LINK_MOSI, LINK_FRAME; LINK_MISO back (FIFO level during the frame); registers over SPI; LRCLK back to MCU PD2 | 3.3 V LVCMOS; 36 MHz, CPOL = 0 | Per the MCU–CPLD interface specification (open item); frames start with a 16-bit header; MCU side GPIO-low or input while AUD_EN is low; 24.576 Mbit/s payload at 384 kHz (68 % of 36 MHz); I²S is not used for the link; second lane reserved (SPI2 PB12–PB15 → two spare CPLD pins) |
| LINK_SCK | MCU PA5 (21) → R224 33 Ω → CPLD pin 9 (global network through ALTCLKCTRL) | 3.3 V output (MCU master) | CPLD weak pull-down; the CPLD writes its FIFO in the SCK domain |
| LINK_FRAME, LINK_MOSI, LINK_MISO | FRAME: PA4 (20) → CPLD 11; MOSI: PA7 (23) → R225 33 Ω → CPLD 10; MISO: CPLD 12 → PA6 (22) | 3.3 V; FRAME active high (idle low) | CPLD weak pull-downs on FRAME and MOSI; low or input while AUD_EN is low |
| MCLK | X201 → R203 33 Ω → DAC XI (pin 7) | 80 MHz CMOS | DAC only (no CPLD branch); X201 pin 1 = MCLK_EN; XI trace ≤ 20 mm; duty measured at XI; TP711 |
| J703 MCLK monitor | MCLK → R703 1 kΩ (DNF, fitted with J703) → J703 | 80 MHz monitor output | Monitor only: an external source never drives MCLK |
| Family clock (shared node) | X202 → R210 and X203 → R211 → one node → CPLD PIN_1 (IO_GB, the only dedicated clock pin) → PLL; R214 10 kΩ pull-down | 24.576 or 22.5792 MHz CMOS | Only one family oscillator drives the node at a time; low while both are off; both oscillators next to CPLD pin 1; TP712; DNF mux option U205/R215 with the R216/R217 input pull-downs |
| BCLK, LRCLK, SDATA | CPLD pins 18/19/20 → R204/R205/R206 33 Ω → DAC DATA_CLK (27), DATA1 (26), DATA2 (25); J703 header | 3.3 V output | SDATA and LRCLK change on the falling BCLK edge; held low during rate-family changes; tristated in test mode (then driven by the external source); TP715–TP717 |
| OSC48_EN | CPLD pin 2 → X202 enable (pin 1) | 3.3 V output; R212 2.2 kΩ pull-down | X202 off by default until the MCU enables it through an SPI register; TP713 |
| OSC44_EN | CPLD pin 3 → X203 enable (pin 1); U205 select if the mux is fitted | 3.3 V output; R213 2.2 kΩ pull-down | X203 off by default; never high together with OSC48_EN (break-before-make timed by the MCU); TP714 |
| I²C SCL, SDA | CH32V307 I2C2 PB10 (29) SCL, PB11 (30) SDA ↔ ES9018K2M SCL (3), SDA (4) | Open drain; 2.2 kΩ to 3V3D | DAC 7-bit address 0x48 (written 0x90; ADDR pin 5 to DGND through R306); I2C2 disabled while 3V3D is off; 9-clock bus recovery |
| DAC RESETB | CH32V307 PA8 (41) → R308 1 kΩ → DAC RESETB (28) | 3.3 V output; R305 pull-down | Stays high (DAC muted) until 3V3A < 0.3 V in the controlled power-down; dropped first in the fault and unplug interrupts; injection < 3 mA |
| CC1_ADC, CC2_ADC | CC1, CC2 → 10 kΩ → MCU ADC | Analog | PA2 (16), PA3 (17) on ADC2; sampled at least every 5 ms while configured; TP726, TP727 |
| VBUS_SENSE | R105/R106 divider → MCU ADC | Analog; watchdog 4.10 V | PA1 (15), ADC1 continuous with the analog watchdog; TP725 |
| DC_SENSE_L+ | Leg L+ at the jack side of its output link → 15 kΩ series resistor (one of R624–R627) → ADC node with 10 kΩ to 3V3A (R628–R631), 22 kΩ to GND (R632–R635) and 2.2 µF (C608–C611) → MCU ADC | Analog: 1.556 V + 0.314 × leg (0.29 … 2.85 V at every corner), corrected with the 3V3A monitor | PC0 (8), ADC2 at 1 kHz with dithered timing; slow DC tier (3.7); zero calibrated at the end-of-line test and re-read at every power-up |
| DC_SENSE_L−, DC_SENSE_R+, DC_SENSE_R−; 3V3A_MON | Legs L−, R+ and R−, each with a network as DC_SENSE_L+; 3V3A → R639/R640 10 kΩ/10 kΩ with C616 100 nF | Analog, as DC_SENSE_L+; 3V3A_MON ≈ 1.65 V | PC1 (9), PC2 (10), PC3 (11) and PC4 (24), ADC14 on ADC2; slow DC tier; 3V3A_MON also times AMP_EN at power-down |
| MCLK_EN | CPLD pin 21 → X201 enable (pin 1) | 3.3 V output; R218 2.2 kΩ pull-down | X201 off by default and during CPLD configuration; high ≥ 2 ms after 1V3 is up (SPI register); low before AVCC goes off |
| DAC_LOCK, LRCLK_FB, CPLD_IRQ | DAC GPIO2 (23) → MCU PC9 (40); CPLD pin 13 (LRCLK) → MCU PD2 (54); CPLD pin 14 → MCU PC8 (39) | 3.3 V inputs to the MCU | DAC_LOCK: default DPLL lock-status output; LRCLK_FB: TIM3_ETR, latched at SOF (feedback and family-change check); CPLD_IRQ optional |
| AUD_EN | MCU PC10 (51) → R521 10 kΩ → U504 CE | 3.3 V output; R508 pull-down | AUD_EN off ≥ 100 ms before re-enabling |
| AVCC_EN | MCU PC11 (52) → R307 10 kΩ → U302 EN; Q611 kill (drain) while the latch is set | 3.3 V output; R303 pull-down | TP720; pulled low in hardware when the latch sets (0.33 mA through R307, no GPIO contention); high only while AMP_EN is high and the rails are good |
| ANA_EN | MCU PC12 (53) → U503 EN | 3.3 V output; R513 pull-down | Before RAIL_EN (≥ 10 ms) |
| ILIM_HI | MCU PA15 (50) → Q501 gate | 3.3 V output; R511 pull-down | Only with ≥ 1.5 A advertised |
| ANA_FAULT | U503 FAULT → MCU PB5 (57); also R638 1 kΩ → D606 cathode (latch set from node F); firmware output to set the latch | Open drain; R512 pull-up to 3V3M; MCU pin floating input or open-drain output low (never push-pull high) | EXTI5; TP723 (node F on TP724); firmware drives it low to set the latch (slow DC tier) |
| RAIL_EN | MCU PB8 (61) → R507 10 kΩ → LM27762 EN+/EN−; Q607 kill | 3.3 V; R506 pull-down | TP719; ≥ 10 ms after ANA_EN; low after AMP_EN in the controlled power-down (2.5) |
| RAILS_OK_N | LM27762 PGOOD → R522 10 kΩ → MCU PB4 (56); Q610 gate; Q609 gate through R623 1 MΩ (C607 10 nF to GND: on 1.9–9.4 ms after PGOOD goes bad); blanking RC | Open drain; R505 10 kΩ pull-up to 5V_ANA | Floating input, EXTI4 on both edges; TP721 |
| AMP_EN | MCU PB9 (62) → R618 10 kΩ → both OPA1622 EN (pin 8); Q608 kill (latch), Q609 slow interlock | 3.3 V; R421 pull-down | TP718; ≥ 250 ms after PGOOD good; low at power-down once 3V3A_MON reads < 0.1 V (deadline t0 + 5.8 ms) |
| FLT_CLR | MCU PA11 (44) → Q606 gate | 3.3 V output; R616 pull-down | ≥ 600 ms after ANA_EN or rails off; second clear at PGOOD good + 20 ms |
| FLT | Latch node B → MCU PB3 (55); Q607, Q608 and Q611 gates | 3.3 V input (interrupt); high when the latch sets | Floating input, EXTI3, highest priority; TP722 |
| LED × 2 | MCU PC6 (37) → R701 1 kΩ → D705 green; PC7 (38) → R702 2.2 kΩ → D706 red | 3.3 V output | Status and fault; both off in suspend |
| Debug SWDIO, SWCLK, NRST; recovery pads | J201 ↔ CH32V307 PA13 (46), PA14 (49), NRST (7); BOOT0 pad; USART1 TX PA9 (42) pad, RX PA10 (43) optional | 2-wire debug | The 3V3 pad on J201 is sense only (the probe must not power 3V3M) |
| CPLD Compact-JTAG JTCK, JTMS, NRST; PROG | J202 ↔ AGRV2KQ32 JTCK (25), JTMS (24), NRST (4); PROG pad TP732 → MCU PC13 (2) | 2-wire Compact-JTAG (AGM Supra); PROG strap with internal pull-up | J-Link or AGM DAP-Link only; the J202 3V3D pin is sense only, the CPLD is powered by the programming mode (2.5) |

# 5. Firmware requirements

The USB firmware and the CPLD logic are our own (owner decision, 24 Sep 2026). The firmware is our own USB Audio Class 2.0 implementation on the CH32V307 high-speed USB port, built on WCH’s register-level USBHS device examples; the chip’s EVT package has high-speed device examples for HID, CDC serial, mass storage and network adapters, but no audio example. TinyUSB (MIT) is an alternative only if its CH32V307 high-speed audio path proves reliable: a 2023 report showed UAC2 control requests failing on this chip, and the driver contributor had not verified isochronous transfers.

The open-source USBAudioToI2S firmware (R7, Apache-2.0) is kept as a known-good comparison on the same hardware and as a reference; licence notices are kept if any code is copied. Its source (audio_cpld.v) ties the MCLK output low, runs one PLL from the 48 kHz-family clock, has no 352.8 kHz path and a combinational clock select, and its descriptors report self-powered (0xC0), bMaxPower 0x32 (100 mA), Feature Unit bmaControls = 0 and VID 0x1A86 / PID 0. The plan below is proven on the JLCPCB-built Stage 2 board, the first hardware, before layout (firmware gate and layout gate); the Stage 1 desk setup is optional.

## 5.1 Firmware plan and proof of concept

| # | Topic | Required behaviour |
| --- | --- | --- |
| 1 | USB core | High-speed port: enumeration, descriptors, standard requests, suspend/resume. Suspend is a v1 requirement: ≤ 2.5 mA within 10 ms of the last bus activity (suspend interrupt 3 ms after the last bus activity → controlled power-down → suspend level typically ≈ 8.2 ms, at worst 9.3 ms after the last bus activity, 2.5; both LEDs off; MCU Stop with LPDS, USBHS PHY suspended, wake on resume, HSE/PHY PLL restart within the 10 ms resume recovery, fallback wake EXTI on PB6/PB7; resume-to-audio ≈ 1.0 s). Descriptors: bus-powered (bmAttributes 0x80), bMaxPower 0xFA (500 mA); HS feedback endpoint wMaxPacketSize 4; isochronous OUT ≥ 392 bytes; bcdDevice bumped on every descriptor change; test VID/PID 1209:0001 during development (pid.codes reserves it for private testing only), a real PID before publishing; serial number from the CH32V307 96-bit unique ID |
| 2 | UAC2 controls | Clock source with the rates (CUR/RANGE: all 8 rates from 44.1 to 384 kHz, 352.8 kHz included), clock valid, mute and volume (CUR/RANGE) mapped to the ES9018K2M attenuator over I²C plus MCU fine scaling for the channel trim, alternate settings for 16/24/32-bit |
| 3 | Streaming | Isochronous OUT → ring buffer → stream DC guard → DMA to the CPLD over SPI1 at 36 MHz with FRAME framing (MCU master; each frame starts with a 16-bit header; 8-bit frames or a halfword swap in the CPLD set the word order, tested with a known pattern). Stream DC guard (R7-H H8): no zero crossing for 25 ms and ¦mean¦ ≥ 0.25 FS → output zeroed at once; 0.2-s mean ≥ −30 dBFS → soft mute; unmute after 1 s below −40 dBFS; reported to the host log. At 384 kHz, 32-bit stereo: 384 bytes per 125 µs microframe, up to 392 bytes with one extra sample; link payload 24.576 Mbit/s (68 % of 36 MHz) |
| 4 | Feedback | The CPLD’s LRCLK (LRCLK_FB on PD2, TIM3_ETR) counted by an MCU timer and latched at SOF, plus a small FIFO-level correction (MISO returns the FIFO level during each frame); sent in 16.16 format (at 384 kHz: 48.0 samples per microframe = 0x300000; the 44.1 kHz family is non-integer, e.g. 44.1 kHz = 0x002C1999); the same timer checks the LRCLK rate after every family change (within ±0.1 % of the target) |
| 5 | Application | ES9018K2M driver (asynchronous mode; register 1 = 0x80: 32-bit I²S, input auto-select off; register 10 = 0x00: slave, asynchronous, stop_div = 0, required above 96 kHz; DPLL bandwidth register 12; soft start register 14; mute, lock status, master trim); power states and sequencing (2.5: order A legs first by default, order A′ and option B selectable; MCLK_EN before RESETB; power-down with MCLK stopped and RESETB high, AMP_EN low once the 3V3A monitor reads < 0.1 V, deadline t0 + 5.8 ms; AUD_EN off ≥ 100 ms); rate and break-before-make family changes sequenced through CPLD SPI registers (5.2); CC monitoring with the 0.66 V / 1.23 V thresholds, VBUS watchdog and ILIM_HI control; calibration storage (master trim, channel trim, factory zero of each DC-sense channel) and the power-up auto-zero; the −4.5 dB pre-calibration safety default; programming mode (PROG strap); test modes (DC fault injection by a digital DC signal with the stream guard bypassed, I²S-header test mode) |
| 6 | Slow DC tier | ADC2 scan (CC1, CC2, four DC-sense, 3V3A monitor; 239.5-cycle sampling; first conversion after each channel switch discarded), each DC-sense channel at 1 kHz with ±20 % dithered timing, corrected ratiometrically with the 3V3A monitor and zeroed at every power-up while muted (accepted only within ±30 mV of the factory zero, otherwise a fault is reported). Tiers on max(¦each leg¦, ¦L+ − L−¦, ¦R+ − R−¦): T1 0.2-s mean ≥ 1.0 V → latch within 0.25 s; T2 0.3-s mean ≥ 0.5 V → 0.35 s; T3 1-s mean ≥ 0.3 V → 1.1 s; T4 2-s mean ≥ 0.11 V nominal → 3 s; by switching the ANA_FAULT pin to open-drain output low (D606 → node F → the hardware kill path); armed whenever AVCC_EN is high; from RAIL_EN until the hardware set path is armed, a 1 kHz start-up watch drops RAIL_EN directly if any leg exceeds 0.5 V |
| 7 | Fault cause capture and recovery | FLT interrupt (EXTI3, highest priority): first (≤ 50 µs) drive AMP_EN, RAIL_EN and AVCC_EN low (mirroring the latch), AUD_EN and RESETB low; then classify the cause from the pre-trip rolling 0.5-s averages in the ring buffer plus the event history, ANA_FAULT, RAILS_OK_N and whether firmware itself set the latch; then ANA_EN low. Classes: DC fault, overload, or “ambiguous” when neither pattern matches (logged and counted toward the DC three-strike limit, R7-F F6); unavailable readings count as a DC fault. FLT during a controlled power-down or before the first FLT_CLR after power-up is expected: not logged, not counted, no ceiling change. IWDG enabled: an MCU reset floats all outputs and the pull-downs switch every domain off. Recovery policy with the −12 dB strike rule in 5.2 |
| 8 | MCU–CPLD interface | Ours to define: SPI1 master with DMA at 36 MHz, CPOL = 0, LINK_FRAME active high (idle low). The CPLD has no free-running system clock: the FIFO is written in the SPI clock domain (SCK on the global network through ALTCLKCTRL) and read in the PLL bit-clock domain (Gray-coded pointers); each frame starts with a 16-bit header so that the pointer synchronisers see SCK edges; registers live in the SPI domain (MCLK_EN, OSC48_EN, OSC44_EN, PLL reset, dividers); MISO returns the FIFO level during the frame; LRCLK back to PD2. I²S is not used for the link (its frame rate is not locked to the audio rate). Register reads at ≤ 18 MHz; second lane reserved (SPI2 PB12–PB15). Write the interface specification and freeze both pin maps before the Stage 2 board and the schematic (open item) |
| 9 | CPLD logic | Our own: SPI link capture in the SPI clock domain; dual-clock FIFO of 1024 × 32 bit in 4 × M9K (≈ 1.33 ms at 384 kHz; written in the SPI domain, read in the PLL bit-clock domain, Gray-coded pointers); I²S output engine on the PLL output with exact ÷2/4/8/16, SDATA and LRCLK changed on the falling BCLK edge; PLL reset and lock handling; OSC48_EN, OSC44_EN and MCLK_EN from SPI registers, with the break-before-make family change timed by the MCU (both family enables low after configuration, X202 enabled by the MCU after ≥ 1 ms); weak pull-downs on SCK, MOSI and FRAME (3.3). Global clocks: PLL output and SCK. No clock multiplexer in the fabric. The CPLD is AG32 silicon in pure-CPLD mode; the Supra/Quartus toolchain is Windows-centred (CPLD builds in CI on a Windows runner, or check Supra on Linux) |
| 10 | Proof of concept, stage 1 (optional) | Optional, because development boards are hard to get in Taiwan (owner, 24 Sep 2026): desk setup on a WCH CH32V307V-EVT-R1 board, an AGM 2K-LUT CPLD board and an ES9018K2M I²S-input DAC board, only if the owner can get them (≈ US$48–57 for the three boards at listing prices, outside the board budget); nothing gates on it |
| 11 | Proof of concept, stage 2 (mandatory, first hardware, layout gate) | Two JLCPCB-built boards with the real chips and the frozen pin map, including the ES9018K2M with its real supplies (3.8); ≈ US$93 ≈ 2,940 NTD for two boards, outside the board budget. With the real CPLD design loaded, 384 kHz streaming, at VBUS 4.60 V and 5.25 V and across rate and family changes: attached-not-configured ≤ 100 mA; configured digital ≤ 262 mA; suspend ≤ 2.5 mA within 10 ms of the last bus activity, including the DAC-first power-down with the real LP5907; watchdog-to-GPIO latency < 5 µs. Layout does not start unless all pass |
| 12 | Gate pass criteria | Enumerates bus-powered with the new descriptors; every rate plays on Windows 10/11, macOS and Linux; bit-perfect path to the DAC at all 8 rates, 352.8 kHz included (lock status, FFT of the check output); BCLK/LRCLK average rates correct for every rate (frequency counter); host volume and mute work; SPI 36 MHz link with the real CPLD capture; PLL lock at both family clocks; clock handover: never both enables high, node quiet ≥ 1 ms before the new oscillator starts, disable time measured, no runt pulses, PLL relock after every family change, on the shared-node board and on the mux board; CPLD meets timing at the SPI clock and at the PLL output (49.152 MHz); unconfigured ≤ 100 mA and configured digital ≤ 262 mA; suspend current against time from the last bus activity, ≤ 2.5 mA within 10 ms; watchdog-to-GPIO latency < 5 µs; AUD_EN cycled 100× at VBUS 4.40 V and 5.25 V; RAILS_OK_N rise, latch capture, blanking and arming times; DC-sense auto-zero, dither and ratiometric correction; programming mode and CPLD programming through J202 |
| 13 | Unit tests | Pure-logic parts (descriptor tables, feedback math, ring buffer, volume mapping, DC-tier averaging per leg and per pair, cause classification with the ambiguous class and recovery counters, the stream DC guard, the GPIO configuration table, the DAC register values) compile on a PC and run as unit tests in GitLab CI |
| 14 | Licences and reference | Keep the licence notices if any code is copied (USBAudioToI2S Apache-2.0, TinyUSB MIT). USBAudioToI2S stays a known-good comparison on the same hardware and a reference, not the base of our firmware |

## 5.2 State machine and policies

| State / event | Required behaviour |
| --- | --- |
| Attached | Only 3V3M is on; the MCU enumerates within 100 mA; the latch is set (node F starts at 0 V through C605); all enables are low. PROG strapped low at reset → programming mode (2.5) instead of USB operation |
| Configured | Read CC1/CC2 (0.66 V / 1.23 V thresholds with hysteresis) → Default or High mode (ILIM_HI high only with ≥ 1.5 A advertised; a USB-A-to-C cable reads as Default); run the power-up sequence (2.5): ANA_EN → FLT_CLR ≥ 600 ms after ANA_EN → RAIL_EN (≥ 10 ms after ANA_EN; PGOOD good within 20 ms; 1 kHz start-up DC watch) → second FLT_CLR at PGOOD good + 20 ms → AMP_EN ≥ 250 ms after PGOOD good (VREF still 0 V) → AVCC_EN → +5 ms AUD_EN → MCLK_EN → RESETB and I²C init → soft unmute when streaming. CC monitoring: sample CC1/CC2 at least every 5 ms; treat a change as valid after tRpValueChange (10–20 ms); on a drop from ≥ 1.5 A to Default: soft mute, ILIM_HI low and continue in Default mode within tSinkAdj (60 ms max) of the change; switch up to High only after the new advertisement is stable (R16) |
| Playing | Monitor RAILS_OK_N, FLT, ANA_FAULT, the four DC-sense channels and the 3V3A monitor (firmware tiers per leg and per pair, armed while AVCC_EN is high), the stream DC guard, CC1/CC2 and the VBUS watchdog |
| Controlled power-down | Suspend, host command or fault command: t0 soft mute; t0 + 0.1 ms MCLK_EN low (RESETB stays high) and AVCC_EN low; AMP_EN low once the 3V3A monitor reads < 0.1 V (typically t0 + 4.7 ms, at the latest t0 + 5.8 ms); then RESETB, AUD_EN, RAIL_EN and ANA_EN low; MCU low-power (2.5) |
| Suspend | Suspend interrupt 3 ms after the last bus activity → controlled power-down → MCU Stop with the USBHS PHY suspended, both LEDs off; ≤ 2.5 mA within 10 ms of the last bus activity is a v1 requirement (suspend level typically ≈ 8.2 ms, at worst 9.3 ms after the last bus activity; budget ≈ 2.30 mA); resume runs the power-up sequence, with AUD_EN only after ≥ 100 ms off (resume-to-audio ≈ 1.0 s) |
| FLT interrupt (cause capture) | First (≤ 50 µs) AMP_EN, RAIL_EN, AVCC_EN, AUD_EN and RESETB low; then classify from the pre-trip rolling 0.5-s averages and the event history, ANA_FAULT, RAILS_OK_N and whether firmware itself set the latch; then ANA_EN low. DC fault, overload or ambiguous (counted as DC toward the three-strike limit); if the readings are unavailable, treat the event as a DC fault (conservative) |
| Expected FLT (masked) | FLT during a controlled power-down, before the first FLT_CLR after power-up, or cleared by the second FLT_CLR at PGOOD good + 20 ms (rail inrush) is expected: not logged, not counted, no ceiling change; firmware also ignores its own ANA_FAULT classification for 20 ms after RAIL_EN |
| Overload | The latch has already removed RAIL_EN, AMP_EN and AVCC_EN in hardware (PGOOD bad via Q610, or a sustained TPS2553 FAULT via D606) → soft mute; volume ceiling −3 dB per event (down to −12 dB); clear after ≥ 1 s (FLT_CLR only ≥ 600 ms after the rails went off); the outputs return through the power-up sequence; the ceiling is kept until replug. Overloads count toward the 3-strike stop only once the ceiling has reached −12 dB (Default mode at full volume into 60 Ω balanced needs three −3 dB steps) |
| DC fault | The latch has already removed RAIL_EN, AMP_EN and AVCC_EN (hardware detector, or the firmware DC tiers through ANA_FAULT) → soft mute, log, clear after 2 s (FLT_CLR only ≥ 600 ms after the rails went off); 3 DC or ambiguous faults within 10 s → latched until USB replug; the red LED blinks. DC in the program material is muted by the stream DC guard before it reaches the tiers (reported to the host log, no latch) |
| VBUS watchdog | ADC1 continuous on VBUS_SENSE with the analog watchdog at 4.10 V nominal (3.95–4.25 V), ≈ 3.8 µs worst-case latency, ISR on a fast (VTF) vector → AMP_EN and RAIL_EN low at once, then RESETB, AUD_EN, AVCC_EN and ANA_EN low (target < 5 µs to the GPIOs); the PGOOD latch is the hardware backstop; the latency is measured on the Stage 2 board |
| Rate change | MCLK (X201) never changes while playing. Within a family: soft mute → CPLD changes the BCLK divider → wait for DPLL lock (lock_status, register 64) → soft unmute. Family change, break-before-make, sequenced by the MCU through CPLD SPI registers: (1) DAC soft mute; (2) I²S output stopped (held low); (3) PLL held in reset; (4) old enable low; (5) wait ≥ 1 ms with both off; (6) new enable high; (7) wait ≥ 10 ms; (8) release the PLL, wait for lock; (9) restart the output; the MCU checks the LRCLK rate with its feedback timer (within ±0.1 % of the target, otherwise stay muted, retry once, then report a fault); (10) DAC re-locks → soft unmute. Never change both enable bits at the same time. AMP_EN stays high (outputs muted digitally); pop measured |
| Calibration | Before calibration apply −4.5 dB (a safety setting, always active until a calibration is stored); the end-of-line test writes the master trim (1.55 Vrms ± 0.1 dB), the channel trim (±0.05 dB) and the zero of each DC-sense channel to flash |
| Test modes | DC fault injection by a digital DC test signal with the stream DC guard bypassed (per leg and opposite-sign per pair, above each tier threshold); stream-guard test with a DC test file (mute, no latch); I²S-header test mode (CPLD I²S outputs Hi-Z; an external source drives BCLK, LRCLK and SDATA only; the DAC stays in asynchronous mode on X201 and configured over I²C; J703 MCLK is a monitor output); pop measurement: pop = peak of the transient at the jack beyond the settled DC step, band-limited 20 Hz–20 kHz (AC-coupled analyzer input) |
| Programming | CH32V307 through the WCH-LinkE pads (J201); CPLD in programming mode (PROG strap: AVCC_EN, then AUD_EN, with RESETB, MCLK_EN, ANA_EN, RAIL_EN and AMP_EN low, no USB configuration) through the J202 Compact-JTAG pads with a J-Link or an AGM DAP-Link; end-of-line order MCU → CPLD → calibration |

# 6. Schematic review checklist

These rules seed the AI datasheet-review agent, and each one can be turned into a planted-error test case. ERC is KiCad’s electrical rules check; a netlist script checks connections, values and counts in the exported netlist; the AI agent reviews rules that need datasheet or part-level interpretation.

| # | Rule | Source | Check method |
| --- | --- | --- | --- |
| 1 | VBUS: 5 × 1 µF before any switch (C102, C501, C503 after R519/R520, C301, C505); D102 TVS footprint at J101 (DNF); attach charge ≤ 50 µC (≈ 42.5 µC at 5.0 V, ≈ 43.8 µC at 5.25 V estimate) | USB; 2.4; 3.1 | Netlist script |
| 2 | CC1/CC2: 5.1 kΩ Rd each; CC → 10 kΩ (R103, R104) → MCU ADC, nothing else on CC | USB-C; 3.1 | Netlist script |
| 3 | VBUS divider R105 100 kΩ / R106 47 kΩ to PA1, the ADC1 pin with the analog watchdog (4.10 V threshold, continuous single-channel conversion) | 3.1 | Netlist script |
| 4 | USBLC6-2SC6 on D+, D− and VBUS at the connector; J101 A6/B6 (D+) and A7/B7 (D−) tied at the connector; USB pair in a 90 Ω differential net class | R1; 3.1 | Netlist script; AI agent (stack-up) |
| 5 | Every IC supply pin decoupled: CH32V307 100 nF at each of the 6 supply pins plus one extra at VDD_2 and VDD_4, C221 1 µF at VDDA and 2.2 µF bulk; CPLD 100 nF at each supply pin on the FB202 branch plus C220 1 µF at VDDA33; each oscillator 100 nF (C217 at X201, C218 at X202, C219 at X203); DAC per 3.4; OPA2210 100 nF per rail pin; OPA1622 100 nF + 10 µF per rail pin; 2 × LM339 C612–C615 100 nF (V+ and V− at each package); LM27762 per 3.6 | R1–R5 | Netlist script |
| 6 | HSE crystal Y201 8 MHz, CL 20 pF, with 33 pF C0G load capacitors (C201, C202) | R1; 3.2 | Netlist script; AI agent |
| 7 | CH32V307 and CPLD pins match the frozen pin maps (3.2, 3.3) and the MCU–CPLD interface specification | 3.2; 3.3; Section 5 | Netlist script (pin-map diff) |
| 8 | MCU pins toward the CPLD (LINK_SCK, LINK_MOSI, LINK_FRAME) and RESETB are GPIO-low or input while AUD_EN is low (no MCU-side pull-ups into 3V3D); I²C pull-ups R207, R208 on 3V3D | 2.5 | Netlist script; AI agent |
| 9 | MCLK: X201 drives only the DAC XI pin (through R203 33 Ω) and the J703 monitor pin through R703; no other MCLK load and no CPLD branch | 3.3; 3.8 | Netlist script; AI agent (XI trace ≤ 20 mm) |
| 10 | DAC in asynchronous mode with MCLK 80 MHz (cycle 12.5 ns ≥ 10 ns, pulse width ≥ 5.6 ns ≥ 4.5 ns): DVCC 3.3 V and the external 1.3 V DVDD (FSR > 192 kHz) | R3 “XI / MCLK Timing” | AI agent |
| 11 | External DVDD from ME6216A13M3G fed from 3V3D (DVDD ≤ DVCC); C310 2.2 µF + 100 nF at the pin | R3; 3.4 | Netlist script |
| 12 | AVCC_L and AVCC_R on separate 2.2 Ω branches from the LP5907; LP5907 EN = AVCC_EN with pull-down R303 and series resistor R307 (rule 57); LP5907 total output capacitance ≤ 10 µF (5.7 µF) | R3; R6 | Netlist script |
| 13 | DAC RESETB: R305 100 kΩ pull-down and MCU drive through R308 (rule 67); ADDR (pin 5) to DGND through R306 10 kΩ for address 0x48 (written 0x90) | R3 | Netlist script |
| 14 | Each DAC output pin connects only to its I/V inverting input and to its clamp (D405–D408, fitted, rule 78): no other load, short trace | R3 p. 27 | Netlist script (pin fan-out); AI agent (trace length) |
| 15 | VREF from 3V3A through R431 / (R432 + R433), R433 1 kΩ (0 Ω option for 1.650 V); C421 33 pF C0G at the divider (τ ≈ 0.29 µs, VREF tracks 3V3A) and no other capacitor or clamp diode on VREF; short traces to all four non-inverting inputs; one shared node | 3.5.1 | Netlist script |
| 16 | Each I/V feedback: 1 kΩ ∥ 5.1 kΩ, with 10 Ω in series with 2 × 330 pF C0G across it | 3.5.1 | Netlist script |
| 17 | C0G only in the signal path (I/V and leg capacitors); no X5R/X7R there | 3.5 | Netlist script (dielectric field) |
| 18 | Four 2.00 kΩ 0.1 % resistors in every difference amplifier; leg+ = I/V(DAC−) − I/V(DAC+), leg− the swap; U401 = L+/L−, U402 = R+/R−; U403 = DACL/DACLB, U404 = DACR/DACRB | 3.5 | Netlist script |
| 19 | AMP_EN: R421 100 kΩ pull-down, R618 10 kΩ series from the MCU, kill FETs Q608 (latch) and Q609 (slow interlock, rule 51) to ground | 3.5.2; 3.7 | Netlist script |
| 20 | OPA1622 thermal pad to V−; pin 3 (GND) to board GND (rule 70); input common mode inside (V−)+1.5 V … (V+)−1 V (≈ 0.85 V on the ±3.8 V rails) | R4 | Netlist script; AI agent |
| 21 | TPS2553: RILIM 100 kΩ + 47 kΩ; R510 through Q501 with gate pull-down R511; RILIM trace short; EN pull-down R513; FAULT pull-up R512 to 3V3M | R9; 3.6 | Netlist script; AI agent (layout) |
| 22 | LM27762 dividers: 100 k + 100 k + 15 k + 10 k over 100 k (FB+, R523 10 kΩ, +3.90 V) and 100 k + 100 k + 5.1 k over 100 k (FB−, R518 5.1 kΩ); bottom resistors ≥ 50 kΩ | R5 | Netlist script |
| 23 | LM27762 EN must not exceed VIN: RAIL_EN only after 5V_ANA (≥ 10 ms after ANA_EN, firmware order); RAIL_EN pull-down R506, series R507, kill FET Q607 to ground | R5; 2.5 | AI agent; netlist script |
| 24 | LM27762 PGOOD (RAILS_OK_N) routed to the MCU through R522, to the Q610 gate, to R623 (Q609 gate delay) and to the blanking RC; pull-up per rule 58; load per rule 75 | R5; 3.7 | Netlist script |
| 25 | Blanking RC R617 1 MΩ + C606 100 nF on the Q603 gate from PGOOD (re-blanked 19–94 ms after PGOOD bad, armed 51–186 ms after PGOOD good); blanking acts on node S only, never on the latch nodes | 3.7 | Netlist script; AI agent |
| 26 | Node F: R611 10 kΩ series, R612 47 kΩ pull-up to 3V3M, C605 100 nF from F to GND, D605 clamp to GND; only R611, R612, C605, D605, the Q601 gate, the Q610 drain and the D606 anode connect to F | 3.7 | Netlist script; AI agent |
| 27 | Latch: Q604/Q605 cross-coupled with R614, R615 47 kΩ pull-ups to 3V3M and no capacitor on the latch nodes; Q606 clear from FLT_CLR with pull-down R616; FLT (node B) to an MCU input (PB3) and to the Q607, Q608 and Q611 gates | 3.7 | Netlist script |
| 28 | DC detect: R601–R604 100 kΩ + C601–C604 1 µF per leg (τ = 100 ms), each filter node directly into its own window of two LM339 comparators (rules 59, 60) | 3.7 | Netlist script |
| 29 | Ferrites: BLM18PG121SN1D in the 5V_ANA path; GZ1608D601TF only on DVCC (FB301) and on the CPLD branch (FB202) | 3.4; 3.6 | Netlist script |
| 30 | ESD diode (LESD5D5.0CT1G) on every jack leg | 3.5.2 | Netlist script |
| 31 | Test pads TP701–TP732 per the allocation table: 8 rails, VREF, AVCC_L, MCLK, family-clock node, OSC48_EN, OSC44_EN, BCLK, LRCLK, SDATA, AMP_EN, RAIL_EN, AVCC_EN, RAILS_OK_N, FLT, ANA_FAULT, node F, VBUS_SENSE, CC1, CC2, 4 GND, PROG | 3.8 | Netlist script |
| 32 | One part per BOM line: one value, one MPN, one LCSC number and a JLCPCB class taken from its part page or the basic-parts list per line (X202 and the DNF D102: assumed Extended); the MPN must decode to the value of its LCSC part (2.2 Ω = UNI-ROYAL 0805W8F220KT5E); no mixed or range values | Parts List v0.6 | Netlist script (BOM) |
| 33 | X202 and X203: C218 and C219 100 nF at their supply pins; R210 and R211 33 Ω in series at each oscillator output, joined into one node at the CPLD’s dedicated PLL clock input | 3.3 | Netlist script; AI agent (placement) |
| 34 | Shared family-clock node routed next to CPLD pin 1 (IO_GB, the PLL input): both family oscillators and the node beside the pin, short traces, no stubs | R2; 3.3 | Netlist script (pin map); AI agent (layout) |
| 35 | Short 80 MHz MCLK route from X201 to the DAC XI pin (≤ 20 mm); duty measured at XI | 3.3 | AI agent (layout) |
| 36 | No clock trace (MCLK, family clock, BCLK, LRCLK) crosses the analog area (I/V converters, output legs, VREF) | 3.3; 3.5 | AI agent (layout) |
| 37 | Family change break-before-make: I²S output (BCLK, LRCLK, SDATA) held low, DAC soft-muted and PLL held in reset; old enable low, ≥ 1 ms with both off, new enable high, ≥ 10 ms start-up; never both enable bits changed at the same time | 3.3; 5.2 | AI agent (CPLD logic review); scope check on Stage 2 |
| 38 | Oscillator enables: 2.2 kΩ pull-downs on all three enables (R212 on OSC48_EN, R213 on OSC44_EN, R218 on MCLK_EN), so every oscillator is off at power-up and during CPLD configuration | 3.3 | Netlist script |
| 39 | X201 tri-state pin 1 = MCLK_EN from CPLD pin 21 with the R218 2.2 kΩ pull-down; firmware enables X201 only after DVDD is up and disables it before AVCC goes off | 3.3; R14 | Netlist script; AI agent (firmware review) |
| 40 | OSC48_EN, OSC44_EN and MCLK_EN driven from CPLD I/O in the same 3V3D domain as the oscillators (no drive from the always-on MCU domain, so no back-powering when 3V3D is off) | 2.5; 3.3 | Netlist script (power domain); AI agent |
| 41 | R214 10 kΩ pull-down from the shared family-clock node to GND | 3.3 | Netlist script |
| 42 | U205 (74LVC1G157GW,125) and R215 33 Ω DNF at the CPLD PLL clock pin, select = OSC44_EN; R210/R211 are the population option (fit U205 and R215 and remove R210/R211 to use the mux) | 3.3 | Netlist script (BOM fit field); AI agent |
| 43 | Q610 2N7002: gate = RAILS_OK_N, drain = node F, source = GND | 3.7 | Netlist script |
| 44 | D606 1N4148WS: anode = node F, cathode through R638 (rule 61) to ANA_FAULT (TPS2553 FAULT and the MCU ANA_FAULT pin, rule 53) | 3.7 | Netlist script |
| 45 | Q611 2N7002: gate = node B (FLT), drain = AVCC_EN node (the U302 EN side of R307), source = GND | 3.7 | Netlist script |
| 46 | Hardware DC sense (R601–R604) and slow DC-sense network (R624–R627) taken at the jack side of the 0 Ω output links R417–R420 | 3.7; 3.8 | Netlist script |
| 47 | J703 MCLK pin connects to MCLK only through R703 1 kΩ (monitor output; DNF with J703) | 3.8 | Netlist script |
| 48 | MCU audio-link SPI clock LINK_SCK (PA5) to CPLD pin 9 (global network through ALTCLKCTRL); LINK_FRAME (PA4, active high) to pin 11; LINK_MOSI (PA7) to pin 10; LINK_MISO (PA6) from pin 12 | R2; 3.3 | Netlist script (pin map) |
| 49 | Idle family-oscillator output traces kept short (only one oscillator drives the shared node; the other may keep running internally) | 3.3 | AI agent (layout) |
| 50 | 3V3D: U504 fed from 5V_SYS through R519 + R520 (2 × 2.2 Ω 0805 in series, UNI-ROYAL 0805W8F220KT5E) before the LDO; CE = AUD_EN through R521 10 kΩ; C503 1 µF at the U504 input after the dropper; C504 1 µF and C512 10 µF at the U504 output; R524 2.2 kΩ bleeder to GND | 3.6; 3.6.3 | Netlist script |
| 51 | Q609 2N7002: drain = AMP_EN, source = GND; gate from RAILS_OK_N through R623 1 MΩ, with C607 10 nF (C15195) from the gate to GND | 3.7 | Netlist script |
| 52 | Slow DC-sense network per leg: R624–R627 15 kΩ from the jack side of R417–R420, R628–R631 10 kΩ to 3V3A, R632–R635 22 kΩ to GND, C608–C611 2.2 µF; each node to its own MCU ADC pin (PC0–PC3); range per rule 76 | 3.7; Section 4 | Netlist script |
| 53 | ANA_FAULT on an open-drain-capable MCU pin (PB5): floating input, or open-drain output low that sets the latch through D606, never push-pull high; FLT (PB3) and RAILS_OK_N (PB4) are floating inputs; no internal pulls on any of the three | 3.2; 3.7 | Netlist script (pin map); AI agent (firmware GPIO table) |
| 54 | R216, R217 10 kΩ pull-downs (DNF, fitted with U205) at both U205 inputs | 3.3 | Netlist script (BOM fit field) |
| 55 | CPLD I²S engine changes SDATA and LRCLK on the falling BCLK edge (≥ 20 ns half period at 24.576 MHz against 4.1 ns setup / 2 ns hold) | R3 p. 30; 3.3 | AI agent (CPLD logic review); scope check |
| 56 | Copper pours on the VIN, GND and VOUT pins of the LDOs U502, U504, U303 and U302; IR or thermocouple check at bring-up | 3.6.3 | AI agent (layout) |
| 57 | Enable series resistors: AVCC_EN through R307 10 kΩ (MCU PC11 → U302 EN node with R303 and the Q611 drain), AUD_EN through R521 10 kΩ (PC10 → U504 CE), RAILS_OK_N through R522 10 kΩ into MCU PB4; RAIL_EN (R507) and AMP_EN (R618) as in rules 19 and 23 | 2.1; 3.6; 3.7 | Netlist script |
| 58 | LM27762 PGOOD pull-up R505 10 kΩ to 5V_ANA (the LM27762 VIN, as TI specifies), not to 3V3M | R5; 3.6 | Netlist script (power domain) |
| 59 | U601, U602 = TI LM339DR (C7948): V+ pin to +3.90 V and the GND pin to −3.72 V, with C612–C615 100 nF from V+ and V− to GND at each package; each leg filter node to one comparator IN− (positive half, IN+ = VTH+) and to one comparator IN+ (negative half, IN− = VTH−); all eight open-collector outputs wired-OR into R611 (node F); no unused comparator | 3.7; TI LM339 datasheet | Netlist script; AI agent (datasheet) |
| 60 | Threshold dividers: VTH+ from V+ through R607 47 kΩ + R636 15 kΩ over R609 10 kΩ to GND; VTH− from V− through R608 47 kΩ + R637 10 kΩ over R610 10 kΩ to GND; C601–C604 return to GND; no diode-OR and no MAX/MIN load resistors | 3.7 | Netlist script |
| 61 | R638 1 kΩ in series between the D606 cathode and ANA_FAULT; D605 anode to GND, cathode to node F | 3.7 | Netlist script |
| 62 | J202 = 2-wire Compact-JTAG only: 3V3D (sense only), JTCK (CPLD pin 25), JTMS (pin 24), NRST (pin 4), GND; the probe never powers 3V3D | R2; 3.3 | Netlist script (pin map) |
| 63 | Family clock (shared X202/X203 node) on CPLD PIN_1 (IO_GB) into the PLL; the CPLD has no 80 MHz input (X201 drives only the DAC XI) | R2; 3.3 | Netlist script (pin map) |
| 64 | FB202 (GZ1608D601TF) from 3V3D to the CPLD VDD33/VDDA33 branch; 100 nF at each CPLD supply pin (C213–C216); C220 1 µF at VDDA33; exposed pad to GND | R2; 3.3 | Netlist script |
| 65 | CH32V307 VBAT (pin 1) tied to 3V3M; C221 1 µF at VDDA (pin 13) | R1; 3.2 | Netlist script |
| 66 | R224, R225 33 Ω at the MCU on LINK_SCK and LINK_MOSI; CPLD weak pull-downs on SCK, MOSI and FRAME enabled in the CPLD design | 3.2; 3.3 | Netlist script; AI agent (CPLD logic review) |
| 67 | R308 1 kΩ in series between the MCU (PA8) and DAC RESETB (pin 28), with R305 100 kΩ pull-down on the DAC side | 3.4 | Netlist script |
| 68 | Bleeders: R524 2.2 kΩ from 3V3D to GND and R525 1 kΩ from 1V3 to GND | 3.6 | Netlist script |
| 69 | USB-C shell: R107 0 Ω to GND (fitted) and C101 DNF | 3.1 | Netlist script (BOM fit field) |
| 70 | OPA1622 (U401, U402) VSON-10 pinout: 1 +INA, 2 V+, 3 GND (EN reference, to board GND, never to V−), 4 V−, 5 +INB, 6 −INB, 7 OUTB, 8 EN, 9 OUTA, 10 −INA; thermal pad to V− | R4; 3.5.2 | Netlist script (symbol pin check); AI agent |
| 71 | ES9018K2M: DATA_CLK (pin 27) = BCLK, DATA1 (26) = LRCLK, DATA2 (25) = SDATA; XI (7) = MCLK; ADDR (5) to DGND through R306 10 kΩ; XO (6) open; GPIO2 (23) to MCU PC9 (DAC_LOCK) | R3; 3.4 | Netlist script (symbol pin check) |
| 72 | U303 ME6216A13M3G symbol and footprint SOT23-3 “M3”: pin 1 VSS, pin 2 VOUT, pin 3 VIN; LM27762 symbol PGOOD = pin 1, FB− = pin 7 | 3.4; 3.6 | Netlist script (symbol pin check) |
| 73 | Jack pin numbers (J701, J702) verified by continuity tests on 2 samples of each jack before layout; the footprint pins match the tested map | 3.5.3 | Bench check (samples); netlist script |
| 74 | PROG pad TP732 to MCU PC13 (internal pull-up, no external part) | 3.2; 3.8 | Netlist script |
| 75 | RAILS_OK_N may carry only the R505 pull-up, ≥ 1 MΩ RC branches (R617/C606, R623/C607) and high-impedance gate or MCU inputs (the Q610 gate, R522 → PB4) | R5; 3.6; 3.7 | Netlist script |
| 76 | Slow DC-sense ADC node stays 0.29–2.85 V over every rail, 3V3A ±2 % and resistor corner (source 4.7 kΩ ≤ 50 kΩ); the bias resistors R628–R631 go to 3V3A, never to 3V3M | 3.7 | Netlist script (power domain); AI agent (corner calculation) |
| 77 | 3V3A monitor: R639/R640 10 kΩ/10 kΩ from 3V3A to MCU PC4 (24), ADC14 with C616 100 nF to GND | 3.2; 3.7 | Netlist script |
| 78 | D405–D408 BAT54S (JSMSEMI, C916425) fitted, one per DAC pin within 5 mm of the DAC: pin 3 (common) on DACL/DACLB/DACR/DACRB, pin 1 (lower anode) to AGND at the DAC AGND pins, pin 2 (upper cathode) to AVCC_L (DACL, DACLB) or AVCC_R (DACR, DACRB) at their decoupling capacitors; BOM fit field fitted | 3.5.1 | Netlist script (BOM fit field, pin connections); AI agent (placement within 5 mm) |

# 7. Open items and review disposition

Open items are verification tasks with a closing gate: MCU–CPLD interface specification and both pin maps (before the Stage 2 board), the Stage 2 board (the first hardware: firmware gate and layout gate; its USB current, suspend, watchdog-latency, clock and protection measurements gate the layout), jack pin numbers from samples (before layout), SPICE before layout, schematic review, the DAC-pin excursion measurement with the fitted clamps on main board 1, JLCPCB quote after layout, bring-up. No item waits for an owner decision on the residual DC: the region below the lowest firmware tier was accepted by the owner on 24 Sep 2026.

## 7.1 Open items

| # | Item | Why it matters | How to close | Gate |
| --- | --- | --- | --- | --- |
| 1 | Own UAC2 firmware and CPLD logic (top risk: host compatibility of high-speed isochronous streaming with feedback) | v1 depends on our own USB stack on the CH32V307 high-speed port; WCH has no high-speed audio example | Stage 2 board, the first hardware (5.1); the Stage 1 desk setup is optional | Firmware gate (Stage 2) |
| 2 | MCU–CPLD interface specification and both pin maps (SPI with FRAME framing) | The link is ours to define; the proposed MCU and CPLD pin maps (3.2, 3.3) depend on it | Write the specification and freeze both pin maps | Before the Stage 2 board and the schematic |
| 3 | CPLD timing at the SPI clock (36 MHz) and at the PLL output (49.152 MHz), including the FIFO crossing | Every bit clock comes from the PLL; the FIFO is written in the SPI clock domain; the CPLD has no system clock | Build and time the CPLD design; frequency counter and duty on every rate; PLL relock and runt-pulse check at family changes | Stage 2 |
| 4 | SPI 36 MHz link capture (R7-D DG-12) | 36 MHz (72 MHz ÷ 2) is the only workable SPI clock (68 % utilised at 384 kHz) | Capture with the real CPLD design; register reads at ≤ 18 MHz; second lane reserved (SPI2 PB12–PB15) | Stage 2 gate |
| 5 | PLL lock at both family clocks (R7-D DG-11) | One PLL must lock at 22.5792 and 24.576 MHz input; the AGM lock range is not published | Measure lock at both inputs; fallback: BCLK ÷ 1 directly from X202/X203 (duty 45–55 % = the DAC limit, zero margin) | Stage 2 gate |
| 6 | CPLD programmer and Supra support (R7-D DG-1) | The AGRV2KQ32 is programmed only over 2-wire Compact-JTAG with a J-Link or an AGM DAP-Link; a generic CMSIS-DAP (WCH-LinkE in DAP mode) is not confirmed | Owner buys a J-Link or AGM DAP-Link for the CPLD (confirm which models Supra accepts; order with the LCSC jack samples if available there) and a WCH-LinkE for the MCU; program through J202 in programming mode on Stage 2; CPLD builds on a Windows CI runner, or check Supra on Linux | Owner action; Stage 2 |
| 7 | USBHS suspend on the CH32V307 (R7-D DG-10) | WCH’s USBHS example has only a placeholder; Stop turns off HSE and the PLL | Stop with LPDS, PHY suspended, wake on resume, HSE/PHY PLL restart within the 10 ms resume recovery; fallback wake: EXTI on PB6/PB7; measure the current against time | Stage 2 gate |
| 8 | Oscillator disable time and disabled-output state are not specified in the SCTF/YXC datasheets | X202 and X203 share one node at CPLD PIN_1 | Measure on Stage 2 (scope on the shared node); fallback: mux U205 with R216/R217 fitted, built on one Stage 2 board | Stage 2 |
| 9 | AGRV2K I/O state and weak pull-up strength during configuration | The 2.2 kΩ pull-downs must hold OSC48_EN, OSC44_EN and MCLK_EN low against ≥ 5.1 kΩ | Datasheet check and Stage 2 scope check | Stage 2 |
| 10 | Shared oscillator node routing | Two oscillator outputs and the PLL clock pin on one net | Both oscillators and the node next to CPLD pin 1, short traces, idle trace short; scope check of the clock | Layout review; bring-up |
| 11 | MCU and CPLD currents are assumptions; oscillator typicals assumed (datasheet maxima used) | The unconfigured state must stay ≤ 100 mA; the configured totals depend on them | Measure on Stage 2 with the real CPLD design (including the CPLD dynamic current, R7-D DG-21); mitigation: lower MCU clock before configuration | Stage 2; bring-up |
| 12 | USB Default-mode estimated maximum 0.475 A (25 mA margin) | Close to the 500 mA USB 2.0 limit; rests on the current estimates | Stage 2 measurement (attached ≤ 100 mA, configured digital ≤ 262 mA), then the main boards | Stage 2 (layout gate); bring-up |
| 13 | MCU Stop current with the USBHS PHY suspended (suspend budget) | ≤ 1.9 mA is an assumption within the ≈ 2.30 mA budget (margin 0.20 mA, LEDs off); suspend ≤ 2.5 mA within 10 ms is a v1 requirement and the power-down leaves ≥ 0.7 ms margin | Measure the current against time from the last bus activity on Stage 2 (real LP5907 and DAC), then on the main board | Stage 2 gate; bring-up |
| 14 | VBUS watchdog-to-GPIO latency | ≈ 3.8 µs ADC latency against the ≈ 6 µs hold-up (VBUS 4.10 V → 3.46 V; target < 5 µs); the PGOOD latch is the backstop | Measure on Stage 2 | Stage 2 gate |
| 15 | Slow DC-sense network on the real hardware (R7-F F3; CH32V307 I/O leakage, R7-B AN-10) | Redesigned network (4.7 kΩ source): 1 µA pin leakage is only 15 mV at the leg; the error budget (≤ 44 mV per leg, ≤ 88 mV per pair worst case) sets the T4 band | Measure the zero, the dithered-sampling residual and the ratiometric correction on Stage 2; confirm the CH32V307 leakage maximum | Stage 2 |
| 16 | Jack pin numbers (R7-B AN-1) | The jack drawings are image-only; contact names are mapped (3.5.3) but the pin numbers are not verified | Buy 2 samples of each jack (LCSC, ships to Taiwan; with the USB-C receptacle if possible) and continuity-test every pin; check the J101 pin functions against the drawing (R7-A P-10) | Before layout |
| 17 | ESS application-circuit figure (R7-B AN-11) | The ESS p. 27 application circuit is image-only, so its values could not be compared | Owner compares the figure with the I/V design by eye | Before the schematic |
| 18 | DAC pin-to-pin DC mismatch (≤ 1 mV assumed) | Offset budget term of 1.18 mV | Measure; 100 % offset screening with a DC-capable DMM | Bring-up |
| 19 | DAC null: 1.702 V (current-output formula) or 1.65 V (voltage-source model) | VREF-to-null offset term 1.17 mV at 1.702 V, 2.8 mV if the null is 1.65 V (totals 9.3 / 18.6 mV) | VREF null test; R433 1 kΩ → 0 Ω sets VREF = 1.650 V if the test shows the 1.65 V null | First boards; bring-up |
| 20 | DAC-pin excursion with the fitted clamps D405–D408 in a failed-output shutdown and at unplug (R7-F F4; clamp decision closed: fitted, owner decision 24 Sep 2026) | The I/V converters may lose the DAC pins for up to ≈ 5.5 ms after an output-stage failure; the clamps hold them within ≈ −0.40 V … AVCC + 0.40 V, a small residual beyond the ESS rating (no margin given); the upper diodes can briefly back-feed 3V3A (≤ 4.4 mA per pin) | Probe the four DAC pins at the R423–R426 DAC-side pads against AVCC_L/AVCC_R during the failed-output emulation and hot unplug: each must stay within −0.4 V … AVCC + 0.4 V; record the 3V3A decay with the clamps fitted | Main board 1 |
| 21 | Pop at AVCC-on and at power-down (R7-B AN-3, R7-H H2, H5) | In reset the DAC outputs are driven to ground: ≈ 18–35 mV at the leg from AVCC-on until RESETB is released; power-down pop at 300 Ω ≈ 4.5 mV typical, ≈ 12.7 mV at the worst-case discharge; the ≤ 5 mV pop target is at risk | Measure orders A and A′ and option B on main board 1 (I/V outputs and legs scoped during the AVCC ramp with RESETB low); choose the order with the lower pop | Bring-up |
| 22 | OPA1622 clean swing and current capability at the ±3.8 V rails | Clean swing modelled with 1.3 V headroom from TI data at ±5 V; 300 Ω balanced is only +0.04 dB inside full scale | Bench-verify the power table and the clean swing | Bring-up |
| 23 | SPICE: OPA2210 stability with the DAC pin capacitance plus ≤ 20 pF per DAC-pin node from the clamps D405–D408 and the 10 Ω + 660 pF network; OPA2210 output swing; OPA1622 disabled-state output with the network attached; protection timing; failed output stage | I/V converter loop; the swing margins are estimates; pop and protection behaviour | SPICE | Before layout |
| 24 | LM27762 charge-pump resistance (3.0 Ω assumed, 3.5 Ω pessimistic) | Sets the negative-rail headroom: +0.05 V worst at 70 mA per leg (4.4 mm 32 Ω, 3.5 Ω, 4.75 V host) | Measure the negative-rail headroom at VBUS 4.60 V and 4.40 V | Bring-up |
| 25 | LM27762 dissipation and VIN ripple | 0.39 W at 5.25 V (≈ +24 °C), 0.46 W at 5.5 V; FB501 + C507 resonance ≈ 80–100 kHz | Copper pour and IR check; scope the VIN ripple at light load and add 0.47–1 Ω + 1 µF damping if needed | Layout; bring-up |
| 26 | VBUS hot-plug ring (R7-A P-7) | VBUS has ceramics only; LP5907 absolute maximum 6.0 V, LM27762 5.8 V | Scope C102 while hot-plugging a USB-A-to-C cable into a live port on board 1; fit D102 only if the peak exceeds 5.8 V, after measuring its leakage at 5.25 V | Bring-up (board 1) |
| 27 | Stock: CH32V307 (LCSC 193 / JLCPCB 364), ES9018K2M (724), AGRV2KQ32 (980), ME6216A13M3G (≈ 3,100), 4.4 mm jack J701 (JLCPCB 21, LCSC 721), X201 (2,503), OPA2210 (not confirmed) | Low stock (Jun–Sep 2026) | Reserve CH32V307RCT6, ES9018K2M and AGRV2KQ32; buy 7 × J701 (5 + 2) into the JLCPCB library | Before ordering |
| 28 | OPA2210 stock unverified; OPA1612 fallback designed in (R7-E BM-1) | The OPA2210 JLCPCB stock figure is 12 months old; at the LCSC price (US$2.67) the run is ≈ 1,825 NTD per board, above the 1,721 NTD order gate. With the OPA1612 fallback (≈ 1,725–1,740 NTD per board, above the gate; analog quiescent ≈ 72 mA) the Default-mode targets at 3.5 mm 32 Ω (≈ 27 mW against ≥ 38 mW), 4.4 mm 60 Ω (≈ 13 mW against ≥ 17 mW) and 4.4 mm 120 Ω (≈ 26 mW against ≥ 35 mW) would not be met; High mode is unaffected | Check OPA2210IDR stock in the JLCPCB Parts Manager now; if ≥ 12, buy 12 into the private library; otherwise switch U403/U404 to OPA1612AIDR (C94590: V+ 3.90 V keeps VREF ≥ 35 mV inside its range; offset 9.7 / 19.4 mV, at the edge; the Default targets at 3.5 mm 32 Ω and 4.4 mm 60/120 Ω not met, 3.5.5) | Before ordering (owner action) |
| 29 | Cost 2 NTD under the 1,721 NTD order gate (conservative fees) | The margin is very small: lower-fee case ≈ 1,621 NTD (no Extended fee on the through-hole jack, no X-ray of the three oscillators); OPA1612 fallback ≈ 1,725–1,740 NTD (above the gate); OPA2210 at the LCSC price ≈ 1,825 NTD; LM339BIDR instead of LM339DR ≈ 1,715 NTD; the gate (1,700 NTD ceiling + the owner-approved 21 NTD for the clamps) covers every JLCPCB line of the quote | JLCPCB quote from the real BOM/CPL files after layout; confirm the X202 class (assumed Extended); record the gated subtotal and the total payable | JLCPCB quote |
| 30 | Rail-stuck hardware failure in High mode; balanced digital DC | Up to 21.4 mJ into 16 Ω at the worst case (limit 30 mJ); balanced digital DC is larger (32.8 mJ at 16 Ω, 21.7 mJ at 32 Ω if the hardware alone acts; ≈ 10 / 12 mJ with the stream DC guard) and not covered by the rail-stuck requirement | Failed-output emulation with dummy loads; balanced opposite-sign DC and stream-guard tests; v2 option: fast rail-proximity detector | Bring-up; v2 |
| 31 | Unplug pop and DAC pins against AVCC at unplug | Fast VBUS collapse; the DAC-first order is impossible in the ≈ 6 µs hold-up | Measure the unplug pop (≤ 20 mV target) and the four DAC pins against AVCC_L/AVCC_R at hot unplug (C421 33 pF: VREF lag ≤ 15 mV; pass within −0.4 V … AVCC + 0.4 V with the clamps fitted); VBUS watchdog at 4.10 V | Bring-up |
| 32 | 4.4 mm plug shorted leg+ to leg− | Outside TI’s one-amplifier short rating (≈ 0.55 W per package) | Survival test | Bring-up |
| 33 | Both jacks live; 4.4-to-3.5 mm adapter | One headphone at a time | Warning stays in the manual | Manual |

## 7.2 Review disposition, seventh round (R7-A…R7-E, full design check)

IDs keep the reviewers' finding numbers: R7-A = power (P-1…P-22), R7-B = analog (AN-1…AN-13), R7-C = protection (PR-1…PR-19), R7-D = digital (DG-1…DG-21), R7-E = parts and cost (BM-1…BM-15); five independent reviews of Spec v0.7, Notes v0.6 and Parts List v0.5, run before any hardware is bought. Severity: 3 blockers (R7-C PR-1 = R7-A P-1 = R7-D DG-3, AVCC_EN contention; R7-D DG-1, CPLD programming port; R7-E BM-1, OPA2210 stock unverified), 13 high, the rest medium, low or info. Findings raised by more than one reviewer are closed together (P-1 = PR-1 = DG-3; P-2 = PR-2; P-3 = AN-8 = BM-3; P-9 = BM-4; AN-13 = BM-2). Status values: Closed – design (changed now); Closed – design, verify (changed; confirmed on hardware); Closed – firmware rule; Closed – documentation; Open – verify on Stage 2; Open – owner action; Accepted (v1); Info.

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| R7-A P-1 | AVCC_EN has no series resistor; the Q611 latch kill fights the MCU push-pull output | New R307 10 kΩ from PC11 to the LP5907 EN node; Q611 pulls the node low against 0.33 mA; the FLT interrupt also drives AVCC_EN low (same as R7-C PR-1 and R7-D DG-3) | Closed – design |
| R7-A P-2 | RAILS_OK_N cannot rise quickly: R505 100 kΩ is loaded by R623/C607 and R617/C606 | R505 10 kΩ to 5V_ANA, R522 10 kΩ into PB4, R623 1 MΩ, C607 10 nF (C15195): rise < 1 µs, so the ≈ 35 µs capture is genuine; timing recalculated (3.7) | Closed – design, verify |
| R7-A P-3 | 2.2 Ω line: MPN and LCSC number disagree | MPN corrected to UNI-ROYAL 0805W8F220KT5E (C17521) (same as R7-B AN-8 and R7-E BM-3) | Closed – documentation |
| R7-A P-4 | U504 CE can exceed VIN + 0.3 V at unplug under load | New R521 10 kΩ in series from AUD_EN (PC10) into U504 CE: clamp current ≤ 0.2 mA | Closed – design |
| R7-A P-5 | Suspend budget leaves out the D+ pull-up and uses a low LDO quiescent current | Budget ≈ 2.31 mA with the D+ pull-up (0.20 mA) and the 60 µA ME6211 quiescent; margin 0.19 mA (2.2) | Closed – documentation |
| R7-A P-6 | Negative-rail headroom mixed average and peak currents and used the 0.5 A cable drop | Peak currents and 0.3 Ω at the real total current from a 4.75 V host; 70 mA envelope at 4.4 mm 32 Ω (+0.05 V worst) (2.3) | Closed – design |
| R7-A P-7 | VBUS has ceramic capacitors only and no surge or TVS clamp | DNF footprint D102 (VRWM 6.0 V); hot-plug test on board 1; fitted only above 5.8 V after a leakage check at 5.25 V | Closed – design, verify |
| R7-A P-8 | 3V3D and 1V3 have no discharge path | Bleeders R524 2.2 kΩ (3V3D) and R525 1 kΩ (1V3); AUD_EN off ≥ 100 ms before re-enabling | Closed – design |
| R7-A P-9 | ME6216A13M3G pinout not recorded; claims differ from the datasheet | SOT23-3 “M3”: pin 1 VSS, pin 2 VOUT, pin 3 VIN; 250 mA max, ±15 mV, Pd 0.54 W; stock ≈ 3,100 (same as R7-E BM-4) | Closed – documentation |
| R7-A P-10 | USB-C J101 pin functions unchecked; D+/D− contact ties not stated | Tie A6/B6 and A7/B7 at the connector; drawing check before layout (7.1) | Open – verify |
| R7-A P-11 | Minimum delay from ANA_EN to RAIL_EN not specified | RAIL_EN ≥ 10 ms after ANA_EN | Closed – firmware rule |
| R7-A P-12 | Rails start inside the TPS2553 current limit (rail inrush) | Firmware ignores its own ANA_FAULT classification for 20 ms after RAIL_EN; the hardware set is blanked; bench check in Default mode at 4.40 V | Closed – firmware rule, verify |
| R7-A P-13 | PGOOD pulled up to 3V3M while the LM27762 is unpowered | Moot: R505 is now on 5V_ANA, as TI specifies | Closed – design |
| R7-A P-14 | LM27762 dissipation understated | 0.39 W at 5.25 V (θJA 62.2 °C/W, ≈ +24 °C), 0.46 W at 5.5 V (3.6.3) | Closed – documentation |
| R7-A P-15 | U504 evaluated only up to 5.25 V | 0.20 W at 5.25 V and 0.24 W at 5.5 V against 300 mW (160 mA incl. the bleeders); input ≥ 3.70 V at VBUS 4.40 V | Closed – documentation |
| R7-A P-16 | 3V3A discharge time used the typical 230 Ω | 5.5 ms worst case (+50 %, 6.7 µF) used in the power-down timing; suspend level ≈ 9.3 ms (2.5) | Closed – documentation |
| R7-A P-17 | CC firmware thresholds not stated | 0.66 V (Default ↔ 1.5 A) and 1.23 V (1.5 A ↔ 3.0 A) with hysteresis; only the CC pin with Rp is active | Closed – firmware rule |
| R7-A P-18 | The < 5 µs VBUS watchdog target needs a dedicated ADC | ADC1 continuous single-channel on VBUS_SENSE with the analog watchdog, 7.5-cycle sampling at 12 MHz (≈ 3.8 µs latency); ADC2 scans CC and DC sense | Closed – design |
| R7-A P-19 | Shell capacitor C101 overstressed by ESD; no ESD device on CC | R107 0 Ω (shell tied to GND), C101 DNF; CC1/CC2 rely on the 10 kΩ series resistors | Closed – design |
| R7-A P-20 | C15849 rating, ME6211 dropout and LM27762 symbol pins stated wrongly | C15849 1 µF 0603 is 50 V; ME6211 dropout 120 mV typical at 100 mA; LM27762 PGOOD = pin 1, FB− = pin 7 | Closed – documentation |
| R7-A P-21 | Start-up into the ME6211/ME6216 foldback current limits | AUD_EN cycled 100× at VBUS 4.40 V and 5.25 V on Stage 2, checking 3V3D and 1V3 reach regulation; MCLK stays off until DVDD is up | Open – verify on Stage 2 |
| R7-A P-22 | FB501 + C507 LC resonance ≈ 80–100 kHz | Check the LM27762 VIN ripple at light load; add 0.47–1 Ω + 1 µF damping if needed | Open – verify |
| R7-B AN-1 | Jack pin numbers not documented or verified | Placeholders beside the contact names (3.5.3); buy 2 samples of each jack and continuity-test every pin before layout; checklist rule 73 | Open – owner action |
| R7-B AN-2 | OPA1622 clean swing overestimated, so calibrated 0 dBFS clips | Swing model with 1.3 V headroom; calibrated 1.55 Vrms per leg; −4.5 dB pre-calibration default; new power table and targets (3.5) | Closed – design |
| R7-B AN-3 | DAC reset state not modelled: tens of mV at the jack at AVCC transitions | Power-down keeps RESETB high and stops MCLK (MCLK_EN) before AVCC goes off; order A by default, option B as a firmware alternative; pop target at risk until measured | Closed – design, verify |
| R7-B AN-4 | Pin-level connections not documented for U401/U402 and U301 | OPA1622 and ES9018K2M pinouts and connections written down (3.4, 3.5.2) and covered by checklist rules | Closed – documentation |
| R7-B AN-5 | Leg-network noise underestimated | 11.5 nV/√Hz (≈ 13.0 nV/√Hz with the OPA1622); DR ≈ 117.4 / 118.6 dB(A) at 1.55 Vrms (3.5.6) | Closed – documentation |
| R7-B AN-6 | Default-mode unbalanced peak rule leaves out the charge-pump factor | Peak = 53 mA + 2 × 1.03 × Ipk; power table recalculated (3.5.5) | Closed – documentation |
| R7-B AN-7 | The 0.47 Ω link option cannot fix capacitive-load instability | Keep the 0 Ω links; test with 1 nF plus long cables; last resort 2.2–4.7 Ω (breaks the ≤ 0.5 Ω target) | Closed – documentation, verify |
| R7-B AN-8 | 2.2 Ω MPN decodes to 22 Ω | 0805W8F220KT5E (same as R7-A P-3 and R7-E BM-3) | Closed – documentation |
| R7-B AN-9 | Disabled-leg DC path; slow DC tier gated by AMP_EN | Interlock: AVCC_EN only while AMP_EN is high and the rails are good; the slow DC tier is armed whenever AVCC_EN is high | Closed – firmware rule |
| R7-B AN-10 | Slow DC-sense accuracy depends on the MCU pin leakage | Superseded by the redesign (R7-F F3, 7.3): 15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF, 4.7 kΩ source, 1 µA leakage = 15 mV at the leg | Closed – design, verify |
| R7-B AN-11 | ESS application-circuit values (p. 27) could not be compared | Owner compares the image-only figure with the I/V design by eye before the schematic (7.1) | Open – owner action |
| R7-B AN-12 | Small numeric inconsistencies | VREF τ = 7.74 kΩ × 330 pF = 2.55 µs and the related figures corrected | Closed – documentation |
| R7-B AN-13 | J701 JLCPCB stock is 21, not 721 | Buy 7 (5 + 2) into the JLCPCB library (same as R7-E BM-2) | Open – owner action |
| R7-C PR-1 | AVCC_EN: Q611 fights the MCU push-pull output on every latch trip (blocker) | R307 10 kΩ; FLT interrupt drives AVCC_EN low (same as R7-A P-1 and R7-D DG-3) | Closed – design |
| R7-C PR-2 | Capture claim ignores the load on RAILS_OK_N | As R7-A P-2: R505 10 kΩ to 5V_ANA, R522, R623 1 MΩ, C607 10 nF; timing recalculated | Closed – design, verify |
| R7-C PR-3 | LM393 input bias current missing from the worst case | U601 → TI LM393BIDR in this round; superseded by the LM339 windows (R7-G G2, 7.3) | Closed – design |
| R7-C PR-4 | Faults present at power-up are blanked in hardware | Firmware: PGOOD timeout 20 ms, 1 kHz start-up DC watch (drop RAIL_EN above 0.5 V), first power-up of every board without headphones | Closed – firmware rule |
| R7-C PR-5 | Arming race after a short rails-off time | FLT_CLR only ≥ 600 ms after ANA_EN or rails off; second FLT_CLR at PGOOD good + 20 ms; AMP_EN ≥ 250 ms after PGOOD good | Closed – firmware rule |
| R7-C PR-6 | Firmware reaction leaves enables, MCLK and RESETB driven after a trip | FLT interrupt order: enables, AUD_EN and RESETB low within 50 µs, then cause capture, then ANA_EN low; IWDG enabled | Closed – firmware rule |
| R7-C PR-7 | MCU internal pulls on the latch-node inputs not specified | FLT, RAILS_OK_N and ANA_FAULT always floating inputs; ANA_FAULT only open-drain low; unit test on the GPIO table | Closed – firmware rule |
| R7-C PR-8 | Cause classification used instantaneous readings | Pre-trip 0.5-s average from a ring buffer; the instantaneous value only confirms a hardware trip, or equal sign on L+/L− is required | Closed – firmware rule |
| R7-C PR-9 | PGOOD pull-up to 3V3M while VIN = 0 V | Moot: R505 on 5V_ANA | Closed – design |
| R7-C PR-10 | No local decoupling for U601 | C612, C613 100 nF from V+ and V− to GND | Closed – design |
| R7-C PR-11 | DC-sense source impedance against the ADC sampling | Superseded by the redesign (R7-F F3, 7.3): dithered 1 kHz sampling, 239.5-cycle sampling, auto-zero and ratiometric correction | Closed – firmware rule, verify |
| R7-C PR-12 | DC-sense node below 0 V at the negative extreme | Fixed by the redesign (R7-F F3, 7.3): the node stays 0.29–2.85 V at every corner | Closed – design |
| R7-C PR-13 | Documented latch and blanking timing out of date | Stretch 1.5–7.7 ms, Q609 on 1.9–9.4 ms, re-blank 19–94 ms, arming 51–186 ms (3.7) | Closed – documentation |
| R7-C PR-14 | Ambiguous connections (BAV99, comparator inputs, returns, gates, OPA1622 pin 3) | Netlist details added in 3.5.2 and 3.7 and in the checklist | Closed – documentation |
| R7-C PR-15 | LM393DR2G ESD rating only 250 V HBM | LM339DR ESD ±2 kV HBM (TI SLCS006Z), which closes this finding for the fitted LM339DR too; the LM339BIDR (2 kV HBM) stays a drop-in if JLCPCB stocks it (R7-G G2, 7.3) | Closed – design |
| R7-C PR-16 | C605 discharges directly into the MCU pin or the TPS2553 FAULT | R638 1 kΩ in series with the D606 cathode | Closed – design |
| R7-C PR-17 | Q609 slow gate ramp may make AMP_EN chatter | Moot with the PR-2 fix: the latch acts first | Closed – design |
| R7-C PR-18 | A quick replug may start with the latch cleared | Harmless: the enables are pulled down | Accepted (v1) |
| R7-C PR-19 | Unplug description inconsistent; PGOOD backstop questioned | Unplug interrupt order documented (2.5); PGOOD goes bad at VBUS ≈ 3.70 V with 5V_ANA ≈ 3.6 V, so Q610 still turns on | Closed – documentation |
| R7-D DG-1 | The CPLD cannot be programmed as documented (blocker) | J202 = 2-wire Compact-JTAG: 3V3D sense, JTCK (25), JTMS (24), NRST (4), GND; owner buys a J-Link or an AGM DAP-Link (7.1) | Closed – design; Open – owner action (probe) |
| R7-D DG-2 | Only one dedicated clock pin on the QFN-32 | Family clock on PIN_1 into the PLL; no 80 MHz input; SPI SCK through ALTCLKCTRL; FIFO written in the SPI domain (3.3) | Closed – design |
| R7-D DG-3 | MCU output fights the Q611 kill FET | R307 (same as R7-A P-1 and R7-C PR-1) | Closed – design |
| R7-D DG-4 | No defined way to power the CPLD for programming | Programming mode on the PROG strap (TP732, PC13); the J202 probe senses 3V3D only (2.5) | Closed – design |
| R7-D DG-5 | The < 5 µs watchdog target needs a dedicated ADC | ADC1 continuous on VBUS_SENSE at fADC 12 MHz; ADC2 for the other channels (as R7-A P-18) | Closed – firmware rule |
| R7-D DG-6 | MCLK can back-power VCCA after a latch trip | FLT interrupt drops AUD_EN and RESETB; MCLK_EN stops X201 before AVCC goes off; IWDG | Closed – firmware rule + design (MCLK_EN) |
| R7-D DG-7 | Missing DAC register stop_div | Register 10 = 0x00 (stop_div = 0, required above 96 kHz); register 1 = 0x80; both in the unit tests | Closed – firmware rule |
| R7-D DG-8 | Idle levels of the MCU → CPLD link | LINK_FRAME active high (idle low), CPOL = 0; outputs low or input while AUD_EN is low; R224/R225 33 Ω; CPLD weak pull-downs; I2C2 recovery | Closed – design |
| R7-D DG-9 | MCU internal pulls on latch-node inputs | As R7-C PR-7 | Closed – firmware rule |
| R7-D DG-10 | Suspend path unproven on this chip | Stop with LPDS, PHY suspended, wake and HSE/PHY PLL restart within 10 ms; fallback wake EXTI on PB6/PB7 | Open – verify on Stage 2 |
| R7-D DG-11 | One PLL must lock at 22.5792 and 24.576 MHz | Lock test at both inputs; fallback BCLK ÷ 1 directly from X202/X203 (duty 45–55 %, zero margin) | Open – verify on Stage 2 |
| R7-D DG-12 | SPI clock quantised: 36 MHz is the only workable setting | 36 MHz, register reads at ≤ 18 MHz; second lane reserved (SPI2 PB12–PB15) | Open – verify on Stage 2 |
| R7-D DG-13 | CPLD supply isolation | FB202 to the VDD33/VDDA33 branch; C220 1 µF at VDDA33; 100 nF per supply pin | Closed – design |
| R7-D DG-14 | J201 has no NRST or BOOT0 pad | J201 with NRST, a BOOT0-to-3V3 pad and a USART1 TX pad | Closed – design |
| R7-D DG-15 | X201 load and duty margin | CPLD branch removed; XI trace ≤ 20 mm; duty measured at XI | Closed – design |
| R7-D DG-16 | DAC pin details not written down | ADDR, XO, N.C., DATA pins and GPIO2 documented (3.4) | Closed – documentation |
| R7-D DG-17 | RESETB injection | R308 1 kΩ in series; RESETB dropped first in the interrupts | Closed – design |
| R7-D DG-18 | VBAT handling not stated | VBAT tied to 3V3M; one 100 nF per supply pin plus two extras; C221 1 µF at VDDA | Closed – design |
| R7-D DG-19 | Link word order | 8-bit frames or a halfword swap in the CPLD; known-pattern test; MISO returns the FIFO level | Closed – documentation |
| R7-D DG-20 | Descriptor details not stated | Feedback wMaxPacketSize 4, isochronous OUT ≥ 392, non-integer 44.1 kHz feedback, bcdDevice bump, VID/PID 1209:0001 for private testing only | Closed – documentation |
| R7-D DG-21 | The CPLD is AG32 silicon in pure-CPLD mode | Dynamic current measured on Stage 2; Windows-centred toolchain (Windows CI runner, or Supra on Linux) | Open – verify |
| R7-E BM-1 | OPA2210 stock rests on a 12-month-old JLCPCB figure (blocker until verified) | Owner checks the JLCPCB Parts Manager now and buys 12 if ≥ 12 are available; otherwise OPA1612AIDR (C94590), designed in with V+ at 3.90 V | Open – owner action |
| R7-E BM-2 | J701 JLCPCB stock is 21, not 721 | Buy 7 (5 + 2) into the JLCPCB library (same as R7-B AN-13) | Open – owner action |
| R7-E BM-3 | Wrong MPN on the 2.2 Ω line | 0805W8F220KT5E (same as R7-A P-3 and R7-B AN-8) | Closed – documentation |
| R7-E BM-4 | ME6216A13M3G package and pinout | As R7-A P-9 | Closed – documentation |
| R7-E BM-5 | Cost-model prices stale and too low for some passives | Prices refreshed to LCSC on 24 Sep 2026; 1,691 NTD per board (Section 8) | Closed – documentation |
| R7-E BM-6 | J702 flagged “Assembly Difficulty High” | Kept (allowed in Economic PCBA); the alternative PJ-332-6A (C962626) is not qualified (pin map unknown) | Accepted (v1) |
| R7-E BM-7 | Mixed price basis | Noted in Section 8 (OPA2210 at the JLCPCB page price, the other lines at LCSC prices) | Info |
| R7-E BM-8 | C15849 is rated 50 V, not 25 V | Corrected in the parts table | Closed – documentation |
| R7-E BM-9 | 2.2 µF 0402 6.3 V X5R running at 3.3 V (DC-bias loss) | Optional C303/C304 as 2.2 µF 16 V 0603 (C23630) | Not adopted (optional) |
| R7-E BM-10 | Green LED current about 3× lower than stated | R701 2.2 kΩ → 1 kΩ; R702 stays 2.2 kΩ | Closed – design |
| R7-E BM-11 | No input-voltage margin on the LP5907 and LM27762 above 5.5 V | Accepted for v1; VBUS measured on the chargers used | Accepted (v1) |
| R7-E BM-12 | Counts and stale figures in the documents | Counts corrected: 152 BOM lines, 263 placed parts, 20 Extended + 27 Basic part types, 755 SMT joints | Closed – documentation |
| R7-E BM-13 | Fee rules against JLCPCB’s Economic PCBA table (08 Sep 2026) | Fee lines match the published table | Info |
| R7-E BM-14 | Economic PCBA limits | Single-sided placement, 2–50 pcs and the standard stack-up are met | Info |
| R7-E BM-15 | Part ratings against their use | All ratings check out | Info |
| R6-B2 (earlier round) | Residual DC region below 0.2 V per leg is a product-safety decision | Accepted by the owner on 24 Sep 2026 (≤ 2.5 mW into 16 Ω, ≤ 1.25 mW into 32 Ω) | Closed – owner decision |

## 7.3 Review disposition, seventh round, external reviews (R7-F, R7-G, R7-H)

IDs: R7-F = external review 1 (items F1–F6), R7-G = external review 2 (G1 negative ADC range, G2 BAV99 threshold, G3 owner decision, G4 release conditions), R7-H = external review 3 (H1–H8 plus minors): three external reviews of Spec v0.7, Notes v0.6 and Parts List v0.5, answered by amending this revision on 24 Sep 2026. Many items were already fixed in the R7-A…E round (7.2); the new design changes are the LM339 window comparators, the redesigned slow DC-sense network with the 3V3A monitor, the per-leg and per-pair firmware tiers, C421 33 pF, the clamps D405–D408 (fitted by owner decision on 24 Sep 2026), the adaptive AMP_EN and the stream DC guard. Status values as in 7.2.

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| R7-F F1 / R7-H H1 | R623/C607 and R617/C606 load RAILS_OK_N (R505 100 kΩ): the node steps to ≈ 1.57 V, Q610 capture up to 15–19 ms, not 35 µs | Already fixed in R7-A P-2 / R7-C PR-2, using both of R7-H’s options: R505 10 kΩ to 5V_ANA (TI datasheet: “connect a 10-kΩ pullup resistor (RPU) to VIN”), R623 1 MΩ, C607 10 nF. Recomputed with the real loads: RAILS_OK_N steps to 4.31–5.15 V (0.98 × 5V_ANA) and crosses 2.5 V in ≈ 1.0–1.3 µs; capture ≈ 35 µs holds. New checklist rule 75: RAILS_OK_N may carry only ≥ 1 MΩ RC branches and high-impedance gate/MCU inputs (R522). The pulse test emulates the real open-drain release (OUT+ pulled below 95 % with a switched load, or the node released through an open-drain FET), never PGOOD forced with a low-impedance generator | Closed – design, verify |
| R7-F F2 / R7-H 2 | AVCC_EN has no series resistor | Already fixed: R307 10 kΩ (R7-A P-1) | Closed – design |
| R7-F F3 / R7-G G1 / R7-H 3 | Slow DC-sense network: 333 kΩ source against the 50 kΩ ADC limit and 1 µA pin leakage (1 V leg error); node reaches −0.18 V, outside the ADC range | Redesigned (3.7): 15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF, source 4.7 kΩ, node 0.29–2.85 V at every corner, leakage error ≤ 15 mV at the leg, full error budget, auto-zero, 3V3A monitor | Closed – design, verify |
| R7-F F4 | VREF tracking does not prove DAC-pin safety in a loaded failed-output shutdown; the test measures VREF, not the DAC pins; “below junction conduction” is not an ESS allowance | Analysed (3.5.1): with a stuck leg the I/V positive common-mode margin is gone 0.11–0.39 ms after the kill while 3V3A needs 3.2–5.6 ms to fall below 0.3 V, so the I/V converters lose the DAC pins for up to ≈ 5.5 ms; unclamped they could be driven to ≈ −0.6 V (≤ 3.9 mA into the DAC pin) or ≈ 0.33 V above AVCC. ESS gives only “GND < Vout < AVCC” with no margin. BAT54S clamps D405–D408 fitted (owner decision 24 Sep 2026): the pins are held within ≈ −0.40 V … AVCC + 0.40 V (≤ 4.4 mA per pin), and the diodes never conduct in normal operation or in controlled transitions. Unplug excursion reduced from ≤ 0.13 V to ≤ 15 mV (C421 330 pF → 33 pF). Tests now probe the four DAC pins (at the R423–R426 DAC-side pads) against AVCC_L/AVCC_R during the failed-output emulation and at unplug: within −0.4 V … AVCC + 0.4 V | Closed – design (clamps fitted, owner decision 24 Sep 2026), verify; small residual: ≤ 0.4 V beyond the rails for ≤ 5.5 ms after an output-stage failure |
| R7-F F5 | Residual-DC acceptance used 0.2 V across the headphone, but the thresholds are per leg: opposite-sign legs give 0.4 V across a balanced driver (4× power) | Firmware tiers now act on every leg and on each balanced pair difference (L+ − L−, R+ − R−); T4 threshold 0.11 V nominal, worst-case trip ≤ 0.198 V differential after the full error budget, so the accepted residual ≤ 2.5 mW (16 Ω) / 1.25 mW (32 Ω) now holds for balanced headphones too | Closed – firmware rule, verify |
| R7-F F6 | The classifier can mistake bass for DC (instantaneous 33 ms-filtered reading) | Already changed (R7-C PR-8) to the pre-trip rolling 0.5-s average plus event history; added: events that match neither pattern are logged as “ambiguous” and counted toward the DC three-strike limit (conservative policy, deliberate) | Closed – firmware rule |
| R7-G G2 | The hardware “worst case” relies on an extrapolated BAV99 VF at ≈ 0.4 µA (datasheet guarantees only 1 mA) | Topology changed (3.7): two quad comparators (TI LM339DR), one window per leg directly on its filter node, no diode-OR, so the threshold is set only by the divider, the comparator offset and the bias current, all datasheet-guaranteed. Leg trip 0.43–0.67 V worst case; 1 V ≤ 125 ms, 2 V ≤ 47 ms; rail-stuck energy ≤ 21.4 mJ (16 Ω) / 11.0 mJ (32 Ω) | Closed – design |
| R7-G G3 | Owner decision on the blind region; suggests testing 0.1 V / 5 s | Owner accepted (24 Sep 2026). T4 is now 0.11 V nominal (≤ 0.198 V guaranteed, 2-s window, ≤ 3 s), effectively the suggested 0.1 V; the worst-case band is stated | Closed – owner decision |
| R7-G G4 | Release conditions: ME6216/LM27762 thermal data, OPA2210 SPICE, Stage 1/2 gates | ME6216 Pd 0.54 W (datasheet) against 0.12 W; LM27762 RθJA 62.2 °C/W (datasheet) → ≈ +24 °C at 0.39 W: closed. OPA2210 stability SPICE: open, before layout. Stage 1 is optional (owner constraint); its gates run on Stage 2 | Partly closed |
| R7-H H2 | Legs live while the DAC powers up or down: unbounded DAC transients reach the jack | Covered by R7-B AN-3: DAC-reset step measured; firmware orders A (legs first), A′ (AUD_EN together with AVCC_EN, allowed by ESS: DVCC with or after VCCA) and B (DAC first, legs last), chosen by the measured pop on main board 1 (Stage 2 has no output stage) | Open – verify |
| R7-H H3 | VBUS watchdog latency in a 7-channel scan | Already fixed (R7-D DG-5): ADC1 continuous on VBUS_SENSE only; the scanned channels are on ADC2 | Closed – design |
| R7-H H4 | LM27762 PGOOD state while unpowered | Moot: PGOOD now pulled up to 5V_ANA (TI recommendation) | Closed – design |
| R7-H minor (power-down pop) | Power-down pop: AMP_EN low while 3V3A ≈ 0.24 V | AMP_EN now drops when the new 3V3A monitor reads < 0.1 V (typically t0 + 4.7 ms, pop ≈ 4.5 mV at 300 Ω incl. the sense-bias path) with a deadline at t0 + 5.8 ms (worst-case discharge: 3V3A ≈ 0.28 V, pop ≈ 12.7 mV at 300 Ω, measured); suspend level typically ≈ 8.2 ms, at worst 9.3 ms after the last bus activity | Closed – design, verify |
| R7-H H5 | Step 4 assumes pair-symmetric DAC pins with DVCC off | Same as H2 / AN-3; test “I/V outputs and legs scoped during the AVCC ramp with RESETB low” added; fallback A′ | Open – verify |
| R7-H H6 | Suspend budget omits the LEDs | Firmware rule: both LEDs off in suspend (0 mA in the budget) | Closed – firmware rule |
| R7-H H7 | Step 2 wording; resume latency | Wording already “≥ 600 ms after ANA_EN or after the rails last went off”; resume-to-audio ≈ 1.0 s (0.6 s blanking charge + rails + 250 ms + DAC init + 105 ms soft start) stated | Closed – documentation |
| R7-H H8 | DC in program material trips the board | New firmware stream DC guard in the MCU before the DAC: (a) no zero crossing for 25 ms and ¦mean¦ ≥ 0.25 FS → output zeroed at once; (b) 0.2-s mean ≥ −30 dBFS → soft mute; unmute after 1 s below −40 dBFS. DC in program material therefore mutes (reported to the host log) instead of latching; stated as behaviour | Closed – firmware rule |
| R7-H envelope | The owner decision should cover the whole sub-hardware-trip envelope | Envelope stated per tier (3.7), all ≤ 28.2 mJ at 16 Ω and ≤ 14.1 mJ at 32 Ω | Closed – documentation |
| R7-H minors | C211 note “≈ 38 µC”; X201 “208 ×”; C605 discharge through Q610 | C211 note ≈ 42.5 µC; “≈ 208 ×”; note added: C605 discharges through Q610 in a sub-µs ≈ 0.5 A pulse, fine for a 2N7002, so no series resistor is added there | Closed – documentation |
| R7-F cost | Workbook consistent at 8,248 NTD (v0.5) | Now 8,594 NTD / 1,719 NTD per board after these changes, including the DAC-pin clamps D405–D408 fitted by owner decision (Section 8) | Info |

## 7.4 Review disposition, sixth round (R6-A, R6-B, R6-C, R6-D)

Kept as history: the values in this table are as closed in Notes v0.6 at that time; the current values are in the body of this revision. IDs: R6-A = review 1 of Spec v0.6, Notes v0.5 and Parts List v0.4 (latch timing, power-down, suspend, fault test, cause capture, cost gate), R6-B = review 2 (worst-case protection, blind region, suspend wording, fixture in the gate), R6-C = review 3 (3-strike lockout, mux inputs, OPA2210 single source, minors), R6-D = review 4 (pop vs offset, latch on controlled power-down, LDO dissipation, DAC centre voltage, OPA2210 output swing, watchdog timing, rail floating, 3V3D bulk, minors). Status values: Closed (fixed in this revision); Closed – design, verify (design change made; a bench or proof-of-concept check confirms it); Closed – verify (confirmed by a later check); Open – owner decision and Open – tracked (item with owner and gate).

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| R6-A1 | Latch can miss short PGOOD faults; latch delay missing from the energy figures | C605 moved to node F (power-on set and ≥ 1.5 ms set-request stretch); 47 kΩ latch pull-ups; capture ≈ 35 µs (budget 0.1 ms); Q609 slowed to a static interlock (6–32 ms), so a PGOOD glitch either latches or does nothing; energy computed through the rail-disable instant | Closed – design, verify |
| R6-A2 | Power-down does not establish safe DAC-pin voltage or the post-trip energy bound | VREF tracks AVCC (C421 330 pF; D401, C422 removed); DAC analog off first with the I/V and legs powered, rails last; latch also kills AVCC_EN (Q611); after-trip bound 0.51 mJ (both rails, C +10 %, maximum rails) + ≤ 1 µJ back-power → ≤ 0.6 mJ; unplug case bench-checked | Closed – design, verify |
| R6-A3 | ≥ 50 ms shutdown conflicts with USB suspend timing | Power-down ≤ 4 ms after the suspend interrupt, suspend level ≈ 7 ms after the last bus activity; suspend is a v1 requirement; Stage 2 carries the real regulators with dummy loads and measures current vs time | Closed – design, verify |
| R6-A4 | Hardware fault test tests detection, not removal | Detector test kept; failed-output emulation from the board’s own rail through an external switch with dummy loads, V²/R integrated; FLT high; no +20 % allowance | Closed |
| R6-A5 | PGOOD read after the trip cannot identify the cause | Per-leg DC readings (independent ADC path) and the firmware-trip flag stored first at the interrupt; ≥ 0.3 V → DC fault; unknown → DC fault | Closed – design, verify |
| R6-A (cost) / R6-B4 | Fixture fee outside the ceiling | Gate covers every JLCPCB line incl. any fixture; gated subtotal and total payable recorded; the fixture sentence is standard page text | Closed |
| R6-B1 | Protection limits are nominal | Full worst case (R, C incl. DC bias, LM393 offset, BAV99 VF, rails); R601–R604 100 kΩ, R609/R610 10 kΩ → 2 V ≤ 59 ms (41 % margin), 1 V ≤ 183 ms (27 %); energy 26.2 / 13.3 mJ worst ≤ 30 / 15 mJ | Closed – design, verify |
| R6-B2 | 0.755 V blind region is a product-safety decision | Firmware slow tier on every leg (≥ 0.8 V in 1 s, ≥ 0.2 V in 3 s) sets the same latch; residual < 0.2 V (≤ 2.5 mW into 16 Ω) | Open – owner decision |
| R6-B3 | Suspend wording contradicts the gate | Suspend ≤ 2.5 mA is a v1 requirement everywhere | Closed |
| R6-B (cost) / R6-C3 | OPA2210 single source decides the ceiling | No qualified second source (OPA2209AIDR stock 0; OPA1612 fails the common-mode range at the minimum rail); reserve 12; fallback LCSC price | Open – tracked |
| R6-C1 | Default-mode overloads can lock the board off | Overloads count toward the 3-strike stop only at the −12 dB ceiling; DC faults counted | Closed |
| R6-C2 | Mux inputs float with U205 fitted | DNF R216, R217 10 kΩ pull-downs fitted with U205 | Closed |
| R6-C (minor) | Clipping before a trip near the Default limit; link framing | Scope test added; SPI with chip-select framing | Closed |
| R6-D1 | Pop target conflicts with the offset limit | Pop defined as the band-limited transient beyond the settled DC step; legs enabled/disabled while VREF = 0; SPICE item for the disabled-state output | Closed – verify |
| R6-D2 | Every controlled power-down sets the latch | Expected and masked by firmware (not logged, counted or applied) | Closed |
| R6-D3 | LDO dissipation not analysed | Dissipation table; R519 + R520 dropper keeps U504 ≤ 0.20 W; copper and IR check | Closed – verify |
| R6-D4 | Two DAC centre voltages | Both ESS values stated; offset 8.6 mV worst if 1.65 V; R433 0 Ω option | Closed – verify |
| R6-D5 | OPA2210 output swing not quantified | ≈ (V+) − 0.3 V at ≈ 3 mA; 0.43 V margin calibrated; −3.5 dB pre-calibration default is a safety setting; SPICE | Closed – verify |
| R6-D6 | VBUS watchdog timing margin | Quantified (≈ 1 µs margin at 0.47 A); PGOOD latch as backstop; latency measured on Stage 2 | Closed – verify |
| R6-D7 | Positive rail can float during the rails-off window | Window removed by the new order; the latch trip collapses AVCC in hardware | Closed |
| R6-D8 | 3V3D bulk capacitance | C512 10 µF at U504 | Closed |
| R6-D (minor) | ≈ 208 ×; attach charge; test-pad table; node F margin; BCLK edge; DR loopback floor; Spec C.2 A-M2 bracket | All updated as listed in this round | Closed |

## 7.5 Review disposition, fifth round (R5-A, R5-B, R5-C)

Kept as history: the values in this table are as closed in Notes v0.5 at that time; the current values are in the body of this revision. IDs: A = R5-A (review 1 of Spec v0.5, Notes v0.4 and Parts List v0.3), B = R5-B (review 2), C = R5-C (review 3); status values as in 7.4.

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| R5-A1 / R5-B1 | Spec §4 Key-parts table showed placeholder tokens | The saved v0.5 table was repaired during the amendment (the review saw an interim copy); v0.6 table rebuilt and the saved file scanned for tokens; CI document-lint check added | Closed |
| R5-A2 / R5-B6 / R5-C1 | Shared oscillator outputs need break-before-make; X203 enabled when OE is open | Both enables 2.2 kΩ pull-downs (off at power-up and during CPLD configuration); node pull-down R214; enables mutually exclusive in CPLD logic; sequence with ≥ 1 ms both-off, ≥ 10 ms start-up, PLL held in reset; SCTF/YXC datasheet facts cited (no disable time specified → measured); DNF mux U205/R215 fitted on one Stage 2 board | Closed – design, verify |
| R5-A3 / R5-B2 | 100 MHz MCLK at or above the DAC limit; “worst duty” wording | 80 MHz SCTF SX3M80 (12.5 ns cycle, ≥ 5.6 ns pulse, 208 × 384 kHz); “ideal duty 50 %”; all 8 rates stay in the gate | Closed – design, verify |
| R5-A4 | 0.473 A is an estimate, not a whole-board limit; CC changes not monitored | “Estimated maximum with hardware-limited analog current”, now 0.469 A; Stage 2 measurements before layout; CC monitoring within tSinkAdj | Closed – design, verify |
| R5-A5 | OPA2197 input-pair transition region at VREF | OPA2210 (single input pair, 0.47 V common-mode margin, 35 µV, 2.2 nV/√Hz) | Closed |
| R5-A6 | Shutdown back-powering; AVCC can collapse faster than VREF | ES9018K2M abs. max cited; rails-first-off power-down; watchdog also drops RAIL_EN; VREF clamp D401 and C421 100 nF | Closed – design, verify |
| R5-A7 / R5-B3 | Overload restarts when PGOOD recovers; overload does not latch | Q610 (PGOOD bad) and D606 (TPS2553 FAULT) set the latch; firmware clear required; ≥ 1 s recovery, ceiling steps, 3-strike stop | Closed – design, verify |
| R5-A8 / R5-B5 | Protection envelope and terminal energy | Envelope by DC magnitude (t(V)); energies recalculated as V²/R × t for digital DC and a rail-stuck failure; both rails 0.46 mJ; hardware fault-injection test added | Closed – design, verify |
| R5-A9 | Divider resistor tolerance omitted | Included; R518 5.1 kΩ (−3.722 V); design-corner margin 0.11 V | Closed |
| R5-A10 / R5-C5 | Balanced DR advantage assumes uncorrelated noise | Correlated model: balanced ≈ 120.3 vs unbalanced ≈ 119.5 dB(A) (+0.85 dB) | Closed |
| R5-A11 | QA403 cannot measure DC offset | DC-capable DMM for the 100 % offset screening | Closed |
| R5-A12 | External MCLK contention at the test header | J703 MCLK is a monitor output via R703 1 kΩ; external source drives BCLK/LRCLK/SDATA only | Closed |
| R5-A13 / R5-B9 | 1,700 NTD called “owner-approved”; ceiling policy | Owner statement recorded (budget flexible as long as it does not surge); 1,700 kept as the hard order gate; the JLCPCB quote must pass; exclusions listed; estimate now 1,647 | Closed |
| R5-A14 | “Every class verified” wording; fixture fee not included | X202 exception stated; possible fixture fee (X202 and OPA2210 pages) listed as excluded, to confirm in the quote | Closed |
| R5-B4 | USB compliance only at bring-up | Stage 2 mandatory; unconfigured, configured and suspend currents measured before layout; layout blocked unless all pass | Closed – design, verify |
| R5-B7 | “Exact sample rates” | “Exact integer clock ratios”; absolute rate within the oscillator tolerance, absorbed by the asynchronous feedback | Closed |
| R5-B8 | Stage 2 should be mandatory | Mandatory, ≈ US$79 ≈ 2,500 NTD for two boards (outside the board budget) | Closed |
| R5-C2 / R5-C3 | Close the X203 datasheet item; “only one runs” wording; PLL ×2 justification | Datasheet facts cited; “only one drives the node”; idle-trace rule; PLL ×2 also fixes the 45–55 % oscillator duty | Closed |
| R5-C4 | Default-mode power from average current | Instantaneous-peak method; new table and targets | Closed |
| R5-C6 | Blanking 28–119 ms vs 100 ms wait | 25–133 ms incl. R/C tolerance; AMP_EN after PGOOD good ≥ 150 ms; FLT_CLR after ≥ 200 ms rails off | Closed |
| R5-C7 | 0.1 V margin on the 4.30 V VBUS watchdog | 4.10 V (3.95–4.25 V), ≥ 0.15 V margin to the 4.40 V stress corner | Closed |
| R5-C8 | K1000 swing margin ≈ 0.06 dB | With resistor tolerance the worst unit reaches the swing limit 0.27 dB below full scale at 120 Ω balanced; rated as swing-limited; bench-verified | Closed – verify |
| R5-C9 | Continuous CC monitoring | Firmware requirement (≤ 5 ms sampling, tRpValueChange debounce, tSinkAdj response) | Closed – design, verify |
| R5-C10 | MCU link sampled by the system clock | Link captured in its own clock domain with a word toggle synchronizer | Closed – design, verify |
| R5-C11 | Versioning and stale rows | New files Spec v0.6 / Notes v0.5 / Parts List v0.4; Spec history notes that v0.5 first used the interim divider; Notes earlier-round rows R1-B1 (7.7 µF) and R2-M5 (1,550 / 1,536) marked as the design at that time with the current values | Closed |

## 7.6 Review disposition, fourth round (R4-A, R4-B, R4-C)

Kept as history: the values in this table are as closed in Notes v0.4 at that time; the current values are in the body of this revision. IDs: A = R4-A (review 1 of Spec v0.4), B = R4-B (review 2), C = R4-C (review 3); status values as in 7.4.

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| A-H1 | FB101 mis-specified (GZ1608D601TF = 200 mA / 450 mΩ, C1002; C1015 is GZ2012D101TF) | Power ferrite now Murata BLM18PG121SN1D (C14709, Basic, 2 A, 50 mΩ) in the 5V_ANA path; GZ1608D601TF (C1002) only on DVCC (≈ 14 mA); drops recomputed (≈ 15–25 mV) | Closed |
| A-H2 | Offset used OPA1678 typical as maximum | Confirmed (±0.5 mV typ / ±2 mV max); I/V op-amp now OPA2197 (±0.25 mV max); budget 7.9 / 15.9 mV worst, balanced limit ±20 mV, 100 % screening; DAC-mismatch assumption Open (measure) | Closed |
| A-H3 | Safe disconnected output not established | Full sequence and current paths; rails first on / last off; PGOOD hardware kill; back-powering bounded (≤ 1.7 mA per pin) and removed in 10 ms; energy ≤ 16.5 mJ before / ≤ 0.22 mJ after the trip; protection defined at the headphone terminals | Closed – design, verify |
| A-H4 | Negative-rail headroom | Rails ±3.78 V; both rails budgeted at 4.60 V and maximum load with a 3.0 Ω charge-pump resistance (margin 0.17 V) | Closed |
| A-H5 | 100 mA ceiling not enforced | Clean envelope (80 mA per leg, swing curve) separated from overload; TPS2553 enforces the analog supply current; PGOOD kill; firmware ceiling steps; overload dissipation bounded; overload tests replace the 30-min 32 Ω balanced test | Closed – design, verify |
| A-H6 | USB power states and descriptors | State-by-state budget; MCU-only before configuration (≤ 83 mA estimate); descriptors 0x80 / 0xFA; CC detection by ADC for High mode; suspend policy (2.5 mA Open) | Closed – design, verify (firmware gate) |
| A-M1 | Mixed purchasing lines and placeholder ranges | One part per line, every value specified, 48 purchasing lines by LCSC number | Closed |
| A-M2 | 13.8 NTD ceiling margin; one Basic→Extended flip fails it | Every class verified on the JLCPCB page except X202 (not shown; assumed Extended); Extended count conservative (19, incl. the through-hole jack, X202 and X203); still a planning estimate until the JLCPCB quote | Closed – verify |
| A-M3 | Gain tolerance defeats the minimum power targets | Per-unit calibration to 1.75 Vrms per leg; targets defined at the calibrated level | Closed |
| A-M4 | Two LDOs do not guarantee sequencing | Separate enables (LP5907 EN, ME6211 CE) in a firmware sequence; DVDD LDO fed from 3V3D so DVDD ≤ DVCC by construction; scope check at bring-up | Closed – design, verify |
| A-M5 | Clock adaptation needs an earlier gate | Three oscillators: X201 100 MHz for the DAC MCLK (asynchronous mode, no PLL in the MCLK path) and the CPLD system clock; X202 24.576 MHz and X203 22.5792 MHz, selected by their enables in front of the CPLD PLL ×2 (all 8 rates exact); two-stage proof of concept on development hardware before layout | Closed – design, verify |
| B-S1 | −4.5 V rail cannot regulate from USB | As A-H4 | Closed |
| B-S2 | 2.0 Vrms per leg does not fit the supply; +1 dB units clip | Calibrated 1.75 Vrms per leg; swing model from OPA1622 Fig. 38/39; pre-calibration trim −3.5 dB | Closed |
| B-S3 | Power targets fail on low-spread units; rating method | As A-M3: “1 % THD+N or calibrated 0 dBFS, whichever first” | Closed |
| B-S4 | Firmware needs much more than the spec said | Firmware scope listed module by module for our own UAC2 firmware and CPLD logic (Section 5); two-stage proof of concept before layout; 352.8 kHz comes from X203 through the same CPLD PLL | Closed – verify |
| B-S5 | 14 NTD margin will be used up (odd values, 1.5 nF C0G, crystal, AMS1117, through-hole jack fee) | Basic E24 values or Basic combinations (100 k + 47 k, 1 k ∥ 5.1 k, 15 k + 1 k, 2 × 330 pF); Basic 5032 crystal; AMS1117 removed; through-hole fee counted; no cheaper 4.4 mm jack exists at LCSC (no SMD option) | Closed – verify |
| B-M1 | Pop at unplug has no hardware control | PGOOD gates AMP_EN in hardware; VBUS watchdog; separate unplug pop target (≤ 20 mV) | Closed – design, verify |
| B-M2 | Fault latch re-trips on clear | Set path blanked ≈ 60 ms after PGOOD good, without clearing the latch | Closed |
| B-M3 | PLL jitter goes to the DAC in sync mode | Asynchronous mode, MCLK straight from the 100 MHz oscillator (no PLL in the DAC MCLK path); the CPLD PLL is only in the bit-clock path (both families), whose jitter the DAC’s DPLL rejects; J-test stays in the plan | Closed |
| B-M4 | > 100 mA before enumeration; phones limit OTG | As A-H6; Default mode ≤ 0.473 A covers phones | Closed |
| B-M5 | AMS1117-ADJ needs a tantalum-type capacitor and a minimum load | Fixed 1.3 V ME6216A13M3G (±1 %, 300 mA, ceramic-stable) | Closed |
| B-minor | Duplicate version rows in the Spec history; U301 “voltage-mode” text; HSE 8 MHz; VREF risk overstated | Spec history cleaned; U301 text fixed; HSE 8 MHz confirmed; VREF risk downgraded and quantified (1.17 mV) | Closed |
| C-B1 | USB power before configuration | As A-H6 | Closed – verify |
| C-B2 | Reference descriptors wrong (self-powered, 100 mA) | 0x80 / 0xFA in the firmware gate | Closed – verify |
| C-B3 | 352.8 kHz MCLK not supported by the reference CPLD | Confirmed in the source; replaced by our own CPLD logic: the family oscillator (X202 24.576 MHz or X203 22.5792 MHz) is selected before the PLL, as the reviewer suggested; all 8 rates exact, 352.8 kHz included | Closed – design, verify |
| C-B4 | Clock-family switching can glitch | MCLK never switches; the family changes by the oscillator enables while the output is stopped and the PLL is held in reset; no clock multiplexer in the CPLD fabric; scope check for runt pulses | Closed – design, verify |
| C-B5 | Host volume not implemented | UAC2 volume/mute with GET/SET CUR/RANGE and I²C mapping, tested on three OSes in the gate | Closed – verify |
| C-B6 | “100 mA design current” not implemented | As A-H5 | Closed – design, verify |
| C-H1 | 500 mA budget has no margin | Default mode ≤ 0.473 A (27 mA below 0.5 A) enforced; High mode only with a ≥ 1.5 A Type-C advertisement | Closed |
| C-H2 | +4.5 V corner | Rails ±3.78 V, both rails budgeted | Closed |
| C-H3 | 7.7 µF table is not an inrush proof | Attach charge 38 µC including the 3V3M output capacitance; staggered, soft-started domains | Closed – verify |
| C-H4 | Cost provisional; “Assign LCSC no.” left; “every part is LCSC” inaccurate | Every line has an LCSC number and a verified class (X202: assumed Extended until the JLCPCB quote) | Closed |
| C-H5 | Offset margin | As A-H2 | Closed |
| C-H6 | ≤ 5 mV pop ambitious without relays | Split targets; full sequence incl. unplug/brownout validated on the bench | Closed – verify |
| C-H7 | USB identity (WCH VID, PID 0) | pid.codes VID 0x1209: test PID 0x0001 during development, a real PID before publishing; UID-based serial | Closed – verify |

## 7.7 Review disposition, earlier rounds (R1–R3)

Findings of the first three review rounds as closed in Notes v0.3, kept as history; values in this table are as closed at that time. R1-B1 and R2-M5 are marked as the design at that time and give the current values.

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| R1-B1 | VBUS_PD ≈ 130 µF vs USB-C 10 µF | No PD port; single 5 V port, VBUS budget 7.7 µF (design at that time; current: 5 × 1 µF on VBUS, 2.4) | Closed – N/A |
| R1-B2 | LMR33630 + SMAJ18A surge stacking ≈ 44 V | No inverting converter or 15 V input; LM27762 runs from 5 V | Closed – N/A |
| R1-B3 | LP5907 on 1V8_D over 10 µF | No 1.8 V rails; the only LP5907 (DAC 3.3 V) is budgeted ≤ 10 µF | Closed – N/A |
| R1-B4 | Paralleling open | No paralleling; one amplifier per leg (100 mA design current) | Closed |
| R1-B5 | LM339 at −12 V pulling 1.8 V logic | Comparator on ±4.5 V drives a PNP into a 3.3 V latch; 3.3 V nodes only see transistor/N-FET pull-downs, never below 0 V | Closed |
| R1-B6 | Offset ±5 mV vs 11 mV | Worst case recomputed (current mode: 7.4 / 14.7 mV); targets ±10 / ±15 mV | Closed |
| R1-H | AMS1117 dropout/noise | ME6211 (digital), LP5907 (DAC analog) | Closed |
| R1-H | XU316 VOH vs AK4493 VIH | All logic 3.3 V | Closed – N/A |
| R1-H | Both jacks live / gain-switch pop | Gain switch removed; both jacks live documented (one headphone at a time) | Closed |
| R1-H | OPA1622 stock 197 vs 400 | Sample run needs 10 (2 per board) | Closed |
| R1-H | LM317/LM337 cost trade | LM27762 low-noise LDOs | Closed – N/A |
| R1-H | CI “every part has an LCSC number” vs Digi-Key/Mouser parts | Every part is now an LCSC/JLCPCB part; the rule stands as written | Closed |
| R1-H | Firmware port map before layout | USBAudioToI2S build on the reference pin map before layout (open item 1) | Closed as a gate |
| R1 | Consistency: v0.2 wording, cost source of truth, THD+N conditions, Zout incl. contacts, DSD pins, grounding/ESD/CC/silkscreen | Fixed in Spec v0.4: single USB-C port removes the silkscreen confusion; CC = 5.1 kΩ Rd; jack ESD PESD15VL1BA; grounding rules added; DSD not in v1 | Closed |
| R2-S1 | TVS does not protect −14.7 V converter; no OVP | Block removed | Closed – N/A |
| R2-S2 | VBUS bulk before contract; analog rails start at 5 V | Block removed | Closed – N/A |
| R2-S3 | Balanced Zout with option B | Legs driven directly (< 0.1 Ω + contacts) | Closed |
| R2-S4 | K1000 1 W no margin | Requirement change (budget redesign); ≈ 0.13 W, 95 dB SPL documented | Closed |
| R2-S5 | Offset target contradicts design | As R1-B6 | Closed |
| R2-M1 | Board ≈ 7 W, PLA case | ≤ ≈ 2.5 W board, ≤ 0.54 W per OPA1622 | Closed |
| R2-M2 | LM317 near dropout; LM337 LC filter | Block removed | Closed – N/A |
| R2-M3 | ±0.05 dB trim cannot use 0.5 dB DAC steps | Optional digital scaling in the MCU firmware | Closed |
| R2-M4 | Max power criterion | Rated at 1 kHz, 1 % THD+N, both channels | Closed |
| R2-M5 | Cost margin 10 NTD / exchange rate | Gate uses fixed 31.7 NTD/US$: target 1,500 NTD per board (WARN above), approved ceiling 1,550 (FAIL above); current 1,536 → WARNING (design at that time; current: 1,650 NTD per board, ceiling 1,700 NTD, Section 8) | Closed |
| R2-M6 | 4.4-to-3.5 mm adapter shorts leg− | One amplifier per package shorted (TI continuous-short rating), ≤ 0.6 W; risk + manual warning | Closed |
| R2-M7 | Unbalanced crosstalk needs ≤ 6 mΩ ground | Layout rule (≤ 5.7 mΩ) | Closed |
| R2 | Minor: stale versions; KC2520Z “(assumed)”; through-hole count; pop requirement; CH224A/AMS1117 unverified claims; NVIDIA paragraph | Versions fixed; oscillators counted leadless conservatively; only the 4.4 mm jack is through-hole (confirm USB-C shell pegs); pop ≤ 5 mV; CH224A removed; AMS1117-3.3 replaced by ME6211C33 (digital 3V3) and LP5907 (DAC analog); an AMS1117-ADJ is used only for the 1.30 V DAC core supply; job-application text moved to the repository README | Closed |
| R3-1 | DC fault latch and recovery | Hardware 3.3 V latch, firmware recovery policy (Section 5, item 6), status to MCU | Closed |
| R3-2 | TVS/converter envelope | Block removed | Closed – N/A |
| R3-3 | Stage-1 gain 0.9009 with the input network | No stage 1; the gain now includes the DAC’s 806 Ω source (current mode, gain = 2 × Rf/806 per leg) | Closed |
| R3-4 | QA403 cannot drive 1.8 V I²S | I²S is now 3.3 V; header + firmware test mode + sequencing specified | Closed |
| R3-5 | 200 mA “limit” not implemented; thermal at 16 Ω balanced | 100 mA/leg design current, rated power at 1 % THD+N, full load matrix; 0.54 W worst per package | Closed |
| R3 | Consistency: rails-good qualification; offset/trim; workbook external links; cost/sourcing reconciliation | LM27762 PG + latch + 50 ms; offset/trim closed; the delivered v0.1 workbook references only its own sheets (links appear when sheets are copied into another workbook; v0.2 adds a self-check); single source of truth, all LCSC | Closed |

# 8. Chosen parts

Parts for the 5-board sample run, one line per LCSC part number (48 purchasing lines); Qty is per board. Unit prices are the tier that matches the run quantity (LCSC prices refreshed on 24 Sep 2026; 1+ tier for one-per-board ICs, 10+ for two-per-board parts); the OPA2210 keeps the JLCPCB part-page price of Sep 2025 (1+ US$1.2180, 10+ US$1.0035, 100+ US$0.7530; LCSC lists US$2.67 at 10+), whose stock is not confirmed now (7.1), so the price basis is mixed (R7-E BM-7). Parts List v0.6 (amended 24 Sep 2026) is the source of truth for cost; the figures below are copied from it. Every line has an LCSC number, and every line’s JLCPCB class is taken from its JLCPCB part page or the basic-parts list (Jun–Sep 2026), except X202, whose class is not shown (assumed Extended). The DNF parts (C101, D102, U205, R215, R216, R217, R703, J703) are not in the purchasing lines; the DAC-pin clamps D405–D408 are fitted (owner decision, 24 Sep 2026) and are line 30.

| # | Part | MPN | LCSC no. | JLCPCB class | Qty | Unit US$ | Refs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Jack, 4.4 mm balanced | G-Switch GT-3321667P-01 | C2762984 | Extended | 1 | 6.8656 | J701 |
| 2 | Op-amp, output legs, VSON-10 3 × 3 mm | TI OPA1622IDRCR | C2876559 | Extended | 2 | 1.7871 | U401, U402 |
| 3 | MCU, USB HS, LQFP-64 10 × 10 mm | WCH CH32V307RCT6 | C2988083 | Extended | 1 | 3.47 | U201 |
| 4 | DAC, QFN-28 5 × 5 mm | ESS ES9018K2M | C2688377 | Extended | 1 | 3.4678 | U301 |
| 5 | Op-amp, I/V converters, SOIC-8 | TI OPA2210IDR | C1849415 | Extended | 2 | 1.0035 | U403, U404 |
| 6 | CPLD, QFN-32 4 × 4 mm | AGM AGRV2KQ32 | C18208899 | Extended | 1 | 1.6503 | U202 |
| 7 | Charge pump + LDOs, ±3.8 V, WSON-12 2 × 3 mm | TI LM27762DSSR | C473398 | Extended | 1 | 1.0174 | U501 |
| 8 | Oscillator, 80 MHz | SCTF SX3M80.000B20F30TNN | C5179978 | Extended | 1 | 0.8916 | X201 |
| 9 | Capacitor, 10 µF, 0805, 25 V X5R | Samsung CL21A106KAYNNNE | C15850 | Basic | 9 | 0.085 | C413–C416, C512, C507, C509, C510, C511 |
| 10 | Current-limited switch, SOT-23-6 | TI TPS2553DBVR | C55266 | Extended | 1 | 0.5966 | U503 |
| 11 | Oscillator, 22.5792 MHz | SCTF SX2M22.5792M20F30TNN | C5452663 | Extended | 1 | 0.5455 | X203 |
| 12 | Resistor, 2.00 kΩ, 0603, 0.1 %, 25 ppm thin film | YAGEO RT0603BRD072KL | C728588 | Extended | 16 | 0.034 | R401–R416 |
| 13 | Oscillator, 24.576 MHz | YXC OT322524.576MJBA4SL | C2831388 | Extended | 1 | 0.5075 | X202 |
| 14 | Comparator, quad, open collector, SOIC-14 | TI LM339DR | C7948 | Extended | 2 | 0.1666 | U601, U602 |
| 15 | Jack, 3.5 mm | HRO PJ-332A-6A | C2848643 | Extended | 1 | 0.2921 | J702 |
| 16 | Crystal, 8 MHz | YXC X50328MSB2GI | C115962 | Basic | 1 | 0.2452 | Y201 |
| 17 | N-MOSFET, 2N7002, SOT-23 | JSCJ 2N7002 | C8545 | Basic | 12 | 0.0177 | Q501, Q601–Q611 |
| 18 | LDO 3.3 V (3V3A), SOT-23-5 | TI LP5907MFX-3.3/NOPB | C80670 | Extended | 1 | 0.2104 | U302 |
| 19 | USB ESD array, SOT-23-6 | ST USBLC6-2SC6 | C7519 | Extended | 1 | 0.1844 | U101 |
| 20 | Capacitor, 100 nF, 0402, 16 V X7R | Samsung CL05B104KO5NNNC | C1525 | Basic | 36 | 0.0045 | C203–C210, C212, C213–C216, C217–C219, C305–C309, C423–C426, C409–C412, C612–C615, C605, C606, C616 |
| 21 | LDO 3.3 V (3V3M, 3V3D), SOT-23-5 | Microne ME6211C33M5G-N | C82942 | Extended | 2 | 0.0603 | U502, U504 |
| 22 | Capacitor, 330 pF, 0603, 50 V C0G | Samsung CL10C331JB8NNNC | C1664 | Basic | 16 | 0.0068 | C417–C420, C427–C430, C401–C408 |
| 23 | Capacitor, 1 µF, 0603, 50 V X5R | Samsung CL10A105KB8NNNC | C15849 | Basic | 18 | 0.006 | C102, C221, C220, C301, C302, C311, C312, C501, C502, C503, C504, C505, C506, C508, C601–C604 |
| 24 | Resistor, 100 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1003TCE | C25741 | Basic | 23 | 0.0033 | R105, R303, R305, R421, R508, R509, R510, R511, R512, R513, R501, R515, R502, R503, R517, R504, R506, R601–R604, R613, R616 |
| 25 | USB-C receptacle | SHOU HAN TYPE-C 16PIN 2MD(073) | C2765186 | Extended | 1 | 0.074 | J101 |
| 26 | Diode, 5 V, SOD-523 | GOODWORK LESD5D5.0CT1G | C41399463 | Extended | 4 | 0.0155 | D701–D704 |
| 27 | LDO 1.3 V (DVDD), SOT23-3 | Microne ME6216A13M3G | C236662 | Extended | 1 | 0.06 | U303 |
| 28 | Resistor, 10 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1002TCE | C25744 | Basic | 23 | 0.002 | R103, R104, R201, R202, R214, R307, R306, R521, R523, R505, R522, R507, R637, R609, R610, R611, R628–R631, R639, R640, R618 |
| 29 | Capacitor, 2.2 µF, 0402, 6.3 V X5R | Samsung CL05A225MQ5NSNC | C12530 | Basic | 8 | 0.005 | C211, C303, C304, C310, C608–C611 |
| 30 | Diode, BAT54S, SOT-23 | JSMSEMI BAT54S | C916425 | Extended | 4 | 0.01 | D405–D408 |
| 31 | Diode, 1N4148WS, SOD-323 | JSCJ 1N4148WS | C2128 | Basic | 2 | 0.016 | D605, D606 |
| 32 | Ferrite bead, 600 Ω @ 100 MHz, 0603 | Sunlord GZ1608D601TF | C1002 | Basic | 2 | 0.0127 | FB202, FB301 |
| 33 | Resistor, 1 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1001TCE | C11702 | Basic | 9 | 0.002 | R308, R423–R426, R433, R525, R638, R701 |
| 34 | LED, Green, 0805 | Hubei KENTO KT-0805G | C2297 | Basic | 1 | 0.0163 | D705 |
| 35 | Resistor, 33 Ω, 0402, 1 % | UNI-ROYAL 0402WGF330JTCE | C25105 | Basic | 8 | 0.002 | R203–R206, R210, R211, R224, R225 |
| 36 | Resistor, 15 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1502TCE | C25756 | Basic | 8 | 0.002 | R431, R432, R516, R636, R624–R627 |
| 37 | Ferrite bead, 120 Ω @ 100 MHz, 0603 | Murata BLM18PG121SN1D | C14709 | Basic | 1 | 0.0157 | FB501 |
| 38 | Resistor, 5.1 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF5101TCE | C25905 | Basic | 7 | 0.002 | R101, R102, R434–R437, R518 |
| 39 | Resistor, 47 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF4702TCE | C25792 | Basic | 7 | 0.002 | R106, R514, R607, R608, R612, R614, R615 |
| 40 | Resistor, 2.2 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF2201TCE | C25879 | Basic | 7 | 0.002 | R212, R213, R218, R207, R208, R524, R702 |
| 41 | Resistor, 2.2 Ω, 0805, 1 % | UNI-ROYAL 0805W8F220KT5E | C17521 | Basic | 4 | 0.003 | R301, R302, R519, R520 |
| 42 | Resistor, 0 Ω, 0402, jumper | UNI-ROYAL 0402WGF0000TCE | C17168 | Basic | 5 | 0.002 | R107, R417–R420 |
| 43 | Resistor, 10 Ω, 0603, 1 % | UNI-ROYAL 0603WAF100JT5E | C22859 | Basic | 4 | 0.002 | R427–R430 |
| 44 | Resistor, 22 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF2202TCE | C25768 | Basic | 4 | 0.002 | R632–R635 |
| 45 | LED, Red, 0603 | Hubei KENTO KT-0603R | C2286 | Basic | 1 | 0.0075 | D706 |
| 46 | Capacitor, 33 pF, 0402, 50 V C0G | FH 0402CG330J500NT | C1562 | Basic | 3 | 0.002 | C201, C202, C421 |
| 47 | Resistor, 1 MΩ, 0402, 1 % | UNI-ROYAL 0402WGF1004TCE | C26083 | Basic | 2 | 0.002 | R617, R623 |
| 48 | Capacitor, 10 nF, 0402, 50 V X7R | Samsung CL05B103KB5NNNC | C15195 | Basic | 1 | 0.002 | C607 |

Cost at this revision (fixed 31.7 NTD per US$, JLCPCB Economic PCBA, LCSC prices of 24 Sep 2026): parts US$143.27 (4,542 NTD), five bare 4-layer PCBs US$10 (estimate), fees US$117.83 → US$271.11 = 8,594 NTD = 1,719 NTD per board. Fees: setup 8.18; stencil 1.53; 21 Extended part types × 3.07 = 64.47 (includes the through-hole 4.4 mm jack, X202, the LM339DR and the BAT54S, conservatively); SMT joints 781 per board × 0.0016 × 5 = 6.25; X-ray 40 leadless parts (8 per board) × 0.82 = 32.80; through-hole 4.15; Confirm Parts Placement 0.45.

Cost gate: target 1,500 NTD per board (WARN above) and ceiling 1,700 NTD per board plus the owner-approved 21 NTD for the DAC-pin clamps (24 Sep 2026) → order gate 1,721 NTD per board, the hard gate (FAIL above) → status WARNING, 219 NTD over the target and 2 NTD under the order gate. The owner decided on 24 Sep 2026 to fit the clamps D405–D408 knowing they add ≈ 21 NTD per board and take the estimate over the 1,700 NTD ceiling. The margin to the gate is very small, and the real JLCPCB quote decides. The gate covers every JLCPCB line of the real quote: PCBs, PCBA fees (including any fixture or other assembly line that appears in the quote) and parts; shipping, import tax, programmers, enclosure and proof-of-concept hardware are outside it. When the quote is approved, both the gated subtotal and the total payable (gated subtotal + shipping + tax from the quote) are recorded. The sentence “A PCB assembly fixture is needed to protect and support this part during the assembly process” appears on every JLCPCB part page, including unrelated parts (checked on the OPA2209AIDR page) and an empty part-page template, so it is standard page text and not a part-specific requirement for X202 or the OPA2210; no fixture fee is expected for Economic PCBA, and any fixture line in the real quote counts toward the gate. If JLCPCB does not charge the Extended fee for the through-hole jack and does not X-ray the three oscillators, the run is ≈ 1,621 NTD per board; JLCPCB-vs-LCSC price differences are ≈ −24 NTD per board (indicative); the LM339BIDR instead of the LM339DR gives ≈ 1,715 NTD per board. The figure is a planning estimate; the gate becomes final with the JLCPCB quote after layout.

Pricing rule: US$0.10 minimum per purchasing line for parts under US$0.05 (attrition and minimum-quantity allowance). OPA2210 (R7-E BM-1): the JLCPCB stock figure is 12 months old and not confirmed; check it in the JLCPCB Parts Manager now and, if ≥ 12 are available, buy 12 (10 + 2 spares) into the private library. Fallback designed in: OPA1612AIDR (C94590, Extended, same SOIC-8 pinout; LCSC 10+ US$1.0942, stock 9,754; JLCPCB part page of 28 Aug 2026 US$1.33, tier not shown, 31,254 available), which fits because V+ is now +3.90 V. With the fallback the run is ≈ 1,725 NTD per board at the LCSC price and up to ≈ 1,740 NTD at the JLCPCB part-page price, i.e. above the 1,721 NTD order gate, so it is an owner decision only if the OPA2210 cannot be bought; offset 9.7 / 19.4 mV (at the edge), lower noise (1.1 nV/√Hz) and +16 mA analog quiescent (≈ 72 mA), so the Default-mode targets at 3.5 mm 32 Ω and 4.4 mm 60 Ω and 120 Ω would not be met (High mode unaffected, 3.5.5). Buying the OPA2210 at the LCSC price (US$2.67) gives ≈ 1,825 NTD per board, also above the gate. Estimate history: 1,540 (one oscillator) → 1,602 (+24.576 MHz) → 1,666 (+22.5792 MHz) → 1,647 (80 MHz MCLK, OPA2210 at the JLCPCB price, protection parts) → 1,650 (protection and power fixes) → 1,691 (seventh round: prices refreshed to LCSC on 24 Sep 2026, LM393B, more joints and parts) → 1,697 NTD per board (amendment for R7-F/G/H: two LM339DR window comparators, the redesigned DC-sense network with the new 22 kΩ line and the 3V3A monitor, 14 more SMT joints) → 1,719 NTD per board (owner decision of 24 Sep 2026: DAC-pin clamps D405–D408 fitted, one more Extended type, 4 more placed parts and 12 more SMT joints).

By block (NTD per board): USB-C input and sensing 9; USB bridge 236; DAC and supplies 121; I/V and output legs 212; power 73; protection 23; outputs, indicators and board 230 (4.4 mm jack 218). Counts: 154 BOM lines (143 fitted; DNF: C101, D102, U205, R215, R216/R217, R703, J703 and the no-part pad lines), 48 purchasing lines, 267 placed parts per board, 21 Extended + 27 Basic part types, 781 SMT + 7 through-hole joints and 8 leadless parts per board; 100-unit reference ≈ 809 NTD per board. Excluded from the ceiling: shipping, import tax, programmers (J-Link or AGM DAP-Link, WCH-LinkE), enclosure and proof-of-concept hardware (Stage 2 ≈ US$93 ≈ 2,940 NTD for two boards).

# Appendix A. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Initial issue: datasheet-derived requirements and starting values for schematic capture; companion to Design Specification v0.2 |
| v0.2 | 23 Sep 2026 | Diego Yang | Remaining parts chosen; CH224A replaces CH224Q; TPS7A4901 replaces LT3045; protection circuit defined (LM339 + 2 × HFD4 relays); open items updated |
| v0.3 | 23 Sep 2026 | Diego Yang | Cost-driven redesign after three design reviews, companion to Design Specification v0.4: CH32V307 + AGRV2KQ32 USB bridge, ES9018K2M DAC, 2 × OPA1622 legs, LM27762 ±4.5 V, relay-less 3.3 V latch protection |
| v0.3 (revised) | 24 Sep 2026 | Diego Yang | Current-mode DAC outputs with OPA1678 I/V converters (option B); estimates, checklist, open items and cost updated |
| v0.4 | 24 Sep 2026 | Diego Yang | Fourth review round (R4-A, R4-B, R4-C), companion to Design Specification v0.5: ±3.78 V rails; calibrated 1.75 Vrms per leg; TPS2553 current limit with Default and High modes; switched USB power domains with CC sensing; OPA2197 I/V; single 100 MHz clock with the DAC in asynchronous mode; completed protection with an all-2N7002 latch; corrected ferrite; fixed 1.3 V LDO; ±10 / ±20 mV offset with screening; one-part-per-line BOM; firmware gate |
| v0.4 amendment | 24 Sep 2026 | Diego Yang | Owner decisions of 24 Sep 2026: family oscillators for both rate families, first X202 (24.576 MHz, 48 kHz family) and then X203 (22.5792 MHz, 44.1 kHz family), selected by their enables in front of the CPLD PLL ×2 so that all 8 rates have exact 50:50 bit clocks, because one 100 MHz clock gave a 40:60 bit-clock duty cycle at 352.8/384 kHz against the ES9018K2M 45:55 requirement; own USB firmware and CPLD logic instead of modifying USBAudioToI2S (kept as a known-good comparison). Currents, power-up sequence, block notes, firmware plan, checklist, open items, review rows, both figures, parts table (48 lines) and cost (1,666 NTD per board, ceiling 1,700 NTD) updated |
| v0.5 | 24 Sep 2026 | Diego Yang | Fifth review round (R5-A, R5-B, R5-C), companion to Design Specification v0.6 and Parts List v0.4: 80 MHz MCLK (SCTF SX3M80, below the DAC limit); break-before-make family oscillators with 2.2 kΩ enable pull-downs, node pull-down R214 and a DNF mux option (U205, R215); OPA2210 I/V converters; overload latch (Q610, D606) with firmware clear and new blanking bounds; rails-first-off power-down with the VREF clamp D401 and C421 100 nF; VBUS watchdog 4.10 V; FB− divider with R518 5.1 kΩ and rails incl. the resistor tolerance; Default-mode power by instantaneous peak current; protection envelope and terminal energy recalculated; correlated balanced noise; Stage 2 mandatory as the layout gate; MCU link captured in its own clock domain; 29 test pads, J703 MCLK monitor via R703 and DMM offset screening; checklist, open items, fifth-round disposition, both figures, parts table and cost (1,647 NTD per board, ceiling 1,700 NTD) updated |
| v0.6 | 24 Sep 2026 | Diego Yang | Sixth review round (R6-A, R6-B, R6-C, R6-D), companion to Design Specification v0.7 and Parts List v0.5: fault latch with C605 moved to node F (power-on set, ≥ 1.5 ms set-request stretch), R614/R615 47 kΩ and ≈ 35 µs capture; Q609 slowed to a static interlock (R623, C607); new Q611 kills AVCC_EN; worst-case detector with R601–R604 100 kΩ (R619–R622 removed) and R609/R610 10 kΩ, removal envelope and terminal energy recalculated at the worst case; slow firmware DC tier (R624–R635, C608–C611, four ADC pins) that sets the latch through ANA_FAULT; cause capture and a recovery policy that counts overloads only at the −12 dB ceiling; VREF tracks AVCC (C421 330 pF; D401 and C422 removed) with the DAC first off and the rails last and the legs enabled while VREF = 0; USB suspend a v1 requirement (≤ 2.5 mA within 10 ms, power-down ≤ 4 ms); LDO dissipation table with the R519/R520 dropper and C512; both DAC centre voltages with the R433 option; OPA2210 output swing; DNF R216/R217; SPI link with CS framing; SDATA/LRCLK on the falling BCLK edge; 31 test pads; Stage 2 with the real regulators (≈ 2,860 NTD); checklist, open items, sixth-round disposition, Figure 2, parts table (47 lines) and cost (1,650 NTD per board, ceiling 1,700 NTD) updated |
| v0.7 | 24 Sep 2026 | Diego Yang | Seventh review round, a full design check in five areas before any hardware is bought (R7-A power, R7-B analog, R7-C protection, R7-D digital, R7-E parts and cost), companion to Design Specification v0.8 and Parts List v0.6: series resistors on every enable the latch can kill (R307 on AVCC_EN, R521 on AUD_EN) and R522 into the MCU; PGOOD pull-up R505 10 kΩ to 5V_ANA with R623 1 MΩ and C607 10 nF, so the ≈ 35 µs capture is genuine; LM393BIDR comparator with re-set thresholds (R636/R637), C612/C613 and R638 (leg trip 0.36–0.80 V); mandatory firmware start-up, FLT-interrupt and pin-configuration rules; V+ raised to +3.90 V (R523) so the OPA1612 fallback fits; OPA1622 clean swing with 1.3 V headroom, calibrated level 1.55 Vrms per leg, −4.5 dB pre-calibration default, 70 mA envelope at 4.4 mm 32 Ω and new power targets; DAC reset state modelled, MCLK_EN (R218) and a power-down that stops MCLK and keeps RESETB high until AVCC has gone; CPLD on 2-wire Compact-JTAG (J202), family clock on PIN_1 with no 80 MHz input, FB202/C220 and a programming mode (TP732); proposed MCU and CPLD pin maps; bleeders R524/R525, DNF D102, R107 0 Ω and C101 DNF; suspend budget 2.31 mA with the suspend level ≈ 9.3 ms after the last bus activity; owner decisions of 24 Sep 2026 (residual DC below 0.2 V per leg accepted; Stage 1 optional, the JLCPCB-built Stage 2 board with the DAC as the first hardware); jack pin numbers to be confirmed from samples; checklist, open items, seventh-round disposition, both figures, parts table (47 lines) and cost (1,691 NTD per board, ceiling 1,700 NTD) updated |
| v0.7 (amended, 24 Sep 2026) | 24 Sep 2026 | Diego Yang | Amendment for three external reviews of Spec v0.7, Notes v0.6 and Parts List v0.5 (R7-F, R7-G, R7-H); same file and version number, companion to Design Specification v0.8 (amended) and Parts List v0.6 (amended): hardware DC detector changed to two TI LM339DR quad comparators with one window per leg directly on its filter node (BAV99 diode-OR D601–D604 and R605/R606 removed; R607/R608 47 kΩ, R636 15 kΩ, R637 10 kΩ; C612–C615; leg trip 0.43–0.67 V; rail-stuck energy ≤ 21.4 / 11.0 mJ); slow DC-sense network redesigned (15 kΩ / 10 kΩ to 3V3A / 22 kΩ + 2.2 µF, node 0.29–2.85 V, 3V3A monitor R639/R640/C616 on PC4) with the full error budget, auto-zero, dithered sampling and ratiometric correction; four firmware DC tiers per leg and per balanced pair with the accepted energy envelope; DAC pins in a failed-output shutdown stated as a residual risk, C421 330 pF → 33 pF, DNF BAT54S clamp footprints D405–D408 and DAC-pin probing in the tests; adaptive AMP_EN at power-down, LEDs off in suspend, resume ≈ 1.0 s, order A′, stream DC guard and an ambiguous fault class; RAILS_OK_N load rule and open-drain PGOOD pulse test; external-review disposition (7.3; older rounds renumbered 7.4–7.7) and updates to R7-C PR-3, PR-11, PR-12, PR-15 and R7-B AN-10; checklist, open items, both figures, parts table (47 lines) and cost (1,697 NTD per board, 3 NTD under the 1,700 NTD ceiling) updated; DAC-pin clamps D405–D408 fitted (owner decision) |