# USB DAC + Balanced Headphone Amplifier — Schematic Design Notes v0.2

Companion to Design Specification v0.3: datasheet-derived requirements and starting component values for the KiCad schematic

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 23 Sep 2026 | v0.2 | Schematic design notes (companion to Design Specification v0.3) |

# 0. Purpose and references

This document collects the datasheet-derived requirements and starting component values for drawing the KiCad schematic of the USB DAC + balanced headphone amplifier. It is a companion to Design Specification v0.3 and is organized by block: key decisions, the power tree, per-block notes, the XU316 signal plan, firmware requirements, a schematic review checklist, the items still to be verified and the chosen parts.

The designer owns and verifies every value. Circuit values are checked in SPICE with TI's OPA1622 and OPA1612 models, and pin-level requirements against the datasheets below (cited as R1 to R7). Figures quoted from a datasheet, or computed from datasheet values, are used exactly as stated; where a value is only a starting point it is marked “starting value — verify in SPICE/datasheet”.

| Ref | Document | Publisher | Document no. | Date |
| --- | --- | --- | --- | --- |
| R1 | XU316-1024-QF60A Datasheet | XMOS | XM014034B | 2023-08-29 |
| R2 | AK4493S Datasheet | AKM | 211100143-E-02 | 2022/03 |
| R3 | AKD4493S-A Evaluation Board Manual | AKM | KM136001 | 2025/08 |
| R4 | OPA1622 Datasheet | TI | SBOS727B | May 2016 |
| R5 | CH224Q/CH224A datasheet | WCH | — | — |
| R6 | LMR33630 datasheet | TI | — | — |
| R7 | Inverting buck-boost application note | TI | SNVA856 | — |

# 1. Key design decisions

The decisions below fix the architecture that the schematic implements. The Basis column records why each choice was made, so a later change can be checked against the original reason.

| Decision | Choice | Basis |
| --- | --- | --- |
| USB audio processor | XU316-1024-QF60A-C24; all I/O 1.8 V only (QF60A); core 0.9 V | Matches the supplied datasheet; 1.8 V logic everywhere |
| DAC digital supplies | AK4493S internal LDO off (LDOE = L); TVDD = AVDD = DVDD = 1.8 V | Exactly AKM's characterization condition (analog specifications are given at AVDD = TVDD = DVDD = 1.8 V, VDDL/R = VREFH = 5.0 V) and matches the XU316's 1.8 V I/O |
| DAC analog supplies | VDDL/R and VREFHL/R = 5.0 V (4.97 V) from a low-noise LDO (TI TPS7A4901) | Output full scale scales with VREFH − VREFL; each VREFH pin has its own 10 Ω + 470 µF (≈ 34 Hz) filter, so regulator noise is filtered before the reference; the LT3045 (≈ 200 NTD more) is kept as an A/B experiment |
| DAC output coupling | AC-couple each DAC output pin (as in AKM's Example Circuit 3) | Removes the ≈ 2.5 V common-mode DC so the DC-coupled amplifier stays near 0 V |
| Stage 1 | OPA1612 unity-gain difference amplifier + low-pass filter per channel | Cancels even-order and common-mode errors so the 3.5 mm output (leg+ only) is clean; AKM's evaluation board uses OPA1612 |
| Output stage | Leg+ non-inverting (1× / 2.74×); leg− is a unity inverter of leg+; each leg is one OPA1622 with both halves paralleled | One gain resistor per channel to switch; 200 mA peak per leg for K1000/Susvara |
| Analog supplies | ±12 V from 15 V USB PD: ST LM317 (+12 V); TI LMR33630 inverting buck-boost to −14.7 V, then onsemi LM337 (−12 V) | 7.75 V peak per leg with headroom (OPA1622 operates from ±2 V to ±18 V) |
| Clocks | Two Kyocera KC2520Z 1.8 V oscillators + MCLK net to XU316 pin X1D11 | X1D11 is the XU316 application-PLL output pin, so a PLL-generated MCLK can be tried without rework |
| Protection | One TI LM339 quad comparator + 2 × Hongfa HFD4/12-SR DPDT relays; a fault also pulls the OPA1622 enable low | All-SMD and in stock; one IC covers DC detection on four legs and PD-loss detection (3.5) |
| USB PD sink | WCH CH224A, 56 kΩ on CFG1 → 15 V | 20,632 in stock at LCSC; leaded ESSOP-10 (no X-ray fee); same resistor table as the CH224Q (3.6) |

# 2. Power tree and current budget

Two USB-C ports feed separate domains. The data port supplies the digital rails from VBUS_DATA (5 V); a USB PD sink on the power port supplies the analog rails from VBUS_PD (15 V). Currents are datasheet typical and maximum figures, or budget estimates where marked.

## 2.1 Regulation chains

| Domain | Input | Regulation chain | Rails |
| --- | --- | --- | --- |
| Digital | USB-C data port, VBUS_DATA 5 V | 5 V → TI TLV62569 buck (2 A, 2.2 µH) → 0V9; 5 V → AMS1117-3.3 → 3V3; 3V3 → TI LP5907-1.8 → 1V8_D; 3V3 → LP5907-1.8 (EN from 1V8_D) → 1V8_A | 0V9, 3V3, 1V8_D, 1V8_A |
| Analog | USB-C power port with CH224A, VBUS_PD 15 V | 15 V → ST LM317 → +12 V; +12 V → TI TPS7A4901 (4.97 V) → +5 V_A; 15 V → TI LMR33630 inverting buck-boost → −14.7 V → onsemi LM337 → −12 V | +12 V, +5 V_A, −14.7 V (intermediate), −12 V |

## 2.2 Rail budget

| Rail | From → regulator | Loads | Current | Notes |
| --- | --- | --- | --- | --- |
| VBUS_DATA 5 V | USB-C data port | Digital rails | ≈ 0.3 A worst case (estimate) | 1–10 µF on VBUS required by USB (2.2 µF typical); ESD array |
| 0V9 | 5 V → TLV62569DBVR buck (2 A, 1.5 MHz) with 2.2 µH SWPA3015S2R2MT; 0.6 V feedback, 100 kΩ / 200 kΩ → 0.900 V | XU316 VDD; PLL_AVDD via ferrite (600 Ω at 100 MHz, DCR < 1 Ω, e.g., BLM18KG601SN1D) + 1 µF | 225 mA typ, 830 mA max (C24) | 0.855–0.945 V; ≥ 8 × 100 nF + ≥ 10 µF bulk |
| 3V3 | 5 V → AMS1117-3.3 (SOT-223); use the output capacitor its datasheet requires | XU316 USB_VDD33 (0.8 mA HS, up to 25 mA FS transmit); feeds the 1.8 V LDOs | ≤ ≈ 185 mA | 3.0–3.6 V; ≈ 0.3 W LDO dissipation |
| 1V8_D | 3V3 → LP5907MFX-1.8 | XU316 VDDIOL/R/T/B18, USB_VDD18 (30 mA typ, 36 mA max HS), QSPI flash, AK4493 TVDD (1–1.5 mA), I²C pull-ups, debug VREF | ≈ 100 mA | All XU316 VDDIO on this one rail, so the on-chip power-on reset works (XMOS checklist G.3); 100 nF per VDDIO pin + ≥ 10 µF |
| 1V8_A (quiet) | 3V3 → LP5907MFX-1.8, EN from 1V8_D | AK4493 AVDD (1–1.5 mA) + DVDD (6/11/17 mA typ at 44.1/96/192 kHz, 26 mA max at 192 kHz) + two KC2520Z oscillators (7.3 mA max each, one enabled at a time) | ≤ ≈ 60 mA | AK4493 rule: TVDD must come up before or with AVDD and DVDD |
| VBUS_PD 15 V | USB-C power port, CH224A (56 kΩ on CFG1) | Analog rails | ≤ 0.71 A (10.7 W at 1.2 W into 60 Ω, both channels) | ≥ 30 W charger; SMAJ18A TVS + ≥ 100 µF bulk |
| +12 V | 15 V → ST LM317D2T-TR (D2PAK), 10 µF ADJ bypass | 4× OPA1622, OPA1612, relay coils, +5 V_A LDO | Up to ≈ 390 mA | ≈ 1.16 W at 388 mA, D2PAK + copper; headroom 3 V nominal, 2.25 V at 14.25 V input |
| −12 V | 15 V → TI LMR33630ADDAR inverting buck-boost to −14.7 V → onsemi LM337BD2TR4G (D2PAK) | 4× OPA1622, OPA1612 | Up to ≈ 280 mA | Converter IC sees Vin + ¦Vout¦ ≈ 30.4 V worst case (15.75 V in) vs its 36 V rating; LM337 headroom 2.66 V, 0.75 W; small hot loop, far from analog |
| +5 V_A | +12 V → TI TPS7A4901DGNR, 31.6 kΩ / 10 kΩ → 4.97 V (1.194 V reference; starting value — verify in datasheet) | AK4493 VDDL + VDDR (27 mA typ, 41 mA max) + VREFHL + VREFHR (1–1.5 mA) | ≈ 45 mA | VREFHL/R each via 10 Ω; VDDL/R routed separately from the regulator (AKM) |

## 2.3 Power sequencing

The sequencing rules from the datasheets are met by the rail structure and the hardware defaults, so the board powers up safely before firmware runs.

| # | Requirement | Implementation | Source |
| --- | --- | --- | --- |
| 1 | AK4493 TVDD must come up before or with AVDD and DVDD | TVDD is on 1V8_D; the 1V8_A low-noise LDO is enabled from 1V8_D | R2 |
| 2 | AK4493 PDN low while the supplies come up and for ≥ 150 ns after all are up | 100 kΩ pull-down on PDN; firmware releases PDN only after PD_GOOD_N and rail settling (Section 5) | R2 |
| 3 | No MCLK, BICK or LRCK until all AK4493 supplies are on | Oscillators off at power-up and enabled only after the rails settle; I²S starts after register configuration | R2 |
| 4 | VREFH must come up with or after VDDL/R | VREFHL/R fed from +5 V_A through 10 Ω into 470 µF | R2 |
| 5 | XU316 on-chip power-on reset | All VDDIO pins on the single 1V8_D rail (XMOS checklist G.3); no external reset circuit | R1 |
| 6 | QSPI flash ready within 300 µs | Select a flash that meets this; otherwise hold RST_N until the flash is ready | R1 |
| 7 | Headphone outputs isolated during power-up and on PD power loss | Relays close 2–3 s after all rails are good and the DAC is running; the LM339 opens them and pulls AMP_EN low on a DC fault or when VBUS_PD < 13.8 V (3.5) | Design |

# 3. Block notes

Pin numbers refer to the XU316-1024-QF60A and AK4493SEQ packages. Entries are datasheet requirements unless they are marked as starting values.

## 3.1 XU316-1024-QF60A (USB audio processor)

### 3.1.1 Requirements and starting values

| Topic | Requirement / value | Notes |
| --- | --- | --- |
| Crystal | 24 MHz crystal (USB needs 12 or 24 MHz): 12 pF load, ESR ≤ 60 Ω, Rf = 1 MΩ, Rd = 680 Ω, CL1 = CL2 = 22 pF | Chosen: YXC X322524MOB4SI (24 MHz, 12 pF, ±10 ppm, ESR 50 Ω; LCSC C70590) |
| Clock alternative | 1.8 V oscillator into XIN | The oscillator must be running before reset is released |
| Decoupling | VDD (0V9): ≥ 8 × 100 nF + ≥ 10 µF bulk; 100 nF per VDDIO pin + ≥ 10 µF on 1V8_D; PLL_AVDD via ferrite (600 Ω at 100 MHz, DCR < 1 Ω) + 1 µF | Place the PLL_AVDD ferrite and 1 µF close to the pin; rail details in 2.2 |
| Boot flash pins | QSPI on fixed pins: SS = X0D01; D0–D3 = X0D04–X0D07; SCLK = X0D10 | Six pins; the remaining port plan is in Section 4 |
| Boot flash part | Winbond W25Q32JWSSIQ TR: 1.7–1.95 V, 32 Mbit QSPI, SOIC-8 208 mil, leaded (LCSC C2456252) | The XMOS flash library verifies the W25Q16JV/32JV/64JV/128JV family and states that devices in the same family should be compatible; the XMOS tools set the Quad Enable bit on SFDP devices. Confirm in the datasheet that the flash accepts commands within 300 µs of power-up (Section 7) |
| Boot-mode pins | X0D04–X0D06 are sampled as boot-mode pins and have internal pull-downs; 000 = QSPI boot | Do not pull them up |
| USB data | USB_DP pin 29, USB_DM pin 28; 90 Ω differential; ≤ 1 mm skew | XMOS example geometry W 0.12 mm / S 0.10 mm over 0.10 mm dielectric; recompute for the JLCPCB stack-up. Keep ≥ 1.27 mm from other high-speed nets and ≥ 0.51 mm from low-speed nets |
| USB-C sink | 5.1 kΩ from CC1 to GND and from CC2 to GND | VBUS decoupling (1–10 µF): see 2.2. Connector: HRO TYPE-C-31-M-12 on both ports; ESD array ST USBLC6-2SC6 |
| VBUS sensing | Optional, because the design is bus-powered | If self-powered: 220 kΩ series, 100 kΩ to GND for 1.8 V I/O, 47 kΩ bleeder, 2.2 µF |
| Reset | RST_N has an internal pull-up; with a single 1.8 V VDDIO rail no external reset is required | Bring RST_N to the debug header |
| Debug | JTAG-only xSYS2 0.05" 10-pin header or TAG-Connect TC2030 footprint (pinouts in 3.1.2) | Needs a 1.8 V XTAG4 adapter |
| Layout and thermal | 16 vias (4 × 4) in the ground paddle; 4 vias under each of the four VDD paddles; decoupling around the device with short ground returns | MSL3 (168 h floor life) |
| Logic levels | XU316 VOH ≥ 1.35 V at 2 mA; AK4493 VIH ≥ 80% × TVDD = 1.44 V at 1.8 V | Works only because the AK4493 inputs draw ≤ 10 µA, so the XU316 outputs sit near 1.8 V. Keep TVDD on the same 1V8_D rail and flag this at review |

### 3.1.2 Debug connectors

| Pin | xSYS2 10-pin, 0.05" (JTAG only) | TAG-Connect TC2030 |
| --- | --- | --- |
| 1 | VDDIOB18 (with decoupler) | TCK |
| 2 | TMS | GND |
| 3 | GND | TMS |
| 4 | TCK | TDI |
| 5 | GND | VREF |
| 6 | TDO | TDO |
| 7 | GND | Not present (6-pin) |
| 8 | TDI | Not present (6-pin) |
| 9 | GND | Not present (6-pin) |
| 10 | RST_N | Not present (6-pin) |

## 3.2 Clocks

Two 1.8 V CMOS oscillators with a standby pin, one per sample-rate family, drive a single MCLK net. The XU316 enables one oscillator at a time, and MCLK feeds both the AK4493 and XU316 pin X1D11 so that a PLL-generated MCLK can be tried later without rework.

### 3.2.1 Clock requirements

| Item | Requirement / value | Notes |
| --- | --- | --- |
| Oscillators | Kyocera KC2520Z22.5792C1KX00 (22.5792 MHz) and KC2520Z24.5760C1KX00 (24.576 MHz): 1.71–3.63 V CMOS, ±20 ppm, 2.5 × 2.0 mm, standby pin (5 µA when disabled), 7.3 mA max | From Digi-Key through JLCPCB global sourcing (not stocked at LCSC). Both drive one MCLK net, so confirm the output is high-impedance in standby (Section 7). Second source: ECS-2520MVLC (1.6–3.6 V, tri-state enable) |
| Oscillator supply | 1V8_A | Quiet rail, shared with AK4493 AVDD and DVDD |
| Oscillator outputs | Each through a ≈ 22–33 Ω series resistor onto the one MCLK net; enabled one at a time by OSC_EN_44K1 and OSC_EN_48K | Series resistor: starting value — verify in SPICE/datasheet |
| MCLK to AK4493 | MCLK pin 46 via a damping resistor | AKM recommends a damping resistor; without it THD+N can degrade |
| MCLK to XU316 | X1D11 (port 1D, tile 1) | Application-PLL output pin: input in oscillator mode, output in PLL mode |
| AK4493 MCLK limits | 2.048–49.664 MHz; duty cycle 40–60%; pulse width ≥ 9.05 ns | Both oscillator frequencies are inside the range |
| Clock changes | MCLK may change only while the DAC is in reset (PDN = L or RSTN bit = 0) | Firmware sequence in Section 5 |
| Routing | Keep MCLK short and away from the VREF pins and the analog outputs | MCLK test point in 3.7 |

### 3.2.2 Sample-rate ratios

| Sample rate | MCLK | MCLK / fs | AK4493S |
| --- | --- | --- | --- |
| 384 kHz | 24.576 MHz | 64 fs | Allowed: the 216–388 kHz range allows 32/48/64/96 fs |
| 352.8 kHz | 22.5792 MHz | 64 fs | Allowed (same 216–388 kHz range) |
| 192 kHz | 24.576 MHz | 128 fs | Allowed |
| 96 kHz | 24.576 MHz | 256 fs | Allowed |
| 48 kHz | 24.576 MHz | 512 fs | Allowed |
| 44.1 kHz | 22.5792 MHz | 512 fs | Allowed |

## 3.3 DAC (AK4493SEQ)

The AK4493SEQ runs in register mode over I²C with its internal LDO disabled. All digital supplies are 1.8 V, which is AKM's characterization condition, and the analog supplies and references are 5.0 V from +5 V_A.

### 3.3.1 Pin connections

| Function | Pin (no.) | Connection | Requirement / note |
| --- | --- | --- | --- |
| Mode select | PSN (13), I2C (14), CSN (7) | PSN = L; I2C = H; CSN → DVSS | Register mode over I²C |
| I²C address | CAD0 (12), CAD1 (17) | Strap to set the I²C address | Record the chosen address for the firmware register map |
| I²C bus | SCL (8), SDA (9) | XU316 1-bit open-drain ports | 2.2 kΩ pull-ups to 1V8_D |
| LDO enable | LDOE (16) | DVSS | Internal LDO off (LDOE = L) |
| Digital I/O supply | TVDD (48) | 1V8_D | Must come up before or with AVDD and DVDD |
| Core supplies | AVDD (44), DVDD (1) | 1V8_A | DVDD 6/11/17 mA typ at 44.1/96/192 kHz, 26 mA max at 192 kHz |
| Analog supplies | VDDL (33, 34), VDDR (27, 28) | +5 V_A | Route VDDL and VDDR separately from the regulator (AKM) |
| Power-down | PDN (2) | XU316 GPIO with 100 kΩ pull-down | Low while the supplies come up and for ≥ 150 ns after all are up; no MCLK/BICK/LRCK until all supplies are on |
| Master clock | MCLK (46) | MCLK net via damping resistor | Limits and ratios in 3.2 |
| Audio interface | BICK (3), SDATA (4), LRCK (5) | XU316 I²S outputs BCLK, SDATA, LRCLK | Test points and I²S input header in 3.7 |
| Reference, high | VREFHL (41, 42), VREFHR (19, 20) | +5 V_A via 10 Ω each | ≈ 34 Hz low-pass with 470 µF; VREFH comes up with or after VDDL/R |
| Reference, low | VREFLL (39, 40), VREFLR (21, 22) | Analog ground (optional 10 Ω) | Output full scale scales with VREFH − VREFL |
| Common mode | VCML (38), VCMR (23) | 10 µF electrolytic each to VREFL | No other load |
| Grounds | AVSS, DVSS, VSSL, VSSR | One solid analog ground plane | THD+N degrades with impedance between VSS pins |
| Unused pins | NC (24, 37, 43); TESTE (18); WCK (6); DEM0 (15) | NC → analog ground; TESTE → DVSS or open; WCK and DEM0 → DVSS | WCK and DEM0 are not used in register-mode PCM |
| Zero detect | DZFL (10), DZFR (11) | Open or to GPIO | Optional DZF input on the XU316 (Section 4) |
| Analog outputs | AOUTLP (36), AOUTLN (35), AOUTRP (25), AOUTRN (26) | Analog chain (3.4) | Load ≥ 450 Ω and ≤ 25 pF per pin; internal current limit 85 mA per pin; filter resistors ≤ 1% (AKM), 0.1% here for common-mode rejection |

### 3.3.2 Decoupling (AKM typical connection, LDO disabled)

| Pin or pin pair | Decoupling | Note |
| --- | --- | --- |
| TVDD | 0.1 µF + 10 µF | Supplied from 1V8_D |
| AVDD | 0.1 µF + 10 µF | Supplied from 1V8_A |
| DVDD | 1 µF + 0.1 µF | Supplied from 1V8_A |
| VDDL, VDDR | Each 0.1 µF + 10 µF | Supplied from +5 V_A, routed separately |
| VREFHL–VREFLL, VREFHR–VREFLR | Each 0.1 µF ceramic at the pins + 470 µF electrolytic | With the 10 Ω feed this forms the ≈ 34 Hz low-pass |
| VCML (38), VCMR (23) | Each 10 µF electrolytic to VREFL | No other load on VCM |

## 3.4 Analog chain (per channel)

Each channel runs from the DAC pins through a passive interface and the OPA1612 difference amplifier (stage 1) to two OPA1622 legs, then through the relays to the jacks. Every component value in 3.4.1 is a starting value — verify in SPICE/datasheet; the results are computed from these values.

### 3.4.1 Starting values

| Stage | Values | Result |
| --- | --- | --- |
| DAC pin interface | 100 Ω series per pin → 2.2 nF C0G across P–N → 100 µF coupling capacitor per pin (audio/bipolar electrolytic) → 10 kΩ bias to ground → stage 1 | Passive pole ≈ 362 kHz; coupling time constant with the 10 kΩ bias ≈ 1 s |
| Stage 1 (one OPA1612 half per channel) | Unity-gain difference amplifier: four 1.00 kΩ 0.1% thin-film resistors; 820 pF C0G across the feedback resistor and across the ground-leg resistor | Active pole ≈ 194 kHz. Combined response −0.06 dB at 20 kHz, −0.23 dB at 40 kHz, −0.89 dB at 80 kHz. Output 2 Vrms at full scale |
| Leg+ (OPA1622 channel A, non-inverting) | Rf = 348 Ω, Rg = 200 Ω (0.1%); 22–47 pF C0G across Rf (verify stability) | 2.74× (+8.8 dB) high gain; the gain switch opens Rg for 1.00× low gain. Low Rf keeps the offset from the OPA1622 input bias current (1.2 µA typ, 2 µA max) at 0.42 mV typ |
| Leg− (OPA1622 channel A, inverting) | Rin = Rf = 1.00 kΩ 0.1%; non-inverting input to ground via 499 Ω | Unity inverter of leg+; the 499 Ω provides bias-current compensation |
| Paralleling, Option 1 (simulate first) | Channel A is the gain stage with feedback from the leg output node; channel B is a unity follower of channel A's output; each output reaches the node through 0.47 Ω 1% (≥ 0.25 W) | One gain network per leg; low output impedance |
| Paralleling, Option 2 (fallback) | Both channels as identical gain stages, each with feedback taken before its own 0.47 Ω ballast | Output impedance ≈ 0.24 Ω; needs a 4-pole gain switch |
| Option footprints | Arrange the footprints so that either option is a 0 Ω-link change | The choice follows the SPICE stability check (Section 7) |
| Gain switch | DPDT slide switch G-Switch MS-22D28-G020 (LCSC C963205), one pole per channel in the Rg path | No spare pole, so GAIN_STATE is dropped: change gain with the volume turned down (a 3- or 4-pole switch would restore GAIN_STATE) |
| Enable | Both halves of a package share EN; all four EN pins are driven from one XU316 GPIO with a 100 kΩ pull-down | EN threshold ≈ 0.82 V referenced to the OPA1622 GND pin; EN is never floating or high-impedance. In shutdown the output is not high-impedance, so the relays isolate the jacks |
| Thermal | OPA1622 thermal pad to V− copper with vias | Dissipation up to 1.05 W per chip (+50 °C at 47.6 °C/W) |
| Do-not-fit footprints | Output Zobel per leg (10 Ω + 100 nF); DC servo per channel | Not fitted by default; the footprints keep both options open |

### 3.4.2 Output connectors (after the relays)

| Signal | 4.4 mm 5-pole (G-Switch GT-3321667P-01, through-hole) | 3.5 mm (HRO PJ-332A-6A, TRRS jack used with TRS plugs) |
| --- | --- | --- |
| L+ | Tip | Tip |
| L− | Ring 1 | Not connected (leg+ only) |
| R+ | Ring 2 | Ring 1 |
| R− | Ring 3 | Not connected (leg+ only) |
| GND | Sleeve | Sleeve and ring 2 (tie both to GND) |

### 3.4.3 Computed estimates

| Parameter | Estimate | Basis |
| --- | --- | --- |
| Dynamic range | ≈ 121 dB(A) at both gains, both jacks | DAC 1.42 µV and stage 1 0.94 µV, A-weighted |
| DC offset, typical | 0.7 mV unbalanced / 1.5 mV balanced | Computed from the 3.4.1 values |
| DC offset, worst case | ≈ 5.0 mV unbalanced / 11.0 mV balanced | Computed; a DC servo footprint is provided (do not fit) |
| OPA1622 dissipation | Up to 1.05 W per chip | +50 °C at 47.6 °C/W; thermal pad to V− copper |
| Output drive per leg | 7.75 V peak, 200 mA peak | K1000/Susvara; the ±12 V rails give headroom |
| Stage 1 full-scale output | 2 Vrms | DAC full scale scales with VREFH − VREFL |

## 3.5 Protection

Two relays keep both jacks disconnected until the system is running and disconnect them again on a DC fault or loss of PD power. One LM339 quad comparator does the detection, and a fault also pulls the OPA1622 enable low, so the amplifiers shut down (pop-suppressed by design) within microseconds while the relays open. The XU316 can also force a mute.

| Function | Requirement / implementation |
| --- | --- |
| Relays | 2 × Hongfa HFD4/12-SR (DPDT, 12 V coil 1.028 kΩ = 11.7 mA, gold-plated AgNi, ≤ 100 mΩ, 3 ms operate/release; polarized coil — observe polarity). Relay 1 switches L+ and L−, relay 2 switches R+ and R−, ahead of both jacks. |
| Coil drive | Coils from +12 V; one MMBT3904 low-side switch for both coils (23.3 mA); base from XU316 RLY_EN through 2.2 kΩ (0.5 mA) with 100 kΩ to GND; 1N4148W across each coil. |
| Comparator | TI LM339DR on ±12 V (V+ = +12 V, GND pin = −12 V); inputs stay inside −12 V to +10.5 V; open-collector outputs wired together as FAULT_N. |
| DC detection | Each leg, taken before its relay → 100 kΩ + 1 µF X7R (τ = 100 ms, fc 1.6 Hz) → BAV99 into a MAX node and a MIN node (each 1 MΩ to GND). Comparator A trips when MAX > +0.6 V, comparator B when MIN < −0.6 V; with ≈ 0.4 V diode drop at microamp currents the trip point is ≈ ±1.0 V DC on any leg (starting values — verify in SPICE). |
| DC timing | Opens in ≈ 69 ms at 2 V DC, ≈ 110 ms at 1.5 V, ≈ 10 ms for a rail-stuck leg (11 V). A full-scale 20 Hz tone (7.75 V peak per leg, high gain) is 0.61 V peak after the filter, below the 1.0 V trip. |
| PD power loss | Comparator C compares VBUS_PD through 100 kΩ / 15 kΩ with 1V8_D and trips below 13.8 V (normal range 14.25–15.75 V); add ≈ 100 mV hysteresis. It acts before the rails collapse. |
| Fault action | FAULT_N pulls the relay-driver base node and the AMP_EN node low through 4.7 kΩ each (2.5 mA, overriding the 0.95 mA from the GPIO); a BAT54 Schottky from GND to each node holds it ≥ −0.3 V, because the LM339 outputs swing to −12 V. Amplifiers shut down in microseconds; relays open within ≈ 3 ms plus the flyback-diode delay (target ≤ 10 ms). |
| Spare | Comparator D spare (e.g., −12 V rail-good); tie its inputs to defined levels if unused. |
| Power-up | Relays open at power-up; firmware closes them 2–3 s after all rails are good and the DAC is running. |
| Firmware mute | RLY_EN low opens the relays; AMP_EN low shuts the amplifiers down. |
| Short circuit | The OPA1622 tolerates a continuous short on one amplifier per package; with paralleled halves, a sustained short relies on thermal shutdown (175 °C) |

## 3.6 Power input

| Item | Requirement |
| --- | --- |
| PD sink | WCH CH224A (ESSOP-10, leaded; LCSC C42459160) in single-resistor mode: 56 kΩ from CFG1 to GND requests 15 V (WCH table: 6.8 kΩ → 9 V, 24 kΩ → 12 V, 56 kΩ → 15 V, 120 kΩ → 20 V, 210 kΩ → 28 V); 1 µF on VHV; VBUS pin to VHV as in the WCH reference; CFG2/CFG3 unused (no I²C) |
| Charger | ≥ 30 W; worst-case draw ≤ 0.71 A (10.7 W at 1.2 W into 60 Ω, both channels) |
| PD_GOOD_N | CH224A PG (open-drain, 32 V tolerant) with 100 kΩ pull-up to 1V8_D → XU316 input; check the PG polarity in the WCH datasheet. The VBUS_PD divider now feeds the LM339 PD-loss comparator (3.5) |
| Input protection | Littelfuse SMAJ18A TVS (18 V standoff, 29.2 V clamp) + ≥ 100 µF, ≥ 35 V electrolytic on VBUS_PD to damp hot-plug ringing |
| −14.7 V converter | TI LMR33630ADDAR (3.8–36 V, 3 A synchronous, 400 kHz, HSOIC-8 leaded) as an inverting buck-boost: RFBT 100 kΩ / RFBB 7.32 kΩ → −14.66 V (1.0 V feedback reference; starting value — verify in datasheet); L = 22 µH Sunlord SWPA6045S220MT (Isat 2.2 A). At 0.285 A load: duty ≈ 0.49, average inductor current ≈ 0.56 A, ripple ≈ 0.84 A, peak ≈ 0.98 A. IC stress 15.75 + 14.66 ≈ 30.4 V vs 36 V; input capacitors ≥ 50 V; FB and EN/UVLO are referenced to the IC ground (−VOUT) — follow TI SNVA856 (R7); keep the hot loop small and far from the analog section. |

## 3.7 Test and DFT

| Item | Requirement |
| --- | --- |
| Rail test points | VBUS_DATA, 0V9, 3V3, 1V8_D, 1V8_A, VBUS_PD, +12 V, −14.7 V, −12 V, +5 V_A, VREFHL, VREFHR |
| Signal test points | MCLK, BCLK, LRCLK, SDATA and PDN; relay drive; each leg before the relay; FAULT_N, relay-driver base, AMP_EN |
| I²S input header | MCLK, BCLK, LRCLK, SDATA, GND, with 0 Ω links (or the XU316 held in reset) so a QA403 can drive the DAC directly |
| Assembly | Three fiducials; edge rails; board ≥ 70 × 70 mm for Standard PCBA |
| Bring-up tests | Hot-plug the charger (converter input stays below 30 V); unplug PD while playing (amplifiers off and relays open before ±12 V fall); DC-fault injection on each leg (trip time vs the targets in 3.5) |

# 4. XU316 signal plan

- About 15 GPIO are needed beyond the six QSPI pins, and 28 are available (34 − 6). Two port rules apply: never mix inputs and outputs on one multi-bit port, and treat X0D04–X0D07 as output-only (XMOS checklist G.7); here those pins carry the QSPI data lines, so no GPIO is mapped to them. Start from XMOS's xcore.ai USB Audio reference design port map and confirm that the map builds in the XMOS tools before layout.

| Signal | Dir | Port rule | Pin / notes |
| --- | --- | --- | --- |
| QSPI: SS, SCLK, D0–D3 (6 signals) | Output, I/O | Fixed pins | SS = X0D01; D0–D3 = X0D04–X0D07; SCLK = X0D10 |
| MCLK | Input (oscillator mode) / output (PLL mode) | X1D11 = port 1D, tile 1 | Application-PLL output pin |
| BCLK, LRCLK, SDATA | Output | 1-bit ports on the tile running I²S | To the AK4493 and the I²S test header |
| I²C SCL, SDA | Open-drain | 1-bit ports, open-drain | 2.2 kΩ pull-ups to 1V8_D |
| PDN, OSC_EN_44K1, OSC_EN_48K, AMP_EN, RLY_EN, 2 LEDs | Output | May share one multi-bit output port | PDN, AMP_EN and RLY_EN each have a 100 kΩ pull-down; AMP_EN and RLY_EN are also pulled low by FAULT_N |
| PD_GOOD_N (CH224A PG), DZF (optional) | Input | Inputs on a separate port | Never on a port that also carries outputs |
| USB DP, DM | USB | Dedicated USB pins | DP pin 29, DM pin 28 |
| JTAG and reset | JTAG | Dedicated pins | TCK 24, TMS 23, TDI 18, TDO 20, RST_N 21 |

# 5. Firmware requirements

The hardware defaults (pull-downs on PDN and EN, relays open) hold the board in a safe state until firmware takes over. Firmware then follows the sequences below.

| # | Event | Required behavior |
| --- | --- | --- |
| 1 | Power-up | PDN low, oscillators off, OPA1622 EN low, relays open (RLY_EN low) |
| 2 | On PD_GOOD_N asserted | Wait for the rails to settle (≈ 100 ms) → enable the oscillator for the current family → release PDN → configure registers over I²C → wait for the oscillator start-up time from standby before releasing RSTN → start I²S → enable the OPA1622s → wait ≥ 2 s → close the relays |
| 3 | Sample-rate family change | Soft mute → RSTN = 0 (or PDN low) → switch oscillator → RSTN = 1 → unmute |
| 4 | PD power loss | Hardware (LM339) pulls AMP_EN and the relay drive low within microseconds; firmware then sets PDN low and drives AMP_EN and RLY_EN low |
| 5 | Volume | Through the AK4493 attenuator (0.5 dB steps); optional per-channel trim, since the DAC gain mismatch is up to 0.3 dB |
| 6 | First programming | First flash programming through the XTAG4 adapter |

# 6. Schematic review checklist

These rules are the first rule set for the AI datasheet-review agent, and each one can be turned into a planted-error test case. ERC is KiCad's electrical rules check; a netlist script checks connections, part values and counts in the exported netlist; the AI agent reviews rules that need datasheet or part-level interpretation.

| # | Rule | Source | Check method |
| --- | --- | --- | --- |
| 1 | XU316 VDD: ≥ 8 × 100 nF + ≥ 10 µF | R1 | Netlist script |
| 2 | XU316 VDDIO: 100 nF per VDDIO pin + ≥ 10 µF | R1 | Netlist script |
| 3 | PLL_AVDD: ferrite + 1 µF, placed close | R1 | Netlist script; AI agent (ferrite part) |
| 4 | All XU316 VDDIO on one 1.8 V rail | R1 (G.3) | Netlist script |
| 5 | 24 MHz crystal network: 12 pF load, ESR ≤ 60 Ω, Rf = 1 MΩ, Rd = 680 Ω, CL1 = CL2 = 22 pF | R1 | Netlist script; AI agent (crystal part) |
| 6 | QSPI flash on the fixed pins; 1.8 V part (W25Q32JWSSIQ); Quad Enable set; ready within 300 µs | R1 | Netlist script (pins); AI agent (part) |
| 7 | X0D04–X0D06 not pulled up (boot mode 000 = QSPI) | R1 | Netlist script |
| 8 | No multi-bit port mixes inputs and outputs | R1 (G.7) | Netlist script with the port map |
| 9 | USB: 90 Ω differential, ≤ 1 mm skew; VBUS 1–10 µF; 5.1 kΩ on CC1 and CC2 | R1; USB | Netlist script; AI agent (differential pair) |
| 10 | Ground paddle 16 vias (4 × 4); 4 vias under each VDD paddle | R1 | AI agent (footprint) |
| 11 | Debug header present (xSYS2 or TC2030) with RST_N | R1 | Netlist script |
| 12 | AK4493 LDOE = L; TVDD = AVDD = DVDD = 1.8 V; TVDD first | R2 | Netlist script; AI agent (sequencing) |
| 13 | AK4493 PDN pull-down | R2 | Netlist script |
| 14 | VREFH pairs 0.1 µF + 470 µF; VCML and VCMR 10 µF each | R2 | Netlist script |
| 15 | VDDL and VDDR 0.1 µF + 10 µF each | R2 | Netlist script |
| 16 | Single analog ground plane for AVSS, DVSS, VSSL, VSSR | R2 | Netlist script |
| 17 | MCLK damping resistor present; MCLK ≤ 49.664 MHz | R2 | Netlist script; AI agent (oscillator) |
| 18 | DAC output pin load ≥ 450 Ω, ≤ 25 pF | R2 | AI agent (load calculation) |
| 19 | Unused and NC pin handling (NC, TESTE, WCK, DEM0, DZFL/DZFR) | R2 | ERC; netlist script |
| 20 | XU316 VOH vs AK4493 VIH margin flagged (1.35 V vs 1.44 V) | R1, R2 | AI agent |
| 21 | OPA1622 thermal pad to V−; EN never floating; paralleled halves in one package | R4 | ERC (EN); netlist script |
| 22 | Relays on all four legs before both jacks; FAULT_N drives both the relay-driver base and AMP_EN; BAT54 clamps present | Design (3.5) | Netlist script |
| 23 | CH224A: 56 kΩ on CFG1 (15 V); 1 µF on VHV; VBUS pin to VHV | R5 | Netlist script |
| 24 | LMR33630 inverting buck-boost: input capacitors ≥ 50 V; FB and EN referenced to −VOUT; inductor Isat ≥ 2 A | R6, R7 | Netlist script; AI agent |
| 25 | LM339 inputs inside −12 V to +10.5 V; its outputs reach 1.8 V nodes only through a series resistor and clamp | TI LM339 datasheet | AI agent |
| 26 | Relay coil polarity per the HFD4 datasheet; flyback diode on each coil | Hongfa HFD4 datasheet | Netlist script; AI agent |

# 7. Open items to verify

The items below are still open. Each one points to the section where the result is used. Closed in v0.2: QSPI flash, oscillators, inverting converter and inductor, relays and coil current, protection circuit, and the CH224 resistor value — see Section 8.

| # | Item | What to verify | Section |
| --- | --- | --- | --- |
| 1 | Paralleling topology stability | SPICE with TI's OPA1622 model; Option 1 first, Option 2 as fallback | 3.4 |
| 2 | OPA1622 output swing at 100 mA | Datasheet Figures 38/39 | 3.4 |
| 3 | USB impedance on the JLCPCB stack-up | 90 Ω differential; recompute the XMOS example geometry | 3.1 |
| 4 | XU316 port map | Map builds in the XMOS tools before layout | 4 |
| 5 | Flash power-up time | W25Q32JW accepts commands < 300 µs after power-up (datasheet), else hold RST_N | 3.1 |
| 6 | Oscillator standby output | KC2520Z output high-impedance in standby; start-up time for the firmware delay | 3.2 |
| 7 | LMR33630 details | 1.0 V feedback reference; inverting buck-boost stability and start-up (TI SNVA856 / WEBENCH) | 3.6 |
| 8 | CH224A PG and CFG2/CFG3 | PG polarity; CFG2/CFG3 handling per the WCH reference schematic | 3.6 |
| 9 | Protection thresholds | BAV99 drop at µA currents, ±0.6 V node thresholds, PD-loss hysteresis — SPICE and bench | 3.5 |
| 10 | Supply | OPA1622 LCSC stock was 197 on 10 Sep 2026 (400 needed for 100 boards); 4.4 mm jack: LCSC 721, JLCPCB 21 | 8 |
| 11 | 4.4 mm jack assembly | Through-hole part: confirm JLCPCB's soldering method and fee at quote time | 8 |

# 8. Chosen parts

Parts chosen for v0.2 from widely stocked, cost-efficient options (distributor listings Aug–Sep 2026, 100-board price tier).

| Block | Part (MPN) | Key specs | Package | Source (LCSC / JLCPCB no.) | Stock | US$ @100 | Why chosen |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Boot flash | Winbond W25Q32JWSSIQ TR | 1.7–1.95 V, 32 Mbit QSPI | SOIC-8 208 mil, leaded | LCSC C2456252 | 11,886 | 1.16 | 1.8 V member of the W25Q family the XMOS flash library verifies; leaded |
| 44.1 kHz clock | Kyocera KC2520Z22.5792C1KX00 | 1.71–3.63 V CMOS, ±20 ppm, standby pin, 7.3 mA max | 2.5 × 2.0 mm, no-lead | Digi-Key (global sourcing) | 2,985 | 0.87 | Runs from 1.8 V; no 1.8 V 22.5792 MHz part in stock at LCSC |
| 48 kHz clock | Kyocera KC2520Z24.5760C1KX00 | as above | 2.5 × 2.0 mm, no-lead | Digi-Key (global sourcing) | 2,419 | 0.88 | Same family as the 44.1 kHz clock |
| USB crystal | YXC X322524MOB4SI | 24 MHz, 12 pF, ±10 ppm, ESR 50 Ω | 3.2 × 2.5 mm | LCSC C70590 | 58,152 (JLCPCB) | 0.08 | Matches the XU316 crystal rule (12 pF, ESR ≤ 60 Ω) |
| USB PD sink | WCH CH224A | PD 3.2; 56 kΩ on CFG1 requests 15 V | ESSOP-10, leaded | LCSC C42459160 | 20,632 | 0.27 | CH224Q is out of stock; same resistor table; leaded (no X-ray fee) |
| Negative-rail converter | TI LMR33630ADDAR | 3.8–36 V in, 3 A synchronous, 400 kHz | HSOIC-8, leaded | LCSC C841384 | 6,718 | 0.46 | 36 V rating covers Vin + ¦Vout¦; synchronous, so no catch diode |
| Converter inductor | Sunlord SWPA6045S220MT | 22 µH, Isat 2.2 A, 89 mΩ | 6 × 6 mm | JLCPCB C83454 | 18,562 | 0.11 | 2.2× margin over the 0.98 A peak current |
| +12 V LDO | ST LM317D2T-TR | 1.5 A adjustable | D2PAK | LCSC C91673 | 8,205 | 0.31 | D2PAK handles the 1.16 W worst case |
| −12 V LDO | onsemi LM337BD2TR4G | 1.5 A adjustable, negative | D2PAK | LCSC C154567 | 1,090 | 0.37 | Matching negative regulator |
| DAC 5 V LDO | TI TPS7A4901DGNR | 3–36 V in, 150 mA, 12.7 µVrms (20 Hz–20 kHz), 72 dB PSRR at 120 Hz | HVSSOP-8 PowerPAD, leaded | LCSC C16430 | 13,858 | 1.05 | About 1/7 of the LT3045 price; each VREFH pin already has its own 10 Ω + 470 µF (≈ 34 Hz) filter |
| 0.9 V core buck | TI TLV62569DBVR | 2.5–5.5 V in, 2 A, 1.5 MHz, 0.6 V feedback | SOT-23-5 | LCSC C141836 | 153,170 | 0.06 | Covers the 830 mA maximum core current |
| Buck inductor | Sunlord SWPA3015S2R2MT | 2.2 µH, 2 A | 3 × 3 mm | JLCPCB C43389 | 79,454 | 0.06 | Value used with the TLV62569 |
| 3.3 V LDO | AMS1117-3.3 | 1 A | SOT-223 | LCSC C6186 (JLCPCB Basic) | 959,525 | 0.17 | Basic part, so no Extended-part fee |
| 1.8 V LDOs (× 2) | TI LP5907MFX-1.8/NOPB | 250 mA, 6.5 µVrms, enable pin | SOT-23-5 | LCSC C92498 | 15,585 | 0.11 | One part number for both 1.8 V rails |
| Protection comparator | TI LM339DR | quad, open-collector, ±1 to ±18 V | SOIC-14 | LCSC C7948 | 120,550 | 0.14 | One IC covers DC detection on four legs and PD-loss detection |
| Relays (× 2) | Hongfa HFD4/12-SR | DPDT, 12 V coil 1.028 kΩ (11.7 mA), gold-plated AgNi, ≤ 100 mΩ, 3 ms | SMD 10 × 6.5 mm | LCSC C44212 | 7,454 | 1.22 | Gold contacts for low-level audio; SMD; in stock |
| Relay driver | MMBT3904 (JSCJ) | NPN, 40 V, 200 mA | SOT-23 | LCSC C20526 (JLCPCB Basic) | 25,800 | 0.01 | Switches fully from 1.8 V logic |
| PD input TVS | Littelfuse SMAJ18A | 18 V standoff, 29.2 V clamp | SMA | JLCPCB C148219 | 16,102 | 0.12 | Standoff above the 15.75 V maximum PD voltage |
| USB ESD | ST USBLC6-2SC6 | 2-line USB ESD array | SOT-23-6 | LCSC C7519 | 37,525 | 0.12 | Standard USB 2.0 protection |
| USB-C (× 2) | HRO TYPE-C-31-M-12 | 16-pin USB 2.0 receptacle, 5 A | SMD | LCSC C165948 | 110,085 | 0.13 | The most widely used USB-C part at JLCPCB |
| 4.4 mm jack | G-Switch GT-3321667P-01 | 5-pole, gold-plated, 0.5 A | Through-hole | LCSC C2762984 | 721 (JLCPCB 21) | 4.28 | The 4.4 mm jack in the JLCPCB library |
| 3.5 mm jack | HRO PJ-332A-6A | TRRS, gold-plated, 1 A | SMD | LCSC C2848643 | 10,085 | 0.23 | Gold contacts, 1 A rating, SMD |
| Gain switch | G-Switch MS-22D28-G020 | DPDT slide | SMD | LCSC C963205 | 32,925 | 0.10 | Tape and reel, so it can be machine-placed |
| Ferrite beads (× 4) | Murata BLM18KG601SN1D | 600 Ω at 100 MHz, 0.15 Ω, 1.3 A | 0603 | LCSC C85833 | 99,850 | 0.03 | Meets the PLL_AVDD rule (DCR < 1 Ω) |

Small discretes, assigned at the BOM step (JLCPCB Basic parts preferred): 4 × BAV99 (DC-detect diode-OR), 2 × 1N4148W (coil flyback), 2 × BAT54 (clamps), 2 status LEDs. Unchanged from v0.1: XU316-1024-QF60A-C24 (Digi-Key), AK4493SEQ (Mouser), 4 × OPA1622IDRCR (LCSC C2876559: US$2.11, only 197 in stock on 10 Sep 2026), OPA1612AIDR (LCSC). The X-ray fee is assumed for the XU316, the four OPA1622 and the two oscillators.

# Appendix A. Rails and signals

The rails and signals referenced in this document, with their source and destination, as a starting list for the schematic net labels.

| Rail or signal | Type | From → to | Section |
| --- | --- | --- | --- |
| VBUS_DATA | Power, 5 V | USB-C data port → 0V9 buck and 3V3 LDO | 2.2 |
| 0V9 | Power | TLV62569 buck → XU316 VDD; PLL_AVDD via ferrite | 2.2 |
| 3V3 | Power | AMS1117-3.3 LDO → XU316 USB_VDD33; input to the 1.8 V LDOs | 2.2 |
| 1V8_D | Power | LP5907-1.8 LDO → XU316 VDDIO and USB_VDD18, QSPI flash, AK4493 TVDD, I²C pull-ups, debug VREF | 2.2 |
| 1V8_A | Power, quiet | LP5907-1.8 LDO (EN from 1V8_D) → AK4493 AVDD and DVDD, both oscillators | 2.2 |
| VBUS_PD | Power, 15 V | USB-C power port (CH224A) → +12 V LDO, inverting converter, PD-loss comparator divider | 2.2, 3.6 |
| +12 V | Power | ST LM317 → four OPA1622, OPA1612, relay coils, +5 V_A LDO | 2.2 |
| −14.7 V | Power, intermediate | LMR33630 inverting buck-boost → LM337 | 2.2 |
| −12 V | Power | onsemi LM337 → four OPA1622, OPA1612 | 2.2 |
| +5 V_A | Power | TPS7A4901 → AK4493 VDDL and VDDR; VREFHL and VREFHR feeds | 2.2 |
| VREFHL, VREFHR | Reference | +5 V_A via 10 Ω each → AK4493 VREFHL (41, 42) and VREFHR (19, 20) | 3.3 |
| MCLK | Clock | Enabled oscillator → AK4493 MCLK (46) via damping resistor; XU316 X1D11 | 3.2 |
| BCLK, LRCLK, SDATA | I²S | XU316 → AK4493 BICK (3), LRCK (5), SDATA (4); I²S input header | 3.3, 3.7 |
| SCL, SDA | I²C | XU316 ↔ AK4493 SCL (8), SDA (9); 2.2 kΩ pull-ups to 1V8_D | 3.3 |
| PDN | Control | XU316 → AK4493 PDN (2); 100 kΩ pull-down | 3.3 |
| OSC_EN_44K1, OSC_EN_48K | Control | XU316 → standby (enable) pins of the 22.5792 MHz and 24.576 MHz oscillators | 3.2 |
| AMP_EN | Control | XU316 → EN of all four OPA1622; 100 kΩ pull-down | 3.4 |
| RLY_EN | Control | XU316 → relay-driver base (MMBT3904); also pulled low by FAULT_N | 3.5 |
| FAULT_N | Protection | LM339 open-collector outputs → relay-driver base and AMP_EN (clamped) | 3.5 |
| PD_GOOD_N | Status | CH224A PG (open-drain) with pull-up to 1V8_D → XU316 input | 3.6 |
| DZF | Status, optional | AK4493 DZFL (10) / DZFR (11) → XU316 input | 3.3 |
| RST_N | Reset | XU316 RST_N (21) ↔ debug header | 3.1 |
| USB_DP, USB_DM | USB | USB-C data port ↔ XU316 pins 29 and 28; 90 Ω differential | 3.1 |
| CC1, CC2 | USB-C | Data port configuration channels; 5.1 kΩ each to GND | 3.1 |
| L+, L−, R+, R− | Analog output | OPA1622 leg outputs → relays → 4.4 mm and 3.5 mm jacks | 3.4, 3.5 |

# Appendix B. Abbreviations

| Term | Meaning |
| --- | --- |
| C0G | Class 1 ceramic dielectric (NP0), stable over voltage and temperature |
| DFT | Design for test |
| DNF | Do not fit: footprint placed, part not assembled |
| DPDT | Double-pole, double-throw (relay or switch) |
| DZF | Zero-detect flag outputs of the AK4493 (DZFL, DZFR) |
| ERC | Electrical rules check (KiCad) |
| ESD | Electrostatic discharge |
| fs | Sampling frequency |
| FS / HS | USB full-speed / high-speed |
| I²C | Two-wire control bus (SCL, SDA) |
| I²S | Three-wire audio serial interface (BCLK, LRCLK, SDATA) |
| IBB | Inverting buck-boost converter |
| LDO | Low-dropout linear regulator |
| MCLK | Master clock |
| MSL | Moisture sensitivity level |
| OE | Output enable |
| PCBA | Printed circuit board assembly |
| PD | USB Power Delivery |
| PDN | Power-down input of the AK4493 |
| PLL | Phase-locked loop |
| QE | Quad Enable bit of a QSPI flash |
| QSPI | Quad serial peripheral interface |
| THD+N | Total harmonic distortion plus noise |
| TVS | Transient voltage suppressor |
| UVLO | Under-voltage lockout |
| VIH / VOH | Input-high threshold / output-high level |
| xSYS2 / XTAG4 | XMOS debug header / XMOS debug adapter |

# Appendix C. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Initial issue: datasheet-derived requirements and starting values for schematic capture; companion to Design Specification v0.2 |
| v0.2 | 23 Sep 2026 | Diego Yang | Remaining parts chosen (Section 8); CH224A replaces CH224Q; TPS7A4901 replaces LT3045; protection circuit defined (LM339 + 2 × HFD4 relays with hardware amplifier shutdown); −14 V rail set to −14.7 V; open items updated |