# USB DAC + Balanced Headphone Amplifier — Schematic Design Notes v0.6

Companion to Design Specification v0.7: datasheet-derived requirements and starting component values for the KiCad schematic after the sixth review round

| Author | Date | Version | Document type |
| --- | --- | --- | --- |
| Diego Yang | 24 Sep 2026 | v0.6 | Schematic design notes (companion to Design Specification v0.7 and Parts List v0.5) |

# 0. Purpose and references

This document collects the datasheet-derived requirements and starting component values for drawing the KiCad schematic of the USB DAC + balanced headphone amplifier. It is the companion to Design Specification v0.7 and Parts List v0.5 and replaces Notes v0.5, which stays unchanged as history. This revision answers the sixth review round, four reviews (R6-A, R6-B, R6-C and R6-D, Section 7.2): a fault latch that captures short PGOOD faults in ≈ 35 µs and sets at power-on (C605 at node F), a detector sized and checked at the worst case (R601–R604 100 kΩ, R609/R610 10 kΩ), a slow firmware DC tier on every leg that sets the same latch, cause capture at the fault interrupt, a VREF that tracks AVCC so the DAC turns off first and the rails last, USB suspend as a v1 requirement with a power-down of ≤ 4 ms, an LDO dissipation analysis with a dropper for the 3V3D regulator, and a cost gate that covers every line of the JLCPCB quote. The fifth and earlier review rounds stay as history in Sections 7.3 to 7.5.

The designer owns and verifies every value. Circuit values are starting values unless marked otherwise, to be checked in SPICE and on the bench. Every number is computed from the stated values or taken from the datasheet named next to it. Labels are kept as written: “assumption” marks a value not yet confirmed, “to verify” marks a check still to be done, and “per datasheet — to verify” marks a value that must come from a datasheet not yet read. All costs use a fixed 31.7 NTD per US$.

| Ref | Document | Publisher | Document no. / note |
| --- | --- | --- | --- |
| R1 | CH32V307 datasheet and reference manual; CH32V307 EVT package (high-speed USB device examples: HID, CDC serial, mass storage, network adapter; no audio example) | WCH | — |
| R2 | AGRV2K datasheet | AGM | —; I/O state and weak pull-up during configuration still to check (7.1) |
| R3 | ES9018K2M datasheet | ESS | v3.7 (22 Apr 2021); pages 8, 27 and 31 used for the output stage, power-up and MCLK limits; page 30 for the audio-interface timing; “XI / MCLK Timing” (cycle ≥ 10 ns, pulse width ≥ 4.5 ns, duty 45:55 to 55:45) and the absolute maximum “DAC Output voltage Range: GND < Vout < AVCC” |
| R4 | OPA1622 datasheet | TI | SBOS727B; Fig. 38/39 (output swing) |
| R5 | LM27762 datasheet | TI | SNVSAF7C |
| R6 | LP5907 datasheet | TI | Output automatic discharge 230 Ω when disabled (3V3A below 0.3 V within ≈ 3.2 ms after AVCC_EN goes low, 2.5); output capacitance ≤ 10 µF; noise < 6.5 µVrms |
| R7 | USBAudioToI2S firmware project | GitHub, LoveLonelyTime | Apache-2.0; known-good comparison and reference only (the firmware and CPLD logic are our own); source (audio_cpld.v) reviewed for the clock path |
| R8 | JLCPCB PCBA price list | JLCPCB | Economic PCBA fees used for the 5-board cost gate |
| R9 | TPS2553 datasheet | TI | SLVS841F (current-limit equations, Figure 24) |
| R10 | OPA197 datasheet (OPA197, OPA2197, OPA4197) | TI | SBOS737C; table 6.8 and section 7.3.6: input-pair transition region typically (V+) − 3 V to (V+) − 1.5 V, VOS not specified for (V+) − 3.5 V < VCM < (V+) − 1.5 V; the reason the OPA2197 was replaced (3.5.1) |
| R11 | USB Type-C specification | USB-IF | CC voltage bands for the current advertisement |
| R12 | LCSC and JLCPCB part pages | LCSC / JLCPCB | Classes, prices and stock, Jun–Sep 2026 (sourcing evidence for Parts List v0.5); the assembly-fixture sentence is standard text on every part page (Section 8) |
| R13 | SCTF 2CM series oscillator datasheet (X203) | SCTF | Manufacturer file “2CM-Series-2024-11-04”: pin 1 “Tri-State”, enabled when OE is logic 1 or open, disabled when logic 0 (≤ 0.3 VDD); start-up ≤ 5 ms; 10 mA max; duty 45–55 %; no disable/enable time, internal pull-up value or disabled-output leakage specified |
| R14 | SCTF 3C series oscillator datasheet (X201, 80 MHz) | SCTF | SX3M80.000B20F30TNN: 3.3 V CMOS, ±20 ppm tolerance / ±30 ppm stability, 40 mA max for 80–160 MHz, duty 45–55 %, tri-state pin 1 |
| R15 | OPA210 datasheet (OPA2210) | TI | SBOS924H: single input pair, common-mode range (V−) + 1.5 V to (V+) − 1.5 V, Vos ±35 µV max, 2.2 nV/√Hz, 18 MHz, 2.2 / 2.5 mA per channel, ±2.25 V to ±18 V; output swing (V+) − 0.2 V at RL = 10 kΩ and (V+) − 0.6 V at 600 Ω (±15 V); input current 10 mA (absolute maximum ratings and section 7.3.5) |
| R16 | USB Type-C timing: tSinkAdj 60 ms max, tRpValueChange 10–20 ms | Zephyr Project; Chromium EC | USB Type-C specification timing as quoted in the Zephyr USB-C stack and the Chromium EC documentation; used for the CC monitoring (5.2) |
| R17 | ME6211 datasheet (ME6211C33M5G-N, U502 and U504) | Microne | SOT-23 power dissipation 300 mW (absolute maximum ratings); used for the LDO dissipation check (3.6.3) |

# 1. Key design decisions

The decisions below fix the v0.6 schematic. The Basis column records why each choice was made and which review finding it answers (finding IDs in Sections 7.2 to 7.5). The clock, firmware and cost rows follow the owner decisions of 24 Sep 2026.

| Decision | Choice | Basis |
| --- | --- | --- |
| Cost target | ≤ 1,500 NTD per board target for the 5-board sample run (WARN above); ceiling 1,700 NTD per board as the hard order gate (FAIL above). The ceiling covers every JLCPCB line of the real quote: PCBs, PCBA fees (including any fixture or other assembly line that appears in the quote) and parts. Outside the ceiling: shipping, import tax, programmers, enclosure and proof-of-concept hardware | Ceiling 1,700 NTD per board, set when the third oscillator was added; on 24 Sep 2026 the owner confirmed the budget is flexible as long as it does not surge; the actual JLCPCB quote from the real BOM/CPL files must pass the ceiling before ordering, and at approval both the gated subtotal and the total payable (gated subtotal + shipping + tax) are recorded (R5-A13, R5-B9, R6-A cost note, R6-B4) |
| Cost status | 1,650 NTD per board (8,248 NTD for 5 boards) | WARNING: 150 NTD over the 1,500 NTD target and 50 NTD under the 1,700 NTD ceiling; ≈ 1,552 NTD per board without the conservative fees; ≈ 1,755 NTD per board if the OPA2210 has to be bought at the LCSC price (Section 8). Parts List v0.5 is the source of truth; final with the JLCPCB quote after layout |
| USB audio bridge | WCH CH32V307RCT6 + AGM AGRV2KQ32 CPLD; our own USB Audio Class 2.0 firmware and our own CPLD logic (Section 5) | Owner decision 24 Sep 2026; USBAudioToI2S (R7) is kept only as a known-good comparison. Two-stage proof of concept before layout; Stage 2 is mandatory and gates the layout (R4-B S4; R4-C B3–B5, H7; R5-B4, R5-B8) |
| Clock | DAC MCLK 80 MHz from X201 (SCTF SX3M80.000B20F30TNN; asynchronous mode, never switched), also the CPLD system clock; family oscillators X202 24.576 MHz and X203 22.5792 MHz share the CPLD PLL input, both off by default (2.2 kΩ enable pull-downs) and switched break-before-make, so only one drives the node at a time; PLL ×2 gives exact integer clock ratios with an ideal 50 % bit-clock duty for all 8 rates, 352.8 kHz included (3.3) | 80 MHz is below the ES9018K2M MCLK limit: 12.5 ns cycle (≥ 10 ns), ≥ 5.6 ns pulse width (≥ 4.5 ns), 80 MHz ≈ 208 × 384 kHz > 192 × FSR (asynchronous mode needs no integer ratio); the previous 100 MHz part sat at the limit (R5-A3, R5-B2). The DAC needs a DATA_CLK duty cycle of 45:55 to 55:45 (R3 p. 30), hence the family oscillators in front of the PLL (owner decisions 24 Sep 2026). Break-before-make answers R5-A2, R5-B6 and R5-C1 |
| DAC | ESS ES9018K2M, I²C register control; differential current-mode outputs into I/V converters (ESS application circuit, R3 p. 27) | DNR 127 dB-A and THD+N −120 dB per datasheet, specified in differential current mode and asynchronous mode |
| DAC core supply | Fixed 1.3 V LDO Microne ME6216A13M3G (±1 %, 300 mA, ceramic-stable) fed from the switched 3V3D domain | External DVDD is required for FSR > 192 kHz (R3). Fed from 3V3D, DVDD cannot exceed DVCC during ramps; the previous adjustable LDO needed a tantalum-type capacitor and a 5–10 mA minimum load (R4-B M5; R4-A M4) |
| Headphone rails | +3.78 V / −3.72 V from TI LM27762: FB+ 215 kΩ / 100 kΩ → +3.780 V (3.673–3.890 V); FB− 205.1 kΩ / 100 kΩ → −3.722 V (3.618–3.828 V magnitude); ranges incl. VFB and the 1 % resistors; each top resistor built from Basic values (100 kΩ + 100 kΩ + 15 kΩ or 5.1 kΩ) | The negative rail (charge pump + negative LDO) keeps regulation at the USB supply corner: +0.11 V margin to the maximum ¦V−¦ at VBUS 4.60 V and maximum load (2.3). R518 is now 5.1 kΩ because the divider-resistor tolerance is included (R4-A H4; R4-B S1; R4-C H2; R5-A9) |
| Output level | Every unit calibrated to 1.75 Vrms per leg at 0 dBFS (3.5 mm 1.75 Vrms, 4.4 mm 3.5 Vrms) with the ES9018K2M 32-bit master trim, stored in MCU flash at the end-of-line test; the −3.5 dB pre-calibration default is a safety setting, active until a calibration is stored (2.92 V peak at the I/V output of the highest-gain unit, 3.5.1); Rf = 1 kΩ ∥ 5.1 kΩ = 836 Ω | Untrimmed unit spread 1.955–2.589 Vrms per leg; the calibrated level fits the +3.78 V / −3.72 V rails. 836 Ω uses Basic values (R4-B S2, S3; R4-A M3; R6-D5) |
| Power rating method | Rated at 1 % THD+N or calibrated 0 dBFS, whichever comes first (both channels, 1 kHz, calibrated level, worst-case rail); the clean output envelope (≤ 80 mA peak per leg and the OPA1622 swing curve) is kept separate from overload behaviour; Default mode is rated by the instantaneous peak current | R4-A H5, M3; R4-B S3; R4-C B6; R5-C4 (the TPS2553 limits instantaneous current) |
| Analog current limit | TI TPS2553 current-limited switch feeds the LM27762 with two limits: Default 0.157–0.213 A (RILIM 147 kΩ) and High 0.393–0.498 A (59.5 kΩ); the limit acts on instantaneous current | Overload → rails droop → LM27762 PGOOD bad → fault latch set in hardware within ≈ 35 µs, which disables the amplifiers and turns the rails and the DAC analog supply off → firmware captures the cause, clears the latch and lowers the volume ceiling (R4-A H5, H6; R4-B M4; R4-C B6, H1; R5-A7, R5-B3; R6-A1) |
| USB power states | Before configuration only the MCU domain 3V3M is powered (≤ 83 mA estimate; assumption on MCU current). Audio domains are switched: ME6211 #2 CE (3V3D), LP5907 EN (3V3A), TPS2553 EN (5V_ANA). CC1/CC2 are read by the MCU ADC through 10 kΩ to select Default (≤ 500 mA; estimated maximum 0.469 A with hardware-limited analog current) or High mode (≥ 1.5 A advertised) and are monitored while configured. Descriptors: bus-powered, bmAttributes 0x80, bMaxPower 0xFA (500 mA). USB suspend is a v1 requirement: ≤ 2.5 mA within 10 ms of the last bus activity (budget ≈ 2.2 mA; controlled power-down ≤ 4 ms after the suspend interrupt, 2.5) | R4-A H6; R4-C B1, B2, H1; R5-A4, R5-C9; R6-A3, R6-B3. The TPS2553 limits only the analog branch; the MCU, CPLD and DAC-core currents are estimates or datasheet values, measured on the Stage 2 board (real regulators with dummy loads) before layout (R5-B4) |
| I/V op-amp | TI OPA2210 (single input pair, common-mode range (V−) + 1.5 V to (V+) − 1.5 V, Vos ±35 µV max, 2.2 nV/√Hz, 18 MHz, 2.5 mA per channel max; LCSC C1849415); output swing ≈ (V+) − 0.3 V at the ≈ 3 mA I/V load, 0.43 V above the calibrated 2.94 V peak (3.5.1) | VREF 1.703 V sat inside the OPA2197 input-pair transition region, where its offset is not specified and its noise rises (R10); the OPA2210 has one input pair and 0.47 V common-mode margin at the minimum +rail (R5-A5, R6-D5); no qualified second source, 12 reserved (Section 8, R6-C3) |
| Output legs | 2 × TI OPA1622, one amplifier per leg; each leg a 4-resistor difference amplifier (2.00 kΩ 0.1 %) fed by two I/V outputs | U401 = L+/L−, U402 = R+/R−, so an adapter that shorts leg− loads only one amplifier per package (TI continuous-short rating, R4) |
| Protection | Fast hardware tier: LM393 window on every leg (R601–R604 100 kΩ, C601–C604 1 µF, R609/R610 10 kΩ), PGOOD bad (Q610) and a sustained TPS2553 overcurrent (D606) set the fault latch through node F (C605 at node F: power-on set and a ≥ 1.5 ms set-request stretch); 47 kΩ latch pull-ups, capture ≈ 35 µs; the latch kills RAIL_EN, AMP_EN and AVCC_EN (Q607, Q608, Q611) until firmware clears it; Q609 is a slow interlock (6–32 ms). Slow firmware tier: per-leg DC-sense networks to four MCU ADC pins; ≥ 0.8 V → latch within 1 s, ≥ 0.2 V → within 3 s, through the ANA_FAULT pin. Cause capture at the FLT interrupt before any other action; latch built from 2N7002 N-MOSFETs only | Worst-case removal ≤ 59 ms at 2 V (limit 100 ms) and ≤ 183 ms at 1 V (limit 250 ms); terminal energy for a rail-stuck failure in High mode 26.2 / 13.3 mJ worst case at 16 / 32 Ω (limits 30 / 15 mJ, including the ≤ 0.6 mJ after the trip) (3.7). R4-A H3; R4-B M1, M2; R4-C H6; R5-A6, R5-A7, R5-B3; R6-A1, R6-A5, R6-B1, R6-B2. The residual DC blind region below 0.2 V per leg is an owner decision pending. The PNP transistor is no longer a JLCPCB Basic part, so the latch uses Basic 2N7002 (C8545) only |
| Power sequencing | VREF tracks AVCC (C421 330 pF C0G, τ ≈ 2.8 µs; D401 and C422 removed). Power-up: 5V_ANA, rails, then AMP_EN while AVCC is still off (VREF = 0, the legs start at ≈ 0 V), then AVCC_EN (VCCA, AVCC and VREF rise together), then AUD_EN and RESETB. Controlled power-down (suspend, host command, fault) ≤ 4 ms: the DAC turns off first (RESETB, AVCC_EN and AUD_EN low together) while the I/V converters and legs stay powered, then AMP_EN, RAIL_EN and ANA_EN | The ES9018K2M outputs must stay between GND and AVCC (R3 absolute maximum): the I/V converters hold the pins at VREF, which now follows AVCC in every ramp and in the latch shutdown; no rails-off, DAC-powered window remains; the legs are enabled and disabled while VREF = 0, so the disabled-state network path does not reach the jack (R6-A2, R6-A3, R6-D1, R6-D7; 2.5) |
| Input ferrite | Murata BLM18PG121SN1D (C14709, Basic, 2 A, 50 mΩ) in the 5V_ANA path (≤ 0.50 A); Sunlord GZ1608D601TF (C1002) only on the DAC DVCC branch (≈ 14 mA) | GZ1608D601TF is a 200 mA / 450 mΩ part (C1002; C1015 is GZ2012D101TF), so it cannot carry the power path (R4-A H1) |
| DC offset requirement | ≤ ±10 mV unbalanced, ≤ ±20 mV balanced; 100 % offset screening in the automated test with a DC-capable DMM | Worst case 7.0 / 14.0 mV with datasheet maxima (≈ 3.9 mV RSS unbalanced); 8.6 / 17.2 mV if the DAC null is 1.65 V (R433 option, 3.4); 20 mV into 16 Ω balanced = 25 µW DC (R4-A H2; R4-C H5; R5-A11; R6-D4) |
| BOM | One part per line; every line has an LCSC number; every line’s JLCPCB class is taken from its JLCPCB part page or the basic-parts list, except X202, whose class is not shown (assumed Extended) | R4-A M1; R4-C H4; R5-A14 |
| Firmware | Our own USB Audio Class 2.0 firmware on the CH32V307 high-speed port, built on WCH’s USBHS device examples (TinyUSB only if proven reliable), and our own CPLD logic; two-stage proof of concept before layout (firmware gate; Stage 2 mandatory and the layout gate, Section 5) | Owner decision 24 Sep 2026. WCH’s high-speed examples cover HID, CDC, mass storage and network, not audio; USBAudioToI2S (R7) stays a known-good comparison (R4-B S4; R4-C B3–B5, H7; R5-B8) |
| Kept | 3.5 mm unbalanced + 4.4 mm balanced jacks (both live); ≥ 384 kHz; single USB-C port, bus-powered; JLCPCB Economic PCBA; CI, AI review and measurement automation | Unchanged requirements |

# 2. Power tree and current budget

One USB-C port supplies the board. VBUS (5V_SYS) feeds four regulators: an always-on 3.3 V LDO for the MCU domain, a switched 3.3 V LDO for the audio-digital domain (through the R519/R520 dropper), the DAC’s analog LDO and the TPS2553 current-limited switch for the headphone rails. The MCU enables the audio domains after USB configuration. Currents are estimates unless marked as datasheet figures; the TPS2553 limits only the analog branch, so the totals are estimated maxima with hardware-limited analog current, measured on the Stage 2 board before layout.

Figure 1. Power domains and enables (5V_SYS, 3V3M, 3V3D with the CPLD and the three oscillators including the 80 MHz MCLK, 1V3, 3V3A, 5V_ANA, +3.78 V / −3.72 V rails for the OPA2210 I/V converters and OPA1622 legs; PGOOD to the overload latch).

## 2.1 Power domains

| Domain | Regulator and parts | Enable | Loads |
| --- | --- | --- | --- |
| 5V_SYS | VBUS from J101 (no switch); U101 USBLC6-2SC6; C102 1 µF | Always | Inputs of U502, U504 (through R519/R520), U302 and U503 (5 × 1 µF on VBUS in total) |
| 3V3M | U502 ME6211C33M5G-N; C501, C502 1 µF | Always on | CH32V307 (C211 2.2 µF bulk), fault latch and its pull-ups, slow DC-sense networks (R628–R631), LEDs, PGOOD and FAULT pull-ups (R505, R512) |
| 3V3D | U504 ME6211C33M5G-N, fed through the dropper R519 + R520 (2 × 2.2 Ω 0805 in series, 4.4 Ω); C503 1 µF at the input, C504 1 µF and C512 10 µF at the output; CE pull-down R508 100 kΩ | AUD_EN | AGRV2KQ32, oscillators X201 (80 MHz), X202 (24.576 MHz) and X203 (22.5792 MHz) with the enable pull-downs R212, R213 (2.2 kΩ) and the node pull-down R214, DAC DVCC via FB301, 1.3 V LDO input, I²C pull-ups (R207, R208) |
| 1V3 | U303 ME6216A13M3G from 3V3D; C311, C312 1 µF; C310 2.2 µF at the DAC pin | Follows 3V3D | ES9018K2M external DVDD |
| 3V3A | U302 LP5907MFX-3.3 from 5V_SYS; C301, C302 1 µF; EN pull-down R303 100 kΩ; 230 Ω output discharge when disabled (R6) | AVCC_EN (pulled low by Q611 when the latch is set) | VCCA, AVCC_L and AVCC_R (via 2.2 Ω) and the VREF divider (VREF = 0.516 × 3V3A, tracks it) |
| 5V_ANA | U503 TPS2553DBVR; C505, C506 1 µF; EN pull-down R513 100 kΩ; RILIM R509 + R514 = 147 kΩ, R510 in parallel via Q501 for High mode; FB501 to the LM27762 VIN | ANA_EN, ILIM_HI | LM27762 (C507 10 µF at VIN) |
| +3.78 V / −3.72 V | U501 LM27762DSSR from 5V_ANA; C508 1 µF flying, C509 10 µF CP, C510 10 µF OUT+, C511 10 µF OUT−; EN+/EN− = RAIL_EN (R506 pull-down, R507 10 kΩ); PGOOD → R505 to 3V3M | RAIL_EN | 2 × OPA1622, 2 × OPA2210, LM393 |

## 2.2 Rail budget by USB state

Digital loads at 5 V (typical / maximum):

| Load | Typ / max | Basis |
| --- | --- | --- |
| MCU CH32V307 incl. USB HS PHY | 60 / 80 mA | Assumption: the datasheet electrical tables were not obtained — to confirm |
| LEDs, latch, pull-ups | 2 / 3 mA | Estimate |
| CPLD AGRV2KQ32 | 15 / 25 mA | Assumption |
| Oscillator X201, 80 MHz | 15 / 40 mA | SCTF 3C datasheet (R14): 40 mA max for 80–160 MHz; typical assumed |
| Oscillator X202, 24.576 MHz | 3 / 5 mA | YXC YSO110TR datasheet: 5 mA max; typical assumed |
| Oscillator X203, 22.5792 MHz | 5 / 10 mA | SCTF 2CM datasheet (R13): 10 mA max; typical assumed. Only one family oscillator drives the node at a time; a disabled output-enable oscillator may keep running internally, so both are counted |
| DAC DVCC | 13.5 / 14.2 mA | Datasheet (R3) |
| DAC external DVDD 1.3 V | 50 / 60 mA | Datasheet: 50 mA at MCLK 80 MHz and 192 kHz; +20 % for 384 kHz and spread |
| AVCC_L + AVCC_R + VCCA + VREF divider | 17 / 17.5 mA | Datasheet and divider |
| Enable pull-downs (2 × 2.2 kΩ, one enable driven high) + node pull-down R214 | 1.6 / 1.7 mA | 1.5 mA per enable when driven high |

Current by USB state (5 V at the receptacle):

| State | Domains on | Current (typ / max) | Notes |
| --- | --- | --- | --- |
| Attached, not configured | 3V3M only (MCU, latch, LEDs) | 62 / 83 mA | ≤ 100 mA unit load; rests on the MCU current assumption; measured on the Stage 2 board before layout (gate); mitigation: lower MCU clock before configuration |
| Configured, idle | All domains, outputs muted | 227 / 309 mA | Digital 182 / 256 mA plus analog quiescent 45 / 53 mA (the OPA2210 adds ≈ 10 mA) |
| Playing, Default mode | All | ≤ 0.469 A estimated maximum | Estimated maximum with hardware-limited analog current: 0.256 A digital + 0.213 A TPS2553 maximum Default limit (31 mA below 0.5 A); covers USB 2.0 500 mA hosts and phones that limit OTG to ≤ 500 mA |
| Playing, High mode | All | ≤ 0.754 A estimated maximum | Only when CC advertises ≥ 1.5 A (0.256 A digital + TPS2553 High limit ≤ 0.498 A) |
| Suspend | 3V3M, MCU low-power | ≤ 2.5 mA within 10 ms of the last bus activity (v1 requirement); budget ≈ 2.2 mA | USB 2.0 suspend limit. Budget (mA): MCU low-power with USB wake-up ≤ 1.9 (assumption, measured), latch pull-up 0.07, node F pull-up 0.07, node S pull-up 0.03, VBUS divider 0.03, 3V3M LDO quiescent 0.04, slow DC-sense networks 0.007, regulators off 0.003, PGOOD pull-up 0.03. Controlled power-down ≤ 4 ms after the suspend interrupt; suspend level ≈ 7 ms after the last bus activity (2.5); measured on the Stage 2 board and on the main board |

Quiescent current per rail (max): OPA1622 13.2 mA, OPA2210 10 mA, LM393 1 mA, plus 3.4 mA DC from the I/V outputs into the leg networks (positive rail). Analog current per load, average and instantaneous peak, is in 3.6.2.

## 2.3 Supply headroom

LM27762 (R5): VOUT+ = 1.2 V × (R1 + R2)/R2 and VOUT− = −1.22 V × (R3 + R4)/R4, with R2 and R4 ≥ 50 kΩ; VFB+ 1.182–1.218 V, VFB− −1.202 to −1.238 V. Including VFB and the 1 % divider resistors, the dividers give +3.780 V (3.673–3.890 V) and −3.722 V (3.618–3.828 V magnitude; R518 is now 5.1 kΩ). Supply chain at the design corner: VBUS 4.60 V at the receptacle (host minimum 4.75 V minus 150 mV cable and connector drop at ≤ 0.5 A — assumption); TPS2553 rDS(on) 135 mΩ max; ferrite 50 mΩ (60 mΩ used); traces 20 mΩ; charge-pump resistance 3.0 Ω (assumption: 2.5 Ω is specified only at 5.5 V). Worst clean load: 4.4 mm, 32 Ω, High mode, 80 mA per leg (average analog input 260 mA, negative-rail peak 184 mA).

| VBUS at the receptacle | LM27762 VIN | Negative-rail capability | Margin to the max ¦V−¦ (3.828 V) | Positive-rail capability | Margin to the max V+ (3.890 V) |
| --- | --- | --- | --- | --- | --- |
| 4.60 V (design corner) | 4.544 V | −3.936 V | +0.11 V | 4.460 V | +0.57 V |
| 4.40 V (stress corner) | 4.344 V | −3.736 V | −92 mV at current peaks (brief dropout) | 4.260 V | +0.37 V |

At the 4.40 V stress corner the negative rail is 92 mV short of the maximum |V−| at current peaks (brief dropout; 4.4 mm 32 Ω, High mode; bench check); the 95 % PGOOD thresholds (+3.59 V, −3.54 V) are not reached. With the previous 210 kΩ FB− divider and the resistor tolerance, the design-corner margin would have been only 46 mV (the reviewer’s 66 mV figure used the OPA2197 quiescent current).

## 2.4 Inrush

| Item | Value | Basis |
| --- | --- | --- |
| Capacitance on VBUS before any switch | 5 × 1 µF: C102, C501, C503, C301, C505 | Connector and the inputs of U502, U504 (after R519/R520), U302 and U503 |
| 3V3M capacitance charged at attach | 4.4 µF: 1 µF LDO output + 12 × 100 nF (C203–C210, C212, C605, C606, C607) + 2.2 µF MCU bulk | Only 3V3M starts at attach; C211 is 2.2 µF for this budget |
| Attach charge | ≈ 40 µC (≈ 5 µF on VBUS at 5 V plus 4.4 µF on 3V3M) | USB-IF limit 50 µC; nominal values (DC-bias derating makes the real figure lower) |
| Later domain enables | Staggered by firmware; the headphone rails charge through the TPS2553 soft start and current limit | VBUS droop at each enable is measured at bring-up |

## 2.5 Power-up and power-down sequence

Pull-downs on AUD_EN, AVCC_EN, ANA_EN, RAIL_EN, AMP_EN, RESETB, OSC48_EN and OSC44_EN and the latch set at power-up (node F starts at 0 V through C605) hold everything off until firmware runs the sequence below. It meets the ES9018K2M recommended power-up (R3 p. 27): VCCA first; AVCC_L/AVCC_R and DVCC at the same time or later; external DVDD at the same time as DVCC or later; RESETB asserted until ≥ 1 ms after all supplies and MCLK are stable; later resets ≥ 10 ns. X201 starts with 3V3D; X202 and X203 stay off through their 2.2 kΩ enable pull-downs until the CPLD enables X202, and RESETB is released only when both clocks are stable. VREF tracks AVCC (0.516 × 3V3A, τ ≈ 2.8 µs), so the I/V converters hold the DAC pins inside GND…AVCC in every controlled transition and in the latch shutdown (R3 absolute maximum GND < Vout < AVCC). The DAC therefore turns off first and the rails last, and the legs are enabled and disabled while VREF = 0.

| # | Step | Hardware and timing | ESS p. 27 / basis |
| --- | --- | --- | --- |
| 1 | Headphone supply | After USB configuration: ANA_EN high → TPS2553 soft start → 5V_ANA | Rails first on; RAIL_EN only after 5V_ANA (EN must not exceed VIN) |
| 2 | Headphone rails | ≥ 200 ms after attach with the rails off: FLT_CLR pulse, confirm FLT low; RAIL_EN high → +3.78 V / −3.72 V → PGOOD good; the set path is armed 25–133 ms later | FLT_CLR only after RAILS_OK_N has been high (rails off) ≥ 200 ms, so the PGOOD-bad set request is blanked |
| 3 | Output legs | PGOOD good ≥ 150 ms (covers the 133 ms maximum arming time): AMP_EN high while AVCC is still off (VREF = 0 V, I/V outputs at 0 V), so the legs start at ≈ 0 V | Legs enabled while VREF = 0: the disabled-state path through the difference-amplifier network is not present at enable (pop definition, 3.8) |
| 4 | DAC analog | AVCC_EN high → LP5907: VCCA, AVCC_L/R and VREF rise together (common mode; the legs are active and hold the jack at ≈ 0 V ± 3.4 mV common-mode leakage) | VCCA first, AVCC with it; VREF lag ≤ 4 mV, so the DAC pins stay inside GND…AVCC |
| 5 | DAC digital | +5 ms: AUD_EN high → 3V3D: DVCC, CPLD and X201 start together; X202 and X203 stay off through R212/R213 until the CPLD, after configuration and with its PLL in reset, enables X202 ≥ 1 ms later; the 1.3 V DVDD LDO is fed from 3V3D, so DVDD ≤ DVCC during the ramp | DVCC later than VCCA; DVDD with or after DVCC |
| 6 | Reset, init and unmute | MCLK (X201) and the family clock (X202, start-up ≤ 3 ms) running and the CPLD PLL locked → RESETB released ≥ 1 ms after all supplies and both clocks are stable → I²C init (asynchronous mode, 32-bit I²S, mute, master trim from flash) → firmware DC tier armed → soft unmute when streaming | RESETB ≥ 1 ms after stable supplies and MCLK (here both clocks). ESS order kept: VCCA first, AVCC with it, DVCC later, DVDD with or after DVCC |
| 7 | Power-down, t0: DAC off | Controlled power-down (USB suspend, host command or fault command), ≤ 4 ms in total. t0 = suspend interrupt (3 ms after the last bus activity) or the command: soft mute (the stream is already silent at suspend); RESETB, AVCC_EN and AUD_EN low together, so the DAC turns off first while the I/V converters and legs stay powered; 3V3A falls below 0.3 V within ≈ 3.2 ms through the LP5907’s 230 Ω output discharge (R6) and VREF follows; the legs stay enabled and hold ≈ 0 V | ES9018K2M absolute maximum GND < Vout < AVCC: VREF lags AVCC by ≤ 4 mV, so the I/V converters keep the DAC pins inside GND…AVCC |
| 8 | Power-down, t0 + 3.5 ms: legs and rails off | AMP_EN low, then RAIL_EN and ANA_EN low; the rails decay on their own, and nothing is left that could back-power the I/V stage | Legs disabled after VREF has collapsed (pop, 3.8); no “rails off, DAC powered” window remains; the latch sets as the rails go off and firmware masks it (5.2) |
| 9 | Power-down, t0 + 4 ms: suspend level | MCU low-power; VBUS current at the suspend level ≈ 7 ms after the last bus activity | USB suspend ≤ 2.5 mA within 10 ms of the last bus activity (3 ms margin); budget ≈ 2.2 mA (2.2) |
| 10 | Latch trip | Q607, Q608 and Q611 pull RAIL_EN, AMP_EN and AVCC_EN low within ≈ 35 µs; 3V3A collapses within ≈ 3.2 ms, about when the I/V rails (≈ 13 mA load with the amplifiers off) fall to the OPA2210 minimum supply (≈ 3.6 ms); firmware then sets RESETB, AUD_EN and ANA_EN low | Any residual back-power ≤ 1.7 mA per DAC pin (806 Ω − 11 %), inside the OPA2210 10 mA input-current rating (R15: absolute maximum ratings and section 7.3.5) |
| 11 | Unplug / brownout | VBUS analog watchdog at 4.10 V (3.95–4.25 V) starts the power-down (target < 5 µs to the GPIOs). At 0.47 A with ≈ 4.5 µF on VBUS, VBUS falls at up to ≈ 0.1 V/µs: 4.10 V → 3.46 V (3.3 V LDO dropout) in ≈ 6 µs, so the watchdog has ≈ 1 µs margin. PGOOD goes bad at VBUS ≈ 3.70 V → latch (hardware backstop). At full load the logic rails may droop before the sequence completes | Unplug pop ≤ 20 mV peak stays a measured target; DAC-pin safety does not depend on this timing (VREF tracking, ≤ 0.15 V above AVCC for microseconds); the watchdog-to-GPIO latency is measured on the Stage 2 board |
| 12 | Domain interfaces | MCU outputs toward the CPLD stay low or high-impedance while 3V3D is off; I²C pull-ups sit on 3V3D | No back-powering of the audio-digital domain |

# 3. Block notes

Entries with a reference are datasheet requirements; all other values are starting values. Designators, values and LCSC numbers follow Parts List v0.5.

Figure 2. Signal chain: 80 MHz oscillator (DAC MCLK and CPLD system clock), enable-selected 24.576 or 22.5792 MHz family oscillator (break-before-make, one on at a time) into the CPLD PLL, DAC in current mode, OPA2210 I/V converters and OPA1622 legs on the +3.78 V / −3.72 V rails; the LM393 DC detect and the MCU slow DC check, PGOOD and overload set the fault latch.

## 3.1 USB-C input and sensing

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| Connector | J101 SHOU HAN TYPE-C 16PIN 2MD(073) (C2765186, Extended): USB 2.0 data + 5 V, sink | No Basic 16-pin USB-C receptacle exists; only the 4.4 mm jack is through-hole |
| CC pull-downs | R101, R102 5.1 kΩ Rd to GND | USB-C sink |
| CC sensing | CC1, CC2 → R103, R104 10 kΩ → MCU ADC; the ADC does not load CC | Bands (R11): Default 0.25–0.61 V, 1.5 A 0.70–1.16 V, 3.0 A 1.31–2.04 V. A USB-A-to-C cable reads as Default |
| Power mode | High mode only when ≥ 1.5 A is advertised; otherwise Default mode; CC1/CC2 monitored while configured (sampled at least every 5 ms; a drop to Default is handled within tSinkAdj, 5.2) | Many phones limit OTG to ≤ 500 mA and run in Default mode; USB Type-C timing per R16 |
| VBUS sense | R105 100 kΩ / R106 47 kΩ → MCU ADC (5.0 V → 1.60 V); analog-watchdog interrupt at 4.10 V nominal (3.95–4.25 V with the ADC reference, divider and ADC errors) | Unplug / brownout early warning → firmware power-down (target < 5 µs to the GPIOs; ≈ 1 µs margin at 0.47 A, 2.5); ≥ 0.15 V margin to the 4.40 V stress corner and ≥ 0.35 V to the 4.60 V design corner |
| ESD | U101 ST USBLC6-2SC6 on D+, D− and VBUS at the connector | — |
| VBUS capacitance | C102 1 µF at the connector; 5 × 1 µF on VBUS in total | Inrush in 2.4 |
| Shield | R107 1 MΩ and C101 4.7 nF from shield to GND | Bleed and RF path |
| USB data | D+/D− to the CH32V307 USB HS pins as a 90 Ω differential pair | Layout per WCH reference |

## 3.2 CH32V307RCT6 (USB MCU)

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| Device | U201, LQFP-64 10 × 10 mm, 144 MHz RISC-V, on 3V3M (always on) | R1; C2988083. LCSC stock 119: reserve before ordering |
| HSE crystal | Y201 YXC X50328MSB2GI, 8 MHz, 5.0 × 3.2 mm, ±10 ppm, CL 20 pF (C115962, Basic); C201, C202 33 pF C0G | Load capacitors 2 × (20 pF − ≈ 3.5 pF stray) ≈ 33 pF. Firmware sets the USB HS PLL reference to 4 MHz as HSE ÷ 2, so 8 MHz is confirmed. No Basic 3.2 × 2.5 mm 8 MHz crystal exists |
| Decoupling | C203–C210 100 nF, one per VDD/VDDA/VIO pin; C211 2.2 µF bulk | 2.2 µF keeps the attach charge ≈ 40 µC (2.4) |
| Reset | R201 10 kΩ NRST pull-up; C212 100 nF | — |
| Boot | R202 10 kΩ BOOT0 pull-down | — |
| USB HS | D+/D− as a 90 Ω differential pair | Layout per WCH reference |
| Debug | J201 2-wire pads for WCH-LinkE: 3V3, SWDIO, SWCLK, GND | Programming |
| Pin map | Set by our MCU–CPLD interface specification (Section 5, open item); new enables and sensing inputs on free pins (Section 4); the SPI link clock goes to a CPLD global-clock-capable input; ANA_FAULT on an open-drain-capable pin (firmware drives it low to set the latch, 3.7) | Frozen before the Stage 2 proof-of-concept board and the schematic |
| ADC inputs | CC1, CC2 (through 10 kΩ; sampled at least every 5 ms while configured), VBUS_SENSE (analog watchdog 4.10 V) and four slow DC-sense inputs, one per leg (DC_SENSE_L+, DC_SENSE_L−, DC_SENSE_R+, DC_SENSE_R−: 4 more ADC pins) | 3.1; CC monitoring per 5.2; slow DC tier per 3.7 and 5.2 (ADC ratiometric to 3V3M; zero of each channel calibrated at the end-of-line test) |
| Audio link and outputs toward the CPLD | SPI with chip-select framing: MCU SPI master with DMA, one CS frame per block of samples; 384 kHz / 32-bit stereo = 24.576 Mbit/s: confirm the CH32V307 SPI maximum clock in Stage 1. I²S is not used for the link (its frame rate is not locked to the audio rate). All outputs toward the CPLD low or high-impedance while 3V3D is off | The CPLD captures the link in the SPI clock domain and hands 32-bit words to the 80 MHz domain through the toggle synchronizer; the 80 MHz system clock never samples the MCU serial clock (3.3). No back-powering through the CPLD pins |
| I²C | SCL, SDA to the DAC; R207, R208 2.2 kΩ pull-ups to 3V3D | Pull-ups switch off with the DAC domain |

## 3.3 CPLD and clock

| Item | Requirement / value | Basis / notes |
| --- | --- | --- |
| CPLD | U202 AGM AGRV2KQ32, 2K LUTs, QFN-32 4 × 4 mm (leadless), on 3V3D; our own logic (Section 5): SPI link capture with CS framing, dual-clock FIFO, I²S output engine (SDATA and LRCLK change on the falling BCLK edge), PLL ×2 on the selected family oscillator with integer division to BCLK/LRCLK for all 8 rates, mutually exclusive oscillator enables OSC48_EN/OSC44_EN, SPI registers | R2; C18208899, LCSC stock 980. I/O state and weak pull-up during configuration to check (7.1) |
| Decoupling | C213–C216 100 nF | — |
| Oscillator X201 | SCTF SX3M80.000B20F30TNN, 80 MHz 3.3 V CMOS, 3.2 × 2.5 mm, ±20 ppm tolerance / ±30 ppm stability, 40 mA max, duty 45–55 % (C5179978, Extended, LCSC stock 2,503), on 3V3D; C217 100 nF; tri-state pin 1 tied to 3V3D (always enabled) | DAC MCLK (never switched) and the CPLD system clock (SPI registers, link hand-over, FIFO write side). SCTF 3C datasheet (R14). Replaces the 100 MHz part (C4353707), which sat at the ES9018K2M limit |
| Oscillator X202 | YXC OT322524.576MJBA4SL, 24.576 MHz 3.3 V CMOS, 3.2 × 2.5 mm, ±10 ppm tolerance, ±20 ppm stability (C2831388), on 3V3D; C218 100 nF; R210 33 Ω into the shared node; OSC48_EN with R212 2.2 kΩ pull-down (off by default) | 48 kHz family. YXC YSO110TR datasheet: pin 1 tri-state (enable ≥ 70 % VDD or floating, disable ≤ 30 % VDD), duty 45–55 %, start-up ≤ 3 ms, 5 mA max; no disable time specified. JLCPCB class not shown, assumed Extended; the assembly-fixture sentence on its JLCPCB page is standard text on every part page, not a part-specific requirement (Section 8) |
| Oscillator X203 | SCTF SX2M22.5792M20F30TNN, 22.5792 MHz 1.62–3.63 V CMOS, 2.5 × 2.0 mm, ±20 ppm tolerance, ±30 ppm stability (C5452663, Extended), on 3V3D; C219 100 nF; R211 33 Ω into the shared node; OSC44_EN with R213 2.2 kΩ pull-down (off by default) | 44.1 kHz family. SCTF 2CM datasheet (R13): pin 1 “Tri-State”, enabled when OE is logic 1 or open, disabled when logic 0 (≤ 0.3 VDD); start-up ≤ 5 ms; 10 mA max; duty 45–55 %; no disable/enable time, internal pull-up value or disabled-output leakage specified (measured on Stage 2) |
| Clock routing | X201 → 33 Ω → DAC MCLK (direct, short) and X201 → R209 33 Ω → CPLD system-clock input; X202 → R210 and X203 → R211 into one node at the CPLD’s dedicated PLL clock input; R203–R206 33 Ω are the series resistors on MCLK (to the DAC), BCLK, LRCLK and SDATA | The 33 Ω resistors are source terminations, not contention protection. Both family oscillators and the shared node next to the CPLD PLL clock pin, short traces; the idle oscillator’s trace kept short |
| Shared-node pull-down | R214 10 kΩ from the shared family-clock node to GND | Defines the node low while both family oscillators are off |
| Fallback mux (DNF) | U205 Nexperia 74LVC1G157GW,125 (C135822, SOT-363, Extended) 2:1 mux and R215 33 Ω at the PLL pin, both DNF; select = OSC44_EN; R216, R217 10 kΩ pull-downs on both U205 inputs (DNF, fitted with U205) | To use it: fit U205, R215, R216 and R217 and remove R210 and R211; with R210/R211 removed a disabled oscillator output is high-impedance, so each mux input needs its pull-down. One of the two Stage 2 boards is built with the mux fitted |
| Oscillator enables | The CPLD drives OSC48_EN (X202) and OSC44_EN (X203); R212 and R213 are 2.2 kΩ pull-downs, so both family oscillators are off at power-up, during CPLD configuration or reprogramming and whenever the CPLD pins are high-impedance; the enables are mutually exclusive in logic and never high in the same cycle; after configuration the CPLD starts with both low and the PLL in reset, then enables X202 after ≥ 1 ms | 2.2 kΩ keeps pin 1 below 0.3 VDD against any combined pull-up ≥ 5.1 kΩ (the oscillators’ internal pull-ups of unknown strength and the CPLD weak pull-up during configuration); 1.5 mA per enable when driven high. Same 3V3D domain as the oscillators, so no back-powering when 3V3D is off. Only one family oscillator drives the node at a time; a disabled output-enable oscillator may keep running internally (both counted in the current budget) |
| DAC MCLK limits | Normal (asynchronous) mode at 80 MHz: cycle 12.5 ns (≥ 10 ns); pulse width ≥ 5.6 ns at 45:55 (≥ 4.5 ns); duty 45–55 % (DAC 45:55 to 55:45); 80 MHz ≈ 208 × 384 kHz (208.3; asynchronous mode needs no integer ratio), above the 192 × FSR minimum (73.7 MHz at 384 kHz); external 1.3 V DVDD still required (FSR > 192 kHz) | R3 “XI / MCLK Timing”. The previous 100 MHz part sat at the cycle-time limit (R5-A3, R5-B2) |
| DAC audio-interface timing | DATA_CLK (BCLK) duty cycle 45:55 to 55:45; pulse width high and low ≥ 4.5 ns; cycle ≥ 10 ns; DATA setup 4.1 ns and hold 2 ns to the DATA_CLK rising edge; the CPLD changes SDATA and LRCLK on the falling BCLK edge (≥ 20 ns half period at 24.576 MHz BCLK against 4.1 ns setup / 2 ns hold) | R3 p. 30. One fixed MCLK cannot give 45:55 bit clocks for both families (the earlier single 100 MHz clock gave 40:60 at 352.8/384 kHz), hence the family oscillators |
| Bit clocks | CPLD PLL ×2 → 49.152 MHz (X202) or 45.1584 MHz (X203) → BCLK = PLL ÷ 2/4/8/16 → 384/192/96/48 kHz or 352.8/176.4/88.2/44.1 kHz; LRCLK = BCLK/64 | All 8 rates are exact integer clock ratios with an ideal 50 % bit-clock duty (table below); the PLL ×2 also turns the oscillators’ 45–55 % duty into a 50:50 bit clock. Absolute rates follow the oscillator tolerance (±20 ppm tol. + ±30 ppm stab. for X203), which the asynchronous USB feedback absorbs |
| Family change | Break-before-make: (1) DAC soft mute; (2) I²S output stopped (held low); (3) PLL held in reset; (4) old enable low; (5) wait ≥ 1 ms with both off; (6) new enable high; (7) wait ≥ 10 ms (start-up ≤ 3 ms X202, ≤ 5 ms X203); (8) release the PLL, wait for lock; (9) restart the output; the MCU checks the LRCLK rate with its feedback timer (within ±0.1 % of the target, otherwise stay muted, retry once, then report a fault); (10) DAC re-locks (lock_status, register 64) → soft unmute. Never change both enable bits at the same time; MCLK (X201) is never touched | The datasheets give no disable time; typical oscillator disable times are well under 1 µs, so 1 ms is a large margin; the disable time is measured in the proof of concept. No clock multiplexer in the CPLD fabric (R5-A2, R5-B6, R5-C1) |
| Clock path | No PLL in the DAC MCLK path; the CPLD PLL only makes the bit clock (both families), whose jitter the DAC’s DPLL rejects | Answers the PLL-jitter concern (R4-B M3) |
| MCU link capture | The CPLD captures the MCU audio link (SPI with chip-select framing, MCU master; one CS frame per block of samples) in the SPI clock domain, with the SPI clock on a CPLD global-clock-capable input, and hands each complete 32-bit word to the 80 MHz system domain through a toggle synchronizer (word period ≥ 1.3 µs at 384 kHz, ≫ 3 system-clock cycles of 12.5 ns); the FIFO stays dual-clock (written in the system domain, read in the PLL bit-clock domain, Gray-coded pointers) | The 80 MHz system clock never samples the MCU serial clock (R5-C10); I²S is not used for the link, because its frame rate is not locked to the audio rate. Global clocks used: X201, PLL output, SPI clock (the AGRV2K has up to four) |
| JTAG | J202 pads: 3V3, TCK, TMS, TDI, TDO, GND | CPLD programming |
| Timing | CPLD timing at 80 MHz (system clock) and at the PLL output (49.152 MHz), including the FIFO crossing and the link hand-over between the clock domains | Part of the proof of concept (Section 5) |

Bit clock by sample rate (BCLK = 64 × fs = PLL output ÷ 2/4/8/16; every rate is an exact integer clock ratio; integer division sets the nominal duty, while PLL jitter and output timing are verified on the bench; the DAC requires a duty cycle of 45:55 to 55:45):

| Sample rate | BCLK | Source | Ideal duty | Within 45:55? |
| --- | --- | --- | --- | --- |
| 48 kHz | 3.072 MHz | 49.152 MHz (24.576 MHz, CPLD PLL ×2) ÷ 16 | 50 % | Yes, integer division |
| 96 kHz | 6.144 MHz | 49.152 MHz ÷ 8 | 50 % | Yes, integer division |
| 192 kHz | 12.288 MHz | 49.152 MHz ÷ 4 | 50 % | Yes, integer division |
| 384 kHz | 24.576 MHz | 49.152 MHz ÷ 2 | 50 % | Yes, integer division |
| 44.1 kHz | 2.8224 MHz | 45.1584 MHz (22.5792 MHz, CPLD PLL ×2) ÷ 16 | 50 % | Yes, integer division |
| 88.2 kHz | 5.6448 MHz | 45.1584 MHz ÷ 8 | 50 % | Yes, integer division |
| 176.4 kHz | 11.2896 MHz | 45.1584 MHz ÷ 4 | 50 % | Yes, integer division |
| 352.8 kHz | 22.5792 MHz | 45.1584 MHz ÷ 2 | 50 % | Yes, integer division |

## 3.4 DAC and its supplies (ES9018K2M)

| Pin / function | Connection | Requirement (R3) / note |
| --- | --- | --- |
| Mode | U301 ES9018K2M (C2688377, LCSC stock 724) in normal (asynchronous) mode: DPLL / jitter eliminator | Datasheet DNR/THD figures are specified in this mode (test condition MCLK 27 MHz, asynchronous) |
| DVCC | 3V3D through FB301 Sunlord GZ1608D601TF (C1002: 600 Ω at 100 MHz, 200 mA, 0.45 Ω) + 100 nF | +1.8 V or +3.3 V ±5 %; 13–14.2 mA |
| VCCA | 3V3A (LP5907) + 100 nF | +3.3 V ±5 %; 0.8 mA |
| AVCC_L, AVCC_R | 3V3A via R301, R302 2.2 Ω on separate branches (≈ 18 mV drop at 8 mA); C303, C304 2.2 µF + 100 nF each | +3.3 V ±5 %; 8 mA |
| DVDD | External 1.3 V from U303 Microne ME6216A13M3G (C236662: ±1 %, 300 mA, ceramic-stable, 6 µA Iq) fed from 3V3D; C311 1 µF in, C312 1 µF out; C310 2.2 µF + 100 nF at the pin | External +1.3 V ±5 % must be used for PCM when MCLK > 50 MHz or FSR > 192 kHz; ≥ 1 µF effective (2.2 µF X5R recommended). DVDD ≤ DVCC during ramps. LCSC stock ≈ 410: reserve |
| LP5907 | U302 LP5907MFX-3.3 (C80670) from 5V_SYS; EN = AVCC_EN with R303 100 kΩ pull-down, pulled low by Q611 when the latch is set; C301, C302 1 µF | Total output capacitance 5.7 µF (C302 + C303/C304 + 3 × 100 nF) ≤ 10 µF (R6); 230 Ω output automatic discharge when disabled: 3V3A below 0.3 V within ≈ 3.2 ms (2.5) |
| MCLK | 80 MHz from X201 through 33 Ω, direct; never switches (both rate families) | “XI / MCLK Timing”: cycle ≥ 10 ns (12.5 ns), pulse width high and low ≥ 4.5 ns (≥ 5.6 ns at 45:55), duty 45:55 to 55:45; > 192 × FSR (80 MHz ≈ 208 × 384 kHz); external DVDD required above 192 kHz |
| BCLK, LRCLK, SDATA | From the CPLD through 33 Ω; 3.3 V logic; SDATA and LRCLK change on the falling BCLK edge; held low while the rate family changes | 32-bit I²S; DATA_CLK duty cycle 45:55 to 55:45, pulse width ≥ 4.5 ns, setup 4.1 ns, hold 2 ns (R3 p. 30; 3.3) |
| I²C | SCL, SDA to the MCU; pull-ups on 3V3D; ADDR strap R306 10 kΩ | Address 0x90 |
| RESETB | R305 100 kΩ pull-down; MCU GPIO | Held low until ≥ 1 ms after all supplies and both clocks are stable; in the controlled power-down driven low together with AVCC_EN and AUD_EN (DAC first off) |
| Outputs | DACL, DACLB, DACR, DACRB in differential current mode, each only to its I/V inverting input (3.5.1) | ESS application circuit (R3 p. 27). Absolute maximum: “DAC Output voltage Range (DACL, DACR, DACLB, DACRB): GND < Vout < AVCC”; the I/V converters hold the pins at VREF, which tracks AVCC (3.5.1), so the pins stay inside GND…AVCC in every controlled transition and in the latch shutdown |
| Output model | Each pin is a voltage source swinging 3.05 Vpp (0.924 × AVCC) around its centre voltage behind 806 Ω ±11 %; unloaded differential full scale 2.156 Vrms. The datasheet gives two centre-voltage values: 1.702 V (current-output formula 2.112 mA − 1000·Vg/806 = 0) and 1.65 V (voltage-source model, AVCC/2) | R3 p. 31, note 1. The design uses VREF = 1.703 V; R433 1 kΩ → 0 Ω sets VREF = 1.650 V, decided after the VREF null test on the proof boards (offset in 3.5.6) |
| Master trim | 32-bit master trim sets the calibrated level (3.5.4) | Value stored in MCU flash |
| Lock status | DPLL lock on GPIO / register 64 (lock_status), checked before unmute after a rate or family change | Section 5 |
| Exposed pad | DGND | — |

## 3.5 Output stage: I/V converters and legs (2 × OPA2210 + 2 × OPA1622)

Each DAC output pin drives its own I/V converter: one OPA2210 amplifier holds the pin at VREF and turns its current into a voltage. Each leg is one OPA1622 amplifier wired as a 4-resistor difference amplifier fed by two I/V outputs: leg+ = I/V(DAC−) − I/V(DAC+), leg− the swap. U403 serves DACL and DACLB, U404 serves DACR and DACRB; U401 carries L+ and L−, U402 carries R+ and R−.

### 3.5.1 I/V converters (U403, U404)

| Item | Value | Basis / notes |
| --- | --- | --- |
| Converters | U403, U404 TI OPA2210IDR (C1849415, SOIC-8, same dual pinout) on +3.78 V / −3.72 V, one amplifier per DAC pin | OPA210 datasheet (R15): single input pair, common-mode range (V−) + 1.5 V to (V+) − 1.5 V → upper limit 2.17 V at the minimum +rail 3.673 V, 0.47 V above VREF; Vos ±35 µV max, 2.2 nV/√Hz, 18 MHz, 2.2 / 2.5 mA per channel; input current 10 mA (absolute maximum ratings and section 7.3.5). Replaces the OPA2197, whose input-pair transition region (typically 0.78–2.28 V on +3.78 V) contains VREF (R10). No qualified second source: 12 reserved (Section 8) |
| Inputs | Inverting input = the DAC pin (its only load, short trace); non-inverting input = VREF | ESS application circuit (R3 p. 27) |
| Feedback resistor | Rf = R423–R426 1 kΩ ∥ R434–R437 5.1 kΩ = 836 Ω, 1 % | Basic values. Per leg L = (Rf/Rs) × 2.156 Vrms = 2.24 Vrms nominal before calibration |
| Feedback capacitor | R427–R430 10 Ω in series with C417–C420 ∥ C427–C430 (two 330 pF C0G = 660 pF per converter) | ≈ 288 kHz with 836 Ω. No Basic C0G above 330 pF; X7R is unsuitable in the signal path |
| VREF | R431 15 kΩ over R432 15 kΩ + R433 1 kΩ from 3V3A → 1.703 V (0.516 × 3V3A through the 7.74 kΩ divider); C421 330 pF C0G at the divider (τ ≈ 2.8 µs); one shared node with short traces to all four non-inverting inputs; C422 and D401 removed; R433 0 Ω option → 1.650 V (3.4) | VREF tracks AVCC: lag ≤ 4 mV in controlled ramps (3V3A falls through the LP5907’s 230 Ω output discharge, R6) and ≤ 0.15 V for a few microseconds at the fastest unplug slope (0.1 V/µs), below the voltage at which a silicon protection junction conducts; bench check TP709 VREF against TP710 AVCC_L. 1.703 V sits at the ESS current-output null (1.702 V). VREF noise (LP5907 < 6.5 µVrms × 0.516) is common mode; residual ≈ 0.2 µVrms per leg through the 3 % pair-gain mismatch, negligible |
| Decoupling | C423–C426 100 nF on V+ and V− of U403 and U404 | — |
| Output swing | At the calibrated level ±1.24 V peak around VREF (0.47 … 2.94 V); untrimmed highest-gain unit at 0 dBFS ±1.83 V (−0.13 … 3.53 V); 2.92 V peak with the −3.5 dB pre-calibration default | OPA210 datasheet (R15): (V+) − 0.2 V at RL = 10 kΩ and (V+) − 0.6 V at 600 Ω (±15 V). The I/V output load is ≈ 3 mA → estimate (V+) − 0.3 V → 3.37 V at the minimum rail: calibrated peak 2.94 V (0.43 V margin). An untrimmed highest-gain unit at 0 dBFS would reach 3.53 V and clip, so the −3.5 dB pre-calibration default is a safety setting, always active until a calibration is stored. SPICE item |
| Back-powering | Latch trip: 3V3A collapses within ≈ 3.2 ms, about when the I/V rails (≈ 13 mA load with the amplifiers off) fall to the OPA2210 minimum supply (≈ 3.6 ms); any residual back-power ≤ 1.7 mA per DAC pin (806 Ω − 11 %). Controlled sequences: the DAC is off before the rails, so there is no rails-off, DAC-powered window, and VREF tracks AVCC, so the I/V converters cannot pull the pins above AVCC | Inside the OPA2210 10 mA input-current rating (R15: absolute maximum ratings and section 7.3.5); ES9018K2M absolute maximum GND < Vout < AVCC (3.7.1) |
| Stability | DAC pin capacitance on the OPA2210 inverting inputs with the 836 Ω ∥ (10 Ω + 660 pF) feedback | SPICE before layout (open item) |

### 3.5.2 Output legs (U401, U402)

| Item | Value | Basis / notes |
| --- | --- | --- |
| Amplifiers | U401, U402 TI OPA1622IDRCR (C2876559) on +3.78 V / −3.72 V; one amplifier per leg; thermal pad to V− copper | Isc +145/−130 mA typ (R4) |
| Resistors | R401–R416 YAGEO RT0603BRD072KL 2.00 kΩ 0.1 % 25 ppm thin film (C728588, Extended); gain 1 | Worst 4-resistor mismatch 0.2 % (common-mode leakage term in 3.5.6) |
| Filter | C401–C408 330 pF C0G across each feedback and ground-leg resistor | ≈ 241 kHz with 2 kΩ; 330 pF is the largest Basic C0G |
| Input mapping | leg+ = I/V(DAC−) − I/V(DAC+); leg− the swap | Each I/V inverts, so leg+ is in phase with DAC+ − DAC− |
| Input common mode | VREF ≈ 1.70 V → non-inverting node ≈ 0.85 V DC | Inside the OPA1622 range (V−)+1.5 V to (V+)−1 V, i.e. −2.22 V to +2.78 V on the nominal rails |
| Output link | R417–R420 0 Ω per leg; the hardware DC sense and the slow DC-sense network (3.7) are taken at the jack side of each link | Fit 0.47 Ω only if the capacitive-load test needs it; removing one link allows the detector test and the failed-output emulation (3.8) |
| Enable | AMP_EN to both EN pins: R421 100 kΩ pull-down, R618 10 kΩ from the MCU; pulled low by Q608 when the latch is set (≈ 35 µs) and by the slow interlock Q609 while the rails are off (on 6–32 ms after PGOOD goes bad) | EN threshold ≈ 0.82 V referred to the OPA1622 GND pin; Q609 also keeps EN below the input limit whenever the rails are off, including after a firmware clear |
| Supplies | C409–C412 100 nF and C413–C416 10 µF on V+ and V− | — |
| Swing model | Vpk ≤ (¦rail¦min − 0.9 V)/(1 + 8 Ω/Rleg) at the worst-case rail ¦rail¦min = 3.618 V; clean envelope ≤ 80 mA peak per leg | R4 Fig. 38/39, typical, 25 °C: rail − 0.8…0.9 V at light load, falling ≈ 8 Ω × Iout; current limit ≈ 110–120 mA at 85 °C die. Extrapolated to the ±3.8 V rails — bench-verify |
| Jack ESD | D701–D704 GOODWORK LESD5D5.0CT1G (C41399463): Vrwm 5 V, 12 pF, one per jack leg | Legs swing ≤ ±3.8 V |
| Output impedance | < 0.1 Ω plus jack contacts (legs driven directly) | Target ≤ 0.5 Ω on both jacks (≤ 1.0 Ω balanced if the 0.47 Ω option is fitted) |
| Ground layout | Unbalanced crosstalk −75 dB at 32 Ω needs ≤ 5.7 mΩ shared ground | Dedicated sleeve return, ≥ 4 vias, measured at the jack |

### 3.5.3 Jack pinout

| Signal | 4.4 mm 5-pole (J701, G-Switch GT-3321667P-01, through-hole) | 3.5 mm (J702, HRO PJ-332A-6A, TRRS body used with TRS plugs) |
| --- | --- | --- |
| L+ | Tip | Tip |
| L− | Ring 1 | Not connected (leg+ only) |
| R+ | Ring 2 | Ring 1 |
| R− | Ring 3 | Not connected (leg+ only) |
| GND | Sleeve | Sleeve and ring 2 (tie both to GND) |

Both jacks are live; use one headphone at a time. JLCPCB marks J702 as assembly difficulty “high”.

### 3.5.4 Level and calibration

| Parameter | Value | Basis |
| --- | --- | --- |
| DAC source | 3.05 Vpp around 1.65 V behind 806 Ω ±11 % per pin; unloaded differential 2.156 Vrms | R3 p. 31, note 1 |
| Untrimmed level per leg | L = (Rf/Rs) × 2.156 Vrms = 2.236 Vrms nominal; unit spread 1.955–2.589 Vrms | Includes Rs ±11 %, Rf 1 % and AVCC (LP5907) ±2 % |
| Calibrated full scale | 1.75 Vrms per leg (2.475 V peak): 3.5 mm 1.75 Vrms, 4.4 mm 3.5 Vrms | Master trim −0.96 dB (lowest-gain unit) to −3.40 dB (highest), nominal −2.13 dB |
| Before calibration | Firmware applies −3.5 dB → ≤ 1.73 Vrms on any unit; 2.92 V peak at the I/V output of the highest-gain unit | Safety setting, always active until a calibration is stored: without it the highest-gain unit at 0 dBFS would reach 3.53 V at the I/V output and clip (limit ≈ 3.37 V, 3.5.1) |
| Channel trim | ±0.05 dB per channel by MCU digital scaling | Stored with the master trim |
| Calibration | Automated end-of-line test (QA403): master trim to 1.75 Vrms ± 0.1 dB and per-channel fine trim, written to MCU flash | 3.8, Section 5 |

### 3.5.5 Output power (High and Default mode)

Both channels driven, 1 kHz, at the calibrated level and the worst-case rail (|rail|min = 3.618 V); rated at 1 % THD+N or calibrated 0 dBFS, whichever comes first. The limiting factor is in brackets. Default mode is rated by the instantaneous peak current, because the TPS2553 limits instantaneous current: peak analog input = quiescent (53 mA max) + 2 × Ipk (unbalanced, both channels on one rail) or + 2 × Ipk × (1 + 1.03) (balanced, both rails) ≤ 157 mA (Default minimum). Estimates: the OPA1622 swing at the ±3.8 V rails is extrapolated, so the table is bench-verified.

| Load | High mode (limit) | Default mode (limit) |
| --- | --- | --- |
| 3.5 mm 16 Ω | 51.2 mW (80 mA envelope) | 21.6 mW (Default limit) |
| 3.5 mm 32 Ω | 73.9 mW (swing) | 43.1 mW (Default limit) |
| 3.5 mm 60 Ω | 47.9 mW (swing) | 47.9 mW (swing) |
| 3.5 mm 300 Ω | 10.2 mW (full scale) | 10.2 mW (full scale) |
| 4.4 mm 32 Ω | 102.4 mW (80 mA envelope) | 10.5 mW (Default limit) |
| 4.4 mm 60 Ω | 153.5 mW (swing) | 19.6 mW (Default limit) |
| 4.4 mm 120 Ω | 95.9 mW (swing) | 39.2 mW (Default limit) |
| 4.4 mm 300 Ω | 40.8 mW (full scale) | 40.8 mW (full scale) |
| 4.4 mm 600 Ω | 20.4 mW (full scale) | 20.4 mW (full scale) |

High-mode peaks stay below the 393 mA High minimum (worst 378 mA at 4.4 mm 32 Ω). At 120 Ω balanced (K1000) and 3.5 mm 60 Ω the worst-case unit reaches the swing limit 0.27 dB below full scale (nominal unit +0.05 dB), so those loads are rated as swing-limited and bench-verified. Proposed power targets (worst unit, both channels, same rating method): High mode — 3.5 mm ≥ 65 mW at 32 Ω; 4.4 mm ≥ 140 mW at 60 Ω, ≥ 85 mW at 120 Ω, ≥ 36 mW at 300 Ω. Default mode — 3.5 mm ≥ 38 mW at 32 Ω; 4.4 mm ≥ 17 mW at 60 Ω, ≥ 35 mW at 120 Ω, ≥ 36 mW at 300 Ω; 32 Ω balanced is not rated in Default mode.

SPL: HIFIMAN Susvara (60 Ω, 83 dB/mW) ≈ 104.9 dB SPL in High mode and ≈ 95.9 dB SPL in Default mode; AKG K1000 (120 Ω, 74 dB/mW) ≈ 93.8 dB SPL in High mode and ≈ 89.9 dB SPL in Default mode.

### 3.5.6 Offset, noise and frequency response

| Offset contribution per leg (25 °C, datasheet maxima) | Value |
| --- | --- |
| Common-mode leakage: VREF 1.703 V × 0.2 % (0.1 % resistors, gain 1) | 3.40 mV |
| OPA1622 Vos 0.5 mV × 2 | 1.00 mV |
| OPA1622 Ios × 1 kΩ × 2 | 0.10 mV |
| OPA2210 Vos 35 µV × noise gain ≤ 2.17 × 2 (two amplifiers) | 0.15 mV |
| DAC pin-to-pin DC mismatch ≤ 1 mV (assumption, measure) × Rf/Rs | 1.18 mV |
| VREF-to-null error (divider tolerance and the assumed ±20 mV null-point uncertainty) × 3 % pair-gain mismatch | 1.17 mV |
| Unbalanced total; requirement ≤ ±10 mV | ≤ 7.0 mV worst (≈ 3.9 mV RSS); margin 3.0 mV |
| Balanced total; requirement ≤ ±20 mV | ≤ 14.0 mV worst; margin 6.0 mV |

Every board is measured by the automated test (100 % offset screening with a DC-capable DMM, because the QA403 inputs are AC-coupled); the DAC-mismatch assumption stays open until measured. If the DAC null is 1.65 V (ESS voltage-source model, 3.4) instead of 1.702 V, the VREF-to-null term is 2.8 mV and the totals become ≤ 8.6 mV unbalanced and ≤ 17.2 mV balanced, still inside ±10 / ±20 mV; R433 0 Ω then moves VREF to 1.650 V.

| Parameter | Estimate | Basis |
| --- | --- | --- |
| I/V output noise | ≈ 6.95 nV/√Hz | OPA2210 2.2 nV/√Hz × 2.04 plus Rf and DAC-source thermal noise |
| Leg noise | ≈ 13.9 nV/√Hz unbalanced (leg-network own noise 9.85 nV/√Hz); ≈ 24.1 nV/√Hz balanced | Balanced: both legs share the same two I/V converters (leg+ = I/V(DAC−) − I/V(DAC+), leg− the swap), so the I/V noise doubles with the signal and only the leg-network noise adds as √2: +0.85 dB, not +3 dB |
| Dynamic range, analog only | ≈ 120.9 dB(A) unbalanced | Estimate |
| Dynamic range, system | ≈ 119.5 dB(A) unbalanced, ≈ 120.3 dB(A) balanced | With the DAC’s 127 dB(A), which is also common to both legs (correlated model). With the OPA2197 the correlated figures would have been 117.0 / 117.4 dB(A), and worse in its transition region. Target ≥ 115 dB(A), minimum ≥ 112 dB(A); measured results are recorded with the analyzer loopback floor |
| I/V pole | 288 kHz | 836 Ω with 660 pF |
| Leg pole | 241 kHz | 2 kΩ with 330 pF |
| Frequency response | −0.05 dB at 20 kHz, −0.20 dB at 40 kHz | Both poles |

## 3.6 Power (ME6211 × 2, TPS2553, LM27762)

| Item | Value | Basis / notes |
| --- | --- | --- |
| 3V3M LDO | U502 Microne ME6211C33M5G-N (C82942), always on; C501, C502 1 µF | ±2 %, 100 mV dropout at 100 mA |
| 3V3D LDO | U504 ME6211C33M5G-N, CE = AUD_EN with R508 100 kΩ pull-down, fed from 5V_SYS through R519 + R520 (2 × 2.2 Ω 0805 in series, 4.4 Ω); C503 1 µF input; C504 1 µF and C512 10 µF bulk at the output | Same part type as U502; the dropper keeps U504 ≤ 0.20 W at VBUS 5.25 V (0.27–0.31 W without it, against the 300 mW SOT-23 rating, R17; 3.6.3); LDO input ≥ 3.71 V at VBUS 4.40 V and 157 mA; C512 is the 3V3D bulk for the CPLD, the 80 MHz oscillator and the 1.3 V LDO input |
| 1.3 V LDO | U303 ME6216A13M3G from 3V3D | 3.4 |
| Current-limited switch | U503 TI TPS2553DBVR (C55266): EN = ANA_EN with R513 100 kΩ pull-down; FAULT → MCU (ANA_FAULT, open-drain-capable pin) with R512 100 kΩ pull-up to 3V3M, and to the latch through D606; firmware can drive ANA_FAULT low to set the latch (3.7); C505, C506 1 µF | rDS(on) 85 mΩ typ / 135 mΩ max; built-in soft start; FAULT open drain with 5/7.5/10 ms deglitch; the limit acts on instantaneous current (short-circuit response ≈ 2 µs, then constant-current limiting) (R9). The auto-retry version stays; the latch-off TPS2553-1 is listed “not recommended for new designs”, and our latch gives an ordered shutdown |
| Current-limit resistors | RILIM = R509 100 kΩ + R514 47 kΩ = 147 kΩ (Default); R510 100 kΩ switched in parallel by Q501 2N7002 (gate = ILIM_HI, R511 100 kΩ pull-down) → 59.5 kΩ (High) | R9: IOS max = 22980/R^0.94, nom = 23950/R^0.977, min = 25230/R^1.016 (mA, R in kΩ, 15–232 kΩ); two-level limit per datasheet Figure 24. Keep the RILIM trace short |
| Ferrite | FB501 Murata BLM18PG121SN1D (C14709, Basic): 120 Ω at 100 MHz, 2 A, 50 mΩ; 5V_ANA → LM27762 VIN | Drop ≈ 15–25 mV |
| Rail generator | U501 TI LM27762DSSR (C473398): C507 10 µF VIN, C508 1 µF flying, C509 10 µF CP, C510 10 µF OUT+, C511 10 µF OUT− | R5: charge-pump output resistance 2.5 Ω typ at VIN 5.5 V, 100 mA; LDO dropout 30 mV (negative) and 45 mV (positive) at 100 mA; input current limit 500 mA typ; UVLO 2.4 V falling / 2.6 V rising |
| FB+ divider | R501 100 kΩ + R515 100 kΩ + R516 15 kΩ = 215 kΩ over R502 100 kΩ → +3.780 V (3.673–3.890 V incl. VFB and 1 % resistors) | 1.2 V × 315/100; R2 ≥ 50 kΩ |
| FB− divider | R503 100 kΩ + R517 100 kΩ + R518 5.1 kΩ = 205.1 kΩ over R504 100 kΩ → −3.722 V (3.618–3.828 V magnitude incl. VFB and 1 % resistors) | −1.22 V × 305.1/100; R4 ≥ 50 kΩ. R518 was 10 kΩ; 5.1 kΩ restores the charge-pump headroom once the resistor tolerance is included |
| Enable | EN+ and EN− = RAIL_EN: R506 100 kΩ pull-down, R507 10 kΩ from the MCU; pulled low by Q607 when the latch is set | EN high ≥ 1.2 V; EN must not exceed VIN, so RAIL_EN goes high only after 5V_ANA is up; in the controlled power-down RAIL_EN goes low at t0 + 3.5 ms, after the DAC and AMP_EN (2.5) |
| PGOOD | Open drain, active low (low = both outputs ≥ 95 % of target: +3.59 V, −3.54 V); R505 100 kΩ pull-up to 3V3M → RAILS_OK_N to the MCU, the Q610 gate, the Q609 gate through R623/C607 and the blanking RC | In shutdown each output has a 50 kΩ pull-down (not relied on for discharge); PGOOD goes bad at VBUS ≈ 3.70 V during an unplug |
| Rail capacitance | ≈ 30.6 µF per rail | After a trip both rails with C +10 % at the maximum rail voltage hold 0.51 mJ; with ≤ 1 µJ DAC back-power the after-trip bound is ≤ 0.6 mJ (3.7.1) |

### 3.6.1 Current limits

| Mode | RILIM | Limit min / nom / max | Use |
| --- | --- | --- | --- |
| Default | 147 kΩ | 157 / 183 / 213 mA | USB 2.0 hosts, USB-A ports and phones; estimated maximum total 0.469 A with hardware-limited analog current |
| High | 59.5 kΩ | 393 / 442 / 498 mA | Only when CC advertises ≥ 1.5 A; estimated maximum total 0.754 A |

Limits include the 1 % resistor tolerance. The limit acts on instantaneous current, so Default-mode power is rated by the instantaneous peak (3.5.5).

### 3.6.2 Analog supply current

| Jack and load | Analog input current, average (max quiescent) | Instantaneous peak | Within the Default limit (157 mA min) at the peak? |
| --- | --- | --- | --- |
| 3.5 mm 16 Ω | 156 mA | 213 mA | No: Default mode 21.6 mW |
| 3.5 mm 32 Ω | 141 mA | 189 mA | No: Default mode 43.1 mW |
| 3.5 mm 60 Ω | 105 mA | 133 mA | Yes |
| 3.5 mm 300 Ω | 64 mA | 70 mA | Yes |
| 4.4 mm 32 Ω | 260 mA | 378 mA | No: Default mode 10.5 mW |
| 4.4 mm 60 Ω | 238 mA | 343 mA | No: Default mode 19.6 mW |
| 4.4 mm 120 Ω | 156 mA | 215 mA | No: Default mode 39.2 mW |
| 4.4 mm 300 Ω | 96 mA | 120 mA | Yes |
| 4.4 mm 600 Ω | 74 mA | 87 mA | Yes |

### 3.6.3 Regulator and power-part dissipation

Dissipation of every regulator and power part at its maximum load (R6-D3). The ME6211 SOT-23 power dissipation rating is 300 mW (R17), so U504 is fed through the R519 + R520 dropper; the ME6216 rating and the LM27762 θJA are confirmed before layout (7.1).

| Part | Load | Dissipation | Note |
| --- | --- | --- | --- |
| U504 3V3D ME6211, without dropper | 157 mA max | 0.27 W at 5.0 V, 0.31 W at 5.25 V | ME6211 SOT-23 power dissipation rating 300 mW (datasheet absolute maximum) → too close |
| U504 with R519 + R520 (2 × 2.2 Ω 0805 in series, 4.4 Ω) | 157 mA max (103 mA typ) | 0.13 W typ, 0.16 W at 5.0 V, 0.20 W at 5.25 V; 54 mW per resistor (0.125 W rating) | LDO input ≥ 3.71 V at VBUS 4.40 V (≈ 0.25 V dropout needed at 157 mA, estimate) |
| U502 3V3M ME6211 | 84 mA | 0.16 W at 5.25 V | — |
| U303 1.3 V ME6216 | 60 mA | 0.12 W | Pd rating to confirm |
| U302 LP5907 | 17.5 mA | 0.03 W | — |
| U501 LM27762 | Worst clean load (4.4 mm 32 Ω, High) | ≈ 0.22 W | Check against its datasheet θJA at layout |
| U503 TPS2553 | 0.26 A | 9 mW normal; ≤ 2.6 W into a short for ≤ 10 ms before the latch/FAULT path | Its thermal shutdown protects |

3V3D load (maximum): CPLD 25, X201 40, X202 5, X203 10, DVCC 14.2, DVDD 60 (through U303), enables and node 1.7, I²C 1.5 mA. C512 10 µF 0805 at the U504 output is the 3V3D bulk (R6-D8). Layout rule: copper pours on the LDO VIN, GND and VOUT pins; IR or thermocouple check at bring-up.

## 3.7 Protection

Protection has a fast hardware tier and a slow firmware tier, and both end in the same fault latch. The hardware tier works without firmware: the DC window detector, a PGOOD-bad request (Q610) and a sustained TPS2553 overcurrent (D606) pull node F low, and the latch then pulls RAIL_EN, AMP_EN and AVCC_EN low through N-MOSFET kill switches (Q607, Q608, Q611) within ≈ 35 µs. The firmware tier reads each leg through its own ADC network and sets the latch through the ANA_FAULT pin. The latch stays set until firmware clears it (FLT_CLR); firmware captures the cause and applies the recovery policy (3.7.2, Section 5). All transistors are JSCJ 2N7002 (C8545, Basic), Q601–Q611.

| Item | Value | Basis / notes |
| --- | --- | --- |
| DC sensing (hardware) | Each leg, taken at the jack side of its 0 Ω output link (R417–R420) → R601–R604 100 kΩ with C601–C604 1 µF (τ = 100 ms; 84–111 ms with 1 % R and C −15 %/+10 % for tolerance and DC bias) → D601–D604 BAV99 into the MAX and MIN nodes (R605, R606 1 MΩ loads) | One filter per leg; sensing at the jack side allows the detector test and the failed-output emulation (3.8) |
| Comparator | U601 onsemi LM393DR2G (C7955, Basic) on the ±3.8 V rails; thresholds R607, R608 100 kΩ over R609, R610 10 kΩ from the rails → node threshold ≈ ±0.34 V (0.323–0.360 V worst case, 0.341 V nominal) | Leg trip 0.47–0.81 V worst case (0.635 V nominal); the thresholds follow the rails |
| Worst-case inputs | Rails 3.673–3.890 V / 3.618–3.828 V; divider resistors 1 %; LM393 input offset ±9 mV (over temperature); BAV99 forward voltage at ≈ 0.4 µA 0.12–0.40 V over 0–50 °C and unit spread; diode load current through the filter resistor; C601–C604 −15 %/+10 % (tolerance and DC bias); node F delay through R611 into C605 (≤ 1.33 ms) and latch capture (35 µs) | BAV99 VF is an estimate from VF(1 mA) ≤ 0.715 V with n ≈ 2, measured on the proof boards (3.8) |
| Node F | LM393 open-collector outputs (wired-OR) → R611 10 kΩ → node F; R612 47 kΩ pull-up to 3V3M; C605 100 nF from node F to GND; D605 1N4148WS clamp to GND | F is 3.3 V normally and ≈ −0.7 V on a DC fault (clamp current ≤ 0.31 mA); the 3.3 V domain never sees the −3.8 V comparator output. Power-on set: F starts at 0 V (C605), so the latch sets at attach, independent of the LM27762 PGOOD pin while that part is unpowered |
| Set-request stretch | After any PGOOD or fault pulse node F stays low 1.5–7.4 ms (R612 47 kΩ into C605) | Every PGOOD-bad pulse that turns Q610 on (≥ ≈ 10 µs) is latched |
| PGOOD set request | Q610 2N7002: gate = RAILS_OK_N (LM27762 PGOOD, high = not good), drain = node F, source = GND | PGOOD bad → node F low → set request → the latch sets unless blanked, so an overload that drops PGOOD latches in ≈ 35 µs |
| Overcurrent and firmware set request | D606 1N4148WS: anode = node F, cathode = ANA_FAULT (TPS2553 FAULT, open drain, 5/7.5/10 ms deglitch); the MCU ANA_FAULT pin is open-drain capable, and firmware sets the latch by switching it to open-drain output low (slow DC tier) | Same hardware kill path, no extra part; in normal operation D606 has 0 V across it and ANA_FAULT still reads correctly at the MCU. With ANA_FAULT low, F sits at VF(D606) ≈ 0.5–0.7 V (≈ 55 µA through R612) against the Q601 VGS(th) minimum of 1.0 V: 0.3–0.5 V margin, smaller at low temperature |
| Set path | Q601 inverter (gate = F) drives node S (R613 100 kΩ pull-up): S high = set request; Q602 (gate = S) sets the latch by pulling node A low | — |
| Blanking | Q603 holds S low while its gate is high; gate = PGOOD (RAILS_OK_N) through R617 1 MΩ with C606 100 nF: the set path is armed 25–133 ms after PGOOD goes good and re-blanked 35–173 ms after PGOOD goes bad (charging through R505 + R617) | 2N7002 VGS(th) 1.0–2.5 V, R 1 %, C 10 %. Blocks only the set request; it cannot clear an already-set latch (a clamp on the latch node would clear it and cause a hiccup). At start-up the PGOOD-bad request is blanked |
| Latch | Q604 and Q605 cross-coupled (drain A → gate B, drain B → gate A); R614, R615 47 kΩ pull-ups to 3V3M; no capacitor on the latch nodes; FLT = node B (goes high when the latch sets) | Capture with the 2N7002 capacitances at their maxima and VGS(th) up to 2.5 V: RAILS_OK_N rise to the Q610 threshold ≈ 8.5 µs, node S ≈ 9.9 µs, node B ≈ 14.0 µs, plus the FET transitions → ≈ 35 µs from PGOOD going bad to RAIL_EN, AMP_EN and AVCC_EN pulled low (budget 0.1 ms; measured) |
| Clear | Q606 (gate = FLT_CLR, R616 100 kΩ pull-down) pulls node B low | The latch stays set until firmware clears it; a clear does not stick while a set request is present, so FLT_CLR is given only after the rails have been off ≥ 200 ms (set path re-blanked); the MCU reads FLT directly |
| Kill paths | While the latch is set: Q607 pulls RAIL_EN low, Q608 pulls AMP_EN low and Q611 (gate = node B, drain = AVCC_EN) pulls AVCC_EN low, so the latch also turns off the DAC analog supply (LP5907) | Hardware; no firmware needed. At unplug the VBUS watchdog (4.10 V) also starts the power-down from firmware (2.5) |
| Slow interlock | Q609 2N7002 pulls AMP_EN low; gate from RAILS_OK_N through R623 100 kΩ with C607 100 nF to GND → on 6–32 ms after PGOOD goes bad, off 2.5–13 ms after it goes good | Keeps AMP_EN low whenever the rails are off (also after a firmware clear) but is no longer a fast kill: a short PGOOD glitch either latches or does nothing, so the amplifiers cannot be disabled and re-enabled without the latch |
| Slow DC-sense network (firmware tier) | Per leg at the jack side of R417–R420 → MCU ADC: R624–R627 1 MΩ (leg series), R628–R631 1 MΩ (to 3V3M), R632–R635 1 MΩ (to GND), C608–C611 100 nF → ADC node = 1.1 V + leg/3 (−0.18 … +2.40 V over the full rail range), τ ≈ 33 ms, leg load ≈ 1 µA | 4 more MCU ADC pins; the ADC is ratiometric to 3V3M; the zero of each channel is calibrated at the end-of-line test (uncalibrated error up to ≈ ±35 mV at the leg) |
| Slow DC tiers | ≥ 0.8 V (0.5-s average) → firmware sets the latch within 1 s; ≥ 0.2 V (2-s average) → within 3 s; active only while AMP_EN is high | No false trip for full-scale content above ≈ 2 Hz. Remaining blind region below 0.2 V per leg: ≤ 2.5 mW into 16 Ω, ≤ 1.25 mW into 32 Ω; proposed: accept (owner decision pending, 7.1) |
| No false trip (hardware) | Full-scale content above 9.9 Hz worst case (6.0 Hz nominal) | The earlier ≈ 4–5 Hz figure was nominal |

Worst-case detection envelope of the hardware tier (removal = RAIL_EN, AMP_EN and AVCC_EN pulled low; the detector, the node F delay and the latch capture are included):

| DC on a leg | Removal time, nominal | Removal time, worst case | Requirement |
| --- | --- | --- | --- |
| 1.0 V | 101 ms | 183 ms | ≤ 250 ms (27 % margin) |
| 1.5 V | 56 ms | 87 ms | — |
| 2.0 V | 39 ms | 59 ms | ≤ 100 ms (41 % margin) |
| 2.47 V (full-scale digital DC) | 30 ms | 45 ms | — |
| 3.67–3.89 V (rail-stuck) | 19–20 ms | 27–29 ms | — |

Requirement (hard, worst case): a DC fault ≥ 2 V on any leg is removed within 100 ms and ≥ 1 V within 250 ms. The hardware tier trips at 0.47–0.81 V on a leg (worst case); the firmware tier covers DC down to 0.2 V.

### 3.7.1 Energy at the headphone terminals and back-powering

Terminal energy E = V²/R × (detector + node F + latch time) + the energy after the trip, at the worst case of the envelope above, for a hardware rail-stuck failure in High mode and for a digital DC signal on a healthy output stage:

| Load | Rail-stuck failure, High mode, worst case (nominal) | Digital DC on a healthy output stage, worst case |
| --- | --- | --- |
| 16 Ω | 26.2 mJ (17.9 mJ) | 1.81 V (swing-limited), 66.7 ms, 13.7 mJ |
| 32 Ω | 13.3 mJ (9.2 mJ) | 2.17 V, 52.8 ms, 7.8 mJ |
| 60 Ω | 7.4 mJ (5.1 mJ) | 2.40 V, 46.8 ms, 4.5 mJ |
| 300 Ω | 1.9 mJ (1.4 mJ) | 2.47 V, 45.1 ms, 0.9 mJ |

After the trip, back-powering and the test criterion:

| Item | Value | Basis / notes |
| --- | --- | --- |
| Default mode | In Default mode a rail-stuck fault into 16–32 Ω exceeds the TPS2553 limit, so the PGOOD/overcurrent path latches first | The energy table above is the High-mode worst case |
| After the trip | Rails disabled; both rails’ capacitors (≈ 30.6 µF per rail) with C +10 % at the maximum rail voltage = 0.51 mJ, plus DAC back-power into a failed stage ≤ 1 µJ → after-trip bound ≤ 0.6 mJ: the most a faulty output stage can deliver after the rails are disabled | Included in the energy table. The load discharges it; the LM27762 50 kΩ discharge is not relied on. OPA1622 shutdown is not treated as isolation (TI: “do not consider the amplifier output to be high-impedance”) |
| Back-powering | Latch trip: Q611 kills AVCC_EN with the rails; 3V3A collapses within ≈ 3.2 ms, about when the I/V rails (≈ 13 mA load with the amplifiers off) fall to the OPA2210 minimum supply (≈ 3.6 ms); residual back-power ≤ 1.7 mA per DAC pin (806 Ω − 11 %). Controlled power-down: the DAC turns off first and VREF follows AVCC, so the pins stay inside GND…AVCC; at a sudden unplug at most ≈ 0.15 V above AVCC for microseconds | Inside the OPA2210 10 mA input-current rating (R15: absolute maximum ratings and section 7.3.5); ES9018K2M absolute maximum GND < Vout < AVCC; bench check TP709 VREF against TP710 AVCC_L (3.8) |
| Test criterion | Hard, worst case: a DC fault ≥ 2 V on any leg is removed within 100 ms and ≥ 1 V within 250 ms; energy at the headphone terminals ≤ 30 mJ at 16 Ω and ≤ 15 mJ at 32 Ω for a rail-stuck failure in High mode, including the after-trip energy (worst-case estimates 26.2 / 13.3 mJ) | Coverage: detector test, failed-output emulation with dummy loads and V²/R integrated, digital DC test signal and SPICE of a failed output stage (3.8). Risk kept: up to 26.2 mJ into a 16 Ω IEM in High mode; v2 option: a fast “leg within 0.5 V of a rail” detector (≈ 1 ms) |

### 3.7.2 Thermal and overload

| Case | Estimate | Basis / notes |
| --- | --- | --- |
| Analog input power | ≤ 1.06 W (Default), ≤ 2.49 W (High) | Bounded by the TPS2553 limit |
| Board worst case | ≈ 3.8 W (3.77 W) | High mode, overload |
| 4.4-to-3.5 mm adapter (leg− shorted) | ≈ 0.35 W in the shorted amplifier + ≈ 0.1 W in the other → ≈ 0.45 W per package, +21 °C | θJA 47.6 °C/W; one amplifier per package, inside TI’s continuous-short rating |
| 4.4 mm plug shorted leg+ to leg− | ≈ 0.55 W per package (both amplifiers of one package) | Outside TI’s one-amplifier continuous-short rating → bench survival test |
| Overload response and recovery | Current limit → rails droop → PGOOD bad → Q610 sets the latch (a sustained TPS2553 FAULT sets it through D606) → within ≈ 35 µs the latch kills RAIL_EN, AMP_EN and AVCC_EN and stays set. At the FLT interrupt, before any other action, firmware stores the four leg DC readings (the 33 ms RC holds the pre-trip state), ANA_FAULT, RAILS_OK_N and whether firmware itself set the latch: DC fault if any leg reads ≥ 0.3 V or firmware tripped, otherwise overload; unavailable readings count as a DC fault. Overload: soft mute, volume ceiling −3 dB per event (down to −12 dB), clear after ≥ 1 s (FLT_CLR only after the rails have been off ≥ 200 ms), ceiling kept until replug; overloads count toward the 3-strike stop only once the ceiling has reached −12 dB. DC fault: clear after 2 s; 3 DC faults within 10 s → latched until USB replug; the red LED blinks | Overload latches (no restart when PGOOD recovers) and needs the firmware clear (Section 5). Default mode at full volume into 60 Ω balanced needs three −3 dB steps; the previous rule could lock the board off within ≈ 5 s (R6-C1). FLT during a controlled power-down or before the first FLT_CLR is expected and masked (5.2). TPS2553 FAULT deglitch 5/7.5/10 ms |

## 3.8 Test and DFT

| Item | Provision | Notes |
| --- | --- | --- |
| Test pads | TP701–TP731 (31 copper pads), allocated in the table below: rails, VREF and AVCC_L, clocks and I²S, enables and protection nodes, USB sensing, 4 GND | — |
| Debug | J201 CH32V307 2-wire pads (WCH-LinkE); J202 CPLD JTAG pads | Section 5 |
| I²S header | J703 1 × 6, 2.54 mm, DNF: MCLK monitor output, BCLK, LRCLK, SDATA, GND, 3V3 reference; the MCLK pin only through R703 1 kΩ (DNF, fitted with J703) | In test mode an external source drives only BCLK, LRCLK and SDATA (CPLD I²S outputs tristated) and the DAC stays in asynchronous mode on X201; the QA403 digital output is 3.3 V-class: use series resistors, power the board first |
| Indicators | D705 KT-0805G green (status), D706 KT-0603R red (fault); R701, R702 2.2 kΩ (≈ 0.5–0.7 mA) from 3.3 V GPIO | No Basic 0603 green LED exists |
| Assembly | JLCPCB Economic PCBA, one side; all SMT on top except the 4.4 mm jack (7 through-hole joints); 3 fiducials, 4 × M3 holes; 4 layers, ≤ 80 × 60 mm | 8 leadless parts per board for X-ray: CPLD, DAC, 2 × OPA1622, LM27762, 3 oscillators |

Test-pad allocation (TP701–TP731, 31 pads):

| Pads | Group | Nets (in pad order) | Used by |
| --- | --- | --- | --- |
| TP701–TP708 | Supplies | TP701 5V_SYS, TP702 5V_ANA, TP703 3V3M, TP704 3V3D, TP705 3V3A, TP706 1V3, TP707 +3.78 V, TP708 −3.72 V | Rails and power; TP707/TP708 are the sources for the failed-output emulation |
| TP709–TP710 | DAC pin safety | TP709 VREF, TP710 AVCC_L | DAC pin against AVCC |
| TP711–TP717 | Clocks and I²S | TP711 MCLK, TP712 family-clock node, TP713 OSC48_EN, TP714 OSC44_EN, TP715 BCLK, TP716 LRCLK, TP717 SDATA | Jitter, clock handover |
| TP718–TP724 | Enables and protection | TP718 AMP_EN, TP719 RAIL_EN, TP720 AVCC_EN, TP721 RAILS_OK_N, TP722 FLT, TP723 ANA_FAULT, TP724 node F | Protection tests, short PGOOD pulses, power sequence |
| TP725–TP727 | USB sensing | TP725 VBUS_SENSE, TP726 CC1, TP727 CC2 | USB tests, watchdog latency |
| TP728–TP731 | Ground | TP728, TP729, TP730, TP731: GND | Probe and DMM returns |

End-of-line and bring-up tests:

| # | Test | Method and pass criterion |
| --- | --- | --- |
| 1 | End-of-line calibration | QA403 + scripts: full-scale level per leg → master trim to 1.75 Vrms ± 0.1 dB and per-channel fine trim, written to flash; zero of each slow DC-sense channel calibrated and stored; THD+N; dynamic range, recorded with the analyzer loopback floor. DC offset of every leg (100 %) with a DC-capable DMM (≥ 4½ digits, SCPI over USB for the automated test), because the QA403 inputs are AC-coupled |
| 2 | Detector test | External lab source on one jack contact with its link (R417–R420) removed, at 1.0 V, 2.0 V and 3.8 V; tests detection and the control signals only: FLT high, RAIL_EN, AMP_EN and AVCC_EN low within the worst-case envelope |
| 3 | Failed-output emulation | The board’s own +3.78 V (TP707) or −3.72 V (TP708) is switched onto a jack contact (link removed) by an external MOSFET switch, bypassing the amplifier enable; dummy loads 16 Ω and 32 Ω; scope the terminal voltage and integrate V²/R until it stays below 0.1 V. Pass: FLT high, RAIL_EN, AMP_EN and AVCC_EN low within the worst-case envelope (no allowance beyond the hard 100 ms / 250 ms / energy limits); total energy ≤ 30 mJ (16 Ω) / ≤ 15 mJ (32 Ω). Also: digital DC test signal on a healthy output stage and SPICE of a failed output stage |
| 4 | Short PGOOD pulses | Force RAILS_OK_N high for 5 µs, 20 µs, 100 µs and 1 ms while the MCU keeps requesting AMP_EN: either nothing happens or the latch sets; never an un-latched AMP_EN dropout |
| 5 | Firmware DC tier | 0.25 V and 0.9 V DC from the digital path → latch within 3 s / 1 s; the event is recorded as a DC fault |
| 6 | Overload (30 min each, High mode, maximum volume) | 4.4 mm 16 Ω; 4.4-to-3.5 mm adapter; 3.5 mm 16 Ω; 60 Ω balanced full scale — all parts within ratings; the latch sets and recovery needs the firmware clear (FLT_CLR); the board recovers. Near the Default-mode limit: scope the rails and the output for clipping before a trip |
| 7 | USB | Current in every state (USB power meter), repeating the Stage 2 limits on the full board: attached-not-configured ≤ 100 mA, configured digital ≤ 256 mA; suspend current against time from the last bus activity (≤ 2.5 mA by 10 ms); attach inrush charge; VBUS droop at each domain enable; Default and High mode on a Type-C 1.5 A source and on a USB-A port; a CC change from ≥ 1.5 A to Default handled within tSinkAdj |
| 8 | Pop | Pop = peak of the transient at the jack beyond the settled DC step, band-limited 20 Hz–20 kHz (AC-coupled analyzer input); the DC before and after each transition is bounded separately by the offset limits (±10 / ±20 mV). ≤ 5 mV peak for controlled transitions (power-on after configuration, suspend/resume, rate change, fault recovery); ≤ 20 mV peak at unplug/brownout (target); 32 Ω and 300 Ω |
| 9 | Jitter | J-test for both families (exact integer clock ratios from the CPLD PLL), including 352.8 and 384 kHz; BCLK duty cycle measured at every rate |
| 10 | Clock handover | Scope both enables and the shared node at every family change: never both enabled, node quiet for ≥ 1 ms before the new oscillator starts, disable time measured, no runt pulses, PLL relock, all 8 rates bit-perfect; on the shared-node board and on the mux board (Stage 2), repeated on the main boards |
| 11 | DAC pin against AVCC | TP709 VREF against TP710 AVCC_L at power-up, power-down, latch trip and unplug: inside GND…AVCC in the controlled transitions and the latch trip; at most ≈ 0.15 V above AVCC for microseconds at unplug; recorded together with the unplug pop |
| 12 | Rails and power | Negative-rail margin at VBUS 4.60 V (predicted ≥ 0.11 V worst case); 3V3A below 0.3 V within ≈ 3.2 ms of AVCC_EN low; LDO temperatures by IR or thermocouple (3.6.3); output power tables in both modes on the bench |
| 13 | Stability | Each leg with the 0 Ω links (fit 0.47 Ω only if the capacitive-load test needs it); each I/V output checked for oscillation |
| 14 | VREF null point | At idle the DC at each I/V output ≈ VREF, confirming the DAC zero-current point; decides R433 on the proof boards (1 kΩ for 1.703 V, 0 Ω for 1.650 V) |
| 15 | BAV99 forward voltage | VF at ≈ 0.4 µA and 0/25/50 °C on the proof boards (detector worst case, 3.7) |

Stage 2 proof-of-concept boards (mandatory, layout gate, 5.1): the two digital boards also carry the real enable-controlled regulators with resistive dummy loads (TPS2553 with 53 mA on 5V_ANA, LP5907 with 17.5 mA on 3V3A, ME6216 with a 60 mA dummy on 1V3, and the 3V3D dropper), so the suspend transition, the watchdog latency and the domain enables are measured on the real path. Estimate ≈ US$90 ≈ 2,860 NTD for two assembled boards (was ≈ US$79 ≈ 2,500 NTD), outside the board budget. Gate before layout: unconfigured ≤ 100 mA; configured digital ≤ 256 mA; suspend ≤ 2.5 mA within 10 ms of the last bus activity; watchdog-to-GPIO latency < 5 µs; clock handover checks; all 8 rates bit-perfect.

# 4. Signal plan

The MCU–CPLD interface is ours to define (Section 5): its pins are set by the MCU–CPLD interface specification, and the new control and sensing signals use free pins (“to assign”). The pin map is frozen before the Stage 2 proof-of-concept board and the schematic. Test pads follow the allocation table in 3.8.

| Signal | From → to | Type / level | Pin / notes |
| --- | --- | --- | --- |
| USB D+, D− | J101 ↔ CH32V307 USB HS | USB 2.0 HS | Dedicated USB HS pins; 90 Ω pair |
| MCU → CPLD audio link | CH32V307 → AGRV2KQ32: SPI with chip-select framing (MCU SPI master with DMA; one CS frame per block of samples); FIFO level and registers over SPI; LRCLK back to a timer-capable MCU pin | 3.3 V LVCMOS | Per the MCU–CPLD interface specification (open item); MCU side low or Hi-Z while 3V3D is off; 24.576 Mbit/s at 384 kHz; I²S is not used for the link |
| MCU link clock | CH32V307 SPI SCK → CPLD global-clock-capable input | 3.3 V output (MCU master) | To assign (CPLD global-clock pin); the CPLD captures the link in the SPI clock domain; the 80 MHz system clock never samples it |
| MCU link chip select | CH32V307 SPI CS → CPLD | 3.3 V output (MCU master) | To assign; one CS frame per block of samples; low or Hi-Z while 3V3D is off |
| MCLK | X201 → 33 Ω → DAC; X201 → R209 33 Ω → CPLD system-clock input | 80 MHz CMOS | Never switches; X201 pin 1 tied to 3V3D; short route to the DAC; TP711 |
| J703 MCLK monitor | MCLK → R703 1 kΩ (DNF, fitted with J703) → J703 | 80 MHz monitor output | Monitor only: an external source never drives MCLK |
| Family clock (shared node) | X202 → R210 and X203 → R211 → one node → CPLD dedicated PLL clock input; R214 10 kΩ pull-down | 24.576 or 22.5792 MHz CMOS | Only one family oscillator drives the node at a time; low while both are off; both oscillators next to the CPLD PLL pin; TP712; DNF mux option U205/R215 with the R216/R217 input pull-downs |
| BCLK, LRCLK, SDATA | CPLD → DAC through 33 Ω; J703 header | 3.3 V output | To assign (CPLD I/O); SDATA and LRCLK change on the falling BCLK edge; held low during rate-family changes; tristated in test mode (then driven by the external source); TP715–TP717 |
| OSC48_EN | CPLD → X202 enable (pin 1) | 3.3 V output; R212 2.2 kΩ pull-down | To assign (CPLD I/O); X202 off by default until the CPLD enables it; TP713 |
| OSC44_EN | CPLD → X203 enable (pin 1); U205 select if the mux is fitted | 3.3 V output; R213 2.2 kΩ pull-down | To assign (CPLD I/O); X203 off by default; never high together with OSC48_EN; TP714 |
| I²C SCL, SDA | CH32V307 ↔ ES9018K2M | Open drain; 2.2 kΩ to 3V3D | To assign; DAC address 0x90 |
| DAC RESETB | CH32V307 → DAC | 3.3 V output; R305 pull-down | To assign; driven low together with AVCC_EN and AUD_EN in the controlled power-down |
| CC1_ADC, CC2_ADC | CC1, CC2 → 10 kΩ → MCU ADC | Analog | To assign (ADC pins); sampled at least every 5 ms while configured; TP726, TP727 |
| VBUS_SENSE | R105/R106 divider → MCU ADC | Analog; watchdog 4.10 V | To assign (ADC pin with analog watchdog); TP725 |
| DC_SENSE_L+ | Leg L+ at the jack side of its output link → 1 MΩ series resistor (one of R624–R627) → ADC node with 1 MΩ to 3V3M (R628–R631), 1 MΩ to GND (R632–R635) and 100 nF (C608–C611) → MCU ADC | Analog: 1.1 V + leg/3 (−0.18 … +2.40 V), ratiometric to 3V3M | To assign (ADC pin); slow DC tier (3.7); zero calibrated at the end-of-line test |
| DC_SENSE_L− | Leg L−, network as DC_SENSE_L+ | Analog, as DC_SENSE_L+ | To assign (ADC pin); slow DC tier |
| DC_SENSE_R+ | Leg R+, network as DC_SENSE_L+ | Analog, as DC_SENSE_L+ | To assign (ADC pin); slow DC tier |
| DC_SENSE_R− | Leg R−, network as DC_SENSE_L+ | Analog, as DC_SENSE_L+ | To assign (ADC pin); slow DC tier |
| AUD_EN | MCU → U504 CE | 3.3 V output; R508 pull-down | To assign |
| AVCC_EN | MCU → U302 EN; Q611 kill (drain) while the latch is set | 3.3 V output; R303 pull-down | To assign; TP720; pulled low in hardware when the latch sets (the DAC analog supply turns off) |
| ANA_EN | MCU → U503 EN | 3.3 V output; R513 pull-down | To assign |
| ILIM_HI | MCU → Q501 gate | 3.3 V output; R511 pull-down | To assign; only with ≥ 1.5 A advertised |
| ANA_FAULT | U503 FAULT → MCU; also the D606 cathode (latch set from node F); firmware output to set the latch | Open drain; R512 pull-up to 3V3M; MCU pin used as input or as open-drain output low | To assign (open-drain-capable pin); TP723 (node F on TP724); firmware drives it low to set the latch (slow DC tier) |
| RAIL_EN | MCU → R507 10 kΩ → LM27762 EN+/EN−; Q607 kill | 3.3 V; R506 pull-down | To assign; TP719; low at t0 + 3.5 ms in the controlled power-down (2.5) |
| RAILS_OK_N | LM27762 PGOOD → MCU, Q610 gate, Q609 gate through R623 100 kΩ (C607 100 nF to GND: on 6–32 ms after PGOOD goes bad), blanking RC | Open drain; R505 pull-up to 3V3M | To assign; TP721 |
| AMP_EN | MCU → R618 10 kΩ → both OPA1622 EN; Q608 kill (latch), Q609 slow interlock | 3.3 V; R421 pull-down | To assign; TP718 |
| FLT_CLR | MCU → Q606 gate | 3.3 V output; R616 pull-down | To assign |
| FLT | Latch node B → MCU; Q611 gate | 3.3 V input (interrupt); high when the latch sets | To assign; TP722 |
| LED × 2 | MCU → D705, D706 through 2.2 kΩ | 3.3 V output | To assign |
| Debug SWDIO, SWCLK | J201 ↔ CH32V307 | 2-wire debug | Dedicated pins |
| CPLD JTAG TCK, TMS, TDI, TDO | J202 ↔ AGRV2KQ32 | JTAG | Dedicated pins |

# 5. Firmware requirements

The USB firmware and the CPLD logic are our own (owner decision, 24 Sep 2026). The firmware is our own USB Audio Class 2.0 implementation on the CH32V307 high-speed USB port, built on WCH’s register-level USBHS device examples; the chip’s EVT package has high-speed device examples for HID, CDC serial, mass storage and network adapters, but no audio example. TinyUSB (MIT) is an alternative only if its CH32V307 high-speed audio path proves reliable: a 2023 report showed UAC2 control requests failing on this chip, and the driver contributor had not verified isochronous transfers.

The open-source USBAudioToI2S firmware (R7, Apache-2.0) is kept as a known-good comparison on the same hardware and as a reference; licence notices are kept if any code is copied. Its source (audio_cpld.v) ties the MCLK output low, runs one PLL from the 48 kHz-family clock, has no 352.8 kHz path and a combinational clock select, and its descriptors report self-powered (0xC0), bMaxPower 0x32 (100 mA), Feature Unit bmaControls = 0 and VID 0x1A86 / PID 0. The plan below is proven in a two-stage proof of concept before layout (firmware gate); Stage 2 is mandatory and its USB current, suspend and watchdog-latency measurements gate the layout.

## 5.1 Firmware plan and proof of concept

| # | Topic | Required behaviour |
| --- | --- | --- |
| 1 | USB core | High-speed port: enumeration, descriptors, standard requests, suspend/resume. Suspend is a v1 requirement: ≤ 2.5 mA within 10 ms of the last bus activity (suspend interrupt 3 ms after the last bus activity → controlled power-down ≤ 4 ms → MCU low-power with USB wake-up). Descriptors: bus-powered (bmAttributes 0x80), bMaxPower 0xFA (500 mA); test VID/PID 1209:0001 during development (pid.codes reserves it for private testing only), a real PID before publishing; serial number from the CH32V307 96-bit unique ID |
| 2 | UAC2 controls | Clock source with the rates (CUR/RANGE: all 8 rates from 44.1 to 384 kHz, 352.8 kHz included), clock valid, mute and volume (CUR/RANGE) mapped to the ES9018K2M attenuator over I²C plus MCU fine scaling for the channel trim, alternate settings for 16/24/32-bit |
| 3 | Streaming | Isochronous OUT → ring buffer → DMA to the CPLD over SPI with chip-select framing (MCU master; one CS frame per block of samples). At 384 kHz, 32-bit stereo: 384 bytes per 125 µs microframe, up to 392 bytes with one extra sample; link payload 24.576 Mbit/s |
| 4 | Feedback | The CPLD’s LRCLK counted by an MCU timer against USB (micro)frames, plus a small FIFO-level correction; sent in 16.16 format (at 384 kHz: 48.0 samples per microframe = 0x300000); the same timer checks the LRCLK rate after every family change (within ±0.1 % of the target) |
| 5 | Application | ES9018K2M driver (asynchronous mode, DPLL bandwidth register 12, 32-bit I²S, soft start, mute, lock status, master trim); power states and sequencing (2.5: legs enabled while VREF = 0, DAC first off and rails last, power-down ≤ 4 ms); rate and break-before-make family changes (5.2); CC monitoring and VBUS sensing, ILIM_HI control; calibration storage (master trim, channel trim, zero of each DC-sense channel); the −3.5 dB pre-calibration safety default; test modes (DC fault injection by a digital DC signal, I²S-header test mode) |
| 6 | Slow DC tier | Four ADC channels, one per leg (3.7), zero-calibrated at the end-of-line test; 0.5-s and 2-s averages: ≥ 0.8 V → set the latch within 1 s, ≥ 0.2 V → within 3 s, by switching the ANA_FAULT pin to open-drain output low (D606 → node F → the hardware kill path); active only while AMP_EN is high, armed after the DAC initialisation in the power-up; no false trip for full-scale content above ≈ 2 Hz |
| 7 | Fault cause capture and recovery | At the FLT interrupt, before any other action: store the four leg DC readings (the 33 ms RC holds the pre-trip state), ANA_FAULT, RAILS_OK_N and whether firmware itself set the latch. DC fault if any leg reads ≥ 0.3 V or firmware tripped, otherwise overload; unavailable readings count as a DC fault. FLT during a controlled power-down or before the first FLT_CLR after power-up is expected: not logged, not counted, no ceiling change. Recovery policy with the −12 dB strike rule in 5.2 |
| 8 | MCU–CPLD interface | Ours to define: SPI with chip-select framing (MCU SPI master with DMA; one CS frame per block of samples); the CPLD captures the link in the SPI clock domain (SPI clock on a CPLD global-clock-capable input) and hands each complete 32-bit word to the 80 MHz system domain through a toggle synchronizer (word period ≥ 1.3 µs at 384 kHz, ≫ 3 system-clock cycles of 12.5 ns); FIFO level over SPI, LRCLK back to a timer-capable MCU pin. I²S is not used for the link (its frame rate is not locked to the audio rate). Payload at 384 kHz / 32-bit stereo = 24.576 Mbit/s: confirm the CH32V307 SPI maximum clock in Stage 1. Write the interface specification and freeze the pin map before the Stage 2 board and the schematic (open item) |
| 9 | CPLD logic | Our own: SPI link capture in the SPI clock domain; dual-clock FIFO (written in the 80 MHz system domain, read in the PLL bit-clock domain, Gray-coded pointers); I²S output engine on the PLL output with exact ÷2/4/8/16, SDATA and LRCLK changed on the falling BCLK edge; PLL reset and lock handling; mutually exclusive oscillator enables OSC48_EN/OSC44_EN with the break-before-make family change (both low after configuration, X202 enabled after ≥ 1 ms); SPI registers (3.3). Global clocks: X201, PLL output, SPI clock. No clock multiplexer in the fabric |
| 10 | Proof of concept, stage 1 | Desk setup: WCH CH32V307V-EVT-R1 board, an AGM 2K-LUT CPLD board and an ES9018K2M I²S-input DAC board (≈ US$48–57 for the three boards at listing prices, outside the board budget). Order: enumeration and controls without audio → streaming into a dummy sink with timer feedback → link clock limit (CH32V307 SPI maximum clock) → full chain with CPLD and DAC |
| 11 | Proof of concept, stage 2 (mandatory, layout gate) | Two proof-of-concept boards with the real chips and pin map: USB-C and ESD, 3V3M/3V3D LDOs with the 3V3D dropper, CH32V307 with its crystal, AGRV2KQ32, X201 80 MHz, X202 and X203 with R210–R217 (shared node on one board, mux fitted on the other), an I²S/MCLK header to the Stage 1 ES9018K2M board, current-measurement points on 3V3M and 3V3D, and the real enable-controlled regulators with resistive dummy loads: TPS2553 (53 mA on 5V_ANA), LP5907 (17.5 mA on 3V3A) and ME6216 + dummy (60 mA on 1V3), so the suspend transition, the watchdog latency and the domain enables are measured on the real path; ≈ US$90 ≈ 2,860 NTD for two assembled boards, outside the board budget. With the real CPLD design loaded, 384 kHz streaming, at VBUS 4.60 V and 5.25 V and across rate and family changes: attached-not-configured ≤ 100 mA; configured digital ≤ 256 mA (datasheet DAC currents added, since the DAC is on the Stage 1 board); suspend ≤ 2.5 mA within 10 ms of the last bus activity; watchdog-to-GPIO latency < 5 µs. Layout does not start unless all pass |
| 12 | Gate pass criteria | Enumerates bus-powered with the new descriptors; every rate plays on Windows 10/11, macOS and Linux; BCLK/LRCLK average rates correct for every rate (frequency counter) and the DPLL locks; host volume and mute work; all 8 rates bit-perfect, 352.8 kHz included; clock handover: never both enables high, node quiet ≥ 1 ms before the new oscillator starts, disable time measured, no runt pulses, PLL relock after every family change, on the shared-node board and on the mux board; CPLD meets timing at 80 MHz and at the PLL output (49.152 MHz); unconfigured ≤ 100 mA and configured digital ≤ 256 mA; suspend current against time from the last bus activity, ≤ 2.5 mA within 10 ms; watchdog-to-GPIO latency < 5 µs |
| 13 | Unit tests | Pure-logic parts (descriptor tables, feedback math, ring buffer, volume mapping, DC-tier averaging, cause classification and recovery counters) compile on a PC and run as unit tests in GitLab CI |
| 14 | Licences and reference | Keep the licence notices if any code is copied (USBAudioToI2S Apache-2.0, TinyUSB MIT). USBAudioToI2S stays a known-good comparison on the same hardware and a reference, not the base of our firmware |

## 5.2 State machine and policies

| State / event | Required behaviour |
| --- | --- |
| Attached | Only 3V3M is on; the MCU enumerates within 100 mA; the latch is set (node F starts at 0 V through C605); all enables are low |
| Configured | Read CC1/CC2 → Default or High mode (ILIM_HI high only with ≥ 1.5 A advertised; a USB-A-to-C cable reads as Default); run the power-up sequence (2.5): ANA_EN → FLT_CLR and RAIL_EN (≥ 200 ms after attach with the rails off) → AMP_EN once PGOOD has been good ≥ 150 ms (VREF still 0 V) → AVCC_EN → +5 ms AUD_EN → RESETB and I²C init → firmware DC tier armed → soft unmute when streaming. CC monitoring: sample CC1/CC2 at least every 5 ms; treat a change as valid after tRpValueChange (10–20 ms); on a drop from ≥ 1.5 A to Default: soft mute, ILIM_HI low and continue in Default mode within tSinkAdj (60 ms max) of the change; switch up to High only after the new advertisement is stable (R16) |
| Playing | Monitor RAILS_OK_N, FLT, ANA_FAULT, the four DC-sense channels (slow DC tier, active while AMP_EN is high), CC1/CC2 and the VBUS watchdog |
| Controlled power-down | Suspend, host command or fault command, ≤ 4 ms in total: t0 soft mute, then RESETB, AVCC_EN and AUD_EN low together (DAC first off; 3V3A below 0.3 V within ≈ 3.2 ms, VREF follows, the legs stay enabled and hold ≈ 0 V); t0 + 3.5 ms AMP_EN low, then RAIL_EN and ANA_EN low; t0 + 4 ms MCU low-power (2.5) |
| Suspend | Suspend interrupt 3 ms after the last bus activity → controlled power-down → MCU low-power with USB wake-up; ≤ 2.5 mA within 10 ms of the last bus activity is a v1 requirement (suspend level ≈ 7 ms after the last bus activity; budget ≈ 2.2 mA); resume runs the power-up sequence |
| FLT interrupt (cause capture) | Before any other action: store the four leg DC readings (the 33 ms RC holds the pre-trip state), ANA_FAULT, RAILS_OK_N and whether firmware itself set the latch. DC fault if any leg reads ≥ 0.3 V or firmware tripped; otherwise overload; if the readings are unavailable, treat the event as a DC fault (conservative) |
| Expected FLT (masked) | FLT during a controlled power-down, or before the first FLT_CLR after power-up, is expected (the rails going off make PGOOD bad): not logged, not counted, no ceiling change |
| Overload | The latch has already removed RAIL_EN, AMP_EN and AVCC_EN in hardware (PGOOD bad via Q610, or a sustained TPS2553 FAULT via D606) → soft mute; volume ceiling −3 dB per event (down to −12 dB); clear after ≥ 1 s (FLT_CLR only after the rails have been off ≥ 200 ms); the outputs return through the power-up sequence; the ceiling is kept until replug. Overloads count toward the 3-strike stop only once the ceiling has reached −12 dB (Default mode at full volume into 60 Ω balanced needs three −3 dB steps) |
| DC fault | The latch has already removed RAIL_EN, AMP_EN and AVCC_EN (hardware detector, or the firmware DC tier through ANA_FAULT) → soft mute, log, clear after 2 s (FLT_CLR only after the rails have been off ≥ 200 ms); 3 DC faults within 10 s → latched until USB replug; the red LED blinks |
| VBUS watchdog | Analog watchdog at 4.10 V nominal (3.95–4.25 V) → controlled power-down from the interrupt (target < 5 µs to the GPIOs; ≈ 1 µs margin at 0.47 A); the PGOOD latch is the hardware backstop; the latency is measured on the Stage 2 board |
| Rate change | MCLK (X201) never changes. Within a family: soft mute → CPLD changes the BCLK divider → wait for DPLL lock (lock_status, register 64) → soft unmute. Family change, break-before-make: (1) DAC soft mute; (2) I²S output stopped (held low); (3) PLL held in reset; (4) old enable low; (5) wait ≥ 1 ms with both off; (6) new enable high; (7) wait ≥ 10 ms; (8) release the PLL, wait for lock; (9) restart the output; the MCU checks the LRCLK rate with its feedback timer (within ±0.1 % of the target, otherwise stay muted, retry once, then report a fault); (10) DAC re-locks → soft unmute. Never change both enable bits at the same time. AMP_EN stays high (outputs muted digitally); pop measured |
| Calibration | Before calibration apply −3.5 dB (a safety setting, always active until a calibration is stored); the end-of-line test writes the master trim (1.75 Vrms ± 0.1 dB), the channel trim (±0.05 dB) and the zero of each DC-sense channel to flash |
| Test modes | DC fault injection by a digital DC test signal (0.25 V and 0.9 V for the firmware tier); I²S-header test mode (CPLD I²S outputs Hi-Z; an external source drives BCLK, LRCLK and SDATA only; the DAC stays in asynchronous mode on X201 and configured over I²C; J703 MCLK is a monitor output); pop measurement: pop = peak of the transient at the jack beyond the settled DC step, band-limited 20 Hz–20 kHz (AC-coupled analyzer input) |
| Programming | CH32V307 through the WCH-LinkE pads; CPLD through the JTAG pads |

# 6. Schematic review checklist

These rules seed the AI datasheet-review agent, and each one can be turned into a planted-error test case. ERC is KiCad’s electrical rules check; a netlist script checks connections, values and counts in the exported netlist; the AI agent reviews rules that need datasheet or part-level interpretation.

| # | Rule | Source | Check method |
| --- | --- | --- | --- |
| 1 | VBUS: 5 × 1 µF before any switch (C102, C501, C503 after R519/R520, C301, C505); attach charge ≤ 50 µC (≈ 40 µC estimate) | USB; 2.4 | Netlist script |
| 2 | CC1/CC2: 5.1 kΩ Rd each; CC → 10 kΩ (R103, R104) → MCU ADC, nothing else on CC | USB-C; 3.1 | Netlist script |
| 3 | VBUS divider R105 100 kΩ / R106 47 kΩ to an ADC pin that has the analog watchdog (4.10 V threshold) | 3.1 | Netlist script |
| 4 | USBLC6-2SC6 on D+, D− and VBUS at the connector; USB pair in a 90 Ω differential net class | R1; 3.1 | Netlist script; AI agent (stack-up) |
| 5 | Every IC supply pin decoupled: CH32V307 100 nF per pin + 2.2 µF; CPLD 4 × 100 nF; each oscillator 100 nF (C217 at X201, C218 at X202, C219 at X203); DAC per 3.4; OPA2210 100 nF per rail pin; OPA1622 100 nF + 10 µF per rail pin; LM27762 per 3.6 | R1–R5 | Netlist script |
| 6 | HSE crystal Y201 8 MHz, CL 20 pF, with 33 pF C0G load capacitors (C201, C202) | R1; 3.2 | Netlist script; AI agent |
| 7 | CH32V307 and CPLD pins match the frozen pin map of the MCU–CPLD interface specification | Section 5 | Netlist script (pin-map diff) |
| 8 | MCU pins toward the CPLD are low or Hi-Z while 3V3D is off (no MCU-side pull-ups into 3V3D); I²C pull-ups R207, R208 on 3V3D | 2.5 | Netlist script; AI agent |
| 9 | MCLK: X201 drives only the DAC MCLK pin (through 33 Ω), the CPLD clock input (through R209) and the J703 monitor pin through R703; no other MCLK load | 3.3; 3.8 | Netlist script |
| 10 | DAC in asynchronous mode with MCLK 80 MHz (cycle 12.5 ns ≥ 10 ns, pulse width ≥ 5.6 ns ≥ 4.5 ns): DVCC 3.3 V and the external 1.3 V DVDD (FSR > 192 kHz) | R3 “XI / MCLK Timing” | AI agent |
| 11 | External DVDD from ME6216A13M3G fed from 3V3D (DVDD ≤ DVCC); C310 2.2 µF + 100 nF at the pin | R3; 3.4 | Netlist script |
| 12 | AVCC_L and AVCC_R on separate 2.2 Ω branches from the LP5907; LP5907 EN = AVCC_EN with pull-down; LP5907 total output capacitance ≤ 10 µF (5.7 µF) | R3; R6 | Netlist script |
| 13 | DAC RESETB 100 kΩ pull-down and MCU drive; ADDR strap for 0x90 | R3 | Netlist script |
| 14 | Each DAC output pin connects only to its I/V inverting input: no other load, short trace | R3 p. 27 | Netlist script (pin fan-out); AI agent (trace length) |
| 15 | VREF from 3V3A through R431 / (R432 + R433), R433 1 kΩ (0 Ω option for 1.650 V); C421 330 pF C0G at the divider (τ ≈ 2.8 µs, VREF tracks 3V3A) and no other capacitor or clamp diode on VREF; short traces to all four non-inverting inputs; one shared node | 3.5.1 | Netlist script |
| 16 | Each I/V feedback: 1 kΩ ∥ 5.1 kΩ, with 10 Ω in series with 2 × 330 pF C0G across it | 3.5.1 | Netlist script |
| 17 | C0G only in the signal path (I/V and leg capacitors); no X5R/X7R there | 3.5 | Netlist script (dielectric field) |
| 18 | Four 2.00 kΩ 0.1 % resistors in every difference amplifier; leg+ = I/V(DAC−) − I/V(DAC+), leg− the swap; U401 = L+/L−, U402 = R+/R−; U403 = DACL/DACLB, U404 = DACR/DACRB | 3.5 | Netlist script |
| 19 | AMP_EN: R421 100 kΩ pull-down, R618 10 kΩ series from the MCU, kill FETs Q608 (latch) and Q609 (slow interlock, rule 51) to ground | 3.5.2; 3.7 | Netlist script |
| 20 | OPA1622 thermal pad to V−; input common mode inside (V−)+1.5 V … (V+)−1 V (≈ 0.85 V on the ±3.8 V rails) | R4 | Netlist script; AI agent |
| 21 | TPS2553: RILIM 100 kΩ + 47 kΩ; R510 through Q501 with gate pull-down R511; RILIM trace short; EN pull-down R513; FAULT pull-up R512 to 3V3M | R9; 3.6 | Netlist script; AI agent (layout) |
| 22 | LM27762 dividers: 100 k + 100 k + 15 k over 100 k (FB+) and 100 k + 100 k + 5.1 k over 100 k (FB−, R518 5.1 kΩ); bottom resistors ≥ 50 kΩ | R5 | Netlist script |
| 23 | LM27762 EN must not exceed VIN: RAIL_EN only after 5V_ANA (firmware order); RAIL_EN pull-down R506, series R507, kill FET Q607 to ground | R5; 2.5 | AI agent; netlist script |
| 24 | LM27762 PGOOD pulled up to 3V3M (R505) and routed to the MCU, the Q610 gate, R623 (Q609 gate delay) and the blanking RC | R5; 3.7 | Netlist script |
| 25 | Blanking RC R617 1 MΩ + C606 100 nF on the Q603 gate from PGOOD (armed 25–133 ms after PGOOD good, re-blanked 35–173 ms after PGOOD bad); blanking acts on node S only, never on the latch nodes | 3.7 | Netlist script; AI agent |
| 26 | Node F: R611 10 kΩ series, R612 47 kΩ pull-up to 3V3M, C605 100 nF from F to GND, D605 clamp to GND; only the Q601 gate, the Q610 drain, the D606 anode and C605 connect to F | 3.7 | Netlist script; AI agent |
| 27 | Latch: Q604/Q605 cross-coupled with R614, R615 47 kΩ pull-ups to 3V3M and no capacitor on the latch nodes; Q606 clear from FLT_CLR with pull-down R616; FLT (node B) to an MCU input and to the Q611 gate | 3.7 | Netlist script |
| 28 | DC detect: R601–R604 100 kΩ + C601–C604 1 µF per leg (τ = 100 ms), BAV99 into MAX/MIN, 1 MΩ loads, thresholds R607/R608 100 kΩ over R609/R610 10 kΩ from the rails | 3.7 | Netlist script |
| 29 | Ferrites: BLM18PG121SN1D in the 5V_ANA path; GZ1608D601TF only on DVCC | 3.4; 3.6 | Netlist script |
| 30 | ESD diode (LESD5D5.0CT1G) on every jack leg | 3.5.2 | Netlist script |
| 31 | Test pads TP701–TP731 per the allocation table: 8 rails, VREF, AVCC_L, MCLK, family-clock node, OSC48_EN, OSC44_EN, BCLK, LRCLK, SDATA, AMP_EN, RAIL_EN, AVCC_EN, RAILS_OK_N, FLT, ANA_FAULT, node F, VBUS_SENSE, CC1, CC2, 4 GND | 3.8 | Netlist script |
| 32 | One part per BOM line: one value, one MPN, one LCSC number and a JLCPCB class taken from its part page or the basic-parts list per line (X202: not shown, assumed Extended); no mixed or range values | Parts List v0.5 | Netlist script (BOM) |
| 33 | X202 and X203: C218 and C219 100 nF at their supply pins; R210 and R211 33 Ω in series at each oscillator output, joined into one node at the CPLD’s dedicated PLL clock input | 3.3 | Netlist script; AI agent (placement) |
| 34 | Shared family-clock node routed next to the CPLD PLL clock pin: both family oscillators and the node beside the pin, short traces, no stubs; X201 reaches the CPLD system-clock input through R209 | R2; 3.3 | Netlist script (pin map); AI agent (layout) |
| 35 | Short 80 MHz MCLK route from X201 to the DAC MCLK pin; the CPLD branch leaves through R209 at the oscillator | 3.3 | AI agent (layout) |
| 36 | No clock trace (MCLK, family clock, BCLK, LRCLK) crosses the analog area (I/V converters, output legs, VREF) | 3.3; 3.5 | AI agent (layout) |
| 37 | Family change break-before-make: I²S output (BCLK, LRCLK, SDATA) held low, DAC soft-muted and PLL held in reset; old enable low, ≥ 1 ms with both off, new enable high, ≥ 10 ms start-up; never both enable bits changed at the same time | 3.3; 5.2 | AI agent (CPLD logic review); scope check in the proof of concept |
| 38 | Oscillator enables: 2.2 kΩ pull-downs on both enables (R212 on OSC48_EN, R213 on OSC44_EN), so both family oscillators are off at power-up and during CPLD configuration | 3.3 | Netlist script |
| 39 | X201 tri-state pin 1 tied to 3V3D; X201 is never disabled | 3.3; R14 | Netlist script |
| 40 | OSC48_EN and OSC44_EN driven from CPLD I/O in the same 3V3D domain as the oscillators (no drive from the always-on MCU domain, so no back-powering when 3V3D is off) | 2.5; 3.3 | Netlist script (power domain); AI agent |
| 41 | R214 10 kΩ pull-down from the shared family-clock node to GND | 3.3 | Netlist script |
| 42 | U205 (74LVC1G157GW,125) and R215 33 Ω DNF at the CPLD PLL clock pin, select = OSC44_EN; R210/R211 are the population option (fit U205 and R215 and remove R210/R211 to use the mux) | 3.3 | Netlist script (BOM fit field); AI agent |
| 43 | Q610 2N7002: gate = RAILS_OK_N, drain = node F, source = GND | 3.7 | Netlist script |
| 44 | D606 1N4148WS: anode = node F, cathode = ANA_FAULT (TPS2553 FAULT and the MCU ANA_FAULT pin, rule 53) | 3.7 | Netlist script |
| 45 | Q611 2N7002: gate = node B (FLT), drain = AVCC_EN, source = GND | 3.7 | Netlist script |
| 46 | Hardware DC sense (R601–R604) and slow DC-sense network (R624–R627) taken at the jack side of the 0 Ω output links R417–R420 | 3.7; 3.8 | Netlist script |
| 47 | J703 MCLK pin connects to MCLK only through R703 1 kΩ (monitor output; DNF with J703) | 3.8 | Netlist script |
| 48 | MCU audio-link SPI clock on a CPLD global-clock-capable input; SPI chip select from the MCU to the CPLD (CS framing) | R2; 3.3 | Netlist script (pin map) |
| 49 | Idle family-oscillator output traces kept short (only one oscillator drives the shared node; the other may keep running internally) | 3.3 | AI agent (layout) |
| 50 | 3V3D: U504 fed from 5V_SYS through R519 + R520 (2 × 2.2 Ω 0805 in series) before the LDO; C503 1 µF at the U504 input after the dropper; C504 1 µF and C512 10 µF at the U504 output | 3.6; 3.6.3 | Netlist script |
| 51 | Q609 2N7002: drain = AMP_EN, source = GND; gate from RAILS_OK_N through R623 100 kΩ, with C607 100 nF from the gate to GND | 3.7 | Netlist script |
| 52 | Slow DC-sense network per leg: R624–R627 1 MΩ from the jack side of R417–R420, R628–R631 1 MΩ to 3V3M, R632–R635 1 MΩ to GND, C608–C611 100 nF; each node to its own MCU ADC pin (four ADC pins) | 3.7; Section 4 | Netlist script |
| 53 | ANA_FAULT on an open-drain-capable MCU pin (input, or open-drain output low that sets the latch through D606) | 3.2; 3.7 | Netlist script (pin map); AI agent (MCU datasheet) |
| 54 | R216, R217 10 kΩ pull-downs (DNF, fitted with U205) at both U205 inputs | 3.3 | Netlist script (BOM fit field) |
| 55 | CPLD I²S engine changes SDATA and LRCLK on the falling BCLK edge (≥ 20 ns half period at 24.576 MHz against 4.1 ns setup / 2 ns hold) | R3 p. 30; 3.3 | AI agent (CPLD logic review); scope check |
| 56 | Copper pours on the VIN, GND and VOUT pins of the LDOs U502, U504, U303 and U302; IR or thermocouple check at bring-up | 3.6.3 | AI agent (layout) |

# 7. Open items and review disposition

Open items are verification tasks with a closing gate: MCU–CPLD interface specification (before the Stage 2 board), firmware gate (Stage 1 desk proof of concept and the mandatory Stage 2 board, before layout; the Stage 2 USB current, suspend and watchdog-latency measurements gate the layout), SPICE before layout, schematic review, JLCPCB quote after layout, bring-up. One item waits for an owner decision: the residual DC blind region below 0.2 V.

## 7.1 Open items

| # | Item | Why it matters | How to close | Gate |
| --- | --- | --- | --- | --- |
| 1 | Own UAC2 firmware and CPLD logic (top risk: host compatibility of high-speed isochronous streaming with feedback) | v1 depends on our own USB stack on the CH32V307 high-speed port; WCH has no high-speed audio example | Two-stage proof of concept on development hardware (5.1) | Firmware gate |
| 2 | MCU–CPLD interface specification (SPI with CS framing) | The link is ours to define and the pin map depends on it | Write the specification (SPI with chip-select framing, one CS frame per block of samples) and freeze the pin map | Before the Stage 2 board and the schematic |
| 3 | CPLD timing at 80 MHz and at the PLL output (49.152 MHz), including the FIFO crossing and the link hand-over | Every bit clock comes from the PLL; the system side runs at 80 MHz; the link has its own clock domain | Build and time the CPLD design; frequency counter and duty on every rate; PLL relock and runt-pulse check at family changes | Firmware gate (Stage 2) |
| 4 | Oscillator disable time and disabled-output state are not specified in the SCTF/YXC datasheets | X202 and X203 share one node at the CPLD PLL input | Measure on Stage 2 (scope on the shared node); fallback: mux U205 with R216/R217 fitted, already built on one Stage 2 board | Stage 2 |
| 5 | AGRV2K I/O state and weak pull-up strength during configuration | The 2.2 kΩ pull-downs must hold both enables low against ≥ 5.1 kΩ | Datasheet check and Stage 2 scope check | Stage 2 |
| 6 | CH32V307 SPI clock limit | The link payload is 24.576 Mbit/s at 384 kHz / 32-bit stereo | Confirm on the desk setup | Stage 1 |
| 7 | Shared oscillator node routing | Two oscillator outputs and the PLL clock pin on one net | Both oscillators and the node next to the CPLD PLL clock pin, short traces, idle trace short; scope check of the clock | Layout review; bring-up |
| 8 | MCU and CPLD currents are assumptions; oscillator typicals assumed (datasheet maxima used) | The unconfigured state must stay ≤ 100 mA; the configured totals depend on them | Measure on Stage 2 with the real CPLD design; mitigation: lower MCU clock before configuration | Stage 2; bring-up |
| 9 | USB Default-mode estimated maximum 0.469 A (31 mA margin) | Close to the 500 mA USB 2.0 limit; rests on the current estimates | Stage 2 measurement (attached ≤ 100 mA, configured digital ≤ 256 mA), then the main boards | Stage 2 (layout gate); bring-up |
| 10 | MCU low-power current with USB wake-up (suspend budget) | ≤ 1.9 mA is an assumption within the ≈ 2.2 mA budget; suspend ≤ 2.5 mA within 10 ms of the last bus activity is a v1 requirement | Measure the current against time from the last bus activity on Stage 2 (real regulators with dummy loads), then on the main board | Stage 2 gate; bring-up |
| 11 | VBUS watchdog-to-GPIO latency | ≈ 1 µs margin at 0.47 A (VBUS 4.10 V → 3.46 V in ≈ 6 µs; target < 5 µs); the PGOOD latch is the backstop | Measure on Stage 2 | Stage 2 gate |
| 12 | DAC pin-to-pin DC mismatch (≤ 1 mV assumed) | Offset budget term of 1.18 mV | Measure; 100 % offset screening with a DC-capable DMM | Bring-up |
| 13 | DAC null: 1.702 V (current-output formula) or 1.65 V (voltage-source model) | VREF-to-null offset term 1.17 mV at 1.702 V, 2.8 mV if the null is 1.65 V (totals 8.6 / 17.2 mV) | VREF null test; R433 1 kΩ → 0 Ω sets VREF = 1.650 V if the test shows the 1.65 V null | Proof boards; bring-up |
| 14 | BAV99 forward voltage at ≈ 0.4 µA (detector worst case) | 0.12–0.40 V estimated over 0–50 °C; sets the 0.47–0.81 V worst-case leg trip | Measure at 0/25/50 °C | Proof boards |
| 15 | Residual DC blind region below 0.2 V per leg | ≤ 2.5 mW into 16 Ω, ≤ 1.25 mW into 32 Ω; a product-safety decision (R6-B2) | Proposed: accept; owner decision pending | Owner decision |
| 16 | OPA1622 swing and current capability at the ±3.8 V rails | Extrapolated from typical ±18 V curves; the worst unit reaches the swing limit 0.27 dB below full scale at 120 Ω balanced and 3.5 mm 60 Ω | Bench-verify the power table | Bring-up |
| 17 | SPICE: OPA2210 stability with the DAC pin capacitance and the 10 Ω + 660 pF network; OPA2210 output swing; OPA1622 disabled-state output with the network attached; protection timing; failed output stage | I/V converter loop; 0.43 V swing margin is an estimate; pop and protection behaviour | SPICE | Before layout |
| 18 | LM27762 charge-pump resistance at 4.5 V (3.0 Ω assumed) | Sets the 0.11 V negative-rail margin | Measure the negative-rail margin at VBUS 4.60 V | Bring-up |
| 19 | Negative-rail brief dropout at the 4.40 V stress corner | −92 mV at current peaks (4.4 mm 32 Ω, High mode) | Bench measurement | Bring-up |
| 20 | ME6216 power-dissipation rating and LM27762 θJA | U303 0.12 W and U501 ≈ 0.22 W (3.6.3) | Datasheet check; copper pours; IR check at bring-up | Before layout |
| 21 | Stock: CH32V307 (119), ES9018K2M (724), AGRV2KQ32 (980), ME6216A13M3G (≈ 410), 4.4 mm jack (721), OPA2210 (76 at JLCPCB), X201 (2,503) | Low stock (Jun–Sep 2026) | Reserve the parts in the JLCPCB parts library before ordering | Before ordering |
| 22 | OPA2210 single source (reserve 12) | No qualified second source (OPA2209AIDR: JLCPCB stock 0, US$6.11; OPA1612: common-mode limit 1.67 V at the minimum rail, below VREF); at the LCSC price the run is ≈ 1,755 NTD per board, above the ceiling | Reserve 12 (10 + 2 spares) in the JLCPCB parts library now; fallback: LCSC price (owner decision) | Before ordering |
| 23 | Cost 50 NTD under the 1,700 NTD ceiling (conservative) | Conservative fees: through-hole Extended fee (19.5 NTD per board) and X-ray of the three oscillators (26.0 NTD per board each); ≈ 1,552 NTD per board without them; the gate covers every JLCPCB line of the quote | JLCPCB quote from the real BOM/CPL files after layout; confirm the X202 class (assumed Extended); record the gated subtotal and the total payable | JLCPCB quote |
| 24 | Rail-stuck hardware failure in High mode | Up to 26.2 mJ into 16 Ω at the worst case (limit 30 mJ) | Failed-output emulation with dummy loads; v2 option: fast rail-proximity detector | Bring-up; v2 |
| 25 | Unplug pop and DAC pins against AVCC at unplug | Fast VBUS collapse; at full load the logic rails may droop before the sequence completes | Measure the unplug pop (≤ 20 mV target) and TP709 VREF against TP710 AVCC_L; VBUS watchdog at 4.10 V | Bring-up |
| 26 | 4.4 mm plug shorted leg+ to leg− | Outside TI’s one-amplifier short rating (≈ 0.55 W per package) | Survival test | Bring-up |
| 27 | Both jacks live; 4.4-to-3.5 mm adapter | One headphone at a time | Warning stays in the manual | Manual |

## 7.2 Review disposition, sixth round (R6-A, R6-B, R6-C, R6-D)

IDs: R6-A = review 1 of Spec v0.6, Notes v0.5 and Parts List v0.4 (latch timing, power-down, suspend, fault test, cause capture, cost gate), R6-B = review 2 (worst-case protection, blind region, suspend wording, fixture in the gate), R6-C = review 3 (3-strike lockout, mux inputs, OPA2210 single source, minors), R6-D = review 4 (pop vs offset, latch on controlled power-down, LDO dissipation, DAC centre voltage, OPA2210 output swing, watchdog timing, rail floating, 3V3D bulk, minors). Status values: Closed (fixed in this revision); Closed – design, verify (design change made; a bench or proof-of-concept check confirms it); Closed – verify (confirmed by a later check); Open – owner decision and Open – tracked (item with owner and gate).

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| R6-A1 | Latch can miss short PGOOD faults; latch delay missing from the energy figures | C605 moved to node F (power-on set and ≥ 1.5 ms set-request stretch); 47 kΩ latch pull-ups; capture ≈ 35 µs (budget 0.1 ms); Q609 slowed to a static interlock (6–32 ms), so a PGOOD glitch either latches or does nothing; energy computed through the rail-disable instant | Closed – design, verify |
| R6-A2 | Power-down does not establish safe DAC-pin voltage or the post-trip energy bound | VREF tracks AVCC (C421 330 pF; D401, C422 removed); DAC analog off first with the I/V and legs powered, rails last; latch also kills AVCC_EN (Q611); after-trip bound 0.51 mJ (both rails, C +10 %, maximum rails) + ≤ 1 µJ back-power → ≤ 0.6 mJ; unplug case bench-checked | Closed – design, verify |
| R6-A3 | ≥ 50 ms shutdown conflicts with USB suspend timing | Power-down ≤ 4 ms after the suspend interrupt, suspend level ≈ 7 ms after the last bus activity; suspend is a v1 requirement; Stage 2 carries the real regulators with dummy loads and measures current vs time | Closed – design, verify |
| R6-A4 | Hardware fault test tests detection, not removal | Detector test kept; failed-output emulation from the board’s own rail through an external switch with dummy loads, V²/R integrated; FLT high; no +20 % allowance | Closed |
| R6-A5 | PGOOD read after the trip cannot identify the cause | Per-leg DC readings (independent ADC path) and the firmware-trip flag stored first at the interrupt; ≥ 0.3 V → DC fault; unknown → DC fault | Closed – design, verify |
| R6-A (cost) / R6-B4 | Fixture fee outside the ceiling | Gate covers every JLCPCB line incl. any fixture; gated subtotal and total payable recorded; the fixture sentence is standard page text | Closed |
| R6-B1 | Protection limits are nominal | Full worst case (R, C incl. DC bias, LM393 offset, BAV99 VF, rails); R601–R604 100 kΩ, R609/R610 10 kΩ → 2 V ≤ 59 ms (41 % margin), 1 V ≤ 183 ms (27 %); energy 26.2 / 13.3 mJ worst ≤ 30 / 15 mJ | Closed – design, verify |
| R6-B2 | 0.755 V blind region is a product-safety decision | Firmware slow tier on every leg (≥ 0.8 V in 1 s, ≥ 0.2 V in 3 s) sets the same latch; residual < 0.2 V (≤ 2.5 mW into 16 Ω) | Open – owner decision |
| R6-B3 | Suspend wording contradicts the gate | Suspend ≤ 2.5 mA is a v1 requirement everywhere | Closed |
| R6-B (cost) / R6-C3 | OPA2210 single source decides the ceiling | No qualified second source (OPA2209AIDR stock 0; OPA1612 fails the common-mode range at the minimum rail); reserve 12; fallback LCSC price | Open – tracked |
| R6-C1 | Default-mode overloads can lock the board off | Overloads count toward the 3-strike stop only at the −12 dB ceiling; DC faults counted | Closed |
| R6-C2 | Mux inputs float with U205 fitted | DNF R216, R217 10 kΩ pull-downs fitted with U205 | Closed |
| R6-C (minor) | Clipping before a trip near the Default limit; link framing | Scope test added; SPI with chip-select framing | Closed |
| R6-D1 | Pop target conflicts with the offset limit | Pop defined as the band-limited transient beyond the settled DC step; legs enabled/disabled while VREF = 0; SPICE item for the disabled-state output | Closed – verify |
| R6-D2 | Every controlled power-down sets the latch | Expected and masked by firmware (not logged, counted or applied) | Closed |
| R6-D3 | LDO dissipation not analysed | Dissipation table; R519 + R520 dropper keeps U504 ≤ 0.20 W; copper and IR check | Closed – verify |
| R6-D4 | Two DAC centre voltages | Both ESS values stated; offset 8.6 mV worst if 1.65 V; R433 0 Ω option | Closed – verify |
| R6-D5 | OPA2210 output swing not quantified | ≈ (V+) − 0.3 V at ≈ 3 mA; 0.43 V margin calibrated; −3.5 dB pre-calibration default is a safety setting; SPICE | Closed – verify |
| R6-D6 | VBUS watchdog timing margin | Quantified (≈ 1 µs margin at 0.47 A); PGOOD latch as backstop; latency measured on Stage 2 | Closed – verify |
| R6-D7 | Positive rail can float during the rails-off window | Window removed by the new order; the latch trip collapses AVCC in hardware | Closed |
| R6-D8 | 3V3D bulk capacitance | C512 10 µF at U504 | Closed |
| R6-D (minor) | ≈ 208 ×; attach charge; test-pad table; node F margin; BCLK edge; DR loopback floor; Spec C.2 A-M2 bracket | All updated as listed in this round | Closed |

## 7.3 Review disposition, fifth round (R5-A, R5-B, R5-C)

Kept as history: the values in this table are as closed in Notes v0.5 at that time; the current values are in the body of this revision. IDs: A = R5-A (review 1 of Spec v0.5, Notes v0.4 and Parts List v0.3), B = R5-B (review 2), C = R5-C (review 3); status values as in 7.2.

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| R5-A1 / R5-B1 | Spec §4 Key-parts table showed placeholder tokens | The saved v0.5 table was repaired during the amendment (the review saw an interim copy); v0.6 table rebuilt and the saved file scanned for tokens; CI document-lint check added | Closed |
| R5-A2 / R5-B6 / R5-C1 | Shared oscillator outputs need break-before-make; X203 enabled when OE is open | Both enables 2.2 kΩ pull-downs (off at power-up and during CPLD configuration); node pull-down R214; enables mutually exclusive in CPLD logic; sequence with ≥ 1 ms both-off, ≥ 10 ms start-up, PLL held in reset; SCTF/YXC datasheet facts cited (no disable time specified → measured); DNF mux U205/R215 fitted on one Stage 2 board | Closed – design, verify |
| R5-A3 / R5-B2 | 100 MHz MCLK at or above the DAC limit; “worst duty” wording | 80 MHz SCTF SX3M80 (12.5 ns cycle, ≥ 5.6 ns pulse, 208 × 384 kHz); “ideal duty 50 %”; all 8 rates stay in the gate | Closed – design, verify |
| R5-A4 | 0.473 A is an estimate, not a whole-board limit; CC changes not monitored | “Estimated maximum with hardware-limited analog current”, now 0.469 A; Stage 2 measurements before layout; CC monitoring within tSinkAdj | Closed – design, verify |
| R5-A5 | OPA2197 input-pair transition region at VREF | OPA2210 (single input pair, 0.47 V common-mode margin, 35 µV, 2.2 nV/√Hz) | Closed |
| R5-A6 | Shutdown back-powering; AVCC can collapse faster than VREF | ES9018K2M abs. max cited; rails-first-off power-down; watchdog also drops RAIL_EN; VREF clamp D401 and C421 100 nF | Closed – design, verify |
| R5-A7 / R5-B3 | Overload restarts when PGOOD recovers; overload does not latch | Q610 (PGOOD bad) and D606 (TPS2553 FAULT) set the latch; firmware clear required; ≥ 1 s recovery, ceiling steps, 3-strike stop | Closed – design, verify |
| R5-A8 / R5-B5 | Protection envelope and terminal energy | Envelope by DC magnitude (t(V)); energies recalculated as V²/R × t for digital DC and a rail-stuck failure; both rails 0.46 mJ; hardware fault-injection test added | Closed – design, verify |
| R5-A9 | Divider resistor tolerance omitted | Included; R518 5.1 kΩ (−3.722 V); design-corner margin 0.11 V | Closed |
| R5-A10 / R5-C5 | Balanced DR advantage assumes uncorrelated noise | Correlated model: balanced ≈ 120.3 vs unbalanced ≈ 119.5 dB(A) (+0.85 dB) | Closed |
| R5-A11 | QA403 cannot measure DC offset | DC-capable DMM for the 100 % offset screening | Closed |
| R5-A12 | External MCLK contention at the test header | J703 MCLK is a monitor output via R703 1 kΩ; external source drives BCLK/LRCLK/SDATA only | Closed |
| R5-A13 / R5-B9 | 1,700 NTD called “owner-approved”; ceiling policy | Owner statement recorded (budget flexible as long as it does not surge); 1,700 kept as the hard order gate; the JLCPCB quote must pass; exclusions listed; estimate now 1,647 | Closed |
| R5-A14 | “Every class verified” wording; fixture fee not included | X202 exception stated; possible fixture fee (X202 and OPA2210 pages) listed as excluded, to confirm in the quote | Closed |
| R5-B4 | USB compliance only at bring-up | Stage 2 mandatory; unconfigured, configured and suspend currents measured before layout; layout blocked unless all pass | Closed – design, verify |
| R5-B7 | “Exact sample rates” | “Exact integer clock ratios”; absolute rate within the oscillator tolerance, absorbed by the asynchronous feedback | Closed |
| R5-B8 | Stage 2 should be mandatory | Mandatory, ≈ US$79 ≈ 2,500 NTD for two boards (outside the board budget) | Closed |
| R5-C2 / R5-C3 | Close the X203 datasheet item; “only one runs” wording; PLL ×2 justification | Datasheet facts cited; “only one drives the node”; idle-trace rule; PLL ×2 also fixes the 45–55 % oscillator duty | Closed |
| R5-C4 | Default-mode power from average current | Instantaneous-peak method; new table and targets | Closed |
| R5-C6 | Blanking 28–119 ms vs 100 ms wait | 25–133 ms incl. R/C tolerance; AMP_EN after PGOOD good ≥ 150 ms; FLT_CLR after ≥ 200 ms rails off | Closed |
| R5-C7 | 0.1 V margin on the 4.30 V VBUS watchdog | 4.10 V (3.95–4.25 V), ≥ 0.15 V margin to the 4.40 V stress corner | Closed |
| R5-C8 | K1000 swing margin ≈ 0.06 dB | With resistor tolerance the worst unit reaches the swing limit 0.27 dB below full scale at 120 Ω balanced; rated as swing-limited; bench-verified | Closed – verify |
| R5-C9 | Continuous CC monitoring | Firmware requirement (≤ 5 ms sampling, tRpValueChange debounce, tSinkAdj response) | Closed – design, verify |
| R5-C10 | MCU link sampled by the system clock | Link captured in its own clock domain with a word toggle synchronizer | Closed – design, verify |
| R5-C11 | Versioning and stale rows | New files Spec v0.6 / Notes v0.5 / Parts List v0.4; Spec history notes that v0.5 first used the interim divider; Notes earlier-round rows R1-B1 (7.7 µF) and R2-M5 (1,550 / 1,536) marked as the design at that time with the current values | Closed |

## 7.4 Review disposition, fourth round (R4-A, R4-B, R4-C)

Kept as history: the values in this table are as closed in Notes v0.4 at that time; the current values are in the body of this revision. IDs: A = R4-A (review 1 of Spec v0.4), B = R4-B (review 2), C = R4-C (review 3); status values as in 7.2.

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| A-H1 | FB101 mis-specified (GZ1608D601TF = 200 mA / 450 mΩ, C1002; C1015 is GZ2012D101TF) | Power ferrite now Murata BLM18PG121SN1D (C14709, Basic, 2 A, 50 mΩ) in the 5V_ANA path; GZ1608D601TF (C1002) only on DVCC (≈ 14 mA); drops recomputed (≈ 15–25 mV) | Closed |
| A-H2 | Offset used OPA1678 typical as maximum | Confirmed (±0.5 mV typ / ±2 mV max); I/V op-amp now OPA2197 (±0.25 mV max); budget 7.9 / 15.9 mV worst, balanced limit ±20 mV, 100 % screening; DAC-mismatch assumption Open (measure) | Closed |
| A-H3 | Safe disconnected output not established | Full sequence and current paths; rails first on / last off; PGOOD hardware kill; back-powering bounded (≤ 1.7 mA per pin) and removed in 10 ms; energy ≤ 16.5 mJ before / ≤ 0.22 mJ after the trip; protection defined at the headphone terminals | Closed – design, verify |
| A-H4 | Negative-rail headroom | Rails ±3.78 V; both rails budgeted at 4.60 V and maximum load with a 3.0 Ω charge-pump resistance (margin 0.17 V) | Closed |
| A-H5 | 100 mA ceiling not enforced | Clean envelope (80 mA per leg, swing curve) separated from overload; TPS2553 enforces the analog supply current; PGOOD kill; firmware ceiling steps; overload dissipation bounded; overload tests replace the 30-min 32 Ω balanced test | Closed – design, verify |
| A-H6 | USB power states and descriptors | State-by-state budget; MCU-only before configuration (≤ 83 mA estimate); descriptors 0x80 / 0xFA; CC detection by ADC for High mode; suspend policy (2.5 mA Open) | Closed – design, verify (firmware gate) |
| A-M1 | Mixed purchasing lines and placeholder ranges | One part per line, every value specified, 48 purchasing lines by LCSC number | Closed |
| A-M2 | 13.8 NTD ceiling margin; one Basic→Extended flip fails it | Every class verified on the JLCPCB page except X202 (not shown; assumed Extended); Extended count conservative (19, incl. the through-hole jack, X202 and X203); still a planning estimate until the JLCPCB quote | Closed – verify |
| A-M3 | Gain tolerance defeats the minimum power targets | Per-unit calibration to 1.75 Vrms per leg; targets defined at the calibrated level | Closed |
| A-M4 | Two LDOs do not guarantee sequencing | Separate enables (LP5907 EN, ME6211 CE) in a firmware sequence; DVDD LDO fed from 3V3D so DVDD ≤ DVCC by construction; scope check at bring-up | Closed – design, verify |
| A-M5 | Clock adaptation needs an earlier gate | Three oscillators: X201 100 MHz for the DAC MCLK (asynchronous mode, no PLL in the MCLK path) and the CPLD system clock; X202 24.576 MHz and X203 22.5792 MHz, selected by their enables in front of the CPLD PLL ×2 (all 8 rates exact); two-stage proof of concept on development hardware before layout | Closed – design, verify |
| B-S1 | −4.5 V rail cannot regulate from USB | As A-H4 | Closed |
| B-S2 | 2.0 Vrms per leg does not fit the supply; +1 dB units clip | Calibrated 1.75 Vrms per leg; swing model from OPA1622 Fig. 38/39; pre-calibration trim −3.5 dB | Closed |
| B-S3 | Power targets fail on low-spread units; rating method | As A-M3: “1 % THD+N or calibrated 0 dBFS, whichever first” | Closed |
| B-S4 | Firmware needs much more than the spec said | Firmware scope listed module by module for our own UAC2 firmware and CPLD logic (Section 5); two-stage proof of concept before layout; 352.8 kHz comes from X203 through the same CPLD PLL | Closed – verify |
| B-S5 | 14 NTD margin will be used up (odd values, 1.5 nF C0G, crystal, AMS1117, through-hole jack fee) | Basic E24 values or Basic combinations (100 k + 47 k, 1 k ∥ 5.1 k, 15 k + 1 k, 2 × 330 pF); Basic 5032 crystal; AMS1117 removed; through-hole fee counted; no cheaper 4.4 mm jack exists at LCSC (no SMD option) | Closed – verify |
| B-M1 | Pop at unplug has no hardware control | PGOOD gates AMP_EN in hardware; VBUS watchdog; separate unplug pop target (≤ 20 mV) | Closed – design, verify |
| B-M2 | Fault latch re-trips on clear | Set path blanked ≈ 60 ms after PGOOD good, without clearing the latch | Closed |
| B-M3 | PLL jitter goes to the DAC in sync mode | Asynchronous mode, MCLK straight from the 100 MHz oscillator (no PLL in the DAC MCLK path); the CPLD PLL is only in the bit-clock path (both families), whose jitter the DAC’s DPLL rejects; J-test stays in the plan | Closed |
| B-M4 | > 100 mA before enumeration; phones limit OTG | As A-H6; Default mode ≤ 0.473 A covers phones | Closed |
| B-M5 | AMS1117-ADJ needs a tantalum-type capacitor and a minimum load | Fixed 1.3 V ME6216A13M3G (±1 %, 300 mA, ceramic-stable) | Closed |
| B-minor | Duplicate version rows in the Spec history; U301 “voltage-mode” text; HSE 8 MHz; VREF risk overstated | Spec history cleaned; U301 text fixed; HSE 8 MHz confirmed; VREF risk downgraded and quantified (1.17 mV) | Closed |
| C-B1 | USB power before configuration | As A-H6 | Closed – verify |
| C-B2 | Reference descriptors wrong (self-powered, 100 mA) | 0x80 / 0xFA in the firmware gate | Closed – verify |
| C-B3 | 352.8 kHz MCLK not supported by the reference CPLD | Confirmed in the source; replaced by our own CPLD logic: the family oscillator (X202 24.576 MHz or X203 22.5792 MHz) is selected before the PLL, as the reviewer suggested; all 8 rates exact, 352.8 kHz included | Closed – design, verify |
| C-B4 | Clock-family switching can glitch | MCLK never switches; the family changes by the oscillator enables while the output is stopped and the PLL is held in reset; no clock multiplexer in the CPLD fabric; scope check for runt pulses | Closed – design, verify |
| C-B5 | Host volume not implemented | UAC2 volume/mute with GET/SET CUR/RANGE and I²C mapping, tested on three OSes in the gate | Closed – verify |
| C-B6 | “100 mA design current” not implemented | As A-H5 | Closed – design, verify |
| C-H1 | 500 mA budget has no margin | Default mode ≤ 0.473 A (27 mA below 0.5 A) enforced; High mode only with a ≥ 1.5 A Type-C advertisement | Closed |
| C-H2 | +4.5 V corner | Rails ±3.78 V, both rails budgeted | Closed |
| C-H3 | 7.7 µF table is not an inrush proof | Attach charge 38 µC including the 3V3M output capacitance; staggered, soft-started domains | Closed – verify |
| C-H4 | Cost provisional; “Assign LCSC no.” left; “every part is LCSC” inaccurate | Every line has an LCSC number and a verified class (X202: assumed Extended until the JLCPCB quote) | Closed |
| C-H5 | Offset margin | As A-H2 | Closed |
| C-H6 | ≤ 5 mV pop ambitious without relays | Split targets; full sequence incl. unplug/brownout validated on the bench | Closed – verify |
| C-H7 | USB identity (WCH VID, PID 0) | pid.codes VID 0x1209: test PID 0x0001 during development, a real PID before publishing; UID-based serial | Closed – verify |

## 7.5 Review disposition, earlier rounds (R1–R3)

Findings of the first three review rounds as closed in Notes v0.3, kept as history; values in this table are as closed at that time. R1-B1 and R2-M5 are marked as the design at that time and give the current values.

| ID | Finding | Disposition | Status |
| --- | --- | --- | --- |
| R1-B1 | VBUS_PD ≈ 130 µF vs USB-C 10 µF | No PD port; single 5 V port, VBUS budget 7.7 µF (design at that time; current: 5 × 1 µF on VBUS, 2.4) | Closed – N/A |
| R1-B2 | LMR33630 + SMAJ18A surge stacking ≈ 44 V | No inverting converter or 15 V input; LM27762 runs from 5 V | Closed – N/A |
| R1-B3 | LP5907 on 1V8_D over 10 µF | No 1.8 V rails; the only LP5907 (DAC 3.3 V) is budgeted ≤ 10 µF | Closed – N/A |
| R1-B4 | Paralleling open | No paralleling; one amplifier per leg (100 mA design current) | Closed |
| R1-B5 | LM339 at −12 V pulling 1.8 V logic | Comparator on ±4.5 V drives a PNP into a 3.3 V latch; 3.3 V nodes only see transistor/N-FET pull-downs, never below 0 V | Closed |
| R1-B6 | Offset ±5 mV vs 11 mV | Worst case recomputed (current mode: 7.4 / 14.7 mV); targets ±10 / ±15 mV | Closed |
| R1-H | AMS1117 dropout/noise | ME6211 (digital), LP5907 (DAC analog) | Closed |
| R1-H | XU316 VOH vs AK4493 VIH | All logic 3.3 V | Closed – N/A |
| R1-H | Both jacks live / gain-switch pop | Gain switch removed; both jacks live documented (one headphone at a time) | Closed |
| R1-H | OPA1622 stock 197 vs 400 | Sample run needs 10 (2 per board) | Closed |
| R1-H | LM317/LM337 cost trade | LM27762 low-noise LDOs | Closed – N/A |
| R1-H | CI “every part has an LCSC number” vs Digi-Key/Mouser parts | Every part is now an LCSC/JLCPCB part; the rule stands as written | Closed |
| R1-H | Firmware port map before layout | USBAudioToI2S build on the reference pin map before layout (open item 1) | Closed as a gate |
| R1 | Consistency: v0.2 wording, cost source of truth, THD+N conditions, Zout incl. contacts, DSD pins, grounding/ESD/CC/silkscreen | Fixed in Spec v0.4: single USB-C port removes the silkscreen confusion; CC = 5.1 kΩ Rd; jack ESD PESD15VL1BA; grounding rules added; DSD not in v1 | Closed |
| R2-S1 | TVS does not protect −14.7 V converter; no OVP | Block removed | Closed – N/A |
| R2-S2 | VBUS bulk before contract; analog rails start at 5 V | Block removed | Closed – N/A |
| R2-S3 | Balanced Zout with option B | Legs driven directly (< 0.1 Ω + contacts) | Closed |
| R2-S4 | K1000 1 W no margin | Requirement change (budget redesign); ≈ 0.13 W, 95 dB SPL documented | Closed |
| R2-S5 | Offset target contradicts design | As R1-B6 | Closed |
| R2-M1 | Board ≈ 7 W, PLA case | ≤ ≈ 2.5 W board, ≤ 0.54 W per OPA1622 | Closed |
| R2-M2 | LM317 near dropout; LM337 LC filter | Block removed | Closed – N/A |
| R2-M3 | ±0.05 dB trim cannot use 0.5 dB DAC steps | Optional digital scaling in the MCU firmware | Closed |
| R2-M4 | Max power criterion | Rated at 1 kHz, 1 % THD+N, both channels | Closed |
| R2-M5 | Cost margin 10 NTD / exchange rate | Gate uses fixed 31.7 NTD/US$: target 1,500 NTD per board (WARN above), approved ceiling 1,550 (FAIL above); current 1,536 → WARNING (design at that time; current: 1,650 NTD per board, ceiling 1,700 NTD, Section 8) | Closed |
| R2-M6 | 4.4-to-3.5 mm adapter shorts leg− | One amplifier per package shorted (TI continuous-short rating), ≤ 0.6 W; risk + manual warning | Closed |
| R2-M7 | Unbalanced crosstalk needs ≤ 6 mΩ ground | Layout rule (≤ 5.7 mΩ) | Closed |
| R2 | Minor: stale versions; KC2520Z “(assumed)”; through-hole count; pop requirement; CH224A/AMS1117 unverified claims; NVIDIA paragraph | Versions fixed; oscillators counted leadless conservatively; only the 4.4 mm jack is through-hole (confirm USB-C shell pegs); pop ≤ 5 mV; CH224A removed; AMS1117-3.3 replaced by ME6211C33 (digital 3V3) and LP5907 (DAC analog); an AMS1117-ADJ is used only for the 1.30 V DAC core supply; job-application text moved to the repository README | Closed |
| R3-1 | DC fault latch and recovery | Hardware 3.3 V latch, firmware recovery policy (Section 5, item 6), status to MCU | Closed |
| R3-2 | TVS/converter envelope | Block removed | Closed – N/A |
| R3-3 | Stage-1 gain 0.9009 with the input network | No stage 1; the gain now includes the DAC’s 806 Ω source (current mode, gain = 2 × Rf/806 per leg) | Closed |
| R3-4 | QA403 cannot drive 1.8 V I²S | I²S is now 3.3 V; header + firmware test mode + sequencing specified | Closed |
| R3-5 | 200 mA “limit” not implemented; thermal at 16 Ω balanced | 100 mA/leg design current, rated power at 1 % THD+N, full load matrix; 0.54 W worst per package | Closed |
| R3 | Consistency: rails-good qualification; offset/trim; workbook external links; cost/sourcing reconciliation | LM27762 PG + latch + 50 ms; offset/trim closed; the delivered v0.1 workbook references only its own sheets (links appear when sheets are copied into another workbook; v0.2 adds a self-check); single source of truth, all LCSC | Closed |

# 8. Chosen parts

Parts for the 5-board sample run, one line per LCSC part number (47 purchasing lines); Qty is per board. Unit prices are the tier that matches the run quantity (1+ tier for one-per-board ICs, 10+ for two-per-board parts); the OPA2210 uses the JLCPCB part-page price, because the PCBA order is charged at JLCPCB prices (1+ US$1.2180, 10+ US$1.0035, 100+ US$0.7530; LCSC lists US$2.67 at 10+). Parts List v0.5 is the source of truth for cost; the figures below are copied from it. Every line has an LCSC number, and every line’s JLCPCB class is taken from its JLCPCB part page or the basic-parts list (Jun–Sep 2026), except X202, whose class is not shown (assumed Extended). The DNF parts (U205, R215, R216, R217, R703, J703) are not in the purchasing lines.

| # | Part | MPN | LCSC no. | JLCPCB class | Qty | Unit US$ | Refs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Jack, 4.4 mm balanced | G-Switch GT-3321667P-01 | C2762984 | Extended | 1 | 6.8656 | J701 |
| 2 | Op-amp, output legs, VSON-10 3 × 3 mm | TI OPA1622IDRCR | C2876559 | Extended | 2 | 1.7871 | U401, U402 |
| 3 | DAC, QFN-28 5 × 5 mm | ESS ES9018K2M | C2688377 | Extended | 1 | 3.4678 | U301 |
| 4 | MCU, USB HS, LQFP-64 10 × 10 mm | WCH CH32V307RCT6 | C2988083 | Extended | 1 | 3.45 | U201 |
| 5 | Op-amp, I/V converters, SOIC-8 | TI OPA2210IDR | C1849415 | Extended | 2 | 1.0035 | U403, U404 |
| 6 | CPLD, QFN-32 4 × 4 mm | AGM AGRV2KQ32 | C18208899 | Extended | 1 | 1.6503 | U202 |
| 7 | Charge pump + LDOs, ±3.8 V, WSON-12 2 × 3 mm | TI LM27762DSSR | C473398 | Extended | 1 | 1.1014 | U501 |
| 8 | Oscillator, 80 MHz | SCTF SX3M80.000B20F30TNN | C5179978 | Extended | 1 | 0.8916 | X201 |
| 9 | Current-limited switch, SOT-23-6 | TI TPS2553DBVR | C55266 | Extended | 1 | 0.5966 | U503 |
| 10 | Oscillator, 22.5792 MHz | SCTF SX2M22.5792M20F30TNN | C5452663 | Extended | 1 | 0.5455 | X203 |
| 11 | Resistor, 2.00 kΩ, 0603, 0.1 %, 25 ppm thin film | YAGEO RT0603BRD072KL | C728588 | Extended | 16 | 0.034 | R401–R416 |
| 12 | Oscillator, 24.576 MHz | YXC OT322524.576MJBA4SL | C2831388 | Extended | 1 | 0.5075 | X202 |
| 13 | Jack, 3.5 mm | HRO PJ-332A-6A | C2848643 | Extended | 1 | 0.2921 | J702 |
| 14 | Crystal, 8 MHz | YXC X50328MSB2GI | C115962 | Basic | 1 | 0.2452 | Y201 |
| 15 | LDO 3.3 V (3V3A), SOT-23-5 | TI LP5907MFX-3.3/NOPB | C80670 | Extended | 1 | 0.2104 | U302 |
| 16 | N-MOSFET, 2N7002, SOT-23 | JSCJ 2N7002 | C8545 | Basic | 12 | 0.0164 | Q501, Q601–Q611 |
| 17 | USB ESD array, SOT-23-6 | ST USBLC6-2SC6 | C7519 | Extended | 1 | 0.1844 | U101 |
| 18 | Capacitor, 10 µF, 0805, 25 V X5R | Samsung CL21A106KAYNNNE | C15850 | Basic | 9 | 0.02 | C413–C416, C512, C507, C509, C510, C511 |
| 19 | LDO 3.3 V (3V3M, 3V3D), SOT-23-5 | Microne ME6211C33M5G-N | C82942 | Extended | 2 | 0.0603 | U502, U504 |
| 20 | Capacitor, 100 nF, 0402, 16 V X7R | Samsung CL05B104KO5NNNC | C1525 | Basic | 36 | 0.003 | C203–C210, C212, C213–C216, C217–C219, C305–C309, C423–C426, C409–C412, C605, C606, C607, C608–C611 |
| 21 | USB-C receptacle | SHOU HAN TYPE-C 16PIN 2MD(073) | C2765186 | Extended | 1 | 0.1 | J101 |
| 22 | Capacitor, 1 µF, 0603, 25 V X5R | Samsung CL10A105KB8NNNC | C15849 | Basic | 16 | 0.006 | C102, C301, C302, C311, C312, C501, C502, C503, C504, C505, C506, C508, C601–C604 |
| 23 | Comparator, SOIC-8 | onsemi LM393DR2G | C7955 | Basic | 1 | 0.09 | U601 |
| 24 | Capacitor, 330 pF, 0603, 50 V C0G | Samsung CL10C331JB8NNNC | C1664 | Basic | 17 | 0.005 | C417–C420, C427–C430, C421, C401–C408 |
| 25 | Diode, BAV99, SOT-23 | Nexperia BAV99,215 | C2500 | Basic | 4 | 0.0211 | D601–D604 |
| 26 | Diode, 5 V, SOD-523 | GOODWORK LESD5D5.0CT1G | C41399463 | Extended | 4 | 0.02 | D701–D704 |
| 27 | LDO 1.3 V (DVDD), SOT-23 | Microne ME6216A13M3G | C236662 | Extended | 1 | 0.06 | U303 |
| 28 | Resistor, 100 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1003TCE | C25741 | Basic | 27 | 0.002 | R105, R303, R305, R421, R508, R509, R510, R511, R512, R513, R501, R515, R502, R503, R517, R504, R505, R506, R601–R604, R607, R608, R613, R616, R623 |
| 29 | Resistor, 1 MΩ, 0402, 1 % | UNI-ROYAL 0402WGF1004TCE | C26083 | Basic | 16 | 0.002 | R107, R605, R606, R617, R624–R627, R628–R631, R632–R635 |
| 30 | Diode, 1N4148WS, SOD-323 | JSCJ 1N4148WS | C2128 | Basic | 2 | 0.016 | D605, D606 |
| 31 | Resistor, 10 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1002TCE | C25744 | Basic | 11 | 0.002 | R103, R104, R201, R202, R214, R306, R507, R609, R610, R611, R618 |
| 32 | Capacitor, 2.2 µF, 0402, 6.3 V X5R | Samsung CL05A225MQ5NSNC | C12530 | Basic | 4 | 0.005 | C211, C303, C304, C310 |
| 33 | LED, Green, 0805 | Hubei KENTO KT-0805G | C2297 | Basic | 1 | 0.0163 | D705 |
| 34 | Ferrite bead, 120 Ω @ 100 MHz, 0603 | Murata BLM18PG121SN1D | C14709 | Basic | 1 | 0.0157 | FB501 |
| 35 | Resistor, 5.1 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF5101TCE | C25905 | Basic | 7 | 0.002 | R101, R102, R434–R437, R518 |
| 36 | Resistor, 33 Ω, 0402, 1 % | UNI-ROYAL 0402WGF330JTCE | C25105 | Basic | 7 | 0.002 | R203–R206, R209, R210, R211 |
| 37 | Ferrite bead, 600 Ω @ 100 MHz, 0603 | Sunlord GZ1608D601TF | C1002 | Basic | 1 | 0.0127 | FB301 |
| 38 | Resistor, 2.2 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF2201TCE | C25879 | Basic | 6 | 0.002 | R212, R213, R207, R208, R701, R702 |
| 39 | Resistor, 2.2 Ω, 0805, 1 % | UNI-ROYAL 0805W8F220JT5E | C17521 | Basic | 4 | 0.003 | R301, R302, R519, R520 |
| 40 | Resistor, 47 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF4702TCE | C25792 | Basic | 5 | 0.002 | R106, R514, R612, R614, R615 |
| 41 | Resistor, 1 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1001TCE | C11702 | Basic | 5 | 0.002 | R423–R426, R433 |
| 42 | Resistor, 10 Ω, 0603, 1 % | UNI-ROYAL 0603WAF100JT5E | C22859 | Basic | 4 | 0.002 | R427–R430 |
| 43 | Resistor, 0 Ω, 0402, jumper | UNI-ROYAL 0402WGF0000TCE | C17168 | Basic | 4 | 0.002 | R417–R420 |
| 44 | LED, Red, 0603 | Hubei KENTO KT-0603R | C2286 | Basic | 1 | 0.0075 | D706 |
| 45 | Resistor, 15 kΩ, 0402, 1 % | UNI-ROYAL 0402WGF1502TCE | C25756 | Basic | 3 | 0.002 | R431, R432, R516 |
| 46 | Capacitor, 33 pF, 0402, 50 V C0G | FH 0402CG330J500NT | C1562 | Basic | 2 | 0.002 | C201, C202 |
| 47 | Capacitor, 4.7 nF, 0402, 50 V X7R | FH 0402B472K500NT | C1538 | Basic | 1 | 0.002 | C101 |

Cost at this revision (fixed 31.7 NTD per US$, JLCPCB Economic PCBA): parts US$138.97 (4,405 NTD), five bare 4-layer PCBs US$10 (estimate), fees US$111.23 → US$260.20 = 8,248 NTD = 1,650 NTD per board. Fees: setup 8.18; stencil 1.53; 19 Extended part types × 3.07 = 58.33 (includes the through-hole 4.4 mm jack and X202, conservatively); SMT joints 723 per board × 0.0016 × 5 = 5.78; X-ray 40 leadless parts (8 per board) × 0.82 = 32.80; through-hole 4.15; Confirm Parts Placement 0.45.

Cost gate: target 1,500 NTD per board (WARN above) and ceiling 1,700 NTD per board as the hard order gate (FAIL above) → status WARNING, 150 NTD over the target and 50 NTD under the ceiling. The ceiling covers every JLCPCB line of the real quote: PCBs, PCBA fees (including any fixture or other assembly line that appears in the quote) and parts; shipping, import tax, programmers, enclosure and proof-of-concept hardware are outside it. When the quote is approved, both the gated subtotal and the total payable (gated subtotal + shipping + tax from the quote) are recorded. The sentence “A PCB assembly fixture is needed to protect and support this part during the assembly process” appears on every JLCPCB part page, including unrelated parts (checked on the OPA2209AIDR page) and an empty part-page template, so it is standard page text and not a part-specific requirement for X202 or the OPA2210; no fixture fee is expected for Economic PCBA, and any fixture line in the real quote counts toward the gate. If JLCPCB does not charge the Extended fee for the through-hole jack (−19.5 NTD per board) and does not X-ray the three oscillators (−26.0 NTD per board each), the run is ≈ 1,552 NTD per board. The figure is a planning estimate; the gate becomes final with the JLCPCB quote after layout.

Pricing rule: US$0.10 minimum per purchasing line for parts under US$0.05 (attrition and minimum-quantity allowance). OPA2210 second source: none is qualified. The OPA2209AIDR (C2057330) has JLCPCB stock 0 at US$6.11 at 1+, and the OPA1612’s common-mode limit (V+) − 2 V = 1.67 V at the minimum rail is below VREF 1.703 V. Mitigation: 12 pieces (10 + 2 spares; the spares ≈ US$2, outside the estimate) are reserved in the JLCPCB parts library now, together with the other low-stock parts; fallback: the LCSC price, ≈ 1,755 NTD per board, above the ceiling (owner decision). Estimate history: 1,540 (one oscillator) → 1,602 (+24.576 MHz) → 1,666 (+22.5792 MHz) → 1,647 (80 MHz MCLK, OPA2210 at the JLCPCB price, protection parts) → 1,650 NTD per board (this revision: protection and power fixes).

By block (NTD per board): USB-C input and sensing 10; USB bridge 234; DAC and supplies 121; I/V and output legs 201; power 64; protection 16; outputs, indicators and board 230 (4.4 mm jack 218). Counts: 136 BOM lines (127 fitted; DNF: U205, R215, R216/R217, R703, J703), 47 purchasing lines, 247 placed parts per board, 19 Extended + 28 Basic part types, 723 SMT + 7 through-hole joints and 8 leadless parts per board; 100-unit reference ≈ 800 NTD per board. Excluded from the ceiling: shipping, import tax, programmers, enclosure and proof-of-concept hardware (Stage 2 ≈ US$90 ≈ 2,860 NTD for two boards).

# Appendix A. Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| v0.1 | 23 Sep 2026 | Diego Yang | Initial issue: datasheet-derived requirements and starting values for schematic capture; companion to Design Specification v0.2 |
| v0.2 | 23 Sep 2026 | Diego Yang | Remaining parts chosen; CH224A replaces CH224Q; TPS7A4901 replaces LT3045; protection circuit defined (LM339 + 2 × HFD4 relays); open items updated |
| v0.3 | 23 Sep 2026 | Diego Yang | Cost-driven redesign after three design reviews, companion to Design Specification v0.4: CH32V307 + AGRV2KQ32 USB bridge, ES9018K2M DAC, 2 × OPA1622 legs, LM27762 ±4.5 V, relay-less 3.3 V latch protection |
| v0.3 (revised) | 24 Sep 2026 | Diego Yang | Current-mode DAC outputs with OPA1678 I/V converters (option B); estimates, checklist, open items and cost updated |
| v0.4 | 24 Sep 2026 | Diego Yang | Fourth review round (R4-A, R4-B, R4-C), companion to Design Specification v0.5: ±3.78 V rails; calibrated 1.75 Vrms per leg; TPS2553 current limit with Default and High modes; switched USB power domains with CC sensing; OPA2197 I/V; single 100 MHz clock with the DAC in asynchronous mode; completed protection with an all-2N7002 latch; corrected ferrite; fixed 1.3 V LDO; ±10 / ±20 mV offset with screening; one-part-per-line BOM; firmware gate |
| v0.4 amendment | 24 Sep 2026 | Diego Yang | Owner decisions of 24 Sep 2026: family oscillators for both rate families, first X202 (24.576 MHz, 48 kHz family) and then X203 (22.5792 MHz, 44.1 kHz family), selected by their enables in front of the CPLD PLL ×2 so that all 8 rates have exact 50:50 bit clocks, because one 100 MHz clock gave a 40:60 bit-clock duty cycle at 352.8/384 kHz against the ES9018K2M 45:55 requirement; own USB firmware and CPLD logic instead of modifying USBAudioToI2S (kept as a known-good comparison). Currents, power-up sequence, block notes, firmware plan, checklist, open items, review rows, both figures, parts table (48 lines) and cost (1,666 NTD per board, ceiling 1,700 NTD) updated |
| v0.5 | 24 Sep 2026 | Diego Yang | Fifth review round (R5-A, R5-B, R5-C), companion to Design Specification v0.6 and Parts List v0.4: 80 MHz MCLK (SCTF SX3M80, below the DAC limit); break-before-make family oscillators with 2.2 kΩ enable pull-downs, node pull-down R214 and a DNF mux option (U205, R215); OPA2210 I/V converters; overload latch (Q610, D606) with firmware clear and new blanking bounds; rails-first-off power-down with the VREF clamp D401 and C421 100 nF; VBUS watchdog 4.10 V; FB− divider with R518 5.1 kΩ and rails incl. the resistor tolerance; Default-mode power by instantaneous peak current; protection envelope and terminal energy recalculated; correlated balanced noise; Stage 2 mandatory as the layout gate; MCU link captured in its own clock domain; 29 test pads, J703 MCLK monitor via R703 and DMM offset screening; checklist, open items, fifth-round disposition, both figures, parts table and cost (1,647 NTD per board, ceiling 1,700 NTD) updated |
| v0.6 | 24 Sep 2026 | Diego Yang | Sixth review round (R6-A, R6-B, R6-C, R6-D), companion to Design Specification v0.7 and Parts List v0.5: fault latch with C605 moved to node F (power-on set, ≥ 1.5 ms set-request stretch), R614/R615 47 kΩ and ≈ 35 µs capture; Q609 slowed to a static interlock (R623, C607); new Q611 kills AVCC_EN; worst-case detector with R601–R604 100 kΩ (R619–R622 removed) and R609/R610 10 kΩ, removal envelope and terminal energy recalculated at the worst case; slow firmware DC tier (R624–R635, C608–C611, four ADC pins) that sets the latch through ANA_FAULT; cause capture and a recovery policy that counts overloads only at the −12 dB ceiling; VREF tracks AVCC (C421 330 pF; D401 and C422 removed) with the DAC first off and the rails last and the legs enabled while VREF = 0; USB suspend a v1 requirement (≤ 2.5 mA within 10 ms, power-down ≤ 4 ms); LDO dissipation table with the R519/R520 dropper and C512; both DAC centre voltages with the R433 option; OPA2210 output swing; DNF R216/R217; SPI link with CS framing; SDATA/LRCLK on the falling BCLK edge; 31 test pads; Stage 2 with the real regulators (≈ 2,860 NTD); checklist, open items, sixth-round disposition, Figure 2, parts table (47 lines) and cost (1,650 NTD per board, ceiling 1,700 NTD) updated |